# Mini-review Article

Comparative Evaluation of GIS-Based and GIS-AHP Approaches for Flood Susceptibility Mapping: Methodological Insights and Climate Resilience Implications

ABSTRACT

Floods remain among the most recurrent and destructive natural hazards worldwide, with their frequency and magnitude projected to increase under global climate change. Robust and context-sensitive flood susceptibility mapping is therefore essential for advancing disaster risk reduction, adaptive land use planning, and climate resilience. Geographic Information Systems (GIS) provide a spatially explicit platform for integrating hydrological, geomorphological, and anthropogenic data. Building on this foundation, multi-criteria decision analysis techniques, particularly the Analytical Hierarchy Process (AHP), offer structured frameworks for incorporating expert judgment and socio-environmental complexity. This paper presents a comparative evaluation of two methodological paradigms: a GIS-based statistical model employing the Relative Frequency Ratio (RFR), and a GIS–AHP framework that combines spatial analysis with expert-driven weighting of hydrological, morphometric, land cover, and anthropogenic parameters. The GIS-RFR approach demonstrates high predictive accuracy through empirical correlations between flood inventories and conditioning factors, while the GIS–AHP model enhances inclusivity by integrating multiple dimensions of environmental and human drivers, validated against remote sensing data. Comparative analysis highlights the methodological trade-offs between the objectivity and reproducibility of statistical modeling versus the adaptability and contextual sensitivity of expert-based decision frameworks. The synthesis emphasizes that no single approach is universally superior, rather, methodological choice should be guided by data availability, regional hydrological conditions, and policy priorities. Future directions point toward hybrid frameworks that integrate statistical rigor, expert knowledge, and machine learning, thereby reducing uncertainty and enhancing the actionable value of susceptibility maps.

Keywords: Climate change; disaster risk reduction; flood susceptibility; hydrological modeling; remote sensing; spatial planning

1. INTRODUCTION

Floods are among the most frequent and destructive natural hazards, causing extensive socio-economic damage, environmental degradation, and loss of human lives globally (Ullah & Zhang, 2020; Nsangou et al., 2022). Each year, approximately 170 million people are affected, making floods one of the most catastrophic climate-related disasters (Swain, Singha, & Nayak, 2020; Khosravi et al., 2019). Flood is also characterized as hydrological hazards in which water overflows onto normally dry land, floods pose substantial threats to human life, agricultural productivity, infrastructure, communication networks, cultural heritage, and ecosystems. The severity and nature of their impacts are influenced by factors such as geomorphology, land use patterns, and the effectiveness of risk management strategies (Samanta et al., 2018; Zou et al., 2013; Calil et al., 2015).

Climate change has exacerbated flood risks by intensifying extreme precipitation events, altering hydrological cycles, and increasing rainfall variability, particularly in monsoon-dependent regions (Munir, 2022; Murali & Srinivasan, 2025; Kundzewicz et al., 2014). Moreover, rapid urbanization, deforestation, and unplanned land use change have compounded the impacts of flooding by reducing infiltration, increasing runoff, and exposing vulnerable populations and assets to recurrent flood events (Swain, Singha, & Nayak, 2020; Pimenta et al., 2025). Developing nations in Asia, Africa, and Latin America face disproportionate risks due to fragile infrastructure, inadequate disaster preparedness, and limited adaptive capacity (Ullah & Zhang, 2020; Addis, 2023; Hirabayashi et al., 2013; Winsemius et al., 2016).

To address these challenges, flood susceptibility mapping has become a critical tool for disaster risk reduction (DRR), sustainable land use planning, and climate-resilient development (Swain, Singha, & Nayak, 2020; Pimenta et al., 2025). Flood susceptibility maps identify areas at varying levels of risk by integrating hydrological, topographic, climatic, and anthropogenic variables (Ullah & Zhang, 2020; Rahman et al., 2023). Advances in Geographic Information Systems (GIS) and Remote Sensing (RS) have transformed flood modeling by enabling the integration of spatially explicit datasets, high-resolution imagery, and digital elevation models (Garg et al., 2024; Megahed et al., 2023). These tools allow for more accurate assessment of flood-prone zones, especially in data-scarce regions where conventional hydrological and hydraulic models are difficult to implement (Nsangou et al., 2022; Tehrany, Pradhan, & Jebur, 2014). Moreover, combining machine learning techniques with multi-criteria decision-making approaches has been shown to significantly improve the precision and reliability of flood susceptibility assessments across varied landscapes (Al-Ruzouq et al., 2024; Chaulagain et al., 2023; Ghassan Abdo et al., 2024).

Beyond GIS-based statistical approaches, Multi-Criteria Decision Analysis (MCDA) techniques have been widely applied to capture the multidimensional nature of flood risk (Swain, Singha, & Nayak, 2020; Pimenta et al., 2025). Among these, the Analytical Hierarchy Process (AHP) remains one of the most frequently used frameworks, enabling structured weighting of diverse flood conditioning factors (Mitra, Saha, & Das, 2022; Swain, Singha, & Nayak, 2020). AHP combines expert knowledge with quantitative datasets, offering flexibility for regional-scale flood risk assessments (Pimenta et al., 2025; Murali & Srinivasan, 2025). However, its reliance on expert judgment introduces subjectivity and potential bias, warranting comparative analyses with purely data-driven approaches (Ullah & Zhang, 2020; Megahed et al., 2023). Statistical models such as the Relative Frequency Ratio (RFR) method, for example, offer a robust alternative by empirically quantifying correlations between flood occurrences and causative factors (Ullah & Zhang, 2020; Addis, 2023; Tehrany, Pradhan, & Jebur, 2014).

This review focuses on a comparative evaluation of two case studies that exemplify these distinct methodological paradigms. Ullah and Zhang (2020) employed a GIS-based RFR approach to delineate flood susceptibility zones in the Panjkora River Basin, Pakistan, integrating parameters such as elevation, slope, land use, NDVI, topographic wetness index, drainage density, and rainfall. In contrast, Murali and Srinivasan (2025) applied a GIS–AHP framework to the Chalakudy River Basin, India, using multiple hydrological, morphometric, land cover, and anthropogenic factors, with weights assigned through expert-based pairwise comparisons. Together, these studies provide an opportunity to critically assess the methodological trade-offs between empirical-statistical and expert-driven MCDA approaches.

The research addresses three core questions: (i) How do GIS-RFR models compare with GIS–AHP frameworks in terms of accuracy, robustness, and applicability across diverse landscapes? (ii) What are the methodological implications of relying on empirical correlations versus expert-derived weightings? (iii) How can insights from these approaches advance climate resilience and inform integrated flood risk management strategies? By answering these questions, this study contributes to the growing body of scholarship linking geospatial technologies, climate adaptation, and sustainable environmental management (Rahman et al., 2023; Swain, Singha, & Nayak, 2020; Pimenta et al., 2025).

2. SUMMARY OF REVIEWED ARTICLES

2.1 GIS-Based RFR Method in Panjkora River Basin, Pakistan

Ullah and Zhang (2020) employed GIS and Relative Frequency Ratio (RFR) model to delineate flood-prone areas in the Panjkora River Basin in the eastern Hindu Kush, Pakistan. The study utilized an inventory of 154 flood locations from the 2010 flood events, with 70% of the locations used for training and 30% for validation. Eight conditioning factors were considered: slope, elevation, land use/land cover (LULC), Normalized Difference Vegetation Index (NDVI), topographic wetness index (TWI), drainage density, curvature, and rainfall. These variables were processed into GIS-based thematic layers, resampled at 30×30 m resolution, and reclassified using the natural break method. The RFR model quantified the relationship between each factor and flood occurrences, a technique widely recognized in susceptibility mapping due to its statistical simplicity and interpretability (Tehrany et al., 2017; Rahmati et al., 2016). Similar approaches have been applied in Iran and India with effective results in integrating hydrological and terrain factors (Khosravi et al., 2019; Samanta et al., 2018). Validation using Receiver Operating Characteristic (ROC) analysis yielded Area Under Curve (AUC) values of 82.04% (success) and 84.74% (prediction), confirming model reliability and predictive capacity for flood susceptibility mapping in data-scarce mountainous environments (Ullah & Zhang, 2020).

2.2 GIS-AHP Method in Chalakudy River Basin, India

Murali and Srinivasan (2025) applied a GIS–AHP approach to assess flood susceptibility in the Chalakudy River Basin, Kerala, India. The study incorporated 12 key factors grouped into hydrological (precipitation, river network density), morphometric (elevation, slope, distance from rivers), land cover dynamics (LULC, NDVI, NDWI, SAVI), and anthropogenic parameters (distance from roads, settlements). Weights were assigned to each parameter using pairwise comparisons in the AHP framework, ensuring a Consistency Ratio (CR) of 4.8%, below the 10% threshold, consistent with Saaty’s (1980) recommendations for decision-making reliability. Flood susceptibility maps were generated through weighted linear combination and validated against Sentinel-1 SAR imagery of the 2018 Kerala floods, a technique widely recognized for capturing real-time inundation patterns (Alfieri et al., 2018; Li et al., 2019). Results showed 7.41% of the basin as very highly susceptible and 47.69% as highly susceptible to flooding, underscoring the dominant influence of morphometric and anthropogenic drivers. The integration of AHP with remote sensing and GIS reflects broader trends in flood risk modeling, where multi-criteria decision analysis improves spatial precision and policy relevance (Agnihotri et al., 2019; Kazakis et al., 2015). Such approaches demonstrate the utility of combining statistical consistency with geospatial realism in flood hazard assessment.

3. COMPARATIVE ANALYSIS

The two studies exemplify contrasting yet complementary approaches to flood susceptibility mapping. While Ullah & Zhang (2020) relied on a data-driven, statistical model, Murali & Srinivasan (2025) integrated expert judgment through AHP to incorporate multiple data dimensions. This section compares their methodologies, datasets, validation strategies, and findings.

3.1. Methodological Framework

The GIS-RFR method (Ullah & Zhang, 2020) is a purely statistical approach that establishes probabilistic correlations between past flood events and causative factors. By contrast, the GIS-AHP method (Murali & Srinivasan, 2025) incorporates expert judgment in assigning weights to parameters, introducing subjectivity but allowing multi-criteria decision analysis (MCDA) flexibility.

3.2.  Data Sources and Parameters

Both studies employed remote sensing and GIS datasets for flood susceptibility analysis, yet they differed in the selection and categorization of parameters. Ullah and Zhang (2020) implemented a data-driven GIS-RFR approach, emphasizing the use of historical flood inventories and geomorphological parameters. Their study incorporated curvature alongside standard hydrological and morphometric factors, providing a statistically robust framework for model training and validation using 154 historical flood locations. In contrast, Murali and Srinivasan (2025) adopted a GIS-AHP methodology that integrated both biophysical and anthropogenic variables. This included multiple vegetation indices, such as the Normalized Difference Vegetation Index (NDVI), Normalized Difference Water Index (NDWI), and Soil Adjusted Vegetation Index (SAVI), along with proximity measures to roads and settlements. The inclusion of anthropogenic factors in the GIS-AHP model highlights the increasing recognition of human-induced influences on flood dynamics.

Table 1. Comparative summary of parameters

| Parameter Category | GIS-RFR (Ullah & Zhang, 2020) | GIS-AHP (Murali & Srinivasan, 2025) |
| --- | --- | --- |
| Hydrological | Rainfall | Rainfall, River Network Density |
| Morphometric | Elevation, Slope, Drainage Density, Curvature | Elevation, Slope, Distance from Rivers, TWI |
| Land Use / Vegetation | LULC, NDVI | LULC, NDVI, NDWI, SAVI |
| Anthropogenic | None | Distance to Roads, Distance to Settlements |
| Other | Flood inventory (154 locations) | Expert judgment weights (AHP) |

This comparison illustrates how GIS-RFR relies primarily on quantitative, historically observed datasets, ensuring objectivity and reproducibility, whereas GIS-AHP incorporates expert knowledge and human-environment interactions, offering flexibility in regions with limited historical flood records. The choice of parameters directly affects the sensitivity and predictive accuracy of the flood susceptibility maps, underlining the importance of tailored parameter selection based on local environmental and socio-economic conditions.

3.3. Validation and Approaches

Ullah & Zhang validated their results statistically using ROC and AUC, achieving strong predictive power (>80%). This approach is objective and reproducible, especially in areas with detailed historical flood records. Their results showed that about 15% of the basin fell into high and very high flood susceptibility zones, highlighting areas most at risk based on past events. On the other hand, Murali & Srinivasan used Sentinel-1 SAR imagery, enabling real-world validation of inundation patterns. This provided realistic spatial results, though it was limited to a single flood event. The model showed a consistency ratio of 4.8%, with around 55% of the basin classified as high and very high flood risk, demonstrating strong agreement with observed flood patterns.

Table 2. Comparative validation results

| Study | Validation Method | Metric | Flood Susceptibility High Zones |
| --- | --- | --- | --- |
| Ullah & Zhang (2020) | ROC Curve (AUC) | AUC Success: 82.04%, Prediction: 84.74% | 15% of basin (High & Very High) |
| Murali & Srinivasan (2025) | Sentinel-1 SAR Flood Inundation | Consistency Ratio 4.8%, spatial match with SAR imagery | 55% of basin (High & Very High) |

The comparison highlights the trade-offs between statistical and remote-sensing-based validation methods. Statistical validation offers robust quantitative assessment and general applicability, whereas SAR-based validation enhances spatial accuracy and realism but may be limited in temporal scope. These differences underscore the importance of selecting validation techniques aligned with study objectives, available data, and regional flood dynamics.

4. SYNTHESIS AND IMPLICATIONS

GIS-RFR and GIS–AHP approaches highlight how statistical and expert-driven flood susceptibility models each bring unique strengths and limitations. GIS-RFR shows that data-driven statistical models can provide highly accurate predictions when reliable historical flood records are available. Its main advantages lie in their objectivity, reproducibility, and ease of validation using metrics like ROC-AUC. This makes RFR and similar statistical approaches particularly useful in regions with well-documented hydrological data, where planners and decision-makers need solid, quantitative guidance for infrastructure development, zoning, and emergency management.

In contrast, the GIS–AHP framework emphasizes flexibility and contextual sensitivity by integrating diverse datasets, including anthropogenic factors such as proximity to settlements and roads, alongside multiple hydrological, morphometric, and vegetation indices. This enables a more comprehensive assessment of flood susceptibility that accounts for human-environment interactions, making it particularly valuable in rapidly urbanizing or data-sparse regions. However, its reliance on expert judgment introduces subjectivity, potentially affecting reproducibility and transferability. Ensuring transparency and incorporating participatory approaches in the weighting process can mitigate these limitations, enhancing credibility for policy and planning applications.

Both studies show that there is no single method that works best in all situations. Instead, the choice of methodology should depend on factors such as data availability, landscape features, and regional management priorities. Statistical models provide rigor and objectivity, while MCDA-based approaches offer flexibility and the ability to account for socio-environmental complexity. Combining these approaches in hybrid frameworks can capture the strengths of both: for example, integrating GIS–AHP with machine learning or ensemble statistical models can reduce subjectivity, improve predictive accuracy, and help quantify uncertainty.

From a broader perspective, the findings carry significant implications for climate adaptation and disaster risk reduction. Incorporating dynamic factors such as climate change projections, land use trends, and socio-economic vulnerability into flood susceptibility assessments can improve the relevance and actionability of these models for long-term resilience planning. Such integrative approaches align with sustainable spatial planning objectives, enabling policymakers, urban planners, and emergency managers to prioritize interventions, allocate resources efficiently, and enhance community resilience against increasingly frequent and severe flood events.

Ultimately, this comparative evaluation reinforces the need for methodological pluralism in flood risk assessment. By bridging empirical rigor with expert-informed contextual insights, researchers and practitioners can produce susceptibility maps that are not only scientifically robust but also socially and policy-relevant, supporting climate-resilient development and evidence-based flood governance.

4. Conclusion

This comparative review demonstrates that GIS-RFR and GIS–AHP approaches offer complementary strengths for flood susceptibility mapping. GIS-RFR provides objectivity, reproducibility, and high predictive accuracy, particularly in data-rich contexts, whereas GIS–AHP introduces flexibility and contextual sensitivity by integrating anthropogenic and environmental variables, proving valuable in data-scarce or rapidly changing regions. No single method is universally optimal, methodological choice should reflect data availability, landscape characteristics, and policy objectives. Hybrid frameworks that combine statistical rigor with expert-informed weighting and machine learning have the potential to reduce subjectivity, enhance predictive performance, and quantify uncertainty. Integrating climate change projections, land use dynamics, and socio-economic vulnerability into flood modeling is essential for long-term resilience planning. Overall, flood susceptibility mapping serves as a strategic tool for evidence-based decision-making, enabling policymakers, planners, and communities to prioritize interventions, allocate resources efficiently, and strengthen resilience against increasingly frequent and severe flood events.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

Author(s) hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.)   and   text-to-image generators have been used during the writing or editing of this manuscript

References

Addis, A. (2023). GIS-based flood susceptibility mapping using frequency ratio and information value models in Ethiopia. Natural Hazards Research, 3(2), 247–256. https://doi.org/10.1016/j.nhres.2023.02.003

Agnihotri, A. K., Ohri, A., Gaur, S., Shivam, Das, N., & Mishra, S. (2019). Flood inundation mapping and monitoring using SAR data and its impact on Ramganga River in Ganga basin. Environmental monitoring and assessment, 191(12), 760.

Alfieri, L., Bisselink, B., Dottori, F., Naumann, G., de Roo, A., Salamon, P., Wyser, K., & Feyen, L. (2017). Global projections of river flood risk in a warmer world. Earth’s Future, 5(2), 171–182. https://doi.org/10.1002/2016EF000485

Al-Ruzouq, R., Shanableh, A., Jena, R., Gibril, M. B. A., Hammouri, N. A., & Lamghari, F. (2024). Flood susceptibility mapping using a novel integration of multi-temporal Sentinel-1 data and eXtreme deep learning model. Geoscience Frontiers, 15(3), 101780. https://doi.org/10.1016/j.gsf.2024.101780

Calil J, Beck MW, Gleason M, Merrifield M, Klausmeyer K, Newkirk S. Aligning natural resource conservation and flood hazard mitigation in California. PLoS One. 2015; 10: 1–14. https://doi.org/10.1371/ journal. pone.0132651 PMID: 26200353

Garg, S., Dasgupta, A., Motagh, M., Martinis, S., & Selvakumaran, S. (2024). Unlocking the full potential of Sentinel-1 for flood detection in arid regions. Remote Sensing of Environment, 315, 114417. https://doi.org/10.1016/j.rse.2024.114417

Ghassan Abdo, H., Zeng, T., Alshayeb, M., Prasad P., Ahmed M., Albanai J. & ALhabri M. (2024). Multi-criteria analysis and geospatial applications-based mapping flood vulnerable areas: A case study from the eastern Mediterranean. Natural Hazards, 121(1), 1003-1031

Hirabayashi, Y., Mahendran, R., Koirala, S., Konoshima, L., Yamazaki, D., Watanabe, S., Kim, H., & Kanae, S. (2013). Global flood risk under climate change. Nature Climate Change, 3(9), 816–821

Kazakis, N., Kougias, I., & Patsialis, T. (2015). Assessment of flood hazard areas at a regional scale using an index-based approach and Analytical Hierarchy Process: Application in Rhodope–Evros region, Greece. Science of the Total Environment, 538, 555–563.

Khosravi, K., Nohani, E., Maroufinia, E., & Pourghasemi, H. R. (2016). A GIS-based flood susceptibility assessment and its mapping in Iran: A comparison between frequency ratio and weights-of-evidence bivariate statistical models with multi-criteria decision-making technique. Natural Hazards, 83(2), 947–987. https://doi.org/10.1007/s11069-016-2357-2

Khosravi, K., Shahabi, H., Pham, B. T., Adamowski, J., Shirzadi, A., Pradhan, B., Dou, J., Ly, H.-B., Gróf, G., Ho, H. L., Hong, H., Chapi, K., & Prakash, I. (2019). A comparative assessment of flood susceptibility modeling using multi-criteria decision-making analysis and machine learning methods. Journal of Hydrology, 573, 311–323. https://doi.org/10.1016/j.jhydrol.2019.03.073

Kundzewicz, Z. W., Kanae, S., Seneviratne, S. I., Handmer, J., Nicholls, N., Peduzzi, P., Mechler, R., Bouwer, L. M., Arnell, N., Mach, K., Muir-Wood, R., Brakenridge, G. R., Kron, W., Benito, G., Honda, Y., Takahashi, K., & Sherstyukov, B. (2014). Flood risk and climate change: Global and regional perspectives. Hydrological Sciences Journal, 59(1), 1–28.

Li, W., Lin, K., Zhao, T., Lan, T., Chen, X., Du, H., & Chen, H. (2019). Risk assessment and sensitivity analysis of flash floods in ungauged basins using coupled hydrologic and hydrodynamic models. Journal of Hydrology, 572, 108-120

Megahed, H. A., Abdo, A. M., AbdelRahman, M. A. E., Scopa, A., & Hegazy, M. N. (2023). Frequency ratio model as tools for flood susceptibility mapping in urbanized areas: A case study from Egypt. Applied Sciences, 13(16), 9445. https://doi.org/10.3390/app13169445

Mitra, R., Saha, P., & Das, J. (2022). Assessment of the performance of GIS-based analytical hierarchical process (AHP) approach for flood modelling in Uttar Dinajpur district of West Bengal, India. Geomatics, Natural Hazards & Risk, 13(1), 2183–2226. https://doi.org/10.1080/19475705.2022.2112094

Munir, A. (2022). Flood susceptibility assessment using frequency ratio modeling approach in Northern Sindh and Southern Punjab, Pakistan. Polish Journal of Environmental Studies, 31(2), 1653–1664. https://doi.org/10.15244/pjoes/145607

Murali, K. V., & Srinivasan, K. (2025). GIS and AHP based flood susceptibility mapping: A case study of Chalakudy River Basin, Kerala, India. International Journal of Environment and Climate Change, 15(9), 18–34. https://doi.org/10.9734/ijecc/2025/v15i94992

Nsangou, D., Kpoumié, A., Mfonka, Z., Ngouh, A. N., Fossi, D. H., Jourdan, C., Zobo Mbele, H., Mouncherou, O. F., Vandervaere, J.-P., & Ndam Ngoupayou, J. R. (2022). Urban flood susceptibility modelling using AHP and GIS approach: Case of the Mfoundi watershed at Yaoundé in the South-Cameroon plateau. Scientific African, 15, e01043. https://doi.org/10.1016/j.sciaf.2021.e01043

Pimenta, L., Duarte, L., Teodoro, A. C., Beltrão, N., Gomes, D., & Oliveira, R. (2025). GIS-based flood susceptibility mapping using AHP in the urban Amazon: A case study of Ananindeua, Brazil. Land, 14(8), 1543. https://doi.org/10.3390/land14081543

Rahman, Z. U., Ullah, W., Bai, S., Ullah, S., Jan, M. A., Khan, M., & Tayyab, M. (2023). GIS-based flood susceptibility mapping using bivariate statistical model in Swat River Basin, Eastern Hindukush region, Pakistan. Frontiers in Environmental Science, 11, 1178540. https://doi.org/10.3389/fenvs.2023.1178540

Rahmati, O., Haghizadeh, A., & Stefanidis, S. (2016). Assessing the accuracy of GIS-based analytical hierarchy process for watershed prioritization: Gorganrood River Basin, Iran. Water Resources Management, 30(3), 1131–1150

Samanta, R. K., Bhunia, G. S., Shit, P. K., & Pourghasemi, H. R. (2018). Flood susceptibility mapping using geospatial frequency ratio technique: A case study of Subarnarekha River Basin, India. Model Earth Systems and Environment, 4, 395–408. https://doi.org/10.1007/s40808-018-0427-z

Swain, K. C., Singha, C., & Nayak, L. (2020). Flood susceptibility mapping through the GIS-AHP technique using clouds. ISPRS International Journal of Geo-Information, 9(12), 720. https://doi.org/10.3390/ijgi9120720

Tehrany, M. S., Pradhan, B., & Jebur, M. N. (2014). Flood susceptibility mapping using a novel ensemble weights-of-evidence and support vector machine models in GIS. Journal of Hydrology, 512, 332–343. https://doi.org/10.1016/j.jhydrol.2014.03.008

Ullah, K., & Zhang, J. (2020). GIS-based flood hazard mapping using relative frequency ratio method: A case study of Panjkora River Basin, eastern Hindu Kush, Pakistan. PLoS ONE, 15(3), e0229153. https://doi.org/10.1371/journal.pone.0229153

Winsemius, H. C., Aerts, J. C. J. H., van Beek, L. P. H., Bierkens, M. F. P., Bouwman, A., Jongman, B., Kwadijk, J. C. J., Ligtvoet, W., Lucas, P. L., van Vuuren, D. P., & Ward, P. J. (2016). Global drivers of future river flood risk. Nature Climate Change, 6(4), 381–385. https://doi.org/10.1038/nclimate2893

Zou, Q., Zhou, J., Zhou, C., Song, L., & Guo, J. (2013). Comprehensive flood risk assessment based on set pair analysis-variable fuzzy sets model and fuzzy AHP. Stochastic Environmental Research and Risk Assessment, 27, 525–546. https://doi.org/10.1007/s00477-012-0598-5