Original Research Article

COMPARATIVE ANALYSIS OF RAINFALL HEIGHTS OVER NIGERIA GEO-CLIMATIC ZONES FOR SATELLITE COMMUNICATION AT KU BAND

# ABSTRACT

Nigeria's diverse geo-climatic zones present varying challenges for satellite communications, particularly at Ku-band frequencies (12-18 GHz). Rainfall height is a critical parameter that directly influences signal propagation and the link margin, as such, must be taken into account when designing Ku-band satellite communication systems to ensure reliable and efficient communication. This study presents a comparative analysis of rainfall heights over Nigeria geo-climatic zones and the impact on Ku band satellite communication system. Three years data was analyzed from four locations one from each geo-climatic region of Nigeria using a purposeful sampling technique. The study also presents a monthly zero degree isotherm height (km) across the selected locations. The result shows seasonal variations of the Zero Degree Isotherm Height (ZDH) over Nigeria geo-climatic zones with lower heights in January (4.1 to 4.2km) and December (around 4.2km), and peak heights during the middle months of the year (July-September), reaching approximately 4.40-4.45km. ESUT and PJAA shows slightly higher values than FUTA and FUGUS most of the year, particularly in August where it reaches about 4.45km. The distribution of measured ZDIH across the zones varies (4.109km – 4.461km) as against 4.5km predicted by ITU-R corresponding to 4% difference. The measured rain height ranges from 4.68km to 4.81km across the zones as against 4.85km predicted by ITU-R for the same zones. ITU-R model maintains a constant height throughout the period, showing no seasonal variation. This implies that, ITU-R model overestimated the Nigerian Zero Degree Isotherm Height. Therefore, this substantial difference must be put into consideration as a critical parameter in Meteorology and for estimating rain attenuation in satellite communication systems in Nigeria.

Keywords: Rainfall height, Ku-band, satellite communications, geo-climatic zones, zero isothermal height

## 1. INTRODUCTION

Rainfall presents a significant challenge for satellite communications, particularly in the Ku-band frequency range (Ojo, & Rotimi, 2015). In tropical region like Nigeria, understanding the variations in rainfall heights across different geo-climatic zones is crucial for designing reliable satellite communication systems (Ojo et al., 2022). The understanding of rainfall pattern and the impact on satellite communication is a global concern for both scientist and engineers. The application of Satellite communication systems operating at frequencies (12-18 GHz), include telecommunications, broadcasting, and navigation. However, one of the major challenges facing Ku-band satellite communications is the impact of rain attenuation on signal quality and reliability. Rain attenuation is the loss of signal power due to absorption and scattering by raindrops, and it is a serious problem for Ku-band systems, particularly in tropical regions with high rainfall rates (Ojo et al., 2022). Rain height, which is the vertical extent of precipitation from the surface, is crucial in determining the extent of rain attenuation. Increase in rain height, increases the signal attenuation, leading to a decrease in signal quality and reliability (Ippolito, 2008). Hence, understanding the impact of rain height on rain attenuation at Ku-band satellite communications is very important for designing and operating reliable and efficient satellite communication systems (Acharya, 2017). A higher rain height results in a longer path length and increasing attenuation. It is used to calculate the effective path length, which is then used to estimate rain attenuation. Inaccurate rain height estimation can lead to underestimation or overestimation of attenuation. Rain height affects the link budget, which determines the signal-to-noise ratio and overall system performance (Orji et al., 2024).

#### Figure 1: Schematic diagram of Earth-Space path showing input parameters

For ITU-R rain attenuation prediction model (ITU-R, 2021). (A - Frozen height; B - Rain height; C: Liquid precipitation; D: Earth-space path)

In the case of Nigeria, the different geo-climatic zones suggest that there will be variations in key factors like rainfall, temperature, elevation, and latitude. However, ITU-R model did not take into consideration the variations that may come as a result of the differences in each climate zone. Therefore, the objective of this study is to carry out analysis of rainfall heights over Nigeria geo-climatic zones for satellite communication at Ku band and to compare ITU-R model with the measured data of the selected locations in Nigeria. The findings of this study will be useful for the development of more accurate rain attenuation prediction models and improve satellite communication system design especially in Nigeria.

2. MATERIALS AND METHODS

  - MATERIALS

The TRMM-PR data product used for this work is the PR 2A23 version 7. The algorithm for extraction and processing 2A23 v7 is contained in the product manual (TRMM-PR manual v7, 2011).

  - METHODS

2.2.1 Study Location

Nigeria is primarily divided into four geo-climatic zones which includes Tropical Monsoon/Rainforest in the south (characterized by year-round high rainfall typically over 2,000 mm, warm and humid. There's a double maxima peak in rainfall, with a short "August break"), Tropical Continental/Savannah in the central regions (has distinct wet season April to September and dry season December to March, with temperatures generally above 18°C), Sahel/Hot Semi-Arid in the far north (hot, dry with a short rainy season from June to September. Annual rainfall is typically low, ranging from 500 mm to 750 mm), and Alpine or Montane in highland areas (typically above 1,520 meters like the Jos Plateau with cooler temperatures) (Edokpa et al., 2024; Ogolo & Mathew, 2022; World Bank Group, 2020).

2.2.2 Method of Data Collection / In-situ Measurement

Rain flag, rain type, bright band height (BBH), bright band status, Intensity, Freezing Height Levels (FHL) also known as Zero Degree Isotherm Heights (ZDIH) h0 and other rain-related parameters were extracted from TRMM-PR. Algorithm 2A23 helps to identify rain type, provides information on freezing height level such as detection of presence of bright band in rain echoes and also the bright band height when it exists.

The study analyzed rainfall height data collected across thirty-six months from four strategic locations in Nigeria. The measurements were recorded in kilometers (km) with tracked seasonal variations throughout the period. To obtain a representative sample of the entire population, a purposeful sampling technique was adopted to select one location from each of the geo-climatic zones where the Weather Stations are installed. This translates to four locations including Akure (FUTA), Enugu (ESUT), Zamfara (FUGUS), and Abuja (PJAA), and. The map of Nigeria showing the geo-climatic zones and the sites where the weather stations ware located is shown in Figure 2 and the location codes with their corresponding GPS locations is presented in Table 1.

Figure 2. Map of Nigeria showing the geo-climatic zones and the sites where the weather stations are located (Lawal et al 2021)

Table 1: Location codes with their corresponding GPS locations

| S/N | GEO-CLIMATIC LOCATION | NAME | STATE CODE | GPS L0CATION |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | LONGITUDE | LATITUDE |
| 1 | SAHEL | GUSAU | FUGUS | 12.100N | 06.150 E |
| 2 | GUINEA SAVANNAH | ABUJA | PJAA | 08.970N | 07.370E |
| 3 | DERIVED SAVANNAH | AKURE | FUTA | 07.310N | 05.140E |
| 4 | COASTAL | ENUGU | ESUT | 07.270N | 06.250E |

The satellite signal was received by the 90 cm offset parabolic dish, at an elevation angle of 42o to the down converter and the beacon receiver. The precipitate measurement was made up of the Davis Vantage Vue weather station, which logs rain rate and other meteorological parameters at the required one-minute integration time. Both satellite signal and precipitation measurement ware done concurrently. The Davis station has an Integrated Sensor Suit (ISS), which is collocated with the outdoor unit of the beacon signal measurement setup i.e. the offset parabolic antenna. Data was collected from January, 2019 to December 2021 in all the locations. During the thirty-six months of experiment, attenuation data were collected for a period of 80% of the time, while rainfall data were collected for about 85% of the time from all study locations except at FUGUS where measurements are for a period of 70% of the time, while rainfall data were collected for about 75% of the time for attenuation and rain rate measurement, respectively. Figure 3 and 4 shows the typical experimental set up used to concurrently measure and record rain-rate, rain attenuation, signal loss, and noise threshold in FUTA and PJAA.

Figure 3. FUTA site weather station’s outdoor and indoor facility

Figure 4. PJAA site weather station’s outdoor and indoor facility

## 2.2.3 Measurement of Rain Height

The rain height affects the path length through the rainfall as shown in Figure 1. The rain height is determined from the 0°C isotherm height, which is the height above the Earth’s surface where the temperature is 0°C. The ZDIH depends on the season and weather conditions. The height associated with the 0°C isotherm is the threshold between the air layers with temperatures above 0°C at lower altitudes and those with temperatures below freezing at higher altitudes. According to ITU-R (2013), ZDIH is related to rain height as follows

hR = ho+0.36km										(1)

## 3. RESULTS

The results of this study highlights the average rainfall accumulation, the zero isothermal heights and rain heights for the various geo-climatic zones in Nigeria as presented in Tables 2 and 3 and Figure 5.

Figure 5. Average rainfall accumulation for the period of 36 months of study

Table 2: Monthly Average ZDIH over the Study Locations

| MONTHS | FUTA (km) | ESUT (km) | FUGUS (km) | PJAA (km) | ITU-R (km) |
| --- | --- | --- | --- | --- | --- |
| JAN | 4.10 | 4.20 | 4.10 | 4.20 | 4.50 |
| FEB. | 4.20 | 4.30 | 4.20 | 4.30 | 4.50 |
| MAR. | 4.30 | 4.30 | 4.30 | 4.30 | 4.50 |
| APR. | 4.35 | 4.40 | 4.35 | 4.40 | 4.50 |
| MAY. | 4.40 | 4.40 | 4.40 | 4.40 | 4.50 |
| JUN. | 4.45 | 4.45 | 4.40 | 4.45 | 4.50 |
| JUL. | 4.45 | 4.45 | 4.40 | 4.45 | 4.50 |
| AUG. | 4.45 | 4.45 | 4.35 | 4.45 | 4.50 |
| SEP. | 4.40 | 4.40 | 4.40 | 4.40 | 4.50 |
| OCT. | 4.35 | 4.35 | 4.35 | 4.35 | 4.50 |
| NOV. | 4.30 | 4.30 | 4.30 | 4.30 | 4.50 |
| DEC. | 4.20 | 4.20 | 4.20 | 4.20 | 4.50 |
| Mean | 4.33 | 4.35 | 4.31 | 4.35 | 4.50 |

Figure 6. Distribution of 0o C isotherm height over the study locations

Figure 7. The mean ZIDH over the geo-climatic zones vs ITU-R prediction

Table 3: Monthly mean rain height over the study locations

| MONTHS | FUTA (km) | ESUT (km) | FUGUS (km) | PJAA (km) | ITU-R (km) |
| --- | --- | --- | --- | --- | --- |
| JAN | 4.46 | 4.56 | 4.46 | 4.56 | 4.85 |
| FEB. | 4.56 | 4.66 | 4.56 | 4.66 | 4.85 |
| MAR. | 4.66 | 4.66 | 4.66 | 4.66 | 4.85 |
| APR. | 4.71 | 4.76 | 4.71 | 4.76 | 4.85 |
| MAY. | 4.76 | 4.76 | 4.76 | 4.76 | 4.85 |
| JUN. | 4.81 | 4.81 | 4.76 | 4.81 | 4.85 |
| JUL. | 4.81 | 4.81 | 4.76 | 4.81 | 4.85 |
| AUG. | 4.81 | 4.81 | 4.71 | 4.81 | 4.85 |
| SEP. | 4.76 | 4.76 | 4.76 | 4.76 | 4.85 |
| OCT. | 4.71 | 4.71 | 4.71 | 4.71 | 4.85 |
| NOV. | 4.66 | 4.66 | 4.66 | 4.66 | 4.85 |
| DEC. | 4.56 | 4.56 | 4.56 | 4.56 | 4.85 |
| Mean | 4.69 | 4.71 | 4.67 | 4.71 | 4.85 |

Figure 8. Distribution of rain height over the study locations

Figure 9. Mean rain height over the geo-climatic zones vs ITU-R prediction

##### The information on the vertical structure and extent of precipitation events is an important factor in the prediction of attenuation due to rain on a satellite to earth slant link. It is assumed that rain is uniform from the ground to the rain height according to the simple vertical structure and the rain height is quoted to be that level up to which rain drops present have a diameter of 0.1 mm or more (Ajayi & Odunewu, 1990). From Figure 5, the average rain period is between March and November. There is moderate rainfall experienced throughout the period with peak values in August and September. Figure 6, presents the distribution of the ZDIH for Nigeria showing variation within the different geo-climatic zones with ESUT and PJAA showing similar trends throughout the year while FUTA and FUGUS trends were quite different in pattern especially during mid-year. From Table 2, the lowest height was 4.10 km experienced in FUTA and FUGUS in January, while the highest was 4.45 km experienced in FUTA, ESUT and PJAA between June and August. Mean ZDIH for the different geo-climatic zones as shown in Figure 7 are 4.33 km, 4.35 km, 4.31 km, and 4.35 km for FUTA, ESUT, FUGUS, and PJAA respectively as against 4.50 km predicted by ITU-R model. Figure 8 is the distribution of rain height over the study locations showing variation within the different geo-climatic zones with similar pattern with the ZDIH. From Table 3 the lowest rain height is 4.46 km while the highest rain height was 4.81 km. The mean rain height for each geo-climatic zones as shown in Figure 9 are 4.69 km, 4.71 km, 4.67 km, and 4.71 km for FUTA, ESUT, FUGUS, and PJAA respectively as against 4.86 km predicted by ITU-R model.

4. DISCUSSION

The rainfall accumulation for 36 months from the selected locations reveals that ESUT has the highest rainfall and that was recorded in September. Based on the information, Enugu State is characterized with heavy rainfall except for the relatively dry period between December and February. For the period of observation, the mean annual rainfall is about 1800 mm with the peak amount of rainfall of about 440 mm occurring in September. Akure station followed Enugu relatively with an annual mean precipitation of about 1750 mm and peak amount of about 382 mm occurring in August and Abuja site with an annual mean rainfall of about 1400 mm. Zamfara station continue to receive minimum rainfall all through the period of study with no rainfall throughout the dry season. Generally, the period of intense rainy months including June through October; these months are needed to be subjected to further analysis in order to select the worst months in the studied locations.

Findings from this studies have revealed that the 0°C isotherm heights are higher during the wet months of the year (April – October) than the rest of the calendar month (dry months- November- March). This is in agreement with the observation made by Mandeep, (2012) that during the period of enhanced shower activities, 0°C isotherm heights is higher than during the dry months due to the convective heating. In overall, the mean 0°C isotherm heights vary from 4.33 km, 4.35 km, 4.31 km, and 4.35 km, over FUTA, ESUT, FUGUS, and PJAA, respectively. However, ITU-R model predicted an annual average value of 4.5 km for this region. The implication is that, rain attenuation estimated using the value predicted by the ITU will be overestimated in this region due to the annual average estimated value of the 0°C isotherm height used.

The mean values of rain heights 4.69 km, 4.71 km, 4.67 km, and 4.71 km, for FUTA, ESUT, FUGUS, and PJAA respectively using the measured 0°C isotherm height puts the average hr over Nigeria at 4.695 km above mean sea level. The percentage error based on each of the locations is 3.78%, 3.33%, 4.22% and 3.33% respectively for FUTA, ESUT, FUGUS, and PJAA. This variation is Significant during peak rainfall months in all regions which suggests a need for region-specific model for accurate rain attenuation prediction in Nigeria. With highest value of 4.81 km recorded at FUTA, ESUT and PJAA, findings of this study have shown that hr does not depend very much on latitude of the stations as earlier observed by Ajayi and Odunewu (1990). Furthermore, the rainfall height directly influences signal attenuation at Ku-band frequencies in the form of lower, moderate and high attenuation across the zones. Other impacts are short, longer and extended duration of significant attenuation events across the locations. The result implies that Maximum signal attenuation is likely during July-September and more stable satellite communication conditions likely to occur during January-February.

## 5. CONCLUSION

The different geo-climatic regions of Nigeria show marked variations in rainfall height patterns, which significantly impact Ku-band satellite communications’ performance. This study reveals some crucial insights for optimizing system design across the country. Derived Savanah experiences the highest rainfall heights, resulting in severe signal attenuation. In contrast, the Sahel with lower rainfall heights enjoy more reliable service and can operate with reduced fade margins. This regional disparity underscores the need for location-specific design approaches rather than implementing a standardized solution across Nigeria. Notable discrepancies between measured rainfall data and ITU-R predictions highlight the necessity for continued local measurements and development of region-specific models tailored to tropical environments. Future Ku-band satellite communication systems in Nigeria should incorporate these regional rainfall height variations to Optimize overall system performance, Maximize service availability and Minimize costs by avoiding unnecessary over-engineering in less challenging climate zones. These findings contribute valuable data for developing more effective satellite communication infrastructure that accounts for Nigeria's diverse climate conditions, ultimately improving service reliability and cost-efficiency.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

Adetan, O., & Obiyemi, O. O. (2016). A Analysis of Raindrop Diameters for Rainfall Attenuation in Southern Africa. International Journal of Electrical & Computer Engineering (2088-8708), 6(1).

Acharya, R. (2017). Satellite Signal Propagation , Impairments and Mitigation. Elsevier Ltd. https://doi.org/10.1016/B978-0-12-809732-8.00001-6

Ajayi, G. O & Barbaliscia, F. (1990), Prediction of attenuation due to rain: characteristics of the 0 C isotherm in temperate and tropical climates. International Journal of Satellite Communications, 8(3), 187-196.

Crane R. K (1984). A comparative evaluation for several rain attenuation prediction models. Final Rep., NASW-3645N, ASA,

Crane, R. K (2002). Analysis of the effects of water on the ACTS propagation terminal antenna, Antennas and Propagation, IEEE Trans., 50, 954–965.

Edokpa, D.O., Ede, P.N., Brown, J., Adeyemi, A.J. & Pukiche, I.U. (2024).The Nigeria climate zones: Variability, trends and analysis from era-interim data (2010-2015). International Journal of Climate Research, 8(1), 1-24. DOI: 10.18488/112.v8i1.3804.

ITU-R P.311-18 (2021). Acquisition, presentation and analysis of data in studies of radiowave propagation. ITU Radio-communication Bureau, Geneva

Ippolito, L. J. (2008). Propagation Effects Handbook for Satellite Systems Design. Online Journal 	of Space Communication, 1(2), 15.

ITU-R P.618-11, 2013: “Propagation data and prediction methods required for the design of Earth-	Space telecommunication systems,” Recommendation, Geneva.

Kodheli, O., Lagunas, E., Maturo, N., Sharma, S. K., Shankar, B., Montoya, J. F. M. & Goussetis, G. (2020). Satellite communications in the new space era: A survey and future challenges. IEEE Communications Surveys & Tutorials, 23(1), 70-109.

Lawal, Y. B., Ojo, J. S., Falodun, S. E., & Olurotimi, E. O. (2021). Geoclimatic 	characterization and latitudinal dependence of rain heights over Nigeria. In Journal of 	Physics: Conference Series (Vol. 2034, No. 1, p. 012010). IOP Publishing.

Mandeep, J. S. (2011). Comparison of rain rate models for equatorial climate in South East Asia. Geofizika, 28(2), 265-274.

Ogolo, E.O. & Matthew, O.J. (2022). Spatial and temporal analysis of observed trends in extreme precipitation events in different climatic zones of Nigeria. Theoretical and Applied Climatology, 148(2), 1335–1351. DOI: 10.1007/s00704-022-04006-7.

Ojo, J. S., Ijomah, C. K., & Akinpelu, S. B. (2022). YOJNA neural networks for earth-space link applications: a prediction approach and inter-comparison of rain-influenced attenuation models. Int. J. Intell. Syst. Appl.(IJISA), 14(5), 47-58.

Ojo, J. S., & Rotimi, O. C. (2015). Diurnal and seasonal variations of rain rate and rain attenuation on Ku-band satellite systems in a tropical region: A synthetic storm techniques approach. Journal of computer and communications, 3(4), 1-10.

Orji, O. P., Chukwuebuka, O. D., Chidinma, O. E., Josephine, U. O., Chinedu, O. P., Uzochukwu, I. E., ... & Chiemeka, I. U. (2024). Rain Attenuation Prediction at Different Time Percentages for Ku, K, and Ka Bands Satellite Communication Systems over Nigeria. Journal of Astronomy and Space Sciences, 41(1), 25-33.

Panagopoulos, A. D., Arapoglou, P. D. M., & Cottis, P. G. (2004). Satellite communications at Ku, Ka, and V bands: Propagation impairments and mitigation techniques. IEEE communications surveys & tutorials, 6(3), 2-14.

Pelton J.N. (2012). Introduction to Satellite Communications (pp. 1–3). https://doi.org/10.1007/978-1-4614-1994-5_1

Semire, F.A, Adekunle, A. J & Abolade R.A, (2019). A Study on Rain Rate Prediction of Southwestern Nigeria. Physical Science International Journal, 23 (4), 1-12

Shih, S. P. (2008). A study of precipitation effect on tropospheric electromagnetic wave propagation at frequency 19.5 GHz. Terrestrial, Atmospheric and Oceanic Sciences, 19(3), 309–319. https://doi.org/10.3319/TAO.2008.19.3.309(AA)

Stigsson, A. (2024). Satellite signal attenuation due to atmospheric influences in northern Sweden.

Shriram R. Imthiyaz A. S. & Kumar L. S. K (2015). Calculation of cloud attenuation at Ku and Ka band frequencies over Indian region.   Conference: 2015 IEEE Applied Electromagnetics Conference (AEMC), DOI: 10.1109/AEMC.2015.7509233.

World Bank Group (2020) Observed Climatology of Average Mean Surface Air Temperature 1991-2020. Climate Change Knowledge Portal for Development Practitioners and Policy Makers.