PRODUCTION OF SOIL CONDITIONERS BASED ON COAL OXIDIZED WITH HYDROGEN PEROXIDE, BENTONITE, PHOSPHATE RAW MATERIALS, AND MINERAL FERTILIZERS

ABSTRACT.

The article presents the results of obtaining soil conditioners based on coal oxidized with hydrogen peroxide, containing a large amount of humic acids, crushed bentonite, phosphate raw materials, urea, ammonium sulfate, and potassium chloride in wide proportions. As a result of the interaction of humic acids in brown coal with phosphate raw material components and bentonite, an increase in the solubility of plant nutrients was observed. The optimal ratio of components and the degree of activation were determined for obtaining soil conditioners with an optimal amount of nutrients in a form that the plant can assimilate.

In addition, the study assessed the influence of these air conditioners on the physicochemical properties of the soil, including pH, cation exchange capacity, water retention, and aggregate stability. The synergistic interaction  of humic acids with mineral fertilizers improved the soil's nutrient supply.

Keywords: soil conditioner, brown coal, humic acids, phosphorite, bentonite, urea, ammonium sulfate, potassium chloride.

1.INTRODUCTION

At present, natural non-metallic minerals and their activated analogs are widely used in agricultural production. The application of these substances for improving the agrochemical, agrophysical, and microbiological properties of soils; in crop production as soil conditioners, fertilizers, and plant growth stimulants; and in animal husbandry as medicines, preparations, and feed additives for farm animals is due to the presence of biogenic macro- and microelements, as well as their high ion-exchange, sorption, and catalytic properties.Studies of minerals, nanostructured substances obtained from them, and investigations of their chemical and mineral composition, structure, and certain properties have shown that minerals such as bentonites, phosphate rock meal, sapropel, and glauconite contain varying amounts of aluminum, silicon, calcium, magnesium, iron, and phosphorus compounds. While their mineral composition showed similarities among the constituent components, their structures differed significantly. Nanostructured minerals possessed a surface area 15–18 times larger than their macro-analogs. Structurally, they represented conglomerates of agglomerated particles ranging from micrometer to nanometer size (0.1–5.0 µm). In terms of physical properties, they consisted of randomly arranged particles of conical, capsule-like, leaf-like, spherical, tubercular, and other forms. Nanostructured minerals differed significantly from their original macro-analogs, with particle sizes ranging from 5.0 to 120 nm, exhibiting clear size distribution. Their morphology included irregularly shaped, sharp-edged conical, kernel-like, and clustered particles, with some nanominerals forming systems of paired or triplet particles [1].

In [2], it was reported that the application of bentonite to dark chestnut soil under winter wheat increased the content of nitrate nitrogen, mobile forms of phosphorus and potassium, cation exchange capacity, and biological activity of the soil. The optimal bentonite dose of 7.5 t/ha ensured a 15.1% increase in wheat yield, as well as a significant improvement in grain protein content and gluten.

In [3], studies on the biosafety and efficiency of using nanostructured water–bentonite suspension for pre-sowing seed treatment were presented. Evaluations on nine test-plant species confirmed its biological safety at concentrations of 0.25–10.0 kg/t of seeds. Application of the suspension increased field germination of common vetch and variable alfalfa seeds up to 90.0–91.0% and boosted above-ground biomass growth to 1.15–1.22 kg/m².

In [4], it was shown that the use of bentonites significantly altered the soil’s pH-dependent characteristics, with a prolonged effect lasting for three years. Alongside the reduction of exchangeable and hydrolytic soil acidity, an increase in exchangeable calcium and magnesium content was observed, while mobile aluminum—which is toxic to many crops—was substantially reduced. Furthermore, the high ameliorative doses of bentonite and its prolonged three-year influence suggested relative stabilization processes in the soil’s mineralogical composition. Agronomically, the interaction of bentonite with soil optimized its nutrient regime and improved yields of major crops. Elevated concentrations of calcium and potassium from high bentonite doses could positively affect agro-phytocenosis productivity, depending on exchangeable magnesium and mobile phosphorus content.

In [5], it was demonstrated that ultrasonic dispersion of sapropel promotes the breakdown of its conglomerates and reduces particle size from 4.0 µm to 45.0–180.0 nm. This also altered the shape, structure, and properties of modified sapropel. Nanostructured sapropel particles ranged from 45.0 to 180.0 nm. Within a 20×20 µm study area, 120–140 particles were detected, with 36–48 large particles (30.0–34.0%) sized 100.0–180.0 nm, resembling macro-analogs but with smoother tetrahedral forms. Over 80 smaller particles (63.0–67.0%) were under 100 nm and randomly distributed. A specific structural feature of nanosapropel was that larger particles were organized into groups of three, equally spaced. Larger particles retained clear tetrahedral shapes, while smaller ones were amorphous. The average particle size of nanostructured sapropel was about 90.0 nm.

Analysis of numerous studies has shown that soil degradation processes are intensifying worldwide. Chemical degradation, linked to agricultural development, is especially widespread. Soil pollution results from global processes and from excessive application of mineral fertilizers, plant protection agents, use of wastewater and sludge for irrigation, and utilization of industrial and agricultural waste as unconventional fertilizers. All these factors lead to soil chemical degradation, i.e., deterioration of agronomic properties [6,7].

In [8], vegetation experiments on alluvial medium loam soil with unconventional soil conditioners, such as cattle manure effluent and organo-mineral fertilizer based on it, demonstrated positive effects on barley yield (main and by-products). The highest yield increase was achieved with 20 t/ha application.

In [9], lysimetric experiments on sod-podzolic sandy loam soil using manure effluent, biocompost from livestock and municipal waste showed positive effects on annual forage yields. The highest hay yield increase of  95.3% was observed with biocompost application. Hay yields with 10 t/ha biocompost were 8.6% higher than with 10 t/ha effluent and 25.8% higher than with mineral fertilizers alone.

It should be noted that virtually all organo-mineral fertilizers can be classified as soil conditioners. The term melioration usually refers to improving soil fertility through drainage or irrigation, but here it concerns chemical melioration. Humic fertilizers, due to their sorption, ion-exchange, and complex-forming properties, are effective chemical soil conditioners and also contribute to desalination of saline soils [10,11].

One of the main requirements for a soil conditioner is high cation exchange capacity, determined by the number of functional groups capable of ion exchange (–COOH and –OH). These are present in small amounts in lignite. Their content can be increased through oxidation. The use of oxidized brown coal from the Gusinoozersk deposit on meadow-sulfate solonchak soils provided a strong desalination effect and a significant yield increase [12].

Chemical melioration of soils with organo-mineral fertilizers based on oxidized brown coals is not limited to the desalination of saline lands. It is also effective for detoxifying water and soil environments from radioactive elements, heavy metals, and residual herbicides in soil. This is attributed to humic acids, which act as natural detoxifying agents [13–15].

METHODOLOGY

In this study, considering the specific beneficial properties of humic acids from brown coal, bentonite, phosphate rock meal, and mineral fertilizers for soils, and building on accumulated experience of their application, the objective was to investigate the processes of obtaining soil conditioners based on these components and to study their chemical and physicochemical properties.In the experiments, a representative sample of coal fines of BR-2 grade from the Angren deposit was used; its composition is given in Table 1.

Table 1..  Composition of the Initial and Oxidized Coal, %

|  | Moisture | Ash | Organic matter | HA + FA (Humic Acids + Fulvic Acids |
| --- | --- | --- | --- | --- |
| Initial brown coal of the Angren deposit, BR-2 grade | 4,28 | 18,48 | 77,27 | 5,96 |
| Coal Oxidized with Hydrogen Peroxide | 36,96 | 11,68 | 51,36 | 30,09 |

The mineralogical analysis of the concentrates showed that the mineral part of the coal contains quartz, calcite, pyroxene, opal, feldspar, magnetite, and muscovite. Table 2 presents the results of mass spectrometric (ICP–MS) analysis of the coal ash. As seen from the table, the coal contains a number of microelements, including Fe, Mg, Ba, B, Mn, Ti, Cu, Zn, and Mo.

Table 2 Results of Mass Spectrometric Analysis of Coal Ash

| Name and Content of Elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Li 127 | Be 2,61 | B 267 | Na 9950 | Mg 26011 | Al 63889 | P 325 | K 3547 | Ca 164077 | Cr 22,8 |
| Mn 5474 | Fe 46174 | Co 7,17 | Ni 8,65 | Cu 63,0 | Zn 4438 | Mo 1,46 | Ag 1,05 | Ba 127 | Ti 126 |

Also, bentonite from the Azkamar deposit was used, the composition of which is given in Table 3. An X-ray diffraction (XRD) analysis was carried out to determine the mineral composition of the raw bentonite, performed on an XRD-6100 diffractometer with computer control. Phase identification was conducted using domestic catalogs and ASTM standards.[16].

The X-ray diffractogram of bentonite shows diffraction maxima at 3.34, 9.97, 4.15, 2.45, and 2.75 Å, which correspond to the minerals quartz (SiO₂), illite [K₀.₆₅Al₂(Si₃.₃₅Al₀.₆₅)O₁₀(OH)₂•nH₂O], kaolinite [Al₂Si₂O₅(OH)₄], montmorillonite [(Na,Ca)₀.₃₃(Al,Mg)₂Si₄O₁₀(OH)₂•nH₂O], potassium feldspar (KAlSi₃O₈), and hematite (Fe₂O₃).Interplanar spacings at 3.04, 2.50, 2.28, 2.09, 1.91, and 1.87 Å correspond to calcite, while 3.34 and 2.14 Å belong to quartz.On the X-ray diffractogram of phosphate rock meal (Fig. 2), diffraction bands at 2.69, 2.62, 2.28, 2.24, 1.83, and 1.72 Å correspond to fluorocarbonate apatite.The presence of calcite is confirmed by interplanar spacings at 3.86, 3.03, 2.49, 2.09, 1.91, 1.87, and 1.60 Å; dolomite at 1.54 Å; gypsum at 7.61, 3.07, 2.77, and 1.42 Å; and tricalcium phosphate at 3.45 Å. Bands at 4.29 and 1.54 Å indicate the presence of insoluble residue—quartz—in the phosphate rock meal.[17].

Table 3.Chemical Composition of Bentonite from the Azkamar Deposit, Navoi Region

| № | SiO2 | TıO2 | Al2O3 | Fe2O3 | MgO | MnO | CaO | Na2O | К2О | P2O5 | SO3 | Н2О | СО2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 49,34 | 0,68 | 14,42 | 5,28 | 2,45 | 0,03 | 5,12 | 1,93 | 2,18 | 0,14 | 1,22 | 6,87 | 2,93 |

Fig. 1. X-ray diffractogram of the raw bentonite.

7,61

4,29

3,86

3,45

3,07

3,03

2,77

2,69

2,62

2,49

2,28

2,24

2,09

1,91

1,87

1,83

1,72

1,60

1,54

1,42

Figure 2. X-ray diffractogram of the raw bentonite.

For the production of soil conditioners, ammonium sulfate was also used (wt.%): moisture – 0.21; total N – 21.1; potassium chloride (wt.%): moisture – 0.9; total K₂O – 60.7; urea (wt.%): moisture – 0.3; total N – 46.2.

At the first stage, in order to increase the yield of humic acids and enhance the number of functional groups, brown coal from the Angren deposit was oxidized with hydrogen peroxide in the presence of oxalic acid. The oxidation process was carried out with hydrogen peroxide at 10%, oxalic acid at 60%, and a weight ratio of coal (organic part) : H₂O₂ : H₂C₂O₄ equal to 1 : 0.1 : 0.01. The experiments were performed as follows: first, the coal was treated in a mechanical mortar with oxalic acid at a coal : H₂C₂O₄ ratio of 1 : 0.01 for 30 minutes. After this treatment, the coal remained in a loose form. The obtained mass was then transferred to a reactor, and under stirring, hydrogen peroxide solution was added. The interaction was carried out for 2 hours. During oxidation of the brown coal with hydrogen peroxide, the temperature of the reaction mass rose to 50–80 °C. Afterwards, the ash content, moisture, organic matter, and yield of humic and fulvic acids were determined. Ash content was determined according to GOST 11022-75, moisture – according to GOST 11014-70, organic matter – by the difference between 100 and the sum of ash and moisture percentages, and the yield of humic acids – according to GOST 9517-76. The composition of the oxidized coal is also given in Table 1.[18].

For the purpose of obtaining a soil conditioner with a balanced set of nutrients based on plant requirements, at the next stage, brown coal of the same composition was oxidized under optimal conditions, i.e., with 10% H₂O₂, 60% H₂C₂O₄, and a weight ratio of coal : H₂O₂ : H₂C₂O₄ = 1 : 0.1 : 0.01. Then, into the wet oxidized coal were added ground bentonite, phosphate rock meal, urea, ammonium sulfate, and potassium chloride. Mixtures were prepared in a wide range of initial component ratios: oxidized coal (organic part) : bentonite : phosphate rock meal : (NH₂)₂CO : KCl : (NH₄)₂SO₄ = 100 : (25–200) : (2–78) : (1–38) : (1–11) : (1–5). They were then ground in a porcelain mortar to a particle size of less than 0.16 mm.The mixtures were dried at 75–80 °C for 60 minutes.[19].

The determination of P₂O₅ was carried out gravimetrically by precipitating the phosphate ion with a magnesia mixture in the form of magnesium ammonium phosphate, followed by calcination of the precipitate at 1000–1050 °C in accordance with GOST 20851.2-75. Nitrogen was determined according to GOST 26715-85. SO₃ was determined by precipitation as barium sulfate, and the CaO content was determined by titration with 0.05 N Trilon B solution in the presence of a fluorexon indicator. Potassium oxide was determined according to GOST 20851.3-93. The remaining elements were determined by mass spectrometric analysis of the ash of the soil conditioners.[20]

The obtained product consisted of irregularly shaped grains ranging in color from dark brown to black, whose composition is given in Tables 4 and 5. The soil conditioner obtained with the component ratio oxidized coal (organic part) : bentonite : phosphate rock meal : (NH₂)₂CO : KCl : (NH₄)₂SO₄ = 100 : 25 : 2 : 1 : 1 : 1 contained total organic matter – 59.88%, humic acids – 35.08%, nitrogen – 0.40%, P₂O₅ – 0.22%, K₂O – 0.77%, CaO – 1.28%, MgO – 0.21%, SiO₂ – 7.36%, Mn – 4261.20 g/t, Zn – 207.84 g/t, Mo – 1.14 g/t, Co – 5.58 g/t, and Ni – 6.73 g/t. The table also shows that, depending on the ratios of the starting materials, the nutrient content can be varied over a wide range. Most importantly, these soil conditioners can be applied together with plant seeds at sowing, providing the plants with nutrients throughout the entire growing season. For each plant, 10–20 g of soil conditioner is sufficient, depending on soil conditions.[21].

The obtained soil conditioners were tested under laboratory conditions, i.e., the water-holding capacity of soils under the influence of humic-based soil conditioners was determined. It is known that the amount of moisture a soil can retain is called moisture capacity. Depending on the form in which the retained water is present, maximum adsorption, field capacity, capillary, and total moisture capacities are distinguished. Maximum adsorption capacity represents the largest amount of tightly bound water held by sorption forces.[22].

RESULTS AND DISCUSSION

Table 4Composition of Soil Conditioners Obtained on the Basis of Oxidized Coal, Bentonite, Phosphate Rock Meal, Urea, Potassium Chloride, and Ammonium Sulfate

|  | Oxidized coal (organic part) : bentonite : phosphate rock meal : (NH₂)₂CO : KCl : (NH₄)₂SO₄ | Total Organic Matter | HA (Humic Acid) | N2 | P2O5 | K2O | СаО | MgO | SiO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100:25:2:1:1:1 | 59,88 | 35,08 | 0,40 | 0,22 | 0,77 | 1,28 | 0,21 | 7,36 |
| 2 | 100:25:4:2:1:2 | 58,47 | 34,26 | 0,78 | 0,42 | 0,75 | 1,81 | 0,23 | 7,19 |
| 3 | 100:25:6:3:1:3 | 56,82 | 33,29 | 1,14 | 0,61 | 0,73 | 2,30 | 0,24 | 6,98 |
| 4 | 100:25:8:4:1:4 | 55,56 | 32,55 | 1,53 | 0,79 | 0,71 | 2,78 | 0,25 | 6,83 |
| 5 | 100:25:10:5:1:5 | 54,05 | 31,67 | 1,87 | 0,96 | 0,69 | 3,22 | 0,27 | 6,64 |
| 6 | 100:50:2:1:1:1 | 51,54 | 30,20 | 0,34 | 0,21 | 1,02 | 1,72 | 0,35 | 12,68 |
| 7 | 100:50:4:2:1:2 | 50,25 | 29,49 | 0,67 | 0,38 | 0,99 | 2,15 | 0,36 | 12,36 |
| 8 | 100:50:6:3:1:3 | 49,26 | 28,86 | 0,99 | 0,54 | 0,97 | 2,58 | 0,37 | 12,11 |
| 9 | 100:50:8:4:1:4 | 48,08 | 28,17 | 1,29 | 0,70 | 0,95 | 2,98 | 0,38 | 11,82 |
| 10 | 100:50:10:5:1:5 | 47,17 | 27,64 | 1,58 | 0,85 | 0,93 | 3,37 | 0,38 | 11,60 |
| 11 | 100:75:2:1:1:1 | 45,25 | 26,51 | 0,30 | 0,20 | 1,20 | 2,04 | 0,45 | 16,69 |
| 12 | 100:75:4:2:1:2 | 44,24 | 25,95 | 0,59 | 0,35 | 1,18 | 2,42 | 0,46 | 16,32 |
| 13 | 100:75:6:3:1:3 | 43,48 | 25,47 | 0,87 | 0,49 | 1,16 | 2,80 | 0,47 | 16,04 |
| 14 | 100:75:8:4:1:4 | 42,43 | 25,04 | 1,15 | 0,64 | 1,14 | 3,16 | 0,47 | 15,76 |
| 15 | 100:75:10:5:1:5 | 42,02 | 24,62 | 1,41 | 0,77 | 1,12 | 3,50 | 0,48 | 15,50 |
| 16 | 100:100:2:1:1:1 | 40,32 | 23,63 | 0,27 | 0,19 | 1,35 | 2,30 | 0,53 | 19,91 |
| 17 | 100:100:4:2:1:2 | 39,53 | 23,16 | 0,53 | 0,32 | 1,32 | 2,63 | 0,54 | 19,52 |
| 18 | 100:100:6:3:1:3 | 38,76 | 22,71 | 0,78 | 0,45 | 1,30 | 2,95 | 0,54 | 19,14 |
| 19 | 100:100:8:4:1:4 | 37,88 | 22,19 | 1,02 | 0,57 | 1,27 | 3,25 | 0,55 | 18,71 |
| 20 | 100:100:10:5:1:5 | 37,04 | 21,72 | 1,24 | 0,69 | 1,24 | 3,53 | 0,55 | 18,29 |
| 21 | 100:110:2:1:1:1 | 38,46 | 22,53 | 0,25 | 1,30 | 1,39 | 2,38 | 0,56 | 20,81 |
| 22 | 100:125:4:2:1:2 | 35,71 | 20,92 | 0,48 | 1,48 | 1,44 | 2,80 | 0,60 | 21,96 |
| 23 | 100:150:6:3:1:3 | 33,33 | 19,54 | 0,67 | 1,73 | 1,58 | 3,33 | 0,68 | 24,60 |
| 24 | 100:175:8:4:1:4 | 31,25 | 18,31 | 0,84 | 1,94 | 1,69 | 3,79 | 0,74 | 26,90 |
| 25 | 100:200:10:5:1:5 | 29,42 | 17,25 | 0,99 | 2,13 | 1,80 | 4,20 | 0,81 | 28,94 |
| 26 | 100:100:11:5,5:0:1 | 37,74 | 22,11 | 1,02 | 0,74 | 1,03 | 3,78 | 0,56 | 18,64 |
| 27 | 100:100:24:12:0,25:2 | 34,48 | 20,20 | 2,01 | 1,50 | 1,01 | 5,60 | 0,59 | 17,03 |
| 28 | 100:100:40:19,5:3,5:3 | 31,75 | 18,61 | 3,03 | 2,23 | 1,50 | 7,58 | 0,63 | 15,68 |
| 29 | 100:100:58:28:7:4 | 29,41 | 17,23 | 4,04 | 3,02 | 2,03 | 9,56 | 0,68 | 14,52 |
| 30 | 100:100:78:38:11:5 | 27,40 | 16,05 | 5,05 | 3,72 | 2,56 | 11,53 | 0,74 | 13,51 |
| 31 | 75:100:9,5:5:0:1 | 31,91 | 18,72 | 1,06 | 0,76 | 1,17 | 3,96 | 0,62 | 21,02 |
| 32 | 75:100:22:10,5:0:2 | 29,41 | 17,23 | 2,07 | 1,54 | 1,08 | 5,99 | 0,66 | 19,37 |
| 33 | 75:100:35:17:2:3 | 27,27 | 15,98 | 3,06 | 2,25 | 1,53 | 7,82 | 0,70 | 17,96 |
| 34 | 75:100:50:24:5:4 | 25,42 | 14,89 | 4,08 | 3,07 | 2,06 | 9,73 | 0,74 | 16,74 |
| 35 | 75:100:67:32:10:5 | 23,81 | 13,95 | 5,09 | 3,76 | 2,54 | 11,68 | 0,78 | 15,68 |
| 36 | 50:100:8:4:0:1 | 25,64 | 15,02 | 1,07 | 0,78 | 1,42 | 4,40 | 0,74 | 25,33 |
| 37 | 50:100:18:9:0:2 | 23,26 | 13,62 | 2,11 | 1,59 | 1,28 | 6,22 | 0,75 | 22,98 |
| 38 | 50:100:30:14:1,5:3 | 21,28 | 12,46 | 3,09 | 2,29 | 1,56 | 8,13 | 0,78 | 21,02 |
| 39 | 50:100:43:21:4:4 | 19,60 | 11,49 | 4,12 | 3,11 | 2,09 | 9,93 | 0,80 | 19,37 |
| 40 | 50:100:59:28:7:5 | 18,18 | 10,65 | 5,13 | 3,80 | 2,58 | 11,99 | 0,85 | 17,96 |

Table 5. Microelement Composition of Soil Conditioners Obtained on the Basis of Oxidized Coal, Bentonite, Phosphate Rock Meal, Urea, Potassium Chloride, and Ammonium Sulfate, in g/t

| № | Oxidized coal (organic part) : bentonite : phosphate rock meal : (NH₂)₂CO : KCl : (NH₄)₂SO₄ | Mn | B | Zn | Mo | Co | Ni |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100:25:2:1:1:1 | 4261,20 | 207,84 | 3454,73 | 1,14 | 5,58 | 6,73 |
| 2 | 100:25:4:2:1:2 | 4161,52 | 202,98 | 3373,92 | 1,11 | 5,45 | 6,58 |
| 3 | 100:25:6:3:1:3 | 4043,31 | 197,23 | 3278,07 | 1,08 | 5,30 | 6,39 |
| 4 | 100:25:8:4:1:4 | 3953,44 | 192,85 | 3205,24 | 1,05 | 5,15 | 6,25 |
| 5 | 100:25:10:5:1:5 | 3846,59 | 187,62 | 3118,59 | 1,03 | 5,01 | 6,07 |
| 6 | 100:50:2:1:1:1 | 3668,14 | 178,92 | 2973,92 | 0,98 | 4,80 | 5,80 |
| 7 | 100:50:4:2:1:2 | 3575,98 | 174,43 | 2899,21 | 0,95 | 4,68 | 5,65 |
| 8 | 100:50:6:3:1:3 | 3505,52 | 170,99 | 2842,07 | 0,92 | 4,59 | 5,54 |
| 9 | 100:50:8:4:1:4 | 3421,25 | 166,88 | 2773,75 | 0,89 | 4,48 | 5,41 |
| 10 | 100:50:10:5:1:5 | 3356,72 | 163,71 | 2721,42 | 0,86 | 4,42 | 5,32 |
| 11 | 100:75:2:1:1:1 | 3220,03 | 157,06 | 2610,59 | 0,86 | 4,23 | 5,09 |
| 12 | 100:75:4:2:1:2 | 3148,76 | 153,58 | 2552,83 | 0,84 | 4,12 | 4,98 |
| 13 | 100:75:6:3:1:3 | 3094,01 | 150,91 | 2508,44 | 0,82 | 4,05 | 4,87 |
| 14 | 100:75:8:4:1:4 | 3041,12 | 148,33 | 2465,56 | 0,80 | 3,98 | 4,81 |
| 15 | 100:75:10:5:1:5 | 2990,02 | 145,84 | 2424,12 | 0,78 | 3,90 | 4,72 |
| 16 | 100:100:2:1:1:1 | 2869,44 | 139,96 | 2326,37 | 0,77 | 3,76 | 4,53 |
| 17 | 100:100:4:2:1:2 | 2812,73 | 137,19 | 2280,42 | 0,75 | 3,68 | 4,44 |
| 18 | 100:100:6:3:1:3 | 2758,22 | 134,53 | 2236,20 | 0,73 | 3,61 | 4,36 |
| 19 | 100:100:8:4:1:4 | 2695,54 | 131,48 | 2185,38 | 0,71 | 3,53 | 4,25 |
| 20 | 100:100:10:5:1:5 | 2635,61 | 128,55 | 2136,81 | 0,69 | 3,49 | 4,16 |
| 21 | 100:110:2:1:1:1 | 2737,04 | 133,61 | 2219,02 | 0,73 | 3,57 | 4,32 |
| 22 | 100:125:4:2:1:2 | 2541,55 | 123,96 | 2060,53 | 0,68 | 3,33 | 4,01 |
| 23 | 100:150:6:3:1:3 | 2372,07 | 115,72 | 1923,14 | 0,63 | 3,12 | 3,75 |
| 24 | 100:175:8:4:1:4 | 2223,81 | 108,47 | 1802,95 | 0,58 | 2,91 | 3,54 |
| 25 | 100:200:10:5:1:5 | 2093,05 | 102,09 | 1696,88 | 0,53 | 2,74 | 3,36 |
| 26 | 100:100:11:5,5:0:1 | 2685,36 | 130,98 | 2177,13 | 0,72 | 3,52 | 4,24 |
| 27 | 100:100:24:12:0,25:2 | 2453,86 | 119,69 | 1989,45 | 0,66 | 3,21 | 3,88 |
| 28 | 100:100:40:19,5:3,5:3 | 2259,11 | 110,17 | 1831,56 | 0,61 | 2,96 | 3,57 |
| 29 | 100:100:58:28:7:4 | 2093,02 | 102,06 | 1696,88 | 0,56 | 2,74 | 3,31 |
| 30 | 100:100:78:38:11:5 | 1949,64 | 95,11 | 1580,67 | 0,52 | 2,55 | 3,08 |
| 31 | 75:100:9,5:5:0:1 | 2282,77 | 111,34 | 1794,09 | 0,62 | 2,99 | 3,61 |
| 32 | 75:100:22:10,5:0:2 | 2103,73 | 102,62 | 1653,37 | 0,57 | 2,76 | 3,32 |
| 33 | 75:100:35:17:2:3 | 1250,82 | 95,15 | 1533,14 | 0,51 | 2,54 | 3,07 |
| 34 | 75:100:50:24:5:4 | 1818,48 | 88,70 | 1429,18 | 0,49 | 2,38 | 2,86 |
| 35 | 75:100:67:32:10:5 | 1703,04 | 83,07 | 1338,45 | 0,46 | 2,23 | 2,69 |
| 36 | 50:100:8:4:0:1 | 1824,67 | 89,03 | 1479,33 | 0,48 | 2,39 | 2,85 |
| 37 | 50:100:18:9:0:2 | 1654,94 | 80,72 | 1341,72 | 0,44 | 2,17 | 2,62 |
| 38 | 50:100:30:14:1,5:3 | 1514,09 | 73,85 | 1227,54 | 0,41 | 1,98 | 2,41 |
| 39 | 50:100:43:21:4:4 | 1395,33 | 68,06 | 1131,25 | 0,37 | 1,83 | 2,23 |
| 40 | 50:100:59:28:7:5 | 1293,85 | 63,14 | 1048,98 | 0,35 | 1,67 | 2,04 |

The field capacity (minimum moisture-holding capacity) is the maximum amount of capillary-suspended water retained by capillary forces after all gravitational water has drained. The minimum moisture capacity depends mainly on the granulometric composition of soils, their structural aggregation, and bulk density. In soils with a heavy granulometric composition and well-developed structure, the minimum moisture capacity is 30–35%, while in sandy soils it does not exceed 10–15%.Capillary moisture capacity is the maximum amount of capillary-held water that can be contained in the soil. In addition, it depends on the distance between the saturated moisture layer and the groundwater table: the greater this distance, the lower the capillary moisture capacity.Total moisture capacity is the maximum amount of water that soil can hold when all pores are completely flooded with water. Like the minimum moisture capacity, it depends not only on the granulometric composition but also on the structure and porosity of the soils.Composition of Soil ConditionersThe results of the soil conditioner formulations based on oxidized coal, bentonite, phosphate rock meal, urea, potassium chloride, and ammonium sulfate are presented in Table 4. The data indicate that varying the ratios of organic and mineral components significantly affects both the macronutrient and humic acid content of the resulting conditioners.

It can be observed that as the proportion of bentonite and phosphate rock meal increases relative to the organic part of coal, the total organic matter (TOM) and humic acid (HA) content slightly decreases .

For example, Sample 1 (100:25:2:1:1:1) shows TOM of 59.88% and HA of 35.08%, while Sample 30 (100:100:78:38:11:5) contains TOM of 27.40% and HA of 16.05%. This trend reflects the dilution effect caused by increasing mineral components, which have relatively low organic content.Similarly, nitrogen (N) content rises proportionally with increased urea addition, while phosphorus (P₂O₅) and potassium (K₂O) contents are predominantly influenced by phosphate rock and potassium chloride concentrations, respectively.Calcium oxide (CaO), magnesium oxide (MgO), and silicon dioxide (SiO₂) levels correlate with both bentonite and phosphate rock content, highlighting the dual role of these components as both structural and nutrient sources.These findings suggest that careful adjustment of organic-to-mineral ratios allows the tailoring of soil conditioners to achieve a desired balance of organic matter, macroelements, and humic acids, which is critical for improving soil fertility and structure.Microelement Composition.Table 5 presents the microelement composition of the soil conditioners. The concentrations of manganese (Mn), boron (B), zinc (Zn), molybdenum (Mo), cobalt (Co), and nickel (Ni) generally decrease as the proportion of mineral additives increases. For instance, Mn decreases from 4261.2 g/t in Sample 1 to 1293.85 g/t in Sample 40. The reduction in trace elements can be attributed to the dilution effect by higher proportions of phosphate rock and bentonite, which contain lower levels of these microelements compared to the organic coal fraction.This trend indicates that, while the addition of minerals enhances macronutrient content and soil structural properties, excessive mineral loading may reduce the availability of essential trace elements . Therefore, optimization of soil conditioner composition is required to maintain a sufficient microelement supply for plant nutrition.

Water-Holding Capacity of Soil The water-holding capacity of soils treated with various soil conditioners is summarized in Table 6. Soil treated with conditioners demonstrates a significant increase in total, capillary, and field capacity compared to untreated control soil.

Control soil: Total moisture capacity averaged 30.12%, capillary moisture 27.94%, and field capacity 13.97%.Soil with 100:50:10:5:1:5 conditioner: Total moisture capacity increased to 42.67%, capillary to 38.19%, and field capacity to 19.93%.Soil with 100:100:78:38:11:5 conditioner: Showed the highest improvement, with total capacity 42.97%, capillary 38.36%, and field capacity 21.02%.The increase in water-holding capacity can be attributed to several factors:

- Organic matter content: Humic substances from oxidized coal improve soil aggregation, increase porosity, and enhance water retention within micropores.

- Bentonite clay: Provides a high cation-exchange capacity and expands when wet, creating additional micropores to retain water.

- Phosphate rock and urea: Contribute to soil nutrient availability, indirectly supporting microbial biomass that stabilizes soil structure and improves moisture retention.

Overall, these results indicate that the application of soil conditioners not only supplies essential nutrients but also substantially enhances the soil’s physical properties, especially its water-holding capacity [30].

Discussion The combined application of organic and mineral components in soil conditioners demonstrates a synergistic effect: organic coal fractions improve soil structure and provide humic acids, while bentonite and phosphate rock supply minerals and maintain soil cation-exchange capacity.

The data show a clear trade-off: higher mineral ratios reduce humic acid content but improve total nutrient supply.Moreover, the improved water-holding capacity correlates positively with humic acid content and bentonite proportion, confirming that both organic and clay components are essential for optimizing soil moisture dynamics.

Microelement content must be carefully monitored, as excessive dilution by mineral additives may reduce the availability of trace nutrients essential for plant growth.

In conclusion, the formulation of soil conditioners should balance organic matter content, mineral nutrient supply, and water retention properties to achieve optimal soil fertility and productivity.

Table 6. Water-Holding Capacity of Soils under the Influence of the Soil Conditioner, %

| # № п/п | # Sample Number | # Oxidized coal (organic part) : bentonite : phosphate rock meal : (NH₂)₂CO : KCl : (NH₄)₂SO₄ | # Moisture Capacity, % of Mass |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | # Tota | # Capillar | # Field Capacity (Minimum) |
| # 1 | # 1 | # Control (soil without additives) | 29,87 | 28,31 | 14,12 |
| # 2 |  |  | 30,02 | 28,09 | 12,95 |
| # 3 |  |  | 29,35 | 27,94 | 13,41 |
| # 4 |  |  | 30,44 | 27,15 | 13,82 |
| # 5 |  |  | 30,92 | 27,87 | 13,25 |
| # Avg.. |  |  | 30,12 | 27,94 | 13,97 |
| # 6 | # 2 | # Soil with soil conditioner 7.5 t/ha (100:50:10:5:1:5) | 43,19 | 38,11 | 20,16 |
| # 7 |  |  | 43,25 | 38,62 | 20,39 |
| # 8 |  |  | 42,92 | 38,84 | 20,23 |
| # 9 |  |  | 42,81 | 37,45 | 19,25 |
| # 10 |  |  | 41,18 | 37,94 | 19,64 |
| # Avg.. |  |  | 42,67 | 38,19 | 19,93 |
| # 6 | # 3 | # Soil with soil conditioner 7.5 t/ha (100:100:10:5:1:5) | 42,45 | 37,19 | 19,28 |
| # 7 |  |  | 42,42 | 37,71 | 19,45 |
| # 8 |  |  | 41,87 | 36,18 | 19,79 |
| # 9 |  |  | 40,68 | 36,89 | 18,29 |
| # 10 |  |  | 40,71 | 36,79 | 18,15 |
| # Avg.. |  |  | 41,62 | 36,95 | 18,99 |
| # 6 | # 4 | # Soil with soil conditioner 7.5 t/ha (100:100:78:38:11:5) | 44,18 | 39,07 | 21,09 |
| # 7 |  |  | 43,71 | 38,91 | 21,22 |
| # 8 |  |  | 42,69 | 37,55 | 21,86 |
| # 9 |  |  | 41,92 | 38,22 | 19,98 |
| # 10 |  |  | 42,38 | 38,09 | 20,99 |
| # Avg.. |  |  | 42,97 | 38,36 | 21,02 |
| # 6 | # 5 | # Soil with soil conditioner 7.5 t/ha (50:100:43:21:4:4) | 43,98 | 38,61 | 21,29 |
| # 7 |  |  | 43,77 | 39,22 | 20,95 |
| # 8 |  |  | 42,16 | 40,15 | 20,09 |
| # 9 |  |  | 43,27 | 38,21 | 20,15 |
| # 10 |  |  | 44,21 | 38,23 | 21,72 |
| # Avg.. |  |  | 43,47 | 38,84 | 2084 |

4. CONCLUSION

The application of an air conditioner consisting of oxidized coal, bentonite, phosphorite flour, urea, KCl, and ammonium sulfate salts at a rate of 7.5 tons per hectare has shown a significant improvement in the water retention capacity of the soil. In laboratory conditions, the total, capillary, and minimum moisture capacities of uncultivated soil were 30.12%, 27.94%, and 13.97%, respectively, while the moisture capacities of cultivated soil were 43.47%, 38.84%, and 20.84%. These results show that the soil conditioner not only improves moisture retention at different soil moisture levels, but also improves soil structure and provides basic nutrients, which indicates that it is an effective correction tool for agricultural soils.

Details of the AI usage are given below:

1. Disclaimer (artificial intelligence)

The author (s) declares that no generative AI technologies, such as Big Language Models (ChatGPT, COPILOT, etc.) or text-image generators, were used in the process of writing, preparing, or editing this manuscript.

REFERENCES

- Ma, B., Bao, Y., Ma, B., McLaughlin, N. B., Li, M., & Liu, J. (2022). Residual Effect of Bentonite-Humic Acid Amendment on Soil Health and Crop Performance 4–5 Years after Initial Application in a Dryland Ecosystem. Agronomy, 12(4): 853.

- Lei, Z., Pan, G., Fu, Y., Kramer, S., Li, X., & Zhou, Y. (2022). Bentonite-Humic Acid Improves Soil Organic Carbon, Water-Filled Pore Space, and Aggregate Stability in Semi-Arid Grassland. Geoderma, 404: 115456.

- Sharonova, N.L., Rakhmanova, G.F., Yapparov, I.A., Sibgatullin, T.A. (2018). Efficiency of nanostructured aqueous-bentonite suspension for pre-sowing seed treatment. Agrochemical Bulletin, No. 6, pp. 53–56. DOI: 10.24411/0235-2516-2018-10062

- Kozlov, A.V., Kulikova, A.Kh., Uromova, I.P. (2019). Physicochemical properties of bentonite and its effect on acid-base indicators and effective fertility of sod-podzolic light loamy soil. Bulletin of V.V. Dokuchaev Soil Institute, Issue 96, pp. 86–112. DOI: 10.19047/0136-1694-2019-96-86-112

- Ezhkov, V. O., Yapparov, A. K., Ezhkova, A. M., Yapparov, I. A., Ezhkova, G. O., Faizrakhmanov, R. N., & Motina, T. Y. (2016). Study of the action of nanostructured sapropel at different doses on the morphological and functional condition of the gastrointestinal tract in albino mice. Nanotechnologies in Russia, 11(7), 497-505.

- Snezhko, V.L., Benin, D.M. (2021). Dynamics of the Condition of Reclaimed Agricultural Lands in the Russian Federation.

- Baishanova, A.E., Kedelbaev, B.Sh. (2016). Problems of soil degradation. Analysis of the current state of fertility of irrigated soils in the Republic of Kazakhstan. Scientific Review. Biological Sciences, No. 2, pp. 5–13.

- Ilyinsky, A.V. (2020). Increasing soil productivity of degraded agricultural lands using unconventional organic ameliorants. Eurasian Union of Scientists (ESU), No. 9 (78), pp. 8–12. DOI: 10.31618/ESU.2413-9335.2020.5.78.1020

- Ilyinsky, A.V. (2020). Use of biocomposts to improve productivity of degraded sod-podzolic sandy loam soil. Eurasian Union of Scientists (ESU), No. 11 (80), pp. 37–39. DOI: 10.31618/ESU.2413-9335.2020.4.80.1127

- Li, X., Zhang, Y., Wang, J., Liu, S., Zhao, Y., & Chen, H. (2023). Biochar amendment has stronger effects than fertilizer regimes on the bacterial community structure and ecological processes in broomcorn millet field on the Loess Plateau. Plant and Soil.

- Ismail, S.M., Almulhim, N., Sedky, A., El-Cossy, S.A.-N., Mahmoud, E. Impact of Soil Ameliorants on Soil Chemical Characteristics, Sugar Beet Water Productivity, and Yield Components in Sandy Soils Under Deficit Irrigation. Sustainability.

- Stepanov, A.A., Yakimenko, O.S. (2023). Humic Soil Modifiers from Peat and Coal: Effect on Chemical and Biological Properties of Model Soil Mixtures. Russian Agricultural Sciences, 49(3), 252–259.

- Kireicheva, L.A., Glazunova, I.V. (1994). New organo-mineral sorbent ameliorants for detoxification of soils contaminated with heavy metals. Reports of the Russian Academy of Agricultural Sciences, No. 4, pp. 16–18.

- Cherdakova, A.S., Galchenko, S.V., Mazhayskiy, Y.A. (2016). Izmenenie soderzhaniya podvizhnyh form tyazhelyh metallov v seroy lesnoy pochve pod vliyaniem razlichnyh guminovyh preparatov. Vestnik Rossiyskogo universiteta druzhby narodov. Seriya: Ekologiya i bezopasnost zhiznedeyatelnosti, No. 4, pp. 63–66.

- Yarchuk, I.I., Bulgakova, M.P. (1991). Physiologically active substances of humic nature as an ecological factor of detoxification of residual herbicide amounts. Biological Sciences, No. 10, pp. 75–80.

- Shen, Y., Jiao, S., Ma, Z., Lin, H., Gao, W., Chen, J. (2020). Humic acid-modified bentonite composite material enhances urea-nitrogen use efficiency. Chemosphere, 255: 126976. DOI: 10.1016/j.chemosphere.2020.126976

- Ma, B., Bao, Y., Ma, B., McLaughlin, N.B., Li, M., Liu, J. (2022). Residual Effect of Bentonite-Humic Acid Amendment on Soil Health and Crop Performance 4–5 Years after Initial Application in a Dryland Ecosystem. Agronomy, 12(4): 853. DOI: 10.3390/agronomy12040853

- Lei, Z., Pan, G., Fu, Y., Kramer, S., Li, X., Zhou, Y. (2022). Bentonite-Humic Acid Improves Soil Organic Carbon, Water-Filled Pore Space, and Aggregate Stability in Semi-Arid Grassland. Geoderma, 404: 115456. DOI: 10.1016/j.geoderma.2022.115456

- Yang, F., Shen, Z., Wang, X., et al. (2021). Sustainable Advances on Phosphorus Utilization in Soil via Regulation Mechanisms of Biochar and Humic Substances. Science of the Total Environment, 756: 143817. DOI: 10.1016/j.scitotenv.2021.143817

- Li, J., Qin, G., Zeng, L., et al. (2022). Biochemical Fulvic Acid Modification for Phosphate Crystal Inhibition and Enhanced Phosphate Availability. Materials, 15(3): 1174. DOI: 10.3390/ma15031174

- Fan, X., Gao, Y., Wang, Y., Wen, Z., Wang, R., Wu, B., ... & Lu, P. (2025). Mechanisms of carbon and nitrogen distribution in soil aggregates in response to soil conditioners. Scientific Reports, 15(1), 4962.

- Zhang, Z., Ling, C., Zhu, Y., et al. (2022). Synergistic Effect of Biochar-Combined Activated Phosphate Rock on Nutrition Supply and Performance of Water Spinach. Journal of Environmental Management.