Assessing Compliance with Nepal’s Building Code and Bylaws: Insights from Residential Construction in Godawari Municipality

ABSTRACT

Purpose/Aim: This study evaluates compliance with the Nepal National Building Code (NNBC) and municipal bylaws in residential construction in Godawari Municipality, Lalitpur District, Nepal, to identify gaps and propose measures for enhancing earthquake-resistant construction.

Design/Methodology/Approach: A qualitative approach was used, combining literature review, field surveys of 147 reinforced concrete (RC) buildings (78 completed, 69 under construction), questionnaires with 32 contractors/masons, and key informant interviews (KII) with municipal officials and consultants. Compliance was assessed using checklists tailored for completed and under-construction buildings, with data analyzed descriptively in MS Excel.

Findings: Only 27 of 78 completed buildings fully complied with approved designs, with deviations in staircase dimensions, beam depth, and plinth area. Under-construction buildings showed high compliance in structural elements (e.g., 100% in columns, 98% in slabs) but significant deficiencies in concrete mix proportions (24% compliance) and rebar lapping (32% non-compliant in columns). Masons exhibited limited awareness of building code provisions (25%) despite better knowledge of earthquake phenomena (93.75%). Non-compliance stemmed from low awareness, inadequate training, and owner-driven design changes.

Originality/Value: The study provides localized insights into building code compliance in Godawari Municipality, comparing findings with Sisne and Birendranagar Municipalities, and offers practical recommendations for improving enforcement, training, and awareness to enhance seismic resilience.

Keywords: Building Code Compliance, Earthquake-Resistant Construction, Godawari Municipality, Reinforced Concrete Buildings, Nepal

1. INTRODUCTION

Nepal, located in a seismically active zone, ranks among the eleven most earthquake-vulnerable countries, with Kathmandu often described as a seismic risk hotspot. Reinforced concrete (RC) construction began in the 1980s, but engineered practices gained traction only after the Nepal National Building Code (NNBC) was enforced in 2006. Despite this, approximately 70% of RC buildings are owner-built, relying on contractors and municipal bylaws or the NNBC’s mandatory rules of thumb (MRT) [1]. The four different levels of sophistication of design and construction that are being addressed in the National Building Code Part I: International State- of –the Art; Part II: Professionally Engineered Buildings; Part III: Mandatory Rules of Thumb; and Part IV: Guidelines for Remote Rural Buildings [2]. Informal construction dominates, with over 80% of housing built outside formal processes, where code application is minimal. Even in formal projects, enforcement is weak, often treated as guidelines rather than binding regulations, a challenge also observed in low- income countries [3]. Municipalities bear legal responsibility for enforcement, but inspections are typically limited to the plinth stage, focusing on basic parameters like plinth area, column dimensions, and reinforcement diameters. Critical aspects such as concrete mix design, reinforcement detailing, and foundation depth are often unchecked, with engineers and architects largely absent during execution [4].

Godawari Municipality requires building approval and compliance checks at three stages: temporary permit (up to plinth), permanent permit (post-plinth), and completion certificate. This study assesses compliance with the NNBC and approved drawings in Godawari, aiming to identify gaps and propose measures for safer, earthquake-resistant construction. Similar challenges in urban planning and implementation have been noted in Godawari's Integrated Urban Development Plan (IUDP), which highlights issues in project prioritization and regulatory enforcement [5].

Objectives:

- To evaluate the status of building code and bylaw compliance in residential construction in Godawari Municipality.

- To assess the awareness and capabilities of contractors/masons regarding building code compliance.

- To identify reasons for non-compliance and propose measures to strengthen compliance.

2. LITERATURE REVIEW

Godawari Municipality, located in Lalitpur District, Bagmati Province, was established in 2014 through the merger of five Village Development Committees (VDCs) and expanded in 2017 to include twelve, with Bajrabarahi as its center. It is notable for the Godawari Botanical Garden and Phulchowki, the highest peak in Kathmandu Valley.

Studies highlight widespread gaps in building code compliance globally and in Nepal. In Cape Town, low compliance was linked to limited awareness and unqualified contractors, with only 5 of 14 sites adhering to National Building Regulations (NBR) [6]. In Karyabinayak Municipality, Nepal, designs showed deficiencies in ductility-related aspects like stirrup spacing and anchorage, with 15% failing strength checks due to irregular land plots [7]. Nepalese buildings often exhibit poor longitudinal reinforcement detailing, undersized bars (12–16 mm, 4–8 per element), inadequate stirrups (6 mm at >150 mm spacing), and use of rounded aggregates, compromising the Mandatory Rules of Thumb (MRT) strength standards (15 MPa). Corrosion of reinforcement is a recurring issue. In Lalitpur, only 27% of respondents were familiar with the NNBC, and inspections occurred in 33% of cases, limited by weak enforcement and lack of seismic risk awareness [8].

Similar challenges exist in Bihar, India, where cost, corruption, and limited institutional capacity hinder compliance [9]. In Nagarjun Municipality, Nepal, MRT-based implementation was in place but ineffective due to inadequate manpower and training [10]. Post-2015 Gorkha Earthquake studies revealed that non-engineered buildings suffered damage due to substandard materials and poor workmanship [11]. However, successful cases exist: in Kakani Rural Municipality, 74% of reconstructed buildings complied with NNBC due to training and oversight by National Reconstruction Authority (NRA) engineers [12]. In Chandragiri, 70% of RC housing adhered to codes with trained professionals [13]. Conversely, Sisne Rural Municipality reported only 1 of 27 completed buildings as fully compliant, with violations in plinth, beams, and staircases due to low technical awareness and owner-driven changes [14]. Birendranagar Municipality showed similar non-compliance, with masons relying on experience rather than training [15].

Compliance parameters include configuration (length–breadth ratios, cantilever lengths), strength (column/beam sizes, slab thickness, concrete grade), ductility (bar numbers, stirrup spacing, anchorage), and connections (lintel/sill bands, footing ties), as outlined in municipal case studies and handbooks [7]. Additional global perspectives include studies on residential code compliance in the U.S. [16] and energy regulation adherence in the U.K. [17], which emphasize the role of enforcement and training in improving compliance. Recent studies in Kathmandu Valley have also explored green building awareness among engineers and consultants, revealing moderate knowledge levels and challenges in policy support for sustainable practices [18]. Furthermore, comparative analyses of environmental protection provisions in construction procurement highlight variations between national and international funding agencies, underscoring the need for stronger safeguards in Nepal [19]. Delays in public building retrofitting projects, often due to impractical schedules and material unavailability, further illustrate implementation challenges in seismic-prone areas [20].

3. METHODOLOGY

This study assessed compliance with approved design plans and the NNBC in residential construction in Godawari Municipality, identifying causes of non-compliance and proposing solutions. A qualitative approach combined literature review, field surveys, questionnaires, key informant interviews (KII), and descriptive analysis.

3.1 STUDY AREA, POPULATION, AND SAMPLE

Godawari Municipality (96.11 km², population 97,633, 24,045 households per 2021 Census) spans 14 wards in Bagmati Province. The study focused on 691 RC frame structures registered for permits in fiscal year 2081/82. A stratified sample of 147 buildings (78 completed, 69 under construction) was selected for ward-wise representation. Respondents included 32 contractors/masons, two elected officials, three technical personnel, one administrative officer, and two consultants (Table 1).

Table 1: List of Population and Sample

| S.N. | Respondent types | Number of Population | Sample |
| --- | --- | --- | --- |
| 1. | Total Permitted Building | 691 | 147 |
| 1a. | Completed Buildings | 379 | 78 |
| 1b. | Buildings under construction | 312 | 69 |
| 2. | Local Contractor/Masons of buildings under construction | 32 | 32 |
| 3. | Municipal Elected Officials (Mayor and Ward Chairperson) | 2 | 2 |
| 4. | Municipal Technical Personnel | 3 | 3 |
| 5. | Municipal Administrative Officer | 1 | 1 |
| 6 | Consultant | 2 | 2 |

3.2 DATA COLLECTION

A. PRIMARY DATA COLLECTION

- Field Surveys: Site investigations of 147 buildings used a checklist covering mason characteristics, configuration, strength, ductility, connection parameters, and causes of non-compliance.

- Questionnaires: Structured questionnaires assessed masons’ awareness, perspectives, and capabilities regarding building code compliance.

- KII: Unstructured interviews with the chairperson, vice chairperson, chief administrative officer, and technical personnel explored reasons for non-compliance and solutions.

B. SECONDARY DATA COLLECTION

Data were sourced from journal articles, textbooks, municipal records, approved drawings, building permits, photographs, acts/regulations, and online platforms (e.g., ResearchGate, Google Scholar).

C. DATA ANALYSIS

Data were coded, processed, and analyzed descriptively in MS Excel, with results presented through tables and figures.

4. RESULTS AND DISCUSSIONS

4.1 BUILDING CODE AND BYLAW COMPLIANCE IN RESIDENTIAL CONSTRUCTION

Compliance was assessed using checklists tailored for completed and under-construction buildings, adapted from municipal guidelines and studies in Karyabinayak, Sisne, and Birendranagar Municipalities [7]; [14]; [15].

4.1.1 BUILDING CODE COMPLIANCE IN COMPLETED RESIDENTIAL BUILDINGS

Of 78 completed buildings, only 27 fully complied with approved designs. Common issues included non-compliant staircase dimensions, beam depth, plinth area, and room layout, though column specifications were universally met (100%) (Figure 1).

Figure 1: Building Code Compliance in Completed Residential Buildings: Percentage compliance of completed buildings across parameters (column size, beam size, plinth area, room size, staircase size, built-up area).

Table 2: Comparison of Compliance in Godawari, Sisne, and Birendranagar Municipalities

| Parameter | Godawari Municipality |  | Sisne Rural Municipality |  | Birendranagar Municipality |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Compliance (%) | Non-compliance (%) | Compliance (%) | Non-compliance (%) | Compliance (%) | Non-compliance (%) |
| Column Size and Position | 100 | - | 85.2 | 14.81 | 90.9 | 9.1 |
| Beam Size and Position | 93.58 | 6.42 | 7.4 | 92.6 | 40.9 | 59.1 |
| Plinth Area (Length to Breadth) | 64.1 | 35.9 | 3.7 | 96.30% | 68.2 | 31.82 |
| Room size | 51.28 | 48.72 | 3.7 | 96.3 | 59.1 | 40.91 |
| Staircase size and position | 87.18 | 12.82 | 7.41 | 92.59 | 27.3 | 72.73 |
| Built-up | 65.38% | 34.62 | 3.7 | 96.3 | 54.6 | 45.45 |

Godawari showed the highest compliance: 100% in column size, 93.58% in beam size, 87.18% in staircase size, 64.10% in plinth area, 51.28% in room size, and 65.38% in built-up area. Birendranagar had moderate compliance (e.g., 68.18% in plinth area), while Sisne showed low compliance (<10% in most parameters except column size at 85.18%) [11, [15].

4.1.2 BUILDING BYLAW COMPLIANCE IN COMPLETED BUILDINGS

Of 78 completed buildings, 27 fully complied with bylaws. Ground coverage ratio and building height showed 100% and 98.72% compliance, respectively, but setbacks (front: 88.46%, side: 91.03%, back: 93.59%) and floor area ratio (91.03%) had minor gaps (Figure 2).

Figure 2: Building Bylaw Compliance for Completed Buildings: Compliance rates for setbacks, floor area ratio, ground coverage, and building height in completed buildings.

Figure 3: Comparative Study of Bylaw Compliance

Godawari outperformed Birendranagar in setbacks (e.g., side setback: 91.03% vs. 27.27%) but was slightly weaker in ground coverage (87.18% vs. 100%) and building height (98.72% vs. 100%) [15].

4.1.3 BUILDING CODE COMPLIANCE IN UNDER-CONSTRUCTION BUILDINGS

Of 69 under-construction buildings (7% at foundation, 12% at DPC, 14% up to first floor, 26% above second floor, 41% at finishing), compliance was high for columns (100%) and slabs (98%) but low for concrete mix proportions (24%), rebar lapping (32% non-compliant in columns, 25% in beams), beam dimensions (39% non-compliant), staircase slab thickness (16% non-compliant), stirrup spacing (30% non-compliant), and sill/lintel bands (11% absent) (Table 3).

Table 3: Checklist of Compliance and Non-Compliance in Under-Construction Buildings

| S.N. | Description of compliance parameters | C | NC | C (%) | NC (%) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Depth of Excavation in the foundation | 12 | 1 | 92.3 | 7.7 | Observed on buildings constructed up to the Plinth level. |
| 2 | Foundation Construction |  |  |  |  |  |
| a | Size | 11 | 2 | 84.6 | 15.4 | Studied buildings constructed up to the Plinth level. |
| b | Rebar placement and Size | 2 | - | 100 | - | Studied buildings constructed up to the Plinth level. |
| c | Strap beam | 10 | - | 100 | - | Studied buildings constructed up to the Plinth level. |
| 3 | Column Size and Position | 69 | - | 100 | - | Studied all ongoing buildings |
| 4 | Beam Size and Position |  |  |  |  |  |
| a | Tie beam |  |  |  |  |  |
| b | Plinth Beam | 8 | 5 | 61.5 | 38.5 | Studied buildings constructed up to the Plinth level. |
| c | Floor Beam | 47 | 5 | 90.38 | 9.62 | Studied on 1st, 2nd and completion buildings |
| 5 | Slab Thickness |  |  |  |  |  |
| a | Floor Slab | 51 | 1 | 98.07 | 1.93 | Studied on 1st, 2nd and completion buildings |
| b | Staircase Slab | 44 | 8 | 84.61 | 15.39 | Studied on 1st, 2nd and completion buildings |
| 6 | Mix Proportion | 10 | 31 | 24.39 | 75.61 | Studied buildings except for the completion stage |
| 7 | Reinforcement detailing |  |  |  |  |  |
| a | Foundation | 2 | - | 100 | - | Studied foundation construction and building |
| b | Column | 41 | - | 100 | - | Studied buildings except for the completion stage |
| c | Beam | 28 | - | 100 | - | Studied on 1st, and 2nd floor constructed building |
| d | Slab | 28 | - | 100 | - | Studied on 1st, and 2nd floor constructed building |
| e | Staircase Slab | 28 | - | 100 | - | Studied on 1st, and 2nd floor constructed building |
| 8 | Stirrups | 29 | 12 | 70.73 | 29.27 | Studied buildings except for the completion stage |
| 9 | Lapping Position of Bar |  |  |  |  |  |
| a | Column | 28 | 13 | 68.29 | 31.71 | Studied buildings except for the completion stage |
| b | Beam Top Bar | 22 | 6 | 78.57 | 21.43 | Studied on 1st, and 2nd floor constructed building |
| c | Beam Bottom Bar | 21 | 7 | 75 | 25 | Studied on 1st, and 2nd floor constructed building |
| 10 | Sill and Lintel Band | 25 | 3 | 89.28 | 10.72 | Studied on 1st, and 2nd floor constructed building |

Note: C: Compliance, NC: Non-Compliance

Table 4: Comparison with Sisne and Birendranagar

| S. No | Description of Parameter | Godawari Municipality |  | Sisne Rural Municipality |  | Birendranagar Municipality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C (%) | NC (%) | C (%) | NC (%) | C (%) | NC (%) |
| 1 | Depth of Excavation in the foundation | 92.3 | 7.7 | 100 | - | 100 | - |
| 2 | Foundation Construction |  |  |  |  |  |  |
| a | Size | 84.6 | 15.4 | 100 | - | 100 | - |
| b | Rebar placement and Size | 100 | - | 100 | - | 100 | - |
| c | Strap beam | 100 | - | 100 | - | - | - |
| 3 | Column Size and Position | 100 | - | 100 | - | 90.9 | 9.1 |
| 4 | Beam Size and Position |  |  |  |  |  |  |
| a | Tie beam |  |  | 100 | - | 100 | - |
| b | Plinth Beam | 61.5 | 38.5 | 100 | - | 100 | - |
| c | Floor Beam | 90.38 | 9.62 | 77.78 | 22.22 | 70.59 | 24.59 |
| 5 | Slab Thickness |  |  |  |  |  |  |
| a | Floor Slab | 98.07 | 1.93 | 100 | - | 64.7 | 35.3 |
| b | Staircase Slab | 84.61 | 15.39 | 77.78 | 22.22 | 88.23 | 11.77 |
| 6 | Mix Proportion | 24.39 | 75.61 | - | 100 | - | - |
| 7 | Reinforcement detailing |  |  |  |  |  |  |
| a | Foundation | 100 | - | 100 | - | 100 | - |
| b | Column | 100 | - | 100 | - | 100 | - |
| c | Beam | 100 | - | 100 | - | 100 | - |
| d | Slab | 100 | - | 100 | - | 100 | - |
| e | Staircase Slab | 100 | - | 100 | - | 100 | - |
| 8 | Stirrups | 70.73 | 29.27 | 100 | - | 88.89 | 11.11 |
| 9 | Lapping Position of Bar |  |  |  |  |  |  |
| a | Column | 68.29 | 31.71 | - | 100 | 38.89 | 61.11 |
| b | Beam Top Bar | 78.57 | 21.43 | 71.42 | 28.57 | 76.92 | 23.08 |
| c | Beam Bottom Bar | 75 | 25 | 71.42 | 28.57 | 76.92 | 23.08 |
| 10 | Sill and Lintel Band | 89.28 | 10.72 | 85.71 | 14.28 | 84.62 | 15.38 |

Godawari showed strong compliance in beams (90%) and slabs (98%) but lagged in foundation size (85%) and plinth beams (62%) compared to Sisne (100%) and Birendranagar (100%). Concrete mixing was a critical issue across municipalities (Godawari: 24%, Sisne: 0%) [11, [15].

4.2 CHARACTERISTICS, AWARENESS, AND CAPABILITIES OF CONTRACTORS/MASONS

CHARACTERISTICS OF MASONS

Of 32 lead masons, 43.75% had 5–10 years of experience, 19% had formal training, and 15.62% had completed secondary education (equivalent to Grade 10 or above) (Table 5). Compared to Sisne and Birendranagar, Godawari’s workforce is moderately experienced but lacks formal training [7]; [15].

Table 5: Experience and Training of Masons

| Attribute | Category | Godawari (%) | Sisne (%) | Birendranagar (%) |
| --- | --- | --- | --- | --- |
| Experience | Up to 5 yrs | 15.62 | 7.69 | 25 |
|  | 5–10 yrs | 43.75 | 53.85 | 43.75 |
|  | 10–15 yrs | 15.62 | 38.46 | 12.5 |
|  | >15 yrs | 25 | 0 | 18.75 |
| Training | Yes | 18.75 | 0 | 18.75 |
|  | No | 81.25 | 100 | 81.25 |

AWARENESS AND CAPABILITIES

Masons in Godawari showed high awareness of earthquake phenomena (93.75%) and municipal approval processes (78.12%) but low knowledge of building code provisions (25%) and earthquake-resistant design (62.5%) (Figure 4).

Figure 4: Awareness of Masons: Comparative awareness levels of masons in Godawari, Sisne, and Birendranagar regarding earthquake phenomena, building codes, and approval processes.

Godawari masons excelled in earthquake-resistant practices (81.25%) and drawing interpretation (46.88%) but were weak in material understanding (15.63%) and construction methodology (31.25%) (Figure 5).

Figure 5: Capabilities of Masons: Comparative technical capabilities of masons across municipalities in earthquake-resistant practices, material understanding, construction methodology, and drawing interpretation.

PERSPECTIVE ON NON-COMPLIANCE

In Godawari, 62.5% of masons adhered to designs, but 31.25% modified them per owner requests. In Sisne, only 15.38% complied, with 69.23% altering designs for owners. Birendranagar had the lowest compliance (6.25%), with 81.25% modifying designs for owners (Figure 6).

Figure 6: Masons’ Perspective on Non-Compliance: Masons’ reported reasons for non-compliance, including adherence to designs, owner-driven modifications, and simplification for ease of work.

4.3 REASONS FOR NON-COMPLIANCE AND SUGGESTED MEASURES

Reasons: Limited awareness among owners and masons, underestimation of seismic risks, cost concerns, inadequate municipal outreach, infrequent monitoring, and weak coordination among stakeholders were identified via KII.

Solutions:

- Strengthen municipal monitoring with regular inspections and a reward–penalty system.

- Expand training for masons, contractors, and municipal staff on earthquake-resistant practices.

- Conduct public awareness campaigns with simplified guidelines and visual aids.

- Offer incentives (e.g., fee reductions) for compliant builders.

- Enhance coordination among authorities, consultants, and contractors.

5. CONCLUSIONS

Compliance with the NNBC and bylaws in Godawari Municipality is inadequate, with only 27 of 78 completed buildings fully adhering to approved designs, showing deviations in plinth area, room sizes, beam depths, and staircases. Under-construction buildings exhibited high compliance in structural elements (100% in columns, 98% in slabs) but critical weaknesses in concrete mix proportions (24%) and rebar lapping (32% in columns). Masons showed high awareness of earthquake phenomena (93.75%) but low knowledge of building code provisions (25%), with limited training (19%) and secondary education (15.62%). Non-compliance is driven by low awareness, owner-driven changes, and weak enforcement. Godawari outperforms Sisne and Birendranagar in most parameters but shares systemic issues like poor concrete mixing.

Recommendations:

- Implement public awareness campaigns and mason training programs.

- Establish clear guidelines and standard operating procedures (SOPs) aligned with the NNBC.

- Simplify the building permit process with visual tools.

- Enforce compliance through monitoring, rewards, and penalties.

- Conduct comprehensive studies with field and lab testing to assess non-compliance impacts on seismic resistance.

Limitations: The study relies on self-reported mason data and municipal records, which may introduce bias. Sampling was limited to one fiscal year, potentially missing seasonal or long-term trends. Laboratory testing of materials was not conducted, limiting insights into material quality.

REFERENCES

| [1] | D. Gautam, H. Rodrigues, K. K. Bhetwal, P. Neupane and S. Yashusi, "Common structural and construction deficiencies of Nepalese," Innov. Infrastruct. Solut. , vol. 1, no. 1, pp. 1-18. DOI 10.1007/s41062-016-0001-3, 2016. |
| --- | --- |
| [2] | S. K. Shrestha, S. K. Bhattarai and R. K. Shrestha, "Building codes and Quality Control," in A Text Book of Construction Management., 3rd ed., Kathmandu, Heritage Publishers & Distributors Pvt. Ltd. Bhotahity, Kathmandu,, 2019, pp. 288-293. DOI: 10.5281/zenodo.17157100. |
| [3] | I. Ahmed, G. Thayaparan, G. Brewer, K. Maund, J. V. Meding, H. Kabir, M. Faruk, H. D. Shrestha and N. Sitoula, "Opportunities and challenges of compliance to safe building codes: Bangladesh and Nepal," APN Science Bulletin, vol. 9, no. 1, pp. 73-80. doi:10.30852/sb.2019.834, 2019. |
| [4] | N. Maharjan, "Compliance of Concrete Strength Used in Low Rise Residential Buildings with NBC 205: A Case Study of Lalitpur Metropolitan City-28, 29.," Pulchwok Campus, Lalitpur, Harisiddhi., 2017. |
| [5] | U. Poudel, A. C. Lal, S. K. Bhattarai and S. S. Paudyal, "IUDP: Prospects and Challenges for Implementation in Nepal: A Case Study of Godawari Municipality.," Journal of Recent Activities in Architectural Sciences, vol. 9, no. 1, pp. 38-62. https://doi.org/10.46610/JoRAAS.2024.v09i01.005, 2024. |
| [6] | A. Windapo and K. Cattell, "A study of building contractors' compliance with national building regulations in Cape Town," 2010. |
| [7] | S. G. Shrestha, S. Pradhan and S. N. Shrestha, "Status of Building Code Compliance: A case study in Karyavinayak Municipality.," 2015. |
| [8] | L. Arendt, M. EERI, A. Hortacsu, K. Jaiswal, S. Shrestha, J. Bevington, F. Lanning and G. Mentor-William, "Implementing Nepal's National Building Code: A Case Study in Patience and Persistence," SAGE Journals, vol. 33, no. 1, 2017. |
| [9] | S. Jones and V. Vasvani, "Compliance with the building byelaws and earthquake safety in urban areas of Bihar: progress, constraints and challenges," Environmental Hazards, vol. 10, no. 16, pp. 1-18, 2017. |
| [10] | S. K. Bhattarai and A. K. Mishra, "Existing Scenario of Building Code Implementation in Newly Formed "Nagarjun Municipality"," Asian Journal of Science and Technology,, vol. 8, no. 11, pp. 6751-6762. https://zenodo.org/records/17157233, 2017. |
| [11] | H. Varum, R. Dumaru, A. F. Furtado , R. Barbosa, D. Gautam and H. Rodrigues, "Seismic Performance of Buildings in Nepal After the Gorkha Earthquake," in Impacts and Insights of Gorkha Earthquake, H. R. Dipendra Gautam, Ed., Elsevier Inc. , 2018, pp. 47-63. |
| [12] | B. K. Niraula, "Status of Implementation of Building Code on Post Earthquake Reconstruction Work in Kakani Rural Municipality, Nuwakot," Nepal Engineering College, Bhaktapur, 2018. |
| [13] | P. Thapa and H. K. Shrestha, "Evaluation of Implementation Process of National Building Code for Reconstruction of Residential Building after Gorkha Earthquake 2015 (A Case Study of Chandragiri Municipality, Kathmandu)," Nepal Engineering College., Bhaktapur, 2022. |
| [14] | K. K.C, S. K. Bhattarai and K. Katuwal, "Compliance of Building Code in Construction of Residential Buildings in Sisne Rural Municipality, Rukum (East) District.," Kathmandu, 2023. |
| [15] | N. Rijal, S. K. Bhattarai and G. Khatri, "Assessing Code Compliance and Mason Proficiency in Residential Construction in Birendranagar Municipality, Surkhet.," The OCEM Journal of Management, Technology & Social Sciences,, vol. 4, no. 1, pp. 155-167. https://doi.org/10.3126/ocemjmtss.v4i1.74756, 2025. |
| [16] | E. L. Vine, "Residential building code compliance: Implications for evaluating the performance of utility residential new construction programs.," Energy, vol. 12, no. 12, p. 1051–1058. https://www.sciencedirect.com/science/article/pii/036054429600076X, 1996. |
| [17] | w. Pan and H. Garmston, "Compliance with building energy regulations for new-build dwellings.," Energy, vol. 48, no. 1, p. 11–22. https://www.sciencedirect.com/science/article/pii/S0360544212004999, 2012. |
| [18] | S. K. Bhattarai, M. Rokaya, S. Arayal, N. Karmacharya and G. K. Joshi, "Empowering Sustainability: Unveiling Green Building Awareness and Solutions in Kathmandu Valley.," Journal of Recent Activities in Infrastructure Science, vol. 10, no. 1, pp. 1-10. https://doi.org/10.46610/JoRAIS.2025.v010i01.001, 2025. |
| [19] | S. Acharya, S. K. Bhattarai, P. Khanal and P. Khanal, "Comparative Analysis of Environmental Protection Provisions in Construction Procurement: A Case Study of Nepal's Projects Funded by National and International Agencies.," International Journal on Engineering Technology, vol. 1, no. 2, pp. 250-268. https://doi.org/10.3126/injet.v1i2.66695, 2024. |
| [20] | S. K. Bhattarai, L. Rayamajhi, S. Lamichhane and S. Arayal, "Navigating Delays: A Comprehensive Study of Public Building Retrofitting Projects in Kathmandu Valley.," Journal of UTEC Engineering Management, vol. 2, no. 1, pp. 37-52. https://doi.org/10.36344/, 2024. |

COMPETING INTERESTS DISCLAIMER

Authors declare no competing financial, non-financial, or personal interests that could influence this work.

AI USAGE DISCLAIMER

Authors declare that no generative AI technologies (e.g., Large Language Models, text-to-image generators) were used during the writing.