Numerical Study of Coupled Heat and Mass Transfer in Laterite Brick Walls

Abstract:

The insulation performance of a wall is an important contributor to the global energy performance of a building. With the results of the study, we intend to help determining suitable thickness and insulation performance of laterite bricks wall in practical applications. The results of a numerical modelling of coupled heat and mass transfers within a wall made of laterite bricks are presented with validation against experimental data. Based on the work of Luikov, a mathematical predictive model of heat and mass transfer is developed. Temperature and moisture are considered as potential drivers of transfers. The simulation results allowed us to plot the spatiotemporal distribution of temperature and moisture fields within the studied wall. The results show that laterite bricks wall acts as a shield against heat and moisture propagation. Besides. The results show that the greater the thickness of the wall made with cut laterite bricks, the better it insulates against heat and humidity. In the context of Burkina Faso, the results allow to determine the suitable thickness of the laterite brick wall according to the desired insulation level.

Key words: laterite brick, mass transfer, heat transfer, moisture content, local building

materials

- INTRODUCTION

Over the past decade, the use of earth as building material, particularly “laterite bricks”, renewed interest because they are easier to implement and have thermal comfort advantage.

“The laterite bricks, used in construction, constitute the interface between the exterior and the interior environments of the building. This interface is an assembly of porous material and the site of numerous coupled exchanges of mass and heat thanks to the gradients of temperature, humidity level, partial vapor pressure, capillary pressure” [1,2,3,4]. These exchanges are established between the indoor and outdoor environments of the building through its constituent elements.

The interface is considered as a thermal filter. Depending on the climatic conditions and the characteristics of the laterite bricks, it is often difficult to reach a level of comfort inside the building. To evaluate the comfort in the building, the temperature and the relative humidity are the parameters used to describe the coupled exchanges of heat and mass [1,5,6,7]. Indeed, the “variation of humidity and the transfer of heat in building materials are strongly coupled and have a very significant effect on the overall energy performance of buildings. Furthermore, perfect control of the atmosphere of a building always requires control of exchanges within the interface. Aware of this issue, many researchers have undertaken experimental and numerical studies to deepen knowledge of the coupled transfers of mass and heat” [7,8,9,10,11,12].

The pioneers in the mathematical modelling of transfers taking into account the coexistence of the liquid and vapor phases within the porous medium and revealing the coupling effects between mass transfer and heat transfer are Luikov in 1954, De Vries and Philip in 1957. “In 1962, another approach of modelling based on the formalism of the thermodynamics of irreversible processes” was introduced by Cary and Taylor [2]. Since then, “a great number of theoretical, numerical and experimental works have been devoted to resolving the systems of coupled equations governing these phenomena” [13,14,15,16,17,18].

Nevertheless, in the literature few studies have been devoted “to solving the problems of coupled heat and mass transfers in 2D/3D under the unsteady state of a building wall on the basis of local porous materials. In this work we propose a 3D numerical study of coupled heat and mass transfers in order to understand the spatio-temporal distribution of moisture content and temperature within a wall made of laterite bricks” (Kabre et al., 2020). This study is carried out by using the simulation software COMSOL 5.2, under non-steady state conditions. The simulation results are validated against experimental data.

- MATERIAL AND METHOD

  - Physical model of the wall

“Laterite bricks are hygroscopic and porous materials in which numerous hygroscopic and/or thermo-hygroscopic phenomena arise under the influence of a temperature gradient and/or air moisture level. These manifestations affect the thermophysical properties of laterite bricks” [9] and the comfort expected when they are used for building. In order to study the phenomena of coupled heat and mass transfers, we consider a physical model of wall made of laterite bricks. Each laterite brick is assimilated to a rectangular parallelepipedon which dimensions are 29 cm x 14 cm x 9 cm. The wall thickness is 9 cm (brick width). The laterite bricks are joined by mortar of one centimetre (01 cm) high made of sand, cement and water.

Figure 1 illustrates the considered wall model subject to heat and mass transfers phenomena.

Interior

(unstressed side)

Outside (stressed side)

Figure 1: Physical model of the considered wall

  - Mathematical model

The physical phenomena taken into account in the mathematical model are:

- conduction,

- convection,

- radiation,

- phase change.

The equations governing heat and mass transfers used in this work are based on the Luikov model [12]. The energy and mass conservation equations are given respectively by:

∇q=-Cp,mρm+Cp,lω∂T∂t                                                                                               (1)

∂ω∂t+∇m=0                                                                                                                                   (2)

By taking into account the assumptions mentioned above, equations (1) and (2) can be written respectively as follow:

CT∂T∂t=∇.C11∇T+C12∇φ+v.(D11∇T+D12∇φ)                                                           (3)

Ω∂φ∂t=∇.C21∇T+C22∇φ+v.(D21∇T+D22∇φ)                                                           (4)

The expressions of coefficients, Cij and Dij  (1≤i≤ 2 and 1 ≤j ≤2) are given in annex.

A matrix representation of equations (3) and (4) are shown below:

da∂T∂t∂φ∂t=∇.C∇Tφ+β.∇Tφ                                                                                            (5)

with

da=CT00Ω                                                                                                                           (6)

C=C11C12C21C22                                                                                                                        (7)

β=v.D11D12D21D22                                                                                                                     (8)

The capillary pressure pc is function of the relative moisture and the relation is given by the low of Kelvin [13,14]:

φ=exp-pcρlRvT                                                                                                                            (9)

pc=-Klnφ                                                                                                                             (10)

with  K=ρlRvT                                                                                                                         (11)

2.3. Finite elements model

In figure 2, the drawing of the considered wall is shown. A volume meshing is applied on the wall by using the simulation tool COMSOL MultiPhysics ® version 5.2.  COMSOL uses Newton-Raphson nonlinear convergence algorithm. Backward Differentiation Formula (BDF) is used for transient simulation,

Figure 2: Considered physical model of the wall with volume meshing

2.4. Initial Conditions

At the initial time, we suppose that:

- the values ​​of the thermophysical parameters of the laterite bricks are those resulting from the measurements of Kabré et al. [9]

- the thermal conductivity depends on the outdoor environment relative moisture which also depends on the period of simulation

- air temperature and moisture rate data for the months of January, April and August are considered for the simulation. These three months represent the main season in Burkina Faso

2.5. Boundaries conditions

The exchanges are made following the normal to the contact surface. The other components of the flows are considered null. These conditions take into account the aforementioned physical phenomena and verify the following equations:

gL+gV.ngT-LVgL.n=(βpvsatTextφext-pvsatTinφint.nαText-Tint.n                                           (12)

Text , exterior ambient temperature and φext , the exterior moisture rate which depends the period (season) during which the experience is curried out.

2.6. Data for mathematical model parameters

The values of the laterite bricks properties are necessary in order to use the mathematical model implemented in COMSOL. Thes values are obtained experimentally from the work of [9].

Air temperature and moisture rate data for the months of January, April and August were measured using MSR.5.28.32 type thermochips. The data for each month represent mean values for the three main season in Burkina Faso as presented below:

- cold and non-rainy season: January;

- warm and non-rainy season: April;

- rainy season: August

Temperature, moisture content in different months and their modelling are given in the table below.

Table 1: Temperature and moisture content in different months modelling

| Temperature and moisture content in different months | Model data |
| --- | --- |
| Sample of temperature for April (°C) | Tt=-0,2672t2+7,2266t -10,694 |
| Sample of moisture content for April (%) | Hrt=-0,0174t4+0,9017t3-16,582t2+125,52t-292,07 |
| Sample of temperature for January (°C) | Tt=-0,0042t4-0,2121t3+3,5764t2-22,637t+68,435 |
| Sample of moisture content for January (%) | Hrt=-0,0049t4+0,2407t3-4,0962t2+27,245t-41,861 |
| Sample of temperature for August (°C) | Tt=-0,0981t2+2,655t+11,362 |
| Sample of moisture rate for August (%) | Tt=-0,2672t2+7,2266t -10,694 |

Remark: The unit of the time is hour. The data used for identifying the temperature and moisture models are measured from 6 am to 6 pm, therefore the equations given in table 1 above are only valid in the same range. The starting value of the time is then t0 = 6 h.

The instruments used to measure temperature and relative humidity are shown in figure 3.

Figure 3: MSR.5.28.32 type thermochips used to measure temperature and relative humidity

2.7. Assumptions

The following assumptions are considered:

- the wall is made with laterite bricks which have same properties,

- the mortar is made of sand-cement-water,

- the medium studied is made up of three phases: solid phase, liquid phase (water) and gas phase (water vapor and air),

- the solid phase is non-deformable,

- the properties of the gas phase are taken as those of an ideal gas,

- The driving forces for transfers are due to the outside (exterior) air temperature T and air humidity level φ,

- liquid water does not flow through the contact surface. Any water flowing to external surfaces is in vapor form.

- RESULTS AND DISCUSSION

| Figure 4a: Measurement data of temperature and model fitting for April | Figure 4b: Measurement data of moisture content and model fitting for April |
| --- | --- |
| Figure 4c: Measurement data of temperature and model fitting for January | Figure 4d: Measurement data of moisture content and model fitting for January |
| Figure 4e: Measurement data of temperature and model fitting for August | Figure 4f: Measurement data of moisture content and model fitting for August |

Temperature and moisture content within different months and their fit with the considered model are given in Figures 4a to 4f.  For each considered month, the corresponding models (see table 1) are used to represent temperature and moisture which are applied on the stressed side of the wall (exterior face). In other words, these models represent the weather conditions (in terms of temperature and moisture) which are applied on the stress side. For each month, the stressed side is exposed for 8 hours and 12 hours. It means that the exterior face is exposed to the sun for 8 hours and 12 hours before recording desired data.

After running the simulation, the temperature and moisture content profiles of the studied wall are plotted as function of the wall thickness and for different periods, Figures 5a to 5f. Theses plots represent the distribution of temperature and moisture content throughout the thickness of the wall (physical model). The exterior face of the wall corresponds to the coordinate x equals zero. The interior face of the wall corresponds to the coordinate x equals to 14 cm, which is the laterite brick thickness. We can notice that the temperature and the moisture content decrease when we are moving from the exterior face to the interior face. It suggests that the wall constitutes a shield against the propagation of heat and mass flow.

For the months of January and April, throughout the wall thickness, the plots of the temperature after 8 and 12 hours (Figures 5c and 5e) are superimposed. It is the same remark for the plots of the moisture, Figures 3d et 3f. It suggests that during these periods, for a given thickness, the temperature and the moisture reach their maximum values before 8 hours of exposure and thus, the wall acts as a shield against the transfer of heat and mass.

For the month of August, the temperature at each step of thickness after 8 hours of exposure is relatively lower than those after 12 hours of exposure, Figures 5a and 5b. On Figure 5e and 5f, we can notice that the variation of the temperature during the day in August is almost three times lower than the one in January and April. Due to the lower gradient of temperature applied on the stressed side on August, the temperature and the moisture rate at each step of thickness vary all along the duration of exposure. By comparing our results with those of [19] whose objective is to follow the evolution of the temperature profiles and the moisture content on the stress side as a function of the thickness, our results are in agreement.

| Figure 5a: Variation of the indoor temperature versus wall thickness - August | Figure 5b: Variation of relative indoor moisture content wall thickness – August |
| --- | --- |
| Figure 5c: Variation of the indoor temperature versus wall thickness - April | Figure 5d: Variation of relative indoor moisture content versus wall thickness – April |
| Figure 5e: Variation of the indoor temperature versus wall thickness - January | Figure 5f: Variation of relative indoor moisture content versus wall thickness – January |

- CONCLUSION

In this work, we used COMSOL software to model the heat and mass exchanges that take place in a wall made of cut laterite bricks. A mathematical predictive model of heat and mass transfer is developed and the simulation results show good fit with experimental data. The study reveals the following:

- a wall's response to thermohydric stress depends on its properties and thickness,

- the thickness of a building's wall is a key parameter in guaranteeing its thermal inertia. The thicker the wall, the more it absorbs temperature and moisture.

These results could be verified experimentally by thermography or magnetic resonance imaging within the wall. Quantitative study of a typical roof insulation performance could be taken into account for a complete comfort analysis of a building made of laterite bricks.

Nomenclature:

Some notations used locally, are not mentioned in the list below. They are described as they appear in the text.

Latin characters:

Cp,a : specific heat of dry air [J/(kg.K)] ;

Cp,l : specific heat of liquid water [J/(kg.K)] ;

Cp,m : specific heat of the dry material [J/(kg.K)];

Lv : latent heat of state change (J/kg)

CT : total specific heat of the porous material [J/(kg.K)]

K : hydraulic conductivity [kg/(Pa.m.s)]

M : molar mass of water (kg/mol)

m : flux density [kg/(m2.s)] ;

n : normal vector to the exchange surface;

ml : liquid flux density [kg/(m2.s)] ;

mv : steam flux density [kg/(m2.s)];

mT : mass flux density under temperature gradient [kg/(m2.s)];

Pvsat : saturation steam pressure (Pa) ;

Pc : capillary pressure (Pa) ;

Pv : partial steam pressure

q : heat flux (W/m2) ;

Rv : water steam constant [J/(kg.K)]

Text : ambient temperature (K);

Tint : wall surface temperature (K);

t : time (s) ;

v : air speed (m/s)

Greek letters:

φext : outdoor relative humidity (%);

φint : indoor relative humidity (%);

β : mass transfer coefficient [kg/(m2.s.Pa)];

δp : wet material permeability to steam [kg/(m.s.Pa)];

α : heat transfer coefficient [W/(m2.K)].

ρv : water steam density (kg/m3) ;

ρm : density of the material (kg/m3)

ρl : density of liquid water (kg/m3)

ω : mass water content (kg/m3) ;

Ω : moisture storage capacity [kg/(m3.Pa)] ;

λ : thermal conductivity of the material [W/(m.K)].

Indices:

a : air ;

c : capillary;

ext. : outdoor/external;

int. : indoor/internal

l : liquid;

m : environment;

p : permeability;

T : temperature;

v : steam/vapor.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

- REFERENCES

[1] - D. SAMRI, Physical analysis and hygrothermal characterization of construction materials: experimental approach and numerical modeling, Doctoral thesis, civil engineering option, National Institute of Applied Sciences of Lyon, France, 2008.

[1] – J. R. PHILIP, D. A. DE VRIES, Moisture Movement in Porous Materials under Temperature Gradients, Transaction American Geophysical Union, Vol. 38, No. 2, April 1957.

[2] – P. CRAUSSE, G. BACON, S. BORIES, Fundamental Study of Coupled Heat-Mass Transfers in Porous Media, International Journal of Heat Mass Transfer, Vol. 24, No. 6, PP 991-1004, 1981.

[3] - K. ABAHRI, R. BELARBI, A. TRABELSI, Contribution to the analytical and numerical study of combined heat and moisture transfers in porous building materials, Building and Environment 46 (2011) 1354-1360,

[4] - B. ALLAM, T. NEHARI, M. L. BENLEKKAM, Building brick wall thermal management optimization and temperature control based on phase change materials integration. Case study of the city of Bechar, Algeria, Journal of Energy Storage Volume 73, Part B, 10 December 2023, 109043

[5] - M. SAIDI, A. S. CHERIF, B. ZEGHMATI, E. SEDIKI, Modeling of heat and mass transfers in a bio-sourced building material, The 9th Tunisian Conference on Flows and Transfers (JTET), 2016.

[6] – S.LARBI, A.AL-SARKHI, A. SZEKRENVES, Heat and Mass Transfer with condensation in capillary porous Bodies, Hindawi Publishing Corporation, http://dx.doi.org/10.1155/2014/194617.

[7] - F. MNASRI, Study of heat and mass transfer in complex media: application to fibrous media and building insulation, Doctoral thesis, Energy Engineering option, University of Lorraine, France, 2016.

[8] – G. Kanellopoulos, V. G. Koutsomarkos, K. J. Kontoleon, K. Georgiadis-Filikas, Numerical analysis and modelling of heat transfer processes through perforated clay brick masonry walls, Procedia environmental sciences, 38, 492-499, 2017,

https://www.sciencedirect.com/science/article/pii/S1878029617301160.

[9] - S. KABRE, F. OUEDRAOGO, B. NAON, A. MESSAN, J.C. BENET, F. ZOUGMORE Evaluation of the thermo-hydro-mechanical properties of compressed earth bricks (CEB) from the Matourkou quarry, Burkina Faso, Afrique SCIENCE 15 (3) (2019)12-22, ISSN 1813 - 548X, http://www.afrique.science.net.

[10] - M. MALIKI, N. LAREDJ, K. BEDANI, H. MISSOUM, Two-Dimensional Transient Modeling of Energy and Mass Transfer in Porous Building Components using, Journal of Applied Fluid Mechanics, Vol.10, N°1, pp 319-328, 2017.

[11] - M. MALIKI, N. LAREDJ, K. BENDANI, H. MISSOUM, Numerical Study of coupled Heat and Mass Transfer in Building Envelopes Based on Temperature and Capillary Pressure Gradients, Proceedings of the 1st Thermal and Fluid Engineering Summer Conference, TFESC, August 9-12, 2015, New York City, USA.

[12] - A. V. LUIKOV, Systems of Differential Equations of Heat and Mass Transfer in Capillary-Porous Bodies, Review, International Journal Heat Mass Transfer, Vol. 18, pp. 1-14, 1975.

[13] – S. MOISSETTE, M. BART, Validation of a heat and moisture transfer code in a wall according to EN 15026, 9th Interuniversity Franco-Quebec Conference on Thermal Systems, May 18-20, 2009, Lille, France.

[14] – J. F. DAÏAN, Equilibrium and Transfers in Porous Media, Manual, Hal-00452876, Version 4-21 October 2013, Joseph Fourier University, Grenoble, France.

[15] - I. TRAORE, Heat and Mass Transfers in the Walls of Timber-Framed Buildings, Doctoral Thesis in Mechanics and Energy, Henri Poincaré University – Nancy University, France, 2011.

[16] – B.A. O. NEBIE, A. LAWANE, C. HEMA, M. SIROUX, Energy Efficiency in Building Sector in Burkina Faso: Literature Review SWOT Analysis, and Recommendation, Energies 2025, 18,2689. https://doi.org/10.33.90/en18112689

[17] – H. N. Suresh, P. A. Aswatha Narayana, K. N. Seetharamu, Conjugate mixed convection heat and mass transfer in brick drying. Heat and mass transfer, 37(2), 205-213, 2001, https://link.springer.com/article/10.1007/s002310000086.

[18] – E. OUEDRAOGO, B. DIANDA, T. S. M. KY, A. OUEDRAOGO, Science Journal of Energy Engineering, Volume 6, Issue 3, 2018.

[19] – H. KAZEOUI, A. SEGHIR, F. BENNAI, N. SAIBI, A. TAHAKOUR, R. BELARBI, A. AIT-MOKHTAR, Modeling of heat and mass transfer in building walls, https://www.researchgate.net/publication/290748934.

20. Kabre S., Kabre M. B., Ouédraogo F., Naon B., Koalaga Z. & Zougmoré F. (2020) Numerical study on heat and mass transfers on a CEB (compressed earth blocks) wall in a sahelian context. IJISET - International Journal of Innovative Science, Engineering & Technology, 7(6)

- ANNEXES

C=C11C12C21C22=λ+hr-vρaCP,a+LVδPφPsat'-φlnφT-psatδPφPsat'-φlnφTKφ-δppsat

β=v.D11D12D21D22=v.-LV∂ρv∂TLV∂ρv∂φ-∂ρv∂T-∂ρv∂φ