[image: image1.png]s

i
Py
iy

(b voressacs o escs

PHVLLITE QUARTE CHLORITE SGHIST AND SREWWAG
e cutonere s cmmHogesTEE

[ preye





[image: image8.png]00000

=~ YN0V ENNS

“+0d LYXL Y)Y
v0s \//0///// i
© @NNONNN 3

=1 @NNONNN 5 7

= NN\ 4
HONNG g &
o QW E ©
sit @\ I\ g
o3 @\ m






EVALUATION OF GROUNDWATER QUALITY USING CORRELATION AND REGRESSION ANALYSIS FROM SHAHAPUR TALUK, NORTHEASTERN KARNATAKA, INDIA
Abstract

An attempt has been made in this work to understand the relationship between  groundwater quality parameters. Groundwater samples from sixty five bore wells of the study area were collected during pre and post monsoon season and concentrations of different physico-chemical parameters were recorded as per the standard methods. A statistical analysis was carried out to understand the linear relation between the best correlated water quality parameters. The relationship for different water quality parameters of the study area was analyzed for two seasons, pre‐monsoon and post‐monsoon, because the water quality varies seasonally. The integration matrix shows that electrical conductivity and total dissolved solids were significantly correlated and have a strong connection with alkalinity, total hardness, calcium, magnesium, chloride, sulphate and sodium and this indicates that these ions significantly contribute to conductivity and dissolved solids. The study showed that total hardness was strongly correlated with calcium and magnesium and these ions were might be major contributors to hardness and  alkalinity was observed to be positively correlated with calcium, magnesium, chloride, sulphate and sodium during pre-monsoon and post-monsoon season. The results of regression analysis reveals that the eight pairs of ground water parameters showing strong regression relationship.

Keywords: Groundwater quality, Correlation coefficient,  Regression analysis, Pre and Post Monsoon season
1. INTRODUCTION
Ground water has become a vital natural resource over the past few decades due to  raise in its usage for domestic, agriculture and industrial uses which results in overexploitation (Wakode et al., 2014: Priya and Arulraj, 2010). The suitability of water for various uses depends on its physico-chemical and biological characteristics of groundwater.  The groundwater quality also varies with depth of water, season, leached dissolved salts and type of geology of the area(Gebrehiwot et al. 2011; Basavaraja Dasappa et al., 2023). Alteration of groundwater quality can reduce quality of drinking water, high costs for alternative water supplies, and probable health risks (Nas and Berktay, 2010). According to the World Health Organization (WHO, 2017), about 80% of all the diseases in human beings are either water or food borne. Therefore, an assessment of water quality is one of the universal issues in underground water studies(Basavaraja Dasappa et al., 2018; Parisa Dargahi, et al., 2022). And also groundwater quality is under ever increasing pressure due to manmade activities and changing climate posing risk to human consumption and affecting vulnerable groups in society. On the other hand,  an excessive groundwater withdrawal makes the sources vulnerable to salinization and contamination processes (Chakraborty et al., 2022). Now a days, an increased attention to groundwater quality, is of greatest importance for the achievement of the Sustainable Development Goals before 2030, especially related to water security (SDG 6), health (SDG 3), and food production (SDG 2)( Sophie Machona et al., 2025). Therefore, an evaluation and monitoring of groundwater quality are vital for the sustainable use of these resources (Taheri et al., 2016, Hemalatha et al., 2025). The correlation coefficient and regression analysis were found to be a highly useful statistical tools for correlating different water quality parameters. Analyzing correlations can help understand the sources of pollution and the interconnected effects of different pollutants. Regression equations can be used to forecast trends in water quality over time, aiding in long-term management strategies(. The main objective of the study was to know relationship between groundwater quality parameters using correlation coefficient and regression analysis in the areas of Shahapur taluk northeastern Karnataka

2. MATERIALS AND METHODS
2.1  Study area

Shahapur is the one of the taluk of Yadgir district of north-eastern region of Karnataka. The study area falls under the survey of India (SOI) toposheets number 56D/10, 56D/13, 56D/14, 56D/15, and 56H/7 lies between 16°22.30' N to 16°53.15 N and longitudes of 77°15.25 E to 77° 37.30 E and covers an area of approximately 1706 km2. The study area experiences a temperature ranging from 12°C to 42°C and 640 mm average annual rainfall. 
[image: image9.png]0 e
W\ 20N0
€-ON ‘ LY LYY /
708 .”'/’/'/ o
AL ALY
+Nm_z "'/./
= @QKRKWN g
HONNN g =
A )\ N
sat| N\ =




Fig. 1 Geology and sampling locations of the study area

2.3 Collection of water samples for analysis

A total of 65 groundwater samples were collected from bore wells in pre-monsoon and post-monsoon season during November and April respectively. The geographical coordinates of the sampling locations were recorded using GPS (Garmin). The groundwater samples of monitoring locations were analysed systematically analysed for various parameters such as the pH, electrical conductivity(EC), total dissolved solids(TDS), total alkalinity, total hardness, calcium(Ca2+), magnesium(Mg2+), sulphate(SO42-), chloride(Cl-), fluoride(F-), nitrate(NO3-), phosphate(PO42-), sodium(Na+) and potassium(K+) as per the standard methods of APHA (2012) and BIS (1991).

2.4 Statistical methods
Descriptive statistics also called as summary statistic viz., Minimum, Maximum, Average, Standard Deviation and Coefficient of variation was calculated.

Correlation analysis: Simple correlation analysis was carried to analyse the linear association between different water quality parameters of different ground water samples at various locations of the study area as mentioned in Dutta and Sarma, 2018: Menon et al ,2023. The correlation analysis was carried out in R studio software. The correlation coefficients were represented in the diagrammatic form also called as correlation plot. The correlation coefficient ( r) was calculated by using the formula given below;
[image: image2.png]nixy—LxLy

vG) = v(y)







Where,




r = Correlation Coefficient (-1≤ r ≤ 1)




n = Number of samples




x and y = two variables (ground water quality parameters)




v(x) and v(y) = variance of x and y respectively
Regression analysis: Simple linear regression analysis was carried out to find increase or decrease in one independent (x) variable will affect the other dependent variable (y). Analysis was carried out in R studio software and regression equation is given as follows;

[image: image3.png]y=a+bx+e






Where, 




y = Dependent variable




x = Independent variable




a = Intercept, [image: image5.png]






b = Regression coefficient, [image: image7.png]Lov (xy)
o







e = Error term

The significance of Correlation coefficient ( r ), Intercept(a) and Regression coefficient ( b ) was tested by Student t test. 
3. RESULT AND DISCUSSION 
3.1 Descriptive statistics for different parameters of ground water samples

Descriptive Statistics (Minimum, Maximum, Average, Standard Deviation and Coefficient of variation (CV)) for different parameters such as PH, EC, TDS, Alkalinity, Total Hardness, Ca2+, Mg2+, Cl-, SO₄²⁻, F-, NO3-, PO42-, Na+ and K+ of ground water samples at different locations of Shahapur taluk for both Pre and Post monsoon season was calculated and the results were tabulated in Table 1a and Table 1b for Pre and Post monsoon season respectively. From Table 1a, it was found that the parameter PO4-- (232. 69) had the highest CV followed by Cl- (176.75), F- (134.42), EC (113.48), and TDS (110.94) for Pre monsoon season. Similarly for Post monsoon season period K+ (223.26) has the highest CV followed by PO42- (174.26), Cl- (142.55), Na+ (136.69) and Ca2+ (117.10). Hence, it can be interpreted that PO42- has high variability compared to other parameters but in post monsoon season K+ parameter was highly variable.  
Table 1a: Descriptive statistics of different quantitative variables of ground water samples of different locations of Shahapur taluk for pre-monsoon season

	
	Minimum
	Maximum
	Average
	Standard deviation
	Coefficient of variation

	pH
	6.08
	7.94
	7.20
	0.34
	4.69

	EC
	654.00
	22600.00
	2881.45
	3269.88
	113.48

	TDS
	352.00
	11500.00
	1530.08
	1697.48
	110.94

	Alk
	136.00
	848.00
	420.62
	146.38
	34.80

	TH
	70.00
	2150.00
	539.09
	434.57
	80.61

	Ca2+
	9.61
	448.45
	105.47
	87.60
	83.06

	Mg2+
	5.78
	267.06
	60.77
	61.45
	101.13

	Cl-
	37.73
	4446.31
	416.41
	736.00
	176.75

	SO₄²⁻,
	45.70
	1906.56
	334.17
	316.32
	94.66

	F-
	0.00
	2.93
	0.38
	0.52
	134.42

	NO3-
	0.242
	7.422
	4.943
	1.893
	38.30

	PO42-
	0.001
	0.197
	0.011
	0.025
	232.29

	Na+
	4.40
	106.45
	42.89
	28.48
	66.40

	K+
	0.31
	170.30
	41.79
	45.59
	109.09


Note: All parameters are in mg/L except pH and electrical conductivity (EC) is in µS/cm
Table 1b: Descriptive statistics of different quantitative variables of ground water samples of different locations of Shahapur taluk for post-monsoon Season

	
	Minimum
	Maximum
	Average
	Standard deviation
	Coefficient of Variation

	pH
	6.83
	8.09
	7.35
	0.29
	3.88

	EC
	562.00
	14200.00
	2591.54
	2741.09
	105.77

	TDS
	297.00
	7450.00
	1351.40
	1424.78
	105.43

	Alk
	356.00
	1656.00
	858.86
	277.99
	32.37

	TH
	104.00
	3220.00
	613.48
	647.08
	105.48

	Ca2+
	16.02
	903.30
	125.20
	146.61
	117.10

	Mg2+
	13.08
	444.28
	69.58
	78.79
	113.24

	Cl-
	56.73
	3191.13
	441.85
	629.85
	142.55

	SO₄²⁻,
	61.44
	1829.76
	356.45
	363.63
	102.01

	F-
	0.15
	5.32
	1.24
	0.87
	69.85

	NO3-
	0.30
	6.79
	4.53
	2.02
	44.67

	PO42-
	0.00
	0.05
	0.00
	0.01
	174.26

	Na+
	3.95
	2804.50
	260.74
	356.41
	136.69

	K+
	0.01
	748.50
	67.95
	151.69
	223.26


Note: All parameters are in mg/L except pH and electrical conductivity (EC) is in µS/cm
3.2 Correlation analysis

Correlation analysis was carried out between fourteen parameters of ground water samples for pre and post monsoon season, which explains the nature of linear association between two variables and the results were represented as correlation matrix depicted in table 2a and 2b for pre and post monsoon season respectively. 


From table 2a it was observed that pH was significantly negatively correlated with most parameters viz., total hardness (-0.81), Ca²⁺ (-0.79), SO₄²⁻ (-0.81), NO₃⁻ (-0.87), suggests that as pH increases, these concentrations decreases. Significantly positively correlated with F⁻ (0.64). EC and TDS (1.00) were significantly positively correlated and EC had strong positive correlation with alkalinity, total hardness, Ca²⁺, Mg²⁺, Cl⁻, SO₄²⁻, Na⁺ indicates that these ions contribute significantly to conductivity and dissolved solids. Similar observation was reported by Mehta(2010) in groundwater of Vadgam taluka  in  Banaskantha  district  of  Gujarat  state. Total Hardness was strongly correlated with Ca²⁺ (0.89) and Mg²⁺ (0.95) these were might be primary contributors to hardness, whereas it was significantly perfectly positively correlated with SO₄²⁻ (1.00). Alkalinity was observed to be positively correlated with Ca²⁺, Mg²⁺, Cl⁻, SO₄²⁻, Na⁺ These ions may buffer pH and contribute to alkalinity. F⁻ was significantly negatively correlated with most ions except pH (0.64) and K⁺ (0.57). While, Nitrate (NO₃⁻) and Phosphate (PO42-) showed weak or non-significant correlation.


Similarly, correlation analysis was employed for post monsoon season and the results were represented in table 2b, from this it can be concluded that pH had strong negative correlation with most parameters viz., EC, TDS, Total Hardness, Ca²⁺, Mg²⁺, Cl⁻, SO₄²⁻ and positive correlation with F⁻. There was a significant perfect positive correlation between EC and TDS and strong positive correlations with Total Hardness, Ca²⁺, Mg²⁺, Cl⁻, SO₄2⁻, Na⁺, and K⁺.  There was significant moderate positive correlation between alkalinity and total hardness, Ca²⁺, Mg²⁺, Cl⁻, SO₄²⁻ and there exists a weak or no correlation with nutrients like NO₃⁻ and PO₄⁻. Further for the parameters Total Hardness, Ca²⁺, Mg²⁺ there was strong inter-correlations among them and also strongly correlated with Cl⁻ and SO₄²⁻. Whereas, F⁻ negative correlation with most parameters. There was weak or no significant correlations between NO₃⁻, PO₄⁻ and other parameters. Finally moderate positive correlation was observed between Na+ and K+ with EC, TDS, and other cations.
Table 2a: Correlation analysis between different parameters of ground water samples at different locations of Shahapur taluk for pre-monsoon season

	
	PH
	EC
	TDS
	Alk
	TH
	Ca2+
	Mg2+
	Cl-
	SO4
	F-
	NO-3
	PO4--
	Na+
	K+

	PH
	1
	-0.59*
	-0.60*
	-0.66*
	-0.81**
	-0.79**
	-0.72**
	-0.59**
	-0.81**
	0.64*
	-0.87**
	-0.03 NS
	-0.36
	-0.69**

	EC
	
	1
	1.00**
	0.85**
	0.84**
	0.83**
	0.78**
	0.98**
	0.81**
	-0.40NS
	0.36 NS
	0.39 NS
	0.84**
	0.18 NS

	TDS
	
	
	1
	0.85**
	0.84**
	0.84**
	0.78**
	0.98**
	0.81**
	-0.40 NS
	0.36 NS
	-0.39 NS
	0.84**
	0.36 NS

	Alk
	
	
	
	1
	0.77**
	0.79**
	0.70**
	0.81**
	0.75**
	-0.36 NS
	0.44 NS
	-0.3 NS
	0.89**
	0.38 NS

	TH
	
	
	
	
	1
	0.89**
	0.95**
	0.83**
	1.00**
	-0.65*
	0.62*
	-0.23 NS
	0.57**
	0.48 NS

	Ca2+
	
	
	
	
	
	1
	0.72**
	0.86**
	0.86**
	-0.66**
	0.55*
	-0.22 NS
	0.64*
	0.26 NS

	Mg2+
	
	
	
	
	
	
	1
	0.75**
	0.96**
	-0.56*
	0.58 NS
	-0.23 NS
	0.50 NS
	0.17 NS

	Cl-
	
	
	
	
	
	
	
	1
	0.79**
	-0.45 NS
	0.38 NS
	-0.39 NS
	0.80**
	0.38 NS

	SO4
	
	
	
	
	
	
	
	
	1
	-0.64*
	0.62*
	-0.21 NS
	0.54*
	-0.51 NS

	F-
	
	
	
	
	
	
	
	
	
	1
	-0.63*
	-0.31 NS
	-0.05 NS
	0.57*

	NO-3
	
	
	
	
	
	
	
	
	
	
	1
	0.02 NS
	0.11 NS
	0.14 NS

	PO4--
	
	
	
	
	
	
	
	
	
	
	
	1
	-0.46 NS
	0.10 NS

	Na+
	
	
	
	
	
	
	
	
	
	
	
	
	1
	0.20NS

	K+
	
	
	
	
	
	
	
	
	
	
	
	
	
	1


*** = Significant at 0.001 level,
** = Significant at 0.01 level, * = Significant at 0.5 level, 



NS = Non-significant

Table 2b: Correlation analysis between different parameters of ground water samples at different locations of Shahapur taluk for post-monsoon season

	
	PH
	EC
	TDS
	Alk
	TH
	Ca2+
	Mg2+
	Cl-
	SO4
	F-
	NO-3
	PO4--
	Na+
	K+

	PH
	1
	-0.91**
	-0.91**
	-0.58*
	-0.95**
	-0.94**
	-0.92**
	-0.91**
	-0.95**
	0.84**
	-0.37 NS
	-0.69**
	-0.63*
	-0.73**

	EC
	
	1
	1.00**
	0.58*
	0.98**
	0.95**
	0.97**
	1.00**
	0.98**
	-0.72**
	0.23 NS
	0.51NS
	0.82**
	0.65*

	TDS
	
	
	1
	0.59*
	0.98**
	0.95**
	0.97**
	1.00**
	0.98**
	-0.72**
	0.23 NS
	0.51NS
	0.82**
	0.65*

	Alk
	
	
	
	1
	0.51*
	0.44*
	0.58*
	0.54*
	0.51**
	-0.28 NS
	0.01 NS
	0.26NS
	0.53*
	0.54*

	TH
	
	
	
	
	1
	0.98**
	0.98**
	0.98**
	1.00**
	-0.81**
	0.28 NS
	0.65*
	0.73**
	0.63*

	Ca2+
	
	
	
	
	
	1
	0.92**
	0.96**
	0.98**
	-0.84**
	0.29 NS
	0.66*
	0.62*
	0.67**

	Mg2+
	
	
	
	
	
	
	1
	0.97**
	0.97**
	-0.74**
	0.25 NS
	0.61*
	0.83**
	0.54*

	Cl-
	
	
	
	
	
	
	
	1
	0.98**
	-0.74**
	0.21 NS
	0.55*
	0.81**
	0.63*

	SO4
	
	
	
	
	
	
	
	
	1
	-0.81**
	0.28 NS
	0.65*
	0.72**
	0.63*

	F-
	
	
	
	
	
	
	
	
	
	1
	-0.52*
	-0.70**
	-0.34NS
	-0.68**

	NO-3
	
	
	
	
	
	
	
	
	
	
	1
	0.01NS
	0.10NS
	0.18NS

	PO4--
	
	
	
	
	
	
	
	
	
	
	
	1
	0.19NS
	0.37NS

	Na+
	
	
	
	
	
	
	
	
	
	
	
	
	1
	0.30NS

	K+
	
	
	
	
	
	
	
	
	
	
	
	
	
	1


*** = Significant at 0.001 level,
 ** = Significant at 0.01 level, * = Significant at 0.5 level, 


NS = Non-significant

                           (1a)                                                                                              (1b)

Fig. 2a & 2b: Correlation plot between different parameters of ground water samples at different locations of Shahapur taluk for pre and post monsoon seasons respectively.

3.3 Regression analysis
Regression analysis was employed as mathematical tool to estimate various dependent water quality characteristics by substituting values of the corresponding independent parameters. The regression analysis identified specific water quality parameters that exhibited stronger and more statistically significant correlations (Dutta and Sarma. 2018). The results of regression analysis for eight pairs of ground water parameters such as EC-Cl-, Mg2+-Cl-, Mg2+- TDS, EC-TDS, total hardness-Ca2+, total hardness - Mg2+, EC-Na+and alkalinity-total hardness for pre- monsoon and post-monsoon season were presented in Table 3a and 3b respectively.  In Table 3a, TDS was found to be the most powerful predictor of EC as R2+ value is 0.997 indicates nearly all the variability in the model was explained and both coefficients were significant. In addition to that, TDS was significantly influenced by Ca2+ and Mg2+ compared to Ca2+, Mg2+ had stronger impact on TDS. Further, the regression equations with R2 value less than 0.5 shows the weaker regression fit though the coefficients were statistically significant. In table 3b, TDS and Cl- are the most influential predictors of EC with R² values of 0.990 and 0.896 respectively. Mg2+ was well predicted by both Cl- and TDS, though the intercept for TDS is not statistically significant. Total Hardness was strongly influenced by both Ca2+ and Mg2+ with slightly higher predictive power from Ca2+. Alkalinity shows minimal dependence on total hardness indicating other factors may be more relevant.
Table 3a: Regression analysis between different parameters of ground water samples at different locations of Shahapur taluk for pre monsoon season  

	Dependent variable (y)
	Independent variable (x)
	Intercept
	Regression coefficient
	R2

	EC
	Cl-
	1297.24***
	3.804***
	0.733

	Mg2+
	Cl-
	43.121***
	0.042***
	0.257

	Mg2+
	TDS
	26.586**
	0.022***
	0.381

	EC
	TDS
	-62.26*
	1.923***
	0.997

	TH
	Ca2+
	160.271**
	3.592***
	0.524

	TH
	Mg2+
	166.87***
	6.125***
	0.750

	EC
	Na+
	-545.15NS
	79.89***
	0.484

	Alk
	TH
	316.49***
	0.193**
	0.328


*** = Significant at 0.001 level, ** = Significant at 0.01 level, * = Significant at 0.5 level, 


NS = Non-significant
Table 3b: Regression analysis between different parameters of ground water samples at different locations of Shahapur taluk for Post monsoon season   
	Dependent variable (y)
	Independent variable (x)
	Intercept
	Regression coefficient
	R2

	EC
	Cl-
	770.6538***
	4.1211***
	0.896

	Mg2+
	Cl-
	22.433***
	0.106***
	0.727

	Mg2+
	TDS
	4.65NS
	0.048***
	0.754

	EC
	TDS
	3.671NS
	1.919***
	0.990

	TH
	Ca2+
	100.110*
	4.100***
	0.863

	TH
	Mg2+
	100.460*
	7.372***
	0.806

	EC
	Na+
	1207.090***
	5.310***
	0.477

	Alk
	TH
	757.763***
	0.164***
	0.147


*** = Significant at 0.001 level,  ** = Significant at 0.01 level, * = Significant at 0.5 level, 


NS = Non-significant

4. CONCLUSION


The statistical correlation and regression analysis were found to be a extremely useful technique. Studying linear correlation between various physico-chemical parameters groundwater quality could be considered as a distinctive step towards management of  the drinking water quality. It can be concluded that the electrical conductivity, total dissolved solids, alkalinity, total hardness, calcium, magnesium, chloride, sodium are important physico-chemical parameters of drinking water quality, because they are strong correlated with most of the water quality parameters in Shahapur taluk. Thus the study could be more useful for predicting groundwater quality changes in future.
ETHICAL APPROVAL
The author(s) have declared ethics committee/IRB approval is not relevant to this content

CONSENT- Not applicable
REFERENCES:
1. APHA.(2012). Standard Methods for the Examination of Water and Wastewater. 22nd Edition, American Public Health Association, American Water Works Association, Water Environment Federation, Washington, D.C.

2. Basavaraja Dasappa, Janardan Bhima Karnbale & Durgappa Kenchappa Hadimani. (2018). Spatial Analysis of Groundwater Quality Using Geographic Information System in Shahapur Town and its Surrounding Area of District Yadgir, Karnataka. Research iournal of Agricultural Sciences, 9(2), 355-371. DI : 4848-2712-2017-079.
3. Basavaraja Dasappa, Janardan Bhima Kambale, & Durgappa Kenchappa Hadimani. (2023). Evaluation of Groundwater Quality for Irrigation Using GIS in Northeastern Karnataka, India. Int.J.Curr.Microbiol.App.Sci.,12(06), 251-259.  DOI: https://doi.org/10.20546/ijcmas.2023.1206.031
4. BIS (Bureau of Indian Standards) 10500, Indian standard drinking water- specification, First revision, 1991, pp 1-8.
5. Dutta, B., & Sarma, B. (2018). Correlation study and regression analysis of ground water quality assessment of Nagaon Town of Assam, India. International Journal of Engineering Research & Technology (IJERT), 7(6), 320-33.
6. Gebrehiwot, A.B., Tadesse, N., & Jigar, E. (2011). Application of Water Quality Index to Assess Suitablity of Groundwater Quality for Drinking Purposes in Hantebet Watershed, Tigray, Northern Ethiopia. ISABB Journal of Food and Agriculture Science, 1, 22-30.
7. Hemalatha, S., Ravanashree, M., Rajesh, G. M., Karthika, V., Arunkumar, N., Narmadha, R., Agila, C., & Suresh, R. (2025). Water Quality Index-based Groundwater Assessment for Irrigation and Drinking Purposes in Erode District, Tamil Nadu, India. Arch. Curr. Res. Int., 25(9), 219-229. 
8.   Mehta,  K.  V.  (2010).  Physicochemical  characteristics  and statistical study of groundwater of some places of Vadgam taluka  in  Banaskantha  district  of  Gujarat  state  (India). J. Chem. Pharm. Res., 2(4), 663-670.

9. Memon, Y. I., Qureshi, S. S., Kandhar, I. A., Qureshi, N. A., Saeed, S., Mubarak, N. M.,& Saleh, T. A. (2023). Statistical analysis and physicochemical characteristics of groundwater quality parameters: a case study. International Journal of Environmental Analytical Chemistry, 103(10), 2270-2291.

10. Chakraborty, M., Tejankar, A., Coppola, G., & Chakraborty, S. (2022).  Assessment of groundwater quality using statistical methods: a case study. Arab J Geosci., 15, 1136. https://doi.org/10.1007/s12517-022-10276-2.
11. Nas, B., & Berktay, A. (2010). Groundwater quality mapping in urban groundwater using GIS. Environ Monit Assess., 160, 215–227. https://doi.org/10.1007/s10661-008-0689-4.
12. Parisa Dargahi., Simin Nasseri., Mahdi Hadi., Ramin Nabizadeh Nodehi., & Amir Hossein Mahvi, (2022). Prediction models for groundwater quality parameters using a multiple linear regression (MLR): a case study of Kermanshah. Iran. J. EnvIron. Health Sci. Eng., 21(1), 63-71.
13. Priya, K. L., &  Arulraj, G. P. (2011). A correlation-regression model for the physicochemical parameters of the groundwater in Coimbatore city, India. Environmental technology, 32(7), 731-738.

14. Soni Chaubey., & Mohan Kumar Patil. (2015). Correlation Study and Regression Analysis of Water Quality Assessment of Nagpur City, India. International Journal of Scientific and Research Publications, 5(11), 753-757
15. Sophie Machona, George Morara Ogendi, & Bayongwa Samuel Ahana. (2025). Assessing the quality of groundwater is therefore crucial to ensuring its safety for drinking and other uses. Water Quality Research Journal, 60(1),151-163. doi: 10.2166/wqrj.2025.047.
16. Taheri, K., Taheri, M., & Parise, M. (2016). Impact of intensive groundwater exploitation on an unprotected covered karst aquifer: a case study in Kermanshah Province, western Iran. Environ Earth Sci., 75, 1221. https://doi.org/10.1007/s12665-016-5995-5.
17. Wakode, H. B., Baier, K., Jha, R., Ahmed, S., & Azzam, R. (2014). Assessment of Impact of Urbanization on Groundwater Resources using GIS Techniques- Case Study of Hyderabad. India. Int. J. Environ. Res., 8(4), 1145-1158.
18. WHO (World Health Organization). (2017). Guidelines for drinking water quality, fourth edition incorporating first addendum. Geneva: WHO.


