OCCURRENCE AND DISTRIBUTION OF SOME CONTAMINANTS OF EMERGING CONCERN IN SEDIMENTS OF IMO RIVER NIGERIA: A NON- TARGETED ANALYSIS

ABSTRACT

This is a non – targeted assessment of the occurrence and distribution of contaminants of emerging concern in sediment samples of the Imo River, Nigeria. Three (3) sampling locations -  Ekenobizi (Imo State), Owerrinta (Abia State) and Oyigbo (Rivers State) along the Imo river were selected for the study based on high human density with much anthropogenic activities. Sediment samples were collected from upstream, midstream and downstream of each of the sampled locations using the grab sampling method and taken to the laboratory for analysis within three hours of sampling.  A non- targeted screening for CECS was done using gas chromatography – mass spectrometry (GC-MS) after preparation of the samples. The results showed contamination of the river with eighty – four (84) unique CECs distributed as follows; 21 CECs at Ekenobizi axis, 32 CECs at Owerrinta axis and 34 CECs at Oyigbo axis across the upper stream (US), midstream (MS) and downstream (DS) points. Some of the CECs were location specific while some were detected across the three sampling points of each location. Three of the detected CECs were common to both the Owerrinta and Oyigbo axes. The Anova results showed no significant difference among the CECs concentrations of the sampled locations, therefore the null hypothesis was accepted (where p > 0.05, p = 0.909).  Among the detected compounds were siloxanes, fatty acids, amines, hydrocarbons, and other organic chemicals. These chemicals are traceable to pharmaceutical and personal care prohucts, agro chemicals, and other industrial chemicals and they exhibit varying degrees of ecological risks.

INTRODUCTION

Contaminants of emerging concern (CECs) also referred to as emerging contaminants (ECs), or emerging pollutants (EPs), can be defined as newly identified synthetic or naturally occurring chemicals or biological agents that are detected in the environment and potentially hazardous or recently determined to be hazardous to humans and ecosystems. The risks associated with these contaminants are not fully understood, their toxicological significance is difficult to assess and generally accepted concentration limits for drinking water and discharge limits for wastewater effluent have not yet been established (Pal et al., 2014)  . They may include pharmaceuticals and personal care products (PPCPs), per- and poly-fluoroalkyl substances (PFAS), endocrine disruptors (EDs),   emerging pathogens, cyanotoxins and other natural toxins, pesticides, industrial chemicals, micro/nano plastics, nanomaterials, antibiotic resistance genes (ARGs), and other exogenous substances that are found in the environment but are not yet well understood in terms of their impacts on humans and natural ecosystems (Fang et al., 2024; Enyoh et al., 2020; Ebele et al., 2017).

The  increasing presence of contaminants of emerging concern (CECs) in the aquatic environment has become a global concern due to potential adverse effects CECs could have on aquatic life, human health and the entire environment even at low concentrations (Sauve & Desrosiers, 2014; Yuange et al., 2024 Jianglu et al., 2024). Previous studies have indicated that the aquatic environment and its sediment serve as sinks for these CECs which have the potential to persist and accumulate, posing risks to benthic organisms and potentially affecting human health through bioaccumulation and biomagnification in the food chain (Fairbairn et al., 2015, Afolabi et al., 2024). In a study by Jobling et al (2006), the presence of CECs was linked to adverse effects like developmental and reproductive problems in aquatic species.

The Imo River located in south eastern Nigeria is a very important water resource that supports different human activities including, but not limited to fishing, agriculture, and domestic usage (Ogbonna et al., 2021). This study aims to assess the occurrence and distribution of contaminants of emerging concern in Imo River sediments, using a non-targeted approach in order to give insights into the types and levels of CECs present in the river and possible ecological risks. This study contributes to the growing body of knowledge on these contaminants, which is necessary to inform strategies for mitigation of their impacts.

METHODS

Study Area

The Imo River, located in Southeastern Nigeria originates from the Imo State, and flows through Abia,, Rivers and Akwaibom states of Nigeria into the  Atlantic ocean (Ogbonna et al., 2021).

According to the location/topographic map of the study region, the Imo River Basin is located between latitudes 4° 38'N and 6° 01'N and between longitudes 6° 53'E and 7° 32'E. It has an approximate area of 9100 km2 (Uma, 1989). Three (3) sampling locations along the Imo river were selected for the study based on high human density with much anthropogenic activities. The three locations were, Ekenobizi (Imo State), Owerrinta (Abia State) and Oyigbo (Rivers State) (Figure 1).

Sampling

Sediment samples were collected upstream, midstream and downstream from each of the locations (Ekenobizi, Owerrinta and Oyigbo axes of the river) using the grab sampling method as described in Blomqvist,S. (1991) through the help of skilled local fishermen. These samples were placed in properly labelled polythene bags and taken to the laboratory for analysis within three hours after sampling.

Figure 1: Map of Study Area and Sampling points

Laboratory Analysis

The method used for the laboratory analysis was the Gas Chromatography/Mass Spectrometry (GC/MS) scan. The Gas Chromatography/Mass Spectrometry (GC/MS) instrument separates chemical mixtures (the GC component) and identifies the components at a molecular level (the MS component). The GC works on the principle that a mixture will separate into individual substances when heated. The heated gases are carried through a column with an inert gas (such as helium). As the separated substances emerge from the column opening, they flow into the MS. Mass spectrometry identifies compounds by the mass of the analyte molecule. A library of known mass spectra, covering several thousand compounds, is stored on a computer. Mass spectrometry detector identifies these analytes and match them with library. Identification is based on the molecular structure, molecular mass and calculated fragments elucidated by the MS Quadrupole mass analyser filtered on a mass to charge basis by the HED (High energy diode). They are qualitatively interpreted with aid of National Institute standard and Technology (NIST) spectrum database NIST 08 model. The name, molecular weight and structure of the components of the material are ascertained by use of the library.

10g from each of the sediment samples was used for extraction using methanol as solvent. The extract was allowed to concentrate in the fume hood and packed in the GC vial at exactly 1ml concentration.

The relative percentage amount of each component was obtained by comparing its average peak area to the total area, the spectrum of the unknown components are compared to the 2008 version, through which the various spectra extractions and interpretation are obtained.

Statististical analysis

The study adopted tabular representation of findings and the use of a pie chart to summarize and describe findings for clarity.

The hypotheses of the study was formulated and tested using the Analysis of Variance (ANOVA) at 95% level of significant. As inferential statistics, ANOVA allows for exploring the statistically significant differences between two or more variances. One-ANOVA was adopted for the hypotheses testing and it is expressed as thus;

F=MSTMSE

MST=i=1kTi2ni-G2nk-1

MSE=i=1kj=1niYij2-i=1kTi=12nin-k

Where;

F= Variance ratio of the overall test

MST= Mean Square due to treatment /groups (between groups)

MSE= Mean square due to error (within groups, residual mean square)

Yij= an Observation

Ti= Group total

G= Grand total of all observations

ni= number in the group i and n is total number of observations

RESULTS

Contaminants of emerging concerns (CECs) in sediment samples along the Imo River

The CECs in the sediment across the sampled points/axis of the Imo river are presented in Table 1a to 1c and Figure 2. From the outcome, a total of eighty-four (84) CECs were detected which detailed  as 21 CECs at Ekenobizi axis (24%), 32CECs at Owerrinta axis (37%) and 34 CECs at Oyigbo axis (39%) across the U..S, MS and D.S points. The sediment samples from Owerrinta and Oyigbo axis had  in common  three (3) CECs; Hexadecane, 2,6,10,14-tetramethyl-, Heptacosane and Dodecane, 2,6,10-trimethyl-.

Figure 2: Total CECs in Sediments across the Sampled Axes of the Imo River

Table 1a: CEC Concentrations in the Sediment Samples at Ekenobizi Axis of Imo River

|  | Name Of Compound | Formulae | Sediment Conc.(mg/kg) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | U.S | M.S | D.S |
| 1 | Acetaldehyde | C2H4O | 816 | 671 | 375 |
| 2 | Ethylene oxide | C2H4O | 527 | 475.5 | 232 |
| 3 | Eicosamethyl cyclodecasiloxane, - | C20H60O10Si10 | 4.43 | 7.62 | ND |
| 4 | Ethyl 2-((diethoxyphosphoryl)oxy)- 3,3,3 trifluoropropanoate | C9H16F3O6P | 761 | 1162 | ND |
| 5 | 2-methyl Adenosine, - | C11H15N5O4 | 522.5 | 567 | 524 |
| 6 | 1-Heptadecanamine | C17H37N | 458 | 371 | ND |
| 7 | Hexadecanoic acid, 2,2,3,3,4,4,5,5 ,6,6,7,7 dodecafluoroheptyl ester | C23H34F12O2 | 589 | 473 | ND |
| 8 | 1-Butanamine | C4H11N | 673 | 1020 | 619 |
| 9 | N-Ethylformamide | C3H7NO | 883.5 | 393.5 | 220 |
| 10 | Nonadecylamine | C19H41N | ND | 322 | ND |
| 11 | Tridecylamine | C13H29N | ND | ND | 439 |
| 12 | 2-Methyl-1,5-(4H)-dihydropyrido-(2 ,3-b)1,4- diazepine-4-one | C9H9N3O | 957 | 320 | ND |
| 13 | 7-methyloct-3-yn-5- yl propyl ester Phthalic acid | C20H26O4 | ND | ND | 329 |
| 14 | Tributyl(pentafluorophenyl)- Stannane | C18H27F5Sn | 1194 | 397 | ND |
| 15 | Glycyl-L-phenyl alanine | C11H14N2O3 | 365 | 364 | ND |
| 16 | 4,5-dimethyl-3-(1-pyrrolyl)-thiophene-2-carboxylic acid, | C11H11NO2S | 405 | 699 | ND |
| 17 | Hydantoic acid | C3H6N2O3 | ND | ND | 720 |
| 18 | L-Alanine, 3-[(aminocarbonyl)amino ]- | C4H9N3O3 | ND | ND | 344 |
| 19 | 4,6-dimethoxy-5-nitro- Pyrimidine | C6H7N3O4 | ND | ND | 935.5 |
| 20 | N-glycyl-DL-Leucine, | C8H16N2O3 | ND | ND | 817 |
| 21 | Hex-5-enylamine | C6H13N | ND | ND | 320 |

U.S = Up Stream, M.S = Mid Stream, D.S = Down Stream, ND = Not detected

From table 1a (Ekenobizi axis), Acetaldehyde (C₂H₄O) was significantly high with 816 mg/kg (U.S) and 375 mg/kg (D.S). Ethyl 2-((diethoxyphosphoryl)oxy) - 3,3,3 trifluoropropanoate (C9H16F3O6P) was 761 mg/kg (U.S), 1162 mg/kg (M.S). Highest values included Tributyl (pentafluorophenyl)stannane at 1194 mg/kg (U.S). Other CECs like Hydantoic acid, N-glycyl-DL-Leucine, 4,6-dimethoxy-5-nitro-Pyrimidine appeared only downstream, suggesting accumulation and sedimentation downstream. From the outcome, Acetaldehyde (C2H4O), Ethylene oxide (C2H4O3), 2-methyl Adenosine (C11H15N5O4), 1-Butanamine (C4H11N), N-Ethylformamide (C3H7NO) were detected across upstream, midstream and downstream.

Table 1b: CEC Concentrations in the Sediment Samples at Owerrinta Axis of Imo River

|  | Name Of Compound | Formulae | Sediment Conc.(mg/kg) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | U.S | M.S | D.S |
| 1 | n-Hexadecanoic acid | C16H32O2 | 2764 | 547 | ND |
| 2 | Hexadecane, 2,6,10,14-tetramethyl- | C20H42 | ND | ND | 8166 |
| 3 | Hexadecane, 2,6,11,15-tetramethyl- | C20H42 | ND | ND | 1306 |
| 4 | Octatriacontyl pentafluoropropionate | C41H77F5O2 | ND | 481.5 | 2 |
| 5 | cis-Vaccenic acid | C18H34O2 | 3042 | ND | ND |
| 6 | Oleic Acid | C18H34O2 | 79 | ND | ND |
| 7 | 1-Nonadecene | C19H38 | 178.5 | 1705 | ND |
| 8 | Sebacic acid, di(2-propylpentyl) ester | C26H50O4 | ND | 860 | ND |
| 9 | Heptacosane | C27H56 | ND | 312 | ND |
| 10 | Tridecane | C13H28 | ND | ND | 380 |
| 11 | L-Serine, N-(trifluoroacetyl)-, 1- methylpropyl ester, trifluoroacetate (ester) | C11H13F6NO5 | ND | ND | 3 |
| 12 | Decanoic acid, 10-(2-hexylcyclopropyl) | C19H36O2 | ND | ND | 1 |
| 13 | Bicyclo[4.4.1]undeca-1,3,5,7,9-pen taene | C11H10 | ND | ND | 3 |
| 14 | Spiro[furan-2(3H),1'(3'H)-isobenzofuran]-3,3'-dione, 4- phenyl | C17H10O4 | ND | ND | 1 |
| 15 | i-Propyl 11,12-methylene-octadecan oate | C22H42O2 | ND | ND | 1 |
| 16 | Dodecane, 2,6,10-trimethyl- | C15H32 | ND | ND | 66 |
| 17 | 3-Methyl-4-(methoxycarbonyl)hexa-2,4-dienoic acid | C9H12O4 | ND | ND | 72 |
| 18 | Cyclohexane, 1-(1,5-dimethylhexyl) | C20H40 | 207 | ND | ND |
| 19 | Octadecane, 1-(ethenyloxy)- | C20H40O | 116 | ND | ND |
| 20 | Octadecanoic acid | C18H36O2 | 623 | ND | ND |
| 21 | 2-Pentene-1,4-dione, 1-(1,2,2-trim ethylcyclopentyl) | C13H20O2 | 97 | ND | ND |
| 22 | 9,17-Octadecadienal, (Z)- | C18H32O | 189 | ND | ND |
| 23 | 9-Octadecenoic acid (Z)-, 2-hydrox y-1- (hydroxymethyl)ethyl ester | C21H40O4 | 110 | ND | ND |
| 24 | 13-Octadecenal, (Z)- | C18H34O | 153 | ND | ND |
| 25 | p-Nitrophenyl nonyl ether | C15H23NO3 | 500 | ND | ND |
| 26 | n-Propyl 11-octadecenoate | C21H40O2 | 1679 | ND | ND |
| 27 | 1-Hexacosene | C26H52 | ND | 394 | ND |
| 28 | Pentadec-7-ene, 7-bromomethyl- | C16H31Br | ND | 1972 | ND |
| 29 | Pyrrolidine-1,2-dicarboxylic acid 2-(4-tertbutylbenzylamide) 1-phenylamide | C23H29N3O2 | ND | 234 | ND |
| 30 | 2-Dodecen-1-yl(-)succinic anhydride | C16H26O3 | ND | 250.5 | ND |
| 31 | 1-Bromo-11-iodoundecane | C11H22BrI | ND | 174 | ND |
| 32 | 17.alpha.(H),21.beta.(H)-Hopane | C30H52 | ND | 238 | ND |

U.S = Up Stream, M.S = Mid Stream, D.S = Down Stream, ND = Not detected

At Owerrinta Axis (Table 1b), Hexadecane, 2,6,10,14-tetramethyl- dominated with 8166 mg/kg . Others included cis-Vaccenic acid (3042 mg/kg), n-Propyl 11-octadecenoate (1679 mg/kg). Traces of esters and fatty acids such as Octadecanoic acid (623 mg/kg) were found upstream. Emerging contaminants like Pyrrolidine-1,2-dicarboxylic acid derivatives were detected in moderate levels (234 mg/kg). From the analysis, most of  the detected CECs were very specific to just one sampling point each while a few were detected across two sampling points.

Table 1c: CEC Concentration in the Sediment Samples around Oyigbo Axis of Imo River

|  | Name Of Compound | Formulae | Sediment Conc.(mg/kg) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | U.S | M.S | D.S |
| 1 | Pentadecane, 2,6,10,14-tetramethyl | C19H40 | 3518 | ND | 1024 |
| 2 | Hexadecane, 2,6,10,14-tetramethyl- | C20H42 | ND | ND | 924 |
| 3 | Dodecane, 2,6,10-trimethyl- | C15H32 | 3945 | ND | ND |
| 4 | 2-Methyl-7-phenylindole | C15H13N | ND | 9635 | ND |
| 5 | Nonadecane | C19H40 | ND | ND | 924 |
| 6 | Eicosane | C20H42 | ND | ND | 956 |
| 7 | Heneicosane | C21H44 | ND | ND | 936 |
| 8 | Docosane | C22H46 | ND | ND | 802 |
| 9 | Tricosane | C23H48 | ND | ND | 772 |
| 10 | Tetracosane | C24H50 | ND | ND | 625 |
| 11 | Tetradecane, 2,6,10-trimethyl- | C17H36 | ND | ND | 252 |
| 12 | Hexadecane | C16H34 | ND | ND | 279 |
| 13 | Pentadecane, 2-methyl- | C16H34 | ND | ND | 404 |
| 14 | Heptacosane | C27H56 | ND | ND | 460 |
| 15 | Octacosane | C28H58 | ND | ND | 390 |
| 16 | Methylene chloride | CH2Cl2 | 39 | ND | ND |
| 17 | 3,4-Dichlorobutane nitrile | C4H5Cl2N | 3 | ND | ND |
| 18 | 2(1H)-Naphthalenone, octahydro-1,4 a-dimethyl- , (1.alpha.,4a.beta.,8a.alpha.)- | C12H20O | 42 | ND | ND |
| 19 | Pentadecane, 3-methyl- | C16H34 | 624 | ND | ND |
| 20 | Sebacic acid, di(2-propylpentyl) ester | C26H50O4 | 1136 | ND | ND |
| 21 | 9-Oxabicyclo[3.3.1]nonan-2-ol, acetate | C10H16O3 | 83 | ND | ND |
| 22 | 28-Nor-17.alpha.(H)-hopane | C29H50 | 95 | ND | ND |
| 23 | 2-(Acetoxymethyl)-3-(methoxycarbonyl)biphenylene | C17H14O4 | 51 | 14 | ND |
| 24 | Benzo[h]quinoline, 2,4-dimethyl- | C15H13N | 342 | ND | ND |
| 25 | Silicic acid, diethyl bis(trimethylsilyl) ester | C10H28O4Si3 | 60 | ND | ND |
| 26 | 5-(p-Aminophenyl)-4-(p-tolyl)-2-thiazolamine | C16H15N3S | 41 | ND | ND |
| 27 | Methyl 3-bromo-1-adamantaneacetate | C13H19BrO2 | 34 | ND | ND |
| 28 | Ethanol, 2,2,2-trichloro- | C2H3Cl3O | ND | 83 | ND |
| 29 | 3-(4-N,N-Dimethylaminophenyl)propenoic acid, 2- (diethoxyphosphinyl)-,ethyl ester | C17H26NO5P | ND | 17 | ND |
| 30 | Cyclononasiloxane, octadecamethyl- | C18H54O9Si9 | ND | 4 | ND |
| 31 | Heptasiloxane, 1,1,3,3,5,5,7,7,9,9,11,11,13,13- tetradecamethyl | C14H44O6Si7 | ND | 4 | ND |
| 32 | 1,1,1,5,7,7,7-Heptamethyl-3,3- bis(trimethylsiloxy)tetrasiloxane | C13H40O5Si6 | ND | 35 | ND |
| 33 | Octasiloxane, 1,1,3,3,5,5,7,7,9,9, 11,11,13,13,15,15-hexadecamethyl | C16H50O7Si8 | ND | 107 | ND |
| 34 | Pyrido[1,2-a][1,3]benzimidazole-3- acetic acid, 4- cyano-1,5-dihydro- -oxo-, methyl ester | C15H11N3O3 | ND | 28 | ND |

U.S = Up Stream, M.S = Mid Stream, D.S = Down Stream, ND = Not detected

At Oyigbo Axis (Table 1c), Dodecane, 2,6,10-trimethyl- had a high concentration of 3945 mg/kg, 2-Methyl-7-phenylindole recorded the highest value of 9635 mg/kg at M.S. various hydrocarbons were  present, ranging from 252 – 956mg/kg. Some esters and derivatives of carboxylic acids were also common. Siloxanes such as Heptasiloxane and Octasiloxane) ranged from 4 to 107 mg/kg. From the result, most of the detected CECs were very specific to just one sampling point each while a few were detected across two sampling points.

Research Hypothesis

From Table 2, the study’s hypothesis was tested using the ANOVA. The hypothesis was tested based on the following statement:

H0: 	There is no significant difference in the CECs concentrations in the sediment samples along the Imo River

H1: 	There is a significant difference in the CECs concentrations in the sediment samples along the Imo River

The p-value was adopted to determine the significance levels (where p ≤ 0.05 rejects the null hypothesis). Based on the outcome, the null hypothesis (Ho) which stated that there is no significant difference in the CECs concentrations in the surface water of samples along  the Imo River was accepted (where p > 0.05, p = 0.909).

Table 2 Significant Different Analysis of CECs in Sediments

|  |  | Sum of Squares | Df | Mean Square | F | Sig. |
| --- | --- | --- | --- | --- | --- | --- |
| Sediment | Between Groups | 159848.762 | 2 | 79924.381 | 0.095 | 0.909 |
|  | Within Groups | 218975114.678 | 260 | 842211.980 |  |  |
|  | Total | 219134963.440 | 262 |  |  |  |

DISCUSSION

The non targeted assessment of sediment samples from the Imo River, Nigeria using the GCMS established the presence of  eighty – four (84) unique CECs. The outcome  corroborates with the submission of Afolabi (2024) which indicated that sediment is the sinker of various pollutants in aquatic environment.. High concentrations of aliphatic amines, such as Nonadecylamine (322 mg/kg), indicate industrial effluent contributions, as similar findings have been reported in contaminated sediments of the Pearl River Delta, China (Liu et al., 2018). Siloxanes, including Cycloheptasiloxane and Heptamethyltrisiloxane, recorded significant levels, particularly at Oyigbo, confirming their sediment-binding potential as demonstrated in European river studies (Loos et al., 2009). The detection of siloxanes in significant amounts reflect the increasing use of personal care products, lubricants, and industrial materials, consistent with global reports of siloxane pollution in aquatic environments (Mojsiewicz-Pieńkowska et al., 2016) Fatty acids, notably n-Hexadecanoic acid (up to 2764 mg/kg in Owerrinta sediment), and their esters such as cis-Vaccenic acid, detected at significant concentrations reinforce findings from aquatic environments impacted by agricultural and municipal waste (Muir et al., 2017, Rudel et al., 2011).

In summary, the non-target analysis (NTA) of surface water from the Imo River at Ekenobizi, Owerrinta and Oyigbo using Gas Chromatography/Mass Spectrometry (GCMS) was able to establish the presence of various contaminant compounds of emerging concerns. A total of 84 unique CECs were detected. The detected compounds were of diverse parent/functional groups with diverse chemical characteristics.

Finally, the detection and concentration patterns of Contaminants of Emerging Concern (CECs) in the Imo River sediments indicates significant contamination of the Imo River system with a broad spectrum of emerging organic pollutants, including siloxanes, fatty acids, amines, fluorinated compounds and others. The study reveals critical trends of environmental contamination and potential ecological risks, consistent with global studies.

RECOMMENDATIONS

- Targeted analysis of detected CECs for setting permissible health limits and standards for aquatic environments.

- Implementation of stricter monitoring and control of industrial effluents, especially from manufacturing, petroleum, and personal care product industries, which are known sources of siloxanes, amines, and fatty acids.

- Promotion of sustainable farming practices to reduce the discharge of fatty acids and nutrient-rich compounds into the river, as supported by global mitigation strategies.

- Replacement of high-risk compounds, particularly persistent siloxanes and fluorinated esters, with biodegradable alternatives, aligning with global green chemistry initiatives.

REFERENCES

Afolabi, O. O., Okwori, P. B.,  Yemi-Jonathan, O. I. T., Wali, E.,  Ugwu, M. C., Ugwa,  C. D.,

Emelu, V. O., Ugwuechendu, T. T., Bosco-Abiahu, L. C. & Asomaku, S. O. (2024) Ecotoxicological status, source apportionment and human health risk assessment of potential toxic elements in surface water and sediments of creeks along Bonny River, Nigeria,Watershed Ecology and the Environment,Volume 6, Pages 105-113, ISSN 2589-4714, https://doi.org/10.1016/j.wsee.2024.06.001.

(https://www.sciencedirect.com/science/article/pii/S258947142400007X)

Blomqvist, S. (1991). Quantitative sampling of soft-bottom sediments: problems and	solutions. Marine Ecology Progress Series, 72, 295-304.

Jianglu Zhao, Changsheng Guo, Queping Yang, Weiling Liu, Heng Zhang, Ying Luo, Yan

Zhang, Lei Wang, Chao Chen, Jian Xu,(2024).Comprehensive monitoring and prioritizing for contaminants of emerging concern in the Upper Yangtze River, China: An integrated approach, Journal of Hazardous Materials, Volume 480, 135835, ISSN 0304-3894, https://doi.org/10.1016/j.jhazmat.2024.135835.

(https://www.sciencedirect.com/science/article/pii/S0304389424024142)

Ebele, A. J, Abou-Elwafa Abdallah, M., & Harrad, S. (2017). Pharmaceuticals and personal care	products (PPCPs) in the freshwater aquatic environment, Emerging Contaminants, Volume

3,	Issue 1, Pages 1-16, ISSN 2405-6650, https://doi.org/10.1016/j.emcon.2016.12.004. (https://www.sciencedirect.com/science/article/pii/S2405665016300488)

Enyoh, C. E., & Isiuku, B. O. (2020). Determination and human health risk assessment of heavy metals in floodbasin soils in Owerri, Southeastern Nigeria. Chemistry Africa, 3(4), 1059-1071.

Fairbairn D.J., Karpuzcu M.E., Arnold W.A., Barber B.L., Kaufenberg EF, Koskinen WC,

Novak PJ, Rice PJ, Swackhamer DL. Sediment-water distribution of contaminants of emerging concern in a mixed use watershed. Sci Total Environ. 2015 Feb 1;505:896-904. doi: 10.1016/j.scitotenv.2014.10.046. Epub 2014 Nov 7. PMID: 25461092.

Fang W., Leilei, X., Kelvin, S. L., Martin, E., Ying, Z., Yuming, G., Bo, P., Hongwen, S.,

Taicheng, A., Guangguo, Y., Bryan, W. B., Deyi, H., Damian E.,	Helbling, J. S., Hao Qiu, Timothy, M. V.l, Wei, Z., Yanzheng, G., Myrna, J.	S., Yi, L., Scott, X. C., & James, M. T. (2024). Emerging contaminants: A One	Health perspective, The Innovation, Volume 5, Issue 4, 100612, ISSN 2666-6758, https://doi.org/10.1016/j.xinn.2024.100612.(https://www.sciencedirect.com/science/article/pii/S266667582400050X)

Jobling S., Williams R., Johnson A., Taylor A., Gross-Sorokin M., Nolan M., Tyler C.R., van

Aerle R., Santos E., Brighty G. (2006 ). Predicted exposures to steroid estrogens in U.K. rivers correlate with widespread sexual disruption in wild fish populations. Environ Health Perspect. Apr;114 Suppl 1(Suppl 1):32-9. doi: 10.1289/ehp.8050. PMID: 16818244; PMCID: PMC1874167.

Liu, J., et al. (2018). Distribution and sources of contaminants of emerging concern in the Pearl River Delta, China. Science of the Total Environment, 619-620, 517–525.

Loos, R., et al. (2009). Occurrence of polar organic contaminants in the aquatic environment. TrAC Trends in Analytical Chemistry, 28(10), 1043-1056.

Muir, D., Wang, X., & Bright, D. (2017). Long-chain perfluorinated compounds and their accumulation in aquatic food webs. Environmental Toxicology and Chemistry, 36(1), 20-31.

Mojsiewicz-Pieńkowska, K., et al. (2016). Siloxanes in the environment—fate and toxicity. Environmental Science and Pollution Research, 23(11), 10919–10934.

Ogbonna, D. N., O. O. Ndukwu, T. K. S. Abam, and S. A. Ngah. (2021). “Ecological Risk	Assessments of Heavy Metal Contamination in Water, Sediment and Biota of Otamiri	and Imo Rivers, Nigeria”. Journal of Scientific Research and Reports 27 (11):104-19.	https://doi.org/10.9734/jsrr/2021/v27i1130462.

Pal, A., He, Y., Jekel, M., Reinhard, M., & Gin, K. Y. H. (2014). Emerging contaminants of public health significance as water quality indicator compounds in the urban water cycle. Environment International, 71, 46-62.

Rudel, R. A., et al. (2011). Bioaccumulation of synthetic chemicals: insights for human exposure and health. Environmental Health Perspectives, 119(9), 1207–1213.

Sauvé, S., Desrosiers, M. (2014) A review of what is an emerging contaminant. Chemistry Central Journal 8, 15. https://doi.org/10.1186/1752-153X-8-15

Yuange Yu, Zhu Wang, Bin Yao, Yaoyu Zhou,( 2024). Occurrence, bioaccumulation, fate,

and risk assessment of emerging pollutants in aquatic environments: A review, Science of The Total Environment, Volume 923, 171388, ISSN 0048-9697, https://doi.org/10.1016/j.scitotenv.2024.171388. (https://www.sciencedirect.com/science/article/pii/S0048969724015298