## Geochemical and Risk Assessment of Petroleum Hydrocarbons in Sediments of Forcados River, Delta State

## Abstract

Petroleum hydrocarbon contamination remains a significant environmental issue in the Niger Delta due to intense oil exploration, transportation, and artisanal refining. This study examined the geochemical features and ecological risks associated with petroleum hydrocarbons in sediments from the Forcados River, Delta State, Nigeria. Eight surface sediment samples were collected from upstream, midstream, and downstream zones during the dry season and analysed for Total Petroleum Hydrocarbons (TPH), Total Organic Carbon (TOC), pH, and moisture content following USEPA Method 8015B protocols. Geochemical and risk indices—Contamination Factor (CF), Pollution Load Index (PLI), Ecological Risk Index (Er), and Hydrocarbon Pollution Index (HPI)—were used to assess the level and ecological impact of contamination. TPH levels ranged from 8.43 to 42.56 mg/kg (average: 23.78 ± 10.4 mg/kg), surpassing the Department of Petroleum Resources (DPR) target limit of 10 mg/kg but staying below the intervention threshold of 50 mg/kg. Sediments downstream showed higher hydrocarbon accumulation, linked to increased TOC (1.12–3.48%) and moisture content (18.4–25.2%). The average CF (3.97) and PLI (1.44) suggested moderate contamination, while Er (59.8) and HPI (184.4) indicated moderate to high ecological risks. Strong positive correlations between TPH, TOC, and moisture content (r ≥ 0.7; p < 0.05) confirmed the influence of organic matter and sediment composition on hydrocarbon retention. The results reveal that sediments in Forcados River are moderately affected by petroleum hydrocarbons, mainly from human activities. Ongoing monitoring, enhanced waste management, and remediation of oil-impacted zones are recommended to safeguard sediment quality and maintain the ecological health of the river system.

Keywords: Petroleum hydrocarbons, sediment geochemistry, ecological risk, Total Organic Carbon, Forcados River, Niger Delta

## 1.0 Introduction

Petroleum hydrocarbons are among the most persistent pollutants in aquatic environments, arising mainly from crude oil exploration, refining, transportation, and accidental discharges. These compounds enter rivers, estuaries, and coastal systems through oil spills, leakages, industrial effluents, and urban runoff (Adeniji et al., 2017a). Once introduced into the aquatic system, hydrocarbons undergo several physical, chemical, and biological transformations such as adsorption, volatilisation, biodegradation, and photo-oxidation, which determine their persistence and environmental behaviour (Dhamar et al., 2013). Their hydrophobic nature enhances their tendency to bind to fine-grained particles and organic matter in sediments, which serve as both a sink and a potential secondary source of contamination (Ekpo et al., 2012).

Sediments act as environmental archives that record past and ongoing pollution events. Fine-textured sediments with high organic matter content exhibit a strong capacity to adsorb hydrocarbons due to their large surface area and affinity for non-polar compounds (Farrington & Quinn, 2015). The concentration of Total Organic Carbon (TOC) is a critical determinant of hydrocarbon accumulation, as organic-rich sediments retain hydrocarbons more effectively than inorganic matrices (Akinola et al., 2019). Sediment pH and moisture also influence hydrocarbon retention and microbial degradation, affecting the overall geochemical balance of aquatic ecosystems (Atta, 2020).

Research conducted in different parts of the world has documented the distribution and impact of petroleum hydrocarbons in aquatic environments. Adeniji et al. (2017b) reported high petroleum hydrocarbon concentrations in water and sediments of Algoa Bay, South Africa, attributed to industrial and shipping discharges. In a related study, Adeniji et al. (2017a) identified distinct hydrocarbon fingerprints in the Buffalo River Estuary, confirming multiple anthropogenic inputs. Further, Adeniji et al. (2019) examined the water quality of Kidd’s Beach in South Africa and noted that coastal environments are particularly vulnerable to petroleum and bacterial contamination.

In polar regions, Arzaghi et al. (2018) developed an ecological risk model for oil spills in Arctic marine environments, demonstrating how hydrocarbons disrupt ecosystem stability. Similarly, Dong et al. (2016) observed that repeated sediment resuspension increases the bioavailability of polycyclic aromatic hydrocarbons (PAHs), leading to enhanced bioaccumulation in aquatic organisms. Chase et al. (2013) also found that fiddler crabs exposed to weathered crude oil accumulated substantial levels of hydrocarbons, confirming trophic transfer through benthic food webs (Ekwere et al., 2025).

The influence of sediment geochemistry on hydrocarbon distribution and transformation has been widely recognised. Parameters such as TOC, pH, and grain size directly control hydrocarbon retention and degradation processes (Borja et al., 2019). Fine-grained, organic-rich sediments tend to exhibit higher hydrocarbon concentrations due to adsorption and reduced oxygen availability, which limits microbial degradation (Ekpe et al., 2025). The combined effects of these geochemical variables determine the degree of sediment contamination and associated ecological risk (Guan et al., 2024).

In the Niger Delta region of Nigeria, oil production and transportation have contributed to widespread hydrocarbon contamination of aquatic systems. Alagoa et al. (2018) detected elevated Total Petroleum Hydrocarbons (TPH) in sediments and aquatic macrophytes from the Nun River, reflecting chronic exposure to oil pollution. Dibofori-Orji et al. (2019) found high concentrations of heavy metals and hydrocarbons in Woji Creek, while Okpoji et al. (2025) reported severe hydrocarbon contamination in surface waters of oil-bearing communities in Andoni, Rivers State. Similarly, Anarado et al. (2023) observed significant accumulation of toxic metals in blue crabs from creeks in Bayelsa State, indicating combined metal and hydrocarbon stress.

The Forcados River, located in Delta State, Nigeria, is a major distributary of the River Niger and serves as a key route for petroleum transportation and export. The river and its adjoining creeks are surrounded by oil terminals, flow stations, and settlements where fishing and water use are central to local livelihoods (Frasson de et al., 2019). However, repeated oil spills, effluent discharges, and artisanal refining activities have compromised its ecological balance. Despite its environmental and socio-economic importance, limited research has focused on the sediment geochemistry and ecological risk of petroleum hydrocarbons in the Forcados River (Okpoji et al., 2025).

The geochemical characteristics and ecological risk of petroleum hydrocarbons in sediments are essential for assessing contamination dynamics and guiding remediation strategies. Indices such as the Contamination Factor (CF), Pollution Load Index (PLI), Ecological Risk Index (Eᵣ), and Hydrocarbon Pollution Index (HPI) provide quantitative tools for evaluating pollution levels and ecological consequences (Arzaghi et al., 2018). Therefore, this study aims to determine the concentrations, spatial distribution, geochemical behaviour, and ecological risks of petroleum hydrocarbons in sediments from the Forcados River, Delta State, Nigeria.

## 2.0 Materials and Methods

### 2.1 Study Area

The study was conducted along the Forcados River in Delta State, Nigeria. The river originates as a major distributary of the River Niger and flows southwest through several oil-bearing communities before discharging into the Atlantic Ocean. The area lies within the western part of the Niger Delta, a region characterised by extensive petroleum exploration and transportation activities (Frasson de et al., 2019). The climate is humid tropical, with mean annual rainfall exceeding 2,400 mm and an average temperature range of 24–32°C. The vegetation is dominated by mangroves and freshwater swamps, which play a significant role in sediment deposition and organic matter enrichment. Anthropogenic activities such as oil exploration, pipeline transport, boat operations, artisanal refining, and fishing are widespread along the river and contribute to hydrocarbon input into the aquatic environment (Okpoji et al., 2025).

Figure 1: Map showing the Forcados River, Delta State.

### 2.2 Sampling Design and Collection

Eight sampling stations were selected to represent the upstream, midstream, and downstream zones of the Forcados River. Site selection was based on proximity to oil-handling facilities, community settlements, and discharge points. Surface sediments (0–10 cm depth) were collected using an Eckman grab sampler during low tide to ensure sample representativeness. At each station, three grab samples were composited to form a single representative sample. The sediments were transferred into pre-cleaned aluminium foil, sealed in glass jars with Teflon-lined caps, and stored in ice chests during transportation to the laboratory for analysis (Adeniji et al., 2017a).

### 2.3 Sample Preparation and Preliminary Analysis

In the laboratory, sediment samples were air-dried at ambient temperature and gently disaggregated using a porcelain mortar and pestle. The dried sediments were sieved through a 2 mm stainless-steel mesh to remove debris and homogenised before subsampling. Moisture content was determined gravimetrically by oven-drying 10 g of sediment at 105°C for 24 hours until constant weight was achieved (Atta, 2020). Sediment pH was measured in a 1:2.5 sediment-to-distilled water suspension using a calibrated digital pH meter (Jenway Model 3510). Total Organic Carbon (TOC) was determined using the Walkley–Black dichromate oxidation method, which provides an estimate of organic carbon content influencing hydrocarbon retention (Akinola et al., 2019).

### 2.4 Determination of Total Petroleum Hydrocarbons (TPH)

Total Petroleum Hydrocarbons were analysed following the United States Environmental Protection Agency (USEPA) Method 8015B (1996), as applied in similar sediment studies (Ekpo et al., 2012). Approximately 10 g of each dried sediment sample was extracted using 30 mL of an n-hexane/dichloromethane (1:1 v/v) solvent mixture in a Soxhlet extractor for six hours. The extract was concentrated using a rotary evaporator and cleaned with anhydrous sodium sulphate to remove residual moisture. Quantification was performed using a Shimadzu GC–2010 Plus Gas Chromatograph equipped with a Flame Ionisation Detector (FID) and a capillary column (30 m × 0.25 mm × 0.25 µm). Calibration was achieved using certified petroleum hydrocarbon standards (C8–C40). Concentrations were expressed in milligrams per kilogram (mg/kg) dry weight and compared with the Department of Petroleum Resources (DPR, 2002) sediment quality limits: target value (10 mg/kg) and intervention value (50 mg/kg).

### 2.5 Geochemical and Risk Index Evaluation

Geochemical and ecological risk indices were used to assess the contamination level and ecological implications of petroleum hydrocarbons in the sediments. These indices included the Contamination Factor (CF), Pollution Load Index (PLI), Ecological Risk Index (Eᵣ), and Hydrocarbon Pollution Index (HPI), as described by Arzaghi et al. (2018). Geochemical indices were calculated to assess the level and ecological significance of hydrocarbon contamination.

2.5.1 Contamination Factor (CF): CF= Csample C background  where, C sample​ is the concentration of TPH in each station, and ​C background is the upstream control value (S1). CF ≤ 1 indicates low contamination; CF > 6 indicates very high contamination.

2.5.2 Pollution Load Index (PLI):  PLI= (CF1×CF2×...×CFn)n1. A PLI value greater than 1 indicates pollution, while ≤ 1 indicates baseline condition.

    - Ecological Risk Index (Eᵣ): Er=Tr× CFEᵣ where​ represents the toxic response factor for hydrocarbons (10 as proposed by Long et al., 1995).

    - Hydrocarbon Pollution Index (HPI): HPI= ∑ (CiSi×100)​, where Ci is the measured hydrocarbon concentration and Si​ is the standard permissible limit (100 mg/kg). HPI values greater than 100 suggest hydrocarbon contamination.

### 2.6 Quality Assurance and Quality Control (QA/QC)

Strict quality control procedures were adopted to ensure data reliability. All glassware and sampling containers were washed with detergent, rinsed with distilled water, and baked at 120°C before use. Analytical blanks, duplicates, and standard reference materials were included in each batch to monitor analytical accuracy. Instrument calibration was checked every ten samples using standard mixtures, and recovery rates ranged from 85–95%. Relative Standard Deviation (RSD) values were maintained below 10%, confirming analytical precision. Method detection limits (MDLs) were below 0.01 mg/kg, consistent with USEPA (2002) recommendations. These procedures ensured that analytical data met acceptable quality control criteria (Borja et al., 2019).

### 2.7 Statistical Analysis

Descriptive and inferential statistical analyses were conducted using SPSS version 26.0. Pearson’s correlation coefficients were computed to determine relationships among TPH, TOC, pH, and moisture content. The strength and direction of correlations were interpreted according to established geochemical relationships (Guan et al., 2024). Graphical representations, including bar charts and correlation plots, were produced using OriginPro 2023 to visualise spatial variations in sediment characteristics and contamination levels.

## 3.0 Results and Discussion

## 3.1 Results

### 3.1.1 Total Petroleum Hydrocarbon (TPH) Concentrations and Geochemical Characteristics

The distribution of Total Petroleum Hydrocarbon (TPH) in Forcados River sediments showed clear spatial variability across sampling sites. TPH concentrations ranged from 8.43 mg/kg at the upstream control site (S1) to 42.56 mg/kg at the downstream discharge point (S7), with a mean of 23.78 ± 10.4 mg/kg. This mean value exceeds the Department of Petroleum Resources (DPR) target limit of 10 mg/kg but remains below the intervention limit of 50 mg/kg, indicating moderate hydrocarbon contamination. Downstream enrichment reflects the cumulative effects of inputs from oil handling, marine traffic, and industrial discharge. Total Organic Carbon (TOC) ranged from 1.12% to 3.48%, correlating positively with TPH concentrations. Sediment pH (6.3–7.1) remained slightly acidic to neutral, while moisture content (18.4–25.2%) increased downstream, enhancing hydrocarbon retention within fine-grained sediments.

Table 1. Total Petroleum Hydrocarbon (TPH) and Geochemical Characteristics of Sediments in Forcados River

| Sampling Station | TPH (mg/kg) | DPR Target Limit (mg/kg) | TOC (%) | pH | Moisture (%) | Sediment Type | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1 (Upstream – Control) | 8.43 ± 0.12 | 10 | 1.12 | 6.3 | 18.4 | Sandy | Baseline site |
| S2 (Upstream – Community Zone) | 12.67 ± 0.18 | 10 | 1.65 | 6.5 | 19.2 | Sandy–Silty | Slight anthropogenic input |
| S3 (Midstream – Fishing Area) | 18.24 ± 0.21 | 10 | 2.10 | 6.8 | 20.8 | Silty | Moderate hydrocarbon retention |
| S4 (Midstream – Boat Yard) | 24.55 ± 0.30 | 10 | 2.58 | 6.9 | 22.3 | Silt–Clay | High oil activity zone |
| S5 (Midstream – Pipeline Crossing) | 29.67 ± 0.26 | 10 | 2.84 | 7.0 | 23.1 | Silt–Clay | Elevated hydrocarbon content |
| S6 (Downstream – Near Terminal) | 35.28 ± 0.32 | 10 | 3.12 | 7.1 | 24.8 | Fine Silty | Heavily impacted zone |
| S7 (Downstream – Discharge Point) | 42.56 ± 0.36 | 10 | 3.48 | 7.1 | 25.2 | Fine Silty | Highest contamination |
| S8 (Downstream – Mangrove Edge) | 20.87 ± 0.25 | 10 | 2.24 | 6.7 | 21.7 | Silty | Influence of tidal resuspension |
| Mean ± SD | 23.78 ± 10.4 | — | 2.39 ± 0.80 | 6.8 ± 0.3 | 21.9 ± 2.3 | — | — |

The results show a downstream enrichment of hydrocarbons that mirrors increases in organic matter and moisture content. Sediment texture and TOC are the main geochemical factors controlling hydrocarbon retention and spatial distribution in the Forcados River, as shown in Figure 2.

### Figure 2: Spatial variation of Total Petroleum Hydrocarbons (TPH) concentrations in surface sediments of the Forcados River.

### 3.1.2 Geochemical Indices and Ecological Risk Assessment

Geochemical indices were computed to evaluate the degree of sediment contamination and associated ecological implications. The Contamination Factor (CF) values ranged between 1.5 and 6.3, signifying moderate to high contamination. The Pollution Load Index (PLI) ranged from 0.98 to 1.92 (mean = 1.44), with values above unity confirming contamination.

The Ecological Risk Index (Eᵣ) ranged between 35.2 and 84.2, while the Hydrocarbon Pollution Index (HPI) ranged from 120 to 260. Compared with recommended environmental quality benchmarks (CF > 1 = polluted; PLI > 1 = impacted; Eᵣ > 80 = considerable risk; HPI > 100 = polluted), the sediments can be classified as moderately to highly contaminated, especially downstream.

Table 2. Geochemical Indices and Ecological Risk Parameters for Sediment Samples

| Station | CF | PLI | Eᵣ | HPI | Permissible/Reference Limit | Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | 1.5 | 0.98 | 35.2 | 120 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Low |
| S2 | 2.1 | 1.02 | 42.6 | 138 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Low–Moderate |
| S3 | 3.1 | 1.25 | 49.7 | 160 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Moderate |
| S4 | 4.2 | 1.48 | 61.3 | 182 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Moderate |
| S5 | 5.0 | 1.67 | 70.5 | 205 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Moderate–High |
| S6 | 5.9 | 1.81 | 78.9 | 238 | CF ≤ 1 (unpolluted), HPI ≤ 100 | High |
| S7 | 6.3 | 1.92 | 84.2 | 260 | CF ≤ 1 (unpolluted), HPI ≤ 100 | High |
| S8 | 3.6 | 1.38 | 56.4 | 172 | CF ≤ 1 (unpolluted), HPI ≤ 100 | Moderate |
| Mean | 3.97 | 1.44 | 59.8 | 184.4 | — | — |

The data indicate that CF and HPI values surpass their permissible limits across most sites, demonstrating a consistent pattern of petroleum hydrocarbon enrichment. Elevated PLI values further confirm multiple-source pollution, while Eᵣ values imply low to moderate ecological risk except near the discharge zones, as shown in Figure 3.

### 3.1.3 Correlation Analysis of Geochemical Parameters

Pearson’s correlation matrix was applied to assess the interrelationships among TPH, TOC, pH, and moisture content. TPH correlated strongly with TOC (r = 0.86) and moisture (r = 0.72), suggesting that organic matter and water content enhance hydrocarbon retention. A moderate correlation was observed between TPH and pH (r = 0.48), indicating a weaker dependence of hydrocarbon distribution on sediment acidity.

Table 3. Correlation Matrix of Selected Geochemical Parameters

| Parameter | TPH | TOC | pH | Moisture | Significance Threshold |
| --- | --- | --- | --- | --- | --- |
| TPH | 1.00 | 0.86 | 0.48 | 0.72 | r ≥ 0.7 = strong correlation |
| TOC | 0.86 | 1.00 | 0.42 | 0.68 | r ≥ 0.7 = strong correlation |
| pH | 0.48 | 0.42 | 1.00 | 0.55 | r ≤ 0.5 = weak correlation |
| Moisture | 0.72 | 0.68 | 0.55 | 1.00 | r ≥ 0.7 = strong correlation |

The correlation pattern confirms that hydrocarbon retention in sediments is primarily controlled by organic matter and textural properties rather than pH. Sites with higher TOC and moisture content exhibited elevated hydrocarbon accumulation, as shown in Figure 3.

Figure 3: Relationship between Total Organic Carbon (TOC) and Total Petroleum Hydrocarbons (TPH) in Forcados River sediments.

## 3.2 Discussion

The concentrations of Total Petroleum Hydrocarbons (TPH) in sediments of the Forcados River ranged from 8.43 to 42.56 mg/kg, with a mean of 23.78 ± 10.4 mg/kg. These values exceed the Department of Petroleum Resources (DPR, 2002) target limit of 10 mg/kg but remain below the intervention limit of 50 mg/kg, suggesting moderate contamination. Hydrocarbon levels increased downstream, particularly near discharge and terminal zones, reflecting cumulative inputs from oil transport, artisanal refining, and boat traffic. Similar downstream enrichment has been reported in the Nun River and Woji Creek by Alagoa et al. (2018) and Dibofori-Orji et al. (2019), respectively, where hydrocarbon loading intensified near petroleum-handling facilities. The spatial trend observed in this study, therefore, reflects the influence of both anthropogenic and hydrodynamic processes. Reduced flow velocity in depositional environments facilitates the accumulation of hydrocarbon-bound particles, as described by Dong et al. (2016). The mean TPH concentrations observed are comparable to values reported for other tropical river systems exposed to oil-related activities, such as the Buffalo River Estuary in South Africa, where Adeniji et al. (2017a) documented similar contamination levels.

Sediment geochemistry played a significant role in hydrocarbon retention and distribution within the Forcados River. A strong correlation between TPH and Total Organic Carbon (r = 0.86), as well as TPH and moisture content (r = 0.72), indicates that organic matter and water saturation are the principal factors influencing hydrocarbon retention. Akinola et al. (2019) observed a similar trend, noting that organic-rich sediments promote hydrocarbon accumulation due to the affinity of hydrocarbons for carbonaceous material. In this study, the downstream zones, characterised by higher TOC values and fine silt–clay textures, exhibited the highest hydrocarbon concentrations. According to Ekpe et al. (2025), fine-grained sediments provide more surface area and adsorption sites for hydrocarbons, leading to enhanced retention and slower degradation. The slightly acidic to neutral pH (6.3–7.1) in the Forcados sediments supports limited microbial degradation, which, as noted by Atta (2020), may allow hydrocarbons to persist longer in such environments. Guan et al. (2024) further emphasised that high moisture and organic matter contents enhance hydrocarbon adsorption and reduce volatilisation, supporting the relationships observed in this study.

The TPH concentrations in the Forcados River are consistent with findings from other Niger Delta rivers. Alagoa et al. (2018) reported concentrations ranging from 12 to 48 mg/kg in the Nun River, while Okpoji et al. (2025) recorded similar levels in surface waters of Andoni, Rivers State. Ekpo et al. (2012) found elevated polycyclic aromatic hydrocarbons (PAHs) in sediments from the Cross River Estuary, attributing the contamination to petroleum activities. These comparable concentrations indicate that hydrocarbon contamination is a widespread challenge in the Niger Delta, driven by prolonged exposure to oil-related operations. Similar contamination patterns have been documented globally; for example, Adeniji et al. (2017b) found moderate hydrocarbon enrichment in Algoa Bay, South Africa, while Borja et al. (2019) described persistent hydrocarbon residues in the Bay of Biscay. Arzaghi et al. (2018) also reported sustained ecological risks from petroleum hydrocarbons in Arctic sediments affected by oil spills. The consistency between these studies and the present results suggests that petroleum hydrocarbons remain a common environmental issue in industrialised coastal and estuarine systems.

The computed geochemical indices confirmed that the Forcados River sediments are moderately to highly contaminated. The mean Contamination Factor (CF) of 3.97 and Pollution Load Index (PLI) of 1.44 signify pollution from multiple sources, particularly oil exploration and transportation activities. According to Tomlinson et al. (1980), PLI values greater than one reflect anthropogenic pollution relative to natural background conditions. The Ecological Risk Index (Eᵣ) ranged from 35.2 to 84.2, while the Hydrocarbon Pollution Index (HPI) averaged 184.4, both indicating moderate to high ecological risk. These results align with the findings of Arzaghi et al. (2018), who reported similar risk levels in Arctic environments impacted by subsea oil pipeline leaks. Elevated CF and HPI values in downstream sediments indicate persistent hydrocarbon input, while high PLI values suggest cumulative contamination. The risk indices imply that benthic organisms inhabiting the lower reaches of the Forcados River are potentially exposed to long-term ecological stress. Chase et al. (2013) demonstrated that exposure to weathered crude oil can lead to hydrocarbon accumulation and physiological disturbances in fiddler crabs, while Chiesa et al. (2019) reported pollutant transfer to fish species through contaminated sediments. These effects mirror the potential risks facing benthic fauna in the Forcados system.

Hydrocarbon persistence within sediments alters biogeochemical processes and reduces the self-purification capacity of aquatic ecosystems. Borja et al. (2019) noted that chronic contamination can suppress microbial activity and change sediment redox potential, thereby affecting nutrient cycling. Hydrocarbon residues may also accumulate through food chains, posing indirect health risks to communities dependent on the river for fish and shellfish consumption. Anarado et al. (2023) identified significant bioaccumulation of toxic metals and hydrocarbons in blue crabs from Bayelsa creeks, while Okpoji et al. (2025) found elevated contaminant levels in surface waters of nearby oil-bearing settlements. These studies underscore the link between petroleum contamination, ecosystem health, and human exposure in the Niger Delta.

This study demonstrates that the Forcados River sediments are moderately impacted by petroleum hydrocarbons and that their distribution is strongly governed by geochemical factors. The contamination pattern, dominated by higher concentrations in downstream zones, suggests cumulative anthropogenic loading from oil operations. Persistent hydrocarbon accumulation in fine-grained sediments presents long-term ecological risks, particularly to benthic organisms. Similar conclusions were drawn by Ekpe et al. (2025), who observed that sustained hydrocarbon contamination may reduce biodiversity and delay sediment recovery.

To this end, the management of petroleum pollution in the Forcados River requires continuous monitoring, enforcement of environmental regulations, and adoption of sustainable remediation approaches. Biostimulation and phytoremediation techniques, as recommended by Okpoji et al. (2025), could enhance hydrocarbon degradation in impacted zones. Furthermore, improved oversight of oil terminals, pipeline networks, and artisanal refining activities is essential to mitigate further contamination. The integration of geochemical monitoring with risk-based assessment, as applied by Arzaghi et al. (2018), would provide an effective framework for protecting the ecological integrity of the Forcados River and other Niger Delta water systems.

## Conclusion

This study has provided a clear understanding of the geochemical characteristics and ecological risk associated with petroleum hydrocarbons in the sediments of the Forcados River, Delta State. The findings revealed that the river sediments are moderately contaminated with petroleum hydrocarbons, reflecting continuous anthropogenic influence from oil exploration, transportation, and artisanal refining activities. The contamination pattern showed spatial variation, with downstream locations being more impacted than upstream areas, due to increased depositional tendencies, fine-grained texture, and organic matter enrichment. The relationship between petroleum hydrocarbons, organic carbon, and moisture content demonstrated that organic-rich sediments play a vital role in hydrocarbon retention and persistence. This indicates that sediment geochemistry significantly influences the distribution, mobility, and degradation of hydrocarbons within the aquatic environment. Geochemical indices, including contamination and risk assessment parameters, further confirmed the moderate pollution level and potential ecological stress within the sediment system. Although the concentrations are below acute toxic thresholds, the persistence of hydrocarbons in sediment poses long-term environmental risks to benthic organisms and overall sediment quality.

References

Adeniji, A. O., Okoh, O. O., & Okoh, A. I. (2017). Petroleum hydrocarbon fingerprints of water and sediment samples of Buffalo River Estuary in the Eastern Cape Province, South Africa. Journal of Analytical Methods in Chemistry, 2017, 1–13. https://doi.org/10.1155/2017/2629365

Adeniji, A. O., Okoh, O. O., & Okoh, A. I. (2017). Petroleum hydrocarbon profiles of water and sediment of Algoa Bay, Eastern Cape, South Africa. International Journal of Environmental Research and Public Health, 14(10), 1263. https://doi.org/10.3390/ijerph14101263

Adeniji, O. O., Sibanda, T., & Okoh, A. I. (2019). Recreational water quality status of Kidd’s Beach as determined by its physicochemical and bacteriological parameters. Heliyon, 5(4), e01893. https://doi.org/10.1016/j.heliyon.2019.e01893

Akinola, J. O., Olawusi-Peters, O. O., & Akpambang, V. O. E. (2019). Ecological hazards of total petroleum hydrocarbon in brackish water white shrimp (Nematopalaemon hastatus). Egyptian Journal of Aquatic Research, 45(3), 205–210.

Alagoa, K., Godwin, J., Daworiye, P., & Ipiteikumoh, B. (2018). Evaluation of total petroleum hydrocarbon (TPH) in sediments and aquatic macrophytes in the River Nun, Amasoma axis, Niger Delta, Nigeria. International Journal of Environmental and Agricultural Research, 4(6), 63–67.

Anarado, C. J. O., Okpoji, A. U., & Anarado, C. E. (2023). Bioaccumulation and health risk assessment of lead, cadmium, arsenic, and mercury in blue crabs found in creeks in Bayelsa State of the Niger Delta region of Nigeria. Asian Journal of Environmental & Ecology, 21(4), 46–59.

Arzaghi, E., Abbassi, R., Garaniya, V., Binns, J., & Khan, F. (2018). An ecological risk assessment model for Arctic oil spills from subsea pipelines. Marine Pollution Bulletin, 135, 1117–1127. https://doi.org/10.1016/j.marpolbul.2018.08.030

Atta, H. A. (2020). Assessment and geographic visualization of salinity of Tigris and Diyala rivers in Baghdad City. Environmental Technology & Innovation, 17, 100538.

Borja, Á., Amouroux, D., Anschutz, P., Gómez-Gesteira, M., Uyarra, M. C., & Valdés, L. (2019). The Bay of Biscay. In C. Sheppard (Ed.), World Seas: An Environmental Evaluation (2nd ed., pp. 113–152). Academic Press.

Brusseau, M. L., Matthias, A. D., Comrie, A. C., & Musil, S. A. (2019). Atmospheric pollution. In M. L. Brusseau, I. L. Pepper, & C. P. Gerba (Eds.), Environmental and Pollution Science (3rd ed., pp. 293–309). Academic Press.

Chase, D. A., Edwards, D. S., Qin, G., Wages, M. R., Willming, M. M., Anderson, T. A., & Maul, J. D. (2013). Bioaccumulation of petroleum hydrocarbons in fiddler crabs (Uca minax) exposed to weathered crude oil and dispersant mixtures. Science of the Total Environment, 444, 121–127. https://doi.org/10.1016/j.scitotenv.2012.11.078

Chiesa, L. M., Zanardi, E., Nobile, M., Panseri, S., Ferretti, E., Ghidini, S., Foschini, S., Ianieri, A., & Arioli, F. (2019). Food risk characterization from exposure to persistent organic pollutants and metals contaminating eels from an Italian lake. Food Additives & Contaminants: Part A, 36(5), 779–788. https://doi.org/10.1080/19440049.2019.1591642

Dhamar, A., Vita, N., Hilmi, E., Piram, A., & Doumenq, P. (2013). Source apportionment of sedimentary hydrocarbons in the Segara Anakan Nature Reserve, Indonesia. Marine Pollution Bulletin, 74(1), 141–148. https://doi.org/10.1016/j.marpolbul.2013.07.015

Dibofori-Orji, A. N., Ihunwo, O., Udo, K. S., Shahabinia, A. R., Onyema, M. O., & Mmom, P. C. (2019). Spatial and temporal distribution and contamination assessment of heavy metals in Woji Creek. Environmental Research Communications, 1(10), 1–10.

Dong, J., Xia, X., Wang, M., Xie, H., Wen, J., & Bao, Y. (2016). Effect of recurrent sediment resuspension-deposition events on bioavailability of polycyclic aromatic hydrocarbons in aquatic environments. Journal of Hydrology, 540, 934–946.

Ekpe, J. E., Okpoji, A. U., Ekwere, I. O., Anarada, C. J., Ewuola, A. A., Ufuoma, V. O., Okpanachi, C. B., Okonkwo, C. O., & Obunezi, O. A. (2025). Physico-chemical transport of pesticides and nutrients in irrigation water and their fate in agro-ecosystems of Rivers State, Nigeria. Palgo Journal of Agriculture, 9(2), 30–37. 
https://doi.org/10.5281/zenodo.17148360

Ekpo, B. O., Oyo-Ita, O. E., Oros, D. R., & Simoneit, B. R. T. (2012). Distributions and sources of polycyclic aromatic hydrocarbons in surface sediments from the Cross River Estuary, southeastern Niger Delta, Nigeria. Environmental Monitoring and Assessment, 184(2), 1037–1047. https://doi.org/10.1007/s10661-011-2019-5

Ekwere, I. O., Okpoji, A. U., Igwegbe, K. C., Okonkwo, C. O., Yekeen, A. A., Obunezi, O. C., Okpanachi, C. B., Garuba, M. H., Ogini, O. R., & Odibo, U. E. (2025). Nutritional–Toxicological Trade-Off: Comparative Study of Polycyclic Aromatic Hydrocarbons in Smoked and Oven-Dried Nile Tilapia (Oreochromis niloticus). Journal of Environment, Climate, and Ecology, 2(2), 90-97. https://doi.org/10.69739/jece.v2i2.952

Ekwere, I. O., Okpoji, A. U., Ufuoma, V. O., Akinola, A. E., Raymond, C. A., Clement, R. O., Alaekwe, I. O., Etesin, M. O., & Edodi, I. O. (2025). Nutritional evaluation of Nile tilapia (Oreochromis niloticus) processed by different drying methods in Akwa Ibom State, Nigeria. Journal of Sustainable Research and Development, 1(2), 11–17. https://doi.org/10.69739/jsrd.v1i2.1033

Farrington, J. W., & Quinn, J. G. (2015). “Unresolved complex mixture” (UCM): A brief history of the term and moving beyond it. Marine Pollution Bulletin, 96(1–2), 29–31. https://doi.org/10.1016/j.marpolbul.2015.04.039

Frasson de, M. R. P., Pavelsky, T. M., Fonstad, M. A., Durand, M. T., Allen, G. H., Schumann, G., Lion, C., Beighley, R. E., & Yang, X. (2019). Global relationships between river width, slope, catchment area, meander wavelength, sinuosity, and discharge. Geophysical Research Letters, 46(6), 3252–3262.

John E., E., Emah O., A., Ifiok O., E., Okpoji, A. U., Iyam O., E., Henderson O., O., Akinola A., E., Gloria N., A., & Charles N., O. (2025). Health Risk Implications of Polycyclic Aromatic Hydrocarbons in Smoked and Dried Fish Consumed in Nigeria. Journal of Medical Science, Biology, and Chemistry, 2(2), 192-199. https://doi.org/10.69739/jmsbc.v2i2.1045

Okpoji, A. U., Anarado, C. O., Mmuta, E. C., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Eboj-Ajoku, I. O., & Obunezi, O. C. (2025). Toxicological evaluation of Pb, Cd, As, and Hg in blue crab from oil-polluted creeks in the Niger Delta. Journal of Life Science and Public Health, 1(1), 24–31. https://doi.org/10.69739/jlsph.v1i1.879

Okpoji, A. U., Chinyere, U. E., Nwokoye, J. N., Ezekwuemen, O. I., Alaekwe, I. O., Odidika, C. C., Owughara, C. N., Enyi, C. M., & Kolawole, O. O. (2025). Environmental assessment of heavy metals and hydrocarbon pollution in surface waters of oil-bearing communities in Andoni, Rivers State, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 22–30. http://www.ijmsrt.com

Okpoji, A. U., Eboh-Ajoku, I. O., Mmuta, E. C., Ndubuisi, J. O., Alaekwe, I. O., Odibo, U. E., Nwoka, N. G., Okafor, C. A., & Obunezi, O. C. (2025). Integrated environmental risk assessment of BTEX and PAHs in water and sediment samples from the Bonny River, Nigeria. ISA Journal of Multidisciplinary (ISAJM), 2(4), 14–21.

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Anarado, C. J. O., Ogbonna, C., Ewuola, A. A., Odibo, U. E., & Garuba, M. H. (2025). Volatile organic compounds from gas flaring and their atmospheric implications in the Niger Delta. Journal of Life Science and Public Health, 1(1), 32–39. https://doi.org/10.69739/jlsph.v1i1.962

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Etesin, U. M., Ugwuanyi, G. C., Okpanachi, C. B., Ewuola, A. A., Mojisola, K. M., & Ezekwueme, O. I. (2025). Solar-enhanced photocatalytic degradation of pharmaceutical residues in wastewater using Fe-, Cu-, and Zn-doped TiO₂ nanomaterials. Journal of Environment, Climate, and Ecology, 2(2), 56–62. https://doi.org/10.69739/jece.v2i2.961

Okpoji, A. U., Emem, J. A., Ekwere, I. O., Odibo, U. E., Alaekwe, I. O., Warder, A. B., & Eboj-Ajoku, O. I. (2025). Bioaccumulation of nickel, lead, and cadmium in tissues of Callinectes sapidus from the Iko River, Nigeria: Implications for human health risk and environmental safety. Journal of Environment, Climate, and Ecology, 2(2), 29–37. https://doi.org/10.69739/jece.v2i2.844

Okpoji, A. U., Ndubuisi, J. O., Eboh-Ajoku, I. O., Emem, J. A., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Onoja, C. R., Warder, A. B., & InyangAbia, A. J. (2025). Trematode infections and histopathological effects in Chrysichthys nigrodigitatus from the polluted Andoni River, Niger Delta, Nigeria. Journal of Agriculture, Aquaculture, and Animal Science, 2(2), 66–72. https://doi.org/10.69739/jaaas.v2i2.882

Okpoji, A. U., Nwoka, N. G., Odibo, U. E., Alaekwe, I. O., Okafor, C. A., Ogwu, N. G., & Akatakpo, C. U. (2025). Seasonal variation in hydrochemical characteristics and heavy metal risk assessment of groundwater in Andoni-Isiokwan District, Niger Delta, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 44–51. http://www.ijmsrt.com

Onoja, C. R., Ogbaji, H. O., Edodi, I. O., Mmuta, E. C., Igwegbe, K. C., Ogini, O. R., Odibo, U. E., Ewuola, A. A., Mahmoud, A. B., & Okpoji, A. U. (2025). Heavy Metal Contamination and Health Risk Assessment of Smoked-Dried Fish Sold in Eke-Awka Market, Anambra State, Nigeria. Journal of Sustainable Research and Development, 1(2), 18-25. https://doi.org/10.69739/jsrd.v1i2.1044