Analysis of Some Environmental Parameters and Their Effect on the Productivity of Fishes

ABSTRACT

Environmental factors directly influence the growth and survival of fishes and the quality of the water, in which they are cultured, is an important environmental factor in this regard. A study was conducted in the Alekargul and Tikarpara village (Badarpur sub-division) of Sreebhumi (earlier Karimganj) district, Assam to investigate some physicochemical parameters of waters of fisheries and non-maintained ponds present in those villages. The water quality parameters such as pH, temperature, dissolved oxygen, biological oxygen demand, and alkalinity were analyzed. The study revealed that there is significant variation in some of the water properties which pose a challenge for good productivity of fishes. This study highlights the scope for utilization of the water resources in the study area by introducing proper management practices.

Key Words: alkalinity, dissolved oxygen, Karimganj, management practice, environmental parameters, water quality.

INTRODUCTION

A fishery is a sunrise sector of our Indian economy.  Fishes are aquatic, cold blooded vertebrates and cultured in ponds and tanks. Their growth and survival is influenced by the quality of the water in which they are cultured. Larger water bodies like reservoirs, lakes and rivers have relatively uniform environmental conditions whereas smaller water bodies such as ponds, tanks, hatcheries etc. in which fishes are raised, are subject to more variable conditions.

Water is the physical support in which fishes carry out their life functions such as feeding, swimming, breeding, digestion and excretion (Bronmark and Hansson, 2005). Many workers have reported the status of water bodies (lentic and lotic) after receiving various kinds of pollutants altering water quality characteristics (physical, chemical and biological). All living organisms have tolerable limits of water quality parameters in which they perform optimally. A sharp drop or an increase within these limits has adverse effects on their body functions (Davenport, 1993; Kiran, 2010). So, good water quality is very essential for survival and growth of fish. As we know fish is an important protein rich food resource and there has been sharp increase in demand of fish products due to increasing population pressure in this century, so to meet the demand of present food supply, water quality management in fish ponds is a necessary step that is required to be taken up. The distribution of fish ponds in the country is determined by several factors such as availability and quality of water, suitable land for fish farming, awareness and motivation within the community on the economic potential of fish farming.

Fish growth depends on environmental water quality in order to boost its production and physicochemical parameters are known to affect the biotic components of an aquatic environment in various ways (Ugwumba and Ugwumba, 1993). Knowledge of hydrological conditions and plankton of any water body is not only useful in assessing its productivity, but will also permit a better understanding of the population and life cycle of the fish community (Adebisi, 1981; Ayodele and Ajani, 1999).

In the presence of environmental stress such as low dissolved oxygen, high temperature and high ammonia (Boyd, 1981) the ability of organisms to maintain its internal environment (i.e. metabolism, catabolism and reproduction) is reduced (Ezra and Nwankwo, 2001). In view of this, monitoring of water quality, which centers on determination of optimal, sub-lethal and lethal values of physicochemical parameters standardized for fish culture should be embraced (Boyd and Lichtkoppler, 1985).

Such physicochemical parameters include dissolved oxygen (DO), temperature, pH, conductivity and biological oxygen demand (BOD). Several of these physicochemical parameters have been studied on indigenous habitats (APHA, 1991, Boyd, 1981, King, 1998, Ezra and Nwankwo, 2001 and Fafioye et al., 2005). However, pond habitats can be easily manipulated by controlling the water characteristics for an optimum environment yielding high level fish production.

Generally fish needs 250C to 320C of water temperature. Dissolved oxygen concentration (DO) is considered the most important water quality variable in fish culture. Biochemical oxygen demand (BOD) is the amount of dissolved oxygen needed by aerobic biological organisms in a body of water to break down organic material present in a given water sample at certain temperature over a specific time period. Total alkalinity of 20 mg/L or more is necessary for good pond productivity. A desirable range of total alkalinity for fish culture is between 75 and 200 mg/L CaCO3 (Boyd & Lichtkoppler, 1985). The ph value is an indicator of the presence of metabolites, photosynthetic activity and fertility of the pond water. The recommended pH range for aquaculture is 6.5 to 9.0.

Hence the present study is aimed at analyzing the above mentioned ecological parameters to assess the productivity of fishes in maintained and non maintained ponds, to review and present a concise opinion regarding the optimum levels of water quality characteristics required for maximum fish production in maintained (fishery) and non-maintained (household) ponds.

STUDY SITES

The studies of fisheries were carried out in the village Alekargul, Badarpur and the ponds were studied in the village Tikarpara, Badarpur which is about 25km from Karimganj. Generally fisheries are maintained and ponds are non-maintained. Therefore, two fisheries and two ponds were selected for the study.

In the fisheries varieties of fishes are cultivated in their stocking season such as Cyprinus carpio (common carp), Hypophthalmichthys molitrix (silver carp), Ctenopharyngodon idella (grass carp), Labeo rohita (rohu), Labeo boggut (Ghonia), Oncochromis mosambicus (tilapia), Labeo calbasu (kalabaus), catla catla (catla), Cirrhinus mrigala (mrigal), Pungasius pungasius (pungus), Carassius cavasius (golden carp) etc. and different types of food products are supplied for their stock. Rice bran and oil cakes are used as fish food.

The fishes cultivated in both the ponds are Ctenopharyngodon idella (grass carp), Anabas (koi),

PIC 1. STUDY SITES

Labeo rohita (rohu), Oncochromis mosambicus (tilapia), Labeo calbasu (kalibaus), Catla catla (catla), Cirrhinus mrigala (mrigal), Channa marulius (soal), Wallago attu (boal), Puntius sp. (puthi), Mystus tengara (Tengra), Clarius batrachus (magur), Heteropneustus fossils (shingi) , Channa punctatus (cheng) etc. These fishes are developed in the ponds and are used for self consumption and some are used for business purpose.

MATERIALS & METHODS

METHOD

Two fisheries and two ponds were studied. In each area the ponds and fisheries were randomly sampled and result presented in a generalized form.

Water samples were collected using plastic bottle of capacity half liters at 15 days interval.

Temperature: Mercury in glass thermometer (50°C) was lowered into water up to 2 cm below the water surface, allowed to stabilize for 2 minutes and readings were taken in degree Celsius (0C).

pH: The pH of samples were measured by a hand held Sigma Hanna Checker-1 pH meter.

DO: Winkler’s titrimetric method was used to estimate the level of DO in mg/l for each pond.

BOD: The value of this parameter was calculated using standard methods described by Boyd (1979) and APHA (1991).

Alkalinity: The values of this parameter were calculated using standard methods.

RESULTS & DISCUSSION

Table 1: Temperature of the water samples collected from different water bodies

| NO. OF SAMPLE COLLECTION | DATE OF SAMPLE COLLECTION | WATER OF POND-1 | WATER OF POND-2 | WATER OF FISHERY-1 | WATER OF FISHERY-2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 02.08.25 | 28.00C | 29.50C | 32.00C | 31.00C |
| 2 | 16.08.25 | 28.00C | 28.00C | 30.00C | 29.00C |
| 3 | 02.09.25 | 28.00C | 29.00C | 31.00C | 30.00C |
| 4 | 16.09.25 | 31.00C | 32.00C | 31.00C | 32.00C |
| 5 | 06.10.25 | 29.00C | 29.50C | 32.00C | 29.00C |
| 6 | 16.10.25 | 31.00C | 28.00C | 30.00C | 30.00C |

Table 2: Variation of dissolve oxygen in water collected from sampled sites

| NO. OF SAMPLE COLLECTION | DATE OF SAMPLE COLLECTION | WATER OF POND-1 | WATER OF POND-2 | WATER OF FISHERY-1 | WATER OF FISHERY-2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 02.08.25 | 5.0mg/l | 4.0mg/l | 9.1mg/l | 6.0mg/l |
| 2 | 16.08.25 | 5.5mg/l | 4.3mg/l | 7.0mg/l | 9.0mg/l |
| 3 | 02.09.25 | 5.4mg/l | 8.5mg/l | 7.5mg/l | 8.5mg/l |
| 4 | 16.09.25 | 4.5mg/l | 6.6mg/l | 5.1mg/l | 7.0mg/l |
| 5 | 06.10.25 | 9.3mg/l | 8.0mg/l | 7.5mg/l | 8.5mg/l |
| 6 | 16.10.25 | 7.4mg/l | 7.0mg/l | 6.6mg/l | 6.0mg/l |

Table 3:  pH value recorded from water samples at different water bodies

| NO. OF SAMPLE COLLECTION | DATE OF SAMPLE COLLECTION | WATER OF POND-1 | WATER OF POND-2 | WATER OF FISHERY-1 | WATER OF FISHERY-2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 02.08.25 | 7.2 | 6.76 | 6.95 | 6.74 |
| 2 | 16.08.25 | 7.0 | 6.98 | 6.64 | 6.82 |
| 3 | 02.09.25 | 6.0 | 7.1 | 6.54 | 6.78 |
| 4 | 16.09.25 | 6.6 | 6.67 | 6.76 | 6.64 |
| 5 | 06.10.25 | 6.0 | 6.0 | 7.0 | 7.2 |
| 6 | 16.10.25 | 7.0 | 6.1 | 7.5 | 6.8 |

Table 4: Total alkalinity of water samples of different water bodies

| NO. OF SAMPLE COLLECTION | DATE OF SAMPLE COLLECTION | WATER OF POND-1 | WATER OF POND-2 | WATER OF FISHERY-1 | WATER OF FISHERY-2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 02.08.25 | 100ppm | 124ppm | 140ppm | 120ppm |
| 2 | 16.08.25 | 190ppm | 188ppm | 176ppm | 200ppm |
| 3 | 02.09.25 | 98ppm | 80ppm | 100ppm | 124ppm |
| 4 | 16.09.25 | 168ppm | 90ppm | 190ppm | 124ppm |
| 5 | 06.10.25 | 50ppm | 90ppm | 150ppm | 100ppm |
| 6 | 16.10.25 | 80ppm | 85ppm | 120ppm | 190ppm |

Table 5: Biological oxygen demand of water samples of different water bodies (After each 3 days incubation)

| NO. OF SAMPLE COLLECTION | DATE OF SAMPLE COLLECTION | WATER OF POND-1 | WATER OF POND-2 | WATER OF FISHERY-1 | WATER OF FISHERY-2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 05.08.25 | 3.5mg/l | 3.8 mg/l | 4.0 mg/l | 5.1 mg/l |
| 2 | 19.08.25 | 4.2 mg/l | 3.5 mg/l | 4.3 mg/l | 4.7 mg/l |
| 3 | 05.09.25 | 3.8 mg/l | 5.2 mg/l | 4.8 mg/l | 4.5 mg/l |
| 4 | 19.09.25 | 3.1 mg/l | 3.0 mg/l | 4.5 mg/l | 4.8 mg/l |
| 5 | 06.10.25 | 4.1 mg/l | 4.2 mg/l | 4.2 mg/l | 4.2 mg/l |
| 6 | 18.10.25 | 5.0 mg/l | 5.5 mg/l | 5.8 mg/l | 4.1 mg/l |

Table 6: Mean variation of the physicochemical parameters of the water samples

collected from the experimental water bodies

| SL. NO. | SAMPLE SITES | TEMPERATURE (0C) | DO (mg/L) | pH | ALKALINITY (ppm) | BOD |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Pond – 1 | 28.30 | 5.4 | 6.6 | 114.4 | 3.9 |
| 2 | Pond – 2 | 28.30 | 6.4 | 6.6 | 109.5 | 4.2 |
| 3 | Fishery – 1 | 31.00 | 7.1 | 6.8 | 146.0 | 4.6 |
| 4 | Fishery - 2 | 30.10 | 7.5 | 6.8 | 143.0 | 4.5 |

| STUDY SITES | WEIGHT OF FISHES (IN KG) |  | AMOUNT (IN INR) |  | PROFIT (IN INR) |
| --- | --- | --- | --- | --- | --- |
|  | Pre-stocking | Post-harvesting | Pre-stocking | Post-harvesting |  |
| Pond-1 | 55kg | 670kg | 3,570/- | 60,600/- | 57,030/- |
| Pond-2 | 45kg | 480kg | 2,800/- | 48,000/- | 45,200/- |
| Fishery-1 | 500kg | 1,200kg | 12,000/- | 5,00,000/- | 4,88,000/- |
| Fishery-2 | 800kg | 25,00kg | 30000/- | 550000/- | 5,20,000/- |

Table 7:  Fish yields of studied aquatic bodies throughout the year

The ranges of various water quality parameters are summarized in the Tables 1 to 6 and it is found that the water temperature varies from 280C-320C (Table 1) and it is suitable for fish culture. DO vary between 4-9 mg/l (Table 2) and the concentration of DO is one of the most important parameters to indicate water purity and to determine the distribution and abundance of various algal groups. Mean pH values vary between 6.0 and 7.5 (Table 3). It indicates slightly basic and alkaline nature of water sample and total alkalinity varies between 80-200ppm (Table 4) and it represents the optimum limit for fish pond. BOD was between 3-9mg/l (Table 5). It is optimum for normal activities of fish.

Fishes do not like any kind of changes in their environment. Any changes add stress to the fishes and the larger and faster the changes, the greater the stress. So the maintenance of all the factors becomes very essential for getting maximum yield in a fish pond. Good water quality is characterized by adequate oxygen, proper temperature, limited levels of metabolites and other environmental factors affecting fish culture. The initial studies of water quality of a fish pond in India were probably conducted by Sewell (1927) and Pruthi (1932). After that many workers have studied the physicochemical condition of inland waters either in relation to fish mortality or as part of general hydrological survey (Alikunhi et al., 1952; Upadhyaya, 1964).

The various physicochemical parameters of water, recorded for the two maintained fisheries and two non-maintained ponds, were favourable for fish culture. Mean surface water temperature of 280C to 320C recorded (Table 1) agreed to the ranges recorded by Ugwumba and Ugwumba (1993). The least temperatures of 250C might be due to shading of water in Fishery-1 by brunches of trees surrounding it. This prevented sunrays from direct contact with the water surface.

Fig. 1: Graphical representation of mean variation of temperature

DO with higher ranges 4-9mg/l recorded (Table 2) is within the ranges documented by Boyd (1985) for good water quality on fish culture. This is because oxidation converts otherwise poisonous compounds to useful materials. It also encourages good feeding, food utilizing and high stocking density for fish eggs, larva and adults.

Fig. 2: Graphical representation of mean variation of DO

The average pH values of 6.0- 7.5 recorded in this study (Table 3) were within pH values of 6.5-9.0 documented by Boyd (1985). This value is most suitable for fish production for maximum productivity.

Fig3: Graphical representation of mean variation of pH

The mean total alkalinity of 80-200 ppm recorded (Table 4) agreed within the ranges documented by Bhatnagar et.al, (2004) is desirable for fish culture.

Fig. 4: Graphical representation of total mean variation of alkalinity

According to Bhatnagar et.al, (2004) the BOD level between 3.0-6.0 ppm is optimum for normal activities of fish. These marked differences of the values of physicochemical parameters in the different ponds might be due to effect of temperature, plankton density and concentration of organic and related factors.

Fig. 5: Graphical representation of mean variation in BOD

The relationship between fish yield and water parameters showed that no single parameters can be singled out in relation to fish growth and health. Among two fisheries and ponds, fishery-2 and pond-1 shows better fish yield than the fishery-1 and ponds-2. However, five of these parameters (i.e., temperature, DO, pH, alkalinity and BOD) must be kept at optimum level to guarantee high fish yield. The high temperatures of 300C, 320C, 310C and 280C recorded for fishery-1, fishery-2, pond-1 and pond-2 respectively might have resulted in better feeding and food conversion for the fishes. This is supported by Lin (1951) who documented that temperature ranges of 270C to 320C will allow tropical fish to eat more and grow faster.

Conclusion

The present study reveals that the environmental physicochemical parameters of water i.e., Temperature, DO, pH, Alkalinity and BOD of two fisheries are within the optimum range for fish growth and development and this is why we found considerable growth and profit.

However though the physicochemical parameters of water of non-maintained ponds are also favourable for fish growth and development, due to non-maintenance of the water, the yield is poor in comparison to that of the fisheries. If non-maintained ponds are properly managed by the owner or by the local community with minimum investment then fish production will increase which will in turn meet the demand of fish for local inhabitants.  In addition to these, this will improve the socioeconomic status of the people involved in this sector.

This study showed that fish yield is dependent on the quality and management of water bodies. In our study it is found that fishery-2 is the best maintained aquatic body for fish production and gives better yield in comparison to the other studied aquatic bodies.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

- Alikunhi K. H., Ramachandra V. and Chaudhuri H. 1952. Mortality of carp fry under supersaturation of dissolved oxygen in water, Proceedings of the national institute of sciences of India, 17 (4), pp 261-264.

- Adebisi A. A. 1981. The physico-chemical hydrology of a tropical seasonal river-Upper Ogun River, Hydrobiologia, 79 (2):157-165.

- APHA. 1991. Standard Methods for the Examination of Water and Waste Water, Including Bottom Sediments and Sludge. 14th ed., American Public Health Association, New York, USA. 1193 p.

- Ayodele I. A. and Ajani E. K. 1999. Essentials of fish farming (Aquaculture). Odufuwa Press, lbadan. 46p.

- Bhatnagar A., Jana S.N., Garg S.K., Patra B.C., Singh G. and Barman U.K. 2004. Water quality management in aquaculture, In: Course Manual of summerschool on development of sustainable aquaculture technology in fresh and saline waters, CCS Haryana Agricultural, Hisar (India), pp 203- 210. Boyd C. E. 1979. Water quality in warm water fish ponds. Anburn University, Alabama. 359p.

- Boyd C. E. 1981. Water Quality in warm water fish ponds. Anburn University, Alabama. 359p. Craftmaster Printers, Inc. Opelika, Alabama.

- Boyd C. E. and Lichtkoppler F. R. 1985. Water Quality Management in Pond Fish Culture Res. Dev. Ser.: 22 Auburn University Aubum Alabama.

- Boyd, C. E. 1985. Pond evaporation. Trans. Am. Fish. Soc. 114: 99-303.

- Bronmark C. and Hansson L. A. 2005. The biology of lakes and ponds, Oxford University Press, Oxford, pp 285.

- Davenport Y. 1993. Responses of the Blennius pholis to fluctuating salinities, Marine Ecology Progress Series, 1, pp 101 – 107.

- Ezra A.G. and Nwankwo D.I. 2001. Composition of phytoplankton algae in Gubi reservoir, Bauchi, Nigeria. Journal of Aquatic Sciences 16(2): 115- 118.

- Fafioye O.O., Olurin K.B. and Sowunmi A.A. 2005. Studies on the physicochemical parameters  of Omi water body of Ago-lwoye, Nigeria. Arican Journal o Biotechnology, 4 (9): 1022-1024.

- King R. P. 1998. Physico-chemical indices of the fisheries potential of a Nigerian rainforest pond. J. Aquatic Sciences 13: 49-54.

- Kiran B. R. 2010. Physicochemical characteristics of fish ponds of Bhadra project at Karnataka, Rasāyan Journal of Chemistry, 3(4), pp 671-676.

- Lin S. Y. 1951. Pond Culture of Warm Water fishes. Prog Pf. The UN scientific conference on the conservation and utilization of resources-(Aug. Sept., 1946).

- Pruthi H.S. 1932. Investigations regarding a recent epidemic of fish mortality in the tank in the Indian museum compound with remarks on the causation of such epidemics in general. Review of Hydrobiology Hydrographic, 26, pp 242-257.

- Sewell R.B.S. 1927. On mortality of fishes, Journal of the Asiatic Society of Bengal, 22, pp 177-204.

- Ugwumba A. O. and Ugwumba A. A. 1993. A study of the physico-chemical hydrology and Plankton of Awba Lake in lbadan, Nigeria. Fish Acadbiz. Comm.1(1-4):20-39.

- Upadhyaya M.P. 1964. Seminar on inland fisheries development in U. P., pp 127-135.