Comparative study of anthropogenic activities influences on dam water quality of Boundiali, Korhogo and Ouangolodougou (Cote d'Ivoire)

ABSTRACT: Aims: This study was conducted in the context of water resource security. The main objective was to highlight human activities that coud have a significant impact on dams water quality of Boundiali, Korhogo, Ouangolodougou. Study design: Sampling of water from dams was firstly done in order to analyze and determine content of water parameters. Then water quality and pollution indexes were calculated to show water quality and pollution levels. Finally, a survey has been realized to highlight potentials dams waters pollution origins. Place and Duration of Study: First sampling campaign was carried out during rainy season from August to September 2021 and second in dry season from December 2021 to January 2022, at Boundiali, Korhogo, Ouangolodougou dams. Methodology: The adopted methodology consisted of water characterization in these infrastructures and their compliance comparison with WHO drinking water supply standards. Subsequently, quality and pollution indices were determined in order to provide an overview of quality and pollution levels of these resources. Then, surveys were carried out with a view to identifying sites likely to be responsible for pollution. Finally, similarities and divergences in human activities that are the source of these waters quality degradation were noted. Results: Characterization results show that these waters have relatively neutral pH values with relatively low electrical conductivities. Metals (iron and arsenic), nitrate, phosphorus and pesticides (POPs and POHs) have concentrations above the WHO drinking water standard. Quality and organic pollution indices indicate that these dams waters have very poor quality in rainy periods and are not drinkable in dry periods with moderate pollution. Field surveys confirmed that the quality of these resources is influenced by human activities, particularly agricultural practices in the immediate perimeters, which are the main pollution factors in Boundiali and Ouangolodougou, unlike Korhogo, where the main sources of pollution are non-seasonal activities such as sewage from riparian houses, livestock farming, artisanal activities and quarries. Conclusion: Waters from these three dams have different pollution sources such as natural incomes but anthropic activities remain to be the main pollution source.

Keywords: Anthropic Activities, dam, pollution, quality, Korhogo

1. INTRODUCTION

Recognized by United Nations, access to drinking water and sanitation is a fundamental issue for sustainable development (Nations Unies, 2015). The goal is to ensure universal and equitable access to safe and affordable drinking water for all by 2030. The right to safe drinking water is essential to life full enjoyment, as it has a directly effect on public health, food security, livelihoods and economic development (UN-Water, 2023). In Cote d'Ivoire, remarkable progress has been made over the past two decades. According to the latest WHO/UNICEF Joint Program (2024) reports, population proportion with access to safe drinking water increased from 46% in 1998 to 82% in 2021, with significant disparities between urban and rural areas. This significant improvement is part of government efforts to achieve national targets of National Development Plan (NDP 2021-2025), which aims to achieve drinking water coverage of 95% in urban areas and 80% in rural areas by 2025 (Ministry of Water and Forests, 2023). Government of Cote d'Ivoire has implented many water infrastructures to achieve these ambitious objectives, particularly in the northern regions (World Bank, 2023), through Project to Strengthen the Supply of Drinking Water in Urban Areas (PREMU). These regions depend largely on dams for water supply since characterized by a Sudano-Sahelian climate with a long dry season from November to April. These water infrastructures are essential for water security of populations, agricultural development and aquatic ecosystems maintenance (Kouamé et al., 2023). However, their durability is threatened by multiple pressure factors. Rapid expansion of agricultural land, unplanned increasing urbanization, accelerated deforestation, and untreated domestic and industrial discharges are all threats to surface water quality (N'Guessan et al., 2024). Deforestation accelerated by agricultural land expansion, rapid urban development, and untreated domestic and industrial effluents are serious threats for surface water quality (N'Guessan et al., 2024). The intensification of agricultural activities in dams watersheds in northern Côte d'Ivoire shown by Soro et al. (2024), leads to a significant increase in nitrate, phosphorus and pesticide contents in waters reservoir. This seriously affects resource quality when combined with natural processes of water erosion, evaporation and runoff (Bamba et al., 2023). The problem of the degradation of the quality of the dam water thus has a health, economic and environmental dimension. Waterborne diseases are still a major morbidity cause in the country northern region (Ministry of Health, 2024), in terms of health. Water quality deterioration results in additional treatment costs, which ultimately reflects in water prices for consumers (SODECI, 2023), economically. Ecosystem resilience are threatened by water bodies eutrophication and aquatic biodiversity loss (Koné et al., 2024). Ecosystem resilience are threatened by water bodies eutrophication and aquatic biodiversity loss (Koné et al., 2024). A comparative and integrated approach is needed, faced with these serious problems, to better understand anthropogenic pressures dynamics on dam water resources. Sustainable dam management strategies must be based on accurate diagnoses of pollution sources and their differential impact depending on local contexts, as recently related by Adingra et al. (2024). The present study has for principal objective to assess and compare the anthropogenic activities impact on water quality of Boundiali, Korhogo and Ouangolodougou dams in northern Côte d'Ivoire. Specific objectives are:

- assess physico-chemical and bacteriological quality and pollution levels of these various dams through seasonal sampling campaigns;

- identify and characterize anthropogenic activities responsible for possible water quality degradation, using survey and direct observation methods;

- Analyze similarities and differences in human pressures effects on these aquatic ecosystems.

2. material and methods

2.1 Material

Presentation of dams geographical situation

The present study covers three cities in the Savannah District in northern Côte d'Ivoire: Boundiali, Korhogo and Ouangolodougou. This border region with Mali and Burkina Faso covers 40,323 km² and has 901,301 inhabitants (RGPH, 2021). Korhogo, the district's capital, covers 1,196.21 km² and encompasses 61 villages (Figure 1). The region is characterized by a Sudano-Sahelian climate marked by a long dry season, making dams essential for water supply. The three study sites have complementary profiles: Boundiali: important agricultural zone, Korhogo: major urban and economic hub, Ouangolodougou: border town with cross-border dynamics. This selection allows a comparative analysis of anthropogenic impacts on water resources in various socio-economic contexts within the same geographical region

Figure 1: Geographical location of study area

Data acquisition and analysis equipment

The equipment used for field data collection includes:

- 1.5 liter plastic bottles, previously rinsed, for collecting water samples

- Storage cooler maintained at 4°C with cold storage for sample preservation

- GARMIN Etrex 10 GPS for precise location of sampling points and peripheral anthropogenic activities

- Structured questionnaires to collect household socio-economic data

- Observation grids to inventory potentially polluting activities around dams

The material used for data analysis:

- a multi-parameter that served as a tool for in situ measurement of physicochemical parameters; (pH, Temperature, Dissolved oxygen, Conductivity, Suspended matter, Turbidity)

- HACH/DR 1900 spectrophotometer for laboratory analysis;

- Glassware for laboratory analysis;

2.2 Methods

2.2.1 Assessment of dams water quality and pollution levels

2.2.1.1 Sampling

Water samples were taken in uncontaminated plastic bottles with a capacity of 1.5 liters. First sampling campaign was carried out during rainy season from August to September 2021 and second in dry season from December 2021 to January 2022. In each campaign, 4 samples were taken at different points on dams (at least about 2 m from banks so as to avoid edge effects as much as possible) and at variable depths of dams (surface, medium and depth). Samples thus collected were subsequently stored in a cooler containing a cold storage tank at 4° C before being transported to laboratory within hours of collection for chemical analyses.

2.2.1.2 Physico-chemical analyses

Several parameters such as pH, temperature, electrical conductivity and dissolved oxygen were measured in situ using a multi-parameter. Analyses of other parameters were carried out in SODECI laboratory by using a laboratory spectrophotometer of HACH/DR 1900 type. These include chemical oxygen demand (COD), biological oxygen demand (BOD), nitrate (NO₃-), nitrite (NO₂-), ammonium (NH4+), metals such as dissolved iron (Fe), manganese (Mn), total phosphate (POᵌ-), lead (Pb), arsenic (As), cyanide (CN), mercury (Hg), organohalogen (POH) and organophosphorus (POP) pesticides and carbamates (CAR). Analyses have been done according to specific methods of analysis (Table I).

Table I: Parameters analysis methods

| Paramètres | Units | Methods |
| --- | --- | --- |
| pH |  | ISO 10523 V 2008 |
| Temperature | °C | Pt 100 |
| Conductivity | µS/Cm | ISO 7888 V 1985 |
| Dissolved Oxygene | %/mg/L | ISO 5814 V 2012 |
| COD | mgO2/L | Tubes fermés |
| BOD5 | mgO2/L | Respirométrique |
| Nitrate | mgNO3- | ISO 7890-3 V 1988 |
| Nitrite | mgNO2- | Diazotation |
| Cyanides | mg/L | Pyrazolone-pyrine |
| Iron | mg/L | AAS ContrAA 700 (Flamme) |
| Manganese | mg/L | ISO 15586 V 2003 |
| Lead | mg/L | ISO 8288 V 1986 |
| Arsenic | µg/L | ISO 17378-2 V2014 |
| Mercure | µg/L | ISO 12846 V 2012 |
| Ammonium | mgNH4+ | NF T90-015-2 V2000 |
| Phosphate | mgPO43- | ISO 6878 V 2004 |
| Organohalogenated pesticides | µg/L | EN NF ISO V6468-97 |
| Organo-chlorinated pesticides | µg/L | EN NF ISO V 11369-97 |
| Carbamates | µg/L | EN NF ISO V11369-97 |

2.2.1.3 Determination of Water Quality Index (WQI)

WQI was calculated using weighted arithmetic index method proposed by Yidana and Yidana (2010). Eight important parameters (pH, OD, EC, T°C, BOD₅, POᵌ-, NH4+ and NO3-) were selected for this purpose in Table II. In this approach, a numerical value called relative weight (Wi), specific to each physico-chemical parameter, is first calculated according to following equation:

Wi=kSi

With

k=1i=1n(1Si)

k = proportionality constant

n: the number of parameters

Si: maximum value of the standard surface water standard (Table II) of each parameter in mg/L in this work except for pH, T°C and electrical conductivity.

Table II: Guide values for drinking water (WHO, 2017)

| Parameters | Units | WHO guides values |
| --- | --- | --- |
| Temperature | °C | 25 |
| Conductivity | µS/cm | 2000 |
| Turbidity | NTU | 5 |
| pH | - | 6,5˂pH˂8,5 |
| NO₃⁻ | mg/L | 50 |
| NO₂⁻ | mg/L | 3 |
| SO₄² ⁻ | mg/L | 250 |
| PO₄ᵌ ⁻ | mg/L | 5 |
| F⁻ | mg/L | 1,5 |
| Cl⁻ | mg/L | 250 |
| Na⁺ | mg/L | ≤150 |
| Mg²⁺ | mg/L | 50 |
| Ca²⁺ | mg/L | 100 |
| NH₄⁺ | mg/L | ≤0,5 |
| Fe²⁺ | mg/L | 0,3 |
| K⁺ | mg/L | ≤12 |

Then, a quality rating scale (Qi) is calculated for each parameter by dividing concentration by the norm of said parameter and multiplying the set by 100 as in following equation:

Qi=(CiSi)×100

Qi: Quality assessment scale for each parameter.

Ci: Concentration of each parameter in mg/L.

Finally, the overall water quality index is calculated by following equation:

WQI=i=1n(Qi×Wi)i=1nWi

Once calculated, Table III allowed to determine water quality class of dams.

Table III: Classification and possible use of water according to WQI (Aher et al, 2016)

| WQI classe | Water type | Possible use |
| --- | --- | --- |
| 0 – 25 | Excellent quality | Potable water, irrigation and industrie |
| > 25– 50 | Good quality | Potable water, irrigation and industrie |
| > 50– 75 | Bad quality | Irrigation et industrie |
| > 75– 100 | Very bad quality | Irrigation |
| > 100 | Non-potable water | Appropriate treatment required before use |

2.2.2.4 Determination of organic pollution index (OPI)

The organic pollution index (OPI) of dams waters was determined using method proposed by Organic pollution index (OPI) of dam water was determined using method proposed by Chahboune et al.( 2012). The principle is to divide beforehand values of polluting elements (ammonia, BOD5, and phosphates) into 5 classes with the average contents of these three parameters obtained during analyses in laboratory. Then, determine the corresponding class number for each parameter using average data proposed by Leclercq (Table IV). In the end, the overall organic pollution index of dam water was determined through arithmetic mean of all parameters pollution classes

Table IV: Organic pollution index classification grid (Leclercq, 2001)

| Classes | NH44+(mg/l) | DBO5 (mgO2/l) | PO₄ᵌ ⁻(μg/l) | IPO | Pollution organique |
| --- | --- | --- | --- | --- | --- |
| 5 | < 0,1 | <2 | < 15 | 4,6 – 5,0 | Nulle |
| 4 | 0,1 – 0 ,9 | 2,1 – 5 | 16 – 75 | 4 – 4,5 | Faible |
| 3 | 1 – 2,4 | 5,1 – 10 | 76 – 250 | 3,0 – 3,9 | Modérée |
| 2 | 2,5 – 6 | 10,1 – 15 | 251– 900 | 2,0 – 2,9 | Forte |
| 1 | > 6 | > 15 | < 900 | 1,0 – 1,9 | Très forte |

2.2.2 Identification of anthropogenic activities likely to pollute the dam water

Identification of human practices that may be harmful to dam water quality was done by individual interview, which consists of administering individual questionnaires to members of each target group. This approach allows to know and evaluate knowledge, attitudes, practices and perceptions of target groups or persons with respect to questions asked (Dawson et al., 1993). These interviews were therefore carried out with the identified resource people living around Boudali, Korhogo and Ouangolodougou dam, who are engaged in any activity around thess dams (farmers, artisans, inhabitants and others). The aim of these survey is to investigate the way that those activities and domestic waste are managed by the latter, and to identify the causes likely to pollute the dam's water.

3. results and discussion

3.1 Dams water physico-chemical characterzation

The results of physico-chemical analyses at each locality show that pH values are neutral and in line with WHO requirements (Figure 2). Boundiali and Korhogo dams have average values of 7.16 and 7.18 respectively. However, for Ouangolodougou dam pH average value is 6.9. In addition, pH in rainy season are higher than in dry season. pH of these dams being neutral and tending to alkalinity (6.1 to 8.2), coincide with results found in Lobo waters (Daloa) whose pH varies successively from 6.65 to 6.93 (Ahoussi et al., 2019). This results may be due to a dissimilarity of natural factors such as climate, vegetation, geology and also activities carried out near resources. pH range recorded at Boundiali, Korhogo and Ouangolodougou dams could be explained by the presence of bicarbonates that have a buffering power in the water or by variation in leaching activities and degradation of organic matter (Abboudi et al., 2014).

Figure 2: pH according seasons

Dams water temperatures are acceptable by WHO standards in both rainy and dry seasons with a guide value of 25°C. Also, these temperatures are higher during the rainy season. This would mean that solubility of salts and gases would have a higher rate in rainy season than in dry season (Figure 3). These high water temperatures (> 30 °C) could probably de due to sunshine in the outer layers during the rainy season as describes Kouakou et al., 2024. High temperature value could affect organoleptic properties of water since it is a factor in modifying chemical and biological properties of water by acting on its density, on gases solubility, on the dissolved salts dissociation, and on biochemical and biological reactions (Makhoukh et al. 2011, Akil et al. 2014).

Figure 3: Temperature according seasons

Electrical conductivity of each dam is characterized by low mineralization (54 to 180 µS/Cm) as shown in Figure 4, which differs from results found in Lobo (Daloa) waters, where conductivity varies from 192.3 µS/cm to 316 µS/cm. EC value also higher in Korhogo and Ouangolo compare to Boundiali.  This difference could be related to dissimilarity of natural factors such as climate, vegetation, geology and to activities carried out near resources as shown by works of Ahoussi et al., 2019.

Figure 4: Conductivity according to seasons

Concentrations of chemical parameters such as nitrate (NO3-), ammonium (NH4+), total phosphate (PO43-) and nitrite (NO2-), OD, COD and BOD5 are shown in Figure 5. OD, COD and BOD5 have highest contents in the 3 dams waters.  Water from Korhogo and Ouangolo dam is more concentrated in COD and BOD5 in dry season than in rainy season, unlike in Boundiali where these values are higher in rainy season than dry season. It is also observed that OD concentration is higher in Boundiali and Korhogo dams in dry season than in rainy season contrary to Ouangolo. Variation of NO₃-, NO₂-, OD and POᵌ- parameters remains less significant (negligible). BOD₅ values are higher than WHO guide values (5 mg/L), reflecting a high organic contents in dam waters. Based on BOD₅ average value and according to works of Kapepula et al. (2015), dam’s water quality would be poor, also similar to those of (Mehounou et al. 2016). Chemical parameters, despite BOD5 and COD, comply with WHO guidance values. These highly variable parameters have values higher than the norm. High values of these parameters could be explained by a high concentration of organic in these dams. This load could be due to direct discharge of house wastewater into dams and use of fertilizers and pesticides in crop fields. Uncontrolled misuse of these products increases pesticides concentration in water. Water bodies with BOD5 levels above 10 mg/L are considered to be poor quality (Beaux 1998), which has been also described in groundwater and surface water of Aplahoué in Benin in cotton production areas (Mehounou et al., 2016).

Figure 5: Concentration of chemical parameters according to seasons

At Korhogo Dam, Metals Trace Elements results show that dissolved iron (Fe) is more concentrated in this resource with an average of 1.35 mg/l. Manganese (Mn) and arsenic have respectively mean values of 0.33 mg/l and 0.09 mg/l (Figure 6).

These high concentrations are thought to be caused by uncontrolled wastes deposits encountered in places near dams, agricultural inputs used in study areas (chemical fertilizers, insecticides or fungicides), industrial wastewater discharged into nature without prior treatment and geological formations since this dam is located in an urban environment. Concentrations of cyanide (CN), lead (Pb) and mercury (Hg) are relatively low with a respective mean of 0.01 mg/l, 0.01 mg/l, 0.0016 mg/l. Dissolved iron, arsenic and manganese are higher in both seasons.

At Ouangolodougou dam, Iron (Fe) and arsenic (As) are the most abundant elements.

Mean value of arsenic is 0.09 mg/L. This value remains 9 times higher than WHO guideline values (10µg/L), constituting a real health risk. For Iron (Fe), mean concentration is 1.4 mg/L, exceeding standards set by WHO (0.5 mg/L). This high concentration of iron and As could be due to the use of chemical fertilizers, insecticides or fungicides, geological formations, domestic, artisanal and gold mining activities. Other parameters such as Manganese (Mn), Cyanide (CN), Lead (Pb), and Mercury (Hg) have their concentrations in line with WHO guideline values. However, all metal trace elements generally have higher concentrations in rainy than dry season except for mercury, cyanide and lead, which are almost identical in both seasons. According to these results in these various dams, arsenic and iron are the two major ones having a concentration much higher than the norm. In the spring waters exploited by the population of the Mangouin-Yrongouin village, researchers found an arsenic value (<0.002 mg/L) very different from that of Ouangolo dam. In their view, it would appear that in absence of industrial activities, concentration of this metalloid in natural waters poses almost no norm conformity problem (Ahoussi et al., 2013). Mohod and Dhote (2013) also confirm that toxic metals are generally present in industrial, municipal and urban effluents. For this purpose, high values of arsenic (0.21 mg/L) and iron (3.29 mg/L) found could be comparable to discharges of municipal wastewater into the dam.

Hama et al. (2023) and Xu et al. (2022) has explained that the occurrence of heavy metal contamination in surface water maybe due to the increase in human population, industrialization, and agricultural activities expansion. For Singh et al. (2022), Heavy metals in environment pose a real public health risk to populations since the exposure to heavy metals can damage kidneys, livers and lungs leading to the development of different cancers, fragilizing Bones and death in case of huge amount of exposure. Zhang et al. (2023) decribed that contaminated water with heavy metal ions like Cr (VI), Cd (II), Pb (II), As (V and III), Hg (II), Ni (II), and Cu (II) could be responsible for several health issues in humans, mental disorders, harmful effects on the reproductive system. Even malignancies can be developed (Singh et al. 2024).

Figure 6: Seasonally Metals trace elements contents

Studied pesticides are organohalogens (POH), organophosphorus (POP) and carbamates (CBT). In general, high concentrations of these various resources are observed during rainy season and more intensively in Korhogo and Ouangolodougou (Figure 7). At Boundiali, dam results show fairly considerable average concentrations above WHO standard (0.1 µg/l), for POH and POP respectively of 0.51 µg/l and 0.26 µg/l. On the other hand, the CBT concentration is low on average by 0.03 µg/l. At the Korhogo dam, average concentration of CBT is very low. POHs have highest value with an average of 2.47 µg/l, then follows POPS with 1.79 µg/l and finally CBTs with 0.05 µg/l. Apart from carbamates which respects limit of WHO guideline values for pesticides (0.1 µg/l), others (POH and POPs) with mean concentrations of 2 and 1 µg/l, are above normal. At the Ouangolodougou dam, shows that average concentration of these pesticides remained high.  These values are above indicated threshold by WHO standard for pesticides (0.1µg). Similar results have been related by Lakhili et al. (2015), where authors explained that pesticides used to control pests, are drained during rainy season toward surface waters and remain persistent in dry period. Once pesticides reach water bodies, it becomes a global health concern as pesticides can impact the whole ecological food chain (Kruć-Fijałkowska et al. 2022).  Another concern is high level toxicity reached by pesticides when mixed are mixed in water bodies as revealed Oltramare et al. (2023) and long-time exposure to the low concentration of pesticides which could resulted in non-carcinogenic health risks (De Souza et al. 2020).

Figure 7: Seasonally concentration of pesticides

3.2 Assessment of water quality and pollution level of various dams by WQI and OPI

WQI calculation has been done using results of physico-chemical analysis and values of WHO drinking water standard (WHO, 2017). WQI was calculated over the two seasons (August-September) and (December-January). Both WQIs results are considered to be high (Tables V, VI, VII), but highest values are found in dry season with 477.620 for Boundiali, 130 for Korhogo and 147.66 for Ouangolodougou, while dry season WQI were 83.467 in Boundiali dam, 190.6404 in Korhogo and 94.81 in Ouangolodougou. In rainy season dams waters are classified as very bad quality to non-potable water for drinking supply water. In dry season, all dams waters are qualified as non-potable water for drinking supply water. These resuts are consistent with those of Vital et al. (2018) on the evaluation of surface water contamination and its impacts on health in mining districts of Kambélé and Bétaré-Oya. Dams Water quality degradation in the present study could be attributed mainly to agricultural activities through leaching of soils with high fertilizer content and urban wastewater discharges (Kim et al. 2023).  Variation of seasonal water quality has been widely reported by (Yao et al. 2020). High value of WQI in dry periods involved the fact that water level drops (no rain) while inputs wastewaters from nearest houses of dams remain important, as documented by works of Talhaoui et al. (2020) and El Hmaidi et al. ( 2020).

Table V: Rainy and dry seasons WQI of Boundiali dam

|  |  |  |  | Saison Pluvieuse |  | Saison Sèche |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| pH | 8,5 | 0,1176 | 0,0435 | 86,6712 | 83,467 | 81,9672 | 477,620 |
| T (°C) | 25 | 0,04 | 0,0148 | 109,88 |  | 90,28 |  |
| Cond (µs/cm) | 2000 | 0,0005 | 0,0002 | 2,5485 |  | 6,351 |  |
| OD (mg/l) | 8 | 0,125 | 0,0462 | 549,375 |  | 9360 |  |
| NH4+(mg/l) | 0,5 | 2 | 0,7399 | 49 |  | 49,4 |  |
| NO3-(mg/l) | 50 | 0,02 | 0,0074 | 0,74 |  | 0,6 |  |
| PO43-(mg/l) | 5 | 0,2 | 0,0740 | 2 |  | 9,8 |  |
| DBO5 (mg/l) | 5 | 0,2 | 0,0740 | 220 |  | 40,6 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

Table VI: Rainy and Dry Season WQI for Korhogo Dam

|  |  |  |  | Saison Pluvieuse |  | Saison Sèche |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| Ph | 8,5 | 0,1176 | 0,0435 | 85,0980 | 190,6404 | 83,9216 | 130,0898 |
| T (°C) | 25 | 0,04 | 0,0148 | 116,8 |  | 106,24 |  |
| Cond (µs/cm) | 2000 | 0,0005 | 0,0002 | 7,575 |  | 8,7116 |  |
| OD (mg/l) | 8 | 0,125 | 0,0462 | 81,25 |  | 107,475 |  |
| NH4+(mg/l) | 0,5 | 2 | 0,7399 | 190,6667 |  | 113,3334 |  |
| NO3-(mg/l) | 50 | 0,02 | 0,0074 | 0,9467 |  | 0,5233 |  |
| PO43-(mg/l) | 5 | 0,2 | 0,0740 | 7,0667 |  | 7 |  |
| DBO5 (mg/l) | 5 | 0,2 | 0,0740 | 538,6667 |  | 480 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

Table VII: Rainy and Dry Season WQI for Ouangolodougou Dam

|  |  |  |  | Saison Pluvieuse |  | Saison Sèche |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| Ph | 8,5 | 0,1176 | 0,0435 | 87,35 | 94,81 | 75 | 147,66 |
| T (°C) | 25 | 0,04 | 0,0148 | 114,70 |  | 92,87 |  |
| Cond (µs/cm) | 2000 | 0,0005 | 0,0002 | 4,95 |  | 7,53 |  |
| OD (mg/l) | 8 | 0,125 | 0,0462 | 89,74 |  | 32,16 |  |
| NH4+(mg/l) | 0,5 | 2 | 0,7399 | 98,00 |  | 172 |  |
| NO3-(mg/l) | 50 | 0,02 | 0,0074 | 2,59 |  | 0,99 |  |
| PO43-(mg/l) | 5 | 0,2 | 0,0740 | 6,70 |  | 4 ,8 |  |
| DBO5 (mg/l) | 5 | 0,2 | 0,0740 | 164,00 |  | 188 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

OPI for rainy and dry seasons were calculated using pollution indicator parameters (NH4+, BOD5 and PO43). In Boundiali, IPO has a high value of 3.667 in rainy season and 2.667 in dry season (Table VIII). However, values show that Boundiali dam has moderate organic pollution in rainy season while in dry season it is strong. At Korhogo, IPO level has a constant value of 2.33 in both seasons (Table IX). So this shows that Kôkô dam has a strong organic pollution. Finally, in Ouangolodougou IPO is 3 in rainy season and 2,667 in dry season (Table X). This therefore reflects a moderate pollution in organic matter. Waters of these dams in dry season have high organic pollution. The pollution index (IPO), generally remains moderate with some difference in value. The qualitative deterioration is mainly anthropogenic related to agricultural activities by leaching of soils heavily loaded with fertilizers (Vital et al., 2018). Similar work has shown that the increase in the pollution index in dry season is linked to the decrease in dam flows due rain reduction, while the loaded of wastewater from the city remain significant (Talhaoui et al., 2020; El Hmaidi et al., 2020). Moderate pollution of these dams means that these resources are subject to high levels load of organic matter (BOD₅) and phosphate. Indeed, domestic and industrial wastewater discharges of Ouangolodougo and Korhogo cities result in an increase in contents of these above mentioned parameters, which favor degradation of water quality (Benbouih et al., 2005).

Table VIII: Boundiali Dam Rainy and Dry Season IPO

|  | Saison Pluvieuse |  |  | Saison Sèche |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Valeurs Moyennes | Classes | IPO | Valeurs Moyennes | Classes | IPO |
| NH4+(mg/l) | 0,245 | 4 | 3,667 | 0,246 | 4 | 2,667 |
| PO43-( µg/l) | 100 | 3 |  | 490 | 2 |  |
| DBO5 (mg/l) | 11 | 4 |  | 2 ,03 | 2 |  |

Table IX: Korhogo Dam Rainy and Dry Season IPO

|  | Saison Pluvieuse |  |  | Saison Sèche |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Valeurs Moyennes | Classes | IPO | Valeurs Moyennes | Classes | IPO |
| NH4+(mg/l) | 0,9533 | 4 | 2,3333 | 0,5667 | 4 | 2,3333 |
| PO43-(mg/l) | 353,3333 | 1 |  | 350 | 1 |  |
| DBO5 (mg/l) | 26,9333 | 2 |  | 24 | 2 |  |

Table X: Ouangolodougou Dam Rainy and Dry Season IPO

|  | Rainy season |  |  | Dry season |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters | Mean | Class | IPO | Mean | Class | IPO |
| NH4+ (mg/L) | 0,49 | 4 | 3 | 0,86 | 2 | 2,66 |
| DBO5 (mg/L) | 8,2 | 3 |  | 9,4 | 3 |  |
| PO43- (µg/L) | 335 | 2 |  | 240 | 3 |  |

3.3 Identification of anthropogenic activities likely to pollute water resources

Watershed of Boundiali Dam is exclusively comprising several agricultural activities, including rice and vegetable production (see Figures 8).

Figure 8: Proportion of activities likely to pollute the water of Boundiali dam

Korhogo Dam watershed includes several identified activities such as agriculture, crafts and housing (Figure 9). Agriculture occupies 56% of this area with a high use of mineral fertilizers and phytosanitary products that have an impact on dam water. Handicrafts occupy 20% and include the informal sector (car and motorcycle garages, scrap metal, and vulcanization...). These actors use different products in the exercise of their activities, especially scrap metal workers who pile up iron on dam edge. All these wastes and residues dumped on the ground which are washed away by rainwater runoff towards the dam. The survey reveals the presence of dwellings in the watershed due to the position of the dam inside the city, some of which discharge their wastewater directly into the dam due to a lack of sanitation. Household waste is deposited close to these homes and is transported during the rains to the dam.

Figure 9: Proportion of activities likely to pollute the water of Korhogo dam

Ouangolodougou dam watershed includes several human activities including agriculture (49%), crafts (27%) and sanitation (24%) (Figure 10). Gardening takes place mainly in the dry season with the use of mineral, organic fertilizers and phytosanitary products that increase the organic and mineral pollution of the dam water. Several herds come daily to drink and leave their waste near this resource also influencing its organic pollution. The craft sector includes the informal sector (vehicules and motocycle repairation, scrap metal, and vulcanization...).

Figure 10: Proportion of activities likely to pollute the water of Ouangolodougou dam

4. CONCLUSION

This study shows that Boundiali, Korhogo and Ouangolodougou dams are characterized by a low temperature compared to WHO standard and a high content of BOD₅, COD, NH₄⁺, pesticides (POP and POH), and metals (iron and arsenic). The active presence of these parameters in water strongly influences their qualities. The assessment of quality and pollution has allowed to qualify these resources as very poor quality in rainy periods and as non-potable in dry periods. This state of deterioration is directly linked to human actions probably economic activities and households. Household waste, municipal wastewater, income-generating activities that take place within watersheds are the real sources of quality degradation and pollution. The IPOs and WQIs showed that water pollution from Korhogo dam is not due to a source that varies according to seasons, unlike that from Boundiali and Ouangolodougou dams which is caused by agriculture. Agriculture's contribution to this degradation is linked to crops protection products and fertilizers that farmers use. In order to reduce pollution and maintain water quality of these dams, it is advisable to:

- Protect them by setting up and respecting the various protection perimeters;

- Raise awareness among farmers for the rational use of fertilizer products;

- Promote the proper management of household waste through the creation of landfills and the recycling of household waste;

- Collect all municipal wastewater for possible treatment before discharge.

Thus, the study perspectives that emerge are:

Strengthen the study by taking into account several parameters, including bacteriological parameters to better characterize the dam. Conduct a hydrobiological study to assess biological quality.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

1.	1. Adingra, A. K. M., Kouassi, A. M., & Eblin, S. G. (2024). Integrated water resources management for sustainable development: A case study of Ivorian watersheds. Journal of Environmental Management, 351, 119832. https://doi.org/10.1016/j.jenvman.2023.119832

2.	Bamba, I., Savané, I., & Gnakri, D. (2023). Impacts of climate variability and land use change on surface water quality in the Bandama watershed, Côte d'Ivoire. Environmental Monitoring and Assessment, 195(2), 314. https://doi.org/10.1007/s10661-022-10872-5

3.	Koné, M., Paturel, J. E., & Oulare, S. (2024). Assessing the ecological health of tropical reservoirs: Phytoplankton community structure as an indicator of eutrophication in Côte d'Ivoire. Lakes & Reservoirs: Science, Policy and Management for Sustainable Use, 29(1), e12455. https://doi.org/10.1111/lre.12455

4.	Kouamé, K. M., Jourda, J. P., Saley, M. B., Kouamé, K. F., & Koffi, K. V. (2023). Assessment of the future water resources for the drinking water supply of Abidjan (Côte d'Ivoire) using the SWAT model. Environmental Science and Pollution Research, 30, 10825–10846. https://doi.org/10.1007/s11356-022-22826-y

5.	Ministry of Health, Public Hygiene and Universal Health Coverage. (2024) Annual Report on the Health Status of Populations 2023. Abidjan, Ivory Coast. (Report No. MSA-2024-01)

6.	N'Guessan, Y. M., Ahoussi, K. E., Koffi, K. V., Soro, G., & Biémi, J. (2024). Assessment of surface water quality and pollution in a tropical African urban river using multivariate statistical analyses and the Water Quality Index (WQI): A case study of the Djibi River (Abidjan, Côte d'Ivoire). Journal of Hydrology: Regional Studies, 52, 101687. https://doi.org/10.1016/j.ejrh.2024.101687

7.	Nations Unies. (2015). 159/5000 Resolution adopted by the General Assembly on 25 September 2015. Transforming our world: the 2030 Agenda for Sustainable Development. A/RES/70/1.

8.	SODECI. (2023). Activity and Sustainable Development Report 2022. Water Distribution Company of Côte d'Ivoire. (Rapport No. SODECI-RADD-2022).

9.	Soro, G. M., Ahoussi, K. E., Koffi, K. V., & Biémi, J. (2024). Hydrochemistry and quality assessment of reservoir water in agricultural watersheds: A case study from northern Côte d'Ivoire. Applied Water Science, 14(3), 58. https://doi.org/10.1007/s13201-024-02115-4

10.	UN-Water. (2023). Summary Progress Update 2023: SDG 6 – water and sanitation for all. United Nations.

11.	WHO/UNICEF Joint Monitoring Programme (JMP). (2024). *Progress on WASH in health care facilities 2000-2023: Special focus on climate resilience*. World Health Organization (WHO) and the United Nations Children’s Fund (UNICEF). (Rapport No. ISBN 978-92-4-008073-8).

12.	World Bank. (2023). Côte d’Ivoire - Urban Water Supply Strengthening Project: Environmental and Social Management Framework (ESMF). World Bank Group. (Rapport No. ESMF-PREMU-2023).

13.	Ministry of Water and Forestry (MEF). (2023) National Report on the State of Water Resources and Sanitation 2022 (Report No. MEF/DGE-REA-2023-01). Abidjan, Côte d'Ivoire: Republic of Côte d'Ivoire.

14.	Organisation mondiale de la Santé (OMS). (2017). Directives de qualité pour l'eau de boisson. Quatrième édition. https://apps.who.int/iris/handle/10665/254637

15.	Aher, S. B., Kuchewar, S. V., & Deshpande, S. M. (2016). Assessment of Water Quality Index (WQI) for the Groundwater in the Coastal Area of Deogad District, Maharashtra, India. International Journal of Current Research, 8(11), 42433-42437. https://www.journalcra.com/article/assessment-water-quality-index-wqi-groundwater-coastal-area-deogad-district-maharashtra-india

16.	Leclercq, L. (2001). Assessment of surface water quality using biotic and physico-chemical indices: application to Walloon watercourses (Belgium). Biotechnology, Agronomy, Society and Environment, 5(1), 45-59. https://popups.uliege.be/1780-4507/index.php?id=15593

17.	Dawson, S., Manderson, L., & Tallo, V. L. (1993). A manual for the use of focus groups. International Nutrition Foundation for Developing Countries (INFDC). https://idl-bnc-idrc.dspacedirect.org/handle/10625/11963

18.	Ahoussi, K. E., Koffi, Y. B., Kouassi, A. M., Soro, G., & Biémi, J. (2019). Hydrogeochemistry of the water of the fissure layers of the Daloa region (West-Central Côte d'Ivoire): case of the villages of Gbada and Lobo. International Journal of Biological and Chemical Sciences, 13(5), 2189-2206. DOI: https://doi.org/10.4314/ijbcs.v13i5.31

19.	Abboudi, M., Makhoukh, M., Sbaa, M., & Claeys, P. (2014). Physico-chemical characterization of the surface waters of the Moulouya watershed (Eastern Morocco). Journal of Materials and Environmental Science, 5(S1), 2345-2352.

20.	Beaux, J.-F. (1998). Water treatment: environmental aspects and technologies. Paris: Tec & Doc Lavoisier. (General reference book on water treatment)21.	Mehounou, M. G., Houssou, C. S., & Mama, D. (2016). Assessment of groundwater and surface water quality in the municipality of Aplahoué (Benin): case of cotton production areas. International Journal of Biological and Chemical Sciences, 10(5), 2257-2272. DOI: https://doi.org/10.4314/ijbcs.v10i5.2922.	Ahoussi, K. E., Koffi, Y. B., Oga, M. S., Soro, G., & Biémi, J. (2013). Caractérisation hydrochimique et microbiologique des eaux de source de la région du Haut-Sassandra (Centre-Ouest de la Côte d’Ivoire) : cas du village de Mangouin-Yrongouin. Journal of Applied Biosciences, 63, 4717–4734. DOI : https://doi.org/10.4314/jab.v63i0.85662

23.	Mohod, C. V., & Dhote, J. (2013). Review of heavy metals in drinking water and their effect on human health. International Journal of Innovative Research in Science, Engineering and Technology, 2(7), 2992-2996.

24.	Serge, M. K., & Ernest, K. K. (2020). Seasonal variation of water quality in tropical reservoirs: A case study of the Ayamé 2 Dam (Côte d'Ivoire). Journal of Water Resource and Protection, 12(5), 414-430. DOI : https://doi.org/10.4236/jwarp.2020.125025

25.	Taybi, A. F., Mabrouki, Y., & Berrahou, A. (2016). Assessment of surface water quality in the Moulouya watershed (Morocco). Journal of Materials and Environmental Science, 7(8), 2799-2812.

26.	Charles Vital, D., Gbombele, S., & Kouame, K. F. (2018). Assessment of seasonal variation of water quality in the Buyo Reservoir (Côte d'Ivoire). International Journal of Engineering and Applied Sciences, 5(3), 61-73.

27.	Talhaoui, A., El Hmaidi, A., & Jaddi, H. (2020). Évaluation de la qualité des eaux de surface du bassin versant de l'Oum Er Rbia (Maroc) par les indices de pollution organique. European Scientific Journal, 16(20), 185-201. DOI : https://doi.org/10.19044/esj.2020.v16n20p185

28.	El Hmaidi, A., Talhaoui, A., & Jaddi, H. (2020). Seasonal variation of water quality in the Oum Er Rbia watershed using multivariate statistical analysis. Journal of Ecological Engineering, 21(5), 1-12. DOI : https://doi.org/10.12911/22998993/122175

29.	Benbouih, H., Nassali, H., & Rami, M. (2005). 182/5000

Assessment of the organic pollution of the waters of the Sebou wadi downstream of the city of Fez (Morocco). Bulletin of the Scientific Institute, Rabat, Life Sciences section, 27, 69-75.

30. Yidana, S. M., & Yidana, A. (2010). Assessing water quality using water quality index and

multivariate analysis. Environmental Earth Sciences, 59 (7), 1461-1473.

32. Chahboune, N., Mehdi, M., Abidi, M., & Douira, A. (2012). Environmental impact and assessment of water quality by chemical and biological methods "Diatoms". International Journal of Biological and Chemical Sciences, 6 (6), 8024 8033. Available:https://www.google.com/url?sa=t&source=web&rct=j&opi=89978449&url=https://www.ajol.info/index.php/ijbcs/article/view/88506/78117&ved=2ahUKEwiOhpLOtMyJAxVfQ6QEHcWHAiAQFnoECBUQAw&usg=AOvVaw3zXD6nwa7aNmYOp1INP8vB

33. KOUAKOU Yao Salomon, SEYHI Brahima and KONAN N’guessan Maxime (2024). Impacts of Anthropogenic Activities on Water Quality of Ouangolodougou Dam, Côte d'Ivoire. International Journal of Environment and Climate Change Volume 14, Issue 12, Page 64-81

34. Makhoukh, M., Sbaa, M., Berrahou, A., & Van Clooster, M. (2011). Contribution to the physicochemical study of the surface waters of the Moulouya wadi (East Morocco). Environmental Science Studies ,21.

35. Akil, A., Hassan, T., Fatima, E. H., Lahcen, B., & Abderrahim, L. (2014). Study of the physicochemical quality and metallic contamination of the surface waters of the Guigou watershed, Morocco. Environmental S cience, 10 (23), 84-94. Available:https://core.ac.uk/download/pdf/236415782.pdf

36. Kapepula, L., Mateso, L.L., Shekani, A., Muyisa,S., Ndikumana, T., & Van der B. (2015). Evaluation de la charge polluante des rivières des eaux usées ménagères et pluviales dans la ville de Bukavu, République Démocratique du Congo. Afrique Science, 11 (2), 195-204. Available:https://www.ajol.info/index.php/afsci/article/view/118488/108021

37. KrućFijałkowska, R., Dragon, K., Drożdżyński ,D., & Górski, J. (2022). Seasonal variation

of pesticides in surface water and drinking water wells in the annual cycle in western Poland, and potential health risk assessment. Scientific Reports, 12, 33-17. Available:https://doi.org/10.1038/s41598022 07385-z

38. Mehounou, J. P., Josse, R. G., Dossou Yovo, P., Senou, S. F., & Toklo, R. M. (2016). Physico chemical and microbiological characterization of groundwater and surface water in the Aplahoué cotton production area. Journal of Applied Biosciences, 103, 9841-9853.

39. Mohod, C. V., & Dhote, J. (2013). Review of the heavy metals in drinking water and their effect on human health. International Journal of Innovative Research in Science, 2, 2992-2996.

40. Hama Aziz, K. H., Mustafa, F. S., Omer, K. M., Hama, S., Hamarawf, R. F., & Rahman, K. O. (2023). Heavy metal pollution in the aquatic environment: Efficient and low-cost removal approaches to eliminate their toxicity: A review. RSC Advances, 13(26), 17595-17610. DOI: 10.1039/d3ra00723e

41. Xu, F., Wang, Y., Chen, X., Liang, L., Zhang, Y., Zhang, F., & Zhang, T. (2022). Assessing the environmental risk and mobility of cobalt in sediment near nonferrous metal mines with risk assessment indexes and the diffusive gradients in thin films (DGT) technique. Environmental Research, 212(C), 113456. Available:https://doi.org/10.1016/j.envres.2022.113456

42. Singh, A., Sharma, A., Verma, K. R., Chopade, L. R., Pandit, P. P., Nagar, V., et al. (2022). Heavy metal contamination of water and their toxic effect on living organisms. In The Toxicity of Environmental Pollutants(pp. 1-26). IntechOpen.   Available:https://doi.org/10.5772/intechopen.105075; http://dx.doi.org/10.5772/intechopen.105075

43. Singh, V., Ahmed, G., Vedika, S., Kumar, P., Sanjay, K., Chaturvedi, S. N., Rai, S. N., et al. (2024). Toxic heavy metal ions contamination in water and their sustainable reduction by eco-friendly methods: Isotherms, thermodynamics and kinetics study. Scientific Reports, 14(1), 7595. Available:https://doi.org/10.1038/s41598-024-58061-3

44. Zhang, P., Yang, M., Lan, J., Huang, Y., Zhang, J., Huang, S., Yang, Y., & Ru, J. (2023). Water quality degradation due to heavy metal contamination: Health impacts and eco-friendly approaches for heavy metal remediation. Toxics, 11(10), 828.

DOI: 10.3390/toxics11100828

45. Lakhili, F., Benabdelhadi, M., Bouderka, N., Lahrach, H., & Lahrach, A. (2015). Study of the physicochemical quality and metallic contamination of the surface waters of the Beht watershed (Morocco). European Scientific Journal, 11(11), 132-147.

Available:https://eujournal.org/index.php/esj/article/view/5443

46. Oltramare, C., Weiss, F. T., Staudacher, P., Kibirango, O., Atuhaire, A., & Stamm, C. (2023). Pesticides monitoring in surface water of a subsistence agricultural catchment in Uganda using passive samplers. Environmental Science and Pollution Research International, 30(4), 10312-10328. Available:https://doi.org/10.1007/s11356-022-22717-2

47. De Souza, R. M., Seibert,D., Quesada, H. B., Bassetti, F. J., Fagundes-Klen, R. F., Bergamasco, R. (2020). Occurrence, impacts and general aspects of pesticides in surface water: A review. Process      Safety and Environmental Protection, 135,22-37.

Available:https://doi.org/10.1016/j.psep.2019.12.035

48. Vital, S. N. C., Robert, N., & Benot, N. M. (2018). Pollution of water for human consumption and short-term health risks: Case of the Menoua watershed (West Cameroon). European Scientific Journal, 14(3), 96-117.

49. Yao, K. S., & Ahoussi, K. E. (2020). Physico-chemical characterization of surface waters in a mining environment in west-central Côte d’Ivoire: The case of the department of Divo. European Scientific Journal, 16(12), 293-315.

DOI:https://doi.org/10.19044/esj.2020.v16n12p293

50. Kim, Y. E., Jeon, D., Lee, H., Huh, Y., Lee, S., Kim, J. G., et al. (2023). Revealing the extent of pesticide runoff to the surface water in agricultural watersheds. Water, 15(22), 3984.

Available:https://doi.org/10.3390/w15223984

51. El Hmaidi, A., Talhaoui, A., Manssouri, I., Jaddi, H., & Ousmana, H. (2020). Contribution of the pollution index and GIS in the assessment of the physico-chemical quality of the surface waters of Moulouya River (NE, Morocco). La Coille Blanche, 3,45-54.

52. Talhaoui, A., El Hmaidi, A., Jaddi, H., Ousmana, H., & Manssouri, I. (2020). Calculation of the water quality index (WQI) for the assessment of the physico-chemical quality of surface waters of L'Oued Moulouya (NE, Morocco). European Scientific Journal, 16(2), 64-85.

53. Benbouih, H., Nassali, H., Leblans, M., & Srhiri, A. (2005). Trace metal contamination of sediments in Lake Fouarat, Morocco. Afrique Sciences, 1(1), 109-125.