A Study on Engineering Properties on Apple Ber

ABSTRACT

The study was conducted from August 2023 to September 2024 at the Department of Agricultural Engineering, College of Horticulture, Munirabad, Karnataka, India, with the objective of determining the engineering properties of apple ber (Ziziphus mauritiana). These properties are essential for the design and development of efficient material handling and food processing equipment. Fresh apple ber fruits were analyzed to evaluate their physical and mechanical characteristics, which serve as a crucial database for post-harvest operations such as grading, conveying, drying, and storage. The moisture content of the fruits ranged between 82–85% on a wet basis. The mean dimensions were recorded as 53 mm in length, 43 mm in width, and 22 mm in thickness, with a geometric mean diameter of 43.72 mm and sphericity of 0.67%. The average pulp-to-stone ratio was 8.00, indicating a high edible portion suitable for processing. The bulk density and true density were observed as 0.932 g cc⁻¹ and 1.04 g cc⁻¹, respectively, while the calculated porosity was about 12%. The angle of repose of the fruits ranged from 27° to 32°, reflecting their flow behavior during handling and storage. These engineering parameters provide valuable insights into the design of processing and handling equipment for apple ber, ensuring minimal mechanical damage and efficient material flow. The findings serve as a baseline for further research and equipment standardization in ber processing and post-harvest technology development.

Keywords: Apple Ber, Bulk Density, Engineering Properties, Moisture, Processing,

- INTRODUCTION

Ber (Ziziphus mauritiana Lamk.), commonly known as Indian jujube, belongs to the genus Ziziphus of the family Rhamnaceae and order Rhamnales. It is widely distributed across tropical and subtropical regions of the northern hemisphere. The family Rhamnaceae includes over 50 genera and more than 600 species (O’Brien et al., 2023), among which Z. mauritiana and Z. spina-christi are the most economically important due to their wide cultivation and diverse uses (El Maaiden et al., 2020; Al-Saif et al., 2021). In India, Z. mauritiana thrives in semi-arid and arid zones, particularly in the northwest and southern regions (Sebastian and Bhandari, 1990; O’Brien et al., 2023). Earlier wild varieties were small with distinct flavour, but breeding advancements have produced improved hybrids with desirable traits like higher pulp-to-seed ratio, larger fruit size, and uniformity (Abdel-Sattar et al., 2021). Among these, the “apple ber” has gained commercial importance for its high yield, low input needs, and adaptability to dry climates (Kumar et al., 2023). In India, two main types are cultivated the red “Kashmiri apple ber” and the “green apple ber,” the latter being preferred for its attractive colour and superior taste (Abdel-Sattar et al., 2024; Al-Saif et al., 2021).

Nutritionally, ber is rich in essential amino acids such as asparagine, arginine, glycine, glutamic acid, and serine, and contains higher vitamin C and phosphorus levels than apples or oranges. Ziziphus mauritiana has long been used in traditional medicine, and modern studies have identified bioactive compounds like flavonoids, terpenoids, alkaloids (e.g., berberine), sterols, and phenolics responsible for its antioxidant, antimicrobial, anti-inflammatory, hepatoprotective, and antidiarrheal properties (Prakash et al., 2021; Alsayari et al., 2021). Extracts from its leaves and roots show strong inhibitory effects against E. coli, S. aureus, and S. pyogenes, while fruit extracts demonstrate notable anti-inflammatory potential. Agronomically, ber thrives in marginal soils with low water, making it a valuable crop for small farmers. Its adaptability and nutritional richness have earned it the title “poor man’s apple,” offering both health and economic benefits at minimal production cost (Kumar et al., 2023).

Understanding the engineering properties of Ziziphus mauritiana is essential for designing efficient post-harvest and processing systems. Operations such as conveying, drying, juicing, and packaging depend on physical, thermal, and mechanical properties like size, shape, density, porosity, moisture content, and mechanical strength (Sharif, 2022). These parameters influence equipment design for sorting, grading, storage, and juice extraction. Since ber’s thin skin and high moisture make it prone to bruising, such knowledge helps reduce post-harvest losses. Studying its thermal and rheological behavior supports the development of effective dryers, heat exchangers, and packaging systems that maintain quality. A comprehensive database of engineering characteristics enables process optimization, automation, and standardized equipment design, especially for small and medium-scale enterprises. Thus, analyzing these properties is vital for sustainable utilization, technological innovation, and efficient post-harvest management (Patel et al., 2022; Kumar et al., 2023).

MATERIALS AND METHODS

The study was conducted during August 2023 - September 2024 at the Department. of Agricultural Engineering, College of Horticulture, Munirabad, Karnataka, India.

2.1 Physical Parameter of Apple ber:

2.1.1 Fruit selection

Green ber apples were collected from local farmers in Karnataka and were chosen for analysis of engineering properties. The samples were completely washed to remove foreign bodies and dried before the analysis was carried out.

2.1.2 Weight of the fruit

The same samples were weighed using an electronic weighing balance, and the mean value was recorded for analysis.

2.1.3 Size and shape

Randomly, ten samples were chosen and linear measurements such as length, diameter and thickness were measured with the help of a digital Vernier calliper. From the average of axial dimensions, the geometric mean diameter Dg in mm was determined by using the following formula (Joshi et al., 1993):

Dg=(abc)1/3                          -1

Where:

a = length was the dimension along the longest axis in mm;

b = width was dimension along the longest axis perpendicular to a in mm;

c = thickness, was the dimension along the longest axis perpendicular to both a and b in mm.

The sphericity was determined using (Mohsenin, 1970):

Sp=      (abc)1/3a                          -2

2.1.4 Pulp-Stone Ratio (Rps)

In order to calculate the pulp-to-stone ratio of fruit samples weight was measured in different ways based on the requirements for the calculations. Whole fruit weight, weight of flesh and weight of stone were measured, and the Pulp-Stone Ratio was calculated using the following formula.

Pulp to stone ratio Rps=Wf-WsWs               -3

Where,

Wf = Weight of fruit (gm).

Ws = Weight of stone (gm).

Pulp-stone ratio forms the basis for any processing industry to finalize the quantity of fruit required to get the targeted volume of fruit juice or the weight of flesh for processing.

2.1.5 Moisture Content (MC)

Moisture content was the basic parameter for all the properties studied. The fruits skins were removed manually. These were sliced into 1 mm thick slices and placed on perforated metal trays and dried at 55 °C in hot air oven till a constant weight was attained. The observations were recorded, and it was calculated by the following equation (Shoba et al., 2017).

Mwb=W1-W2W1                - 4

Where,

Mwb -Moisture content, per cent wet basis

W1 - Initial weight of the sample, g

W2 - Final weight of the sample, g

2.2 Bulk Density

It indicated the weight of the sample (unit volume) -1, and variation of densities tends to cause the undesirable stratification as bags or bins were filled. The porous nature of some agricultural materials presented a number of problems in volume and density measurement. It was determined by filling fruits of known volume of a steel container and it was weighed. The ratio between the mass and volume (10 X 10 X 10 cm3) was calculated as the bulk density. The average value of bulk density was reported by repeating the experiments three times.

Bulk Density kgm3=weight of the sample   KgVolume of the sample  m3                         -5

2.3 True density

The true density of ber was calculated using the liquid displacement method, whereby the mass of fruits was measured and the fruits were then poured into the measuring cylinder containing toluene. The volume of toluene displaced was noted and the true density was calculated.

2.4 Porosity

The porosity of ber fruit was computed from the value of bulk density and true density using a relationship.

Porosity %=ρt-ρbρt                            -6

Where, ρt = true density in g cc-1

ρb = Bulk density in g cc-1

2.5 Specific Gravity of Fruits

Another important parameter to be calculated was the specific gravity of the fruit. The weight of the fruit was measured in two states. First, being the usual method of weighing and second, by keeping the fruit submerged under water. Fruit was pushed into water with the aid of a wire loop, which assured proper replacement of water from the container. Specific gravity was measured using the following formula.

Specific gravity=Weight of fruit in air (Wfa)weight of fruit in air Wfa-weight of fruit in water(Wfw)                                -7

2.6 Angle of Repose

It was calculated by placing the samples at the center of the horizontal platform. The platform was inclined by rotating the handle at a minimum speed until the samples began to roll. At this point, the angle of inclination of the platform was recorded. For the next sample, the platform was placed into the initial horizontal position. The experiment was repeated three times.

2.7 Coefficient of friction

The coefficients of static friction were obtained with respect to four different surfaces, namely, galvanized iron, plywood, glass surfaces and acrylic surface, by using an inclined plane apparatus as described by Rastogi et al. (2015).

μ = tan θ             -8

Where, μ = Coefficient of friction,

θ = Inclined angle (°)

3 RESULT AND DISCUSSION

3.1 Weight of the fruit

Twenty fruit samples were selected and weighed on a weighing balance with a least count of 0.01g the maximum weight of fruit was recorded to be 90 g, whereas, minimum weight was recorded to be 55 g and the average weight was 68g, with their standard deviation and coefficient of variation being 12.73 and 18.77 (Table. 1)

3.2 Size and shape

The size of ber fruit was measured from its length, width and its thickness. The range of length, width and thickness was found to be 60 to 45 mm, 47-37mm and 30-16 mm, respectively and mean value were found to be 53 mm, 43mm and 22mm, respectively. Shape describes the object in terms of a geometrical body and it was also important in calculation of heat and mass transfer designing of grading screening equipment it was also expressed in terms of its sphericity, the range of sphericity value of ber fruits found to be 0.69-0.65 (%) whereas average value was found to be 0.67 % according to Mohsenin, (1986) the shape of the ber fruit was prolate spheroid in which the polar ax was was greater than the equatorial diameter.

3.3 Pulp-stone ratio

Twenty samples were taken for this evaluation during the calculation, the range of pulp to stone ratio in apple ber fruit was observed to be 9 to 7.5 mm and the mean value observed to be 8 mm, whereas standard deviation, standard error mean and coefficient of variation were 1.60, 0.71 and 22.49, respectively. (Table. 1)

3.4 Moisture content

The moisture content of apple ber fruit was observed to be in the range of 82 to 85 % on wet basis. The physical property was very much important in storage studies, material handling concentration, evaporation, drying, freezing, designing of separation equipment and harvesting equipment etc.,

3.5 Bulk density, true density and porosity

The whole apple ber fruit Bulk density and true density was measured and found to be 0.931- 0.938  g cc-1 and  1.02 - 1.09 g cc-1 with average values of 0.932 and 1.04 g cc-1 respectively. The maximum porosity value was observed to be 14.5 % and the minimum value was observed to be 10.25 % with an average value of 12.00 %.

3.6 Angle of repose

The angle of repose of apple ber fruit was observed to be in the range of 27o to 32o with a mean value of 32o whereas standard deviation and coefficient of variation value were 1.92 and 6.25.

3.7 Coefficient of friction

The mean coefficient of friction value on four different rolling surfaces like, acrylic surface, glass, galvanized iron and wood was measured and observed to be 0.42, 0.46, 0.56 and 0.63 (Table.2) among the four surfaces the maximum coefficient of static friction was observed on plywood whereas minimum static friction was observed on acrylic surface followed by glass and galvanized iron (figure 1.). Which was mainly due to frictional properties between fruit and rolling surfaces (Gupta et.al., 2015) Similar results were reported by Patil et al. (2012) on mucuna bean and Shirkole et al. (2011) on soybean. These observations might be helpful in the design of graders, sorters, and transportation and separation equipment.

| Table 1: Engineering properties of apple ber fruit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl no | Parameters | Mean Value | Range | SD | SE±m | CV P= 0.05% |
| 1 | Moisture content (%) | 82 | 82 - 85 | 1.30 | 0.58 | 1.61 |
| 2 | Length (mm) | 53 | 45 - 60 | 5.50 | 2.24 | 10.31 |
| 3 | Width (mm) | 43 | 37 - 47 | 3.97 | 1.62 | 9.27 |
| 4 | Thickness (mm) | 22 | 16 - 30 | 5.94 | 2.42 | 27.23 |
| 5 | Weight (gm) | 68 | 55 - 90 | 5.19 | 12.73 | 18.77 |
| 6 | Geometric mean (mm) | 43.72 | 41.79 – 83.71 | 1.01 | 0.58 | 0.02 |
| 7 | Sphericity (%) | 0.67 | 0.65 - 0.69 | 0.02 | 0.01 | 3.58 |
| 8 | Pulp to stone ratio | 8.00 | 7.5 - 9 | 1.60 | 0.71 | 22.49 |
| 9 | Bulck Density (g cc-1) | 0.932 | 0.931 - 0.938 | 0.02 | 0.01 | 2.66 |
| 10 | True density (g cc-1) | 1.04 | 1.02 - 1.09 | 0.01 | 0.00 | 0.81 |
| 11 | Porosity (%) | 12.00 | 10.25 - 14.5 | 1.58 | 0.71 | 11.29 |
| 12 | Specific Gravity | 0.96 | 0.92 - 0.98 | 0.02 | 0.11 | 2.52 |
| 13 | Angle of repose (o) | 32 | 29 - 32 | 1.92 | 0.86 | 6.25 |

| Table.2: Coefficient of friction on different rolling surfaces. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rolling Surface material |  |  |  |  |
| Parameter | Acrylic surface | Glass | Galvanized iron | Plywood |
| Average | 0.42 | 0.46 | 0.56 | 0.63 |
| Range | 0.45-0.40 | 0.48-4.00 | 0.59-0.53 | 0.67-0.61 |
| SD | 0.019 | 0.34 | 0.023 | 0.024 |
| CV P= 0.05% | 4.494* | 7.702* | 4.015** | 3.78* |

Figure-1: The coefficient of friction value on different rolling surfaces.

- CONCLUSION

The study concluded that the engineering properties of apple ber, including size, shape, sphericity, moisture content, bulk density, true density, porosity, and angle of repose, are vital for designing efficient post-harvest handling and processing equipment. These parameters influence material behaviour during operations such as drying, storage, and transportation. The findings provide a valuable database for researchers and industries to optimize equipment design, ensuring reduced losses, improved processing efficiency, and better-quality management of apple ber fruits.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

7. REFERENCES

Abdel-Sattar M., El-Gendy A. and El-Kenawy M. (2024). Genetic diversity and relationships among Indian jujube cultivars: Morphometric and molecular marker approaches. Diversity, 16(6), 313.

Abdel-Sattar M., El-Sayed A. and El-Beltagi H. (2021). Fruit properties during the harvest period of eleven Indian jujube (Ziziphus mauritiana Lamk.) cultivars. Food Science & Nutrition, 9(4), 2001–2012.

Al-Saif A. M., Al-Qahtani M. M. and Al-Khalifa K. S. (2021). Identification and characterization of Indian jujube (Ziziphus mauritiana) varieties cultivated in Saudi Arabia. Journal of Horticultural Science & Biotechnology, 96(3), 321–331.

Alsayari A., Al-Numair K. S., Al-Johani H. A., Alqahtani A. M., Alotaibi M. F., Almuqati N. I. and Alenezi T. M. (2021). Genus Ziziphus for the treatment of chronic inflammatory diseases: A review. Frontiers in Pharmacology, 12, 678-999.

El Maaiden E., El Kharrassi Y., Qarah N. A., Essamadi A., Moustaid K. and El Kebbaj M. S. (2020). Genus Ziziphus: A comprehensive review on ethnopharmacological, phytochemical and pharmacological properties. Journal of Ethnopharmacology, 259, 112950.

Gupta S. V., Wankhade V.R., Patil B.N. and Nimkar P.M. (2015). Physico-mechanical properties of sapota (Achras sapota L.). Journal of Applied Horticulture, 17(3), 225–229.

Joshi P. S. and Dwivedi I. H. (1993). Naked singularities in spherically symmetric inhomogeneous Tolman–Bondi dust cloud collapse. Physical Review D, 47(12), 5357–5369.

Kumar R., Singh S., Patel J. and Meena K. (2023). Ber (Ziziphus mauritiana) as a sustainable fruit crop for arid ecosystem: Nutritional and economic importance. International Journal of Bioresource and Stress Management, 14(2), 225–231.

Mohsenin N. N. (1970). Physical properties of plant and animal materials. New York, NY: Gordon and Breach Science Publishers.

Mohsenin N. N. (1986). Physical Properties of Plant and Animal Materials, 2nd Edition. Gordon and Breach Science Publishers, New York, USA.

O’Brien C. J., Jarrad F. and  Raghu S. (2023). Chinee apple (Ziziphus mauritiana): A comprehensive review of its distribution, cultivation, and uses. Plants, 12(18), 3213.

Patel G. R., Desai P. R., Gamit P. K. and Bhatt R. S. (2022). Physico-textural properties and mass modelling of Indian jujube (Ziziphus mauritiana L.). Fruits, 77(2), 89–98.

Patil B. N., Nimkar P. M., Kunghadkar A. A. and Wasekar M. A. (2012). Effect of moisture content on some properties of mucuna bean. Journal of Soils and Crops, 22(2), 404–409.

Prakash O., Usmani S., Singh R., Singh N., Gupta A. and Ved A. (2021). A panoramic view on phytochemical, nutritional, and therapeutic attributes of Ziziphus mauritiana Lam.: A comprehensive review. Phytotherapy Research, 35(1): 63–77.

Rastogi S., Borgo B., Pazdernik N., Fox P., Mardis E. R., Kohara Y., Havranek J. and Schedl T. (2015). Caenorhabditis elegans glp‐4 encodes a valyl aminoacyl tRNA synthetase. G3, 5(12), 2719–2728.

Sebastian M.K. and Bhandari M.M. (1990). Edible wild plants of the forest areas of Rajasthan, India. Journal of Economic and Taxonomic Botany, 14: 689–694.

Sharif, M. 2022. A comprehensive investigation of novel ber (Ziziphus spp.) .Physico-chemical and processing suitability. Journal of Biological & Medical Research, 12(1), 45–58.

Shoba H., Rajeshwari N. and Nagaraja G. (2017). A study on physico-mechanical properties of onion varieties under Koppal District (Karnataka). Current Agriculture Research Journal, 3(5), 381–386.

Shirkole S. S., Nimkar P. M. and Madwe D. S. (2011). Moisture-dependent physical properties of soybean. International Journal of Engineering Science and Technology, 3(5), 3814–3822.