ENERGY UTILIZATION PATTERN IN DRYLAND PRODUCTION SYSTEMS OF COTTON AND MAIZE

Abstract

The energy use pattern for unit operations of crop production varies under different agro climatic zones and across various farm categories. This study was conducted to determine the energy utilization pattern for cotton and maize crops for different farm size holdings under dryland conditions. Medium size farm holdings were observed as highest energy utilization for crop production and lowest utilization of energy was observed in case of small farm holdings for both the crops, due to more use of power sources and inputs. Among field operations, land preparation consumed maximum energy across all categories of farmers and fertilizer was observed as the dominant source of input energy for cotton (55.1 to 64.1%) and maize (48.7 to 58.8%). Lowest machinery energy ratio and mechanization index values were observed in small farmers when compared with the medium and large farmers.

Introduction

In development of agricultural process over a period of time, energy played a key role in Indian agriculture during which, During the year 1971-72, animate power dominated with 69% share in total available farm power for agricultural operations, whereas, mechanical power dominated with 95% share in total farm power availability of 3.04 kW/ha in 2021-22 (Mehta et al., 2023). Energy use patterns of crops differ with the type of machines available and extent of farm operations, irrigation modalities and inputs uses. Farm operations under various crops consume 37-62% of total energy in the country (Dipankar, 2006). The structure of energy use pattern in Indian agriculture has experienced a marked shift from animate to mechanical sources since four decades due to introduction of various types of machines.

Cotton (Gossypium sp.) and maize (Zea mays L.) are two major important commercial crops playing a key role in farmer’s economy of dryland zones of Telangana State. Over few years the yields of cotton and maize crops are decreasing, the reasons attributed to decrease in yields of these two crops are, inadequate inputs use, untimely field operations, lack of irrigation (70% area under rainfed conditions) and inefficient other crop production technologies. In dryland areas, cotton and maize farmers still use traditional farm implements that have low field capacity and demand more power. Several labor intensive field operations like planting, weeding and harvesting which could be semi-mechanized/mechanized are still being carried manually and labor shortage is frequent in peak seasons. Any delay in timely completion of these operations leads to yield loss. The energy use pattern for unit production of crop has varied under different agro-climatic zones. The use of energy in crop production depends on the availability of energy sources in particular region and also on the capacity of the farmers (Kalbande and More, 2008). Hence, the investigation on energy use pattern for crop production operations was carried out under different categories.

Materials and Methods

A field survey was conducted to collect data and analyze the source and operation wise direct and indirect energy use in cotton and maize production systems. Accordingly, a survey was conducted in selected cluster of villages, which represented the predominant cultivation practices of cotton and maize. Three categories of farmers were selected for survey purpose based on their farm holding size. The farm categories were small farmers (1 to 2 ha), medium farmers (4 to 10 ha) and large farmers (10 ha and above) as studied by Sanjeeva Reddy et al., (2009). Under each category 10 farmers were selected randomly in each cluster for each crop. Overall 60 farmers were selected for face to face interview for each crop to collect the relevant data which needed for the study.

Energy calculation and Analysis

The energy use from direct energy sources (Human, animal and mechanical) and indirect energy sources (FYM, seed, fertilizers and chemicals) were quantified. The Human, Animal and Mechanical energy input (MJ/ha) at every stage in production process were estimated by the following equation (Vivek Khambalkar et al, 2010).

Labour × Time

Human Energy = -------------------- × LABENF

Area

Where, 	TIME		=	Operating time, h

AREA		=	Operating area, ha

LABENF	=	Labour energy factor, MJ/h

Energy for bullock (MJ/ha) = Energy co-efficient of bullock × time (h/ha)

TW×CED

Mechanical Energy = --------------- × h × RU ×AFU × PEU

UL

Where, TW	=	Total weight of the specific machine, kg

CED	=	Cumulative energy demand for machinery, MJ/kg

UL	=	Wear-out life of machinery, h

h	=	Specific working hours per run, h/ha

RU	=	Runs, number of applications in the considered field operations

AFU	=	Average fuel use for working hour, l/h

PEU 	= 	Specific energy value per liter of fuel, MJ/l

The indirect energy per unit area for other production inputs such as fertilizer, pesticides and seed was expressed as,

ED = RATE x MATENF

Energy use efficiency: The comparison of how efficiently different crops convert input energy into output energy is called energy use efficiency and defined as the ratio of output energy to input energy (Prasanna Kumar et al, 2013).

Crop yield (kg)

Energy productivity (kg/MJ) = ----------------------------

Input energy (MJ)

Output energy (MJ)

Energy ratio = --------------------------------

Input energy (MJ)

Machinery Energy Ratio (Machine Index)

The machinery energy ratio is an index which represents the fraction of the total energy inputs through the various tools and implements used in different operations for cultivation practices of a particular crop. The machinery energy ratio was determined using equation described by Adrian et al. (2007).

Me(a, i)

MER = ----------------------------

Te(a, i)

Where, MER is the ratio of the machinery energy to the total energy input

Me(a, i) is the energy input through the various machines/implements

Te(a, i)  is the total energy input (from human labour, animals, machine/hand tools, seed, and farm yard manures) for the production of crop ‘i’ in the production unit ‘a’.

Mechanization Index

Mechanization index is an index based on the ratio of the cost of use of machinery to the total animate and machinery cost for the estimation of the mechanization. The mechanization index based on the matrix of use of animate and machinery energy inputs was determined as suggested by Singh (2006) and is given below.

CEMi

MER = ----------------------------

CEHi + CEAi + CEMi

Where, 	Imi	= 	Mechanization index of the ith crop,

CEMi	= 	Cost of use of machinery ith crop,

CEHi	= 	Cost of use of Human labour ith crop

CEAi	= 	Cost of use of bullock pair ith crop

Results and Discussions

# Operation-wise and Source-wise Energy Utilization (MJ ha-1) for Cotton

# In the field operational activities, land preparation consumed maximum percentage of total operational energy across all farm categories, in which medium farmers spent highest energy 1723 MJ ha-1 followed by large farmers 1421 MJ ha-1 and small farmers 1142 MJ ha-1, which include mainly of machinery and animate power sources usage (Table 1). After land preparation, the next highest energy consuming operation was interculture and weeding, whose quantity ranged from 23.9% to 31.8% of the total operational energy. Plant protection was observed as lowest energy consuming operation in all categories of farmers in which large, medium and small farmers were recorded 97, 84 and 114 MJ ha-1, respectively.

# The source wise energy use pattern in cotton cultivation under different categories of farmers is given in Table 2. The major dominant energy spent input source was fertilizers irrespective of different categories of farmers and its contribution in total energy in selected categories of farmers ranged from 55.1 to 64.1% followed by the mechanical energy the next highest in the range of 8.9 to 17.3%. As the source wise energy uses were concerned, medium farmers used highest chemical fertilizers 9859 MJ ha-1, mechanical energy 3005 MJ ha-1 and farm yard manure (FYM) 2249 MJ ha-1. The two lowest energy expended operations reported were input seed and chemicals to control pest and diseases in cotton crop across all the categories of farmers.

# Table 1. Operation wise energy expenditure (MJ ha-1) for cotton and maize crop

| Farm category | Farm operation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Land preparation | Sowing | Interculture & weeding | Plant protection | harvesting | Stalk uprooting/ Threshing | FYM application | Total energy |
| Cotton crop |  |  |  |  |  |  |  |  |
| Large | 1421 | 146 | 1052 | 97 | 605 | 478 | 594 | 4392 |
| Medium | 1723 | 146 | 1034 | 84 | 417 | 453 | 883 | 4739 |
| Small | 1142 | 165 | 1107 | 114 | 582 | 35 | 327 | 3472 |
| Maize crop |  |  |  |  |  |  |  |  |
| Large | 1673 | 356 | 612 | 96 | 565 | 1065 | 736 | 5103 |
| Medium | 1378 | 510 | 669 | 123 | 495 | 822 | 687 | 4685 |
| Small | 1213 | 296 | 718 | 104 | 525 | 1195 | 370 | 4422 |

Table 2. Source wise energy expenditure (MJ ha-1) for cotton and maize crop

| Farm category | Source wise energy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Human energy | Bullock energy | Mechanical energy | FYM | Seed | Fertilizers | Chemicals | Total energy |
| Cotton |  |  |  |  |  |  |  |  |
| Large | 1480 | 492 | 2420 | 1623 | 68 | 8016 | 432 | 14531 |
| Medium | 1244 | 490 | 3005 | 2249 | 70 | 9859 | 381 | 17298 |
| Small | 1515 | 778 | 1179 | 808 | 72 | 8488 | 400 | 13239 |
| Maize |  |  |  |  |  |  |  |  |
| Large | 1161 | 248 | 3694 | 1919 | 272 | 7258 | 328 | 14881 |
| Medium | 1050 | 319 | 3316 | 1558 | 272 | 9774 | 324 | 16613 |
| Small | 1361 | 538 | 2524 | 1103 | 272 | 7634 | 299 | 13730 |

# Operation-wise and Source-wise Energy Utilization Pattern (MJ ha-1) for Maize

# The operations and source wise energy utilization data collected for maize was shown in Table 1 and 2. In the field operational activities, land preparation consumed maximum total operational energy across all farm categories, in which large farmers spent 1673 MJ ha-1 followed by medium farmers 1378 MJ ha-1 and small farmers 1213 MJ ha-1, which include mainly of machinery and animate power sources. After land preparation, the next highest energy consumed operation was threshing, whose quantity ranged from 17.5 to 27.0% of the total operational energy. Farm yard manure (FYM) application was observed as lowest energy consumption operation in all categories of farmers; large, medium and small farmers were recorded 736, 687 and 370 MJ ha-1, respectively.

# In maize crop also, major dominant input source reported was fertilizers irrespective of different categories of farmers and its contribution in total energy in selected categories of farmers ranged from 48.7 to 58.8 % followed by the next highest was mechanical energy in the range of 18.3 to 24.8%. As the sources wise energy uses were concerned, medium farmers used highest chemical fertilizers 9774 MJ ha-1 and large farmers highest mechanical energy 3694 MJ ha-1 and farm yard manure (FYM) 1919 MJ ha-1. The two lowest energy expenditure operations reported were input seed and chemicals to control pest and weed growth in maize crop across all the categories of farmers.

# Energy Ratio, Energy Productivity for Cotton and Maize

# The energy use efficiencies for cotton and maize production systems were presented in Table 3. For cotton, highest output energies was observed with medium farmers 66602 MJ ha-1 , when compared with large farmers 62633 MJ ha-1 and small farmers 55769 MJ  ha-1. The highest energy use efficiency was observed with large farmers about 4.31 and lowest was 3.85 in case of medium farmers. The energy productivity did not shown much variation between different categories of farmers, because of the fact that, the productivity increases with increase in energy within the optimum yield range. Although, medium farmers used a higher amount of input energy than large and small farmers, it did not result in higher energy productivity. The energy productivity ranged from 0.15 and 0.17 kg MJ-1 in cotton.

# In maize production, the highest output energy was found in medium farmers 77079 MJ ha-1 and similar trend as that of cotton was observed in maize also. The energy use efficiency was observed among three category of farmers was ranged from 4.54 to 4.85. The reason for low energy ratio for medium farmers than small and large farmers was due to less output energy, more input energy and lesser yield per unit area. The farmers of these clusters are completely dependent on the rainfall and inputs, it indicated that the yield depends upon the climatic characteristics and inputs use pattern for the crop. From the study, the yield was increased with increase in input energy and energy ratio was increased with increase in farm size and the similar observations were reported by Singh (2013) and Shahin et al., (2008).

Table 3.  Energy use efficiency in cotton and maize production system

|  | Cotton |  |  | Maize |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | large | medium | Small | large | medium | Small |
| Input energy MJ/ha | 14531 | 17298 | 13239 | 14881 | 16613 | 13730 |
| Output energy MJ/ha | 62633 | 66602 | 55769 | 72099 | 77079 | 62325 |
| Yield, kg/ha | 2506 | 2664 | 2231 | 4905 | 5244 | 4246 |
| Energy use efficiency | 4.31 | 3.85 | 4.21 | 4.85 | 4.64 | 4.54 |
| Specific energy, MJ/kg | 5.80 | 6.49 | 5.93 | 3.03 | 3.17 | 3.23 |
| Energy productivity, kg/MJ | 0.17 | 0.15 | 0.17 | 0.33 | 0.32 | 0.31 |
| Net energy return, MJ/ha | 48102 | 49304 | 42530 | 57218 | 60466 | 48595 |

# Machinery Energy Ratio and Mechanization Index

Machinery energy ratio and mechanization index were mainly depends upon the availability of power sources and their extent of utilization pattern in crop production. The values of these indices for the both crops were given in Table 4. The highest machine energy ratio was observed in medium farmers 0.43 and 0.52 for cotton and maize, respectively. The small farmers were reported as lowest machinery energy ratio in both the crops.  Overall, machinery energy ratio values decreased with decrease in land holding size, indicating that, the large and medium farmers are able to use machinery energy in the range of 16.6 to 17.3 % of input energy in cotton and 19.9 to 24.8% in maize.

The highest machine energy ratio was observed in medium farmers 0.36 and 0.39 for cotton and maize, respectively. Like machinery energy ratio, difference was observed between small farmers and other category of farmers. In both crops, small farmers were the lowest machinery energy utilized category for crop production. It indicated that, small farmers had hindrance for use of machinery in the clusters in comparison to large and medium farmers. In these clusters, no appropriate implement/self-propelled machinery was observed for various field operations except land preparation and transport of FYM, inputs, agricultural produce etc. For all other operations like sowing, intercultural & weeding and harvesting were carried out by hand labour, which also affected mechanization index. Mechanization index values indicated that, these clusters were poorly mechanization, which causes stagnation in crop productivity.

# Table 4. Machinery energy ratio (MER) and mechanization index (MI)

|  | Cotton |  |  | Maize |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | large | medium | Small | large | medium | Small |
| Machine Energy ratio | 0.40 | 0.43 | 0.26 | 0.51 | 0.52 | 0.44 |
| Mechanization index | 0.29 | 0.36 | 0.17 | 0.38 | 0.39 | 0.23 |

Conclusions

Cotton and maize crops are predominantly grown in the dry land agriculture and productivity levels of these two crops are very low due to inadequate use of inputs and inefficient crop production technologies. From the study it was observed that, the crop productivity increases as the energy input increases and energy input increased with increase in farm size. The farmers in the selected clusters were still depends on the traditional power sources and implements for majority of operations except few selected operations, which has less capacity to do operations. Though the farmers are aware of the availability of improved machinery, they are not able to use them due to cost considerations and low scale of operations for crop production in these clusters.

References

Adrian, A. R., Akira, O., Narsimha, H., Miyasaka, J., and Katsuaki, O. 2007. Mechanization index and machinery energy ratio assessment by means of an artificial neural network: a Mexican case study. Agricultural Engineering International: the CIGR Journal IX: 1-21.

Kalbande, S. R and More, G. R. 2008. Assessment of energy requirement for cultivation of Kharif and Rabi Sorghum. Karnataka Journal of Agricultural Science. 21(3): 416-420.

Mehta, C.R., Bangale, Rashmi & Chandel, N. S. and Kumar, Mohit. (2024). Farm Mechanization in India: Status and Way Forward. Agricultural Mechanization in Asia, Africa and Latin America. 54(2): 75-88.

Prasanna Kumar, P. S., Hundre, S., Ranjith Kumar, P. S and Hugar, L. B. 2013. Energy use pattern in cotton cultivation: An economic analysis. Research Journal of Agricultural Sciences. 4(2): 250-253.

Sanjeeva Reddy. B., Adake. R.V., Thyagaraj. C. R and Srinivas Reddy. K. 2009. Utilization pattern of power sources on productivity of groundnut and cotton dry land production systems. Journal of Agricultural Engineering. 46(4): 17-23.

Shahin, S., Jafari, A., Mobli, H., Rafiee, S and Karimi, M. 2008. Effect of farm size on energy ratio for wheat production: A case study from Ardabil province of Iran. American-Eurasian J. Agric. & Environ. Sci. 3(4): 604-608.

Singh, G. 2006. Estimation of mechanization index and its impact on production and economic factors – a case study in India. Journal of Biosystems Engineering. 93(1): 99-106.

Singh, R. S., Singh, S and Singh, S.P. 2015. Farm Power Machinery availability on Indian farms. Journal Agricultural Engineering. 39(1): 45-56.

Singh, R. S. 2013. Optimization of energy inputs for gram production under different farming systems in Madhya Pradesh. Agricultural Mechanization in Asia, Africa and Latin America. 44(1): 27-39.

Vivek Khambalkar., Jyoti Pohare., Sachin Katkhede., Dipak Bunde and Shilpa Dahatonde., 2010. Energy and economic evaluation of farm operationsin crop production. Journal of Agricultural Science. 2(4): 191-200.