Urban Green Infrastructure for Climate Change Adaptation and Mitigation: Roles, Innovations, and Challenges in Sustainable Cities

Abstract

Urban green spaces have become increasingly recognized as essential components in the adaptation and mitigation of climate change impacts within rapidly urbanizing environments. This review critically examines the multifaceted roles of urban green infrastructure-encompassing parks, urban forests, green roofs, vertical gardens, and permeable landscapes-in enhancing urban resilience to climate extremes. The paper synthesizes recent advances in the understanding of how urban vegetation and landscape interventions contribute to reducing the urban heat island effect through evapotranspiration, shading, and albedo modification, with empirical evidence demonstrating local temperature reductions of up to 4°C. Additionally, the review explores the effectiveness of green spaces in managing stormwater runoff via bio-swales, rain gardens, and permeable pavements, which collectively reduce peak flow rates and improve groundwater recharge. The carbon sequestration potential of urban green infrastructure is assessed, highlighting the capacity of mature urban forests and innovative green roof systems to capture and store atmospheric carbon. The integration of smart technologies, such as IoT-based soil moisture sensors and remote sensing for vegetation health monitoring, is discussed as a means to optimize the management and performance of urban green spaces. The review also addresses the socio-economic co-benefits, including improved air quality, enhanced public health, increased property values, and greater social cohesion. Key challenges are identified, such as fragmented governance, limited funding, and inequitable access to green infrastructure, particularly in marginalized urban communities. Case studies from global cities illustrate successful strategies for mainstreaming nature-based solutions and integrating green infrastructure into urban planning frameworks. The paper concludes by emphasizing the need for interdisciplinary collaboration, policy innovation, and community engagement to maximize the adaptive and mitigative potential of urban green spaces. This comprehensive synthesis aims to inform urban planners, policymakers, and researchers about the strategic importance of green infrastructure in fostering climate-resilient and sustainable cities.

Keywords: Green space, climate change, policy, mitigation, sustainable development

- Introduction

Cities worldwide are progressively exposed to intensified climate impacts thus advance urban climate adaptation is required. City climates are dominated by the urban heat island (UHI) (Akbari and Kolokotsa, 2016). The surface energy balance alters when natural and semi-natural land cover types become impermeable surfaces and built-up regions. Anthropogenic heat emissions and surface longwave radiation increase, solar radiation absorption and storage increase, and evapotranspiration decreases (Quan et al., 2016; Monteiro et al., 2016). The UHI may increase heat stress, air pollution, and carbon dioxide emissions, negotiating health and comfort of urban residents, especially during extreme heat events (Beniston and Diaz, 2004; Gabriel and Endlicher, 2011; Singh et al., 2020; Wong et al., 2016). Climate change and growing urbanisation are global human welfare and development challenges. Global urbanisation has increased from 13% in 1900 to 56% in 2021. According to the UN's World Bank report today, 4.4 billion inhabitants which is 56% of the world’s population live in cities. The urban population is expected to more than doubling its current size by 2050, at which point about 70% population will live in cities. Due to the increase in urban impermeable surfaces and surface material thermal characteristics, surface sealing is one of the most significant structural changes in urban regions (Yu et al., 2015). The 2022 IPCC report shows a 0.86 °C global surface temperature increase. By next two decades, the temperature will rise 1.5 °C. In most cities, excessive heat and heat stress are likely (Pörtner, 2022). To adapt to climate change with urbanisation, one must comprehend urbanization's climatic consequences. Urban vegetation regulates latent and sensible heat exchange anOked selectively absorbs and reflects incident radiation through photosynthesis, evapotranspiration, and shadowing (Oke, 2002). Greenspace reduces temperatures and creates a cool island in urban settings, cooling adjacent areas (Bowler et al., 2010). The urban green-blue system can also reduce the UHI and is linked to the urban cooling island (UCI) effect (Bowler et al., 2010; Yu et al., 2015). Trees, bushes, and grassland have the greatest UCI effect on greenspace (Kong et al., 2014). The UCI effect of greenspaces associated to plant cover size, form, structure, and configuration is still debated (Monteiro et al., 2016). Greenspace size affects studies differently due to data collection methods. Rosenfeld et al. (1998) found that even shade trees cool and save energy. As climate change continues to exert its influence on the planet, urban areas face escalating challenges in mitigating and adapting to its effects. Rising temperatures, extreme weather events, and the urban heat island effect are some of the consequences impacting cities worldwide. To combat these challenges, the integration of urban green spaces becomes paramount. This paper delves into the critical role of urban green spaces in climate change adaptation and mitigation, highlighting their benefits, challenges, and potential strategies for maximizing their impact.

2.	The Significance of Urban Green Spaces in Mitigating Climate Change

Urban green spaces play a pivotal role in mitigating the impacts of climate change by offering a range of ecological and social benefits to urban environments (James et al., 2009; Maimaitiyiming, 2014). As cities continue to expand and face the challenges of rising temperatures, air pollution, and changing weather patterns, the strategic integration of green spaces becomes increasingly crucial (Figure 1). These areas, including parks, gardens, urban forests, and green roofs, contribute to climate change mitigation efforts in several key ways.

Figure. 1.  Urban Green Space Types and Associated Benefits

2.1 	Carbon Sequestration

Carbon sequestration is the process of capturing and storing carbon dioxide (CO2) or other greenhouse gases from the atmosphere and preventing their release back into the air. This process helps in mitigating climate change by reducing the greenhouse gases concentration, particularly CO2, which is a key driver of global warming (Shahbazi and Nasab, 2016).

There are various natural and artificial methods of carbon sequestration:

2.1.1 Natural Carbon Sequestration:

Forests: Trees and vegetation naturally absorb CO2 during photosynthesis and store it as biomass.

Soils: Healthy soils can store significant amounts of carbon through the decomposition of organic matter and other processes.

Oceans: The world's oceans act as a massive carbon sink, absorbing CO2 from the atmosphere.

2.1.2 Artificial Carbon Sequestration

Carbon Capture and Storage (CCS): The CO2 emissions from power plants or industrial processes are captured before released into the atmosphere and store in underground geological formations, like gas reservoirs or deep saline aquifers and depleted oil.

Afforestation and Reforestation: Planting new forests (afforestation) or replanting trees in areas previously deforested (reforestation) can enhance carbon sequestration.

Bioenergy with Carbon Capture and Storage (BECCS): It includes growing plants biomass to generate bioenergy, capturing the CO2 emitted during energy production, and then storing it using CCS technology (Figure 2).

Figure. 2. Classification of Carbon Sequestration Techniques

Carbon sequestration in urban areas contributes to climate change mitigation and also improves the overall well-being and livability of cities by creating healthier and more resilient environments for residents. While cities are significant sources of greenhouse gas emissions due to industrial activities, transportation, and energy consumption, they also offer opportunities for carbon sequestration through various strategies: urban green spaces, urban forests and tree canopy, urban agriculture, sustainable urban planning and etc. Urban green spaces, including community gardens, parks, and urban forests, act as carbon sinks, capturing CO2 from the atmosphere and storing in underground. Trees, in particular, play an important role in carbon sequestration. Their ability to absorb carbon helps greenhouse gas emissions reduction and combat climate change. Studies on the carbon sequestration potential of urban green spaces will be explored, along with initiatives to increase tree canopy cover in urban areas (Zhang et al., 2022; Ariluoma et al., 2021; Amoatey and Sulaiman, 2020).

2.2 	Cooling Effects and Energy Reduction

The urban heat island (UHI) effect is a well-documented and widely observed phenomenon in cities worldwide. It refers to the tendency of urban areas to experience higher temperatures compared to their surrounding rural or natural environments. This temperature disparity arises due to a combination of human activities and the physical characteristics of urban landscapes. As urbanization continues to increase, the UHI effect becomes more pronounced and presents unique challenges for cities and their inhabitants. Various studies and research papers have explored the causes and impacts of the urban heat island effect in different cities. One study conducted by the U.S. Environmental Protection Agency (EPA) analyzed UHI effects in several major cities in the United States. The research found that urban areas consistently experienced higher temperatures, with the magnitude of the UHI effect varying depending on the population density, city size, and land use land cover (Lee at al., 2019). Another comprehensive study published in the journal Nature Communications examined the UHI effect in multiple cities across the globe. The researchers concluded that the urban heat island effect is a pervasive and consistent phenomenon, affecting cities of varying sizes and geographic locations. Addressing the effect of UHI become a priority for urban planners, policymakers, and researchers. Numerous studies have explored potential mitigation strategies, like implementing cool roof technologies, increasing urban green spaces, promoting energy-efficient buildings, and adopting sustainable urban planning and design practices. Urban green spaces offer cooling benefits through shade and evapotranspiration, mitigating the heat island effect. As a result, the demand for energy-intensive cooling systems decreases, leading to reduced energy consumption and associated greenhouse gas emissions.

2.3 	Air Quality Improvement

Air pollution is a significant concern in urban environments and contributes to climate change (Schmitz et al., 2018). Urban green spaces act as natural filters thus helps in improving air quality by removing pollutants and particulate matter from the air (Kumar et al., 2019). Here are several ways in which urban green spaces contribute to air quality improvement:

Air Pollutant Absorption: Plants, especially trees and shrubs, can absorb various air pollutants, such as sulfur dioxide (SO2), nitrogen dioxide (NO2), particulate matter (PM) and ozone (O3). Through a process known as phytoremediation, plants take up these pollutants through their leaves and roots, helping to reduce their concentration in the air.

Particulate Matter Filtration: Urban greenery serves as a natural filter for particulate matter, which includes tiny particles suspended in the air. The leaves and branches of plants can trap and capture these particles, preventing them from becoming airborne and contributing to respiratory and cardiovascular health issues.

Oxygen Production: Photosynthesis, the process in which carbon dioxide is converted into oxygen by plants in presence of sunlight, results in the release of oxygen into the atmosphere. The existence of greenery and trees in urban areas increases the overall oxygen content, which is crucial for human and animal respiration.

Cooling Effect: Urban green spaces offer shade and reduce the "urban heat island " effect.  The urban heat island phenomenon in which cities are warmer than nearby rural areas due to lack of vegetation and human activities. Cooler temperatures can lead to reduced emissions of air pollutants, as some pollutants are more likely to form under high temperatures.

Reduction of Ground-Level Ozone: Urban green spaces help mitigate the ground-level ozone formation, which intensifies respiratory problems. Trees release volatile organic compounds (VOCs), that react with ozone precursors and reduce ozone formation.

Microclimate Improvement: Urban green spaces create microclimates that are often more favorable than surrounding built-up areas. They can moderate temperature extremes and help maintain more stable humidity levels, which can impact the dispersion and concentration of air pollutants.

Biodiversity Promotion: A diverse range of plant species in urban green spaces can attract various insects and birds. These creatures can contribute to pollination, ecosystem health, and natural pest control, reducing the need for chemical pesticides that can contribute to air pollution.

Noise Reduction: Some urban green spaces, particularly those with dense vegetation, can act as noise barriers, absorbing and reflecting sound waves. This noise reduction can indirectly contribute to better air quality by reducing stress and promoting overall urban livability.

3. 	The Role of Urban Green Spaces in Climate Change Adaptation

Urban green spaces play important role in climate change adaptation by helping cities and their residents to cope with the changing climate. Table 1 shows detailed reviews of the impact of green spaces in urban areas.

Table 1. Summary of Recent Studies on the Role of Urban Green Spaces in Mitigating Urban Heat and Enhancing Environmental Quality

| Authors (Year) | Study Area | Method used | Findings | limitations |
| --- | --- | --- | --- | --- |
| Sinnett, 2018 | Rotterdam, Netherlands | Literature review on ecosystem services from urban trees; discussed tree characteristics and placement for maximizing benefits | Urban trees reduce air pollution and urban heat; global efforts to increase canopy cover for environmental and health benefits | Did not discuss implementation challenges or limitations of using trees for regulation |
| Reis et al., 2019 | Lisbon, Portugal | Used Landsat satellite images and remote sensing to estimate urban biomass; linear regression analyzed vegetation density vs. air temperature; kriging for temperature interpolation | Urban vegetation significantly enhances cooling and human thermal comfort; biomass higher in winter; 50 m² vegetation can reduce air temp by 1°C | Did not account for local topography/climate; satellite resolution too coarse for detailed vegetation estimation |
| Kamble et al., 2022 | Raipur, India | Used land use change detection, fragmentation and accessibility indices to assess UGS status (2010–2020) | Built-up area increased by 24.3%; UGS availability index declined; fragmentation increased from city center to outskirts | UGS distribution is uneven across the city |
| Yaşlı et al., 2023 | Global | Synthesized global evidence on urban green spaces and cooling; reviewed planning and restoration strategies | Extensive shade trees and afforestation reduce urban temperatures and urban heat island effect; strategic planting is key | No explicit discussion of challenges in implementing shade species in cities |
| Ettinger et al., 2024 | Tacoma, Washington | Measured air temperature across locations with varying tree canopy cover; analyzed probability of exceeding high temp thresholds; compared areas with 0% vs. 100% canopy cover | Tree canopy creates significant cooling (2.57°C difference); linear decrease in air temperature with more nearby tree cover | Data limited to summer 2022; lacks multi-season/multi-year trends |
| Singh et al., 2024 | Bhopal, India | Geospatial analysis using remote sensing to map and assess Urban Green Spaces (UGS) distribution (2010–2020) | UGS in Bhopal declined, became fragmented and degraded; highlights urgent need for targeted urban greening strategies | Only two time points analyzed (2010, 2020) |

3.1 	Temperature Regulation and Heat Stress Reduction

Climate change exacerbates heatwaves and heat-related illnesses in urban areas. The existence of green spaces helps in regulate temperatures, reduce heat stress, and improve overall urban livability (Turner‐Skoff and Cavender, 2019). Here are some of the key mechanisms by which green spaces help mitigate heat stress and their significance in adapting to climate change:

Shade: Trees and vegetation in green spaces provide natural shade, reducing direct exposure to sunlight and lowering ambient temperatures. This shade is particularly important in densely developed urban areas where buildings and pavements can trap heat.

Evapotranspiration: Plants release water vapor into the atmosphere through a process called evapotranspiration (Khose et al., 2022). This cools the surrounding air, similar to the cooling effect of sweat evaporating from our skin. Evapotranspiration from green spaces helps counteract the heat generated by buildings, roads, and other surfaces.

Thermal Mass: Green spaces have a lower heat-absorbing capacity compared to concrete and asphalt. As a result, they don't retain heat as much, contributing to cooler microclimates within urban areas.

Albedo Effect: The albedo effect measures the reflectivity of surface. Natural surfaces like grass and soil have higher albedo values than concrete or asphalt, meaning they absorb less heat and reflect more sunlight. Green spaces contribute to reducing the overall heat absorption in urban areas.

Cooling through Irrigation: Well-maintained green spaces often require irrigation, which can provide additional cooling benefits through the water's evaporative cooling effect.

Heat Dissipation: Green spaces act as "heat sinks" during the day, absorbing heat and gradually releasing it during the cooler nighttime hours. This moderate temperature fluctuation and can provide relief during heatwaves.

Airflow Enhancement: Vegetation can improve air circulation, allowing cooler air to flow through urban areas. This is especially important in reducing heat stagnation in areas with limited natural airflow.

Localized Cooling: Green spaces create localized cooling effects, creating pockets of cooler air that offer relief for residents and reduce the overall heat island effect.

Carbon Sequestration: Green spaces trees and plants capture and store CO2, a greenhouse gas that contributes to climate change. This mitigating effect on greenhouse gas emissions is important for overall climate change adaptation.

3.2 	Stormwater Management and Flood Prevention

Due to climate change, extreme weather events, like heavy rainfall and flood are more frequent than ever. Urban green spaces, when designed as multifunctional green infrastructure, can aid in stormwater management and flood prevention (Green et al., 2021). Here's how they are effective in managing stormwater and reducing flood risks:

Absorption and Infiltration: Green spaces, including lawns, gardens, and vegetated swales, have permeable surfaces that allow rainwater to be absorbed into the soil. This process, called infiltration, reduces the volume of stormwater flowing into drainage systems and enhance groundwater recharge.

Retention and Delay: Green spaces can retain stormwater for a period of time, allowing it to gradually infiltrate the ground or be taken up by plants. This delay in stormwater release can help reduce the peak flow rates in drainage systems during heavy rainfall events, mitigating the risk of flooding.

Water Storage: Shrubs, trees, and other vegetation in green spaces can capture and store water in their root systems and canopies. This reduces the overall volume of runoff and slows down the rate at which water enters drainage systems.

Soil Improvement: Green spaces with healthy soils can absorb more water and facilitate better infiltration. These areas act like natural sponges, helping to manage excess rainwater.

Erosion Control: The roots of plants and trees help bind soil together, preventing erosion and reducing sediment runoff into water bodies. This, in turn, helps maintain the capacity of drainage systems and reduces the risk of watercourse blockages.

Water Quality Improvement: Green spaces can filter and treat stormwater by allowing it to percolate through soil and vegetation. This natural filtration process helps remove pollutants and contaminants from the water before it reaches water bodies.

Floodplain Restoration: Some green spaces are designed as floodplains or wetlands. These areas have the capacity to temporarily hold and gradually release large amounts of stormwater, reducing downstream flood risks.

Combined Sewer Overflow Reduction: In urban areas with combined sewer systems, green spaces can help reduce the frequency of combined sewer overflows (CSOs) by absorbing and storing stormwater during heavy rainfall events, thus minimizing the release of untreated sewage into water bodies.

3.3 	Biodiversity and Ecosystem Services

Urban green spaces offer opportunities to promote biodiversity within cities. Biodiverse ecosystems contribute to climate change adaptation by increasing urban resilience and supporting ecological balance (Bush and Doyon, 2019). Preserving and enhancing biodiversity in urban areas is of paramount significance for both environmental and human well-being. Biodiversity is diversity of life forms, from plants and animals to microorganisms, and the ecosystems they form. While urbanization can lead to habitat loss and fragmentation, actively promoting and protecting biodiversity in cities offers numerous benefits:

Ecosystem Services: Biodiverse urban ecosystems provide vital ecosystem services such as air and water purification, crops pollination, soil fertility, climate regulation, and natural pest control. These services contribute to the overall health and functioning of urban environments.

Improved Air and Water Quality: Biodiverse green spaces can act as filters, removing pollutants from the air and water. Plants absorb pollutants and release oxygen, leading to better air quality, reduced heat islands, and improved overall environmental health.

Climate Change Mitigation: Urban biodiversity helps mitigate the climate change effects. Trees and vegetation capture carbon dioxide through photosynthesis, mitigating greenhouse gas emissions and aiding in temperature regulation.

Pollinator Support: Biodiverse urban environments provide habitat for pollinators like butterflies, bees and birds. Pollinators are crucial for the reproduction of various plants, including food crops, contributing to food security and ecosystem health.

Natural Pest Control: A diverse range of species in urban ecosystems can help control pests naturally, reducing the need for chemical pesticides. Predatory insects and birds can keep pest populations in check.

Resilience to Disturbances: Biodiverse ecosystems are often more resilient to disturbances including invasive species, disease outbreaks, and extreme weather events. Species diversity can enhance the capacity of ecosystems to recuperate from such disruptions.

Genetic Diversity: Urban biodiversity can preserve genetic diversity in plant and animal species, that is crucial for their long-term survival and ability to adjust to changing conditions.

4. 	Challenges and Barriers to Implementing Urban Green Spaces

Implementing urban green spaces can offer several benefits, such as improving air quality, providing recreational opportunities, enhancing biodiversity, and promoting mental well-being. However, there are several challenges and barriers that cities often face when trying to establish and maintain these spaces. Some of the key challenges include:

Limited Space and Land Availability: Urban areas have high population densities and limited available land. Finding suitable areas for green spaces can be difficult, especially in densely built-up cities where land is already in high demand for housing, commercial development, and infrastructure.

Land Costs: Even if suitable land is identified, the cost of acquiring and developing it into green spaces can be prohibitive. This is particularly true in cities where prices of real estate are high. Allocating funds for purchasing and developing land for green spaces can compete with other budgetary priorities.

Infrastructure and Development Pressure: Rapid urbanization can lead to pressure on available land for infrastructure development. As cities expand, the demand for roads, buildings, and other structures can encroach upon potential green space areas.

Community Engagement and Participation: Participation of local community is essential for successful planning and design of urban green spaces. However, achieving consensus among diverse stakeholders, addressing conflicting preferences, and ensuring that the needs of all residents are considered can be challenging.

Maintenance and Management: Once green spaces are established, ongoing maintenance is necessary to ensure they remain attractive and functional. This requires financial resources, skilled labor, and effective management strategies. Without proper maintenance, green spaces can become neglected and lose their appeal.

Bureaucratic and Regulatory Hurdles: Navigating the regulatory processes, obtaining permits, and meeting various legal requirements can be time-consuming and complex. Bureaucratic hurdles can slow down the implementation of green space projects.

Equitable Access: Ensuring equitable access to green spaces is important, as certain communities may have limited access due to factors like socio-economic disparities or geographical location. Addressing these inequalities requires deliberate planning and resource allocation.

Environmental Concerns: Introducing green spaces can have unintended ecological consequences, such as disrupting local ecosystems, introducing invasive species, or altering natural drainage patterns. Balancing environmental preservation with the creation of new green spaces is a challenge.

Funding and Resources: Financial investments is required for establishment and sustaining green spaces which includes land acquisition, infrastructure development, landscaping, and constant maintenance. Securing funding from public and private sources can be difficult, especially in times of budget constraints.

Changing Climate: Urban green spaces can be affected by climate change, including extreme weather events like floods, heatwaves, and storms. Designing resilient green spaces that can adapt to changing climate conditions is a significant challenge.

Perception of Value: Some policymakers and urban planners may not fully appreciate the value of green spaces or may prioritize other development goals over creating or preserving these areas.

To overcome these challenges, cities can adopt strategies such as conducting thorough planning and community engagement, leveraging public-private partnerships, adopting sustainable design principles, and prioritizing long-term funding for maintenance and management. The successful implementation of flexibility, innovation, and comprehensive tactic to urban planning are key to sustaining urban green spaces.

5. 	Strategies for Maximizing the Impact of Urban Green Spaces

Maximizing the impact of urban green spaces involves thoughtful planning, design, and management to ensure that these spaces effectively contribute to the well-being of the community and the environment. Here are some strategies to consider:

Integrated Planning and Design:

- Integrate green spaces into urban planning from the early stages to ensure they are well-integrated with other urban elements.

- Design green spaces to serve multiple functions, such as providing recreational areas, promoting biodiversity, and managing stormwater.

Community Engagement:

- Involve the local community in the planning and design process to ensure that green spaces meet their needs and preferences.

- Create opportunities for community members to participate in maintenance and programming activities, fostering a sense of ownership.

- Conduct awareness campaigns for communities and capacity building programs for ground workers on the practical steps that could be taken by societies, communities like terrace gardens (to add up the extra area for green cover)

Accessibility and Inclusivity:

- Design green spaces with universal accessibility in mind, ensuring they are usable by people of all ages and abilities.

- Consider the diverse needs and preferences of different cultural and socioeconomic groups when designing and programming green spaces.

Biodiversity and Native Planting:

- Incorporate native plants and habitats to support local wildlife and promote biodiversity.

- Incorporate

- Design landscapes that mimic natural ecosystems, which require less maintenance and provide greater ecological benefits.

Multifunctional Design:

- Design green spaces to serve a variety of functions, such as providing recreational areas, urban agriculture, cultural spaces, and educational opportunities.

- Incorporate amenities like seating, shade, and water features to enhance the usability and comfort of the space.

Sustainable Infrastructure:

- Implement sustainable design practices, such as permeable surfaces and rain gardens, to manage stormwater runoff and reduce the urban heat island effect.

- Utilize green infrastructure elements that contribute to water conservation and purification.

- Create awareness on insulated infrastructure and reduce the energy demand for cooling, which can further avoid pumping heat in surroundings.

Programming and Activities:

- Organize diverse programs and events in green spaces, such as fitness classes, workshops, markets, and cultural celebrations, to attract a wide range of users.

- Create spaces that can accommodate a variety of activities, from relaxation to active sports.

Education and Interpretation:

- Incorporate educational elements within green spaces to teach visitors about local ecology, sustainability, and the importance of green spaces.

- Install signage, displays, and interactive features that engage visitors and encourage learning.

- Incorporate this information in school curriculum so that the young generation are equipped with necessary knowledge for future interventions to find innovative solutions for this crisis.

Maintenance and Management:

- Develop a comprehensive maintenance plan to ensure that green spaces remain well-kept and attractive.

- Involve community members in maintenance activities through volunteer programs or partnerships with local organizations.

Public-Private Partnerships:

- Collaborate with private entities, nonprofits, and community organizations to share the responsibilities of funding, designing, and managing green spaces.

- Leverage resources from various stakeholders to enhance the impact of green space projects.

Continuous Monitoring and Adaptation:

- Regularly monitor the usage and condition of green spaces to identify areas for improvement.

- Adapt green space management strategies based on changing community needs and evolving environmental conditions.

Demonstrate Economic Value:

- Collect data on the fiscal profits of green spaces, like reduced healthcare costs and improved property values to make a convincing circumstance for continued investment.

By implementing these strategies and fostering a collaborative approach among various stakeholders, urban green spaces can accomplish their full potential in improving the quality of life for city residents while contributing positively to the environment and local communities

6.	Conclusion

Urban green spaces play a critical role in climate change adaptation and mitigation. By sequestering carbon, regulating temperatures, improving air quality, and enhancing urban resilience, these green spaces contribute significantly to combat climate change. However, challenges related to limited space, funding, and social equity must be addressed through collaborative efforts between governments, communities, and private sectors. By embracing innovative strategies and recognizing the importance of urban green spaces, cities can emerge as sustainable and climate-resilient centers, creating a positive impact on both the environment and the well-being of their inhabitants.

Reference

Akbari, H., Kolokotsa, D. (2016). Three decades of urban heat islands and mitigation technologies research. Energy Build 133, 834–842.

Amoatey, P., & Sulaiman, H. (2020). Quantifying carbon storage potential of urban plantations and landscapes in Muscat, Oman. Environment, Development and Sustainability, 22, 7969-7984.

Ariluoma, M., Ottelin, J., Hautamäki, R., Tuhkanen, E. M., & Mänttäri, M. (2021). Carbon sequestration and storage potential of urban green in residential yards: A case study from Helsinki. Urban Forestry & Urban Greening, 57, 126939.

Beniston, M., Diaz, H.F. (2004). The 2003 heat wave as an example of summers in a greenhouse climate? Observations and climate model simulations for Basel
Switzerland. Glob. Planet. Change 44, 73–81.

Bowler, D.E., Buyung-Ali, L., Knight, T.M., Pullin, A.S. (2010). Urban greening to cool towns and cities: a systematic review of the empirical evidence. Landsc. Urban Plan. 97, 147–155.

Bush, J., & Doyon, A. (2019). Building urban resilience with nature-based solutions: How can urban planning contribute?. Cities, 95, 102483.

Ettinger, A. K., Bratman, G. N., Carey, M., Hebert, R., Hill, O., Kett, H., ... & Wyse, L. (2024). Street trees provide an opportunity to mitigate urban heat and reduce risk of high heat exposure. Scientific Reports, 14(1), 3266.

Gabriel, K.M., Endlicher, W.R. (2011). Urban and rural mortality rates during heat waves in Berlin and Brandenburg. Germany Environ. Pollut. 159, 2044–2050.

Green, D., O'Donnell, E., Johnson, M., Slater, L., Thorne, C., Zheng, S., ... & Boothroyd, R. J. (2021). Green infrastructure: The future of urban flood risk management?. Wiley Interdisciplinary Reviews: Water, 8(6), e1560.

James, P., Tzoulas, K., Adams, M. D., Barber, A., Box, J., Breuste, J., ... & Thompson, C. W. (2009). Towards an integrated understanding of green space in the European built environment. Urban Forestry & Urban Greening, 8(2), 65-75.

Kamble, T., Bahadure, S., & Punglia, S. (2022). Availability and accessibility of urban green spaces in a high-density city: The Case of Raipur, India. The Professional Geographer, 74(2), 290-303.

Khose, S. B., Mailapalli, D. R., Biswal, S., & Chatterjee, C. (2022). UAV-based multispectral image analytics for generating crop coefficient maps for rice. Arabian Journal of Geosciences, 15(22), 1681.

Kong, F., Yin, H., James, P., Hutyra, L.R., He, H.S. (2014). Effects of spatial pattern of greenspace on urban cooling in a large metropolitan area of eastern China. Landsc. Urban Plann. 128, 35–47.

Kumar, P., Druckman, A., Gallagher, J., Gatersleben, B., Allison, S., Eisenman, T. S., ... & Morawska, L. (2019). The nexus between air pollution, green infrastructure and human health. Environment international, 133, 105181.

Lee, K., Kim, Y., Sung, H. C., Ryu, J., & Jeon, S. W. (2019). Trend analysis of urban heat island intensity according to urban area change in Asian mega cities. Sustainability, 12(1), 112.

Maimaitiyiming, M., Ghulam, A., Tiyip, T., Pla, F., Latorre-Carmona, P., Halik, Ü., ... & Caetano, M. (2014). Effects of green space spatial pattern on land surface temperature: Implications for sustainable urban planning and climate change adaptation. ISPRS Journal of Photogrammetry and Remote Sensing, 89, 59-66.

Monteiro, M.V., Doick, K.J., Handley, P., Peace, A. (2016). The impact of greenspace size on the extent of local nocturnal air temperature cooling in London. Urban for. Urban Green. 16, 160–169.

Oke, T.R. (2002). Boundary Layer Climates. Routledge, London and New York.

Pörtner, H. O., Roberts, D. C., Poloczanska, E. S., Mintenbeck, K., Tignor, M., Alegría, A., ... & Okem, A. (2022). IPCC, 2022: Summary for policymakers.

Quan, J., Zhan, W., Chen, Y., Wang, M., & Wang, J. (2016). Time series decomposition of remotely sensed land surface temperature and investigation of trends and seasonal variations in surface urban heat islands. Journal of Geophysical Research: Atmospheres, 121(6), 2638-2657.

Reis, C., & Lopes, A. (2019). Evaluating the cooling potential of urban green spaces to tackle urban climate change in Lisbon. Sustainability, 11(9), 2480. Reis, C., & Lopes, A. (2019). Evaluating the cooling potential of urban green spaces to tackle urban climate change in Lisbon. Sustainability, 11(9), 2480.

Rosenfeld, A.H., Akbari, H., Romm, J.J., Pomerantz, M. (1998). Cool communities: strategies for heat island mitigation and smog reduction. Energy Build. 28, 51–62.

Schmitz, S., Weiand, L., Becker, S., Niehoff, N., Schwartzbach, F., & von Schneidemesser, E. (2018). An assessment of perceptions of air quality surrounding the implementation of a traffic-reduction measure in a local urban environment. Sustainable Cities and Society, 41, 525-537.

Shahbazi, A., & Nasab, B. R. (2016). Carbon capture and storage (CCS) and its impacts on climate change and global warming. J. Pet. Environ. Biotechnol, 7(9).

Singh, S., Shukla, A., & Jain, K. (2024, July). Assessment of Spatiotemporal Dynamics of Urban Green Spaces in Bhopal Using a Geospatial Approach. In IGARSS 2024-2024 IEEE International Geoscience and Remote Sensing Symposium (pp. 4753-4756). IEEE.

Singh, N., Singh, S., & Mall, R. K. (2020). Urban ecology and human health: implications of urban heat island, air pollution and climate change nexus. In Urban ecology (pp. 317-334). Elsevier.

Sinnett, D. (2020). Mitigating air pollution and the urban heat island effect: the roles of urban trees. In The Routledge Handbook of Urban Ecology (pp. 639-648). Routledge.

Turner‐Skoff, J. B., & Cavender, N. (2019). The benefits of trees for livable and sustainable communities. Plants, People, Planet, 1(4), 323-335.

Wong, P.P.-Y., Lai, P.-C., Low, C.-T., Chen, S., Hart, M. (2016). The impact of environmental and human factors on urban heat and microclimate variability. Build. Environ. 95, 199–208.

Yaşlı, R., Yücedağ, C., Ayan, S., & Simovski, B. (2023). The role of urban trees in reducing land surface temperature. SilvaWorld, 2(1), 36-49.

Yu, Z.W., Guo, Q.H., Sun, R.H. (2015). Impact of urban cooling effect based on landscape scale: a review. Chin. J. Appl. Ecol. 26, 636–642.

Zhang, Y., Meng, W., Yun, H., Xu, W., Hu, B., He, M., ... & Zhang, L. (2022). Is urban green space a carbon sink or source?-A case study of China based on LCA method. Environmental Impact Assessment Review, 94, 106766.