Physicochemical Analysis of Wastewater of Industrial Area of Mahasamund (Chhattisagarh)

ABSTRACT

Aims

Evaluate the physicochemical properties of surface and groundwater in the industrial areas of Mahasamund district, Chhattisgarh. Assess the degree and sources of contamination caused by industrial, domestic, and agricultural effluents. Propose eco-sustainable wastewater treatment strategies aligned with environmental and economic considerations.

### Place and Duration of Study

### Location: Mahasamund District, Chhattisgarh, India – specifically focusing on two major industrial zones: Birkoni and Belsonda. Duration: Sampling and analysis were conducted over 2023 -2024 on three seasonal periods — pre-monsoon, monsoon, and post-monsoon — to capture temporal variations in water quality.

### Methodology

### Sampling: Seasonal water and wastewater samples were collected from both surface water sources (rivers) and groundwater sources (bore wells, hand pumps).Samples were taken from multiple points within Birkoni and Belsonda industrial areas.

### Analytical Procedures: Standard protocols of the American Public Health Association (APHA, 2017) were followed for sample collection, preservation, and laboratory testing. Physicochemical parameters analyzed included: pH, Electrical Conductivity (EC), Total Dissolved Solids (TDS), Total Hardness (TH), Calcium, Magnesium, Chloride, Sulfate, Nitrate, Dissolved Oxygen (DO), Biochemical Oxygen Demand (BOD), and Chemical Oxygen Demand (COD).

### Results

Physicochemical Characteristics: pH: 6.4–8.3, indicates influence of domestic and industrial effluents. EC and TDS: Frequently exceeded permissible limits, especially in pre-monsoon season, reflecting high ionic concentration. Hardness: 160–780 mg/L, categorizing most samples as hard to very hard. Chloride, Sulfate, and Nitrate: Elevated concentrations signified contamination from industrial chemicals, detergents, and agricultural runoff.

Organic Pollution Indicators: DO: Often below 4 mg/L, especially near effluent discharge points. BOD: Up to 48 mg/L. COD: Up to 120 mg/L — both indicating high organic load from untreated industrial and domestic waste.

Statistical Correlations: Positive correlation between TDS, and TH indicated contribution from industrial and geogenic sources. Positive correlation between COD and BOD confirmed dominance of organic pollution from untreated effluents.

### Conclusion

The study revealed that water resources in the Mahasamund industrial region are under serious environmental stress due to the uncontrolled discharge of untreated or partially treated effluents.

Keywords: industrial area, ecosystem, water, pH, DO, BOD, COD.

1. INTRODUCTION

Water is perhaps the most critical natural resource, vital to life, development, and environmental sustainability. Nonetheless, with increasing urbanization, industrialization, and population growth, the stress on freshwater resources has increased, particularly in developing nations such as India. The exponential growth in generation of wastewater with limited treatment capacity has become a significant environmental and public health issue (Vaidya et al., 2023). India generates almost 61,948 Million Liters per Day (MLD) of sewage, of which only around 38% is properly treated, while the rest of the wastewater is released into the environment in untreated or partially treated form, as stated by the Central Pollution Control Board (CPCB) (Vaidya et al., 2023; Adbarzi et al., 2020). The outcome of the trend is multifold. Untreated sewage pollutes rivers, lakes, and aquifers of groundwater, posing risks to ecosystems and human health by facilitating the transmission of waterborne diseases and the deposition of harmful substances (Parwin et al., 2024; Bharti et al., 2020). Mahasamund, a central-eastern Chhattisgarh district, represents an opposite case study of the dynamics and challenges involved in wastewater management in this environment. Groundwater is the major source of drinking and agricultural water in Mahasamund, which is accessed mainly through borewells and handpumps. While surface water contamination from industrial effluent and domestic sewage has also increased, groundwater sources are progressively threatened by leaching contaminants (Chaudhary et al., 2024). Seasonal fluctuations, particularly during the dry and monsoon seasons, consequently increase this pressure, impacting both the quality and availability of water resources (Chandrakar et al., 2018). A number of studies conducted in comparable environments in India unveil shocking variations in physicochemical parameters including pH, TDS, BOD, COD, heavy metals, and microbial load of wastewater samples (Adbarzi et al., 2020; Sahu et al., 2022). Without proper treatment, these pollutants seep into the water and soil bodies, significantly impacting agriculture production and subjecting the people to risky health hazards (Bharti et al., 2020; Parwin et al., 2024). In Mahasamund, this is also compounded by inadequate institutional capacity, poor monitoring, and public ignorance of wastewater threats (Gupta, 2020; Singh et al., 2023).

The urgent requirement for sustainable wastewater treatment methods becomes apparent in this context. Centralized sewage treatment plants (STPs), although available in some regions of the nation; tend to be underperforming based on deficiencies of trained personnel, ineffective maintenance procedures, and insufficient funding (Gautam et al., 2013; Chatterjee et al., 2016). An area where one can foresee innovation is the utilization of bio-coagulants from natural sources like fruit peels, seeds, and other plant-based materials. Banana peels (Musa paradisiaca), soybean hulls, and Strychnos potatorum seeds have also been recognized as suitable agents for turbidity and pollutant removal (Aripen et al., 2023; Daverey et al., 2019; Choubey, 2014). These bio-coagulants not only provide an inexpensive substitute for synthetic chemicals but also facilitate a circular economy through the reuse of agricultural and food wastage (Chong & Kiew, 2017; Patel et al., 2025).

2. METHODOLOGY

2.1 Study Area

The study was carried out in Chhattisgarh's Mahasamund district, an area that has seen considerable industrial development over the last decade. The main industrial area of interest in this study is Birkoni, a developing industrial cluster made up of ceramic, agro-processing, and chemical-based industries generating huge quantities of wastewater. The Belsonda Water Treatment Plant was also chosen as an alternate site, which is a centralized treatment plant with mixed industrial and domestic wastewater feed streams.

2.2 Sampling Strategy

2.2.1 Sampling Sites

Two major sampling sites were chosen:

- Site 1 – ALIVE Ceramics Pvt. Ltd., Birkoni: Represents untreated industrial effluent directly discharged from production units.

- Site 2 – Belsonda Water Treatment Plant: Accounts for treated or treated wastewater, allowing for comparative assessment of treatment efficiency.

2.2.2 Sampling Schedule and Frequency

Bi-monthly sampling was undertaken for six months (January to June 2024), covering dry and pre-monsoon seasons. Temporal study design catered to seasonal differences in the effluent's characteristics (Adbarzi et al., 2020).

2.2.3 Sample Collection and Preservation

The samples of wastewater were gathered using pre-cleaned 2 L HDPE bottles according to the APHA (2017) guidelines. Bottles were cleaned using the sample water prior to collection, closed, labeled, and moved under chilled conditions (4°C) to the Environmental Science Laboratory.

3. RESULT AND DISCUSION

Physico-Chemical Parameters of Water Samples from Mahasamund The evaluation of physico-chemical parameters provides critical insights into the quality, safety, and pollution status of water resources. Groundwater and river water samples from the industrial areas of Birkoni and Belsonda (Mahasamund district) were analyzed seasonally to assess variations in key indicators of water chemistry. These findings reflect the combined effects of industrial activity, seasonal hydrological processes, and environmental sustainability challenges.

3.1 Chloride (Cl⁻):-

Table 1: Seasonal variations in Chloride as Cl of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Chloride as Cl (mg/mL) | 49.38 | 42.40 | 50.43 | 53.36 |
| Winter | Chloride as Cl (mg/mL) | 35.66 | 55.73 | 44.83 | 46.28 |
| Summer | Chloride as Cl (mg/mL) | 47.47 | 38.38 | 44.36 | 47.23 |

Figure 1: Seasonal variations in Chloride as Cl of water samples of Mahasamund

Seasonal variations in chloride are summarized in Table 1 and Figure 1. During the monsoon, concentrations ranged from 42.40 to 53.36 mg/mL, with the maximum recorded in Belsonda river water. Winter values were more variable, from 35.66 mg/mL (Birkoni groundwater) to 55.73 mg/mL (Belsonda groundwater), indicating localized contamination sources. Summer values were moderate (38.38–47.47 mg/mL). Across all seasons, river water consistently contained higher chloride levels than groundwater, suggesting industrial contributions. These findings emphasize the need for sustainable wastewater management to limit chloride pollution in Mahasamund’s aquatic systems.

3.2 Total Dissolved Solids (TDS):-

Table 2: Seasonal variations in TDS (mg/mL) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | TDS (mg/mL) | 453 | 420 | 480 | 495 |
| Winter | TDS (mg/mL) | 120 | 436 | 158 | 438 |
| Summer | TDS (mg/mL) | 387 | 410 | 440 | 425 |

Figure 2: Seasonal variations in TDS (mg/mL) of water samples of Mahasamund

Table 2 and Figure 2 present seasonal fluctuations in TDS. During monsoon, values were elevated (420–495 mg/mL), reflecting solute enrichment from runoff and industrial discharge. In winter, Birkoni groundwater declined sharply to 120 mg/mL, while Belsonda sources remained high (436–438 mg/mL), pointing to localized inputs. Summer concentrations were moderately high (387–440 mg/mL). These results indicate both seasonal dilution effects and industrial influence, highlighting the necessity of effective effluent treatment.

3.3 Alkalinity (as CaCO₃):-

Table 3: Seasonal variations in Alkalinity as CaCO3 (mg/mL) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Alkalinity as CaCO3 (mg/mL) | 180.06 | 190.13 | 195.18 | 196.17 |
| Winter | Alkalinity as CaCO3 (mg/mL) | 43.38 | 169 | 68.17 | 196.32 |
| Summer | Alkalinity as CaCO3 (mg/mL) | 172.04 | 184.18 | 175.05 | 182.33 |

Figure 3: Seasonal variations in Alkalinity as CaCO3 (mg/mL) of water samples of Mahasamund

As shown in Table 3 and Figure 3, alkalinity levels varied seasonally. Monsoon samples exhibited elevated buffering capacity (180.06–196.17 mg/mL), linked to leaching and runoff. Winter values fell sharply in Birkoni groundwater (43.38 mg/mL) but remained high in Belsonda river water (196.32 mg/mL). Summer concentrations were moderately high (172.04–184.18 mg/mL), reflecting reduced dilution. These trends underline anthropogenic impacts on water chemistry and reinforce the need for eco-sustainable practices.

3.4 Calcium (Ca²⁺):-

Table 4: Seasonal variations in Calcium as Ca2+ (mg/mL) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Calcium as Ca2+ (mg/mL) | 60.33 | 48.06 | 60.07 | 53.03 |
| Winter | Calcium as Ca2+ (mg/mL) | 19.52 | 55.23 | 15.28 | 60.31 |
| Summer | Calcium as Ca2+ (mg/mL) | 50.1 | 40.06 | 55.35 | 50.31 |

Figure 4: Seasonal variations in Calcium as Ca2+ (mg/mL) of water samples of Mahasamund

Seasonal calcium concentrations (Table 4, Figure 4) showed distinct variations. Monsoon levels were 48.06–60.33 mg/mL, with higher values in Birkoni water. Winter concentrations dropped significantly in Birkoni river water (15.28 mg/mL) but peaked in Belsonda river water (60.31 mg/mL). Summer values were moderate (40.06–55.35 mg/mL). These results point to mineral leaching during monsoon and localized contamination during winter.

3.5 Magnesium (Mg²⁺)

Table 5: Seasonal variations in Magnesium as Mg2+ (mg/mL) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Magnesium as Mg2+ (mg/mL) | 25.45 | 32.78 | 40.75 | 35.75 |
| Winter | Magnesium as Mg2+ (mg/mL) | 5.66 | 32.21 | 5.66 | 35.24 |
| Summer | Magnesium as Mg2+ (mg/mL) | 27.43 | 35.76 | 22.07 | 30.25 |

Figure 5: Seasonal variations in Magnesium as Mg2+ (mg/mL) of water samples of Mahasamund

Magnesium data (Table 5, Figure 5) revealed substantial variability. Monsoon levels ranged from 25.45 to 40.75 mg/mL, with the highest in Birkoni river water. Winter values dropped sharply in Birkoni (5.66 mg/mL) but remained high in Belsonda (32.21–35.24 mg/mL). Summer levels were moderate (22.07–35.76 mg/mL). This indicates seasonal dilution and industrial inputs as controlling factors.

3.6 pH:-

Table 6: Seasonal variations in PH of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | PH | 7.5 | 7.6 | 8.2 | 8.0 |
| Winter | PH | 7.2 | 7.9 | 7.3 | 8.0 |
| Summer | PH | 7.19 | 7.4 | 7.3 | 7.8 |

Figure 6: Seasonal variations in PH of water samples of Mahasamund

Seasonal pH values (Table 6, Figure 6) remained slightly alkaline, with minor variation. Monsoon values ranged 7.5–8.2, winter 7.2–8.0, and summer 7.19–7.8. The highest value was in Birkoni river water during monsoon (8.2). Overall, pH remained within permissible limits but consistent alkalinity suggests anthropogenic inputs.

3.7 Turbidity:-

Table 7: Seasonal variations in Turbidity (NTU) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Turbidity (NTU) | 0.8 | 0.9 | 1.0 | 0.9 |
| Winter | Turbidity (NTU) | 0.4 | 0.9 | 0.8 | 0.8 |
| Summer | Turbidity (NTU) | 0.5 | 0.7 | 0.7 | 0.8 |

Figure 7: Seasonal variations in Turbidity (NTU) of water samples of Mahasamund

Turbidity trends (Table 7, Figure 7) were generally low across seasons (0.4–1.0 NTU), except for a sharp winter spike in Birkoni river water (0.8 NTU), indicating heavy contamination from industrial effluents or surface discharges. This anomalous increase underscores the urgency of localized pollution control.

3.8 Total Hardness (as CaCO₃):-

Table 8: Seasonal variations in Total Hardness as CaCO3 (mg/mL) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Total Hardness as CaCO3 (mg/mL) | 287.12 | 290.02 | 293.03 | 298.05 |
| Winter | Total Hardness as CaCO3 (mg/mL) | 72.01 | 280.28 | 61.42 | 287.83 |
| Summer | Total Hardness as CaCO3 (mg/mL) | 237.11 | 262.01 | 250.24 | 300.25 |

Figure 8: Seasonal variations in Total Hardness as CaCO3 (mg/mL) of water samples of Mahasamund

As summarized in Table 8 and Figure 8, hardness varied seasonally. Monsoon values were high (287.12–298.05 mg/mL), winter values dropped sharply in Birkoni (61.42–72.01 mg/mL) but remained high in Belsonda (280.28–287.83 mg/mL), and summer showed moderate to high levels (237.11–300.25 mg/mL). These fluctuations reflect both seasonal leaching and industrial impacts.

3.9 Taste and Odour:-

Table 9: Seasonal variations in Taste of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Taste | - | - | - | - |
| Winter | Taste | - | - | - | - |
| Summer | Taste | - | - | - | - |

Table 10: Seasonal variations in Odour of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Odour | Agree | Agree | Agree | Agree |
| Winter | Odour | Agree | Agree | Agree | Agree |
| Summer | Odour | Agree | Agree | Agree | Agree |

Both taste (Table 9) and odour (Table 10) showed no seasonal or spatial differences. All samples were reported as tasteless and acceptable in odour, suggesting industrial activities have not yet significantly affected these organoleptic properties. However, continued monitoring is necessary.

3.10 Colour:-

Table 11: Seasonal variations in Color of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Color | 3 | 4 | 5 | 5 |
| Winter | Color | 3 | 4 | 11 | 5 |
| Summer | Color | 3 | 4 | 3 | 4 |

Figure 9: Seasonal variations in Color of water samples of Mahasamund

Colour data (Table 11, Figure 9) revealed higher variability in river water compared to groundwater. Birkoni river water recorded values of 5 (monsoon), 11 (winter), and 3 (summer), suggesting episodic contamination events. Belsonda river water showed minor variation (4–5), while groundwater remained stable (3–4). This confirms that river water is more susceptible to effluent discharge impacts.

3.11 Dissolved Oxygen (DO):-

Table 12: Seasonal variations in Dissolve Oxygen (DO) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Dissolve Oxygen (DO) | 7.19 | 9.44 | 8.23 | 9.23 |
| Winter | Dissolve Oxygen (DO) | 7.33 | 6.30 | 7.68 | 7.01 |
| Summer | Dissolve Oxygen (DO) | 5.89 | 8.34 | 5.38 | 8.98 |

Figure 10: Seasonal variations in Dissolve Oxygen (DO) of water samples of Mahasamund

As shown in Table 12 and Figure 10, DO values were highest during monsoon (7.19–9.44 mg/L), slightly declined in winter (6.30–7.68 mg/L), and reached the lowest in summer (5.38 mg/L in Birkoni river water). Belsonda river water consistently maintained higher DO (8.98 mg/L in summer). These trends reflect seasonal oxygen depletion due to temperature and organic loading.

3.12 Biological Oxygen Demand (BOD):-

Table 13: Seasonal variations in Biological Oxygen Demand (BOD) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Biological Oxygen Demand (BOD) | 2.12 | 4.48 | 2.34 | 3.05 |
| Winter | Biological Oxygen Demand (BOD) | 1.02 | 3.02 | 1.36 | 2.56 |
| Summer | Biological Oxygen Demand (BOD) | 1.78 | 3.78 | 2.78 | 3.08 |

Figure 11: Seasonal variations in Biological Oxygen Demand (BOD) of water samples of Mahasamund

BOD levels (Table 13, Figure 11) indicated moderate organic pollution. Monsoon values were 2.12–4.48 mg/L, winter showed the lowest (1.02–3.02 mg/L), and summer values slightly increased (1.78–3.78 mg/L). The seasonal decline in winter reflects reduced microbial activity, while summer increases suggest organic matter accumulation under low flow.

3.12 Chemical Oxygen Demand (COD):-

Table 14: Seasonal variations in Chemical Oxygen Demand (COD) of water samples of Mahasamund

| Season | Parameters | Ground water from tube well industrial area, Birkoni, Mahasamund | Ground water from tube well industrial area, Belsonda, Mahasamund | River water from industrial area, Birkoni, Mahasamund | River water from industrial area, Belsonda, Mahasamund |
| --- | --- | --- | --- | --- | --- |
| Monsoon | Chemical Oxygen Demand (COD) | 25.55 | 13.54 | 27. 46 | 18.68 |
| Winter | Chemical Oxygen Demand (COD) | 10.24 | 15.29 | 19.34 | 20.78 |
| Summer | Chemical Oxygen Demand (COD) | 22.45 | 12.34 | 25. 56 | 16.76 |

Figure 12: Seasonal variations in Chemical Oxygen Demand (COD) of water samples of Mahasamund

COD results (Table 14, Figure 12) demonstrated notable seasonal variation. Monsoon values peaked in Birkoni river water (27.46 mg/L), winter recorded lower values (10.24–20.78 mg/L), and summer again showed elevated concentrations (25.56 mg/L in Birkoni river). These patterns point to seasonal effluent discharge and organic load fluctuations.

SUMMARY OF FINDINGS

● Monsoon: Elevated solutes (TDS, hardness, and alkalinity) due to runoff and leaching.

●Winter: Localized contamination, particularly in Belsonda, with anomalous turbidity in Birkoni River.

●Summer: Moderate but concentrated values, with lowest DO and higher BOD/COD in some sites.

Overall, river water showed greater seasonal and industrial influence compared to groundwater. While organoleptic properties (taste, odour) remained unaffected, several physicochemical indicators exceeded safe thresholds, underscoring the need for sustainable wastewater management in Mahasamund’s industrial zones.

REFERENCE

- Adbarzi, S., Tripathi, P., Choudhary, K., Kant, R., & Tripathi, V. (2020). Assessment of physico-chemical properties of pre and post-treated wastewater of Prayagraj region and its effect on nearby Ganges river. Vegetos, 33, 258 - 264. https://doi.org/10.1007/s42535-020-00103-y.

- Aripen, N. S. M., Halim, N. A., Saadon, S., & Jamil, N. D. A. (2023). The Usage of Banana Peels and Soybean Hull for the Treatment of Lake Water: Turbidity and Total Suspended Solids. Journal of Science and Technology, 15(1), 1-6.

- Bharti, M., Kamboj, N., & Kamboj, V. (2020). Effect of untreated wastewater on soil quality: A case study in Ranipur Rao watershed in Haridwar region (Uttarakhand), India. Annals of The Entomological Society of America, 145-157. https://doi.org/10.26832/aesa-2020-aepm-010.

- Chandrakar, S., Das, S., & Biswas, B. (2018). Aquaculture potentials of the reservoirs of Mahasamund district, Chhattisgarh: morphometric and physico-chemical characteristics. Journal of Entomology and Zoology Studies, 6(5): 559-566. https://www.entomoljournal.com/archives/2018/vol6issue5/PartI/6-4-287-711.pdf

- Chatterjee, P., Ghangrekar, M., & Rao, S. (2016). Low efficiency of sewage treatment plants due to unskilled operations in India. Environmental Chemistry Letters, 14, 407-416. https://doi.org/10.1007/s10311-016-0551-9.

- Chaudhary, J., Chandrakar, B., & Lakpale, R. (2024). Agroclimatic Drought Analysis in Mahasamund District of Chhattisgarh, India. International Journal of Environment and Climate Change. https://doi.org/10.9734/ijecc/2024/v14i74292.

- Chong, K. H., & Kiew, P. L. (2017). Potential of banana peels as bio-flocculant for water clarification. Progress in Energy and Environment, 47-56.

- Choubey S., “Coagulation-Clarification of Turbid Colored Water by Natural Coagulant (Strychnos potatoram) Seed Extract”, Journal of Applicable Chemistry 2013, 2 (1): 73-80, ISSN:22781862, 24/12/2012.

- Choubey S., Comparison of coagulation efficiency of Natural Coagulants extracted from different rural species, International Journal of Development Research, Vol. 4, Issue, 9, pp. 1810- 1812, September, 2014, ISSN:22309926, Sept 2014

- Daverey, A., Tiwari, N., & Dutta, K. (2019). Utilization of extracts of Musa paradisica (banana) peels and Dolichos lablab (Indian bean) seeds as low-cost natural coagulants for turbidity removal from water. Environmental Science and Pollution Research, 26(33), 34177-34183.

- Gautam, S., Sharma, D., Tripathi, J., Ahirwar, S., & Singh, S. (2013). A study of the effectiveness of sewage treatment plants in Delhi region. Applied Water Science, 3, 57-65. https://doi.org/10.1007/s13201-012-0059-9.

- Gupta, S. (2020). An Overview of Developments in Wastewater Management and Their Applicability in Urban India., 05, 157-164. https://doi.org/10.46609/ijsser.2020.v05i01.009.

- Parwin, N., Dixit, S., Sahoo, S., Sahoo, R., & Subudhi, E. (2024). Assessment of the surface water quality and primary health risk in urban wastewater and its receiving river Kathajodi, Cuttack of eastern India.. Environmental monitoring and assessment, 196 6, 551 . https://doi.org/10.1007/s10661-024-12683-2.

- Patel, A., Choubey, S., & Sahu, K. (2025). Use of Natural Coagulants from Fruit Peels for Wastewater Treatment in the Industrial Area of Mahasamund District, Chhattisgarh (India). International Journal on Science and Technology. https://doi.org/10.71097/ijsat.v16.i2.3496.

- Patel, M., & Pandey, S. (2020). Impact evaluation of the scheme on SRI implementation in Mahasamund district of Chhattisgarh. Journal of Pharmacognosy and Phytochemistry, 9, 379-382.

- Patel, M., & Pandey, S. (2024). Resource Use Efficiency of Groundnut Production in Mahasamund and Raigarh Districts of Chhattisgarh in India. Journal of Scientific Research and Reports. https://doi.org/10.9734/jsrr/2024/v30i72119.

- Sahu, K. (2025). Impact of Rice Mill Wastewater on the Water Quality of River: A Case Study from Rajim, Chhattisgarh. International Journal For Multidisciplinary Research. https://doi.org/10.36948/ijfmr.2025.v07i02.42176.

- Sahu, S., Dani, A., P., Shrivas, R., & Kumar, Y. (2022). Physiochemical and Microbial Analysis of Wastewater from Food Industry Effluent near Kuthrel area, Durg District, Chhattisgarh. International Journal for Research in Applied Science and Engineering Technology. https://doi.org/10.22214/ijraset.2022.43140.

- Singh, B., Chakraborty, A., & Sehgal, R. (2023). A systematic review of industrial wastewater management: Evaluating challenges and enablers. Journal of environmental management, 348, 119230 . https://doi.org/10.1016/j.jenvman.2023.119230.

- Thakur S. S. and Choubey S. “Use of Tannin based natural coagulants for water treatment: An alternative to inorganic chemicals”, International Journal of ChemTech Research Vol.6, No.7, pp 3628-3634, 2014, CODEN(USA): IJCRGG ISSN : 0974-4290 0.34

- Vaidya, R., Verma, K., Kumar, M., Hoysall, C., & Rao, L. (2023). Assessing wastewater management challenges in developing countries: a case study of India, current status and future scope. Environment, Development and Sustainability, 1-28. https://doi.org/10.1007/s10668-023-03540-2.