MODELING OF A HYBRID SYSTEM FOR CURRENT AND FUTURE ENERGY SUSTAINABILITY WITH IMPROVED ECONOMIC DEVELOPMENTS USING PRESENT GRID NETWORK TOPOLOGY

ABSTRACT

Sustainable development can be achieved through; energy sustainability, industrial revolution, political stability, improved agricultural sector and mechanized farming, improved and adequate transportation network, adequate communication infrastructure, etc. Developing a sustainable Energy model that drives economic development is timely. A nation's energy availability as in Nigeria where numerous economic activities pose a challenge. It is observed that many of these private sectors, such as Manufacturers, run petrol or diesel generators, which add to air pollution. The nation is challenged by the presence of these dangerous gasses. This makes it necessary to improve the energy mix using a hybrid system. Thus, this paper models a hybrid (Solar PV, Wind, and the National grid) automated power system to provide constant energy to the thickly populated nation with diverse economic activities, thereby guaranteeing energy stability, reliability, and security.  The work is aimed at providing sustainable power that is environmentally friendly, less expensive, and has low maintenance costs. The model is developed using ETAP software which gives a better and reliable solution to power system utilizations and sizing. The present and future worth, initial cost, maintenance cost, solar irradiant, and load demand estimate were among the parameters of interest considered. When comparing the results with different energy sources in the network during simulation, it is observed that the renewable in the network has an 80.8% penetration level, and power quality availability rose 81%, the system witnessed 35% reductions in maintenance cost. These results justify the proposal of a hybrid system compared to a stand-alone system.

KEYWORDS: Hybrid System, Future Energy Sustainability, Economic Development, Energy Improvement and Planning

1.0	INTRODUCTION

Energy plays a pivotal role in the technological and economic development of contemporary society, serving as the linchpin for humanity’s aspiration for a better world. The sustainable growth of any nation hinges significantly on its Electricity infrastructure, with electricity acting as the lifeblood and propelling force of a burgeoning economy [1]. High penetration of renewable (hybrid) into the network gives a better effect on the bus voltage, voltage drop, energy stability, and reliability of the energy on the network and its connections with the inclusion of renewable energy on the network, energy stability is certain, economic and social development will improve and a voltage profile also improves on the network. Thus, an increase in energy generation results in end-user satisfaction with usage from the utility company [8, 9]. Due to the significant economic growth and increasing Industrialization, energy consumption has drastically increased in the last decade. A new survey by the International Energy Agency (IEA) predicts an increase of 4.5% registered for the total energy in 2021 [2]. The economic development deeply depends on the stability of electricity in the area. The increase in energy consumption, and the increase in economic and social development of the nation, thereby improving the welfare of the people [3]. Currently, energy systems are undergoing a paradigm shift towards more sustainability and diversification, this shift is happening quickly worldwide [4-6]. Energy services are widely recognized as the primary catalyst for socio-economic progress. Electric lighting, motors, electronics, and other applications of electricity offer indispensable services that have become integral to every society. Among the Due to their direct or indirect connection to human existence, health facilities account for a significant portion of all entities that depend on a steady supply of electricity. Therefore, making sure that health facilities have access to off-grid electricity is essential to allowing them to function at full capacity. Generally, the options are generators and/or solar PV systems with battery storage, though micro-hydro may be a viable alternative in some parts of Ethiopia. Although the cost of a hybrid PV–Generator is lower than that of solely battery-charged PV, the initial capital outlay is higher [10]. system is the best option. Local utility companies struggle with regular rolling blackouts, voltage surges, and excessive line losses.

However, in Ethiopia, funding for fuel procurement for rural health facilities is consistently lacking or unavailable, making the effective design and implementation of a battery-charged stand-alone PV system the best option. Local utility companies face frequent rolling blackouts, voltage swells, and excessive line losses. The decision to incorporate renewable energy into the hybrid system for Shinshicho Primary Hospital is motivated by the goal of lowering maintenance costs and diesel consumption. Renewable energy sources, like wind, solar, and hydropower, have the peculiar quality of not being consistently available on demand; rather, their availability is somewhat sporadic. The revolutionary idea of hybrid power systems is the real breakthrough in the field of power generation. Contrary to the conventional belief that cost savings derive from utilizing the most potent solar panels or the most efficient diesel engines, the key lies in harmonizing the most economical energy production with the prevailing energy demand. Through the strategic coupling and coordination of diverse energy sources, these hybrid systems not only ensure a more dependable power supply but also enhance the overall quality of electricity delivered. Photovoltaic (PV) systems, in particular, have witnessed widespread deployment, ranging from expansive PV plants generating cumulative power in the order of several tenths of Watts to smaller-scale applications such as cameras, watches, and mobile devices. The versatility of PV systems extends beyond their environmental friendliness—requiring no external power input and being free from pollution—they also possess the distinct advantage of being deployable in any location, irrespective of local grid availability. The primary energy source for PV systems is solar power, widely acknowledged for its environmentally clean profile in energy production. These PV arrays harness the radiant energy from the sun to generate electricity, and notably, they operate without the need for any conventional fuel.  The combination of different generating systems whose sources differ with the emphasis of providing sustainable, reliable, and clean/cheap energy for economic improvements of the common user terms hybrid […]. These can be possible by joining two generators of different sources, or it can be three or more depending on the investors' ability and technical know-how. It can be renewable-renewables, renewable–renewable-renewable sources. For instance, solar PV and wind, wind and gen set, solar PV and nuclear. However, it depends on the availability of sources in a particular area. Toward the southern part of the country solar Pv and hydro hybrid, toward the west, solar Pv and nuclear with wind as a backup, on the East, Solar Pv and wind and towards the North, solar Pv and wind hybrid with gen-set as a backup. Nothing that, there stand-alone and grid-connected hybrid. These will help tremendously in energy sustainability [1, 2]. Economic development depends on energy usage by individuals and nations, and constant energy for production helps reduce the cost of living and commodities. [1, 3] Compared to other energy vectors, electricity is a high-quality energy carrier that is more productive, adaptable and emits no pollution when used. When it comes to applications like lighting, electricity is significantly more thermodynamically efficient than any other technology. Several high-value uses cannot be replaced by electricity, like computing and telecommunications. The high cost of energy restricts its usage to relatively high-value applications when there are alternatives. Nevertheless, where power is subsidized, it will also be employed in low-value applications. Conventional fuels are dirty and can need a large amount of household labor for collection, processing, and usage. The development offers ways to mitigate the harmful impacts of traditional fuels and creates job possibilities in the market. Thus, households gradually move up an "energy ladder" as earnings rise by using higher-quality fuels, like electricity.  A reliable electricity supply necessitates both expensive investment and expert management of the electrical grid. Network collapse can only be avoided by maintaining a constant instantaneous balance between supply and demand for electricity. In the meantime, the demand for energy fluctuates throughout the day and night, throughout the week, during the seasons, and in response to meteorological phenomena. Network and generation capacity that is occasionally only needed for a few hours a year are needed to meet annual peak demand. [4,5,6]. Among the available renewable energy resources, solar and wind power are considered to yield the highest social, economic, and environmental benefits, making their amalgamation a favorable option [7]. Renewable energy, derived from natural resources such as sunlight, rain, tides, wind, and geothermal heat, is characterized by its renewability these sources are replaceable and reusable [1].  It serves as a viable alternative to conventional fossil fuels and has often been referred to as alternative energy. The phrase refers to any naturally existing energy in the environment that may be used for human advantage; the sun, gravity, and the rotation of the Earth are the main sources. Notably, the growth of the Gross National Product (GNP) per capita is directly correlated with the annual rise in energy consumption. The present trajectory of fossil fuel usage is anticipated to result in an inadequate energy supply to fulfill global demand post-2020.  Compared to conventional energy sources, renewable energy sources (RESs) provide a number of benefits, such as instant access to dependable electricity, the avoidance of lengthy wait times for grid extension, decreased reliance on changes in oil prices, lower fuel transportation costs, better healthcare and rural development, increased economic productivity, and the creation of local job opportunities. Additionally, by enabling a more effective use of regional natural resources, RESs aid in the global battle against poverty and climate change [10].  Urban dwellers connected to grid supplies are increasingly turning to renewable energy due to concerns about limited fossil fuel reserves and global environmental issues associated with their usage in power generation. These sustainable energy sources, which are referred to as "green energy," include geothermal, solar, wind, and others. Opportunities for exploiting solar resources to generate electricity globally have been made possible, especially by photovoltaic technology. These technologies are now economically viable enough to be used in small-scale standalone applications for public, private, and commercial settings in addition to large-scale ones. In response to the growing need for grid stability and dependability, the idea of renewable energy hybrid systems first surfaced in the middle of the 1980s and gained popularity in the early 1990s. The incorporation of significant amounts of wind power into tiny autonomous grids was the driving force behind this explosion in hybrid system literature. The investigation and use of renewable energy solutions remain at the forefront of sustainable development as the globe struggles to fulfill energy demands while reducing environmental impact [11,12]. The output characteristics of conventional photovoltaic (PV) systems are inherently unstable and are strongly linked to the erratic weather during short time periods [13]. A deliberate attempt has been made to integrate solar PV with other renewable resources or conventional electricity in order to increase efficiency and make use of energy conversion technologies. There are difficulties with this integration since it has to deal with the complicated issues brought on by the variety of solar energy sources.

The complex web of difficulties is influenced by aspects like stochastic elements, low energy densities, and sporadic availability. The main focus of energy harvesting efforts is solar energy, a ubiquitous substance made up of the sun's heat and radiant light. A constantly developing range of technologies, including solar thermal, photovoltaic, and heating, enable this harnessing. Photovoltaic (PV), wind turbines, hydropower, generators driven by different prime movers like diesel, biogas, or coal-fired choices, electric utility networks, microturbines, or fuel cells are just a few of the many types of power sources that are present in the vast terrain of power sources. [14-18]. In the context of energy storage, battery banks and hydrogen repositories are mentioned.  Primary and deferred loads are introduced as a dichotomy by load considerations.  Primary loads, like lighting, must be completed right away, but deferrable loads, like water pumping, must be completed within a certain window of time. [16-20].

2.0	Modelling

Modeling an electricity network of 72 buses, 14 generators, and 69-line connections is done using ETAP19.0. The load flow simulations on the network is modeled using the fast decouple method as follows

The voltage equation for (k+1)th iteration is

Ax1=J11      J12J21      J22; by choosing  Ax1 = J11          00          J22                                        						(1)

∆P=J11∆δ                                                                                           						(2)

∆Q=J22∆|V|                                                                                       						(3)

The principle is based on the following.

∂Pi∂δk  is much larger than ∂Qi∂δk approximately zero and

∂Qi∂|Vk|  is much larger than ∂Pi|∂Vk|  approximately zero.

By incorporating these approximations in the Jacobian equation, we have

∂P2∂δ2              ⋯            ⋯            ∂P2∂δ4⋮               J11                              ⋮⋮                   ⋮                 ⋮              ⋮∂P4∂P4            ⋯                ⋯            ∂P4∂δ4  ∆δ2⋮⋮∆δ4=∆P2⋮⋮∆P4       (4)

|V2|∂Q2∂|V2|          ⋯          ⋯          |V4|∂Q2∂|V4|⋮                  ⋮           J22               ⋮⋮                   ⋮              ⋮                  ⋮|V2|∂Q4∂|V2|          ⋯          ⋯            |V4|∂Q4∂|V4|∆|V2||V2|⋮⋮∆|V4||V4|=∆Q2⋮⋮∆Q4   (5)

The equations are decoupled in nature

Thus

∆P∆Q= H00L∆δ⃒∆V⃒⃒V⃒				        (6)

2.1	Decoupled Newton Method

It can be further factored out as;

∆P= H∆δ                                      	  	        (7)

∆Q= Lx⃒∆V⃒⃒V⃒     		              	 	       (8)

Hik=Lik=⃒ViVk⃒(Giksinδik-Bik cosδik)     i≠k     (9)

The elements of the Jacobian Equation 8 can be simplified as:

Hik= ∂Pi∂δk         and           Lik= ∂Qi⃒Vi⃒∂⃒⃒Vi⃒⃒      	    (10)

From Eq. 9 & 10

∂Pi∂δk= Pi= ⃒ViVkYii⃒cosθii+K=1,K≠iN⃒ViVkYik⃒cosθik+δk-δi          	   (11)

Qi=⃒ViVkYii⃒sinθii+K=1,K≠iN⃒ViVkYik⃒sinθik+δk-δi                                  		 		 (12)

Off-diagonal elements of H (i.e. when i≠k) are:

Hik= ∂Pi∂δk=⃒ViVkYik⃒ sinθik+δk-δi		(13)

Expanding by trigonometric function yields,

= ⃒ViVk⃒Yiksinθikcosδk-δi+sin⃒Yik⃒cosθiksinδk-δi                          				(14)

We know that

Yiksinθik= -Bik           		 		(15)

and

Yikcosθik= -Gik                               		(16)

then

Yik=Gik-jBik                                   		 (17)

HiK= ⃒ViVK⃒-BiK cosδK-δi+GiK sinδK-δi                                        		 				(18)

Likewise, the off-diagonal element of L (i.e. normalized form) is

Lik= ∂Qi∂⃒VK⃒VK=⃒ViVKYiK⃒sinθiK+δK-δi   	(19)

Lik=ViVk-Bikcosδk-δi+Giksinδk-δi                                       						(20)

From Eqs 18 and 19 we can see that

HiK=Lik=⃒ViVk-Giksinδk-δi-Bikcosδk-δi                                     						(21)

The diagonal elements of H are given as:

Hi=∂Pi∂δk=K=1,K≠iN⃒ViVKYiK⃒sinθiK+δK-δi 	(22)

=  - K=1,K≠iN⃒ViVKYiK⃒sinθiK+δK-δi-⃒ViVKYiK⃒sinθik=- θik+⃒ViVKYiK⃒sinθii        	 ( 23)

Hii=-Qi-Vi2Bii 				 (24)

Similarly, the diagonal element of L are given as:

Lii= ∂Pi∂Vi⃒Vi⃒= ⃒2Vi2Yii⃒sinθii+  K=1N⃒ViVKYiK⃒sinθiK+δK-δi		(25)

Lii=⃒2Vi2Yii⃒sinθii+ K=1N⃒ViVKYiK⃒sinθiK+δK-δi-⃒ViVKYiK⃒sinθii=⃒2Vi2Yii⃒sinθii+ Qi-⃒Vi2Yii⃒sinθii       				(26)

Lii=⃒Vi2Yii⃒sinθii+ Qi ,

but   Yiisin θii=-Bii 				(27)

=-Vi2Bii + Qi,Lii=Qi-Vi2Bii 		(28)

2.2	Modelling of the Pv System

Modelling of photo voltaic source

Considering the equivalent circuit of the solar cell.

Fig: 1.: equivalent circuit of the solar cell

In an ideal solar cell, RS= 0 and RSH = infinity. The net current of the cell is the difference between the photocurrent, IL, and the normal diode current, which is given by

(29)

But the photocurrent depends on the reference temperature T1 and T2 for the first and second temperatures respectively.

(30)

(31)

(32)

Where G is the present solar radiation and Gnorm is the solar radiation at the reference test.

The saturation current of the diode Io is given by

(33)

(34)

Figure 2-V characteristics curve of a solar cell

The series resistance of a solar cell is given by

Where

(35)

The voltage at no light condition of the cell

(36)

Where  is known as the thermal voltage and T is the absolute cell temperature.

The maximum power point is at the operating point A in which the power dissipated in the resistance load is its maximum value and is given by

(37)

The maximum efficiency of a solar cell is the ratio between the maximum power and the incident light power and is expressed as

(38)

where A is the area of the Pv modules and Ga is radiation.

The fill factor (FF) is a measure of the real I-V characteristics for efficient solar cells the value should be greater than 0.7.  The FF diminishes as the cell temperature increase. The FF is expressed as

(39)

Considering the diagram below

Fig. 2.: Double diode Pv model

The output current base on the double diode model is describe by

(40)

Where ID1 and ID2 are the diode saturation currents of first and second diodes respectively and Vt1 and Vt2 are the diode thermal voltages and they can be given as

(41)

(42)

Where a1 and a2 are the diode identity factor that represent the component of diffusion and recombination current.

2.2	Modelling of Pv panel and array: Due to inadequate voltage produced by a solar cell, there is a need to combine several cells together. This is done by connecting them in series to increase the voltage. Several of the cells can also be connected in parallel to increase the current.

Fig. Schematic diagram of a Pv module

The relationship between the cell is voltage (Vc) and current (Ic), and the modules voltage (Vm) and current (Im) are given by the following equation.

(43)

(44)

(45)

Where Nsm is the number of series cells, Npm is the number of parallel cells and Rsm is the equivalent series resistance of the Pv module.

2.3	Modelling of Pv system power electronic features

- Maximum power point tracker

- DC – AC inverter

- Pv array tilt angle

- Storage battery

2.3.1	Modelling of maximum power point tracker

The maximum power extracted from a Pv array depends on three parameters, which are insulations, load impedance and cell temperature. When Pv system is directly connected to load. The system will operate at the intersection of the current – voltage (I – V) curve and load line. Here one need an MPP which is based on the load line adjustment under varying atmospheric condition. This variation has to be as a function of temperature and irradiation shows that, the temperature affect the output voltage while the irradiation affects the Pv module, there is a unique point in the I – V and a P – V curve, namely, the MPP in which the Pv system operate at its maximum efficiency and produces its maximum power output.

2.3.2	IC Method

This is derived by differentiating the Pv power with respect to voltage and setting the results to zero.

(46)

If 				(47)

The left-hand side of equation 3.54 represents the opposite of the instantaneous conductance,  whereas the right-hand side represent its IC. The incremental variations, dVPv and dIPv can be approximated by the increments of both the parameters,  and  with the aim of measuring the actual values of VPv and IPv. The incremental variation dVPv and dIPv are expressed as follows

(48)

(49)

Fig. 3. shows the basics of the IC Method

2.4	DC – AC Inverters

The inverter has two main functions in a Pv system. For a standalone system, the inverter converts the DC to an AC signal. For a grid-connected Pv system, the inverter is responsible for synchronizing the output signal with the grid in term of frequency and phase shift.

The inverter is model in terms of it efficiency curve, and it can be described by a power function as

(50)

Where PPv and PINVC are Pv module output power and inverter’s rated power respectively. Which C1, C2, and C3 are model coefficients.

2.5	Modelling of battery storage

Fig. 4: equivalent circuit of a storage battery

Considering Figure 4. as an equivalent circuit of a storage battery. The internal voltage of the battery is represented by V1 and an internal resistance R1. The charging and discharging current Ibat depends on the system’s voltage level. If the applied voltage, Vbat, the current Ibat, will flow in the battery as charging current, that is, if the applied voltage is less than the battery voltages the current will flow out of the battery as discharging currents.

The state of charge (SoC) of bathing is expressed as

(51)

From equation 3.58, Q represents the battery charge and C represents the battery capacity.

The depth of charge (DoD) of the battery is given by

(52)

During the charging mode, suppose that V1 = Vch and R1 = Rch, the charging voltage Vch is given by

(53)

In which

SoC, represents initial state of charge of the battery, and SOCm represent the maximum value of battery SOC. Ns is the number of 2V series cells.

Rch represents the charging resistance and it is calculated by

(54)

But Vbat = Vch + Ich Rch			(55)

In order to calculate the SOC of the battery during charging mode, equation 3.62 is used

(56)

Where K and D are charging efficiency and the self-discharging rate respectively

(57)

(58)

(59)

One of the important parts of the battery storage model is the estimation of the instantaneous value of the SoC. The following equation describes the SoC at time (t + dt)

(60)

Simplifying previous equation

(61)

The right side of equation 3.65 is the first derivation of the SoC(t), so the instantaneous SoC can be obtained by integration as shown below

(62)

Where t is the interval time response of the simulations.

2.6	Modelling of Pv array tilt angle

The components of incident global solar radiation on a tilted surface can be expressed by

(63)

Where GTLT, DTLT, BTLT, and RTLT are global, direct (beam), diffuse and reflected solar energy on tilt surface. However, equation 3.70 can be rewritten as

(64)

Where G and D are the global and diffuse solar energy on horizontal surface.

Meanwhile, RB, RD, and RR are coefficients and ꝭ is the ground albedo. RB is the ratio between global solar energy on a horizontal surface and global solar energy on a tilt surface RD is the ratio between diffuse solar energy on a horizontal surface and diffuse solar energy on a tilt surface, RR is the amount of reflected solar energy on a tilt surface.

Calculating RB

(65)

The surface in the southern hemisphere slopes toward the equator. The RD is given by

(66)

Where LAT is the latitude of the location and TLT is the tilt angles. DEC and Wss are the angles of declination and sunshine hour angle respectively. DEC is given by

(67)

While Wss is given by

(68)

The equation for RR is given by

(69)

(70)

Meanwhile, other formulas for RD are

(71)

Using equation 3.78, RD can be calculated as

(72)

RD is defined as

(73)

By considering the anisotropy property on the RD can be defined as

(74)

A more complex equation of RD is

.						(75)

The equation for calculating RD is

(76)

Fig.5.: DC – AC three-phase inverter circuit

The fundamental frequency (50Hz) phase-to-neutral voltage is

[1, 5, 19]								(77)

3.0	Modelling of Wind Turbine

Power Equation

The power extracted from the wind is given by:

P = (1/2) * ρ * A * v^3 * Cp			(78)

Where P = power output,  ρ = air density, A = swept area of the rotor, v = wind speed, Cp = power coefficient

Torque Equation

The torque produced by the wind turbine is given by:

T = (1/2) * ρ * A * v^2 * Ct * R			(79)

Where: T = torque, Ct = thrust coefficient, R = rotor radius

Power Coefficient (Cp)

The power coefficient is a measure of the turbine's efficiency:

Cp = P / (1/2 * ρ * A * v^3)			(80)

Tip-Speed Ratio (λ)

The tip-speed ratio is the ratio of the blade tip speed to the wind speed:

λ = ω * R / v					(81)

Where ω = angular velocity of the rotor

Dynamic Equations

The dynamic behavior of the wind turbine can be modeled using equations of motion:

J * dω/dt = T - Tload				(82)

Where:

J = moment of inertia of the rotor,  Tload = load torque

Fig.6.: Graph of Etap model of the network with hybrid PV solar and Wind power with grid network.

- RESULTS AND DISCUSSION

Figure 6. was used to generate the results, which are shown in Tables 1., 2., and 3.. Fig. 7. shows the voltages against bus simulation graph. outcomes of the hybrid network concept (wind and solar PV). In terms of increased generation and energy availability, it displays the network's advancements. Lower voltages are experienced by some buses (such as ALAOJI T.S, Gamo, IKOT ABASI, and OKEARO), which may indicate locations with higher demand or those that are farther from generating stations. The greater voltages experienced by other buses (such as ALAIJA, Bus23, LEKKI, and YENOGOA) may be caused by their close proximity to renewable energy sources or by a lower local load. When evaluating voltage stability and identifying busses that could need strengthening or voltage management, the graph is helpful.

TABLE 1.: SIMULATION OF THE 52 BUS NETWORK

There are noticeable variations in the voltage profile across different buses in the simulated power network using hybrid renewable energy sources (wind and solar PV). Although most bus voltages stay within a reasonable range around the stated 100%, certain buses show notable variations. A number of buses, including ALAOJI T.S., IKOT ABASI, and OKEARO, exhibit undervoltage, with values that are close to or fall below 97%. These might be signs of poor network connectivity or strong local demand. Buses like ALADJA, Bus23, and YENOGOA exhibit overvoltage circumstances, with voltages exceeding 105%, which may cause connected equipment to be overstressed.

The variance in voltages across the network illustrates the impact of intermittent renewable energy, especially where municipal generation is not matched with suitable management techniques or reactive power compensation. The long-term economic benefits of hybrid solar and wind systems, such as low operating costs, energy security, grid dependability, and environmental benefits, make them an affordable option for sustainable power growth, even though the initial investment may be high. Strategic policy support and investment in grid infrastructure are crucial to fully achieving these economic gains.

| BUS ID | %MAG | ANG | BUS ID | %MAG | ANG | BUS ID | %MAG | ANG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AES | 99.068 | -5.7 | GEREGU | 99.865 | -2.0 | OSHOGBO | 99.903 | -1.9 |
| AFAM | 99.800 | -0.6 | GOMBE | 99.310 | 6.6 | OWERRI | 99.072 | -4.7 |
| AJA | 98.761 | 1.1 | GWAGWALADA | 99.272 | -3.9 | PAPALANTO | 100.007 | -1.4 |
| AJAKUTA | 99.859 | -2.0 | IKEJA WEST | 99.957 | -1.3 | PORT HARCOURT | 99.710 | -0.6 |
| AJAKUTA2 | 99.876 | -2.3 | IKOT ABASI | 99.815 | -0.6 | SAKETE | 99.963 | -1.3 |
| AKANKPA | 99.963 | -1.3 | IKOT EKPENE | 99.809 | -0.6 | SAPELLE | 100.026 | -1.8 |
| ALADE | 99.053 | -4.0 | JALINGO | 99.187 | -16.1 | SHIHIRO | 99.216 | -4.0 |
| ALADJA | 99.893 | -1.9 | JEBBA | 99.566 | -3.9 | YOLA | 98.906 | -14.8 |
| ALAGBON | 97.550 | 2.3 | JOS | 97.869 | -4.5 | OSHOGBO | 99.903 | -1.9 |
| ALAHDE | 99.961 | -1.6 | KADUNA | 98.888 | -4.1 | OWERRI | 99.072 | -4.7 |
| ALAOJI | 99.865 | -0.6 | KANJI | 99.699 | -4.2 | PAPALANTO | 100.007 | -1.4 |
| BENIN | 99.950 | -1.6 | KANO | 104.480 | 45.8 | PORT HARCOURT | 99.710 | -0.6 |
| BENIN KEBBI | 100.041 | -7.7 | KATEMPE | 99.317 | -3.7 | SAKETE | 99.963 | -1.3 |
| BININ NORTH | 99.884 | -1.7 | LOKOJA | 99.751 | -2.4 | SAPELLE | 100.026 | -1.8 |
| Bus13 | 101.355 | -6.0 | MADUGURI | 98.918 | -6.3 | SHIHIRO | 99.216 | -4.0 |
| CALABAR | 99.795 | -0.6 | MARKUDI | 98.466 | -4.3 | YOLA | 98.906 | -14.8 |
| DAMATURU | 99.103 | -6.2 | NEW HEAVEN 1 | 99.631 | -3.7 |
| DELTA | 99.959 | -1.7 | NEW HEAVEN 2 | 99.807 | -3.0 |
| egbema | 99.030 | -5.2 | OKPAI | 99.927 | 0.0 |
| *EGBIN | 330.000 |  | Omoku | 98.940 | -5.3 |
| EYEAN | 99.990 | -1.9 | OMOTOSHO | 99.959 | -1.4 |
| GANMO | 99.677 | -2.9 | ONITSHA | 103.114 | -3.7 |

| BUS ID | %MAG | ANG | BUS NO. | BUS ID | %MAG | ANG | BUS ID | %MAG | ANG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AES | 103.060 | -3.7 |  | GEREGU | 99.911 | -2.0 | OSHOGBO | 99.903 | -1.9 |
| AFAM | 98.200 | -0.3 |  | GOMBE | 99.322 | 16.6 | OWERRI | 99.072 | -1.7 |
| AJA | 100.761 | 2.8 |  | GWAGWALADA | 100.272 | -3.9 | PAPALANTO | 100.007 | -1.4 |
| AJAKUTA | 101.259 | -2.0 |  | IKEJA WEST | 101.757 | -1.3 | PORT HARCOURT | 99.710 | -0.6 |
| AJAKUTA2 | 99.356 | -2.3 |  | IKOT ABASI | 99.995 | -0.6 | SAKETE | 96.663 | -1.3 |
| AKANKPA | 99.993 | -1.3 |  | IKOT EKPENE | 99.809 | -0.6 | SAPELLE | 102.026 | -1.8 |
| ALADE | 97.053 | -4.0 |  | JALINGO | 99.187 | -16.1 | SHIHIRO | 99.216 | -4.0 |
| ALADJA | 99.993 | -1.5 |  | JEBBA | 99.266 | -3.9 | YOLA | 98.906 | -12.6 |
| ALAGBON | 98.550 | 1.4 |  | JOS | 97.163 | -4.5 | OSHOGBO | 100.103 | -1.3 |
| ALAHDE | 99.969 | -1.6 |  | KADUNA | 99.668 | -4.1 | OWERRI | 99.072 | -2.8 |
| ALAOJI | 99.865 | -0.4 |  | KANJI | 99.331 | -4.2 | PAPALANTO | 100.007 | -1.4 |
| BENIN | 99.950 | -1.2 |  | KANO | 103.900 | 35.8 | PORT HARCOURT | 99.710 | -0.6 |
| BENIN KEBBI | 101.041 | -3.3 |  | KATEMPE | 99.317 | -3.7 | SAKETE | 98.963 | -1.3 |
| BININ NORTH | 99.884 | -1.7 |  | LOKOJA | 99.751 | -2.4 | SAPELLE | 100.076 | -1.6 |
| Bus13 | 104.355 | -4.0 |  | MADUGURI | 98.918 | -4.3 | SHIHIRO | 98.162 | -3.0 |
| CALABAR | 99.795 | -0.6 |  | MARKUDI | 98.116 | -4.3 | YOLA | 105.00 | -12.8 |
| DAMATURU | 99.103 | -7.2 |  | NEW HEAVEN 1 | 99.316 | -3.7 |
| DELTA | 102.959 | -1.1 |  | NEW HEAVEN 2 | 99.207 | -3.0 |
| egbema | 98.130 | -2.2 |  | OKPAI | 104.971 | 0.0 |
| *EGBIN | 330.000 |  |  | Omoku | 98.640 | -5.3 |
| EYEAN | 99.995 | -1.4 |  | OMOTOSHO | 99.339 | -1.4 |
| GANMO | 99.977 | -2.1 |  | ONITSHA | 103.112 | -3.7 |

Fig.7.: Graph of Voltages against Buses Simulation Results of the network model with Hybrid (Solar Pv & Wind)

TABLE 2: CASE 1, SIMULATION OF THE 52 BUS NETWORK WITH WIND AND SOLAR PV

Table 3.: Branch Results of the Grid network with Hybrid (Solar Pv & Wind)

| From Bus | To Bus | kW Flow | kvar Flow | Amp Flow | % PF | % Voltage Drop | KW Losses | kvar Losses |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kno | damaturu | 18587.3 | -39918.8 | 77.53 | -42.21 | 6.36 | 27567.3 | 744.3 |
| kaduna | shiroro | 497475 | 287726 | 1010 | 86.56 | 155 | 1811479 | -1671258 |
| kebbi | KANJI | 14539.1 | -95972.9 | 182.1 | -14.98 | 7.26 | 4135.7 | -102422 |
| KANJI | jebba | 266945 | -212247 | 581 | -78.27 | 11.1 | 196206 | 40122.7 |
| kno | kaduna | 35904.6 | -45753.5 | 102.2 | -61.73 | 0.24 | 727 | -9983.7 |
| shiroro | jebba | 1973440 | -928183 | 3715 | -90.49 | 451.3 | 10388315 | 3808744 |
| kaduna | jos | 44183.2 | 92124.4 | 226.4 | 43.24 | 24.3 | 21813.3 | -39717.7 |
| gombe | jos | 95546.7 | -147694 | 389.8 | -54.32 | 11.53 | 97178.1 | -8166.7 |
| damaturu | gombe | 3553.1 | -77125.2 | 152.2 | -4.6 | 6.16 | 6320.6 | -39823.8 |
| marduguru | damaturu | 5609.3 | -75565.3 | 141.1 | -7.4 | 5.45 | 4275 | -75701.1 |
| yola | gombe | 4245.3 | -61919.5 | 122.4 | -6.84 | 4.36 | 2936.1 | -62146.3 |
| jos | makurdi | 365657 | -150642 | 620.4 | -92.46 | 38.38 | 314027 | 82783.8 |
| jebba | osogbo | 84528 | -1399268 | 1168 | -6.03 | 126.4 | 116000 | -1835994 |
| jebba | ganmo | 511726 | -1146850 | 2139 | -40.75 | 47.57 | 989804 | 497039 |
| ganmo | osogbo | 469632 | 1630048 | 2074 | 27.68 | 78.83 | 858082 | 393488 |
| osogbo | IKEJA WEST | 2734889 | 2463341 | 3067 | 74.3 | 42.29 | 587576 | 239635 |
| osogbo | AYEDE | 502432 | 526372 | 606.4 | 69.05 | 45.6 | 113498 | -178114 |
| AYEDE | IKEJA WEST | 3594976 | 1600601 | 2801 | 91.35 | 3.31 | 39648.4 | 18148.3 |
| IKEJA WEST | SAKETE | 63868.6 | 25551.9 | 49.07 | 92.85 | 0.66 | 143.5 | -48737.1 |
| IKEJA WEST | AKANGBA | 391038 | 197196 | 311.7 | 89.29 | 3.94 | 5277.9 | -38071.6 |
| EGBIN G.S | AJA | 1114305 | 512154 | 861.9 | 90.86 | 8.6 | 32319.6 | -17573.6 |
| LEKKI | ALAGBON | 178728 | 115419 | 145.6 | 84.01 | 2.31 | 1377.7 | -56755.5 |
| AJA | ALAGBON | 100234 | 108534 | 100.9 | 67.85 | 1.57 | 609.1 | -62846.4 |
| AJA | LEKKI | 116717 | 21283.1 | 81.2 | 98.38 | 0.74 | 281.1 | -33602 |
| IKEJA WEST | OKEARO | 49762.9 | 100332 | 79.7 | 44.43 | 1.19 | 339.6 | -71281 |
| EGBIN G.S | OKEARO | 132756 | 61456.9 | 103.7 | 90.75 | 2.47 | 1078.2 | -85404.3 |
| Bus11 | KATEMPE | 1364628 | -1184781 | 1097 | -75.51 | 58.25 | 474079 | -98048.6 |
| Bus11 | GWAGWALADA | 1415635 | -1199577 | 1120 | -76.29 | 61.39 | 497188 | -87801.7 |
| LOKOJA | GWAGWALADA | 415429 | -1729079 | 1074 | -23.36 | 131.2 | 474810 | -1849950 |
| GWAGWALADA | KATEMPE | 3458321 | -1449160 | 2264 | -92.23 | 3.14 | 25916.6 | 9595 |
| LOKOJA | AJAOKUTA | 61401.2 | -122627 | 497.7 | -44.77 | 0.15 | 3882.7 | 1712.4 |
| Bus9 | BENIN | 80165.3 | -450899 | 878.9 | -17.5 | 68.54 | 414610 | 86410.6 |
| BENIN | EGBIN G.S | 352846 | 715575 | 934.8 | 44.23 | 117.4 | 485636 | -108061 |
| BENIN | OMOTOSHO | 215944 | 592371 | 738.7 | 34.25 | 50.06 | 180311 | -17694.3 |
| OMOTOSHO | IKEJA WEST | 402403 | 577367 | 642 | 57.18 | 63.69 | 165096 | -162753 |
| BENIN | IHOVBOR | 856483 | 1916714 | 2460 | 40.8 | 42.19 | 527941 | 258226 |
| osogbo | IHOVBOR | 1232008 | 2096635 | 2298 | 50.66 | 29.27 | 345177 | 133116 |
| sapelle TS | ALADJA | 79909.2 | 512329 | 573.1 | 15.41 | 13.92 | 58870.6 | -24071.6 |
| Bus13 | ALADJA | 199809 | -127015 | 257.7 | -84.39 | 2.43 | 5925.1 | -21493.7 |
| EGBIN G.S | IKORODU | 103363 | 145950 | 125.7 | 57.79 | 1.4 | 748.8 | -35432 |
| AES | EGBIN G.S | 145232 | 172604 | 158.5 | 64.38 | 4.6 | 2692.8 | -115919 |
| IKORODU | SHAGAMU | 46282.1 | -56484.5 | 51.08 | -63.38 | 0.54 | 124.8 | -74152.6 |
| UGWUAJI | makurdi | 339333 | -158045 | 566.3 | -90.65 | 4.13 | 19962.2 | 2393.5 |
| UGWUAJI | NEW HEAVEN | 579001 | -38920.7 | 855.8 | -99.77 | 2.98 | 15192.4 | 5238.6 |
| NEW HEAVEN | ONITSHA | 564822 | -45412.9 | 683.1 | -99.68 | 26.51 | 111168 | 18608.2 |
| ONITSHA | BENIN | 26937.7 | 81971.2 | 104 | 31.22 | 4.93 | 1234.8 | -95354.5 |
| BENIN | ASABA | 1176039 | 1692180 | 2451 | 57.07 | 2.2 | 30370.2 | 15184.4 |
| ONITSHA | ASABA | 1141987 | 1331900 | 2115 | 65.09 | 1.98 | 22619.2 | 11049.3 |
| ONITSHA | Bus15 | 518564 | -1183291 | 1783 | -40.14 | 18.36 | 548021 | 261182 |
| ONITSHA | ALAOJI T.S | 360013 | -23689.5 | 677.6 | -99.78 | 51.98 | 201108 | 41838.2 |
| ALAOJI T.S | OWERRI | 601227 | -46746 | 1133 | -99.7 | 30.38 | 221162 | 91904.6 |
| OWERRI | AHOADA | 355000 | -149195 | 1073 | -92.19 | 6.8 | 64973.3 | 32924.4 |
| Bus18 | YENOGOA | 81562.1 | 26288.5 | 297.9 | 95.18 | 3.81 | 5270.3 | 1166.6 |
| IKOT EKPENE | ALAOJI T.S | 90557.3 | -55585.3 | 194.6 | -85.23 | 2.36 | 4287.1 | -8630.5 |
| UGWUAJI | IKOT EKPENE | 138978 | 122168 | 338.9 | 75.11 | 20.14 | 27880.7 | -30680.6 |
| ALAOJI T.S | ABA | 35561 | 52979.7 | 119.8 | 55.73 | 7.21 | 3535.5 | -36908.9 |
| ITU | ABA | 23400.5 | 20785.6 | 54.56 | 74.76 | 2.64 | 583.9 | -36466.3 |
| ITU | EKET | 5281.4 | 40213.2 | 68.89 | 13.02 | 1.49 | 337 | -47367.6 |
| Line62~ | IKOT ABASI | 9130.9 | -8245.7 | 22.02 | -74.22 | 0.12 | 19.78 | -6309.6 |
| IKOT EKPENE | IKOT ABASI | 56570.8 | 8160.3 | 104.7 | 98.98 | 2.22 | 1285 | -10782.1 |
| IKOT EKPENE | AFAM | 46063.7 | 166522 | 336 | 26.66 | 5.56 | 8215.3 | -39613.7 |
| Bus17 | AFAM | 55081.3 | -232094 | 463.9 | -23.09 | 1.67 | 16393.6 | 176.1 |
| IKOT EKPENE | ODUKPANI TS | 48597.9 | 59290.3 | 147.7 | 63.39 | 4.7 | 3396.4 | -14460.9 |
| Bus20 | ADIABO TS | 140308 | 82942.5 | 278.8 | 86.08 | 3.11 | 3707.7 | -29663.1 |
| ITU | ADIABO TS | 26240.2 | -79650.3 | 143.4 | -31.29 | 0.72 | 2968.7 | -18161.3 |
| AFAM | Bus23 | 93454.4 | -337274 | 721.9 | -26.7 | 5.14 | 69737.8 | 20632 |
| Bus23 | PH MAIN TS | 375207 | -249650 | 779.1 | -83.25 | 16.38 | 275597 | 80718.6 |
| BENIN | Bus7 | 247460 | -406313 | 552.9 | -52.02 | 1.43 | 44517.1 | -20276.7 |
| BENIN | Bus12 | 41107.3 | -451769 | 594.7 | -9.06 | 18.71 | 101302 | -30994.8 |
| KANJI | Bus4 | 45953 | -162824 | 296 | -27.16 | 0.06 | 2.19 | 98.54 |
| Bus8 | jebba | 699103 | -693357 | 3125 | -71 | 0.23 | 61.02 | 2745.9 |
| Bus11 | shiroro | 13004264 | 4984159 | 9305 | 93.38 | 3.74 | 5410.5 | 243471 |
| EGBIN G.S | IKEJA WEST | 109593 | 1715925 | 1208 | 6.37 | 3.66 | 476.9 | 21459.8 |
| AJAOKUTA | Bus9 | 88667.2 | -182696 | 389.7 | -43.66 | 1.38 | 65.27 | 2937.1 |
| Bus7 | sapelle TS | 266786 | 480060 | 638.3 | 48.58 | 5.2 | 365.6 | 18280.3 |
| Bus12 | Bus13 | 168927 | -341746 | 415 | -44.31 | 3.45 | 205.6 | 9250.6 |
| Bus14 | AJAOKUTA | 37780.3 | -55810.8 | 2444 | -56.06 | 0.09 | 1.47 | 66.29 |
| Bus15 | OKPAI T.S | 126281 | -301313 | 448.5 | -38.65 | 0.68 | 38.1 | 1905.2 |
| ALAOJI T.S | Bus17 | 18532.8 | -285471 | 537.3 | -6.48 | 1.53 | 104.8 | 4714 |
| AHOADA | Bus18 | 38857.3 | -96306.6 | 324.6 | -37.42 | 0.61 | 27.32 | 1229.2 |
| ODUKPANI TS | Bus20 | 234896 | 76404.3 | 475.9 | 95.1 | 14.58 | 89.65 | 4034.2 |
| PH MAIN TS | Bus24 | 19965.3 | -2455 | 26.08 | -99.25 | 0.02 | 1.1 | 49.35 |
| PH MAIN TS | Bus25 | 73805.2 | -60683.8 | 165.1 | -77.24 | 0.02 | 130.1 | 195.1 |

5.0	CONCLUSION

Energy plays a critical role in the economy of any Nation, and enhance her prosperity and development. A Country like Nigeria that is naturally blessed with diverse natural resources located in different Geographical zones such as abundant sun (solar) and wind, and with the current surge in population, there is need to boost the Generation capacity of the current National Grid by adding a Hybrid energy system (Solar PV, Wind) into the present Grid Network. Hybrid energy system helps the present Grid Network by increasing the capacity of energy generated and providing constant energy to the dense population of Nigeria with diverse economic activities, hence guaranteeing energy stability, reliability and security. The increment of Solar PV and Wind energy (Hybrid energy system) in Nigeria helps in reducing Carbon emissions drastically making Nigeria to fulfill her international pledge of Carbon reduction with this shift towards renewable energy sources, and also significant reduction in maintenance cost, hence saving funds for other Infrastructural developments for the benefits of Nigerian citizens. The constant power blackout and outages have brought hardship to Nigerians and can be reduced drastically with the use of solar and wind (Hybrid energy system). Increased energy reliability and sustainability is crucial for economic expansion and continuous development, employment opportunities in different sectors of the economy such as manufacturing, operations and maintenance are facilitated by the development and installation of solar and wind (Hybrid and wind energy system) infrastructure. This helps in reducing unemployment situation and brings about the much-anticipated economic development of the Nation. The economy of Nigeria can be more diverse with the help of renewable energy sources. Effective energy storage policies are needed to manage the erratic nature of renewable energy sources (solar and wind) by constructing energy storage facilities, which will be crucial for energy preservation and a steady supply of electricity.

# COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

[1] Ake David, Asuquo Eke, Okon Ekpenyong, Gertrude Fischer, Akpama E. J. Modeling 72 bus networks with high penetration of hybrid (Solar PV and Wind) renewable energy for Nigeria's future energy sustainability and Economic Development. International Journal of Engineering and Science Invention (IJESI) www.ijesi.org ||Volume 13 Issue 6 June 2024 || PP 61-67, 2024

[2] M. Mustafa, G. Anadhakumare, A.A. Jacob, P. Singh, S.Asha, S.A. Jayadhas, “Hybrid renewable power generation for modelling and controlling the battery storage photovoltaic system”, Int. J. Photo energy 94901808 (2022) , https:// doi.org/10.1155/2022/9491808

[3] Amisa Emram, Asmae Bernada, “A comprehensive review on techno-economic assessment of hybrid energy storage systems integrated with renewable energy.

[3] Hoxha, Bukurije, and Risto FILKOSKI. “Role of wind energy in sustainable development in coal-based systems: Case of Kososvo.” Journal of Energy Systems 7.2 (2022) : 187 – 198. 2022

[4] Zhang, C., Y., Deng, Z.Y. “Robustly coordinated operation of a multi energy micro-grid in grid-connected and islanded modes under uncertainties.”  IEEE Trans. Sustain. Energy 2020,11, 640-651.

[5] Zheng, S.; Hai, Q.; Zhou, X.; Stanford, R.J. “A nove multi-generation system for sustainable power, heating, cooling, freshwater, and methane production: “Thermodynamic, economic, and environmental analysis.” Energy 2024, 290, 130084

[6] Hong, B.; Chen, J.; Zhang, W.; Shi, Z.; Li, J.; Miao, W. “Integrated energy system planning at modular regional-user level based on a two-layer bus structure. CSEE J. Power Energy Syst. 2018, 4, 188-196

[7] H. Zhao.; Q Wu.; S hu.; 2015 “Review of energy storage system for wind power integration support” Appl Energy 137545553 https:// doi.org/10.1.16/j.apenergy. 2014.04.103

[8] Tabatabaee, Sajad, Seyed Saeedallah Mortazavi, and Taher Niknam.; “Stochastic scheduling of local distribution systems considering high penetration of plug-in electric vehicles and renewable energy sources” Energy 121 (2017): 480-490. 2017

[9] Telukunta, Vishnuvardham, et al. “Protection challenges of renewable energy sources in power systems: A review.” CSEE Journal of power and energy systems 3.4 (2017): 365-379

[10] L. Olatomiwa, S. Mekhilef, A.S.N. Huda, O.S. Ohunakin, “Economic evaluation of hybrid energy systems for rural electrification in six geo-political zones of Nigeria.” Renew Energy 83 (2015) 435–446, https://doi.org/10.1016/j. renene.2015.04.057.

[11] Y.F. Nassar, H.J. El-khozondar, A.A. Ahmed, A. Alsharif, M.M. Khaleel, R.J. El- Khozondar.  “A new design for a built-in hybrid energy system, parabolic dish solar concentrator and bioenergy (PDSC/BG): a case study – Libya.” J. Clean.

Prod. 441 (15) (2024) 140944, https://doi.org/10.1016/j.jclepro.2024.140944.

[12] A. Elmariami, W. El-Osta, Y. Nassar, et al., “Life Cycle Assessment of 20 MW Wind Farm in Libya.” Appl. Sol. Energy 59 (2023) 64–78, https://doi.org/10.3103/ S0003701X22601557

[13] Hong-Sung Kim, Naotaka Okada, Kiyoshi Takigawa, “Advanced grid connected PV system with functions to suppress disturbance by PV output variation and customer load change.” Sol. Energy Mater. Sol. Cells 67 (2001) 559–569.

[14] H.J. El-Khozondar, F. El-batta, R.J. El-Khozondar, Y. Nassar, M. Alramlawi, S. Alsadi, “Standalone hybrid PV/wind/diesel-electric generator system for a COVID- 19 quarantine centre, Environ.”   Prog. Sustain. Energy (2023), 10.1002/ep.14049.

[15] Y.F. Nassar, M.J. Abdunnabi, M.N. Sbeta, A.A. Hafez, A. Ahmad, K.A. Amer, Y. Ahmed, Y. Abdulaziz, B. Belgasim, “Dynamic analysis and sizing optimization of a pumped hydroelectric storage-integrated hybrid PV/Wind system: a case study.” Energy Convers. Manage. 229 (2021) 113744, https://doi.org/10.1016/j. enconman.2020.11374.

[16] O.O. Osaloni, A.S. Akinyemi, A.A. Adebiyi, A.O. Salau, “An Effective Control Technique to Implement an IUPQC Design for Sensitive Loads in a Hybrid Solar PV- Grid Connection.” WSEAS Transactions on Power Systems 18 (2023) 26–38, https:// doi.org/10.37394/232016.2023.18.4.

[17] H. Awad, Y.F. Nassar, A. Hafez, M.K. Sherbiny, A.F.M. Ali, “Optimal design and economic feasibility of rooftop photovoltaic energy system for Assuit University, Egypt.” Ain Shams Eng. J. 13 (3) (2022) 101599, https://doi.org/10.1016/j. asej.2021.09.026.

[18] Y.F. Nassar, S.Y. Alsadi, H.J. El-Khozondar, et al., “Design of an isolated renewable hybrid energy system: a case study.” Mater Renew Sustain Energy 11 (2022) 225–240, https://doi.org/10.1007/s40243-022-00216-1.

[19] Y.F. Nassar, S.Y. Alsadi, “Assessment of solar energy potential in Gaza Strip- Palestine.”  Sustainable Energy Technol. Assessments 31 (2019) 318–328, https:// doi.org/10.1016/j.seta.2018.12.010.

[20] RET Screen International Clean Energy Decision Support Centre. “Wind project Analysis Chapter.” Canada (2005) 1-84. [Online]www.retscreen.net.

[21] G. Bekele, “The Study into the Potential and Feasibility of Standalone Solar-Wind Hybrid Electric Energy Supply System For Application in Ethiopia.” KTH Royal Institute of Technology, Stockholm, Sweden,Ph.D. thesis (2009) 1-136.

[22] H. Ibrahimab, A. Illincaa, J. Perronb, “Comparison and analysis of different energy storage techniques based on their performance index.” 393 (2007) 398, https://doi. org/10.1109/epc.2007.4520364.

[23] E. Muljadi, J.T. Bialasiewicz, “Hybrid power system with a controlled energy storage.” IECON’03. 29th Annual Conference of the IEEE Industrial Electronics Society (IEEE Cat. No.03CH37468), Roanoke, VA, USA, (2003) 1296-1301, doi:10 .1109/IECON.2003.1280240.