Modelling the Soil Wetting Pattern under Pulse and Continuous Drip Irrigation

Abstract:

There were some problems that face continuous drip irrigation system in Gujarat, India. such as emitter clogging, due to the narrow paths of water which give small discharge and in sandy soil, the depth of wetting pattern is relatively higher than the width. They cause deep percolation beyond root zone. Thus, the need to increase the width of wetting pattern and use of high emitter discharge suggest the use of pulse drip irrigation. The information on depths and widths of wetted zone of soil plays the greatest role in design and management of drip irrigation system. There is a lack of models to predict wetting pattern under pulse and intermittent flow regime, since the applicability of the available models were limited to continuous flow regime only. Therefore, the objective of this research was to develop a dimensional analysis model to estimate both depth and width of wetting pattern under different flow regimes. Thus, a model was developed using semi-empirical approach and dimensional analysis method for determining geometry of wetted root zone under. The predicted values of wetted depth and width were compared with those obtained through field level experiments conducted. Experiment included determination of maximum depths and widths of wetted zone after 30, 60 and 120 min of water application under different flow regimes (2, 4 and 8 LPH). Predicted and observed values were compared to test model applicability in field conditions. On the basis of root mean square, mean error and model efficiency parameters model performance was found good. Thus, developed models can be used to predict wetting pattern under continuous, intermittent and pulse flow with line source of water application. Pulse flow results showed that the wetted width increased and wetted depth decreased as the operating on-time decreased for the same amount of applied water. As the pulsed flow was increased from one to four times the continuous flow, deep percolation was reduced and horizontal spread increased. This result shows the advantage of pulse flow, for reducing the deep percolation of water under the crop root zone, while obtaining a wide horizontal spread of wetting. This enables using a highly discharge emitter with the same amount of water.

Key words: Drip irrigation Dimensional analysis Soil wetting pattern Pulse flow Intermittent    flow Continuous flow

INTRODUCTION

Gujarat is located in a semi-arid to arid climate zone, where rainfall is unevenly distributed and often scarce, particularly in the northern and western regions. A large portion of the state experiences frequent droughts, and desert-like conditions prevail in areas such as Kutch. The main sources of fresh water for Gujarat include surface water from rivers such as the Narmada, Tapi, Mahi, and Sabarmati, along with limited groundwater resources. The Sardar Sarovar Project on the Narmada River has become a vital source for irrigation, drinking, and industrial water supply across the state (Government of Gujarat, 2023). In addition to this, groundwater reservoirs are used extensively, though overexploitation in several regions has led to declining water tables and salinity intrusion (Central Ground Water Board, 2023).

The total irrigated area in Gujarat is around 5.0 million hectares and is expected to increase further through improved irrigation efficiency and the adoption of modern irrigation techniques such as drip and sprinkler systems, particularly in water-scarce regions (Department of Agriculture, Farmers Welfare and Co-operation, Government of Gujarat, 2023). Meanwhile, the water demand continues to grow rapidly due to population increase, industrial expansion, and rising living standards. As a result, the per capita availability of water has been steadily declining and, in several regions, has approached the “water stress” or “water scarcity” threshold (NITI Aayog, 2022). Therefore, water shortage remains a critical challenge and one of the major constraints to achieving sustainable agricultural and economic development in the state (Patel & Shah, 2021).

Drip irrigation has the tendency to increase water use efficiency only if the system is designed to meet the soil and plant conditions. Information on wetting pattern width and depth under emitters is a prerequisite for the design and operation of drip irrigation systems. This ensures precise placement of water and fertilizer in the crop root zone. Among the various problems associated with drip irrigation, emitter clogging is considered the most serious issue. Water losses from deep percolation and the small horizontal wetted width in sandy soils are also challenges when applying drip irrigation in such soils. An alternative approach to mitigate clogging is to increase the size of the water passage in emitters; however, this may raise the emitter discharge, consequently changing the wetting pattern in the soil profile.

Oron (1996) reported that continuous water application is associated with increased water percolation below the root zone. Using higher emitter discharge in the form of intermittent application is termed pulse flow. The principles of pulsing were first proposed by Karmelli and Peri (1974), who described a pulse as consisting of an operating phase—during which water is applied to the soil—and a resting phase when the flow stops. The average pulsed discharge equals the continuous discharge; thus, the same amount of water can be applied using either method, provided the irrigation period or cycle is the sum of operating and resting phases.

Jackson and Kay (1988) demonstrated that the effect of pulsing depends not only on soil type but also on discharge rate and operating/resting times. A key factor limiting high pulsed discharges is surface ponding—once this exceeds local infiltration capacity, ponding spreads rapidly, potentially altering the wetting pattern, causing runoff, and increasing soil erosion. Although pulsing may reduce clogging, it can increase system costs due to the need for larger pipes and automatic sequencing valves. However, these costs can be offset by reduced maintenance and emitter replacement. Pumping costs remain largely unaffected if the pulsed discharge matches that of a continuous system, achievable through careful selection of the number of trickle laterals operated during each pulse.

Mostaghimi and Mitchell (1983) found that pulsed drip irrigation significantly reduces deep percolation losses in sandy soils and enhances lateral water spread. Similarly, Levin and Van Rooyen (1979) reported that pulsing reduces percolation losses and that wetting patterns under pulsed and continuous flows are nearly identical at low discharges from point sources. Intermittent irrigation, based on discharge pulses followed by breaks, can improve water management and efficiency by aligning mean irrigation rates with soil hydraulic conductivity and minimizing deep percolation losses (Oron, 1996).

Schwartzman and Zur (1986) developed a simplified semi-empirical method to determine the geometry of wetted soil zones under line-source water application for continuous flow regimes. Their model, derived via dimensional analysis, predicts the wetting front position under surface drip irrigation as a function of applied water and basic soil properties such as saturated hydraulic conductivity. This approach simplifies the design process by eliminating the need for complex numerical or analytical models. While information on matric potential distribution is not always necessary, data on the width and depth of the wetted zone remain essential for field design (Zur, 1986).

However, there is a lack of predictive models for wetting patterns under pulsed and intermittent flow regimes, as most existing models apply only to continuous flow. Therefore, there is a strong need to develop a dimensional analysis model capable of predicting wetted soil depth and width under different flow regimes with line sources of water application.

MATERIALS AND METHODS

A field experiment was conducted during the winter season of 2021–22 at Muvaliya Farm, Anand Agricultural University, Dahod, located at 22°44′–22°58′ N latitude and 74°07′–74°27′ E longitude, with an elevation of 217 m above mean sea level. The region falls under the Middle Gujarat Agro-Climatic Zone-III and is characterized by a semi-arid climate, receiving 700–900 mm of annual rainfall, mostly during the southwest monsoon (June–September). Average temperatures range from 9.5 °C to 47 °C, with relative humidity between 45 % and 80 %. The experimental soil was sandy loam, well-drained, and moderately fertile with pH 7.3–7.5, low organic carbon, medium phosphorus, and high potassium content.

Emitter discharge characteristics and manufacturing variation were evaluated under operating pressures ranging from 0.8 to 1.7 kg/cm², following ASAE (1996) and ISO (1991) standards. The field was well-prepared using a tractor-drawn cultivator and rotavator to obtain fine tilth, ensuring uniform soil conditions for accurate assessment of wetting patterns under pulse and continuous flow drip irrigation systems.

Experimental setup for soil wetting geometry.

In the present study, the effects of pulse flow compared to continuous flow on soil wetting geometry and moisture distribution patterns were evaluated under varying discharge rates and pulse frequencies. Two experimental factors were considered: emitter discharge rate and the number of pulses applied. Three emitter discharge rates—2, 4, and 8 liters per hour (lph)—and four pulse combinations (1, 2, 3, and 4 pulses) were tested. The total volume of water applied was kept constant across all treatments, with continuous flow converted into equivalent pulsed intervals. For example, a 4 lph emitter operated continuously for 2 hours (delivering 8 liters) was pulsed into two 1-hour irrigation events separated by a 1-hour rest period. All treatment combinations followed a similar on–off pulsing schedule.

At the end of each irrigation event, the soil surrounding the emitter was excavated to expose the vertical soil profile and to measure the wetting front in both horizontal and vertical directions. A coordinate system was established with the origin at the soil surface directly above the emitter, and plastic sheets were used to minimize evaporation during observation. The maximum wetting distances in each direction were recorded, and soil samples were collected from specific coordinates within the wetted zone. The samples were analyzed to determine soil moisture content using the gravimetric method, providing data on the distribution and extent of moisture movement under different flow regimes.

Table 1: Details of Experimental Factors

| Main Factor: Discharge Rate | Sub Factor: Number of Pulse |
| --- | --- |
| Q1 = 2 LPH Q2 = 4 LPH Q3 = 8 LPH | P1 = Continues flow P2 = two pulse (total time of operation divide in two pulse) P3 = three pulse P4 = four pulse |

Table 2: Details of Various Flow and Pules Combination

| Q (lph) | Time (min) | Volume (lit.) | Number of pulse(min) |  |  |  |  |  |  |  |  |  |  | Total time for operation(min) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P1 | P2 |  | P3 |  |  | P4 |  |  |  | P1 |  | P2 | P3 | P4 |
| 2 | 30 | 1 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 2 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 4 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |
| 4 | 30 | 2 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 4 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 8 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |
| 8 | 30 | 4 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 8 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 16 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |

Determination of Wetting Pattern Dimension

The laterals used for determination of wetting patterns were 16 mm having drippers of inline with 2 lph and 4 lph. The laterals were connected to sub
main line. The system was continuously operated for 2 hours and 1st time zone’s cock
valve was closed off after 30 minutes operating time, similarly it was repeated for all operating time zone i.e. 30, 60 and 120 minutes. After that system was turned off and wetting pattern dimension was measured.

Development of mathematical model

Schwartzman and Zur (1986) developed a simplified semi empirical method for determining the geometry of wetted soil zone under line sources of water application placed on surface for continuous flow regime. They assumed that the geometry of wetted soil (the width and depth of wetting at the end of irrigation) depends on the soil type, emitter discharge per unit length of laterals and soil water content in the soil. The soil type was represented by the saturated hydraulic conductivity.

z = f1 (Qc, Ks, V)                                                        (1)

d = f2 (Qc, Ks, V)                                                        (2)

where f1 and f2 are function signs; 
z Is the wetted soil depth (cm).
d Is the wetted soil width (cm)
Qc Is the discharge rate of continuous drip irrigation (cm3/ min).
Ks Is the saturated hydraulic conductivity of soil (cm / min)

T Is irrigation time (min)

z = αz Ksa Qcb Vc                                                         (3)

d = αd Ksa Qcb Vc                                                        (4)

Where αz, αd, a, b and c are constant

Ismail et al. (2014) modified Schwartzman and Zur (1986) equation and introduce two term. they suggested that discharge rate is not same in pulse flow compare to continuous one, because on and off system of operation it was change. And second one was cycle ratio, which is ratio of system operating time with total time of one cycle. And they developed relationship of wetted soil depth and width.

Pulse and Intermittent Flow: The dimensions of wetted pattern of pulse and intermittent drip irrigation depend on water discharge rate, soil saturated hydraulic conductivity and accumulated cycle times. There were two separate functional relationships of pulse and intermittent drip irrigation, one for wetted soil depth (z) and the other for wetted soil width (d). The two functions can be written as follows:

z = f1 (Qp, Qc, Ks, T)                                                   (5)

d = f2 (Qp, Qc, Ks, T)                                                   (6)

where f1 and f2 are function signs; 
z Is the wetted soil depth (cm).
d Is the wetted soil width (cm).
Qc Is the discharge rate of continuous drip irrigation (cm3/ min).

Qp Is the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ).
Ks Is the saturated hydraulic conductivity of soil (cm / min)

T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

n Is number of cycles.

The basic dimension of each variables can be expressed as follows:

z = L, d = L, Qp = L3T-1, Qc = L3T-1, Ks = LT-1 and T = T

The number of π terms (free variables) is equal to (n-m), where n=5 which is the total number of variables in the experiment (z, Qp, Qc, Ks, T) and m=2 which is the number of reference dimensions (L, T).

free variables = n-m = 5-2 =3

F (π1, π2, π3) = 0                                             (7)

The dependent variables (repeating variables) were selected as Ks and Qc, which involve all fundamental dimensions. The π term was formed by grouping one of the free variables with all dependent variables, starting with the free variable, z, the first π term can be formed by combining z with the dependent variables such that

π1z = Ksa1. Qcb1. Z                                           (8)

Estimating a1, b1 so that the combination is dimensionless

(LT-1) a1.( L3T-1) b1. (L) = L0T0

So: a1+3b1= 0 (for L)  and -a1-b1= 0 (for T)

Therefore: a1= 1/2 and b1= -1/2

The π term them becomes

π1z = zKsQc                                            (9)

Repeating the preceding steps, the second and third π term can be obtained as follows:

π2z = QpQc                                                     (10)

π3z = TKs3Qc                                               (11)

Now base on all π terms one final equation for z term become like:

π1z = F (π2z , π3z)                                      (12)

π1z = az (π2z)bz (π3z)cz                                    (13)

The values of unknowns az, bz, cz and were estimated using the data of the laboratory experiments with curve fitting techniques then Substituting all π term then the final equation which estimates the depth of wetted pattern can be written as follows:

z = αz Ksa Qpb Qcc Td                                               (14)

d = αd Ksa Qpb Qcc Td                                              (15)

Where αz, αd, a, b, c and d are constant

In case of pulse flow, Qc can be calculated as follow;

Qc = CR Qp (16)

where the cycle ratio is, the ratio between the on time (Ton) and the cycle time (Ton+Toff), cycle ratio can be calculated as follow CR = TonTon+Toff (17)

RESULTS AND DISCUSSION

Since, simulation of soil water movements depends upon soil characteristic
requiring location specific information on the soil water movement pattern, the developed model must be validated against the experimental condition for their adoption. These models of the soil moisture movement from different point source of irrigation were simulated in order to find the best fit one for the sandy loam soils of the region.

Ismail et al. (2014) modified Schwartzman and Zur (1986) equation and introduce two term. T and Qp which is the elapsed time (min) and the discharge rate of pulse drip irrigation which changes according to the pulse ration. And generalize equation z = f1 (Qp, Qc, Ks, T) and d = f2 (Qp, Qc, Ks, T) (3.8 and 3.9). for solving this with empirical method used 3 π term introduce. And make other generalize equation 3.10 (F (π1, π2, π3) = 0) by formulating problem and then solving this equation finalize empirical equation was developed which is discuss below.

Depth of Wetting Front

The generalized expression for radius of spread below emitting point can be expressed as:

z = αz Ksa Qpb Qcc Td

And all constant can be determined by The Buckingham Pi theorem for that we have to solved which is π1z = az (π2z)bz (π3z)cz based on experimental data finalize equation is

π1z = 1.94*π2z0.95 *π3z0.51                       (18)

zKsQc=1.94*QpQc0.95* TKs3Qc0.51            (19)

Z = 1.94  Ks0.265 T0.51 Qp0.95 Qc-0.71                   (20)

where z is the wetted soil depth (cm), Qc Is the discharge rate of continuous drip irrigation (cm3/ min), Qp Is the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ), Ks Is the saturated hydraulic conductivity of soil (cm / min) and T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

Table 3: Parameter Estimates of the Samir Ismail model for predicting Depth of wetting front.

| Parameter | Estimate | Std. Err | 95% Confidence Interv |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Bound | Upper Bound |
| az | 1.94 | 0.09 | 1.85 | 2.03 |
| bz | 0.95 | 0.11 | 0.85 | 1.06 |
| cz | 0.51 | 0.05 | 0.46 | 0.55 |

Table 4: Correlations of Parameter Estimates for Depth of wetting front.

| Variable | π2z | π3z | π1z |
| --- | --- | --- | --- |
| π2z | 1.000 | -0.008 | 0.516 |
| π3z | -0.008 | 1.000 | 0.716 |
| π1z | 0.516 | 0.716 | 1.000 |

The value of az, bz and cz for predicting wetted soil depth was calculated to be 1.94, 0.95 and 0.51. The standard error for the model was varying from 0.05 to 0.11. The lower bound and upper bound for az, bz and cz were also given in table 4. The correlation matrix for the parameter estimates for wetted soil depth is presented in table 4. The coefficient of determination was found to be 0.844.

The scatter plot of observed Vs predicted wetted soil depth front from surface was plotted in fig. 1. A good agreement between the observed and predicted values were obtained. The residual plot for various observed wetted soil depth values is also shown in fig. 2. The residues in respect of the observed values were within the tolerable limits.

Fig. 1 scatter plot of observed Vs predicted wetted soil depth.

Fig. 2: Residual plot for observed values of wetted soil depth.

Radius of Spread

The generalized expression for radius of spread below emitting point can be expressed as:

d = αd Ksa Qpb Qcc Td

And all constant can be determined by The Buckingham Pi theorem for that we have to solved which is π1d = ad (π2d)bd (π3d)cd based on experimental data finalize equation is

π1d = 2.06 *π2d-1.32 *π3z0.21                       (21)

dKsQc=2.06*QpQc-1.32* TKs3Qc0.21            (22)

d = 2.06 Ks-0.185 T0.21 Qp-1.32 Qc1.81                     (23)

where d is the wetted soil width (cm), Qc Is the discharge rate of continuous drip irrigation (cm3/ min), Qp Is the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ), Ks Is the saturated hydraulic conductivity of soil (cm / min) and T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

Table 5: Parameter Estimates of the Samir Ismail model for predicting Radius of spread.

| Parameter | Estimate | Std. Err | 95% Confidence Interv |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Bound | Upper Bound |
| ad | 2.06 | 0.31 | 1.76 | 2.37 |
| bd | -1.32 | 0.28 | -1.60 | -1.03 |
| cd | 0.21 | 0.07 | 0.14 | 0.28 |

Table 6: Correlations of Parameter Estimates for Radius of spread.

| Variable | π2d | π3d | π1d |
| --- | --- | --- | --- |
| π2d | 1.000 | -0.022 | -0.635 |
| π3d | -0.022 | 1.000 | 0.317 |
| π1d | -0.635 | 0.317 | 1.000 |

The value of az, bz and cz for predicting Radius of spread was calculated to be 2.06, -1.32 and 0.21. The standard error for the model was varying from 0.07 to 0.31. The lower bound and upper bound for az, bz and cz were also given in table 5. The correlation matrix for the parameter estimates for radius of spread is presented in table 6. The coefficient of determination was found to be 0.840.

The scatter plot of observed Vs predicted Radius of spread at surface was plotted in fig.3. A good agreement between the observed and predicted values were obtained. The residual plot for various observed Radius of spread values is also shown in fig. 4. The residues in respect of the observed values were within the tolerable limits.

Fig. 3 scatter plot of observed Vs predicted Radius of spread.

Fig. 4: Residual plot for observed values of Radius of spread.

(Boundary Conditions and Applicability of develop model are: Soil type: sandy loam, Operating time: up to 120 min, Dripper discharge: 2 LPH to 8 LPH)

CONCLUSIONS

There is a lack of models to predict wetting pattern dimension under pulse and intermittent flow regime, since the applicability of the available models were limited to prediction under continuous flow regime only.

Laboratory results on sandy soil indicated that, in pulse flow, the wetted width increased and wetted depth decreased as the on-time decreased for the same amount of applied water volume. This result shows the advantage of pulse flow, for reducing the deep percolation of water under the crop root zone, while obtaining a wide horizontal spread of wetting. This enables using a large emitter discharge with the same amount of water. Using large emitter discharge reduces the emitter clogging.

The developed dimensional analysis models were able to predict soil wetted depth and width under continuous, intermittent and pulse flow with line source of water application. Predicted and observed values were compared to test models applicability in laboratory conditions. Predictability of model was expressed in terms of model efficiency, which was estimated as 98% and 97%, respectively, for prediction of wetted depth and width under continuous flow regime. However, the model efficiency was estimated as 94% and 83%, respectively, for prediction of wetted depth and width under intermittent flow regime. Under pulse flow regime, the model efficiency was estimated as 86% and 79%, respectively, for prediction of wetted depth and width. This shows that developed models can be used to predict wetting pattern under continuous, intermittent and pulse flow with line source of water application.

REFERENCES

ASAE. (1996). Standards, Engineering Practices Data: EP405.1 – Design and installation of microirrigation systems. American Society of Agricultural Engineers.

Central Ground Water Board. (2023). Dynamic groundwater resources of India (2023 report). Ministry of Jal Shakti, Government of India.

Department of Agriculture, Farmers Welfare and Co-operation, Government of Gujarat. (2023). Annual report 2022–23. Government of Gujarat.

Government of Gujarat. (2023). Sardar Sarovar Narmada Nigam Limited: Water resources management report. Government of Gujarat.

ISO. (1991). Irrigation equipment—Emitters—Specifications and test methods (ISO 9261:1991). International Organization for Standardization.

Ismail, S. M., Moghazy, S. M., & El Marazky, M. S. A. (2014). Modified Schwartzman and Zur model for predicting wetting front under surface trickle irrigation. Soil and Water Research, 9(2), 64–71. https://doi.org/10.17221/7/2013-SWR

Jackson, R. D., & Kay, B. D. (1988). The influence of soil properties on water movement and distribution under trickle irrigation. Irrigation Science, 9(1), 1–12. https://doi.org/10.1007/BF00257940

Karmeli, D., & Peri, G. (1974). Basic principles of pulse irrigation. Journal of Irrigation and Drainage Division, 100(IR3), 309–319.

Levin, I., & Van Rooyen, P. C. (1979). The effect of discharge rate and intermittent water application by point-source irrigation on the soil moisture distribution pattern. Soil Science Society of America Journal, 43(1), 8–11. https://doi.org/10.2136/sssaj1979.03615995004300010003x

Mostaghimi, S., & Mitchell, J. K. (1983). Trickle irrigation: Effects of pulsed application on water and solute distribution. Transactions of the ASAE, 26(6), 1683–1688. https://doi.org/10.13031/2013.33833

NITI Aayog. (2022). Composite water management index (CWMI 2.0): Water resources management report. Government of India.

Oron, G. (1996). Soil water distribution and uptake under subsurface trickle irrigation. Advances in Agronomy, 56, 45–85. https://doi.org/10.1016/S0065-2113(08)60180-9

Patel, A., & Shah, R. (2021). Water management challenges and opportunities in Gujarat: A case of sustainable irrigation practices. Journal of Water Resource Management, 35(4), 512–528.

Schwartzman, M., & Zur, B. (1986). Emitter spacing and geometry of wetted soil volume. Journal of Irrigation and Drainage Engineering, 112(3), 242–253. https://doi.org/10.1061/(ASCE)0733-9437(1986)112:3(242)

Zur, B. (1986). The use of dimensional analysis for the design of drip irrigation systems. Agricultural Water Management, 12(1–2), 37–45. https://doi.org/10.1016/0378-3774(86)90019-0