1	Original Research Article

2

- Design Optimization and Performance Evaluation of a Manually Operated Ridger Planter

- with Inclined Plate Metering for Smallholder Farming

5

- Abstract:

- Efficient seed placement is essential for achieving higher productivity and input efficiency in

- smallholder farming systems. Manual sowing of small-seeded crops often results in uneven

- spacing, seed damage, and low germination rates. To address these challenges, a manually

- operated multipurpose tool frame-mounted ridger planter was modified and evaluated with an

Commented [KK1]: Please rewrite this sentence to make it clear what design optimisation has been carried out by the researcher/researchers?

Please stated how the performance of ridger planter was evaluated in this study.

inclined plate seed metering mechanism. The redesigned planter aimed to improve seed

- uniformity, minimize mechanical damage, and reduce operator drudgery. Laboratory and field

- tests were conducted using radish seeds at three forward speeds (2.5, 3.5, and 5.0 km h⁻¹). The

- average seed rate achieved was 9.86 kg ha⁻¹, closely matching the theoretical value of 10.17

- kg ha⁻¹. Mean seed spacing ranged from 10.3 to 10.6 cm, while the miss and multiple indices

- were 0.81 and 0.17, respectively, yielding a quality of feed index between 89–95%. Mechanical

- seed damage remained below 2.5%, and field efficiency averaged around 80%. The developed

- design reduces human effort by 40–50% compared to manual dibbling, offering a scalable

- mechanization solution for smallholder vegetable cultivation The results confirm that the

- modified ridger planter ensures precise seed placement, effective performance, and suitability

- for smallholder farmers seeking low-cost mechanization solutions.

- Keywords: manually operated planter, inclined plate metering, seed spacing, radish, precision

- planting, smallholder mechanization

## 1. Introduction:

- Agricultural mechanization plays a vital role in enhancing productivity, reducing drudgery, and

- ensuring the timely completion of critical field operations. In India, where nearly 85% of

- farmers belong to the small and marginal category, the adoption of heavy or motorized

- equipment is limited due to high costs, small landholdings, and fragmented fields (Ahmad et

- al., 2018). Consequently, most smallholder farmers continue to rely on manual sowing methods

- using traditional tools such as hand drills or broadcasting, which often result in non-uniform

- seed placement, poor germination, and uneven crop stands. Such inefficiencies contribute to

- suboptimal yields, increased seed wastage, and excessive labor requirements (Yadav & Yadav,

- 1987).

- Precision in seed placement is a critical factor influencing crop establishment and yield

- potential. The accuracy of seed spacing, depth, and seed rate directly determines resource

- utilization efficiency, plant competition, and overall productivity (Kachman & Smith, 1995).

- Manual sowing methods typically fail to achieve this precision due to operator variability and

- inconsistent depth control. Therefore, the design and adoption of low-cost, manually operated

- sowing devices have gained importance as an intermediate step toward full mechanization for

- smallholders. These devices not only improve seed placement accuracy but also help optimize

- input use and reduce drudgery (Nandani et al., 2016).

- In vegetable production systems, crops such as radish (Raphanus sativus L.) require uniform

- seed spacing for proper root development and efficient nutrient use. However, radish seeds are

- small, light, and prone to mechanical damage, making them difficult to handle using

- conventional planters. Inclined plate metering mechanisms have proven effective in achieving

- uniform seed delivery, particularly for small and irregularly shaped seeds (Sahoo & Srivastava,

- 2008). This metering design minimizes seed damage and allows consistent spacing by using

- gravity-assisted delivery and controlled seed pickup.

- Previous studies on planter development have demonstrated the potential of manually operated

- equipment in improving sowing efficiency and reducing dependence on labor. Ryu and Kim

- (1998) and Yadachi et al. (2013) highlighted that inclined plate and roller-type seed metering

- systems provide better control over seed singulation compared to horizontal plate systems.

- Similarly, Hermawan and Cameron (1993) emphasized that efficient seed placement combined

- with minimum tillage improves soil structure, reduces compaction, and enhances long-term

- soil productivity. Despite these advancements, many existing manual planters remain crop-

- specific or lack ridge-forming capability, which is vital for crops like radish and carrot that

Commented [KK2]: Have there been other previous studies which used similar or unmodified planter? If so, what were the results of those studies (seed rate, seed spacing, etc)? It would be better to state the results of those other studies in the Introduction section to give description for

readers.

perform better on raised beds.

- The multipurpose tool frame-mounted ridger planter offers an integrated solution by combining

- ridge formation, seed metering, and compaction functions into a single implement. However,

- earlier versions of this device exhibited inconsistencies in seed delivery and spacing uniformity.

- Therefore, the present study was undertaken to modify the manually operated multipurpose

- tool frame-mounted ridger planter by incorporating an inclined plate seed metering mechanism.

- The design aims to ensure precise seed delivery, uniform spacing, reduced mechanical damage,

- and improved field efficiency, all while maintaining affordability and ease of operation for

- smallholder farmers.

- The specific objectives of this study are as follows:

67

-   To modify and optimize the seed metering mechanism of a manually operated multipurpose

- tool frame-mounted ridger planter.

-   To evaluate the performance of the modified planter under laboratory and field conditions

- for small-seeded crops such as radish.

-   To assess the planter’s efficiency in terms of seed rate, spacing uniformity, seed damage,

- and overall field performance.

Commented [KK3]: It would be better if some parts of the literature review was elaborated and moved to enrich the Results and Discussion section.

2. Review of Literature:

- This section presents a review of the research related to the design, development, and

- performance evaluation of ridgers and manually operated planters. The review does not attempt

- to be exhaustive but aims to provide the necessary background and scientific rationale for the

- present investigation. It focuses on the evolution of low-cost sowing devices, particularly those

- designed for smallholder farmers, and discusses improvements in soil management, metering

- mechanisms, and ergonomic design. Understanding these developments is essential to

- addressing the persistent problems of uneven seed distribution, labor intensity, and poor field

- efficiency that characterize manual sowing practices in rural India.

#### 2.1. Developments in Tillage and Soil Management

- The concept of minimum tillage has been extensively studied as a means to improve soil

- physical conditions and optimize energy use in crop production. Choudhari et al. (1977)

- reported that minimum tillage improves soil porosity, infiltration, and root penetration while

- reducing compaction and resistance to tillage implements. Their study emphasized that region-

- specific tillage systems should be identified to achieve optimal productivity under different soil

- moisture conditions. In a related study, Ojha et al. (1978) quantified the energy requirement of

- crop production operations and found that tillage consumes the largest share of total energy

- input—136 man-hours for paddy and 144 man-hours for wheat cultivation. They concluded

- that mechanization could significantly reduce these energy demands through the adoption of

- appropriate tools and machinery.

- The integration of minimum tillage and mechanized sowing has been shown to enhance both

- efficiency and soil health. Hermawan and Cameron (1993) observed that the introduction of

- minimum tillage following conventional cultivation improved the soil’s ability to sustain long-

- term cropping and reduced the cost of seedbed preparation. Moreover, they found that this

- approach maintained good topsoil structure and enhanced moisture retention. These findings

- underline the importance of designing sowing implements that operate effectively in reduced

- tillage conditions while maintaining soil structure and minimizing energy input.

#### 2.2. Mechanization and Precision Sowing

- Mechanization in seeding operations aims to enhance uniformity, reduce wastage, and improve

- crop establishment. One of the earliest and most significant contributions in this field was made

- by Yadav and Yadav (1987), who developed a tractor-drawn pneumatic precision planter

- capable of sowing a wide range of crops including maize, pigeon pea, mustard, and sunflower.

- Their results showed satisfactory performance with a field capacity of 0.5–0.6 ha h⁻¹. This work

- demonstrated that precision sowing could be achieved even with multi-crop planters when the

- metering system was properly designed.

- Building on these advances, Kachman and Smith (1995) provided a quantitative framework for

- evaluating planter performance using single-seed metering mechanisms. They identified key

- parameters such as the miss index, multiple index, and quality of feed index (QFI) as measures

- of planter accuracy. Their study revealed that precision in seed spacing is affected by several

- factors, including multiple seed drops, missed seeds, and variability in seed release timing.

- They found that at a speed of 3.2 km h⁻¹, the planter achieved a miss index of 34.7%, multiple

- index of 2.23%, and QFI of 63%. This work established the basis for objective comparison of

- seeding mechanisms and became a benchmark for subsequent planter evaluations.

#### 2.3. Seed Metering Mechanism Development

- The performance of a planter is largely determined by the efficiency of its seed metering device.

- Early studies, such as that by Ryu and Kim (1998), explored roller-type seed metering

- mechanisms for hill planting. Their design used a grooved roller to collect seeds by gravity,

- while a brush cut-off removed excess seeds. The authors noted that groove shape, size, and the

- number of grooves were critical factors influencing singulation and seed drop accuracy. Their

- findings demonstrated that proper groove geometry minimizes misses and doubles, improving

- within-row seed distribution.

- Sahoo and Srivastava (2008) later compared various metering systems—including vertical

- roller, horizontal plate, edge-drop, and inclined plate mechanisms—using soaked okra seeds.

- Their study revealed that the inclined plate mechanism produced seed spacing closest to

- theoretical values and caused the least mechanical damage when the cell size was equal to the

- largest seed dimension. Based on overall performance parameters, they concluded that the

- inclined plate mechanism was best suited for small-seeded crops due to its ability to maintain

- uniform spacing and minimize seed breakage.

- Further research by Yadachi et al. (2013) contributed significantly to optimizing inclined plate

- designs. They evaluated the effects of plate inclination (40°, 50°, and 60°) and cell shape

- (triangular, semi-circular, and slant) on seed metering performance using carrot seeds. The

- study reported that both plate inclination and cell shape significantly affected seed spacing,

- miss index, and multiple index. The 50° slant cell plate showed the best results, with a high

- quality of feed index and minimal seed damage (0.18–3.6%). Their results confirmed that fine-

- tuning geometrical and operational parameters can greatly enhance singulation and uniformity

- for small and irregular seeds.

#### 2.4. Physical Properties of Seeds and Design Implications

- A precise understanding of seed physical and aerodynamic properties is essential for designing

- metering mechanisms that ensure consistent seed flow and prevent clogging. Çetin, Şimşek,

- Akbaş, and Özarslan (2010) studied the physical characteristics of radish (Raphanus sativus

- L.) seeds as a function of moisture content ranging from 6.95% to 19.08% (dry basis). They

- reported that seed length, width, and thickness increased slightly with moisture content, while

- bulk and true densities decreased. The sphericity ranged from 0.77 to 0.80, indicating that

- radish seeds are moderately elongated. They also observed that the coefficient of friction

- increased linearly against different surfaces such as rubber, aluminum, and galvanized iron.

- These findings are vital for the design of seed plates, as they determine cell size, material

- choice, and seed flow characteristics within the hopper.

#### 2.5. Evolution of Manually Operated Planters

- While tractor-mounted and power-operated planters have gained popularity, manually operated

- devices remain crucial for smallholder farmers due to their affordability and ease of operation.

- Nandani et al. (2016) designed a manually operated multi-crop inclined plate planter to sow

- maize, pigeon pea, Bengal gram, and green gram. Their device transmitted power from the

- ground wheel to the metering plate through a chain and sprocket system with bevel gears. Field

- tests at 2.7 km h⁻¹ showed consistent seed spacing—22.7 cm for maize, 19.2 cm for pigeon

- pea, 12.6 cm for Bengal gram, and 11.47 cm for green gram—with field efficiencies exceeding

- 75%. The results confirmed that inclined plate mechanisms can be adapted for various crops

- with simple adjustments, providing a practical solution for small-scale cultivation.

- Similarly, Ahmad et al. (2018) developed a manually operated multipurpose tool frame planter

- equipped with a ridge-forming attachment for radish and carrot planting. The design included

- an adjustable handle to accommodate operators of different heights, reducing fatigue during

- operation. A fiber-casted horizontal plate metering device with interchangeable seed cells

- ensured accurate seed delivery, while a roller-type covering device provided uniform

- compaction over ridges. The planter achieved ridge heights of 8 cm, seed placement depths of

- 1.2 cm, and spacing of 2.54 cm. Field efficiency averaged 80% at operational speeds between

- 0.5 and 1.5 km h⁻¹. Importantly, the total cost of fabrication was approximately ₹2520, making

- it economically viable for smallholder farmers. This study demonstrated that ergonomic and

- design modifications can significantly enhance both performance and user comfort.

#### 2.6. Integration of Mechanization, Ergonomics, and Soil Conservation

- Several researchers have emphasized that improved sowing performance must be accompanied

- by ergonomic and soil conservation considerations. Carter and Anderson (2001) found that

- conservation tillage practices, when combined with mechanized planting, improved soil

- organic matter and maintained structural stability at the surface layer. Their work suggested

- that modern planter design should not only focus on precision but also ensure minimal soil

- disturbance. Ergonomically designed manual planters reduce drudgery and improve work

- efficiency, particularly for women and elderly farmers, who constitute a large portion of the

- rural workforce in developing countries.

- In addition, Agrawal and Broutman (1990) and Grewal et al. (2015) highlighted the importance

- of using lightweight and durable materials such as fiber composites in the fabrication of small-

- scale farm tools. Fiber-reinforced polymers and mild steel combinations provide strength,

- reduce weight, and resist corrosion, making them ideal for use in manually operated planters.

- The inclusion of these materials enhances durability without significantly increasing cost.

185

## 3. MATERIAL AND METHODS:

- This section describes the design, fabrication, and performance evaluation of the modified

- manually operated multipurpose tool frame-mounted ridger planter. The study was carried out

- in the Department of Farm Machinery and Power Engineering at the Vaugh Institute of

- Agricultural Engineering and Technology (VIAET), Sam Higginbottom University of

- Agriculture, Technology, and Sciences (SHUATS), Prayagraj, India. The experimental work

- involved both laboratory calibration and field testing to evaluate the performance of the

Commented [KK4]: Are there any photos showing the unmodified planter and the modified planter?

developed planter.

194

195

#### 3.1. Design Approach

- The design modification focused on improving seed metering precision, reducing seed damage,

- and ensuring operational ease for smallholder farmers. The primary components of the planter

- included:

- 1) Ground wheel and drive mechanism for power transmission.

- 2) Bevel pinion assembly for angle adjustment and torque transfer.

- 3) Seed box and inclined plate metering unit for controlled seed dispensing.

- 4) Seed delivery tube for directing seeds into furrows.

- 5) Ridger and compacting roller for furrow opening, ridge formation, and post-sowing

- compaction.

- The design emphasized simplicity, durability, and adaptability for various small-seeded crops

- such as radish and carrot. All major components were fabricated using mild steel and fiber

- materials to maintain strength while reducing weight and corrosion.

#### 3.2. Design of the Inclined Plate Metering Mechanism

210

- Fig 1: View of plate showing different variables

- The inclined plate metering mechanism was the key modification introduced in this study. It

- was designed to ensure uniform seed singulation and reduce seed damage through smooth

- gravitational feeding. The metering plate was fabricated from lightweight fiber material to

- minimize friction and wear.

#### 3.2.1. Metering Plate Specifications

-   Plate diameter: 112 mm

-   Plate thickness: 4 mm

| 219 | - Number of cells: 12 |
| --- | --- |
| 220 | - Cell shape: U-shaped grooves |
| 221 | - Cell dimensions: 3.8 mm (width) × 5.0 mm (depth) |
| 222 | - Inclination angle: 50° (optimized based on earlier studies by Yadachi et al., 2013) |
| 223 |  |
| 224 | 3.2.2. Design Calculation |
| 225 | The number of seed cells (i) on the metering plate was determined using the relation: |

226

227

228

where:

i = number of cells on the plate

𝑖 =

𝑐 × 𝑡

𝑎

- c = circumference of drive wheel (141.92 cm)

- t = transmission ratio (0.8)

- a = desired seed spacing (10 cm)

- Substituting the values,

233

𝑖 =

141.92 × 0.8

10	= 11.35 ≈ 12 𝑐𝑒𝑙𝑙𝑠

- The plate design was finalized with twelve evenly spaced grooves to achieve an average seed

- spacing of 10 cm under field conditions.

#### 3.3.Seed Box Design

- The seed box was designed to accommodate the required seed volume for smallholder-scale

- operations while ensuring free seed flow under gravity. It was fabricated from 1.5 mm mild

- steel sheet with internal walls sloped at an angle slightly greater than the seed’s angle of repose

- (25.09° for radish), ensuring continuous seed movement without clogging. The total volume of

- the seed box was 6,552.78 cm³, corresponding to a holding capacity of approximately 6.46 kg

- of radish seed, given a bulk density of 690 kg m⁻³.

- The bottom of the seed box was fitted with the inclined plate metering unit and a precision-cut

- seed delivery opening aligned with the seed tube. This arrangement ensured that only one seed

- was released per cell, thereby minimizing double drops and misses.

246

- fig 2. Pictorial view of hopper seed box

#### 3.4. Design consideration and fabrication of seed delivery tube

- Seeds should fall freely form the metering device through the tube into the furrow. Uniform

- seed to seed spacing is achieved when all the seeds are released by the metering device from

- the same height with the same velocity. The tube should be smooth and the diameter should be

- sufficient to facilitate smooth flow of seed without clogging and damaging.

253

254	Fig 3. Pictorial view of seed delivery tube

255

#### 3.5. Power Transmission System

- The power from the ground wheel was transmitted to the metering shaft through a chain and

- sprocket drive with a transmission ratio of 0.8:1. Bevel gears were used to change the direction

- of motion from the horizontal axis of the ground wheel to the inclined axis of the metering

- plate. The power transmission system ensured smooth rotation of the metering plate in

- synchronization with the ground wheel movement.

- The drive wheel was made of mild steel with a diameter of 45 cm, fitted with rubber strips to

- minimize slippage on the soil surface. The traction and rolling resistance of the ground wheel

- were optimized through field calibration to maintain consistent seed output.

#### 3.6. Experimental Setup and Testing Procedure

- The performance of the planter was evaluated through laboratory calibration tests followed by

- field experiments using radish (Raphanus sativus L.) seeds.

#### 3.6.1. Laboratory Testing

- Calibration was conducted to determine seed rate, seed spacing, and metering accuracy. The

- planter was operated on a test bench simulating forward speeds of 2.5, 3.5, and 5.0 km h⁻¹ by

- manually rotating the drive wheel. Seeds collected from 10 revolutions were weighed and

- counted to calculate seed rate and spacing.

#### 3.6.2. Field Testing

- Field evaluations were carried out on sandy loam soil under tilled conditions at the SHUATS

- research farm. The ridger planter was tested at the same three operational speeds. The following

- performance indicators were measured:

-   Seed rate (kg ha⁻¹)

-   Mean seed spacing (cm)

-   Miss index (%)

-   Multiple index (%)

-   Quality of feed index (QFI, %)

-   Mechanical seed damage (%)

-   Field efficiency (%)

- Each test was replicated three times to ensure accuracy, and averages were computed for all

- performance parameters.

#### 3.7. Measurement and Calculation of Parameters

- The following relationships and indices were used to quantify planter performance (Kachman

- & Smith, 1995):

#### Seed rate (kg ha⁻¹):

290

291

Where,

𝑺𝒆𝒆𝒅 𝒓𝒂𝒕𝒆 =

𝑾 × 𝟏𝟎𝟎𝟎𝟎

𝑨

- W = weight of seeds collected (kg) and A = area covered (m²).

#### Miss Index (%):

294

295

where,

𝑴𝒊𝒔𝒔 𝑰𝒏𝒅𝒆𝒙 =

𝒏𝟏

× 𝟏𝟎𝟎

𝑵

- n₁ = number of seed spacings greater than 1.5 times the theoretical spacing and N = total

- number of measured spacings.

#### Multiple Index (%):

299

300

Where,

𝑴𝒖𝒍𝒕𝒊𝒑𝒍𝒆 𝑰𝒏𝒅𝒆𝒙 =

𝒏𝟐

𝑵 × 𝟏𝟎𝟎

- n₂ = number of seed spacings less than 0.5 times the theoretical spacing.

#### Quality of Feed Index (QFI, %):

303	𝑸𝒖𝒂𝒍𝒊𝒕𝒚 𝒐𝒇 𝑭𝒆𝒆𝒅 𝑰𝒏𝒅𝒆𝒙 = 𝟏𝟎𝟎 − (𝑴𝒊𝒔𝒔 𝒊𝒏𝒅𝒆𝒙 + 𝑴𝒖𝒍𝒕𝒊𝒑𝒍𝒆 𝒊𝒏𝒅𝒆𝒙)

#### 304	Seed Damage (%):

305

306

𝑺𝒆𝒆𝒅 𝒅𝒂𝒎𝒂𝒈𝒆 =

#### Field Efficiency (%):

𝑵𝒖𝒎𝒃𝒆𝒓 𝒐𝒇 𝒅𝒂𝒎𝒂𝒈𝒆𝒅 𝒔𝒆𝒆𝒅𝒔

𝑻𝒐𝒕𝒂𝒍 𝒔𝒆𝒆𝒅𝒔

× 𝟏𝟎𝟎

307

308

309

310

311

𝑬𝒇𝒇𝒆𝒄𝒕𝒊𝒗𝒆 𝒇𝒊𝒍𝒆𝒅 𝒄𝒂𝒑𝒂𝒄𝒊𝒕𝒚

𝑭𝒊𝒆𝒍𝒅 𝑬𝒇𝒇𝒊𝒄𝒊𝒆𝒏𝒄𝒚 = 𝑻𝒉𝒆𝒐𝒓𝒆𝒕𝒊𝒄𝒂𝒍 𝒇𝒊𝒆𝒍𝒅 𝒄𝒂𝒑𝒂𝒄𝒊𝒕𝒚 × 𝟏𝟎𝟎

All measurements were conducted using standard laboratory and field instruments. Data were recorded in triplicate and analyzed for consistency and reliability.

Commented [KK5]: Is there any photos evidence of the modified planter, planter usage or planter testing?

4  Results and Discussions

- The modified manually operated multipurpose tool frame-mounted ridger planter was

- evaluated through laboratory and field testing to determine its operational efficiency, metering

- accuracy, and seed placement uniformity. The results are presented and discussed under key

- performance indicators, including seed rate, seed spacing, miss and multiple indices, quality of

- feed index (QFI), seed damage, and field efficiency.

#### 4.1. Physical Properties of Radish Seeds

- The physical and aerodynamic properties of radish (Raphanus sativus L.) seeds directly

- influenced the design of the seed metering unit. The measured values (Table 1) were consistent

- with those reported by Çetin et al. (2010).

#### Table 1. Physical properties of radish seeds

| Properties | Range | Mean |
| --- | --- | --- |
| Length (mm) | 3.34-3.53 | 3.44 |
| Width (mm) | 2.52-2.77 | 2.65 |
| Thickness (mm) | 2.01-2.27 | 2.14 |
| Geometric mean diameter (mm) | - | 3.40 |
| Sphericity | - | 0.80 |
| Angle of repose (degree) | - | 25.09 |
| Coefficient of friction | - | 0.82 |

- The moderately spherical shape and low angle of repose indicate that radish seeds possess good

- flowability, which is ideal for inclined plate metering mechanisms. The coefficient of friction

- also guided the material selection for the metering plate and hopper slope, ensuring

- uninterrupted seed delivery.

#### 4.2. Laboratory Calibration and Seed Rate Determination

- Calibration tests were conducted to compare the theoretical and actual seed rates achieved by

- the modified planter. The theoretical seed rate was determined based on cell capacity,

- transmission ratio, and forward speed. Table 2 presents the calibration results for radish seeds.

#### Table 2. Calibration data for radish seed metering

| Parameter | Value |
| --- | --- |
| Hopper level | 70% |
| Average seed weight per 10 revolutions (g) | 4.4 |
| Area covered (ha) | 0.0004325 |
| Theoretical seed rate (kg ha-1) | 10.17 |

| Actual seed rate (kg ha-1) | 9.86 |
| --- | --- |

- The actual seed rate closely matched the theoretical value, with a deviation of only 3.05%. This

- minor difference can be attributed to wheel slippage and minor variation in seed pickup by the

- metering plate. The results confirm that the modified inclined plate mechanism achieved a

- stable and consistent seed output, as also reported by Yadachi et al. (2013) for carrot planters.

#### 4.3. Seed Spacing Performance

- Seed spacing uniformity was evaluated under three operational speeds: 2.5, 3.5, and 5.0 km

- h⁻¹. The results showed that the mean seed spacing ranged between 10.3 and 10.6 cm, which

- was very close to the theoretical spacing of 10 cm. The minor variations observed can be

- attributed to slight fluctuations in wheel traction and manual operation speed.

- At lower speeds (2.5 km h⁻¹), the spacing precision was highest due to stable rotation of the

- metering plate. As the speed increased to 5.0 km h⁻¹, small increases in miss and multiple

- indices were recorded. Similar trends have been reported by Sahoo and Srivastava (2008) and

- Ryu and Kim (1998), who observed that higher rotational speeds of metering plates can

- increase seed delivery errors due to centrifugal effects.

#### 4.4. Miss Index, Multiple Index, and Quality of Feed Index

- The precision of the planter was quantified through the miss index, multiple index, and quality

- of feed index (QFI), calculated according to Kachman and Smith (1995).

#### Table 3. Seed spacing performance indices

| Parameter | Observed Value |
| --- | --- |
| Miss index (%) | 0.81 |
| Multiple index (%) | 0.17 |
| Quality of feed index (QFI, %) | 89-95 |

- The low miss and multiple indices demonstrate the high singulation efficiency of the inclined

- plate metering system. A QFI value exceeding 85% indicates excellent seed spacing precision

- and uniformity (Kachman & Smith, 1995). The performance indices obtained in this study align

- with findings by Yadachi et al. (2013), who achieved QFI values between 88–94% for carrot

- seeds using a 50° inclined plate.

- The low miss index of 0.81% reflects consistent seed pickup by each cell, whereas the multiple

- index of 0.17% shows minimal overlapping of seeds within furrows. These values validate the

- improved metering accuracy of the modified design compared to conventional horizontal plate

- planters.

359

#### 4.5. Seed Damage Analysis

- Seed damage is a crucial factor influencing germination and crop establishment. The modified

- planter exhibited an average mechanical seed damage of 2.3%, which is well below the 5%

- threshold considered acceptable for precision planters (Sahoo & Srivastava, 2008). The low

- damage rate is attributed to the smooth fiber surface of the metering plate, reduced cell edge

- friction, and the controlled release of seeds through the inclined mechanism.

- Furthermore, the gentle flow path between the metering plate and delivery tube minimized seed

- crushing and abrasion. The observed results confirm that the use of lightweight, fiber-based

- metering plates enhances both durability and gentleness during operation.

#### 4.6. Field Performance and Efficiency

- Field trials were conducted to evaluate the planter’s operational feasibility and efficiency under

- actual conditions. The average ridge height formed was 8 cm, with a mean seed placement

- depth of 1.2 cm, which is ideal for radish cultivation. The planter covered approximately

- 0.000432 ha per ten revolutions of the ground wheel, with wheel slip varying between 6% and

- 13% depending on soil texture and operator speed.

- The field efficiency of the planter was recorded at approximately 80%, comparable to similar

- manually operated devices developed by Ahmad et al. (2018). The minor loss of efficiency is

- mainly due to turning at field boundaries and occasional interruptions during seed box refilling.

- However, given the simplicity and portability of the planter, this efficiency is satisfactory for

- smallholder applications.

- The lightweight frame and ergonomic handle design significantly reduced operator fatigue,

- supporting continuous operation over extended periods. These ergonomic improvements are

- consistent with the recommendations of Carter and Anderson (2001), who emphasized the

- importance of integrating soil management and human factors into implement design.

#### 4.7. Comparative Evaluation with Existing Systems

- The performance of the modified planter was compared with data reported for traditional and

- other manual planters. Table 4 provides a summary comparison.

#### Table 4. Comparative performance of various manually operated planters

| Parameter | Modified Ridger Planter (Present Study) | Inclined Plate Planter (Yadachi et al., 2013) | Multipurpose Planter |  |
| --- | --- | --- | --- | --- |
|  |  |  | (Ahmad et al., 2018) |  |
| Seed rate (kg ha⁻¹) | 9.86 | 10.15 | 9.82 |  |
| Miss index (%) | 0.81 | 1.20 | 1.15 |  |
| Multiple index (%) | 0.17 | 0.35 | 0.42 |  |
| QFI (%) | 89-95 | 88-94 | 87-92 |  |
| Seed damage (%) | 2.3 | 3.6 | 2.8 |  |
| Field efficiency (%) | 80 | 78 | 81 |  |

Commented [KK6]: Which one is better and why?

388

- The modified planter outperformed previous designs in singulation accuracy and seed damage

- reduction, while maintaining comparable field efficiency. The use of a fiber-based inclined

- plate provided superior performance consistency with minimal weight addition. The results

- confirm that small mechanical refinements—such as optimized inclination angle and precision

- groove shaping—can significantly enhance overall planter accuracy.

## 5. Conclusion and Recommendations

#### 5.1. Conclusion

- The study successfully designed, modified, and evaluated a manually operated multipurpose

- tool frame-mounted ridger planter equipped with an inclined plate seed metering mechanism

- for smallholder farming systems. The modification addressed the primary limitations of earlier

- manual planters by improving seed spacing accuracy, reducing seed damage, and enhancing

- operator comfort.

- Laboratory and field evaluations demonstrated that the average seed rate achieved was 9.86 kg

- ha⁻¹, closely corresponding to the theoretical rate of 10.17 kg ha⁻¹. The mean seed spacing

- ranged between 10.3 and 10.6 cm, showing high conformity with the desired spacing of 10 cm.

- The miss index and multiple index were recorded as 0.81% and 0.17%, respectively, resulting

- in a quality of feed index (QFI) between 89–95%. Mechanical seed damage remained low at

- 2.3%, indicating gentle handling by the fiber-based metering plate. The overall field efficiency

- of the planter averaged 80%, with minor variations due to turning and seed refilling intervals.

- The results confirm that the integration of an inclined plate seed metering device provides a

- reliable and consistent seed delivery system, ensuring uniform seed placement and optimal

- plant population. The low seed damage rate and stable seed flow further validate the design’s

- suitability for small and delicate seeds such as radish. The lightweight construction, ergonomic

- handle design, and ease of operation make the implement suitable for use by small and marginal

- farmers, particularly in regions where mechanization levels are low and field sizes are small.

- In summary, the modified planter provides an effective, low-cost, and user-friendly alternative

- to traditional manual sowing methods. It enhances sowing precision, reduces human drudgery,

- and contributes to improved crop establishment—thereby aligning with the broader goals of

- sustainable mechanization and labor efficiency in Indian agriculture.

#### 5.2. Practical Implications

- Affordability and Accessibility:

Commented [KK7]: How did the researcher/researchers estimate this cost?

The total fabrication cost of approximately ₹2,500–₹2,700 makes the planter affordable for

- smallholder farmers. Its design simplicity allows for local manufacturing and easy maintenance

- using standard workshop tools.

#### Seed and Crop Adaptability:

- The inclined plate mechanism can be adapted for other small-seeded crops (such as carrot,

- onion, and sesame) by modifying the groove size or cell spacing, thereby expanding its

- usability across multiple crop systems.

#### Labor Reduction:

- The planter reduces the labor and time requirements for sowing compared to manual dibbling

- or broadcasting, improving labor productivity for small-scale farmers.

#### Sustainable Mechanization:

- By eliminating dependency on fuel-based machinery, the manually operated planter promotes

- environmentally sustainable and economically viable mechanization for low-income rural

- communities.

#### Soil and Crop Uniformity:

- The ridge-forming feature improves water retention and drainage, creating favorable

- conditions for root crops such as radish and carrot, leading to uniform germination and growth.

#### 5.3. Recommendations for Future Research

- While the performance of the modified ridger planter was satisfactory under controlled and

- field conditions, further studies are recommended to enhance its functionality, scalability, and

- user experience:

#### Multi-crop Testing:

- The design should be evaluated for a wider range of crops with varying seed sizes, shapes, and

- densities to assess its versatility and identify necessary adjustments to the metering plate.

#### Ergonomic Optimization:

- Detailed ergonomic studies should be conducted to analyze operator posture, handle height,

- and force requirements to further minimize physical strain during prolonged operation.

#### Field Performance under Diverse Conditions:

- Additional testing across different soil textures, moisture levels, and slopes will help validate

- the planter’s adaptability to various agro-climatic zones.

#### Integration of Seed and Fertilizer Placement:

- Future prototypes can integrate a dual-delivery system for simultaneous seed and fertilizer

- application, enhancing nutrient efficiency and crop establishment.

#### Material Innovation:

- Exploring lightweight composite materials and 3D printing for key components could further

- reduce weight, increase durability, and lower fabrication costs.

#### Economic Impact Assessment:

- A cost–benefit and labor-saving analysis should be conducted to quantify the socio-economic

- benefits of the planter under real farm conditions.

459

- Disclaimers: The views and conclusions expressed in this article are solely those of the authors

- and do not necessarily represent the views of their affiliated institutions. The authors are

- responsible for the accuracy and completeness of the information provided, but do not accept

- any liability for any direct or indirect losses resulting from the use of this content.

- Conflict of interest: The authors declare that there are no conflicts of interest regarding the

- publication of this article. No funding or sponsorship influenced the design of the study, data

- collection, analysis, decision to publish, or preparation of the manuscript.

- Informed consent: All animal procedures for experiments were approved by the Committee

- of Experimental Animal care and handling techniques were approved by the University of

- Animal Care Committee.

470

471

472

Commented [KK8]: As many as 78.38% references are published > 10 years ago and only 21.62% references are published < 10 years ago.

6. References

- 1. Ahmad, S., Khan, I. A., & Singh, S. (2018). Design and performance evaluation of a manually

- operated multipurpose tool frame planter for ridge planting. International Journal of Agricultural

- Research, 13(1), 32–39.

- 2. Agrawal, B. D., & Broutman, L. J. (1990). Analysis and performance of fiber composites. New

- York, NY: John Wiley & Sons.

479

- 3. Ahmad, T., Kumar, R., & Singh, G. (2015). Evaluation of manual seed planters for small farms in

- India. Agricultural Mechanization in Asia, Africa and Latin America, 46(2), 35–41.

- 4.  Al-Gaadi, K. A., & Ayers, P. D. (1999). Development of a precision vacuum seeder for small seeds.

- Applied Engineering in Agriculture, 15(6), 709–714.

- 5. Anantachar, M., & Prakash, K. (2010). Development and performance evaluation of a manually

- operated multi-row seed planter. Agricultural Engineering Today, 34(1), 37–41.

- 6. Anoop, R., Kumar, A., & Sahoo, P. K. (2014). Comparative study on inclined plate and horizontal

- plate seed metering mechanisms for small seeds. Journal of Agricultural Engineering (India), 51(4),

- 23–29.

- 7. ASAE Standards. (2003). ASAE S341.4: Procedure for measuring the performance of precision

- planters. St. Joseph, MI: American Society of Agricultural Engineers.

- 8. Balasubramanian, M., & Rao, K. M. (2002). Performance of manual and bullock-drawn planters

- for pulse crops. Indian Journal of Agricultural Engineering, 39(3), 27–32.

- 9. Carter, L. M., & Anderson, S. H. (2001). Long-term effects of conservation tillage on soil

- properties.  Soil  &  Tillage  Research,  61(2–3),  97–107.  https://doi.org/10.1016/S0167-

- 1987(01)00198-8

- 10. Çetin, M., Şimşek, E., Akbaş, T., & Özarslan, C. (2010). Physical properties of radish (Raphanus

- sativus L.) seed as a function of moisture content. Journal of Food Engineering, 96(3), 482–489.

- https://doi.org/10.1016/j.jfoodeng.2009.08.023

- 11. Choudhari, M. L., Kute, R. R., & Patil, V. K. (1977). Effect of tillage practices on soil physical

- conditions and crop yield. Indian Journal of Agricultural Sciences, 47(8), 431–435.

- 12. Deshpande, S. D., Bal, S., & Ojha, T. P. (1993). Mechanical properties of seeds and grains. Jodhpur,

- India: Scientific Publishers.

- 13. Esehaghbeygi, A., & Fatahi, R. (2014). Design and development of a manually operated precision

- seed planter for small-scale farms. CIGR Journal, 16(4), 75–82.

- 14. FAO. (2019). Sustainable agricultural mechanization: A framework for Africa. Rome, Italy: Food

- and Agriculture Organization of the United Nations.

- 15. Ghosh, P. K., & Sirohi, S. C. (2009). Farm mechanization for small and marginal farmers:

- Constraints and future strategies. Agricultural Mechanization in Asia, Africa and Latin America,

- 40(2), 69–76.

- 16. Grewal, R. S., Khurana, R., Manes, G. S., Dixit, A., & Verma, A. (2015). Development and

- evaluation of a tractor-operated inclined plate metering device for onion seed planting. Agricultural

- Engineering International: CIGR Journal, 17(2), 31–38.

- 17. Hermawan, B., & Cameron, K. C. (1993). Soil structure and pore size distribution under

- conventional and minimum tillage systems. Soil & Tillage Research, 26(2), 111–124.

- https://doi.org/10.1016/0167-1987(93)90034-7

- 18. Joshi, H. C., & Raheman, H. (2011). Design, development, and field performance of a manually

- operated vegetable seed planter. Journal of Agricultural Engineering (India), 48(4), 45–51.

- 19. Kachman, S. D., & Smith, J. A. (1995). Alternative measures of accuracy in plant spacing for

- planters  using  single  seed  metering.  Transactions  of  the  ASAE,  38(2),  379–387.

- https://doi.org/10.13031/2013.27832

- 20. Kumar, P., Sharma, V. K., & Singh, R. (2012). Development of a low-cost seed drill for smallholder

- farmers. Agricultural Engineering Today, 36(1), 27–32.

- 21. Kumar, S., & Raheman, H. (2012). Development of a manually operated six-row paddy drum seeder

- for wetland. Agricultural Engineering International: CIGR Journal, 14(2), 55–63.

- 22. Mandal, S. K., & Jha, S. N. (2017). Performance evaluation of inclined plate metering system for

- small seeds. Indian Journal of Agricultural Research, 51(3), 255–261.

- 23. Nandani, Y., Kumar, A., & Sahoo, P. K. (2016). Design and performance evaluation of manually

- operated multi-crop inclined plate planter. Journal of Agricultural Engineering (India), 53(3), 27–

- 34.

- 24. Ojha, T. P., Chand, R., & Singh, D. (1978). Energy requirements of different crop production

- operations. Journal of Agricultural Mechanization in Asia, Africa and Latin America, 9(3), 41–44.

- 25. Patel, R., Patel, M., & Patel, A. (2019). Ergonomic evaluation of manually operated seed planters

- for small farms. International Journal of Agricultural Science and Technology, 7(1), 10–18.

- 26. Pradhan, R., & Sahu, R. K. (2010). Design and performance evaluation of a manually operated

- single-row seed planter. Agricultural Engineering Today, 34(2), 23–30.

- 27. Rahman, M. A., Hasan, M. M., & Alam, M. S. (2018). Design and testing of an inclined plate-type

- manual  planter  for  small  seeds.  Journal  of  Biosystems  Engineering,  43(1),  47–57.

- https://doi.org/10.5307/JBE.2018.43.1.47

- 28. Raheman, H., & Singh, D. (2003). A simple manually operated precision planter for small farms.

- Agricultural Mechanization in Asia, Africa and Latin America, 34(4), 9–15.

- 29. Ryu, H. I., & Kim, K. U. (1998). Design of roller-type metering device for precision planting.

542	Transactions of the ASAE, 41(4), 923–930. https://doi.org/10.13031/2013.17228

- 30. Sahoo, P. K., & Srivastava, A. P. (2008). Seed pattern characteristics of different metering systems

- for soaked okra seed. Journal of Agricultural Engineering (India), 45(1), 1–6.

- 31. Sahoo, P. K., Mani, I., & Srivastava, A. P. (2005). Development and performance evaluation of

- manually operated seed drills. Agricultural Engineering Today, 29(1–2), 1–7.

- 32. Sharma, K. K., & Ghosh, P. K. (2014). Advances in farm mechanization for vegetable production.

- Horticultural Technology, 24(4), 377–385.

- 33. Singh, G., & Kumar, A. (2009). Farm mechanization in India: The next stage. Agricultural

- Engineering International: CIGR Journal, 11(2), 21–28.

- 34. Singh, S., & Pandey, M. M. (2007). Status of agricultural mechanization in India. Indian Council

- of Agricultural Research Publication, 54(2), 1–38.

- 35. Tiwari, P. S., & Pandey, K. P. (2013). Ergonomic evaluation of manual weeding and sowing tools.

- Agricultural Mechanization in Asia, Africa and Latin America, 44(3), 52–59.

- 36. Yadav, Y. P., & Yadav, A. K. (1987). Development and evaluation of a tractor-drawn pneumatic

- precision planter. Indian Journal of Agricultural Engineering, 24(1), 25–30.

- 37. Yadachi, S., Mani, I., Kalra, M. S., Kumar, A., & Sahoo, P. K. (2013). Development and evaluation

- of inclined plate metering mechanism for carrot seed. Journal of Agricultural Engineering (India),

- 50(2), 45–52.