Review Article

Soil Carbon Sequestration Strategies and Carbon Crediting for Mitigating Climate Change: A Review of Recent Developments

Abstract

Soil carbon sequestration represents a critical natural climate solution for mitigating atmospheric CO₂ concentrations while simultaneously enhancing soil health and agricultural productivity. This comprehensive review examines recent developments in soil carbon sequestration strategies, analysing mechanisms, quantification methods, and implementation approaches across diverse ecosystems. Recent studies demonstrate that optimised agricultural practices can increase soil organic carbon stocks by 0.4-1.2 Mg C ha⁻¹ yr⁻¹, contributing significantly to climate mitigation goals. Key strategies include conservation agriculture, cover cropping, biochar application, agroforestry systems, and enhanced weathering techniques. However, permanence challenges, measurement uncertainties, and socioeconomic barriers remain significant constraints. Advanced monitoring technologies, including remote sensing and machine learning approaches, show promise for improving carbon stock assessments. Emerging credit carbon markets create financial incentives for soil carbon sequestration, with voluntary markets currently dominating transactions. Policy frameworks incorporating carbon credits and incentive mechanisms are emering globally to promote adoption. This review synthesises current knowledge on sequestration potential across different soil types and climatic zones, evaluates technological innovations, and identifies research priorities. Future directions emphasise integrated landscape management, precision agriculture technologies, and nature-based solutions. Achieving substantial climate mitigation through soil carbon sequestration requires coordinated efforts combining scientific advancement, policy support, and stakeholder engagement to realise the full potential of soils as carbon sinks.

Keywords: Soil Carbon, Climate Mitigation, Sequestration Strategies, Agricultural Practices, Carbon Stocks

Introduction

The escalating climate crisis demands urgent implementation of carbon dioxide removal strategies to complement emission reduction efforts. Among natural climate solutions, soil carbon sequestration emerges as a particularly promising approach, offering the potential to remove substantial atmospheric CO₂ while delivering co-benefits for agricultural productivity, biodiversity conservation, and ecosystem resilience. Global soils contain approximately 2,500 Pg of carbon in the top meter, representing three times the atmospheric carbon pool and four times the biotic carbon pool. This vast reservoir positions soils as critical components in global carbon cycling and climate regulation.

Soil organic matter is the largest terrestrial carbon pool (Poeplau, 2021). Historical agricultural practices have depleted soil organic carbon (SOC) stocks by 50-70% in cultivated lands, releasing 133 Pg C to the atmosphere since the advent of agriculture. However, this depletion creates opportunities for carbon restoration through improved management practices. Recent assessments suggest global agricultural soils could sequester 0.9-1.85 Pg C yr⁻¹, offsetting 10-20% of annual anthropogenic emissions. India, with 142 million hectares of agricultural land experiencing severe SOC depletion, presents substantial sequestration potential through targeted interventions. Carbon sequestration involves the long-term storage of carbon dioxide or other forms of carbon in the oceans, soils, vegetation (particularly forests), and geologic formations in order to prevent or delay global warming (Kaushal et al., 2023).

The mechanisms underlying soil carbon sequestration involve complex interactions between biological, chemical, and physical processes. The net removal of carbon dioxide (CO2) from the atmosphere or the prevention of emission of carbon dioxide (CO2) into the atmosphere by terrestrial ecosystems is referred to as carbon sequestration (Das et al., 2025; Enebe et al., 2025). Plant photosynthesis captures atmospheric CO₂, transferring carbon to soils through root exudates, biomass decomposition, and microbial transformations. Stabilisation mechanisms include physical protection within aggregates, chemical association with mineral surfaces, and biochemical recalcitrance of organic compounds. Understanding these mechanisms enables the development of management strategies that enhance carbon inputs while minimising losses through decomposition and erosion.

This review synthesises recent advances in soil carbon sequestration science, evaluating strategies across agricultural, grassland, and forest ecosystems. We examine technological innovations in measurement and monitoring, analyse policy frameworks and economic incentives, and identify barriers to large-scale implementation. By integrating multidisciplinary perspectives, this review aims to provide comprehensive guidance for researchers, policymakers, and practitioners working to harness soil carbon sequestration for climate mitigation while advancing sustainable land management objectives.

Table 1: Global Soil Carbon Stocks and Sequestration Potential

| Ecosystem Type | Area (Mha) | Current SOC Stock (Pg C) | Depletion Level (%) | Sequestration Potential (Pg C yr⁻¹) | Time Frame (years) | Confidence Level |
| --- | --- | --- | --- | --- | --- | --- |
| Croplands | 1,550 | 160 | 50-70 | 0.4-0.8 | 20-50 | High |
| Grasslands | 3,400 | 470 | 10-30 | 0.2-0.5 | 30-60 | Medium |
| Degraded lands | 2,000 | 120 | 60-80 | 0.3-0.6 | 25-40 | Medium |
| Paddy soils | 160 | 35 | 40-60 | 0.05-0.15 | 20-30 | High |
| Peatlands | 400 | 550 | 20-40 | 0.1-0.3 | 50-100 | Low |
| Forest soils | 4,000 | 780 | 10-25 | 0.2-0.4 | 40-80 | Medium |
| Urban soils | 250 | 15 | 70-90 | 0.02-0.05 | 15-25 | Low |

Figure 1: Soil Carbon Stabilisation Mechanisms

Mechanisms of Soil Carbon Sequestration

Carbon Input Pathways

Soil carbon accumulation fundamentally depends on the balance between carbon inputs and outputs. Primary input pathways include above-ground plant residues, root biomass and exudates, microbial necromass, and organic amendments. Recent isotopic studies reveal that root-derived carbon contributes 2-3 times more to stable SOC formation than shoot residues, highlighting the importance of deep-rooting species and perennial vegetation. Mycorrhizal fungi, particularly arbuscular mycorrhizae, contribute significantly through hyphal biomass and glomalin production, accounting for 15-20% of total SOC in agricultural systems.

Stabilization Mechanisms

Modern understanding of SOC stabilisation has shifted from recalcitrance-based models to emphasise organo-mineral interactions and physical protection. Mineral-associated organic matter (MAOM) represents 50-80% of total SOC in agricultural soils, formed through the adsorption of dissolved organic compounds onto clay minerals and metal oxides. Fe and Al oxides demonstrate particularly strong stabilisation capacity, protecting organic matter from microbial decomposition for decades to centuries. Aggregate formation creates physical barriers limiting enzyme and microbial access, with microaggregates (<250 μm) providing the greatest protection. Recent research identifies pyrogenic carbon from incomplete combustion as highly stable, persisting for centuries to millennia.

Table 2: Carbon Stabilisation Mechanisms and Residence Times

| Stabilization Mechanism | Carbon Pool | Typical Size | Residence Time | Controlling Factors |
| --- | --- | --- | --- | --- |
| Mineral association | MAOM | <53 μm | 50-1000 years | Clay content, pH |
| Aggregate occlusion | iPOM | 53-250 μm | 10-100 years | Aggregate stability |
| Biochemical recalcitrance | Plant polymers | Variable | 5-50 years | Chemical composition |
| Pyrogenic carbon | Biochar | >250 μm | 100-1000 years | Pyrolysis temperature |
| Deep soil burial | Subsoil SOC | Variable | 100-500 years | Soil depth |
| Anaerobic preservation | Wetland SOC | Variable | 50-200 years | Water saturation |
| Cryogenic protection | Permafrost C | Variable | 1000+ years | Temperature |

Soil Carbon Sequestration Strategies

Conservation Agriculture Practices

Conservation agriculture, encompassing minimal soil disturbance, permanent soil cover, and crop diversification, demonstrates substantial carbon sequestration potential. No-till systems reduce SOC mineralisation by maintaining aggregate structure and limiting oxygen exposure. Meta-analyses indicate no-till practices sequester 0.3-0.6 Mg C ha⁻¹ yr⁻¹ compared to conventional tillage, with the greatest benefits in humid climates. However, carbon accumulation often concentrates in surface layers (0-10 cm), with limited changes at depth. Combining no-till with cover crops enhances sequestration to 0.5-1.0 Mg C ha⁻¹ yr⁻¹ through increased carbon inputs and year-round soil coverage.

Cover Cropping and Green Manures

Integration of cover crops provides multiple carbon sequestration pathways through biomass inputs, enhanced root production, and nitrogen fixation by leguminous species. Winter cover crops in temperate systems contribute 1-4 Mg C ha⁻¹ yr⁻¹ of biomass carbon, with 10-20% retained as SOC after decomposition. Legume-grass mixtures optimise carbon-nitrogen dynamics, promoting microbial biomass and aggregate formation. Crotalaria juncea, Sesbania aculeata, and Vigna radiata demonstrate particular effectiveness in tropical systems, contributing 2-5 Mg biomass ha⁻¹ while fixing 50-150 kg N ha⁻¹.

Figure 2: Cover Crop Carbon Sequestration Pathways

Biochar Application

Biochar represents a targeted approach for introducing recalcitrant carbon into soils while improving fertility and water retention. Pyrolysis temperatures of 400-700°C optimise carbon stability and surface functionality. Application rates of 10-30 Mg ha⁻¹ increase SOC stocks by 10-30%, with 70-90% of added carbon persisting beyond 100 years. Co-benefits include enhanced cation exchange capacity, improved water holding capacity, and reduced greenhouse gas emissions. Economic constraints currently limit large-scale adoption, though integrated biochar-bioenergy systems show promise for improving feasibility.

Table 3: Biochar Properties and Carbon Sequestration Potential

| Feedstock Type | Pyrolysis Temperature | Carbon Content | Stability Factor | Application Rate | SOC Increase | Co-benefits |
| --- | --- | --- | --- | --- | --- | --- |
| Wood residues | 500-700°C | 75-85% | 0.85-0.95 | 20-40 Mg ha⁻¹ | 15-25% | High porosity |
| Crop residues | 400-600°C | 60-75% | 0.70-0.85 | 10-30 Mg ha⁻¹ | 10-20% | Nutrient retention |
| Manure | 350-500°C | 40-60% | 0.60-0.75 | 15-25 Mg ha⁻¹ | 8-15% | P availability |
| Rice husk | 450-650°C | 65-80% | 0.75-0.90 | 10-20 Mg ha⁻¹ | 12-22% | Si supplement |
| Bamboo | 550-750°C | 80-90% | 0.90-0.98 | 25-45 Mg ha⁻¹ | 20-30% | High stability |
| Municipal waste | 400-550°C | 45-65% | 0.65-0.80 | 20-35 Mg ha⁻¹ | 10-18% | Waste management |
| Sewage sludge | 300-450°C | 35-55% | 0.55-0.70 | 15-30 Mg ha⁻¹ | 7-12% | Heavy metal fixation |

Agroforestry Systems

Agroforestry integrates trees with crops or livestock, substantially enhancing carbon storage above and below ground. Systems sequester 0.5-3.0 Mg C ha⁻¹ yr⁻¹ in soils, with additional biomass carbon in woody vegetation. Deep tree roots access subsoil layers, depositing carbon beyond typical crop rooting zones. Leaf litter provides continuous organic inputs while modifying microclimate conditions favourable for SOC accumulation. Leucaena leucocephala, Gliricidia sepium, and Acacia mangium demonstrate exceptional performance in tropical agroforestry, fixing nitrogen while contributing high-quality organic matter.

Figure 3: Agroforestry Carbon Pools Distribution

Integrated Nutrient Management

Balanced nutrient management optimises plant productivity and carbon inputs while minimising losses through enhanced decomposition. Combining organic and inorganic fertilisers promotes microbial activity and aggregate formation. Farmyard manure applications of 10-15 Mg ha⁻¹ yr⁻¹ increase SOC by 0.3-0.7 Mg C ha⁻¹ yr⁻¹ while improving soil structure. Precision nutrient management using soil testing and variable rate technology reduces over-fertilisation which accelerates SOC mineralisation. Silicon and calcium amendments enhance carbon stabilisation through organo-mineral complex formation.

Table 4: Nutrient Management Effects on Soil Carbon

| Management Practice | Application Rate | SOC Change | Mechanism | Duration | Cost-effectiveness |
| --- | --- | --- | --- | --- | --- |
| FYM application | 10-15 Mg ha⁻¹ yr⁻¹ | +0.4-0.8% yr⁻¹ | Direct C input | Long-term | High |
| Compost addition | 5-10 Mg ha⁻¹ yr⁻¹ | +0.3-0.6% yr⁻¹ | Stable C input | Long-term | Medium |
| Green manuring | 20-30 Mg ha⁻¹ | +0.2-0.5% yr⁻¹ | Fresh biomass | Seasonal | High |
| Biosolids | 10-20 Mg ha⁻¹ yr⁻¹ | +0.5-0.9% yr⁻¹ | Organic matter | Medium-term | Medium |
| Integrated NPK+OM | Balanced doses | +0.3-0.7% yr⁻¹ | Enhanced productivity | Long-term | High |
| Biofertilizers | 20-30 kg ha⁻¹ | +0.1-0.3% yr⁻¹ | Microbial activity | Short-term | Very high |
| Crop residue retention | 5-8 Mg ha⁻¹ yr⁻¹ | +0.2-0.4% yr⁻¹ | Surface mulch | Continuous | Very high |

Measurement and Monitoring Technologies

Traditional and Advanced Sampling Methods

Accurate quantification of SOC stocks requires representative sampling accounting for spatial heterogeneity and depth distribution. Traditional grid sampling with 20-50 points per hectare provides baseline accuracy, though labour costs limit frequency. Stratified random sampling based on soil maps and terrain attributes improves efficiency. Recent advances in electromagnetic induction and ground-penetrating radar enable rapid field-scale carbon mapping. Portable spectrometers using visible near-infrared reflectance (VNIR) provide real-time SOC estimates with 85-95% accuracy compared to laboratory analysis.

Figure 4: Soil Carbon Monitoring Technologies

Remote Sensing Applications

Satellite and aerial remote sensing increasingly support large-scale SOC monitoring. Remote Sensing platforms, including Synthetic Aperture Radar and Multispectral Imager, offer high-resolution data for assessing soil and vegetation characteristics (Beisekenov et al., 2025). Multispectral indices correlate with surface SOC content, particularly in bare soil conditions. Machine learning algorithms integrating Sentinel-2 imagery, digital elevation models, and climate data predict SOC stocks with R² values of 0.65-0.85. Hyperspectral sensors demonstrate enhanced capability for SOC mapping, though costs remain prohibitive for routine monitoring. Unmanned aerial vehicles equipped with multispectral cameras enable field-scale assessment with centimetre-level resolution.

Modeling Approaches

Process-based models simulate SOC dynamics under different management scenarios, supporting decision-making and carbon credit verification. RothC, CENTURY, and DayCent models represent established tools, though parameter uncertainty limits accuracy. Machine learning approaches using random forests and neural networks show promise for improving predictions by capturing non-linear relationships. Integration of remote sensing data with process models through data assimilation techniques reduces uncertainty in regional carbon assessments.

Table 5: Comparison of SOC Measurement Methods

| Method | Spatial Scale | Accuracy | Cost per Sample | Time Required | Advantages | Limitations |
| --- | --- | --- | --- | --- | --- | --- |
| Dry combustion | Point | ±2-5% | $15-30 | 2-4 hours | Gold standard | Destructive |
| Wet oxidation | Point | ±5-10% | $5-15 | 1-2 hours | Low cost | Incomplete oxidation |
| VNIR spectroscopy | Point/Field | ±10-15% | $2-10 | Minutes | Rapid, portable | Calibration needed |
| Eddy covariance | Ecosystem | ±15-20% | $50,000+ setup | Continuous | Net flux measurement | High cost |
| Remote sensing | Landscape | ±20-30% | $0.1-1/ha | Days | Large area coverage | Surface only |
| Modeling | Regional | ±25-40% | $0.01-0.1/ha | Hours-days | Scenario testing | Parameter uncertainty |
| Isotopic analysis | Point | ±3-5% | $50-100 | 1-2 weeks | Source partitioning | Expensive |

Figure 5: Global Soil Carbon Monitoring Network

Figure 6: Global Carbon Credit Projects

Fig 7: Growth of Carbon Credit Global Market

Economic and Policy Dimensions

Carbon Credit Markets

Carbon markets, epitomised by emissions trading systems and carbon offset mechanisms, have witnessed rapid expansion and evolution (Kristjanpoller & Minutolo, 2025). To mitigate global greenhouse emissions and climate change, the Kyoto Protocol was ratified in 1997, which proposed the concept of carbon credits and carbon market (Gorain et al., 2021). Emerging carbon markets create financial incentives for soil carbon sequestration, with voluntary markets currently dominating transactions. Carbon credit prices range from $5-50 per Mg CO₂, depending on verification standards and co-benefits. However, high transaction costs and measurement uncertainties limit smallholder participation. Blockchain technology shows promise for reducing verification costs and improving transparency. Aggregation models enabling collective participation of small farmers demonstrate success in Kenya and India.

Policy Frameworks and Incentives

National and international policies increasingly recognise soil carbon's mitigation potential. The "4 per 1000" initiative targets annual SOC increases of 0.4% to offset anthropogenic emissions. India's National Mission for Sustainable Agriculture promotes soil health cards and organic farming to enhance carbon stocks. Payment for ecosystem services schemes in Costa Rica and Mexico include soil carbon in benefit calculations. However, policy coherence remains challenging, with agricultural subsidies often favouring intensive practices that deplete SOC.

Table 6: Economic Analysis of Carbon Sequestration Practices

| Practice | Implementation Cost | Annual Maintenance | Carbon Price Needed | Break-even Time | Co-benefit Value | Risk Level |
| --- | --- | --- | --- | --- | --- | --- |
| No-till adoption | $50-150/ha | $20-40/ha | $15-25/Mg CO₂ | 3-5 years | High | Low |
| Cover cropping | $80-200/ha | $60-120/ha | $20-35/Mg CO₂ | 4-7 years | Medium | Medium |
| Biochar application | $500-1500/ha | $50-100/ha | $40-80/Mg CO₂ | 8-15 years | High | Low |
| Agroforestry | $300-800/ha | $50-150/ha | $10-20/Mg CO₂ | 5-10 years | Very high | Medium |
| Rotational grazing | $100-300/ha | $30-80/ha | $12-22/Mg CO₂ | 3-6 years | High | Low |
| Wetland restoration | $1000-5000/ha | $100-300/ha | $25-50/Mg CO₂ | 10-20 years | Very high | High |
| Precision agriculture | $200-500/ha | $50-150/ha | $18-30/Mg CO₂ | 4-8 years | Medium | Medium |

Challenges and Limitations

Permanence and Reversibility

Soil carbon sequestration faces inherent permanence challenges, as stored carbon remains vulnerable to release through land-use change, climate extremes, or management shifts. Drought and warming accelerate decomposition, potentially converting soils from sinks to sources. Contracts ensuring 20-100 year permanence struggle with enforcement and farmer commitment. Insurance mechanisms and buffer pools provide partial solutions, though costs reduce economic viability.

Saturation and Sink Capacity

Soils exhibit finite capacity for carbon storage, reaching new equilibria after 20-50 years of improved management. Saturation levels depend on clay content, climate, and management intensity. Fine-textured soils approach saturation at 50-80 Mg C ha⁻¹, while sandy soils saturate at 20-40 Mg C ha⁻¹. Continued carbon inputs maintain stocks but don't increase sequestration rates, limiting long-term mitigation potential.

Figure 8: Soil Carbon Saturation Curves

Socioeconomic Barriers

Adoption barriers include high initial costs, technical knowledge requirements, and cultural resistance to practice changes. Smallholder farmers, managing 80% of global farms, face particular constraints in accessing capital and information. Gender disparities in land tenure and decision-making authority limit women's participation despite their dominant role in agricultural labour. Extension services often lack the capacity to support soil carbon management, requiring substantial investment in training and infrastructure.

Table 7: Barriers to Soil Carbon Sequestration Adoption

| Barrier Type | Affected Stakeholders | Severity | Geographic Variation | Potential Solutions | Implementation Timeline |
| --- | --- | --- | --- | --- | --- |
| Economic costs | Smallholder farmers | High | Higher in developing nations | Subsidies, carbon credits | 2-5 years |
| Technical knowledge | All farmers | Medium | Variable | Extension services | 1-3 years |
| Land tenure | Tenant farmers | Very high | Critical in Asia, Africa | Policy reform | 5-10 years |
| Market access | Remote communities | High | Severe in isolated regions | Infrastructure, cooperatives | 3-7 years |
| Cultural practices | Traditional farmers | Medium | Strong in indigenous areas | Participatory approaches | 2-4 years |
| Policy support | Large-scale farmers | Low | Limited in some regions | Advocacy, lobbying | 1-2 years |
| Climate risks | All farmers | Increasing | Higher in vulnerable zones | Insurance, adaptation | Ongoing |

Figure 9: Adoption Barriers and Solutions

Future Perspectives and Research Priorities

Emerging Technologies

Artificial intelligence and Internet of Things (IoT) technologies promise to revolutionise soil carbon management. Smart sensors continuously monitor soil conditions, enabling precision interventions. Gene editing techniques develop crops with enhanced root biomass and exudate production. Microbial inoculants engineered for carbon stabilisation show early promise. Blockchain-based measurement, reporting, and verification systems reduce transaction costs while ensuring transparency.

Integrated Landscape Approaches

Future strategies emphasise landscape-scale management recognising interconnections between agricultural lands, forests, wetlands, and urban areas. Watershed-based planning optimises carbon sequestration while managing water resources and biodiversity. Circular economy principles integrate urban organic waste into agricultural carbon cycles. Climate-smart agriculture bundles adaptation and mitigation practices, building resilience while sequestering carbon.

Research Priorities

Critical knowledge gaps require targeted research investment. Understanding deep soil carbon dynamics below 30 cm remains limited despite representing 50-70% of total stocks. Microbial mechanisms governing carbon stabilisation need elucidation to optimise management. Developing region-specific sequestration factors improves carbon credit accuracy. Social science research on adoption behaviours and policy effectiveness guides implementation strategies. Long-term experiments tracking carbon permanence under climate change provide essential data for model validation.

Figure 10: Future Research Framework

Figure 11: Integrated Landscape Carbon Management

Conclusion

Soil carbon sequestration represents a critical yet underutilised strategy for climate change mitigation, offering substantial potential while delivering multiple co-benefits. This review demonstrates that implementing optimal management practices across global agricultural lands could sequester 0.9-1.85 Pg C annually, significantly contributing to climate targets. However, realising this potential requires overcoming technical, economic, and social barriers through integrated approaches combining scientific innovation, supportive policies, and stakeholder engagement. Future success depends on developing robust monitoring systems, creating viable economic incentives, and building capacity among land managers. As climate urgency intensifies, soil carbon sequestration must transition from a promising concept to widespread practice, recognising soils as essential natural infrastructure for planetary health.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References

- Amelung, W., Bossio, D., de Vries, W., Kögel-Knabner, I., Lehmann, J., Amundson, R., ... & Chabbi, A. (2020). Towards a global-scale soil climate mitigation strategy. Nature Communications, 11(1), 5427.

- Baveye, P. C., Berthelin, J., Tessier, D., & Lemaire, G. (2018). The "4 per 1000" initiative: A credibility issue for the soil science community? Geoderma, 309, 118-123.

- Chen, S., Martin, M. P., Saby, N. P., Walter, C., Angers, D. A., & Arrouays, D. (2018). Fine resolution map of top-and subsoil carbon sequestration potential in France. Science of the Total Environment, 630, 389-400.

- Dignac, M. F., Derrien, D., Barré, P., Barot, S., Cécillon, L., Chenu, C., ... & Basile-Doelsch, I. (2017). Increasing soil carbon storage: mechanisms, effects of agricultural practices and proxies. A review. Agronomy for Sustainable Development, 37(2), 14.

- Fuss, S., Lamb, W. F., Callaghan, M. W., Hilaire, J., Creutzig, F., Amann, T., ... & Minx, J. C. (2018). Negative emissions—Part 2: Costs, potentials and side effects. Environmental Research Letters, 13(6), 063002.

- Gogoi, B., Nath, T., Deka, B., Kalita, S., Gajurel, S., Borah, D., ... & Das, K. (2021). Soil carbon sequestration potential of different land use systems in India: A review. Environmental Monitoring and Assessment, 193(12), 1-25.

- IPCC. (2022). Climate Change 2022: Mitigation of Climate Change. Contribution of Working Group III to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change. Cambridge University Press.

- Lal, R. (2018). Digging deeper: A holistic perspective of factors affecting soil organic carbon sequestration in agroecosystems. Global Change Biology, 24(8), 3285-3301.

- Lehmann, J., Cowie, A., Masiello, C. A., Kammann, C., Woolf, D., Amonette, J. E., ... & Whitman, T. (2021). Biochar in climate change mitigation. Nature Geoscience, 14(12), 883-892.

- Minasny, B., Malone, B. P., McBratney, A. B., Angers, D. A., Arrouays, D., Chambers, A., ... & Winowiecki, L. (2017). Soil carbon 4 per mille. Geoderma, 292, 59-86.

- Paustian, K., Lehmann, J., Ogle, S., Reay, D., Robertson, G. P., & Smith, P. (2016). Climate-smart soils. Nature, 532(7597), 49-57.

- Poulton, P., Johnston, J., Macdonald, A., White, R., & Powlson, D. (2018). Major limitations to achieving "4 per 1000" increases in soil organic carbon stock in temperate regions: Evidence from long‐term experiments at Rothamsted Research, UK. Global Change Biology, 24(6), 2563-2584.

- Rumpel, C., Amiraslani, F., Koutika, L. S., Smith, P., Whitehead, D., & Wollenberg, E. (2018). Put more carbon in soils to meet Paris climate pledges. Nature, 564(7734), 32-34.

- Smith, P., Adams, J., Beerling, D. J., Beringer, T., Calvin, K. V., Fuss, S., ... & Keesstra, S. (2019). Land-management options for greenhouse gas removal and their impacts on ecosystem services and the sustainable development goals. Annual Review of Environment and Resources, 44, 255-286.

- Sokol, N. W., Sanderman, J., & Bradford, M. A. (2019). Pathways of mineral‐associated soil organic matter formation: Integrating the role of plant carbon source, chemistry, and point of entry. Global Change Biology, 25(1), 12-24.

- Sykes, A. J., Macleod, M., Eory, V., Rees, R. M., Payen, F., Myrgiotis, V., ... & Smith, P. (2020). Characterising the biophysical, economic and social impacts of soil carbon sequestration as a greenhouse gas removal technology. Global Change Biology, 26(3), 1085-1108.

- Tao, F., Palosuo, T., Valkama, E., & Mäkipää, R. (2019). Cropland soils in China have a large potential for carbon sequestration based on literature survey. Soil and Tillage Research, 186, 70-78.

- Vermeulen, S., Bossio, D., Lehmann, J., Luu, P., Paustian, K., Webb, C., ... & Wollenberg, E. (2019). A global agenda for collective action on soil carbon. Nature Sustainability, 2(1), 2-4.

- Zomer, R. J., Bossio, D. A., Sommer, R., & Verchot, L. V. (2017). Global sequestration potential of increased organic carbon in cropland soils. Scientific Reports, 7(1), 15554.

- Poeplau, C. (2021). Grassland soil organic carbon stocks along management intensity and warming gradients. Grass and Forage Science, 76(2), 186-195.

- Beisekenov, N., Banakinaou, W., Ajayi, A. D., Hasegawa, H., & Tadao, A. (2025). Remote Sensing-Based Soil Organic Carbon Monitoring Using Advanced Machine Learning Techniques Under Conservation Agriculture Systems. Smart Agricultural Technology, 101036.

- Kristjanpoller, W., & Minutolo, M. C. (2025). Exploring the Dynamic Interplay: Carbon Credit Markets and Asymmetric Multifractal Cross-Correlations with Financial Assets. Fractal and Fractional, 9(10), 638.

- Gorain, S., Malakar, A., & Chanda, S. (2021). An analysis of carbon market and carbon credits in India. Asian Journal of Agricultural Extension, Economics & Sociology, 39(2), 40–49. https://doi.org/10.9734/ajaees/2021/v39i230528

- Kaushal, S., Sharma, N., Singh, I., & Singh, H. (2023). Soil carbon sequestration: A step towards sustainability. International Journal of Plant & Soil Science, 35(11), 160–171. https://doi.org/10.9734/ijpss/2023/v35i112957

- Das, S., Beegum, S., Acharya, B. S., & Panday, D. (2025). Soil Carbon Sequestration: A Mechanistic Perspective on Limitations and Future Possibilities. Sustainability.

- Enebe, M. C., Ray, R. L., & Griffin, R. W. (2025). Carbon sequestration and soil responses to soil amendments–A review. Journal of Hazardous Materials Advances, 100714.