Optimization of High-Pressure Processing Parameters for Microbial Load Reduction in Young Coconut Water Using Response Surface Methodology

Abstract

This study investigated the effect of high-pressure processing (HPP) on the microbial load of young coconut water and optimized the processing parameters using Central Composite Design (CCD) and Response Surface Methodology (RSM). Young coconut water was treated at different pressure levels (300–600 MPa) and holding times (3–6 minutes). The microbiological analyses included Total Plate Count (TPC), Yeast and Mould, Coliform, Escherichia coli, Psychrophilic Bacteria and Staphylococcus aureus. A second-order polynomial response surface model was developed to describe the relationship between pressure, holding time, and microbial reduction. The model showed good prediction accuracy, with a high coefficient of determination (R² = 0.9914) and a non-significant lack-of-fit, indicating its adequacy in predicting microbial behaviour under HPP. The results showed that treatments above 450 MPa effectively reduced all microbial counts to undetectable levels (<1 log CFU/ml), while lower pressures (300 MPa) resulted in measurable TPC values. Optimization using the response surface model indicated that the optimum condition for microbial inactivation was 450 MPa for 3 minutes, achieving a desirability value of 1.0. Pressure was identified as the dominant factor influencing microbial reduction, whereas holding time had a lesser effect. These results indicate that HPP can effectively prolong the shelf life of young coconut water while ensuring microbial safety and product freshness. The developed model can be used  as a predictive tool for industrial-scale processing, highlighting the potential of HPP as a non-thermal preservation method to maintain the natural characteristics of young coconut water.

Keywords: high-pressure processing, young coconut water, microbial reduction, response surface methodology, optimization

Introduction

Young coconut water (YCW) is the clear liquid endosperm of immature coconuts, and its volume depends on the maturity stage of the fruit (Burns et al., 2020). It naturally comprises sugars and electrolytes in a balanced ratio (Alchoubassi et al., 2021; Kumar et al., 2021), making it a nutritious isotonic beverage and a healthier option than many commercial sports drinks that have synthetic ingredients (Kailaku et al., 2015). YCW is also rich in phytochemicals and antioxidant compounds (Lobo et al., 2021; Shayanthavi et al., 2024), which provide numerous health benefits and contribute to its growing global demand (Rethinam & Krishnakumar, 2022). However, its high perishability caused by microbial growth after opening reduces its shelf life to about one week under refrigeration at 4 °C (Zhang et al., 2020).

Plant-based beverages like YCW are highly perishable and prone to microbial spoilage due to their natural enzyme activity and the presence of contaminating microorganisms such as Escherichia coli, Listeria monocytogenes, and Salmonella spp. (Silva & Evelyn, 2023). Conventional extraction methods can result in microbial contamination up to 10⁶ CFU/mL within a day (Balter et al., 2005). Therefore, effective preservation methods are critical to ensure product safety and extended shelf life during distribution and storage. Conventional heat-based preservation methods such as pasteurization and sterilization can extend shelf life but often lead to the degradation of nutrients (Nindo et al., 2007), bioactive compounds (Lund, 1988), and antioxidants (Orellana-Palma et al., 2021), as well as undesirable changes in flavour and colour (Negri Rodríguez et al., 2021). Although conventional thermal processing effectively inactivates microorganisms, it often causes undesirable quality deterioration, including the loss of antioxidants, vitamins, and pigments, as well as the development of off-flavours and browning.

To address these limitations, non-thermal preservation technologies such as membrane filtration, ultraviolet (UV) irradiation, and high-pressure processing (HPP) have been explored as alternatives to heat treatment. Membrane filtration allows clarification and cold sterilization (Debien et al., 2013; Karmakar & De, 2017), whereas UV irradiation effectively inactivates enzymes and microorganisms (Maguluri et al., 2021; Yannam et al., 2020). Among these methods, HPP has become one of the most promising non-thermal pasteurization techniques for fruit juices and beverages, as it efficiently inactivates vegetative microorganisms while preserving nutritional and sensory qualities (Waghmare, 2024). HPP can achieve up to a 5-log reduction in major pathogens such as E. coli O157:H7, Salmonella, and L. monocytogenes (USFDA, 2004). Although bacterial spores are resistant to HPP (Black et al., 2007), the technology is widely used to meet the growing demand for minimally processed, fresh-like products, with the global HPP food market projected to exceed USD 10 billion by 2028 (Visiongain, 2015). The effectiveness of HPP depends on processing parameters such as pressure and holding time, with higher pressures generally providing greater microbial inactivation (Matías et al., 2024). In addition, the composition of the food matrix, including pH, water activity, and initial microbial load, strongly influences treatment effectiveness (Augusto et al., 2018; Koutsoumanis et al., 2022). Therefore, each product matrix should be evaluated individually to determine optimal HPP conditions.

Response Surface Methodology (RSM) is a collection of statistical and mathematical techniques used to develop, improve, and optimize processes. It is widely utilized across various industries, including the food sector, where multiple process variables may influence product quality. RSM allows researchers to systematically study the effects of several factors and their interactions on desired responses while reducing the number of experiments required. This approach has proven effective in designing, formulating, and refining both new and existing products (Myers et. al, 2016; Yolmeh & Jafari, 2017).

RSM is particularly useful for optimizing non-thermal technologies such as high-pressure processing (HPP), where parameters like pressure and holding time significantly affect microbial safety and product stability (Matías et al., 2024). Optimizing these parameters is crucial to guarantee the microbial safety and extend the shelf life of young coconut water (YCW) without compromising its natural characteristics. Therefore, this study aimed to investigate the effects of pressure and holding time during HPP on the microbial quality of young coconut water. RSM was used to determine the optimal conditions for microbial inactivation, with the goal of enhancing microbial safety, extending shelf life and maintaining product quality.

Materials and Method

### Preparation of Young Coconut Water

Young coconuts were obtained from MARDI Bagan Datuk, Perak, Malaysia, and transported by lorry directly to the Food Science and Technology Research Centre, MARDI Serdang, Selangor, Malaysia. Coconuts were washed and soaked in a chlorine solution (50 mg/L) for 30 minutes to reduce surface microorganisms and prevent contamination before opening to collect the coconut water. The collected water was filtered through a muslin cloth (150-200µm) to remove solid impurities. The filtrate of young coconut water was then packed into PET/AL/PE pouches (200 mL), sealed and stored at 4 °C prior to treatment.

### High-Pressure Processing (HPP) Treatment

The packed young coconut water was subjected to high-pressure processing at pressure levels ranging from 300 to 600 MPa for holding times between 3 and 6 minutes. HPP treatment was conducted using a high-pressure unit (Hiperbaric 55, Burgos, Spain) located at the Faculty of Food Science and Technology, Universiti Putra Malaysia (UPM). The processing parameters (pressure and holding time) were set according to the experimental design. Following treatment, the samples were stored under refrigerated conditions (2–7 °C) prior to microbiological analysis.

Microbiological analysis

Microbiological analysis was carried out to determine the total plate count (TPC), yeast and moulds (Y&M), coliform, Escherichia coli (E. coli), Staphylococcus aureus (S. aureus), and psychrophilic bacteria in both fresh and HPP-treated young coconut water samples. The enumeration of microorganisms was performed according to the procedures described in the United States-Food and Drug Administration (US-FDA) Bacteriological Analytical Manual (BAM) standard methods (Feng et al., 2002). Samples were serially diluted using sterile peptone water, and 1 mL aliquots were plated on specific culture media appropriate for each microbial group. The plates were incubated under optimal conditions as prescribed in the guidelines. Microbial counts were expressed as colony-forming units per millilitre (CFU/mL) and converted to logarithmic values (log CFU/mL) for statistical analysis. All microbial data were expressed as number of colony forming units (log CFU/g) with plates enumeration based on 25 to 250 CFU/g.

Experimental Design and Optimisation

Response Surface Methodology (RSM) was employed to investigate the effects of high-pressure processing (HPP) parameters on the microbial load of young coconut water. A 22Central Composite Design (CCD) was applied using Design-Expert software version 6.0.10 (Stat-Ease Inc., Minneapolis, MN, USA), which was used to design the experiment, perform regression analysis, estimate model coefficients and conduct the analysis of variance (ANOVA).

Two independent variables, pressure (A) and holding time (B), were studied. Based on previous research on HPP-treated coconut water (Lukas, 2013; Raghubeer et al., 2020; Waghmare, 2024) and preliminary trials, the pressure and holding time were set within the ranges of 300–600 MPa and 3–6 minutes, respectively. The processing temperature was maintained at 20 °C to ensure non-thermal sterilization effects.

The dependent variables (responses) were total plate count (TPC), yeast and moulds (Y&M), coliforms, Escherichia coli (E. coli), Staphylococcus aureus (S. aureus), and psychrophilic bacteria. A face-centred CCD was selected, in which the star points are located at the centre of each face of the factorial space. Each factor was investigated at three coded levels (−1, 0, +1).

The RSM design generated 14 experimental runs, calculated as:

Tn=r(2f)+2f+K=2(22)+(2×2)+2=14        (1)

Where Tn  represents the total number of experiments, f is the number of factors, r is the number of replications of the factorial points, and K is the number of centre points. The 14 runs consisted of 8 factorial points (each combination replicated once), 4 axial (star) points positioned at the centre of each face of the factorial space, and 2 replicated centre points used  to estimate experimental error and assess the lack of fit of the model (Table 1).

The experiments were performed in randomized order to minimize bias, with each run conducted in triplicate. Data were analysed using analysis of variance (ANOVA), and model adequacy was assessed based on the coefficient of determination (R²), adjusted R², p-values, and the lack-of-fit test. The relationship between independent and dependent variables was expressed by a second-order polynomial model:

y=β0+β1x1+β2x2+β11x12+β22x22+β12x1x2         (2)

where y is the predicted response (microbial load); β0, β1 and β2 are the intercept and linear coefficients; β11and β22are quadratic coefficients; and β12 represents the interaction coefficient.

In the Response Surface Methodology (RSM), the relationship between the independent variables, namely pressure level (A) and holding time (B), was expressed using a second-order polynomial model. The experimental data were fitted to this model as shown in Equation (2) and analysed for adequacy through analysis of variance (ANOVA). For a model to be valid, the regression and model terms must be statistically significant at p ≤ 0.05.

Numerical optimization was performed using Design-Expert software to determine the optimal combination of pressure and holding time that minimized microbial counts while maintaining product quality. The desirability function approach was applied to identify the best overall operating conditions that satisfied all response criteria simultaneously.

Table 1. Experimental design of two-level CCD-RSM

| Run | A:Pressure (MPa) | B: Time (min) |
| --- | --- | --- |
| 1 | 600 | 3 |
| 2 | 600 | 4.5 |
| 3 | 450 | 3 |
| 4 | 450 | 6 |
| 5 | 300 | 3 |
| 6 | 600 | 3 |
| 7 | 600 | 6 |
| 8 | 300 | 6 |
| 9 | 450 | 4.5 |
| 10 | 600 | 6 |
| 11 | 300 | 3 |
| 12 | 300 | 6 |
| 13 | 450 | 4.5 |
| 14 | 300 | 4.5 |

#### Model Validation

Model validation was carried out to assess the adequacy and predictive capability of the developed response surface model. Confirmation experiments were conducted at the predicted optimum conditions to compare experimental and model-predicted microbial counts. The model’s validity was evaluated based on the agreement between predicted and experimental values, as well as key statistical parameters such as the coefficient of determination (R²), adjusted R² and the lack-of-fit test.

RESULTS AND DISCUSSIONS

Microbiological analysis

The microbial analysis of fresh and HPP-treated young coconut water revealed a significant reduction in total plate count (TPC) with increasing pressure levels (Table 2). In untreated samples (control), the initial TPC was 2.39 log CFU/mL, indicating the presence of naturally occurring microorganisms. This result is in accordance with Ma et al. (2019), who reported a TPC of 2.03 ± 0.65 log CFU/mL in untreated coconut water. However, after HPP treatment at pressures above 450 MPa, TPC decreased to undetectable levels (<1 log CFU/mL), confirming the effectiveness of pressure in microbial inactivation. Treatments at 300 MPa showed partial reduction, with residual TPC values ranging from 1.3 to 1.9 log CFU/mL, suggesting that this pressure level was insufficient to achieve complete sterilization.

No detectable growth of yeast and moulds (Y&M), coliform, Escherichia coli (E. coli), Staphylococcus aureus (S. aureus), or psychrophilic bacteria was observed in samples treated at or above 450 MPa. The absence of these microorganisms indicates that HPP effectively eliminated both spoilage and potential pathogenic microorganisms without the need for heat treatment. These findings are consistent with previous reports that demonstrated the capability of HPP to inactivate vegetative cells of microorganisms in liquid foods, such as fruit juices and coconut water, while maintaining product quality (Lukas, 2013; Raghubeer et al., 2020; Waghmare, 2024).

The enhanced microbial safety of HPP-treated young coconut water can be attributed to the disruption of microbial cell membranes and enzyme systems under high pressure, leading to cell inactivation. Since no thermal damage occurs, the natural freshness and nutritional properties of the beverage are preserved. Therefore, HPP is an effective non-thermal preservation technique for extending the shelf life of young coconut water while ensuring microbiological safety.

Table 2. Experimental design of two-level CCD-RSM and microbial counts of young coconut water

| Run | A:Pressure (MPa) | B: Time (min) | TPC (log CFU/ml) | Y&M (log CFU/ml) | Coliform (log CFU/ml) | E.coli (log CFU/ml) | PB (log CFU/ml) | S. aureus (log CFU/ml) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control | - | - | 2.39 | 1.50 | 0 | 0 | 1.53 | 1.02 |
| 1 | 600 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 600 | 4.5 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 450 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 450 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 300 | 3 | 1.90 | 0 | 0 | 0 | 0 | 0 |
| 6 | 600 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | 600 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 300 | 6 | 1.5 | 0 | 0 | 0 | 0 | 0 |
| 9 | 450 | 4.5 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 600 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | 300 | 3 | 1.79 | 0 | 0 | 0 | 0 | 0 |
| 12 | 300 | 6 | 1.30 | 0 | 0 | 0 | 0 | 0 |
| 13 | 450 | 4.5 | 0 | 0 | 0 | 0 | 0 | 0 |
| 14 | 300 | 4.5 | 1.87 | 0 | 0 | 0 | 0 | 0 |

Model fitting and Analyses

As shown in Table 3, Total Plate Count (TPC) was found to be significant (p < 0.05), while yeast and mould (Y&M), Escherichia coli (E. coli), Staphylococcus aureus (S. aureus), coliform, and psychrophilic bacteria were not significant (p > 0.05). Therefore, TPC was the only microbial response selected for model development and optimization. The fitted quadratic model for TPC as a function of pressure (A) and time (B) is expressed as:

Y1=0.038-0.84A-0.089B+0.86A2-0.076B2+0.11AB(3)

where Y1represents the Total Plate Count (TPC), A is the pressure level (MPa), and B is the holding time (min). The model expressed in natural factors (actual units) can be written as:

TPC = 0.038 − 0.84(Pressure) − 0.089(Time) + 0.86(Pressure)² − 0.076(Time)² + 0.11(Pressure)(Time)  (4)

The model for TPC was highly significant (p < 0.0001) and exhibited a strong correlation coefficient (R² = 0.9914) and an adjusted R² value of 0.9860. According to Paucar-Menacho et al. (2017), an R² value greater than 0.75 indicates a good fit between the predicted and observed data. The high R² values obtained in this study suggest that the model accurately represented the experimental data and effectively explained the variations in TPC.

Pressure (A) and time (B) both showed significant linear effects on TPC (p < 0.05), with pressure having a more dominant influence. The quadratic term of pressure (A²) was also significant (p < 0.0001), indicating the presence of a curvature effect in the response surface. Furthermore, the interaction between pressure and time (AB) significantly affected TPC (p = 0.0136). The lack of fit for the model was not significant (p = 0.1086), confirming that the model adequately described the experimental data.

Table 3: Regression coefficients and analysis of variance (ANOVA) for Total Plate Count (TPC)

| Response | Coefficient | F-value | p-value |
| --- | --- | --- | --- |
| Constant, | 0.038 | 184.09 | <0.0001 |
| A- Pressure | -0.84 | 700.83 | <0.0001 |
| B- Time | -0.089 | 7.94 | 0.0226 |
| A2 | 0.86 | 192.31 | <0.0001 |
| B2 | -0.076 | 1.51 | 0.2538 |
| AB | 0.11 | 9.93 | 0.0136 |
| Lack of fit | - | 3.44 | 0.1086 |
| R2 | 0.9914 |  |  |
| Adj R2 | 0.9860 |  |  |

Optimisation

Optimization was performed using the numerical optimization function (Desirability function) in Design-Expert software, where the desired goals for both independent variables and the response were specified. Among all microbial parameters, only TPC was included for optimization because no microbial growth was detected for yeast and mould (Y&M), Coliform, E. coli, psychrophilic bacteria, and S. aureus in most treatment combinations. During the optimization process, TPC was set as the response to be minimized, while both pressure and holding time were set “in range.” The software generated an optimal condition of 450 MPa for 3 minutes, with a predicted TPC value of 0 log CFU/ml and a desirability value of 1.0 (Table 3). This indicates that high-pressure processing at 450 MPa for 3 minutes effectively eliminated the microbial load in young coconut water, providing the best preservation condition among the tested treatments (Table 4).

Table 4. Optimal solution for young coconut water

| No. | Pressure (MPa) | Time (min) | TPC (log CFU/ml) | Desirability |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 450 | 3 | 0.00000 | 1.0 | Selected |

### Model Validation

To verify the adequacy of the developed model, a validation experiment was conducted under the predicted optimal condition (450 MPa for 3 minutes). The experimental TPC was determined using the same microbiological procedures described in the Materials and Methods section, including serial dilution of the treated coconut water, plating 1 mL aliquots on Plate Count Agar, and incubating the plates according to BAM standard conditions before enumeration. The experimental TPC obtained under this condition was consistent with the predicted value, showing less than 10% deviation (Table 5). This confirms the model’s reliability and predictive accuracy and demonstrates that the optimized HPP parameters were effective in reducing microbial load in young coconut water.

Table 5: The predicted and actual values from a response under optimum conditions

| Responses | Predicted Value | Experimental Value |
| --- | --- | --- |
| TPC | 0.00000 | 0.00000 |

CONCLUSION

This study confirmed that high-pressure processing (HPP) is an effective non-thermal preservation method for young coconut water. Using Central Composite Design (CCD) and Response Surface Methodology (RSM), pressure and holding time were successfully optimized to minimize total plate count (TPC). The optimal condition was identified as 450 MPa for 3 minutes, under which the experimental TPC matched the predicted value, with less than 10% deviation. Other microbial indicators, including yeast and mould, coliform, E. coli, S. aureus, and psychrophilic bacteria, were already undetectable (<1 log CFU/mL) under the tested conditions (Table 2). The high coefficient of determination (R² = 0.9914) and non-significant lack-of-fit confirmed the model’s adequacy in explaining the relationship between HPP parameters and TPC reduction. These findings demonstrate that pressure is the dominant factor influencing microbial inactivation. The optimized HPP treatment can effectively extend the shelf life of young coconut water while maintaining its natural freshness, quality and safety. Further studies on physicochemical properties and sensory attributes during storage are recommended to support large-scale commercial application.

ACKNOWLEDGEMENT

The authors gratefully acknowledge all parties involved in this project, particularly the staff at the Food Science and Technology Research Centre, MARDI, for their invaluable contributions. This study was conducted under the Development of Standard Operating Procedure (SOP) for Minimally-processed Products using HPP Technology (Project No. K-RFS03-1001).

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.Chatgpt 4.0

2.

3.

References

Alchoubassi, G., Kińska, K., Bierla, K., Lobinski, R., & Szpunar, J. (2021). Speciation of essential nutrient trace elements in coconut water. Food Chemistry, 339, 127680. https://doi.org/10.1016/j.foodchem.2020.127680

Augusto, P. E. D., Tribst, A. A. L., & Cristianini, M. (2018). High hydrostatic pressure and high-pressure homogenization processing of fruit juices. In Fruit juices: Extraction, composition, quality and analysis (pp. 393–421). Elsevier. https://doi.org/10.1016/B978-0-12-802230-6.00020-5

Balter, S., Weiss, D., Hanson, H., Reddy, V., Das, D., & Heffernan, R. (2005). Three years of emergency department gastrointestinal syndromic surveillance in New York City: What have we found? MMWR Morbidity & Mortality Weekly Report, 54(Suppl.), 175–180.

Black, E. P., Setlow, P., Hocking, A. D., Stewart, C., Kelly, A. L., & Hoover, D. G. (2007). Response of spores to high-pressure processing. Comprehensive Reviews in Food Science and Food Safety, 6(4), 103–121.

Debien, I. C. D., Gomes, M., Ongaratto, R. S., & Viotto, L. A. (2013). Ultrafiltration performance of PVDF, PES, and cellulose membranes for the treatment of coconut water (Cocos nucifera L.). Food Science and Technology, 33(4), 676–684. https://doi.org/10.1590/S0101-20612013000400012

Feng, P., Weagant, S. D., Grant, M. A., Burkhardt, W., Shellfish, M. and Water, B. (2002). Bacteriological Analytical Manual (BAM), Chapter 4 and 5. Retrieved on 14 August 2023 from FDA BAM website: https://www.fda.gov/food/foodscienceresearch/laboratorymethods/ucm064948.htm

Kailaku, S. I., Alam Syah, A. N., Risfaheri, Setiawan, B., & Sulaeman, A. (2015). Carbohydrate-electrolyte characteristics of coconut water from different varieties and its potential as a natural isotonic drink. International Journal of Advanced Science, Engineering and Information Technology, 5(3), 174–177. https://doi.org/10.18517/ijaseit.5.3.515

Karmakar, S., & De, S. (2017). Cold sterilization and process modeling of tender coconut water by hollow fibers. Journal of Food Engineering, 200, 70–80. https://doi.org/10.1016/j.jfoodeng.2016.12.021

Koutsoumanis, K., Alvarez-Ordóñez, A., Bolton, D., Bover-Cid, S., Chemaly, M., Davies, R., De Cesare, A., Herman, L., Hilbert, F., Lindqvist, R., Nauta, M., Peixe, L., Ru, G., Simmons, M., Skandamis, P., Suffredini, E., Castle, L., Crotta, M., Grob, K., … Allende, A. (2022). The efficacy and safety of high-pressure processing of food. EFSA Journal, 20(3), e07128. https://doi.org/10.2903/j.efsa.2022.7128

Kumar, M., Saini, S. S., Agrawal, P. K., Roy, P., & Sircar, D. (2021). Nutritional and metabolomics characterization of coconut water at different nut developmental stages. Journal of Food Composition and Analysis, 96, 103738. https://doi.org/10.1016/j.jfca.2020.103738

Lobo, A. P., Arpitha, K. V., Shetty, K. D., Rai, S. T., Nayak, P., Kumar, A., et al. (2021). Phytochemical and pharmacological evaluation of yellow and green varieties of Cocos nucifera water. Journal of Pharmaceutical Research International, 33(51A), 180–186. https://doi.org/10.9734/jpri/2021/v33i51A33483

Lukas, A. R. (2013). Use of high-pressure processing to reduce foodborne pathogens in coconut water [Master’s thesis, Virginia Tech]. Virginia Tech Digital Library. http://hdl.handle.net/10919/24760

Lund, D. (1988). Effects of heat processing on nutrients. In E. Karmas & R. S. Harris (Eds.), Nutritional evaluation of food processing (pp. 319–354). Springer. https://doi.org/10.1007/978-94-011-7030-7_12

Ma, Y., Xu, L., Wang, S., Xu, Z., Liao, X., & Cheng, Y. (2019). Comparison of the quality attributes of coconut waters by high‐pressure processing and high‐temperature short time during the refrigerated storage. Food science & nutrition, 7(4), 1512-1519.

Maguluri, R. K., Nettam, P., Chaudhari, S. R., & Yannam, S. K. (2021). Evaluation of UV-C LEDs efficacy for microbial inactivation in tender coconut water. Journal of Food Processing and Preservation, 45(9), e15727. https://doi.org/10.1111/jfpp.15727

Matías, C., Ludwig, I. A., Cid, C., Sáiz-Abajo, M., & De Peña, M. (2024). Exploring optimal high-pressure processing conditions on a (poly)phenol-rich smoothie through response surface methodology. LWT, 206, 116595. https://doi.org/10.1016/j.lwt.2024.116595

Myers, R. H., Montgomery, D. C., & Anderson-Cook, C. M. (2016). Response surface methodology: Process and product optimization using designed experiments (4th ed.). Wiley.

Negri Rodríguez, L. M., Arias, R., Soteras, T., Sancho, A., Pesquero, N., Rossetti, L., et al. (2021). Comparison of the quality attributes of carrot juice pasteurized by ohmic heating and conventional heat treatment. LWT, 145, 111255. https://doi.org/10.1016/j.lwt.2021.111255

Nindo, C. I., Powers, J. R., & Tang, J. (2007). Influence of refractance window evaporation on quality of juices from small fruits. LWT - Food Science and Technology, 40(6), 1000–1007. https://doi.org/10.1016/j.lwt.2006.07.006

Orellana-Palma, P., Guerra-Valle, M., Gianelli, M. P., & Petzold, G. (2021). Evaluation of freeze crystallization on pomegranate juice quality in comparison with conventional thermal processing. Food Bioscience, 41, 101106.

Paucar-Menacho, L. M., Martínez-Villaluenga, C., Dueñas, M., Frias, J., & Peñas, E. (2017). Optimization of germination time and temperature to maximize the content of bioactive compounds and the antioxidant activity of purple corn (Zea mays L.) by response surface methodology. LWT - Food Science and Technology, 76(B), 236–244.

Raghubeer, E. V., Phan, B. N., Onuoha, E., Diggins, S., Aguilar, V., Swanson, S., et al. (2020). The use of high-pressure processing (HPP) to improve the safety and quality of raw coconut (Cocos nucifera L.) water. International Journal of Food Microbiology, 331, 108697. https://doi.org/10.1016/j.ijfoodmicro.2020.108697

Rethinam, P., & Krishnakumar, V. (2022). Composition, properties and reactions of coconut water. In P. Rethinam & V. Krishnakumar (Eds.), Coconut water: A promising natural health drink—distribution, processing and nutritional benefits (pp. 77–138). Springer. https://doi.org/10.1007/978-3-031-10713-9_4

Shayanthavi, S., Kapilan, R., & Wickramasinghe, I. (2024). Comprehensive analysis of physicochemical, nutritional, and antioxidant properties of various forms and varieties of tender coconut (Cocos nucifera L.) water in Northern Sri Lanka. Food Chemistry Advances, 4, 100645. https://doi.org/10.1016/j.focha.2024.100645

Silva, F. V. M., & Evelyn. (2023). Pasteurization of food and beverages by high-pressure processing (HPP) at room temperature: Inactivation of Staphylococcus aureus, Escherichia coli, Listeria monocytogenes, Salmonella, and other microbial pathogens. Applied Sciences, 13(2), 1193. https://doi.org/10.3390/app13021193

U.S. Food and Drug Administration. (2004). Guidance for industry: Juice hazard analysis critical control point hazards and controls guidance (1st ed.). https://www.fda.gov/regulatory-information/search-fda-guidance-documents/guidance-industry-juice-hazard-analysis-critical-control-point-hazards-and-controls-guidance-first

Visiongain. (2015). The food high pressure processing (HPP) technologies market forecast 2015–2025. https://www.visiongain.com/Report/1406/-2015-2025

Waghmare, R. (2024). High pressure processing of fruit beverages: A recent trend. Food and Humanity, 2, 100232. https://doi.org/10.1016/j.foohum.2024.100232

Yannam, S. K., Patras, A., Pendyala, B., Vergne, M., Ravi, R., Gopisetty, V. V. S., et al. (2020). Effect of UV-C irradiation on the inactivation kinetics of oxidative enzymes, essential amino acids and sensory properties of coconut water. Journal of Food Science and Technology, 57(10), 3564–3572. https://doi.org/10.1007/s13197-020-04388-4

Yolmeh, M., & Jafari, S. M. (2017). Applications of response surface methodology in the food industry processes. Food and Bioprocess Technology, 10, 413–433. https://doi.org/10.1007/s11947-016-1855-2

Zhang, Y., Chen, W., Chen, H., Zhong, Q., Yun, Y., & Chen, W. (2020). Metabolomics analysis of the deterioration mechanism and storage time limit of tender coconut water during storage. Foods, 9(1), 4. https://doi.org/10.3390/foods9010004