Original Research Article

## Nutrient Enrichment and Eutrophication Potential of Agricultural Runoff in Otuoke, Bayelsa State, Nigeria

## Abstract

The present study evaluated the extent of nutrient enrichment and eutrophication potential of the Otuoke River in Bayelsa State as influenced by agricultural runoff. Nine surface water samples were collected from three monitoring stations located upstream, midstream, and downstream during the 2025 wet season and analysed according to the APHA (2017) standard method. Selected physicochemical parameters—pH, dissolved oxygen (DO), biological oxygen demand (BOD₅), nitrate, phosphate, ammonium, and chlorophyll-a—were determined using a Hanna HI98194 multi-parameter probe and Shimadzu UV-1800 spectrophotometer. Results showed a decrease in DO, which dropped from 6.8 mg/L at the upstream site to 4.1 mg/L at the downstream site, with a concurrent increase in BOD₅, rising from 2.8 to 5.4 mg/L, indicating organic enrichment occurred. Nutrient concentrations showed downstream increases in nitrate and phosphate, with nitrate ranging, and ammonium, indicators of runoff and fertiliser, increasing from 0.31 to 0.68 mg/L. Chlorophyll-a concentration ranged from 8.4 to 23.5 µg/L, while the Secchi depth decreased from 1.8 to 0.7 m. The Trophic State Index (TSI) recorded values ranging from 45.2 to 67.5, classifying the river from mesotrophic to eutrophic. The study revealed that strong positive correlations between nutrient concentration and chlorophyll-a (r ≥ 0.88) indicate nutrient loading was the primary factor controlling algal productivity and consequently reducing oxygen levels in the Otuoke River. The study concluded that agricultural practices are significant contributors to the trophic status of the river.

Keywords: nutrient enrichment; eutrophication; agricultural runoff; Otuoke River; Bayelsa State

1.0 Introduction

The enrichment of nutrients and eutrophication are critical issues affecting aquatic ecosystems worldwide, especially in the developing world. Here, agricultural intensification, human population growth, and weak regulation enforcement drive nutrient loading into water bodies. Eutrophication is the process by which increased levels of nitrogen (N) and phosphorus (P) stimulate algal growth, leading to hypoxia, habitat change, and a decline in aquatic biodiversity (Chapman, 1996; WHO, 2008). These two aspects of eutrophication have direct effects on water quality, fishery productivity, and the stability of aquatic ecosystems, creating ecological and socio-economic challenges. This is particularly evident in the tropics, including Nigeria's Niger Delta, where seasonal flooding and high annual rainfall contribute to runoff containing nutrients, compounded by the widespread use of fertilisers rich in nitrates and phosphates to boost crop yields. The overuse of fertilisers is problematic because improved crop yields often result in nutrient loss through surface runoff and leaching into nearby rivers, creeks, or drainage systems (Ma et al., 2016; Ekpe et al., 2025). Once nutrients enter waterways, they upset natural nutrient balances, promoting algal blooms and oxygen-consuming decomposition processes. This nutrient overload is further exacerbated by domestic effluents, livestock waste, and poor land management practices (Nriagu et al., 2016; Okpoji et al., 2025a).

Phosphorus is typically known as the limiting nutrient in freshwater ecosystems because even a slight increase can induce a rapid eutrophic response (Zhu & Ma, 2020). The United States Environmental Protection Agency (USEPA, 2015) suggests that total phosphorus concentrations below 0.1 mg/L can prevent ecological degradation, although tropical waters often reflect phosphorus concentrations well above this recommendation. Similar indicators of anthropogenic input can be seen when reflecting upon elevated nitrate and ammonium concentrations due to agricultural, organic waste or microbially mineralised nutrients (Ikpe et al., 2023; Dada et al., 2025). Each of these forms of nutrients promotes not only increased primary productivity but also increased frequency of harmful algal blooms or hypoxic events that may jeopardise aquatic life and water usability.

The Niger Delta is among the world's largest wetland systems and represents a distinctive environment for nutrient pollution and the risk of eutrophication. The region's hydrology is characterised by interconnected rivers, creeks, and floodplains, which facilitate nutrient accumulation and redistribution during the wet season (Nwilo & Badejo, 2005). Agricultural expansion, open defecation, and inadequate waste management contribute to nutrient enrichment, while oil-related activities further stress already vulnerable aquatic ecosystems (Ugochukwu & Ertel, 2008; Nriagu et al., 2016). Numerous studies have documented symptoms of eutrophication in the region, including algal blooms, fish kills, and anoxic conditions. These phenomena indicate that increased nutrient input poses a significant threat to aquatic biodiversity and ecosystem function (Ikpe et al., 2020; Dada et al., 2025). The Otuoke River in Bayelsa State exemplifies these challenges, as it traverses agricultural land where fertiliser application, organic waste disposal, and periodic high-flow flood events influence water chemistry.

Despite the river’s well-established ecological and socio-economic importance, limited research has addressed its nutrient dynamics and eutrophication status. Assessing the nutrient status of the river is essential for developing effective management strategies to maintain water quality and support healthy aquatic ecosystems. This study evaluates nutrient enrichment and the potential for eutrophication in the Otuoke River by quantifying concentrations of key nutrients (nitrate, phosphate, and ammonium), measuring algal productivity indicators (chlorophyll-a and Secchi depth), and determining trophic status using the Carlson Trophic State Index (TSI). The results provide scientific evidence to inform sustainable nutrient management, promote best agricultural practices, and support public water quality monitoring in the Niger Delta.

## 2.0 Materials and Methods

2.1 Study Area

The research was carried out in Otuoke in the Ogbia Local Government Area of Bayelsa State, Nigeria, located in the central Niger Delta area at a latitude of 4°48′N and a longitude of 6°18′E. The Otuoke River is a tributary of the Nun River and runs through an agricultural setting, having freshwater swamp forest and riparian grasses. Subsistence agriculture, especially crops such as cassava, plantain and vegetables, is widely practised on the floodplains. Water and nutrient runoff caused by rainfall, coupled with low topography and extensive agriculture, sends pollutants into the river. The climate is classified as humid tropical, and the region has both a wet season (March–October) and a dry season (November–February). The average annual rainfall is between 2,500 and 3,000 mm, while the average temperature is between 26 °C and 30°C

.

Figure 1: Map of the Study Area indicating Otuoke and the sample stations

2.2 Sampling Design and Collection

Three sampling stations were established along the river course to represent the upstream, midstream, and downstream sections. Sampling was carried out during the peak of the wet season (July–August 2025) to capture maximum nutrient runoff. At each station, triplicate surface water samples were collected at a depth of about 30 cm using pre-cleaned 1 L polyethene bottles. The bottles were rinsed with the river water before sampling to prevent contamination. Samples were stored in ice chests at 4 °C and transported to the laboratory within 24 hours. Field duplicates and blanks were collected to ensure quality control.

2.3 In-situ Measurement of Physicochemical Parameters

Temperature, pH, electrical conductivity (EC), and dissolved oxygen (DO) were measured in situ with a calibrated Hanna multiparameter probe (model HI98194). Daily calibration was conducted using standard buffers and conductivity solutions. Biological oxygen demand (BOD₅) was determined by the Winkler titrimetric method after dark incubation at 20 °C for five days in accordance with APHA (2017) procedures. Triplicate measurements were taken, and mean values were reported.

2.4 Laboratory Analysis of Nutrients

Nutrient parameters included nitrate (NO₃⁻), phosphate (PO₄³⁻), ammonium (NH₄⁺), total nitrogen (TN), and total phosphorus (TP), which were analysed according to standard methods published by APHA (2017). Nitrate was determined using the cadmium reduction method (APHA 4500-NO₃⁻ E) and read at 543 nm using a Shimadzu UV-1800 spectrophotometer. Phosphate was analysed through the ascorbic acid (molybdenum blue) method (APHA 4500-P E) and measured using a spectrophotometer at 880 nm. Ammonium was measured by the Nesslerisation method (APHA 4500-NH₃ B) and read at 425 nm. Total nitrogen and total phosphorus were measured using spectrophotometric detection following persulphate digestion. All reagents were analytical grade, and calibration standards were made from certified stock solutions. Triplicate measurements were conducted and expressed in milligrams per litre (mg L⁻¹).

2.5 Determination of Algal Productivity Indicators

## Indicators of algal biomass were assessed by measuring chlorophyll-a concentration and water transparency. Chlorophyll-a was obtained according to the USEPA (2015) protocol using 90% acetone. Water samples (500 mL) were filtered using Whatman GF/C filters, and pigments were extracted in 90% acetone at 4 °C for 24 h. Absorbance was measured with a UV-Vis spectrophotometer at wavelengths of 664 nm and 750 nm, and chlorophyll-a concentrations were expressed in µg L⁻¹. Water transparency was evaluated using a 20 cm Secchi disc that was lowered into the river until it was no longer visible. The mean of three observations was used as the Secchi depth for that site. The Carlson Trophic State Index was calculated using chlorophyll-a and Secchi depth values to determine the trophic state of the river: oligotrophic (<40), mesotrophic (40–50), eutrophic (50–70), and hypereutrophic (>70).

## 2.6 Quality Assurance and Quality Control (QA/QC)

## Comprehensive QA/QC protocols were implemented for field and laboratory methodologies. Instruments were calibrated daily with certified standards. Reagent blanks, field blanks, and duplicates were included in all analytical batches. Reported replicate analyses were within ±5%, relative percent difference (RPD), and calibration curves were maintained, with a correlation coefficient (R²) of ≥0.995. The method detection limits (MDLs) were 0.01 mg L⁻¹ for nitrate, 0.005 mg L⁻¹ for phosphate, and 0.01 mg L⁻¹ for ammonium. Acid-washed glassware and deionised water were used to limit contamination.

## 2.7 Statistical Analysis

## Descriptive statistics were calculated for all parameters (mean and standard deviation). Pearson correlation analysis was performed utilising SPSS version 25, in order to describe relationships between nutrients, chlorophyll-a, biological oxygen demand (BOD₅), and dissolved oxygen (DO). A statistical significance of p < 0.05 was used in the analysis. Results were interpreted against WHO (2008) and USEPA (2015) guidelines, and ecological thresholds for eutrophication were used to assess the Otuoke River's trophic status.

## 3.0 Results and Discussion

## 3.1 Results

The physicochemical characteristics of surface water in the Otuoke River exhibited notable spatial variation across the three sampling stations (upstream, midstream, and downstream) (Table 1). Temperature values increased gradually from 27.2 °C upstream to 29.1 °C downstream, reflecting the influence of direct solar radiation and reduced vegetative cover at lower reaches. The pH ranged from 6.45 to 6.89, indicating slightly acidic to near-neutral conditions that fall within the permissible range (6.5–8.5) recommended by WHO (2008) for aquatic life. Electrical conductivity (EC) values rose from 112.5 µS cm⁻¹ upstream to 194.3 µS cm⁻¹ downstream, signifying increased ionic strength and the accumulation of dissolved solids likely derived from fertiliser residues and eroded soil materials transported by surface runoff.

Dissolved oxygen (DO) concentration decreased markedly from 6.8 mg L⁻¹ upstream to 4.1 mg L⁻¹ downstream, whereas biological oxygen demand (BOD₅) increased from 2.8 to 5.4 mg L⁻¹. These inverse trends indicate that organic matter decomposition and microbial respiration intensified downstream, leading to moderate oxygen depletion. BOD₅ values approaching 6 mg L⁻¹ suggest moderate organic pollution associated with nutrient and organic waste inflows from adjacent farmlands and settlements.

Table 1: Physicochemical parameters of surface water in Otuoke River (mean ± SD)

| Parameter | Upstream | Midstream | Downstream | WHO (2008) limit |
| --- | --- | --- | --- | --- |
| Temperature (°C) | 27.2 ± 0.4 | 28.4 ± 0.3 | 29.1 ± 0.2 | – |
| pH | 6.45 ± 0.11 | 6.72 ± 0.08 | 6.89 ± 0.10 | 6.5–8.5 |
| EC (µS cm⁻¹) | 112.5 ± 5.6 | 156.8 ± 8.2 | 194.3 ± 9.1 | 1000 |
| DO (mg L⁻¹) | 6.8 ± 0.3 | 5.2 ± 0.2 | 4.1 ± 0.4 | ≥5 |
| BOD₅ (mg L⁻¹) | 2.8 ± 0.2 | 3.9 ± 0.3 | 5.4 ± 0.5 | 6 |

Nutrient concentrations increased progressively downstream (Table 2). Nitrate (NO₃⁻) concentrations ranged from 1.84 mg L⁻¹ upstream to 5.46 mg L⁻¹ downstream, while phosphate (PO₄³⁻) increased from 0.45 to 1.22 mg L⁻¹. Ammonium (NH₄⁺) levels varied between 0.31 and 0.68 mg L⁻¹, exceeding the recommended threshold of 0.5 mg L⁻¹ at the lower reach. Total nitrogen (TN) and total phosphorus (TP) similarly rose from 1.95 to 5.71 mg L⁻¹ and 0.51 to 1.18 mg L⁻¹, respectively. These increasing trends reflect nutrient enrichment through agricultural runoff, soil erosion, and possible domestic discharges. Although the measured nitrate and phosphate values were below WHO drinking-water limits, they exceeded ecological thresholds typically associated with eutrophic conditions (TP > 0.1 mg L⁻¹, NO₃⁻ > 1 mg L⁻¹).

Table 2: Nutrient concentrations in surface water (mg L⁻¹)

| Parameter | Upstream | Midstream | Downstream | Guideline limit |
| --- | --- | --- | --- | --- |
| Nitrate (NO₃⁻) | 1.84 ± 0.20 | 3.21 ± 0.30 | 5.46 ± 0.40 | 50 (WHO, 2008) |
| Phosphate (PO₄³⁻) | 0.45 ± 0.05 | 0.79 ± 0.08 | 1.22 ± 0.10 | 0.10 (USEPA, 2015 ecological) |
| Ammonium (NH₄⁺) | 0.31 ± 0.03 | 0.47 ± 0.05 | 0.68 ± 0.06 | 0.50 (WHO, 2008) |
| Total N (TN) | 1.95 ± 0.20 | 3.42 ± 0.30 | 5.71 ± 0.40 | – |
| Total P (TP) | 0.51 ± 0.06 | 0.84 ± 0.07 | 1.18 ± 0.09 | – |

Algal productivity indicators corroborated the nutrient data (Table 3). Chlorophyll-a, an index of phytoplankton biomass, increased sharply from 8.4 µg L⁻¹ upstream to 23.5 µg L⁻¹ downstream, exceeding the oligotrophic benchmark of ≤10 µg L⁻¹ (USEPA, 2015). Secchi depth declined from 1.8 m to 0.7 m, indicating reduced transparency and higher turbidity from phytoplankton proliferation. Calculated Carlson’s Trophic State Index (TSI) values ranged from 45.2 (mesotrophic) to 67.5 (eutrophic), confirming a downstream shift toward eutrophic status driven by nutrient enrichment and organic loading.

Table 3: Algal productivity indicators

| Parameter | Upstream | Midstream | Downstream | Standard (USEPA 2015) |
| --- | --- | --- | --- | --- |
| Chlorophyll-a (µg L⁻¹) | 8.4 ± 0.6 | 14.9 ± 1.2 | 23.5 ± 1.8 | ≤10 (oligotrophic) |
| Secchi depth (m) | 1.8 ± 0.1 | 1.2 ± 0.1 | 0.7 ± 0.1 | ≥1.0 |
| TSI | 45.2 | 56.8 | 67.5 | <50 (mesotrophic) |

Correlation analysis (Table 3) revealed strong positive relationships between nutrients and biological indicators. Nitrate (r = 0.91) and phosphate (r = 0.88) were highly correlated with chlorophyll-a, demonstrating that increased nutrient availability directly enhanced algal biomass. BOD₅ also showed positive correlations with nitrate (r = 0.83) and chlorophyll-a (r = 0.84), while DO exhibited a significant negative correlation with both parameters (r = –0.88 to –0.92), indicating oxygen depletion linked to high biological activity.

Table 4: Correlation matrix of selected water-quality parameters

| Parameter pairs | r - value | Relationship |
| --- | --- | --- |
| NO₃⁻ – PO₄³⁻ | 0.89 | Strong positive |
| NO₃⁻ – Chl-a | 0.91 | Strong positive |
| PO₄³⁻ – Chl-a | 0.88 | Strong positive |
| BOD₅ – NO₃⁻ | 0.83 | Positive |
| BOD₅ – Chl-a | 0.84 | Positive |
| DO – Chl-a | –0.88 | Strong negative |
| DO – BOD₅ | –0.92 | Strong negative |

3.2 Discussion

This research's results suggest that the Otuoke River experiences substantial nutrient enrichment and ongoing eutrophication, mainly from agricultural runoff and associated organic materials. The differing physicochemical and nutrient factors reflect the growing influence of human activities downstream, mirroring trends seen in lowland tropical rivers affected by agricultural and domestic activities (Dada et al., 2025; Ekpe et al., 2025). The temperature values of 27.2 – 29.1 °C reflect the usual range found in tropical aquatic systems and support increased metabolic activity of microbes and biogeochemical processes such as nitrification and mineralisation of organic matter (Chapman, 1996). Warm temperatures promote biochemical oxygen demand and nutrient turnover processes, thereby catalysing indirect eutrophication processes. The pH measured between 6.45 – 6.89 reflects soft acidic to neutral water, favourable for aquatic life and nutrient bioavailability. The observed slight acidification is attributed to microbial decomposition of organic residues and the consequent production of carbonic acids, as noted by Ikpe et al. (2023) in the Qua Iboe River. Electrical conductivity overall progressively increased moving downstream (112.5 – 194.3 µS cm⁻¹), reflecting greater quantities of dissolved ions, largely as a result of fertiliser residues, eroded soil particles, and leachates from organic matter and sediment. An increase in EC downstream has previously been associated with runoff and sediment–nutrient dynamics, an observation noted within agricultural watersheds (Nienie et al., 2017; Ma et al., 2016). Such ionic enrichment serves as an indicator of diffuse nutrient pollution rather than point-source discharges.

The inverse association between dissolved oxygen (DO) and biological oxygen demand (BOD₅) also supports the notion of organic enrichment resulting in eutrophic stress. DO decreased from 6.8 mg L⁻¹ upstream to 4.1 mg L⁻¹ downstream, whereas BOD₅ increased from 2.8 mg L⁻¹ to 5.4 mg L⁻¹, indicating increased microbial respiration from decomposing organic matter. Waters with BOD₅>3 mg L⁻¹ are considered moderately polluted (APHA, 2017). This finding is similar to Dada et al. (2025), who similarly reported oxygen depletion from nutrient enrichment in the Douglas Creek system (Nriagu et al. 2016). Degradation of dissolved oxygen can have negative impacts on aquatic biodiversity, fish production, and benthic community composition. Nutrient enrichment in the Otuoke River was apparent with downstream accumulation of nitrate (1.84–5.46 mg L⁻¹), phosphate (0.45–1.22 mg L⁻¹), and ammonium (0.31–0.68 mg L⁻¹). Although nitrate levels were below the WHO (2008) drinking-water guideline of 50 mg L⁻¹, they exceeded ecological thresholds (>1 mg L⁻¹) associated with algal blooms (USEPA, 2015). Phosphate concentrations far exceeded the 0.10 mg L⁻¹ ecological criterion, demonstrating substantial enrichment (Al Abbar et al. 2020). Elevated levels of ammonium at the lower reach (>0.5 mg L⁻¹) are a clear indicator of continued decomposition, and likely nitrogenous organic matter from fertilisers and livestock waste, which can contribute to eutrophication. These observations agree with the results of Ekpe et al. (2025) and Nienie et al. (2017), who identified agricultural leachates and sewage inflows as major nutrient sources in the Niger Delta rivers.

The elevated total nitrogen (TN = 1.95–5.71 mg L⁻¹) and total phosphorus (TP = 0.51–1.18 mg L⁻¹) concentrations confirm the river's nutrient-rich status. An average TN/TP ratio of about 5:1 indicates phosphorus limitation; however, both nutrients are available at concentrations likely to support eutrophic conditions (Zhu & Ma, 2020; Chapman, 1996). This low ratio also suggests that modest additional phosphorus inputs would exacerbate algal blooms. Ratios such as this have also been observed in sediment-dominated tropical streams where P-release under anoxic conditions sustains eutrophic cycling (Ekwere et al., 2025). Chlorophyll-a concentrations (8.4–23.5 µg L⁻¹) increased downstream with nutrient concentrations, indicating nutrient-stimulated primary productivity. Carlson’s (1977) threshold of 20 µg L⁻¹ for eutrophic conditions was reached. Similarly, the Trophic State Index (TSI = 45.2–67.5) confirmed that the Otuoke River lies in the mesotrophic to eutrophic class. Other studies have also documented transitions to eutrophic states in other Niger Delta rivers receiving nutrient loads from farmlands and settlements (Ikpe et al., 2020; Okpoji et al., 2025a). Secchi depth declined from 1.8 m to 0.7 m, confirming decreased transparency and increased turbidity due to an algal bloom and suspended particulate matter (Steichen et al. 2019).

Correlation analysis further confirms the relationship between nutrient enrichment and eutrophication. Strong positive correlations between nitrate, phosphate, and chlorophyll-a (r ≥ 0.88) illustrate that the availability of nutrients directly drives the accumulation of algal biomass. In contrast, the strong negative correlation between DO and BOD₅ (r = -0.92) indicates oxygen stress due to excessive biological activity and microbial decay. These patterns are indicative of a eutrophic system experiencing oxygen depletion due to excessive loading of organic materials (Ma et al., 2016; Nriagu et al., 2016). The study affirms that agricultural land use within close proximity to Otuoke really impacts water quality in a riverine setting. The effect of excess fertiliser application, together with surface runoff and waste management, has compounded both detrimental nutrient accumulation and resultant biological productivity, leading to the transition toward eutrophic conditions (Ugochukwu & Ertel, 2008).

The impacts of this trophic transfer pose ecological threats, including hypoxia, reduced biodiversity, and habitat loss, to the point that fisheries and the surrounding community livelihoods could be depleted. Furthermore, alga proliferation could also be sustained in systemic situations as a result of internal loading process mechanisms. Under low oxygen conditions, phosphorus bound to metal oxides in the sediment can be remobilised into the water column above, providing additional opportunity for algal growth even in low conditions of surface inputs (Ekwere et al., 2025). Feedback loops such as these constitute complexity in a potential remediation and imply an integrated watershed management approach that considers both control of nutrient sources along the process to restore habitat.

Conclusion

The findings of this study suggest that agricultural activities near the community of Otuoke have played an ongoing role in nutrient enrichment and the resulting eutrophication of the Otuoke River (Note that there is also information in the TSI that must be considered). The increases in nitrate, phosphate, and ammonium concentrations downstream, along with rising chlorophyll-a levels and decreasing dissolved oxygen, indicate a strong influence of agriculture and organic waste on the water quality of the Otuoke River. The Trophic State Index (TSI) values calculated characterise the river as mesotrophic to eutrophic (indicating moderate to high biological productivity and declining ecological health), while the association of nutrients, chlorophyll-a, and BOD5 values clearly shows that the primary cause of the eutrophication is nutrient loading into the river system. Based on these findings, sustainable agriculture practices within the catchment should be studied, including proper fertiliser use, timing of application, and vegetative buffer strips to reduce nutrient inflow. Regular water-quality monitoring should be conducted to assess seasonal changes and detect any emerging eutrophic conditions. Public awareness and community involvement in watershed management are also essential to protect the river from degradation.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Al Abbar, A., Ngai, S. C., Nograles, N., Alhaji, S. Y., & Abdullah, S. (2020). Induced pluripotent stem cells: Reprogramming platforms and applications in cell replacement therapy. BioResearch Open Access, 9(1), 121–136. https://doi.org/10.1089/biores.2019.0046

American Public Health Association. (1995). Standard methods for the examination of water and wastewater (19th ed.). APHA.
https://www.scirp.org/reference/referencespapers?referenceid=1916575

Anarado, C. J. O., Okpoji, A. U., & Anarado, C. E. (2023). Bioaccumulation and health risk assessment of lead, cadmium, arsenic, and mercury in blue crabs found in creeks in Bayelsa State of the Niger Delta region of Nigeria. Asian Journal of Environmental & Ecology, 21(4), 46–59.
http://95.179.195.156:8080/handle/123456789/853

Benson, R., Conerly, O. D., Sander, W., Batt, A. L., Boone, J. S., Furlong, E. T., Glassmeyer, S. T., Kolpin, D. W., Mash, H. E., Schenck, K. M., & Simmons, J. E. (2017). Human health screening and public health significance of contaminants of emerging concern detected in public water supplies. Science of the Total Environment, 579, 1643–1648.
https://doi.org/10.1016/j.scitotenv.2016.03.146

Chapman, D. (1996). Water quality assessments: A guide to the use of biota, sediments, and water in environmental monitoring (2nd ed.). UNESCO/WHO/UNEP.
https://www.taylorfrancis.com/books/mono/10.1201/9781003062103/water-quality-assessments-deborah-chapman

Dada, O. J., Obadimu, C. O., Ubuo, E. E., & Ekwere, I. O. (2025). Seasonal variation in some physicochemical parameters of water from Douglas Creek, Ibeno LGA, Akwa Ibom State. Journal of Environment, Climate, and Ecology, 2(2), 44–55.
https://doi.org/10.69739/jece.v2i2.976

Ekpe, J. E., Okpoji, A. U., Ekwere, I. O., Anarada, C. J., Ewuola, A. A., Ufuoma, V. O., Okpanachi, C. B., Okonkwo, C. O., & Obunezi, O. A. (2025). Physico-chemical transport of pesticides and nutrients in irrigation water and their fate in agro-ecosystems of Rivers State, Nigeria. Palgo Journal of Agriculture, 9(2), 30–37. 
https://doi.org/10.5281/zenodo.17148360

Ekwere, I. O., Okpoji, A. U., Igwegbe, K. C., Okonkwo, C. O., Yekeen, A. A., Obunezi, O. C., Okpanachi, C. B., Garuba, M. H., Ogini, O. R., & Odibo, U. E. (2025). Nutritional–Toxicological Trade-Off: Comparative Study of Polycyclic Aromatic Hydrocarbons in Smoked and Oven-Dried Nile Tilapia (Oreochromis niloticus). Journal of Environment, Climate, and Ecology, 2(2), 90-97. https://doi.org/10.69739/jece.v2i2.952

Ekwere, I. O., Okpoji, A. U., Ufuoma, V. O., Akinola, A. E., Raymond, C. A., Clement, R. O., Alaekwe, I. O., Etesin, M. O., & Edodi, I. O. (2025). Nutritional evaluation of Nile tilapia (Oreochromis niloticus) processed by different drying methods in Akwa Ibom State, Nigeria. Journal of Sustainable Research and Development, 1(2), 11–17. https://doi.org/10.69739/jsrd.v1i2.1033

Ikpe, E. E., Ekwere, I. O., Ukpong, E. G., Effiong, J. O., & Okon, O. E. (2020). Assessment of heavy metals and hydrocarbons in Rhizophora mangle, Callinectes sapidus, & sediment in Qua-Iboe River, Akwa Ibom State, Nigeria (wet and dry seasons). Global Journal of Advanced Engineering Technology and Sciences, 7(3).
https://gjaets.com/index.php/gjaets/article/view/23

Ikpe, E. E., Ekwere, I. O., Ukpong, E. G., Effiong, J. O., & Okon, O. E. (2019). Evaluation of the levels of heavy metals, total petroleum hydrocarbon and total hydrocarbon content in Tympanotonus fuscatus and sediment, Qua Iboe River, Akwa Ibom State, Nigeria (dry vs. wet season comparison). Chemical Science International Journal, 27(4), 1–17.
https://doi.org/10.9734/CSJI/2019/v27i430121

Ikpe, E. E., Etesin, U. M., Ubuo, E. E., & Ekwere, I. O. (2023). Bioaccumulation of heavy metals and hydrocarbons in sediment, shell and flesh of the fauna in Qua-Iboe River at Ibeno, Akwa Ibom State, Nigeria (samples collected in dry and wet seasons). Asian Journal of Chemical Sciences, 13(6), 71–93.
https://doi.org/10.9734/ajocs/2023/v13i6264

Ite, A. E., Harry, T. A., Obadimu, C. O., & Ekwere, I. O. (2019). Comparison of indoor air quality in schools: Urban vs. industrial “oil & gas” zones in Akwa Ibom State – Nigeria (rainy vs. dry seasons). Journal of Environment Pollution and Human Health, 7(1), 15–26.
https://doi.org/10.12691/jephh-7-1-3

John E., E., Emah O., A., Ifiok O., E., Okpoji, A. U., Iyam O., E., Henderson O., O., Akinola A., E., Gloria N., A., & Charles N., O. (2025). Health Risk Implications of Polycyclic Aromatic Hydrocarbons in Smoked and Dried Fish Consumed in Nigeria. Journal of Medical Science, Biology, and Chemistry, 2(2), 192-199. https://doi.org/10.69739/jmsbc.v2i2.1045

Ma, Y. H., Su, C. L., Liu, W. J., Zhu, Y. P., & Li, J. X. (2016). Identification of sulfate sources in the groundwater system of Zaozhuang: Isotopic and hydrochemical evidence. Environmental Science, 37, 4690–4699.
https://europepmc.org/article/med/29965309

Nienie, A. B., Sivalingam, P., Laffite, A., Ngelinkoto, P., Otamonga, J. P., Matand, A., Mulaji, C. K., Mubedi, J. I., Mpiana, P. T., & Poté, J. (2017). Seasonal variability of water quality by physicochemical indexes and traceable metals in a suburban area in Kikwit, DRC. International Soil and Water Conservation Research, 5(2), 158–165.
https://doi.org/10.1016/j.iswcr.2017.02.002

Nriagu, J. O., Udofia, E. A., Ekong, I., & Ebuk, G. (2016). Health risks associated with oil pollution in the Niger Delta, Nigeria. International Journal of Environmental Research and Public Health, 13(3), 346. https://doi.org/10.3390/ijerph13030346

Nwilo, P. C., & Badejo, O. T. (2005). Oil spill problems and management in the Niger Delta. Proceedings of the International Oil Spill Conference, 2005(1), 567–573.
https://doi.org/10.7901/2169-3358-2005-1-567

Okpoji, A. U., Anarado, C. O., Mmuta, E. C., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Eboj-Ajoku, I. O., & Obunezi, O. C. (2025). Toxicological evaluation of Pb, Cd, As, and Hg in blue crab from oil-polluted creeks in the Niger Delta. Journal of Life Science and Public Health, 1(1), 24–31.
https://doi.org/10.69739/jlsph.v1i1.879

Okpoji, A. U., Chinyere, U. E., Nwokoye, J. N., Ezekwuemen, O. I., Alaekwe, I. O., Odidika, C. C., Owughara, C. N., Enyi, C. M., & Kolawole, O. O. (2025). Environmental assessment of heavy metals and hydrocarbon pollution in surface waters of oil-bearing communities in Andoni, Rivers State, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 22–30.
http://www.ijmsrt.com

Okpoji, A. U., Eboh-Ajoku, I. O., Mmuta, E. C., Ndubuisi, J. O., Alaekwe, I. O., Odibo, U. E., Nwoka, N. G., Okafor, C. A., & Obunezi, O. C. (2025). Integrated environmental risk assessment of BTEX and PAHs in water and sediment samples from the Bonny River, Nigeria. ISA Journal of Multidisciplinary (ISAJM), 2(4), 14–21.

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Anarado, C. J. O., Ogbonna, C., Ewuola, A. A., Odibo, U. E., & Garuba, M. H. (2025). Volatile organic compounds from gas flaring and their atmospheric implications in the Niger Delta. Journal of Life Science and Public Health, 1(1), 32–39. https://doi.org/10.69739/jlsph.v1i1.962

Okpoji, A. U., Ekwere, I. O., Igwegbe, K. C., Etesin, U. M., Ugwuanyi, G. C., Okpanachi, C. B., Ewuola, A. A., Mojisola, K. M., & Ezekwueme, O. I. (2025). Solar-enhanced photocatalytic degradation of pharmaceutical residues in wastewater using Fe-, Cu-, and Zn-doped TiO₂ nanomaterials. Journal of Environment, Climate, and Ecology, 2(2), 56–62. https://doi.org/10.69739/jece.v2i2.961

Okpoji, A. U., Emem, J. A., Ekwere, I. O., Odibo, U. E., Alaekwe, I. O., Warder, A. B., & Eboj-Ajoku, O. I. (2025). Bioaccumulation of nickel, lead, and cadmium in tissues of Callinectes sapidus from the Iko River, Nigeria: Implications for human health risk and environmental safety. Journal of Environment, Climate, and Ecology, 2(2), 29–37.
https://doi.org/10.69739/jece.v2i2.844

Okpoji, A. U., Ndubuisi, J. O., Eboh-Ajoku, I. O., Emem, J. A., Ekwere, I. O., Alaekwe, I. O., Odibo, U. E., Igwegbe, K. C., Onoja, C. R., Warder, A. B., & InyangAbia, A. J. (2025). Trematode infections and histopathological effects in Chrysichthys nigrodigitatus from the polluted Andoni River, Niger Delta, Nigeria. Journal of Agriculture, Aquaculture, and Animal Science, 2(2), 66–72.
https://doi.org/10.69739/jaaas.v2i2.882

Okpoji, A. U., Nwoka, N. G., Odibo, U. E., Alaekwe, I. O., Okafor, C. A., Ogwu, N. G., & Akatakpo, C. U. (2025). Seasonal variation in hydrochemical characteristics and heavy metal risk assessment of groundwater in Andoni-Isiokwan District, Niger Delta, Nigeria. International Journal of Modern Science and Research Technology, 3(8), 44–51.
http://www.ijmsrt.com

Onoja, C. R., Ogbaji, H. O., Edodi, I. O., Mmuta, E. C., Igwegbe, K. C., Ogini, O. R., Odibo, U. E., Ewuola, A. A., Mahmoud, A. B., & Okpoji, A. U. (2025). Heavy Metal Contamination and Health Risk Assessment of Smoked-Dried Fish Sold in Eke-Awka Market, Anambra State, Nigeria. Journal of Sustainable Research and Development, 1(2), 18-25. https://doi.org/10.69739/jsrd.v1i2.1044

Steichen, C., Hannoun, Z., Luce, E., Hauet, T., & Dubart-Kupperschmitt, A. (2019). Genomic integrity of human induced pluripotent stem cells: Reprogramming, differentiation and applications. World Journal of Stem Cells, 11(10), 729–747. https://doi.org/10.4252/wjsc.v11.i10.729

Ugochukwu, C. N. C., & Ertel, J. (2008). Negative impacts of oil exploration on biodiversity management in the Niger Delta area of Nigeria. Impact Assessment and Project Appraisal, 26(2), 139–147. https://doi.org/10.3152/146155108X316397

Zhu, X., & Ma, J. (2020). Recent advances in the determination of phosphate in environmental water samples: Insights from practical perspectives. TrAC Trends in Analytical Chemistry, 127, 115908. https://doi.org/10.1016/j.trac.2020.115908