# Assessment of Trace Metal Contamination in Market Garden Soils of Korhogo City, Ivory Coast.

.

ABSTRACT

The aim of this study is to assess the level of trace metals (TMEs) contamination in market garden soils in the Korhogo city. The trace metal content of soils from three market gardening sites in Korhogo (Koko, Natio and Téguéré) was determined. Soil sampling was carried out in January 2025 and July 2025. The results of the soil analyses show that Pb levels vary between 5.650 and 6.970 mg/kg, while Cd levels range from 0.21 to 0.245 mg/kg. Cu concentrations ranged from 17.125 to 23.65 mg/kg, while Zn concentrations ranged from 27.255 to 34.85 mg/kg. Cr levels ranged from 1.175 to 1.305 mg/kg. The degree and sources of contamination were determined by calculating metal pollution indices: contamination factor (CF), geoaccumulation index (Igeo), overall pollution index (PLI), and ecological risk factor ( Eri). The Igeo values show that the various soils studied are not contaminated with Pb, Cu, Zn and Cr, while all these sites are moderately contaminated with Cd. The PLI values obtained show that there is no progressive deterioration of the soil due to TMEs. Furthermore, the TMEs studied present a low ecological risk in the soils, while Cd presents a moderate ecological risk.

Keywords: Trace metal, pollution, market garden soils, Korhogo

1. INTRODUCTION

The development of urban, industrial and agricultural activities in recent decades has led to numerous problems of agricultural soil pollution with trace metals (TMEs) [1]. Trace metals have a significant toxicological impact on plants, everyday consumer products and humans [1]. Some of their impacts, particularly on soil, are still poorly understood [2]. TMEs are pollutants frequently found in soil. They are now found in several economic sectors, such as market gardening [3].

In Korhogo, market gardening sites are located close to numerous industrial activities and roads with heavy traffic from a large number of cars and mopeds. The vegetables produced on these market gardens are therefore exposed to constant industrial effluent discharges, exhaust fumes and all kinds of waste [4]. The TMEs contained in the waste can accumulate in the soil and pollute groundwater through infiltration [5]. They can be absorbed by cultivated plants and enter the food chain, posing a risk of contamination to humans [6-8].

In addition, market gardeners use compost and other organic fertilisers such as poultry manure and animal droppings to improve the soil, as well as wastewater from swamps to irrigate their crops [9, 10]. They treat crop diseases and pests with excessive doses of pesticides [11]. These agricultural practices contribute to the contamination of soil and plants with heavy metals. The contamination of market garden soils by these heavy metals remains a serious and increasingly worrying environmental problem. The aim of this study is to assess heavy metal pollution in market garden soils in Korhogo city.

2. material and methods

2.1. Study area

The study sites are located in the city of Korhogo. This city lies between 9°20 and 9°50 north latitude and between 5°30 and 5°50 west longitude (Figure 1). It is located in northern Côte d'Ivoire, 565 km from Abidjan, and covers an area of 12,500 km², or 3.9% of the national territory [12].

Figure 1: Map of the study area (Korhogo city).

In addition, these market gardening sites were chosen for their large size, accessibility and similar environment. They are located in low-lying areas and along roads with very heavy traffic. In the vicinity of these sites, there are also dwellings, small industries repairing and manufacturing agricultural tools, and small open dumps that are sources of heavy metal pollution [13]. These criteria facilitate both sampling and observation of farming practices in the field. The proximity of roads is a factor in soil contamination, particularly vehicle exhaust fumes and particles from mechanical wear, which can introduce heavy metals such as lead and zinc into the soil at the roadside, potentially affecting crop quality [14].

2.2. Study materials

The instruments used to carry out the survey were: a camera, a Garmin Global Positioning System (GPS) and a manual auger.

2.3. Methods

2.3.1. Soil sampling

During this study, an exploratory mission was carried out in October 2024 to identify market gardening soil sampling sites in the city of Korhogo. To this end, the coordinates of the various sampling points were recorded using a Garmin Global Positioning System (GPS).

Two soil sampling campaigns (one in the dry season and the other in the rainy season) were carried out. Homogeneous 20 cm soil cores (the horizon worked by market gardeners and explored by the roots of the species studied) were collected using a manual auger.

Sampling was carried out randomly to ensure that the samples were representative and to avoid any spatial bias. Each market gardening site was divided into ten parts. In each part, sampling points were chosen at random to cover the different parts of the field. Soil samples were taken from each selected section using an auger. The samples collected from each section were then mixed to form a composite sample representative of that part of the site. Twenty (20) composite samples were obtained from each market garden site during the two campaigns. A total of sixty (60) samples were obtained from the three market garden sites during the two campaigns. Furthermore, this random approach ensured reliable and representative samples from each site, while minimising the risk of bias. All samples collected were stored in a cooler and transferred to the laboratory for analysis.

2.3.2. Methods and standards for analysing certain physical and chemical parameters

The methods for analysing pH, CEC and TOC are presented in Table 1.

Table 1: Analytical methods and standards

| Paramètres | Methodes d’analyse | Normes |
| --- | --- | --- |
| pHeau | Measuring pH using a pH-meter |  |
| CEC | Metson method | NFX 31-130 : 1999 |
| TOC | Oxidation of carbon by potassium dichromate (K₂Cr₂O₇) in an acidic environment | NF ISO 10 694 : 1995 |

2.4. Analysis of trace metals in soils

The soil samples were dried in the open air in the laboratory, then finely ground before being dissolved in hydrofluoric and perchloric acid in accordance with standard NF X 31-147. The principle of the digestion method is based on the decomposition of soils by hydrofluoric acid (HF) in combination with aqua regia (HNO₃: HCl; 1:3, v/v) when heated. The use of HF is essential because it completely dissolves both silicate lattices and all metals [15].

For each soil sample, 0.1 g of ground material was weighed and placed in Teflon boxes or bombs, then 1 mL of aqua regia (HNO₃, HCl, 1:3 v/v) and 6 mL of hydrofluoric acid (HF) were slowly added to each bomb. The bombs were first hermetically sealed using a torque wrench and then left to stand for one (1) hour under a fume hood. They were then placed in a water bath for 2 hours and 30 minutes at 120°C. During this time, 2.7 g of boric acid was weighed into 50 mL propylene tubes, to which 20 mL of distilled water was added and then homogenised.

Once the bombs were removed from the water bath, they were first left to cool at room temperature for 3 hours. They were opened to transfer the contents into 50 mL propylene tubes containing the previously prepared boric acid solution. Finally, the bombs were rinsed with distilled water and then added to the propylene tubes until the gauge line (50 mL) was reached.

After being dissolved, the samples were analysed using VARIAN AA240FS flame atomic absorption spectrometry coupled with a VARIAN EL0608 hydride furnace in accordance with standard NF EN 1483.

2.5. Metal accumulation indices

2.5.1. Geoaccumulation index (Igeo)

The Geoaccumulation Index (Igeo) provides information on the level of accumulation of metallic elements in the soil. It is assessed using the following formula:

𝐈𝐠e𝐨 =log2( Ci1,5. Cref)

Ci : concentration of metal i in the soil

Cref : geochemical background concentration in the Upper Continental Crust (UCC) value for the metal.

1,5: while 1.5 is the background matrix correction factor due to lithogenic effects.

The Igeo values enable seven contamination level classes to be defined, as shown in Table 2 [16].

Table 2: Different classes according to Müller (1981).

|  | Values | Level of contamination |
| --- | --- | --- |
| Class 0 | Igéo <0 | uncontaminated |
| Class 1 | 0< Igéo <1 | from uncontaminated to moderately contaminated |
| Class 2 | 1< Igéo <2 | moderately contaminated |
| Class 3 | 2< Igéo <3 | from moderately to strongly contaminated |
| Class 4 | 3< Igéo <4 | strongly contaminated) |
| Class 5 | 4< Igéo <5 | from strongly to extremely contaminated |
| Class 6 | 5<Igéo | extremely contaminated |

2.5.2. Contamination factor (CF)

The contamination factor (CF), also known as the contamination index, provides information on the level of metal contamination in soil. It is calculated using the following formula:

𝐅𝐂 = CiCref

Ci : concentration of metal i in the soil

Cref : geochemical background concentration in the Upper Continental Crust (UCC) value for the metal.

The different levels of contamination according to CF values are grouped together in Table 3.

Table 3: Level of contamination according to CF values.

| CF values | Contamination |
| --- | --- |
| CF <1 | Low contamination |
| 1 ≤ CF < 3 | Moderate contamination |
| 3 ≤ CF < 6 | High contamination |
| CF ≥ 6 | Very high contamination |

2.5.3. Pollution load index (PLI)

The pollution load index (PLI) estimates the overall degree of soil contamination in the study area based on the total concentration of all metals studied. The following equation was developed by [17] to calculate the pollution load index (PLI):

𝐏𝐋𝐈 = nFC1×FC2×FC3×….×FCn

FCi: metal contamination factor  i.

The metal pollution index provides cumulative information on metal pollution in soils. Thus, we have:

𝐏𝐋𝐈 = 5FCPb×FCCd×FCCu×FCZn×FCCr

For PLI = 0, there is no deterioration;

for 0 <PLI ≤1, only reference levels of pollutants are present;

and a PLI value > 1 indicates progressive soil deterioration.

2.5.4. Determination of the ecological risk factor

The ecological risk factor (Er) is used to assess the ecological risk of a given metal. It also allows for a large-scale estimation of the environmental risk associated with metals and their biological toxicity and is defined by [18] according to the following equation:

Eri = Tri  CiCref

The toxic response factor (Tri) values for TMEs such as lead (Pb), cadmium (Cd), copper (Cu), zinc (Zn) and chromium (Cr) are 5, 30, 5, 1 and 2 respectively, according to [19]. In addition, ecological risk factors have been classified into five categories, as shown in Table 4 [20].

Table 4: Categories of environmental risk factors

| Ecological risk levels: Eri |  |
| --- | --- |
| Eri <40 | low risk, |
| 40 ≤ Eri< 80 | moderate risk, |
| 80 ≤ Eri < 160 | considerable risk |
| 160 ≤ Eri < 320 | high risk |
| Eri ≥ 320 | very high risk |

3. results and discussion

3.1. RESULTS

3.1.1. Some physical and chemical parameters of the soils

Table 5 shows the physical and chemical parameters of the soil samples from the sites studied. It shows their means and standard deviations. The pH values of the soils in Koko (7.17) and Téguéré (7.02) are above 7. Unlike the soils at the Koko and Téguéré market gardening sites, the soil in Natio has a pH of 6.62, which is below 7.

Table 5: Physicochemical parameters of soils

|  | Sites |  |  |
| --- | --- | --- | --- |
| Physicochemical parameters | Koko | Natio | Téguéré |
| pHeau | 7,17 ± 0,27a | 6,62 ± 0,65a | 7,02 ± 0,43a |
| CEC (cmol/kg) | 12,65 ± 1,40a | 10,14 ± 1,76a | 12,40 ± 2,50a |
| TOC (mg/Kg) | 356,58 ± 32,98 b | 263,99 ±16,33c | 456,62 ± 18,92a |

The cation exchange capacity of the soil in Natio is lower than that of the soils in Koko and Téguéré. The CEC values of the soils at the three sites are above 8. The Koko and Téguéré sites are richer in organic matter than the Natio site. The TOC content of the soils varies significantly across the three sites. However, the pH and CEC do not vary significantly from one site to another.

3.1.2. Trace metal content in soil

The averages and standard deviations of trace metal content in soil are presented in Table 6. At the Téguéré site, the average Cu (23.65 mg/kg) and Zn (34.85 mg/kg) content is higher than at the Koko and Natio sites. The highest levels of other trace metals (Pb, Cd and Cr) were found at the Koko site. These values are 6.970, 0.245 and 1.305 mg/kg for Pb, Cd and Cr, respectively.

Pb, Cu and Zn are the TMEs with the highest levels at the three sites. The order of TMEs levels at the three sites is: Zn ˃ Cu ˃ Pb ˃ Cr ˃ Cd.

Furthermore, a comparison of the heavy metal concentrations in the soil at the three sites shows a significant difference (p ˂ 0.05) between these concentrations, except for Cr, whose value does not vary significantly (p ˃ 0.05).

Table 6: Heavy metal concentrations (mg/kg) in soil.

|  | Sites |  |  |  |
| --- | --- | --- | --- | --- |
| TMEs (mg/Kg) | Koko | Natio | Téguéré | UCC |
| Pb | 6,970 ± 1,01a | 6,443 ± 0,548c | 5,650 ± 0,732b | 17 |
| Cd | 0,245 ± 0,008a | 0,235 ± 0,014ab | 0,21 ± 0,030b | 0,102 |
| Cu | 18,15 ± 3,133b | 17,125 ± 0,881c | 23,65 ± 1,47a | 14,3 |
| Zn | 33,24± 0,712a | 27,255 ± 1,168b | 34,85 ± 1,096a | 52 |
| Cr | 1,305 ± 0,164a | 1,27 ± 0,12a | 1,175 ± 0,11a | 35 |

Values followed by different letters in the same row are significantly different at the 5% threshold.

UCC: global pre-industrial values (metal background noise)

3.1.3. Calculation of metal accumulation indices

3.1.3.1. Geoaccumulation index (Igéo)

Figure 2 shows the geoaccumulation indices for all TMEs. The values obtained show that the Koko, Natio and Téguéré sites have negative geoaccumulation indices (Igéo < 0) for Pb, Cu, Zn and Cr. These three sites are therefore not contaminated with Pb, Cu, Zn and Cr.  However, the soils at these three sites are moderately contaminated (0 < Igéo < 1) with Cd.

Figure 2: Soil geoaccumulation indices

3.1.3.2. Contamination factor (CF)

The CF values for TMEs in soils are shown in Figure 3. Soil samples taken at Koko, Natio and Téguéré are slightly contaminated with Pb, Zn and Cr, with CF< 1. However, the soils at these three sites are moderately contaminated (1 ≤CF≤ 3) with Cd and Cu.

Figure 3: Soil contamination factors

3.1.3.3. Pollution load index (PLI)

PLI values range from 0.021 ≤ PLI ≤ 0.030. There is no progressive deterioration of soils due to TMEs (Table 7).

Table 7: Pollution load index values

|  |  | Sites |  |
| --- | --- | --- | --- |
|  | Koko | Natio | Téguéré |
| PLI | 0,030 | 0,021 | 0,024 |

3.1.3.4. Ecological risk factor (Eri)

The soils of Koko, Natio and Téguéré have ecological risk values ranging from 0.067 to 8.279 (Eri < 40) for Pb, Cu, Zn and Cr (Table 5). These TMEs therefore present a low ecological risk in the soils. On the other hand, Cd presents a moderate ecological risk in the soils of these three sites (Table 8) with ecological risk values ranging from 61.765 to 72.060 (40 ≤ Eri < 80).

Table 8: Ecological risk factors

|  |  |  | Sites |  |
| --- | --- | --- | --- | --- |
| Eri |  | Koko | Natio | Téguéré |
|  | Pb | 1,662 | 1,895 | 2,052 |
|  | Cd | 69,118 | 72,060 | 61,765 |
|  | Cu | 8,279 | 5,989 | 6,346 |
|  | Zn | 0,639 | 0,524 | 0,678 |
|  | Cr | 0,075 | 0,074 | 0,067 |

DISCUSSION

Soil analyses from the various sites in Koko and Téguéré show that they are richer in organic matter. Furthermore, the pH values at these sites, which are above 7, show that the soils at these market gardening sites immobilise TMEs more easily. These results are consistent with those of [21], who showed that the accumulation of heavy metals is favoured in soils with a pH > 7. This accumulation of heavy metals could also be attributed to the high organic matter content of these sites. TMEs can be immobilised by soil organic matter. This is the case for Cd and Cr, which accumulate preferentially in horizons rich in organic matter [22]. Similar results were obtained by [23] and [24]. According to these authors, organic matter plays an important role in the formation of metal oxides that retain TMEs in the soil. The CEC values of the three sites, which are above 8, prove that these soils retain TMEs. These results corroborate those of [25].

The Cd concentrations at the Koko, Natio, and Téguéré sites are significantly higher than the pre-industrial value for the upper continental crust. These high concentrations suggest anthropogenic contamination [26]. Furthermore, contamination indices have shown a high anthropogenic contribution to Cd in the soils of the three sites. These sites are both agricultural and urban areas. The heavy metals in the soils of these sites may be of agricultural and urban origin, hence the high Cd contamination. This heavy metal pollution could be due to the various activities carried out around the market gardening sites. Garages and vehicle repair and coating workshops near the Koko and Téguéré sites are sources of heavy metals.  This heavy metal contamination may also be due to local activities such as the common use of herbicides and fertilisers to treat market garden crops.

However, the average concentrations of Pb, Cu, Zn and Cr are lower than global pre-industrial UCC values, indicating that the main source of these heavy metals is natural. Furthermore, the calculation of the pollution load index (PLI) showed that there is no progressive deterioration in soil quality due to heavy metals.

Furthermore, the TMEs content of the three sites shows a significant difference (p ˂ 0.05) from one site to another, except for Cr, whose content does not vary significantly (p ˃ 0.05). The concentration of heavy metals in urban soils is significantly linked to industrial production, vehicle exhaust fumes, solid waste and fossil fuels. Similar work carried out by [27] in market gardening areas located along busy roads showed that the vegetables grown there were polluted by Pb. He noted that the Pb content in the soil and vegetables decreased with distance from the main roads. As air pollution is very intense in market gardening areas, particles from hydrocarbon combustion gases can fall back onto the soil [27].

Furthermore, studies have shown that heavy metals probably have three sources: ‘anthropogenic elements’, ‘natural elements’ and ‘mixed elements’ (natural and anthropogenic) [28-30].

This pollution varies depending on the terrain and the pollutants. The widespread availability of these pollutants, such as TMEs in general and lead in particular, in soil solution poses a real health risk.

4. Conclusion

In this study, the levels of lead, cadmium, copper, zinc and chromium contamination in the market garden soils of Koko, Natio and Téguéré were assessed. The results showed high Cd contamination in the soils of these three sites, indicating contamination from anthropogenic activities. However, the average concentrations of Pb, Cu, Zn, and Cr are below global pre-industrial UCC values, indicating that the main source of these heavy metals is natural. Furthermore, the calculation of the pollution load index (PLI) showed that there is no progressive deterioration in soil quality due to these heavy metals. However, the development of market gardening activities at these sites could have a further impact on soil quality.

Moreover, the study highlighted the presence of heavy metals, including some of the most dangerous (Pb and Cd), in the market gardening soils of Korhogo city. Even heavy metals detected in relatively low concentrations must be taken into account due to chronic toxicity and interaction effects. All these biocontaminants pose certain risks to human health and the quality of natural resources. Consequently, continuous and/or periodic collection of data on soil quality is essential to preserve human health and the ecosystem. For all these reasons, we recommend periodic monitoring of all market gardening sites in the study area in order to protect the environment from the negative effects of TMEs. It would also be advisable to train market gardeners in good farming practices.

Competing interests

Authors have declared that no competing interests exist.

Authors’ Contributions

Author A' designed the study, wrote the protocol, managed the literature searches and wrote the first draft of the manuscript. ‘Authors B’ and ‘Author C’ managed the analyses of the study and performed the statistical analysis. All authors read and approved the final manuscript.

References

[1] 	Waseem, A., Arshad, J. Iqbal, F., Sajjad, A., & Murtaza, G., 2014. Pollution Status of Pakistan: A Retrospective Review on Heavy Metal Contamination of Water, Soil, and Vegetables. BioMed Research International. Volume 2014, 29 pages. https://onlinelibrary.wiley.com/doi/full/10.1155/2014/813206

[2] 	Masciandaro, G., Macci, C., Peruzzi, E, Ceccanti, B., & Doni, S., 2013. Organic matter- microorganismplant in soil bioremediation : a synergic approach, Environnemental Science and Bio- technology, 12 (4), 399 -419. https://link.springer.com/article/10.1007/s11157-013-9313-3

[3]	Bayebila Menanzambi, T., Dufour, P., Pirard, C., Nsangu, J., Mufusama, J. P., Mbinze Kindenge, J.,  & Charlier, C. (2021). Bio-surveillance of environmental pollutants in the population of Kinshasa, Democratic Republic of Congo (DRC): a small pilot study. Archives of Public Health, 79(1), 197.

file:///C:/Users/DELL/Downloads/s13690-021-00717-x.pdf

[4] 	Kouakou, K. J., Bekro, Y-A., Sika, A. E. Denis, B., & Denezon, O. D., 2008. Diagnostic d’une contamination par les éléments traces métalliques de l’épinard (Spinacia Oleracea) cultivé sur des sols maraîchers de la ville d’Abidjan (Côte d’Ivoire) amendés avec de la fiente de Volaille. European Journal of Scientific Research, 21(3), 471-487.

http://psasir.upm.edu.my/id/eprint/7019/1/114.pdf#page=101

[5] 	Yehouenou, P.E.A. Soton, A., Azocli, D. Acakpo, H., Boco, M., Fourn, L., D. Houinsa, D., Keke, J-C, & Fayomi, B., 2010. Contamination du sol, de l’eau et des produits maraîchers par des substances toxiques et des métaux lourds sur le site de Houéyiho (Cotonou) en République du Bénin. International Journal of Biological and Chemical Sciences, 4(6), 2160-2168. https://www.ajol.info/index.php/ijbcs/article/view/64951

[6]        Alain, T. K., Karim, K., dit Corneille, T. M., Moumouni, D., Luc, B. T., Inoussa, Z., & François, Z. (2025). Heavy Metal Pollution of Agricultural Soils and Health Implications at Korsimoro Market Gardening Sites. Current Journal Applied Science and Technology, 44(9), 29-41.

file:///C:/Users/SEKA%20Office/AppData/Local/Microsoft/Windows/INetCache/IE/SRYV3MRQ/Alain4492025CJAST1432261[1].pdf

[7] 	Blais, J.M., Froese, K.L., Kimpe, L.E. Muir, D. C. G., Backus, S., Comba, M., & Schindler, D.W., 2003. Assessment and characterization of polychlorinated biphenyls near a hazardous waste incinerator: Analysis of vegetation, snow and sediments. Environmental Toxicology and Chemistry, 22, 126-133.

https://academic.oup.com/etc/article-abstract/22/1/126/7769757

[8] 	Sakai, S.T., Yamamoto, Y., & Giraud, R., 2006. Formation and control of toxic polychlorinated compound during incineration of wastes containing polychlorinated naphthalenes. Environmental Science and Technology, 40, 2247-2253.

https://pubs.acs.org/doi/abs/10.1021/es052156a

[9]	Mpanga, I. K., Adjei, E., Dapaah, H. K., & Santo, K. G. (2021). Poultry manure induced garden eggs yield and soil fertility in tropical and semi-arid sandy-loam soils. Nitrogen, 2(3), 321-331.

https://www.mdpi.com/2504-3129/2/3/22

[10] 	Courade, G., 2000. La sécurité alimentaire en Afrique à l'aube du xxe siècle. Actes du colloque international Eau-santé-Ouaga, impact sanitaire et nutritionnel des hydro-aménagements en Afrique. Ouagadougou (Burkina Faso), 21-24 novembre 2000, 2 (3), 1-8. https://www.documentation.ird.fr/hor/fdi:010025208

[11] 	Kanda, M., Wala, K., Djaneye-Boundjou, G., Ahanchede, A., & Akpagana, K., 2006.-Utilisation des pesticides dans les périmètres maraîchers du cordon littoral togolais. J. Rech. Sci. Univ. Lomé (Togo), série A, 8 (1), 19-26.

https://www.ajol.info/index.php/jrsul/article/view/51957

[12]       Silué, S., Kouassi, A. A., N'datchôh, E. T., & Yoboué, V., 2021.  Meteorological factors contribution over two urban sites in Côte. Sciences des structures et de la matière. 3,  12-39.

https://www.researchgate.net/profile/Ndatchoh-Toure/publication/352131041_Annual_variability_of_aerosol_optical_thickness_and_analysis_of_meteorological_factors_contribution_over_two_urban_sites_in_Cote_d'Ivoire_Abidjan_and_Korhogo/links/60ba33e4a6fdcc22ead916af/Annual-variability-of-aerosol-optical-thickness-and-analysis-of-meteorological-factors-contribution-over-two-urban-sites-in-Cote-dIvoire-Abidjan-and-Korhogo.pdf

[13] 	Mpundu, M., 2013. Teneurs en éléments traces métalliques dans les sols de différents jardins potagers de la ville minière de Lubumbashi et risques de contamination des cultures potagères. Journal of Applied Biosciences, 65, 4957- 4968.      https://www.m.elewa.org/JABS/2013/65/6.pdf

[14]	Al-Sabbagh, T. A., & Shreaz, S. (2025). Impact of lead pollution from vehicular traffic on highway-side grazing areas: challenges and mitigation policies. International Journal of Environmental Research and Public Health, 22(2), 311.      https://www.mdpi.com/1660-4601/22/2/311

[15] 	Ouattara, A. A., Sangare, N., Yao, K. M., Trokourey, A., & Diaco, T. (2021). Evaluation de la contamination des éléments traces métalliques dans les sédiments de la rivière N’zi, Côte d’Ivoire: Assessment of trace metals contamination in sediments of N’zi River, Côte d’Ivoire. International Journal of Biological and Chemical Sciences, 15(5), 2199-2208.

file:///C:/Users/DELL/Downloads/ajol-file-journals_314_articles_220370_submission_proof_220370-3745-540398-1-10-20220124%20(2).pdf

[16]	Müller, G., 1981. Die Schwermetallbelastung der sediment des Neckars und seiner Nebenflusse: eine Bestandsaufnahme. Chemical Zeitung, 105, 157-164.

https://www.scirp.org/reference/ReferencesPapers?ReferenceID=1552622

[17]	Tomlinson, D.L., Wilson, J. G., Harriis, C.R., & Jeffry, D.W., 1980. Problem in assessment of heavy levels in estuaries and the formation of pollution index. Helgoländer Meeresuntersuchungen, 33(1-4), 566-575.

https://link.springer.com/article/10.1007/BF02414780

[18]	Hakanson, L., 1980. An ecological risk index for aquatic pollution control. A sedimentological approach. Water Research, 14, 975–1001.            https://doi.org/10.1016/0043-1354(80)90143-8

[19]	 Rehman, I., Ishaq, M., Ali, L., Khan, S. I., Ahmad, & Din, U.I., 2018. Enrichment, spatial distribution of potential ecological and human health risk assessment via toxic metals in soil and surface water ingestion in the vicinity of Sewakht mines, district Chitral, Northern Pakistan. Ecotoxicology and Environmental Safety, 154, 127–136.

https://www.sciencedirect.com/science/article/abs/pii/S0147651318301180

[20] 	Baah, D.S., Gikunoo, E., Arthur, E.K., Agyemang, F.O., Foli, G., & Koomson, B., 2023. Anthropogenic Sources and Risk Assessment of Heavy Metals in Mine Soils: A Case Study of Bontesso in Amansie West District of Ghana. Journal of Chemistry, 2023.

https://onlinelibrary.wiley.com/doi/full/10.1155/2023/6760154

[21]	Bravo, S., Amorós, J. A., Pérez-De-Los-Reyes, C., García, F. J., Moreno, M. M., Sánchez-Ormeño, M., & Higueras, P. (2017). Influence of the soil pH in the uptake and bioaccumulation of heavy metals (Fe, Zn, Cu, Pb and Mn) and other elements (Ca, K, Al, Sr and Ba) in vine leaves, Castilla-La Mancha (Spain). Journal of Geochemical Exploration, 174, 79-83.

https://sci-hub.se/https://doi.org/10.1016/j.gexplo.2015.12.012

[22] 	Feszterová, M.; Kowalska, M.; Hudec, M. Assessing the Impact of Soil Humic Substances, Textural Fractions on the Sorption of Heavy Metals (Cd, Pb). Appl. Sci. 2024, 14, 2806. https://doi.org/10.3390/app14072806

file:///C:/Users/SEKA%20Office/Downloads/applsci-14-02806.pdf

[23] 	Violante, A., Krishnamurti, G.R.S., & Huang, P.M., 2002. Impact of organic substances on the formation and transformation of metal oxides in soil environments. Interactions Between Soil Particles and Microorganism : Impact on the Terrestrial Ecosystem, 8, 133-188. http://cabidigitallibrary.org/doi/full/10.5555/20023032512

[24] 	Hang, P.M., 2004. Soil mineral–organic matter–microorganism interactions : fundamentals and impacts. Advances in Agronomy, 82, 391-472. https://www.researchgate.net/publication/222363859_Soil_Mineral-Organic_Matter--Microorganism_Interactions_Fundamentals_and_Impacts

[25] 	Ondo, J.A., 2011. Vulnérabilité des sols maraîchers du Gabon (région de Libreville) : acidification et mobilité des éléments métalliques. Mémoire de Thèse de doctorat de l’Université de Provence, France, 253 p.

file:///C:/Users/Editor-05/Downloads/111129_Ondo_02TQAH024S8_TH.pdf

[26]	Paramasivam, K., Ramasamy, V., & Suresh, G., 2015. Impact of sediment characteristics on the heavy metal concentration and their ecological risk level of surface sediments of Vaigairiver, Tamilnadu, India. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 137, 397-407.

DOI: http://dx.doi.org/10.1016/j.saa.2014.08.0 56.

https://www.sciencedirect.com/science/article/abs/pii/S1386142514012475

[27]     Boahen, E. (2024). Heavy metal contamination in urban roadside vegetables: origins, exposure pathways, and healthimplications. Discover Environment, 2(1), 145.

file:///C:/Users/DELL/Downloads/s44274-024-00182-7.pdf

[28] 	Facchinelli, A., Saechi, E., & Mallen, L., 2001. Multivariate statistical and GIS-based approach to identify heavy metal sources in soils. Environmental Pollution, 114, 313–324. https://www.sciencedirect.com/science/article/abs/pii/S0269749100002438

[29] 	Imperato, M., Adamo, P., Naimo, D., Arienzo, M., Stanzione, P. & Violante, P., 2003. Spatial distribution of heavy metals in urban soils of Naples city (Italy). Environmental Pollution, 124, 247-256. https://www.sciencedirect.com/science/article/abs/pii/S0269749102004785

[30] 	Afshin, Q., Farid,M., & Giti, F., 2009. Characterization of metal pollution in soils under two land use patterns in the Angouran region, NW Iran; a study based on multivariate data analysis. Journal of Hazardous Materials, 172, 374-384. https://www.sciencedirect.com/science/article/abs/pii/S0304389409011339