Characteristics of convective drying of white-skinned sweet potato spheres (Ipomoea batatas [L.] Lam)

Abstract

White-skinned sweet potatoes are a highly sought-after food product in Burkina Faso. However, there are times when this product is very scarce or even unavailable in the country. It is therefore necessary to find a way to preserve it. Despite the drying method already used for preservation, we encounter dried products of poor quality that do not last long. It is in this context that our study aims to understand and optimise the best method for preserving white-skinned sweet potatoes through drying. To carry out this study, we first cut them into spherical shapes with diameters of 2 cm and 3 cm, which we then placed in an oven at fixed temperatures of 80°C, 70°C, 60°C, and 50°C. The behaviour of each size was studied for 6, 8, 10 and 12 hours respectively at different temperatures. The results showed that for a residual water content of approximately 5%, the drying time for potatoes with a diameter of 2 cm was 500, 800, 1040 and 1900 minutes, while potatoes with a diameter of 3 cm took 780, 890, 940 and 2400 minutes respectively, depending on the temperatures studied. These results allowed us to conclude that the drying time is proportional to the shape of the white-skinned sweet potato. Added to this is the evolution of the water velocity of the two different diameters. The results show that the 2 cm potato dries faster than the 3 cm potato.

Keywords: drying, spherical shape, water content, drying speed, sweet potato

Introduction

“Food security is one of the major global challenges facing the world today. One of the factors contributing to food insecurity is the loss of food and agricultural products due to spoilage throughout the food and agricultural chain and/or during the post-production period. A key method of enhancing food security is to reduce losses due to the deterioration of these agricultural products after harvest. To improve shelf life and reduce the deterioration of agricultural products, drying is the most used method” [1,20]. Drying is a unit operation aimed at removing water from a product and thereby reducing its water activity. Convection drying is the process of removing water with air through simultaneous transfer of heat, mass, and momentum.

Drying food products has many advantages, such as inhibiting the growth of microorganisms and deterioration reactions by reducing water activity, as well as reducing transport and storage costs due to the decrease in product weight and volume. Several studies have been conducted in the scientific community on various products to improve their preservation. In 2018, Castro and his colleagues studied fruit drying [2], while Compaoré studied onion drying [3]. With the aim of reducing food self-sufficiency, potatoes were dried by Mukhopadhyay 2021 [4]. The complexity of the structure and composition of moist foods, the variety of transport phenomena and biological diversity make food drying a challenge. It is in this context that we conducted our study on the drying of white-skinned sweet potatoes.

“The sweet potato (Ipomoea batatas) is a tuber crop with immense potential because it produces more edible energy on marginal land than any other major food crop. In addition to this useful property, it can withstand undesirable abiotic and biotic stresses and does not require intensive care. It therefore plays an important role in the economy of poor households, where it is a major source of subsistence and is considered a “famine relief crop”. In addition to this important function, its cultivation has immense industrial value for starch extraction and animal feed production. For all these reasons, sweet potatoes offer great potential for improving food and nutritional security in developing and underdeveloped countries. Most farms belong to vulnerable population groups” [4,20]. Sweet potatoes offer enormous benefits. They are rich in tubers, fibre, manganese, starch, vitamins, minerals, and bioactive compounds [5]. They contain essential carotenoid, phytochemical, anti-cancer, and antimicrobial properties that are beneficial to human health. Raw or processed sweet potatoes can be consumed by humans as a staple food, snack or baked good.

“Sweet potatoes are susceptible to microbial activity, which can cause rapid degradation and deterioration due to their high moisture content. In addition, sweet potatoes are seasonal and difficult to maintain at optimal quality for long periods after harvest. As a result, they are often used shortly after harvest or preserved using hot air convection drying methods” [6]. Panigrahi et al [19], “dried two varieties of protein-rich sweet potatoes (the white-fleshed Bosbok variety and the orange-fleshed Carmel variety) at air temperatures of 40°C, 60°C and 80°C to examine the effects on their nutritional value”. “The sweet potato variety and air temperature were important factors influencing the nutritional value of sweet potato tubers” [7].

The temperature of the drying air has a significant impact on the drying speed. This is due to the heat transferred to the product, which increases with the air temperature. It is also due to the temperature of the product, which is higher when the air temperature is higher. As a result, the conductivity of the water in the product becomes significant. In addition, the composition and structure of the material (soft potato) have been studied [5]. The effect of drying air temperature has been studied by many researchers, for example, the components of okra, potato [9], and green beans [10]. In addition to the various vegetables described by Doymaz Ibrahim [11], there is also the phenotypic diversity of taro [12] and the local variety (Tabouchi) of taro [13]. The change in the physical state of the potato depending on temperature was examined by Ouoba et al [14] and the variety of the exchange surface and the quality of the potato by Sanchez et al. [15]. The influence of size and temperature on cubic and parallelepiped samples showed that drying time decreased with increasing drying temperature [8]. However, there is little information on the drying process of pale-fleshed sweet potatoes in the literature.

Due to the need to increase the use of sweet potato flour, reduce annual post-harvest losses and ensure product availability, it is necessary to study the convective drying of pale-fleshed, white-skinned sweet potatoes. The main objective of this study is to determine the kinetics of air drying of spherical shapes as a function of drying conditions.

Materials and methods

Materials

Sweet potatoes (Ipomoea batatas [L.] Lam) were used as drying material in this study. Samples of the local variety of sweet potato with pale flesh and white skin, widely consumed in low-income households, were purchased during July 2024 at the fruit and vegetable market in the city of Bobo Dioulasso (Coordinates: 11° 11′ 00″ North, 4° 17′ 00″ West), located in the Upper Basin region of Burkina Faso.

Methods

The sweet potato samples were transported and stored under refrigerated conditions (4 ± 0.5 °C) prior to the drying process at the GERME & TI (Groupe D’étude et de Recherche en Mécanique Énergétique et Techniques Industrielles) laboratory at Nazi Boni University. Before drying, the sweet potato samples were placed in the laboratory to reach room temperature (25 ± 1 °C). Sweet potato samples were selected, washed, peeled, and cut into spheres with diameters ranging from 2 ± 1% cm to 3 ± 1% cm, measured manually using a digital caliper. The spherical samples were immersed in distilled water to remove excess surface starch film.

“Excess water on the spherical samples was removed using blotting paper, and these sweet potato spheres were arranged in a single layer on a drying tray. The initial dry basis (db) moisture content of the sweet potato was determined using the convective oven method at 105 ± 5 °C for 24 hours” [17]. Triplicate samples were used to determine the moisture content, and the average values were 2.259 kg of water/kg of dry matter. The choice of material (sweet potato) is due to the homogeneity of its macrostructure, and we consider that diffusion takes place over the entire exchange surface [9].

Drying procedure

The samples obtained are placed in an oven set at the appropriate temperature for drying. We used an oven (AIR Concept, temperature ranging from 40°C to 250°C). The experiments were conducted at temperatures of 50, 60, 70 and 80°C, and the differential mass loss of the samples was measured by weighing them every twenty minutes using a digital balance (MH-999 Ming Heng Electronic). The weighing process is very quick (20 seconds), which means that variations in relative humidity can be ignored. The dry mass of each sample was obtained after a minimum of 24 hours, depending on the drying temperature used.

Establishing drying curves

The amount of water present in the material is quantified by a quantity called the water content. For the study of drying, we define:

The initial water content X0 of each product is determined by relation (1) [18]:

X0=mems=m0-msms                                                                                                      (1)

with me (kg): the initial mass of water contained in the product; m0 (kg): the initial mass of the sample; ms (kg): the mass of the dried sample.

Wet basis water content: this is calculated in relation to the total mass of the material (solid + initial water) and is expressed using the following equation:

X'=m(t)-msmi                                                                                                               ( 2)

where X′ is the water content in the wet basis, m(t) is the mass of the sample at time t, m_i is the initial mass of the sample, and m_s is the final mass (mass of the solid matrix) of the sample, i.e. after drying in an oven at between 105 and 110°C until a constant mass is reached.

Dry basis water content: this is calculated in relation to the final mass of solid material. It is obtained by dividing the mass of water present in the material by the mass of the completely dry sample:

X=mt-msms                                                                                                                    ( 3)

X the water content on a dry basis,

The normalised or reduced water content: This is obtained by calculating the ratio between the water content of the product at time t and its initial water content:

XtX0                                                                                                                                                    ( 4)

Reduced speed:

We associate each time value with the corresponding water content value. The relationship X(t)→t allows us to obtain the curves  -dXdt→XX0

The speed is then normalised by its initial value to obtain the reduced speed curves as a function of X/X₀, according to relation (5)

vt=v(t)v0(t=0)=dX(t)dtdXdt(t=0)                                                                               (5)

Relationships (4) and (5) facilitate comparison between products with different initial water content and drying rates.

RESULTS AND DISCUSSION

Characterisation of water content

Figure 1 shows the water content results for spherical samples with a diameter of d=2±1cm. The curves X(t)/X0 of reduced water content for spherical samples with a diameter of 2cm show the same trend and decrease in the same way. An increase in temperature

Drying reduces the water content of the product and shortens the drying time. Furthermore, for a drying time (t=400 min), the samples are at of their initial water content at 80°C, 70°C, 60°C and 50°C respectively.

In fact, the time taken to reduce the water content from 1 (KgeKgms-1)  to a final water content of 0.1 (KgeKgms-1) is 500 min, 800 min, 1040 min and 1900 min at temperatures of 80°C, 70°C, 60°C and 50°C, respectively.

Figure 1: Water content evolution curves for spherical samples with a diameter of 2 cm as a function of drying time.

Figure 2 shows a time profile of water content as a function of temperature for spherical sweet potato samples with a diameter of d = 3 cm. It can be seen that the curves are almost identical for the first fifty minutes. The curves show the same trend during drying. An increase in temperature reduces the drying time. We can see that after the first 40 minutes; the samples go from 100% of their water content to 72% of its initial value. However, after 40 minutes, between 360 and 400 minutes, the sample goes from 7% to 2% of its initial water content at 80°C, i.e. a loss of 5% in 40 minutes, compared to a loss of 28% in the first 40 minutes.

The results obtained show that for a residual water content of 0.1 (K_ge 〖Kgms〗^(-1)), the drying time is 780, 890, 940 and 2400 minutes at 80°C, 70°C, 60°C and 50°C, respectively. This allows us to say that the drying time is an increasing function of temperature.

Figure 2: Curves showing changes in water content of spherical samples with a diameter of 3 cm as a function of drying time.

From the above, size or thickness can be affected by drying conditions, as shown by the X(t)/X0 curves of reduced water content in spherical samples of sweet potato with diameters of 2 cm and 3 cm as a function of time (t). Furthermore, drying time is an increasing function of temperature.

Characterisation of drying speed

Figure 3: Time profiles of the drying rate of spherical samples with a diameter of d=2 cm (a) as a function of temperature.

At the start of the experiment, the curves are indistinguishable and evolve in the same way (Figure 3). The duration of a phase depends on the temperature. We observe durations of 30 minutes, 45 minutes, 60 minutes, and 80 minutes at 80°C, 70°C, 60°C and 50°C respectively. This slowdown in speed at the end of drying compared to the start of drying can be seen in Figure 3. We can see that the speed suddenly increases from 0 to 0.016 at the 60th minute and varies only slightly from 0.0007 to 0.0004 kg/kg/s from 340 minutes to 400 minutes of drying at 80°C.

Figure 4: Time profiles of drying speed for spherical samples with diameter d=3cm (b) as a function of temperature.

Figure 4 shows the time profile of the drying kinetics of spherical samples with a diameter of d=3cm at different temperatures. The curves converge after the first 20 minutes of drying. In this section, two phases can be observed: the drying phase at increasing speed and the phase at decreasing speed. The latter phase occurs after a drying time of 60 minutes, 80 minutes, 100 minutes, and 100 minutes at temperatures of 80°C, 70°C, 60°C and 50°C, respectively.

This result can be attributed to the phenomenon of the state of water, which is free at the beginning of drying and highly bound at the end of drying. The size of the samples and the drying conditions play a role in the transfer process during convective drying of sweet potatoes.

Conclusion

In this section, we conducted an experimental study on the drying of pale-fleshed sweet potatoes. The transfer process during drying due to the activation of water in the product is visualised through the drying curves. The results showed that the samples went from 100% water content to 72% of their initial value after the first 40 minutes. However, at the end of drying, between 360 min and 400 min, the sample went from 7% to 2% of its initial water content at a fixed temperature of 80°C. It appears that the sample lost 5% of its initial water content in the last 40 minutes, compared to a loss of 28% in the first 40 minutes.

This result is also confirmed by the drying speeds. The speed suddenly increases from 0 to 0.16 at the 60th minute and varies from 0.0007 to 0.0004 kg/kg/s respectively after 340 min to 400 min of drying at a fixed temperature of 80°C.

Furthermore, size or thickness can be affected by drying conditions such as temperature, air intensity, and humidity.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

References:

[1]	D. I. Onwude, Norhashila Hashim, Khalina Abdan, Rimfiel Janius, et Guangnan Chen., " Combination of computer vision and backscattering imaging for predicting the moisture content and colour changes of sweet potato (Ipomoea batatas L.) during drying " Computers and Electronics in Agriculture, vol. 150, pp. 178-87, 2018.

[2]	A. M. Castro, E. Y. Mayorga, et F. L. Moreno "Mathematical modelling of convective drying of fruits: A review " Journal of Food Engineering, vol. 223, pp. 152-67, 2018.

[3]	A. Compaoré, Aditya Putranto, A. O. Dissa, Samuel Ouoba, Romain Rémond, Yann Rogaume, André Zoulalian, Antoine Béré, et Jean Koulidiati. , " Convective Drying of Onion: Modeling of Drying Kinetics Parameters," Journal of Food Science and Technology, vol. 56, no. 7, pp. 3347-54, 2019.

[4]	S. K. Mukhopadhyay, Arup Chattopadhyay, Ivi Chakraborty, et Indrabrata Bhattacharya., "Crops That Feed the World 5. Sweetpotato. Sweetpotatoes for Income and Food Security " Food Security, vol. 3, no. 3, pp. 283-305, 2011.

[5]	F. C. Amagloh, Benard Yada, Gaston A. Tumuhimbise, Francis K. Amagloh, et Archileo N. Kaaya.  , "The Potential of Sweetpotato as a Functional Food in Sub-Saharan Africa and Its Implications for Health: A Review,". Molecules vol. 26, no. 10, p. 2971, 2021.

[6]	T. B. Ayeleso, Khosi Ramachela, et Emmanuel Mukwevho. 2016. , " A Review of Therapeutic Potentials of Sweet Potato: Pharmacological Activities and Influence of the Cultivar," Tropical Journal of Pharmaceutical Research, vol. 15, no. 12, pp. 2751-61, 2016.

[7]	S. Panigrahi, E. B. Oguntona, et B. R. Roberts. , "Effects of oven‐drying tubers of two high‐protein sweet potato varieties at different temperatures on their feeding value in broilers " British Poultry Science, vol. 37(1), pp. 173-188, 1996.

[8]	G. Abdou-Salam, Honore, O. K., & François, Z. , "Taking into Account the Complex Nature and the Intrinsic Parameters of Agro-Food," Journal of Biophysical Chemistry,, vol. 11, no. 01, 2020, Art. no. (1).

[9]	OUOBA.K.Hononé., "sechage des produits agroalimentaires influence de la taille,de la forme et de la découpe.," doctora Sciences Appliquée:Genie des procédés, Transferts des Milieux Poreux et Complexes, , Laboratoire des matériaux et Environnement LA.ME université de Ouagadougou, 07/02/2013.

[10]	s. l. Doymaz I., "Drying Characteristics and Kinetics of okra. [s.l.]," Journal of Food Engeneering, 2005b.

[11]	İ. b. Doymaz, Cüneyt Tunçkal, et Zekiye Göksel, " Comparison of Drying Kinetics, Energy Efficiency and Color of Dried Eggplant Slices with Two Different Configurations of a Heat Pump Dryer " Energy Sources, Part A: Recovery, Utilization, and Environmental Effects and Environmental Effects vol. 45, no. 1, pp. 690-707, 2023.

[12]	Koffi Kouamé Kevin et Koffi Niambet Jean-Michel, "Diversité phénotypique du « taro » cultivé et/ou spontané consommé en Côte d’Ivoire," Journal of Applied Biosciences, vol. 163, pp. 16862 – 16871, 2021.

[13]	S. S. Donwière Some, Marie Claire Cece, and Renan Ernest Traore: , "Réponse d’une variété locale (Tabouchi) de taro (Colocasia esculenta L. schott) à une phytohormone (6 benzylaminopurine) en culture in vitro. ," International Journal of Innovation and Applied Studies de Février 2024

[14]	A.-S. G. Kondia Honoré Ouoba, Désiré Bama, Abdoul Salam Ibrango, Salifou Ouedraogo, "Physical Transformations of Agri-Food Products during Their Convective Drying: Characterization of the Contraction and Isotropicity of Okra," Journal of Minerals and Materials Characterization and Engineering, vol. 11, pp. 249-259, 2023.

[15]	P. D. C. Sanchez, Norhashila Hashim, Rosnah Shamsudin, et Mohd Zuhair Mohd Nor. , "Quality evaluation of sweet potatoes (Ipomoea batatas L.) of different varieties using laser light backscattering imaging technique," Scientia Horticulturae vol. 260, no. 108861, 2020.

[16]	S. K. Mukhopadhyay, Arup Chattopadhyay, Ivi Chakraborty, et Indrabrata Bhattacharya., " Crops That Feed the World 5. Sweetpotato. Sweetpotatoes for Income and Food Security," Food Security vol. 3 no. (3), p. 283‑305, 2021.

[17]	A. Compaoré, Aditya Putranto, A. O. Dissa, Samuel Ouoba, Romain Rémond, Yann Rogaume, André Zoulalian, Antoine Béré, et Jean Koulidiati. , "Convective Drying of Onion: Modeling of Drying Kinetics Parameters," Journal of Food Science and Technology vol. 56, no. 7, pp. 3347-54, 2019.

[18]	K. H. Ouoba, Abdou-Salam Ganame, et François Zougmore.  , "Research of the Characteristic Dimension of the Transfers during the Convective Drying of the Sweet Potato for the Parallelepipedic and Spherical Shapes " Advances in Materials Physics and Chemistry, vol. 11, no. 12, pp. 267-76, 2021.

[19]	S. Panigrahi, E. B. Oguntona, et B. R. Roberts, « Effects of oven‐drying tubers of two high‐protein sweet potato varieties at different temperatures on their feeding value in broilers », British Poultry Science, vol. 37, no 1, p. 173‑188, mars 1996, doi: 10.1080/00071669608417846.

20.  Compaore, Aboubakar & Ouedraogo, Salifou & Ouoba, Honoré & Ouedraogo, Emmanuel & Gasa, Siyabonga & Ilboudo, Fayçal & Naon, Bétaboalé. (2025). Mathematical Modelling of Thin-Layer Drying of Pale-Fleshed, White-Skinned Spherical Sweet Potato (Ipomoea Batatas). IRA-International Journal of Applied Sciences (ISSN 2455-4499). 20. 39. 10.21013/jas.v20.n3.p1.