Original Research Article

Comparative study of anthropogenic activities influences on water qualities of Boundiali, Korhogo and Ouangolodougou dams

(Cote d'Ivoire)

ABSTRACT: Aims: This study was conducted in the context of water resource security. The main objective was to highlight human activities that could have a significant impact on dams water quality of the dams of Boundiali, Korhogo, and Ouangolodougou. Study design: Water samples were analysed. Then, water quality and pollution levels have been evaluated. Finally, waters pollution origins were highlighted. Place and Duration of Study: The First sampling campaign was carried out during the rainy season from August to September 2021, and the second in the dry season from December 2021 to January 2022, at the Boundiali, Korhogo, and Ouangolodougou dams. Methodology: The adopted methodology consisted of water characterisation in these infrastructures and their compliance comparison with the WHO drinking water supply standards. Waters characterization concerns the following parameters: pH, temperature, electrical conductivity and dissolved oxygen, chemical oxygen demand (COD), biological oxygen demand (BOD5), nitrate (NO₃-), nitrite (NO₂-), ammonium (NH4+), iron (Fe), manganese (Mn), total phosphate (POᵌ-), lead (Pb), arsenic (As), cyanide (CN), mercury (Hg), organohalogen (POH) and organophosphorus (POP) pesticides and carbamates (CAR). Subsequently, quality and pollution indices were determined in order to provide an overview of the quality and pollution levels of these resources. Then, surveys were carried out with a view to identifying sites likely to be responsible for pollution. Finally, similarities and divergences in human activities that are the source of this water quality degradation were noted. Results: Characterization results show that these waters have relatively neutral pH values with relatively low electrical conductivities. Metals (iron and arsenic), nitrate, phosphorus and pesticides (POPs and POHs) have concentrations above the WHO drinking water standard. Quality and organic pollution indices indicate that these dams’ waters have very poor quality in rainy periods and are not drinkable in dry periods with moderate pollution. WQIs results are considered to be high, with 477.620 for Boundiali, 130 for Korhogo and 147.66 for Ouangolodougou in dry season, while WQIs were 83.467 in Boundiali dam, 190.6404 in Korhogo and 94.81 in Ouangolodougou in dry season. In Boundiali, OPI has a high value of 3.667 in the rainy season and 2.667 in the dry season. At Korhogo, the OPI level has a constant value of 2.33 in both seasons. In Ouangolodougou, IPO is 3 in rainy season and 2,667 in the dry season. Field surveys confirmed that the quality of these resources is influenced by human activities, particularly agricultural practices in the immediate perimeters, which are the main pollution factors in Boundiali and Ouangolodougou, unlike Korhogo, where the main sources of pollution are non-seasonal activities such as sewage from riparian houses, livestock farming, artisanal activities and quarries. Conclusion: Waters from these three dams have different pollution sources such as natural incomes, but anthropogenic activities remain the main pollution source.

Keywords: Anthropogenic Activities, dam, pollution, quality, Korhogo

1. INTRODUCTION

Recognised by the United Nations, access to drinking water and sanitation is a fundamental issue for sustainable development (Nations Unies, 2015). The goal is to ensure universal and equitable access to safe and affordable drinking water for all by 2030. The right to safe drinking water is essential to life, full enjoyment, as it has a direct effect on public health, food security, livelihoods and economic development (UN-Water, 2023). In Côte d'Ivoire, remarkable progress has been made over the past two decades. According to the latest WHO/UNICEF Joint Program (2024) reports, population proportion with access to safe drinking water increased from 46% in 1998 to 82% in 2021, with significant disparities between urban and rural areas. This significant improvement is part of government efforts to achieve national targets of National Development Plan (NDP 2021-2025), which aims to achieve drinking water coverage of 95% in urban areas and 80% in rural areas by 2025 (Ministry of Water and Forests, 2023). The Government of Cote d'Ivoire has implemented many water infrastructures to achieve these ambitious objectives, particularly in the northern regions (World Bank, 2023), through Project to Strengthen the Supply of Drinking Water in Urban Areas (PREMU). These regions depend largely on dams for water supply since characterised by a Sudano-Sahelian climate with a long dry season from November to April. These water infrastructures are essential for the water security of populations, agricultural development and aquatic ecosystems maintenance (Yao et al., 2024). However, their durability is threatened by multiple pressure factors. Rapid expansion of agricultural land, unplanned increasing urbanization, accelerated deforestation, and untreated domestic and industrial discharges are all threats to surface water quality (Yao et al., 2025). Deforestation accelerated by agricultural land expansion, rapid urban development, and untreated domestic and industrial effluents are serious threats for surface water quality (Yao et al., 2025). The intensification of agricultural activities in dams' watersheds in Côte d'Ivoire shown by Akatchi et al. (2024), leads to a significant increase in nitrate, phosphorus and pesticide contents in water resources. This seriously affects resources quality when combined with natural processes of water erosion, evaporation and runoff (Soro et al., 2017). The problem of the degradation of the quality of the dam water thus has a health, economic and environmental dimension. Waterborne diseases are still a major morbidity cause in the country northern region (Ministry of Health, 2024), in terms of health. Water quality deterioration results in additional treatment costs, which ultimately reflect in water prices for consumers (SODECI, 2022), economically. Ecosystem resilience are threatened by water bodies eutrophication and aquatic biodiversity loss (Koffi et al., 2021). Ecosystem resilience is threatened by water bodies' eutrophication and aquatic biodiversity loss (Koffi et al., 2021). A comparative and integrated approach is needed, faced with these serious problems, to better understand anthropogenic pressures on dam water resources. Sustainable dam management strategies must be based on accurate diagnoses of pollution sources and their differential impact depending on local contexts, as recently related by Varis et al. (2008). The present study has for its principal objective to assess and compare the anthropogenic activities on the water quality of Boundiali, Korhogo and Ouangolodougou dams in northern Côte d'Ivoire. Specific objectives are: i) Assess physico-chemical and bacteriological quality and pollution levels of these various dams through seasonal sampling campaigns; ii) Identify and characterise anthropogenic activities responsible for possible water quality degradation, using survey and direct observation methods; iii) Analyse similarities and differences in human pressure effects on these aquatic ecosystems.

2. Materials and methods

2.1 Material

Presentation of the dam’s geographical situation

The present study covers three cities in the Savannah District in northern Côte d'Ivoire: Boundiali (9.5217° N, 6.4869° W), Korhogo (9.4580° N, 5.6296° W) and Ouangolodougou (9.9684° N, 5.1488° W). This border region with Mali and Burkina Faso covers 40,323 km² and has 901,301 inhabitants (GPHC, 2021). Korhogo, the district's capital, covers 1,196.21 km² and encompasses 61 villages (Figure 1). The region is characterised by a Sudano-Sahelian climate marked by a long dry season, making dams essential for water supply. The three study sites have complementary profiles: Boundiali: important agricultural zone, Korhogo: major urban and economic hub, Ouangolodougou: border town with cross-border dynamics. This selection allows a comparative analysis of anthropogenic impacts on water resources in various socio-economic contexts within the same geographical region

Figure 1: Presentation of the study area (yellow)

Data acquisition and analysis equipment

The equipment used for field data collection includes:

- 1.5-litre plastic bottles, previously rinsed with distilled water, for collecting water samples

- Storage cooler maintained at 4°C with cold storage for sample preservation

- GARMIN Etrex 10 GPS for precise location of sampling points and peripheral anthropogenic activities

- Structured questionnaires to collect household socio-economic data

- Observation grids to inventory potentially polluting activities around dams

The material used for data analysis:

- a multi-parameter that served as a tool for in situ measurement of physicochemical parameters (pH, Temperature, Dissolved oxygen, Conductivity, Suspended matter, Turbidity)

- HACH/DR 1900 spectrophotometer for laboratory analysis;

- Glassware for laboratory analysis;

2.2 Methods

2.2.1 Assessment of the dam's water quality and pollution levels

2.2.1.1 Sampling

Water samples were taken in uncontaminated plastic bottles with a capacity of 1.5 litres. The first sampling campaign was carried out during the rainy season from August to September 2021, and second, in the dry season from December 2021 to January 2022. In each seasonal campaign 12 samples were taken according to, 3 samples (at variable depths of dams (surface, medium and depth)), at 4 different points on each dam (at least about 2 m from banks so as to avoid edge effects as much as possible). Samples thus collected were subsequently stored in a cooler containing a cold storage tank at 4° C before being transported to the laboratory within hours of collection for chemical analyses.

2.2.1.2 Physico-chemical analyses

Several parameters such as pH, temperature, electrical conductivity and dissolved oxygen were measured in situ using a multi-parameter. Analyses of other parameters were carried out in the SODECI laboratory by using a laboratory spectrophotometer of the HACH/DR 1900 type. These include chemical oxygen demand (COD), biological oxygen demand (BOD5), nitrate (NO3−), nitrite (NO2−), ammonium (NH4+), metals such as dissolved iron (Fe), manganese (Mn), total phosphate (PO43−), lead (Pb), arsenic (As), cyanide (CN), mercury (Hg), organohalogen (POH) and organophosphorus (POP) pesticides and carbamates (CAR). Analyses have been done according to specific methods of analysis (Table 1).

Table 1: Parameter analysis methods

| Parameters | Units | Methods |
| --- | --- | --- |
| pH |  | ISO 10523 V 2008 |
| Temperature | °C | Pt 100 |
| Conductivity | µS/Cm | ISO 7888 V 1985 |
| Dissolved Oxygen | %/mg/L | ISO 5814 V 2012 |
| COD | mgO2/L | Tubes fermés |
| BOD5 | mgO2/L | Respirometric |
| Nitrate | mgNO3- | ISO 7890-3 V 1988 |
| Nitrite | mgNO2- | Diazotation |
| Cyanides | mg/L | Pyrazolone-pyrine |
| Iron | mg/L | AAS ContrAA 700 (Flamme) |
| Manganese | mg/L | ISO 15586 V 2003 |
| Lead | mg/L | ISO 8288 V 1986 |
| Arsenic | µg/L | ISO 17378-2 V2014 |
| Mercury | µg/L | ISO 12846 V 2012 |
| Ammonium | mgNH4+ | NF T90-015-2 V2000 |
| Phosphate | mgPO43- | ISO 6878 V 2004 |
| Organohalogenated pesticides | µg/L | EN NF ISO V6468-97 |
| Organo-chlorinated pesticides | µg/L | EN NF ISO V 11369-97 |
| Carbamates | µg/L | EN NF ISO V11369-97 |

2.2.1.3 Determination of Water Quality Index (WQI)

WQI was calculated using weighted arithmetic index method proposed by Yidana and Yidana (2010). Eight important parameters (pH, OD, EC, T°C, BOD₅, NO3−, NH4+, PO43−) were selected for this purpose in Table 2. In this approach, a numerical value called relative weight (Wi), specific to each physico-chemical parameter, is first calculated according to the following equations:

Wi=kSi

With

k=1i=1n(1Si)

k = proportionality constant

n: the number of parameters

Si: maximum value of the standard surface water standard (Table 2) of each parameter in mg/L in this work, except for pH, T°C and electrical conductivity.

Table 2: Guide values for drinking water (WHO, 2022)

| Parameters | Units | WHO guides values |
| --- | --- | --- |
| Temperature | °C | 25 |
| Conductivity | µS/cm | 2000 |
| Turbidity | NTU | 5 |
| pH | - | 6,5˂pH˂8,5 |
| NO₃⁻ | mg/L | 50 |
| NO₂⁻ | mg/L | 3 |
| SO₄² ⁻ | mg/L | 250 |
| PO₄ᵌ ⁻ | mg/L | 5 |
| F⁻ | mg/L | 1,5 |
| Cl⁻ | mg/L | 250 |
| Na⁺ | mg/L | ≤150 |
| Mg²⁺ | mg/L | 50 |
| Ca²⁺ | mg/L | 100 |
| NH₄⁺ | mg/L | ≤0,5 |
| Fe²⁺ | mg/L | 0,3 |
| K⁺ | mg/L | ≤12 |

Then, a quality rating scale (Qi) is calculated for each parameter by dividing concentration by the norm of said parameter and multiplying the set by 100 as in following equation:

Qi=(CiSi)×100

Qi: Quality assessment scale for each parameter.

Ci: Concentration of each parameter in mg/L.

Finally, the overall water quality index is calculated by following equation:

WQI=i=1n(Qi×Wi)i=1nWi

Once calculated, Table 3 was used to determine the water quality class of dams.

Table 3: Classification and possible use of water according to WQI (Meenakshi et al, 2019)

| WQI classe | Water type | Possible use |
| --- | --- | --- |
| 0 – 25 | Excellent quality | Potable water, irrigation and industry |
| > 25– 50 | Good quality | Potable water, irrigation and industry |
| > 50– 75 | Bad quality | Irrigation et industrie |
| > 75– 100 | Very bad quality | Irrigation |
| > 100 | Non-potable water | Appropriate treatment is required before use |

2.2.2.4 Determination of organic pollution index (OPI)

The Organic pollution index (OPI) of dam water was determined using the method used by Hachi et al. (2022) and Kouakou et al. (2024). OPI for rainy and dry seasons were calculated using pollution indicator parameters (NH4+, PO43− and BOD5). The principle is to divide beforehand values of polluting elements (ammonia, BOD5, and phosphates) into 5 classes with the average contents of these three parameters obtained during analyses in the laboratory. Then, determine the corresponding class number for each parameter using the average data used by Hachi (Table 4). In the end, the overall organic pollution index of the dam water was determined through the arithmetic mean of all parameters' pollution classes

Table 4: Organic pollution index classification grid (Hachi et al., 2022

| Classes | NH44+(mg/l) | DBO5 (mgO2/l) | PO₄ᵌ ⁻(μg/l) | IPO | Organic pollution |
| --- | --- | --- | --- | --- | --- |
| 5 | < 0,1 | <2 | < 15 | 4,6 – 5,0 | None |
| 4 | 0,1 – 0 ,9 | 2,1 – 5 | 16 – 75 | 4 – 4,5 | Low |
| 3 | 1 – 2,4 | 5,1 – 10 | 76 – 250 | 3,0 – 3,9 | Moderate |
| 2 | 2,5 – 6 | 10,1 – 15 | 251– 900 | 2,0 – 2,9 | High |
| 1 | > 6 | > 15 | < 900 | 1,0 – 1,9 | very high |

2.2.2 Identification of anthropogenic activities likely to pollute the dam water

Identification of human practices that may be harmful to dam water quality was done by individual interview, which consists of administering individual questionnaires to members of each target group. This approach allows for knowing and evaluating knowledge, attitudes, practices and perceptions of target groups or persons with respect to questions asked (Dawson et al., 1993). These interviews were therefore carried out with the identified resource people living around Boudali, Korhogo and Ouangolodougou dam, who are engaged in any activity around these dams (farmers, artisans, inhabitants and others). The aim of this survey is to investigate the way that those activities and domestic waste are managed by the latter, and to identify the causes likely to pollute the dam's water.

3. results and discussion

3.1 Dams water physico-chemical characterisation

The results of physico-chemical analyses at each locality show that pH values are neutral and in line with WHO requirements (Figure 2). Boundiali and Korhogo dams have average values of 7.16 and 7.18, respectively. However, for Ouangolodougou dam pH average value is 6.9. In addition, pH in the rainy season is higher than in the dry season. pH of these dams being neutral and tending to alkalinity (6.1 to 8.2), coincides with results found in Guiglo-Duekoué zone (west Côte d’Ivoire), whose pH varies successively from 6,51 et 7,13 (Kouassi et al., 2012). These results may be due to a dissimilarity of natural factors such as climate, vegetation, geology and also activities carried out near resources. pH range recorded at Boundiali, Korhogo and Ouangolodougou dams could be explained by the presence of bicarbonates that have a buffering power in the water or by variation in leaching activities and degradation of organic matter (Mohammed et al., 2023).

Figure 2: Seasonal values of pH

Dams' water temperatures are acceptable by WHO (2022) standards in both rainy and dry seasons, with a guide value of 25°C. Also, these temperatures are higher during the rainy season. This would mean that the solubility of salts and gases would have a higher rate in the rainy season than in the dry season (Figure 3). These high water temperatures (> 30 °C) could probably be due to sunshine in the outer layers during the rainy season, as described by Kouakou et al., 2024. High temperature value could affect organoleptic properties of water since it is a factor in modifying chemical and biological properties of water by acting on its density, on gases' solubility, on the dissolved salts dissociation, and on biochemical and biological reactions (Makhoukh et al. 2011, Asfers et al. 2017).

Figure 3: Seasonal values of Temperature

Electrical conductivity of each dam is characterised by low mineralisation (54 to 180 µS/Cm) as shown in Figure 4, which differs from results found in groundwater (Guiglo-Duekoué zone) waters, where conductivity varies from 106 μS/cm et 349 μS/cm. EC value is also higher in Korhogo and Ouangolo compared to Boundiali.  This difference could be related to dissimilarity of natural factors such as climate, vegetation, geology and to activities carried out near resources, as shown by works of Kouassi et al., (2012).

Figure 4: Seasonal values of Conductivity

Concentrations of chemical parameters such as nitrate (NO3−), nitrite (NO2−), ammonium (NH4+), phosphate (PO43−), DO, DOC and BOD5 are shown in Figure 5. DO, DOC and BOD5 have highest contents in the 3 dams waters.  Water from the Korhogo and Ouangolo dam is more concentrated in DOC and BOD5 in the dry season than in the rainy season, unlike in Boundiali where these values are higher in the rainy season than dry season. It is also observed that DO concentration is higher in Boundiali and Korhogo dams in dry season than in the rainy season, contrary to Ouangolo. Variation of NO3−, NO2−, DO and PO43− parameters remains less significant (negligible). BOD₅ values are higher than WHO guide values (5 mg/L), reflecting a high organic contents in dam waters. Based on BOD₅ average value and according to works of Kapepula et al. (2015), dam’s water quality would be poor, also similar to those of (Mehounou et al. 2016). Chemical parameters, despite BOD5 and DOC, comply with WHO guidance values. These highly variable parameters have values higher than the norm. High values of these parameters could be explained by a high concentration of organic matter in these dams. This load could be due to direct discharge of house wastewater into dams and use of fertilisers and pesticides in crop fields. Uncontrolled misuse of these products increases pesticide concentration in water. Water bodies with BOD5 levels above 10 mg/L are considered to be of poor quality (Khan et al.,2022), which has also been described in groundwater and surface water of Aplahoué in Benin in cotton production areas (Akoma et al., 2017).

Figure 5: Seasonally concentrations of chemical parameters

At Korhogo Dam, Trace Elements results show that dissolved iron (Fe) is more concentrated in this resource, with an average of 1.35 mg/L. Manganese (Mn) and arsenic have respective mean values of 0.33 mg/L and 0.09 mg/l (Figure 6).

These high concentrations are thought to be caused by uncontrolled wastes deposits encountered in places near dams, agricultural inputs used in study areas (chemical fertilizers, insecticides or fungicides), industrial wastewater discharged into nature without prior treatment and geological formations since this dam is located in an urban environment. Concentrations of cyanide (CN), lead (Pb) and mercury (Hg) are relatively low with respective means of 0.01 mg/L, 0.01 mg/L, and 0.0016 mg/L. Dissolved iron, arsenic and manganese are higher in both seasons.

At Ouangolodougou dam, Iron (Fe) and arsenic (As) are the most abundant elements.

Mean value of arsenic is 0.09 mg/L. This value remains 9 times higher than the WHO guideline values (10µg/L), constituting a real health risk. For Iron (Fe), the mean concentration is 1.4 mg/L, exceeding the standards set by WHO (0.5 mg/L). This high concentration of iron and As could be due to the use of chemical fertilizers, insecticides or fungicides, geological formations, domestic, artisanal and gold mining activities. Other parameters, such as Manganese (Mn), Cyanide (CN), Lead (Pb), and Mercury (Hg), have their concentrations in line with WHO guideline values. However, all metal trace elements generally have higher concentrations in rainy than dry season, except for mercury, cyanide and lead, which are almost identical in both seasons. According to these results in these various dams, arsenic and iron are the two major ones having a concentration much higher than the norm. The waters exploited by the population of Guiglo-Duekoué village appear to pose almost no norm conformity problem since there are no industrial activities (Kouassi et al., 2012. Mohod and Dhote (2013) also confirm that toxic metals are generally present in industrial, municipal and urban effluents. For this purpose, high values of arsenic (0.21 mg/L) and iron (3.29 mg/L) found could be comparable to discharges of municipal wastewater into the dam.

Hama et al. (2023) and Xu et al. (2022) have explained that the occurrence of heavy metal contamination in surface water maybe due to the increase in human population, industrialization, and agricultural activities expansion. For Singh et al. (2022), Heavy metals in the environment pose a real public health risk to populations since the exposure to heavy metals can damage kidneys, livers and lungs leading to the development of different cancers, fragilizing Bones and death in case of huge amount of exposure. Zhang et al. (2023) described that contaminated water with heavy metal ions like Cr (VI), Cd (II), Pb (II), As (V and III), Hg (II), Ni (II), and Cu (II) could be responsible for several health issues in humans, mental disorders, harmful effects on the reproductive system. Even malignancies can be developed (Singh et al. 2024).

Figure 6: Seasonal Metal trace element contents

Studied pesticides are organohalogens (POH), organophosphorus (OP) and carbamates (CBT). In general, high concentrations of these various resources are observed during the rainy season and more intensively in Korhogo and Ouangolodougou (Figure 7). At Boundiali, dam results show fairly considerable average concentrations above WHO standard (0.1 µg/L), for POH and POP, respectively of 0.51 µg/L and 0.26 µg/L. On the other hand, the CBT concentration is low on average by 0.03 µg/L. At the Korhogo dam, the average concentration of CBT is very low. POHs have the highest value with an average of 2.47 µg/L, followed by POPS with 1.79 µg/L and finally CBTs with 0.05 µg/L. Apart from carbamates which respects limit of the WHO guideline values for pesticides (0.1 µg/L), others (POH and POPs) with mean concentrations of 2 and 1 µg/L, are above normal. At the Ouangolodougou dam, shows that average concentration of these pesticides remained high.  These values are above indicated threshold by the WHO standard for pesticides (0.1 µg/L). Similar results have been related by Lakhili et al. (2015), where authors explained that pesticides used to control pests, are drained during the rainy season toward surface waters and remain persistent in dry period. Once pesticides reach water bodies, it become a global health concern as pesticides can impact the whole ecological food chain (Kruć-Fijałkowska et al., 2022).  Another concern is high-level toxicity reached by pesticides when mixed are mixed in water bodies as revealed Oltramare et al. (2023) and long-time exposure to the low concentration of pesticides which could resulted in non-carcinogenic health risks (De Souza et al., 2020).

Figure 7: Seasonal concentration of pesticides

3.2 Assessment of water quality and pollution level of various dams by WQI and OPI

WQI calculation has been done using the results of physico-chemical analysis and values of the WHO drinking water standard (WHO, 2017). WQI was calculated over the two seasons (August-September) and (December-January). Both WQIs results are considered to be high (Tables 5, 6, 7), but the highest values are found in dry season with 477.620 for Boundiali, 130 for Korhogo and 147.66 for Ouangolodougou, while dry season WQI were 83.467 in Boundiali dam, 190.6404 in Korhogo and 94.81 in Ouangolodougou. In the rainy season dams waters are classified as very bad quality to non-potable water for drinking supply water. In the dry season, all dams' waters are qualified as non-potable water for drinking supply water. These results are consistent with those of Vital et al. (2018) on the evaluation of surface water contamination and its impacts on health in mining districts of Kambélé and Bétaré-Oya. Dams Water quality degradation in the present study could be attributed mainly to agricultural activities through leaching of soils with high fertilizer content and urban wastewater discharges (Kim et al., 2023).  Variation of seasonal water quality has been widely reported by Yao et al. 2020. High value of WQI in dry periods involved the fact that water level drops (no rain) while inputs of wastewaters from nearest houses of dams remain important, as documented by works of Hachem et al. (2020) and Barakat et al. (2016).

Table 5: Rainy and dry seasons WQI for Boundiali dam

|  |  |  |  | Rainy season |  | Dry season |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| pH | 8,5 | 0,1176 | 0,0435 | 86,6712 | 83,467 | 81,9672 | 477,620 |
| T (°C) | 25 | 0,04 | 0,0148 | 109,88 |  | 90,28 |  |
| Cond (µS/cm) | 2000 | 0,0005 | 0,0002 | 2,5485 |  | 6,351 |  |
| DO (mg/L) | 8 | 0,125 | 0,0462 | 549,375 |  | 9360 |  |
| NH4+(mg/L) | 0,5 | 2 | 0,7399 | 49 |  | 49,4 |  |
| NO3-(mg/L) | 50 | 0,02 | 0,0074 | 0,74 |  | 0,6 |  |
| PO43-(mg/L) | 5 | 0,2 | 0,0740 | 2 |  | 9,8 |  |
| BOD5 (mg/L) | 5 | 0,2 | 0,0740 | 220 |  | 40,6 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

Table 6: Rainy and Dry Season WQI for Korhogo Dam

|  |  |  |  | Rainy season |  | Dry season |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| pH | 8,5 | 0,1176 | 0,0435 | 85,0980 | 190,6404 | 83,9216 | 130,0898 |
| T (°C) | 25 | 0,04 | 0,0148 | 116,8 |  | 106,24 |  |
| Cond (µS/cm) | 2000 | 0,0005 | 0,0002 | 7,575 |  | 8,7116 |  |
| DO (mg/L) | 8 | 0,125 | 0,0462 | 81,25 |  | 107,475 |  |
| NH4+(mg/L) | 0,5 | 2 | 0,7399 | 190,6667 |  | 113,3334 |  |
| NO3-(mg/L) | 50 | 0,02 | 0,0074 | 0,9467 |  | 0,5233 |  |
| PO43-(mg/L) | 5 | 0,2 | 0,0740 | 7,0667 |  | 7 |  |
| BOD5 (mg/L) | 5 | 0,2 | 0,0740 | 538,6667 |  | 480 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

Table 7: Rainy and Dry Season WQI for Ouangolodougou Dam

|  |  |  |  | Rainy season |  | Dry season |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Si | 1/Si | Wi | Qi | IQE | Qi | IQE |
| pH | 8,5 | 0,1176 | 0,0435 | 87,35 | 94,81 | 75 | 147,66 |
| T (°C) | 25 | 0,04 | 0,0148 | 114,70 |  | 92,87 |  |
| Cond (µS/cm) | 2000 | 0,0005 | 0,0002 | 4,95 |  | 7,53 |  |
| DO (mg/L) | 8 | 0,125 | 0,0462 | 89,74 |  | 32,16 |  |
| NH4+(mg/L) | 0,5 | 2 | 0,7399 | 98,00 |  | 172 |  |
| NO3-(mg/L) | 50 | 0,02 | 0,0074 | 2,59 |  | 0,99 |  |
| PO43-(mg/L) | 5 | 0,2 | 0,0740 | 6,70 |  | 4 ,8 |  |
| BOD5 (mg/L) | 5 | 0,2 | 0,0740 | 164,00 |  | 188 |  |
|  | TOTAL | 2,7031 | 1 |  |  |  |  |
|  | K | 0,3699 |  |  |  |  |  |

In Boundiali, OPI has a high value of 3.667 in rainy season and 2.667 in dry season (Table 8). However, values show that Boundiali dam has moderate organic pollution in rainy season while in dry season it is strong. At Korhogo, OPI level has a constant value of 2.33 in both seasons (Table 9). So this shows that Kôkô dam has a strong organic pollution. Finally, in Ouangolodougou IPO is 3 in rainy season and 2,667 in dry season (Table 10). This therefore reflects a moderate pollution in organic matter. Waters of these dams in dry season have high organic pollution. OPI, generally remains moderate with some difference in value. The qualitative deterioration is mainly anthropogenic related to agricultural activities by leaching of soils heavily loaded with fertilisers (Vital et al., 2018). Similar work has shown that the increase in the pollution index in dry season is linked to the decrease in dam flows due rain reduction, while the loaded of wastewater from the city remain significant (Hachem et al., 2020; Barakat et al., 2016). Moderate pollution of these dams means that these resources are subject to high levels of load of organic matter (BOD₅) and phosphate. Indeed, domestic and industrial wastewater discharges of Ouangolodougo and Korhogo cities result in an increase in the contents of these above mentioned parameters, which favour degradation of water quality (Perrin et al., 2014).

Table 8: Boundiali Dam Rainy and Dry Season IPO

|  | Saison Pluvieuse |  |  | Saison Sèche |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Valeurs Moyennes | Classes | IPO | Valeurs Moyennes | Classes | IPO |
| NH4+(mg/l) | 0,245 | 4 | 3,667 | 0,246 | 4 | 2,667 |
| PO43-( µg/l) | 100 | 3 |  | 490 | 2 |  |
| DBO5 (mg/l) | 11 | 4 |  | 2 ,03 | 2 |  |

Table 9: Korhogo Dam Rainy and Dry Season IPO

|  | Saison Pluvieuse |  |  | Saison Sèche |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Paramètres | Valeurs Moyennes | Classes | IPO | Valeurs Moyennes | Classes | IPO |
| NH4+(mg/l) | 0,9533 | 4 | 2,3333 | 0,5667 | 4 | 2,3333 |
| PO43-(mg/l) | 353,3333 | 1 |  | 350 | 1 |  |
| DBO5 (mg/l) | 26,9333 | 2 |  | 24 | 2 |  |

Table 10: Ouangolodougou Dam Rainy and Dry Season IPO

|  | Rainy season |  |  | Dry season |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters | Mean | Class | IPO | Mean | Class | IPO |
| NH4+ (mg/L) | 0,49 | 4 | 3 | 0,86 | 2 | 2,66 |
| DBO5 (mg/L) | 8,2 | 3 |  | 9,4 | 3 |  |
| PO43- (µg/L) | 335 | 2 |  | 240 | 3 |  |

3.3 Identification of anthropogenic activities likely to pollute water resources

Watershed of Boundiali Dam is exclusively comprises several agricultural activities, including rice and vegetable production (Figures 8).

Figure 8: Proportion of activities likely to pollute the water of the Boundiali dam

Korhogo Dam watershed includes several identified activities such as agriculture, crafts and housing (Figure 9). Agriculture occupies 56% of this area with a high use of mineral fertilisers and phytosanitary products that have an impact on dam water. Handicrafts occupy 20% and include the informal sector (car and motorcycle garages, scrap metal, and vulcanisation...). These actors use different products in the exercise of their activities, especially scrap metal workers who pile up iron on the dam edge. All these wastes and residues are dumped on the ground, which are washed away by rainwater runoff towards the dam. The survey reveals the presence of dwellings in the watershed due to the position of the dam inside the city, some of which discharge their wastewater directly into the dam due to a lack of sanitation. Household waste is deposited close to these homes and is transported during the rains to the dam.

Figure 9: Proportion of activities likely to pollute the water of the Korhogo dam

Ouangolodougou dam watershed includes several human activities including agriculture (49%), crafts (27%) and sanitation (24%) (Figure 10). Gardening takes place mainly in the dry season with the use of mineral, organic fertilisers and phytosanitary products that increase the organic and mineral pollution of the dam water. Several herds come daily to drink and leave their waste near this resource, also influencing its organic pollution. The craft sector includes the informal sector (vehicles and motorcycle repairs, scrap metal, and vulcanisation...).

Figure 10: Proportion of activities likely to pollute the water of the Ouangolodougou dam

4. CONCLUSION

This study shows that Boundiali, Korhogo and Ouangolodougou dams are characterised by a low temperature compared to the WHO standard and a high content of BOD₅, COD, NH₄⁺, pesticides (POP and POH), and metals (iron and arsenic). The active presence of these parameters in water strongly influences their qualities. The assessment of quality and pollution has allowed to qualify these resources as of very poor quality in rainy periods and as non-potable in dry periods. This state of deterioration is directly linked to human actions probably economic activities and households. Household waste, municipal wastewater, and income-generating activities that take place within watersheds are the real sources of quality degradation and pollution. The IPOs and WQIs showed that water pollution from the Korhogo dam is not due to a source that varies according to seasons, unlike that from the Boundiali and Ouangolodougou dams, which is caused by agriculture. Agriculture's contribution to this degradation is linked to crop protection products and fertilisers that farmers use. In order to reduce pollution and maintain the water quality of these dams, it is advisable to:

- Protect them by setting up and respecting the various protection perimeters;

- Raise awareness among farmers for the rational use of fertiliser products;

- Promote the proper management of household waste through the creation of landfills and the recycling of household waste;

- Collect all municipal wastewater for possible treatment before discharge.

Thus, the study perspectives that emerge are:

Strengthen the study by taking into account several parameters, including bacteriological parameters, to better characterise the dam. Conduct a hydrobiological study to assess biological quality.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

1. Nations Unies. (2015). 159/5000 Resolution adopted by the General Assembly on 25 September 2015. Transforming our world: the 2030 Agenda for Sustainable Development. A/RES/70/1

2. UN-Water. (2023). The Blueprint for Acceleration: SDG 6 Synthesis Report on Water and Sanitation. United Nations. A/CONF.236/6 (Document Symbol) or 4030567 (Record Number). 92p. https://www.un-ilibrary.org/content/books/9789210026444

3. WHO/UNICEF Joint Monitoring Programme (JMP). (2024). WASH in health care facilities 2023 data update: Special focus on primary health care. WHO, UNICEF. 132p. https://data.unicef.org/resources/jmp-wash-in-health-care-facilities-2024/

4. Ministry of Water and Forestry (MEF). (2023) Bilan des Activités 2023. Rapport de Bilan (Contient section Ressources en Eau). 40p. https://eauxetforets.gouv.ci/sites/default/files/communique/bilan_2023_eaux_et_forets_ci_web-compresse.pdf

5. World Bank. (2023). Côte d’Ivoire - Urban Water Supply Strengthening Project: Environmental and Social Management Framework (ESMF). World Bank Group. (Rapport No. ESMF-PREMU-2023).

6. Yao B.A.F., Soro G.E., Larbi I., Limantol A.M., Goula B.T.A. (2024). Effects of climate variability and/or land use dynamics on the hydrological balance of the Cavally river catchment at Toulepleu, West Africa. Journal of Water and Climate Change 15 (5): 2092–2109. https://doi.org/10.2166/wcc.2024.512

7. Yao, K. L., Kouakou, K. E., Kouakou, A. B. P., Yapo, N. S., & Kouassi, A. M. (2025). Assessment of the water pollution level of the N’ZI River (Côte d’Ivoire). Journal of Water Resources and Ocean Science, 14(1), 18–34. https://doi.org/10.11648/j.wros.20251401.12

8. Akatchi A.A., Attoungbre K.S., Niamien-Ebrottie J.E., Boussou K.C. (2024). Characterization of water quality in the river Lobo and tributaries Center-West of Côte d’Ivoire. Int. J. Biosci. 24(2), 10-19. DOI: http://dx.doi.org/10.12692/ijb/24.2.10-19

9. Soro, G. E., Yao, A. B., Kouame, Y. M., & Bi, T. A. G. (2017). Climate Change and Its Impacts on Water Resources in the Bandama Basin, Côte d’Ivoire. Hydrology, 4(1), 18. https://doi.org/10.3390/hydrology4010018

10. Ministry of Health, Public Hygiene and Universal Health Coverage. (2024) Annual Report on the Health Status of Populations 2023. Abidjan, Ivory Coast. (Report No. MSA-2024-01). https://guce.gouv.ci/health?lang=en

11. SODECI. (2022). Sustainable Development Report 2022. Water Distribution Company of Côte d'Ivoire. https://www.eranove.com/wp-content/uploads/2023/10/SODECI-Rapport-DD-2022.pdf

12. Koffi, A., Yao, D.R., Yao, E., Quiblier, C., Niamen-Ebrottié, J., Hamlaoui, Sahima et al. (2021). Assessment of some key indicators of the ecological status of an African freshwater lagoon (Lagoon Aghien, Ivory Coast). PLoS ONE. 16. 1-18. https://doi.org/10.1371/journal.pone.0251065 kone

13/ Varis, O., Rahaman, M. M., & Stucki, V. (2008). Integrated water resources management plans: the key to sustainability. Modern myths of the Mekong. A critical review of water and development concepts, principles and policies. Water & Development Publications, Helsinki University of Technology, Helsinki, 173-184.

14.General Population and Housing Census (GPHC) (2021). https://www.plan.gouv.ci/assets/fichier/RGPH2021-RESULTATS-GLOBAUX-VF.pdf

15. Yidana, S. M., & Yidana, A. (2010). Assessing water quality using water quality index and

multivariate analysis. Environmental Earth Sciences, 59 (7), 1461-1473.

16. World Health Organization (WHO). (2022). Guidelines for drinking-water quality: Fourth edition, incorporating the second addendum. Geneva: World Health Organization. ISBN 978-92-4-004506-4. https://iris.who.int/items/7462ffb0-b73d-420b-94b5-6450ad3b768f

17. Jatayan, Meenakshi. (2019). Assessment of Water Quality Index of Groundwater of Panchkula & Derabassi Region. International Journal for Research in Applied Science and Engineering Technology. 7. 650-653. 10.22214/ijraset.2019.2090.

18. Hachi, T., Hachi, M., Essabiri, H., Boumalkha, O., Doubi, M., Khaffou, M., Abba, E. (2022). Moroccan Journal of Chemistry Statistical Assessment of the Water Quality using Water Quality Index and Organic Pollution Index -Case study, Oued Tighza. Morocco. 10.48317/IMIST.PRSM/morjchem-v10i3.33139. chabourlne leclerq

19. Kouakou Y.S, Seyhi B Konan N.M. (2024). Impacts of Anthropogenic Activities on Water Quality of Ouangolodougou Dam, Côte d'Ivoire. International Journal of Environment and Climate Change Volume 14, Issue 12, Page 64-81

20. Dawson, S., Manderson, L., & Tallo, V. L. (1993). A manual for the use of focus groups. International Nutrition Foundation for Developing Countries (INFDC). https://idl-bnc-idrc.dspacedirect.org/handle/10625/11963

21. Kouassi, M.A., Ahoussi, E.K., Koffi, B.Y., Ake, Y.A., Biemi, J. (2012). Caractérisation hydrogéochimique des eaux des aquifères fissurés de la zone Guiglo-Duekoué (Ouest de la Côte d’Ivoire). International Journal of Biological and Chemical Sciences. 6. 10.4314/ijbcs.v6i1.45. ahoussi

22. Mohammed, M., Arabi, M., Sbaa, M., Smiri, Y. (2023). Contribution to the evaluation of the physicochemical and metallic quality of the surface waters of the Moulouya River (Lower Moulouya, Eastern Morocco). 10.48317/IMIST.PRSM/morjchem-v11i1.31165. Aboudi

23. Makhoukh, M., Sbaa, M., Berrahou, A., & Van Clooster, M. (2011). Contribution to the physicochemical study of the surface waters of the Moulouya wadi (East Morocco). Environmental Science Studies ,21.

24. Asfers, Y., Hamid, T., Amine, A., Elanz, S. Ahmed, S.. (2017). Study of Physico-Chemical Quality of the Surface Waters of the Estuary of the Western Basin of the Oued Oum Errabia-Morocco. IOSR Journal of Applied Chemistry. 10. 41-49. 10.9790/5736-1005024149. Akil

25. Kapepula, L., Mateso, L.L., Shekani, A., Muyisa,S., Ndikumana, T., & Van der B. (2015). Evaluation de la charge polluante des rivières des eaux usées ménagères et pluviales dans la ville de Bukavu, République Démocratique du Congo. Afrique Science, 11 (2), 195-204. https://www.ajol.info/index.php/afsci/article/view/118488/108021

26. Akoma, O. C., & Uhunmwangho, O. S. (2017). Quantitative estimation of pollution in groundwater and surface water in Benin City and environs. Ethiopian Journal of Environmental Studies and Management, 10(3), 340-350.

27. Khan S.A.R., Ponce P., Yu Z., Golpîra H, Manoj M. 2022. Environmental technology and wastewater treatment: Strategies to achieve environmental sustainability. Chemosphere, Volume 286, Part 1, 131532,. https://doi.org/10.1016/j.chemosphere.2021.131532.

28. Mehounou, J. P., Josse, R. G., Dossou Yovo, P., Senou, S. F., & Toklo, R. M. (2016). Physico chemical and microbiological characterization of groundwater and surface water in the Aplahoué cotton production area. Journal of Applied Biosciences, 103, 9841-9853.

29. Mohod, C. V., & Dhote, J. (2013). Review of heavy metals in drinking water and their effect on human health. International Journal of Innovative Research in Science, Engineering and Technology, 2(7), 2992-2996.

30. Hama Aziz, K. H., Mustafa, F. S., Omer, K. M., Hama, S., Hamarawf, R. F., & Rahman, K. O. (2023). Heavy metal pollution in the aquatic environment: Efficient and low-cost removal approaches to eliminate their toxicity: A review. RSC Advances, 13(26), 17595-17610. DOI: 10.1039/d3ra00723e

31. Xu, F., Wang, Y., Chen, X., Liang, L., Zhang, Y., Zhang, F., & Zhang, T. (2022). Assessing the environmental risk and mobility of cobalt in sediment near nonferrous metal mines with risk assessment indexes and the diffusive gradients in thin films (DGT) technique. Environmental Research, 212(C), 113456. Available:https://doi.org/10.1016/j.envres.2022.113456

32. Singh, A., Sharma, A., Verma, K. R., Chopade, L. R., Pandit, P. P., Nagar, V., et al. (2022). Heavy metal contamination of water and their toxic effect on living organisms. In The Toxicity of Environmental Pollutants(pp. 1-26). IntechOpen.  https://doi.org/10.5772/intechopen.105075; http://dx.doi.org/10.5772/intechopen.105075

33. Singh, V., Ahmed, G., Vedika, S., Kumar, P., Sanjay, K., Chaturvedi, S. N., Rai, S. N., et al. (2024). Toxic heavy metal ions contamination in water and their sustainable reduction by eco-friendly methods: Isotherms, thermodynamics and kinetics study. Scientific Reports, 14(1), 7595.https://doi.org/10.1038/s41598-024-58061-3

34. Zhang, P., Yang, M., Lan, J., Huang, Y., Zhang, J., Huang, S., Yang, Y., & Ru, J. (2023). Water quality degradation due to heavy metal contamination: Health impacts and eco-friendly approaches for heavy metal remediation. Toxics, 11(10), 828. DOI: 10.3390/toxics11100828

35. Kruć-Fijałkowska, R., Dragon, K., Drożdżyński ,D., & Górski, J. (2022). Seasonal variation

of pesticides in surface water and drinking water wells in the annual cycle in western Poland, and potential health risk assessment. Scientific Reports, 12, 33-17. https://doi.org/10.1038/s41598022 07385-z

36. Oltramare, C., Weiss, F. T., Staudacher, P., Kibirango, O., Atuhaire, A., & Stamm, C. (2023). Pesticides monitoring in surface water of a subsistence agricultural catchment in Uganda using passive samplers. Environmental Science and Pollution Research International, 30(4), 10312-10328. https://doi.org/10.1007/s11356-022-22717-2

37. De Souza, R. M., Seibert,D., Quesada, H. B., Bassetti, F. J., Fagundes-Klen, R. F., Bergamasco, R. (2020). Occurrence, impacts and general aspects of pesticides in surface water: A review. Process      Safety and Environmental Protection, 135,22-37. https://doi.org/10.1016/j.psep.2019.12.035

38. Lakhili, F., Benabdelhadi, M., Bouderka, N., Lahrach, H., & Lahrach, A. (2015). Study of the physicochemical quality and metallic contamination of the surface waters of the Beht watershed (Morocco). European Scientific Journal, 11(11), 132-147. https://eujournal.org/index.php/esj/article/view/5443

39. Vital, S. N. C., Robert, N., & Benot, N. M. (2018). Pollution of water for human consumption and short-term health risks: Case of the Menoua watershed (West Cameroon). European Scientific Journal, 14(3), 96-117.

40. Kim, Y. E., Jeon, D., Lee, H., Huh, Y., Lee, S., Kim, J. G., et al. (2023). Revealing the extent of pesticide runoff to the surface water in agricultural watersheds. Water, 15(22), 3984.

https://doi.org/10.3390/w15223984

41. Serge, Y. K., & Ernest, A. K. (2020). Physico-chemical characterization of surface waters in a mining environment in west-central Côte d’Ivoire: The case of the department of Divo. European Scientific Journal, ESJ, 16(12), 293. https://doi.org/10.19044/esj.2020.v16n12p293

https://eujournal.org/index.php/esj/article/view/12831

42. Hachem Bekri, M., El Hmaidi, A., Jaddi, H. J., Kasse, Z., Mati El Faleh, E. M. E. F., Essahlaoui, A.,  El Ouali, A. (2020). Utilisation Des Indices De Qualité Et De Pollution Organique Dans L’évaluation De La Qualité PhysicoChimique Des Eaux Superficielles Des Oueds Moulouya et Ansegmir (Haute Moulouya, NE Du Maroc). European Scientific Journal, ESJ, 16(27), 55. https://doi.org/10.19044/esj.2020.v16n27p55

43. Barakat, A., El Baghdadi, M., Rais, J., Aghezzaf, B., & Slassi, M. (2016). Assessment of spatial and seasonal water quality variation of Oum Er Rbia River (Morocco) using multivariate statistical techniques. International soil and water conservation research, 4(4), 284-292.

44. Perrin, J. L., Raïs, N., Chahinian, N., Moulin, P., & Ijjaali, M. (2014). Water quality assessment of highly polluted rivers in a semi-arid Mediterranean zone Oued Fez and Sebou River (Morocco). Journal of hydrology, 510, 26-34.