Original Research Article

Ecological Risk Assessment of Selected Heavy Metals in Nwaniba River, Akwa Ibom State

ABSTRACT

This study assessed the ecological risk of heavy metals in sediments and evaluated metal transfer between sediments and water in the Nwaniba River. Water and sediment samples were collected from four sites during the rainy season and analyzed for seven metals (Tl, As, Pb, Cd, Cr, Co, Ni) using the Inductively Coupled Plasma Optical Emission Spectroscopy (ICP-OES) technique. Single-element indices indicated sediment pollution ranging from low to high levels: contamination Factor (ND – 12.71), enrichment Factor (ND – 9.48) and geo-accumulation index (-1.98 – 3.08), while multi-element indices revealed elevated ecological risk, with pollution load index (PLI) values of 1.94, 1.92, 0.82, and 1.94, indicating PLI > 1 for the majority of the study area and potential ecological risk index (PERI) values of 406.60, 388.84, 187.00, and 293.79 across the four sites, respectively, indicating moderate to high ecological risk at most locations. Cadmium (Cd) was identified as the primary contributor to the calculated ecological risks. Transfer factor (TF) values determined for metals with measurable concentrations in both water and sediment exceeded 1, (TF >1) suggesting potential metal accumulation and increased ecological risk due to mobility within the river system. These findings confirm that the sediments are contaminated and pose ecological threats, likely influenced by agricultural runoff and unregulated municipal waste. Regular monitoring and sustainable land-use practices are recommended to mitigate ecological risks and protect the Nwaniba River ecosystem. By informing sediment monitoring and risk mitigation in data-scarce, rapidly urbanizing regions, this study contributes to Sustainable Development Goals six and eleven, emphasizing clean aquatic ecosystems and sustainable urban environments.

Keywords: Anthropogenic; Ecological risks; Pollution, geo-accumulation index

1. INTRODUCTION

Sediment and soil geochemistry is essential to infer the primary source of potential mineralization, sediment provenance, environmental conditions, and heavy metal pollution in the ecosystem (Afahnwie et al., 2025a; Jean-Lavenir et al., 2025; Yiika et al., 2025). The introduction of poisonous substances into aquatic ecosystems has been a significant issue for several decades, leading to widespread environmental damage and disruption of these delicate habitats (Tiabou et al., 2024a; Simou et al., 2024a). One of the most prevalent contaminants that harm our aquatic ecosystems and are of ecological significance are trace or heavy metals, which are metallic chemical elements with a relatively high density (more than 6g/cm3) and toxicity at low concentrations (Yiika et al., 2022; Sanad et al., 2025a). Heavy metals are a common pollutant of waterbodies, with their distribution facilitated by increased population, urbanization and industrialization (Kachoueiyan et al., 2024a; Sigue et al., 2025). Heavy metals present significant ecological threats due to their high toxicity, non-biodegradability, bioaccumulation and biomagnification (Yiika et al., 2023; Awakama, et al., 2024). Intrusion of heavy metals into aquatic environments results from both geogenic and anthropogenic processes, affecting the concentration, speciation and bioavailability of these pollutants (Simou et al., 2024c; Afahnwie et al., 2025b; Tiabou et al., 2025a). Anthropogenic activities, such as agriculture, aquaculture, mining, and ship transportation are the primary cause of elevated concentrations of heavy metals in the aquatic environment (Ubong, et al., 2021; Jean-Lavenir et al., 2024; Tiabou et al., 2024b). This pollution degrades aquatic ecosystems and negatively impacts all living and non-living things within them.

Numerous heavy metals are essential for the proper functioning of plants and animals, although certain metals (such as arsenic, cadmium, lead and mercury) have no identified role in these organisms (Onoyima, et al., 2021; Afahnwie et al., 2025c; Sigue et al., 2025). Excessive levels of both essential and non-essential metals might result in detrimental health effects (Obadimu, et al., 2024). Lead (Pb) exposure is reported to account for around 3% of cardiovascular disease and 2% of the global burden of ischaemic heart disease (WHO, 2010). Metals like nickel, cadmium and lead lack biological purpose and are potentially hazardous even at trace levels. Generally, they are not biodegradable and their bioaccumulation in seafood, sediments, and other elements of aquatic ecosystems has been documented worldwide (Ubong, et al., 2023a; Yiika et al., 2024; Kachoueiyan et al., 2024b).

Rivers and other aquatic bodies serve as the last depository for human waste, resulting in the deposition of some harmful heavy metals into coastal sediments (Yawo & Akpan, 2021; Tiabou et al., 2025b). Sediments are an intrinsic component of the aquatic ecosystem, as they function as a sink for heavy metals (Obadimu, et al., 2025b; Sanad et al., 2025b; Afahnwie et al., 2025d). Over 90% of the total heavy metal load in aquatic settings is associated with suspended particulate matter and sediments (Islam, et al., 2017; Simou et al., 2024c). Heavy metals attached to suspended particles accumulate in sediments via precipitation, coprecipitation, chelation, and biological impacts resulting in the highest concentrations of hazardous metals in aquatic environments being stored in sediments (Hamuna & Wanimbo, 2021). Research indicates that activities disturbing the seabed, such as trawling, dredging, excavation, piling or drilling frequently expose sediment-encased heavy metals and other pollutants, resulting in their redistribution and recontamination throughout the components of impacted ecosystems, either temporarily or for an extended duration (Fennel, et al., 2019, Isotuk, et al., 2023).

In a riverine ecosystem, sediments serve as environmental indicators of metal pollution owing to their significant physicochemical stability (Sharma, et al., 2021; Jean-Lavenir et al., 2024). The quantitative evaluation of pollution indices in river sediments provides valuable insights into the pollution status and potential ecotoxicity of the aquatic environment. Heavy metal contamination in soil and sediments has been assessed using a variety of evaluation techniques (Tiabou et al., 2025c). These techniques include geochemical normalization techniques (e.g., ecological risk index, pollutant load index, contamination factor, enrichment factor, sediment quality guidelines, and geo-accumulation index) (Algül & Beyhan, 2020, Onoyima, 2021; Tiabou et al., 2024c), and multivariate statistical analysis such as Pearson correlation, principal component analysis and cluster analysis (Okibe, et al., 2020; Djibril et al., 2024). Islam et al. (2015) and Varol (2011) independently reported considerable heavy metal enrichment in riverine sediments, as indicated by elevated contamination factors, geo-accumulation indices, and pollution load indices. Both studies revealed moderate to severe contamination, particularly by Cr, As, and Pb, and a progressive decline in sediment quality. In contrast, Isotuk, et al. (2023) recorded negligible ecological risk in the Obolo, Amadaka, and Emeremen rivers, while low to moderate enrichment of As, Th, and U was detected along the Atlantic coastline and Iko River.

The Nwaniba River, located in Uruan Local Government Area of Akwa Ibom State, serves as a vital water source for the local communities. The region is currently under stress from human activities like fishing, farming, deforestation and dredging of the river bed for shellfish and river-run aggregates (sand and gravel). Despite the growing understanding of heavy metal contamination in aquatic ecosystems, specific data on contamination levels, sources, and ecological risks within the Nwaniba River remain limited. This study is aimed at evaluating the levels of seven selected heavy metals (Tl, As, Pb, Cd, Cr, Co and Ni ) from the water and sediment samples obtained from the Nwaniba River, subjecting the data obtained to transfer factor and ecological risk indices (contamination factor, enrichment factor, geo-accumulation index, pollution load index and potential ecological risk index) computation in an attempt to ascertain the extent of metal mobility, accumulation and ecological impact posed to the aquatic ecosystem. This approach will provide critical insight for environmental management and pollution mitigation in the region.

2. MATERIALS AND METHODS

2.1	The Study Area

The Nwaniba River, encompassing a catchment area of approximately 449 km² and traversing the Mbiakong River within Uruan Local Government Area, was designated as the study location. Geographically situated at 6°40'N latitude and 7°20'E longitude (Figure 1), the river is delineated by several administrative boundaries: Odukpani Local Government Area in Cross River State to the east, Okobo Local Government Area to the south, Nsit Atai and Ibesikpo Asutan Local Government Areas to the west, and Itu Local Government Area to the north (Moses, et al., 2022). Functionally, the river constitutes a major water resource for the adjoining communities. This study examined four sampling sites. The precise geographic coordinates of each sampling site were determined using a Garmin GPSMAP 785 Global Positioning System (GPS) device and are detailed in Table 1.

Fig. 1.  Map of Study Area Showing Sampling Sites.

Table 1.         Sampling Sites with Corresponding Geographic Coordinates

Sites			Site Name 				GPS Co-ordinates

1		Ikoneto, Cross River, Nigeria            Latitude 5.041174˚N; Longitude 8.1007˚E

2		Central Uruan, Akwa Ibom, Nigeria   Latitude 5.034249˚N; Longitude 8.095301˚E

3		Central Uruan, Akwa Ibom, Nigeria   Latitude 5.021154˚N; Longitude 8.077039˚E

4		Central Uruan, Akwa Ibom, Nigeria   Latitude 5.021154 ˚N; Longitude 8.077039˚E

2.2   Sampling and Sample Collection

Sampling was conducted in June 2024, during the rainy season, following the methodology outlined by Moses, et al. (2022). Water and sediment samples, representing the environmental matrices, were collected from four locations along the Nwaniba River to evaluate the spatial distribution of heavy metal concentration. The sampling sites were selected based on their specific characteristics and significance as key pollution sources in order to accurately represent the anthropogenic activities within the catchment area. Composite water samples were obtained from each of the four designated sampling sites and individually stored in well-labelled 500 mL polyethylene bottles. To ensure sample integrity and prevent metal precipitation, the collected water samples were immediately acidified with 2ml of concentrated nitric acid (HNO₃) (USEPA, 2023). Additionally, at each sampling site, triplicate sediment samples were collected from a depth of 0–10 cm using a Van Veen grab sampler, following the methodology outlined by Isotuk, et al. (2023). The triplicate samples from each site were subsequently combined to obtain composite samples respectively. The collected sediment samples from the four (4) Sites were placed into pre-labeled polyethylene bags that had been pre-washed and rinsed with 20% HNO₃ to minimize potential adsorption of trace metals onto the inner surfaces of the bags. Sampling was conducted within the intertidal and littoral zones. Water and sediment samples were placed in distinct compartments of an ice-cooled box to prevent cross-contamination and were promptly transported to the laboratory for metal analysis.

2.3   Sample Preparation and Digestion

Aliquots of 20 mL were taken from each water sample and transferred into individual beakers. Subsequently, 400 μL of HNO₃ and 1 mL of concentrated hydrochloric acid (HCl) were added to each sample. The mixtures were heated at 95˚C for one hour under controlled conditions to prevent boiling. After digestion, the beakers were removed from the heat source and allowed to cool to room temperature. The resulting solutions were filtered through Whatman filter paper to remove any suspended particulates, and the final volumes were made up to 20 mL using deionized water. Sediment samples were air-dried at ambient room temperature for 48 hours. Once dried, the samples were gently disaggregated using a porcelain mortar and subsequently sieved through a polyethylene sieve with a particle size of less than 75 µm to eliminate coarse materials and extraneous debris. The sediment samples were then homogenized, and 0.5 g of each sample was accurately weighed. The samples were transferred into clean beakers, followed by the addition of 20 ml of aqua regia. Digestion was performed on a heating block within a fume hood at a temperature not exceeding 90 °C for approximately one hour. After cooling to room temperature, 2 ml of hydrogen peroxide (H₂O₂) was added to each beaker, and the mixture was heated for an additional 10 minutes. Upon completion of the digestion process, the total volume of each digestate was measured. The solutions were subsequently filtered and diluted to a final volume of 50 ml using ultra-pure deionized water for heavy metal analysis.

2.4   Analytical Technique

The concentration of heavy metals in the digested samples was determined using an Inductively Coupled Plasma Optical Emission Spectrometer (Perkin-Elmer OptimaTM 2000DV, ICP-OES). The heavy metals examined in this study were thallium (Tl), arsenic (As), lead (Pb), cadmium (Cd), chromium (Cr), cobalt (Co), and nickel (Ni).

2.5   Statistical Analysis

Data analysis was carried out using Microsoft Excel 2024. Mean and standard deviation were calculated to summarize the concentrations of heavy metals in water and sediment samples. Bar charts were generated to illustrate the spatial variation of metal levels across sampling locations. Pearson’s correlation analysis between metals in sediment from the four sampling sites were also determined.

2.6   Ecological Risk Assessment

2.6.1   Contamination Factor

The contamination factor (CF) is commonly used to assess the levels of contaminants and identify the sources of heavy metal pollution. The CF was calculated using the equation:

CF=McBc……. (1)

where Mc represents the concentration of the metal in the sample, and Bc denotes the concentration of the background metal. According to previous studies, CF values are categorized into four contamination levels: low contamination (Class 1, CF < 1), moderate contamination (Class 2, 1 ≤ CF < 3), considerable contamination (Class 3, 3 ≤ CF < 6), and very high contamination (Class 4, CF > 6) (Singh, et al., 2021; Tiabou et al., 2024a, 2025a).

2.6.2   Enrichment Factor

The enrichment factor (EF) of the selected metals in the sediment was calculated by normalizing the concentration of each target element to that of a reference element (Fe) in order to assess the potential origin of the metals—whether geogenic or anthropogenic (Tiabou et al., 2024b). An enrichment factor > 1 suggests a contribution from anthropogenic sources beyond natural background levels (Kolawole, et al., 2018; Sigue et al., 2025). Iron (Fe) with reference background value of 47,200 mg/kg was used as the normalization element in this study (Isotuk, et al., 2023). Fe concentrations (76838.10mg/kg, 58937.78mg/kg, 38973.04mg/kg and 62049.40mg/kg) from the general elemental analysis were used solely for EF calculations, though Fe was not among the heavy metals selected for discussion. The EF was calculated by the following equation:

EF =  CiCrefBiBref  ……… (2).

(Ci/Cref) represents the concentration ratio of the element of interest to the reference element within the sample, while (Bi/Bref) denotes the corresponding ratio in the background sample. The enrichment factor (EF) is typically interpreted using five established contamination categories:  EF < 2 indicates absence to minimal enrichment; 2<EF<5 reflects moderate enrichment; 5<EF<20 suggest significant enrichment; 20<EF<40 denotes very high enrichment; and EF > 40 indicates extremely high enrichment (Okoye, et al., 2022; Tiabou et al., 2024c; Afahnwie et al., 2025b).

2.6.3   Geo-accumulation Index

The geoaccumulation index (Igeo) serves as a quantitative measure to assess the degree of heavy metal pollution, categorizing contamination levels based on established classification standards (Xie, et al., 2022b). The Igeo is mathematically expressed as:

Igeo=log2Csample1.5×Cbackground……….(3)

Where Csample is the Heavy metal concentration in sediment and Cbackground​ is the geochemical background level (mg/kg), a constant of 1.5 is applied to attenuate anthropogenic contributions, predominantly mitigating influences from weathering and lithogenic sources. Müller (1969),  proposed a classification system comprising seven categories for the Geo-accumulation Index (Igeo), which are defined as follows: Igeo ≤ 0 – uncontaminated status (Class 0); 0 < Igeo ≤ 1 – uncontaminated to moderately contaminated status (Class 1); 1< Igeo ≤ 2 – moderate contamination status (Class 2); 2<Igeo≤3 – moderate to heavy contamination status (Class 3); 3<Igeo≤4 – heavy contamination status (Class 4); 4< Igeo ≤5 – heavy to extreme contamination status (Class 5); and Igeo>5 – extreme contamination status(Class 6).

2.6.4    Pollution Load Index

The Pollution Load Index (PLI) is a comprehensive tool used to assess the overall level of heavy metal pollution in a given site or region. It gives a single value that reflects the cumulative pollution status based on multiple heavy metals. The Pollution Load Index (PLI) is calculated based on the Contamination Factors (CFs) of individual metals and is expressed as:

PLI= n(CF1×CF2×CF3⋯×CFn)…………(4)

where CF represents the contamination factor of a given metal, i and n is the total number of metals or contaminants assessed at a sampling site (Isotuk, et al 2023; Aljahdali & Alhassan, 2020). According to the classification proposed by Onoyima, et al. (2021), the Pollution Load Index (PLI) can be interpreted using defined thresholds: PLI > 1 indicates pollution, PLI < 1 indicates no pollution, and PLI = 1 reflects baseline or background contamination levels.

2.6.5   Potential Ecological Risk Index

The Potential Ecological Risk Index (PERI) is an assessment method that incorporates the toxic response factor and contamination level of individual heavy metals. It enables the evaluation of both the ecological risk posed by individual elements and the cumulative effects of multiple contaminants within an ecosystem. This index was originally developed by Swedish geochemist Lars Hakanson to evaluate the potential ecological risks posed by heavy metals present in sediments (Ma and Han, 2020, Onoyima et al., 2021). The Potential Ecological Risk Index (PERI) was estimated as follows:

PERI=∑Eri and Eri= Tri×Cfi……………(5)

Where Eri= ecological risk factor of metal i, Tri= toxic response factor for metal i, Cfi = contamination factor of metal i, and PERI= overall potential ecological risk index. The toxic response factors (Tri​) adopted from previous studies include: Cd (30), Cr (2), Ni (5), Hg (40), Pb (5), Zn (1), As (10), Mn (1), V (2), Cu (5), and Ti (1) (Ma and Han, 2020, Isotuk et al., 2023). The Eri and PERI values are categorized as follows:  Eri < 40, low risk; 40 ≤ Eri< 80, moderate risk; 80 ≤ Eri < 160, considerable risk; Eri ≥ 320, very high risk while PERI < 150 (low ecological risk), 150 ≤ PERI < 300(moderate ecological risk), 300 ≤ PERI < 600 (high ecological risk), and PERI ≥ 600 (very high ecological risk) (Hakanson, 1980).

2.7 Transfer Factor

The transfer factor (TF) of metals from water to sediment reflects the ability of sediments to accumulate heavy metals from the surrounding aquatic environment. It serves as an indicator of metal mobility and potential bioavailability. High TF values suggest strong metal retention and possible ecological risk in sediment compartments. Obadimu et al. (2025a) define the transfer factor (TF), in the context of water-to-sediment transfer, as the ratio between the concentration of a dissolved metal in water and its accumulated concentration in sediment. Transfer factor of the metals from the four sites were determined using the expression:

TFsediment/water=Metal concentration in sedimentMetal concentration in water

As reported by Davies et al. (2021), metal transfer factor (TF) values can be categorized as follows: TF < 1 – low transfer (excluders), TF = 1 – no influence, and TF > 1 – high transfer (accumulators).

3.  RESULTS AND DISCUSSION

3.1   Results

Heavy metal concentrations in water and sediments were evaluated at four sites along the Nwaniba River. Descriptive statistics are provided in Table 2, with full datasets in the Supplementary Information (Tables S1–S2) and metal distributions shown in Figures 2 and 3. Sediments generally had higher mean concentrations of heavy metals than the overlying water (Table 2). This observation corroborates existing literature identifying sediments as primary sinks for contaminants, particularly heavy metals, within aquatic ecosystems (Isotuk et al., 2023; Davies et al., 2021; Obadimu et al., 2025b). The results presented in Table 2 reveal that Cr was highest in sediments (73.516 ± 33.038 mg/kg) and Tl was highest in water (0.0243 ± 0.0485 mg/L). The raw concentrations of metals in water and sediments, detailed in the supplementary information (Tables S1–S2), were the input data used for calculating the summarized statistics (Table 2) and all subsequent pollution indices (Tables 3 – 7). Tl was detected exclusively at Site 1 in both water samples (0.0970 mg/L) and sediment samples (4.370 mg/kg) (Tables S1–S2). A similar spatially restricted occurrence of Tl was reported by Isotuk et al. (2023), who detected Tl at only one out of five sampling locations. The isolated occurrence of Tl may be attributed to atmospheric deposition or rainfall-induced runoff. Potential sources include fossil fuel combustion, biomass burning or sediment dredging activities occurring near site 1 (upstream) (McMaster et al., 2020).

Table 2.  Descriptive Statistics of Heavy Metal Concentrations in the sampling sites

Mean ± Standard Deviation                              Range

Heavy metal     water(mg/l)      sediment(mg/kg)         water(mg/l)                sediment(mg/kg)

Tl	      0.0243±0.0485           1.218±2.435		ND – 0.0970	            ND – 4.870

As	      0.0225±0.0261	   14.891±10.694            ND – 0.0477	            4.936 – 25.806

Pb	      0.0103±0.0170	   48.714±17.841            ND – 0.0354	            27.167 – 64.302

Cd	      0.00±0.00                    2.907±0.957                ND	   	            1.732 – 3.813

Cr	      0.0140±0.0209	   73.516±33.038            ND – 0.0452	             35.755 – 101.896

Co	      0.0111±0.0221	   32.715±15.548	ND – 0.0442                  14.026 – 45.847

Ni	      0.0226±0.0176	   41.780 ± 16.580	ND – 0.0379            	17.535 – 54.610

KEY: ND = NOT DETECTED

Fig 2: Concentrations of Heavy Metals in Water across Sampling Sites

Fig 3.  Concentrations of Heavy Metals in Sediment across Sampling Sites

3.2. Discussion

Single-element pollution indices (contamination factor (CF), enrichment factor (EF), and geo-accumulation index (Igeo)) and multi-element pollution indices (pollution load index (PLI) and potential ecological risk index (PERI)) were utilized to evaluate the ecological risks associated with the analyzed heavy metals in the environment (Kachoueiyan et al., 2024a; Sanad et al., 2025a). CF, EF and Igeo are used to evaluate the contamination levels of individual heavy metals, whereas the PLI and PERI assess the overall pollution arising from the combined presence of multiple heavy metals in sediments (Onoyima, et al., 2021; Tiabou et al., 2025c). The global average background values for shales, as reported by Turekian & Wedepohl (1961) (Table S2), were utilized in this study. The results for the computed CF, EF and Igeo values are listed in Table 3.

The classification of CF values according to Hakanson (1980), provides a robust framework for assessing pollution levels across the four studied sites. Cd recorded the highest contamination factors (CFs) among all studied metals, with values ranging from 5.77 (Site 3) to 12.71 (Site 1). These values reflected considerable contamination to very high contamination, indicating intense anthropogenic input. The extremely high CF at Site 1 suggests a localized point source, likely tied to indiscriminate disposal of domestic and industrial waste, as well as agricultural runoff (Jean-Lavenir et al., 2023). Pb exhibited moderate contamination to considerable contamination across all sites, with CF values ranging from 1.36 (Site 3) to 3.22 (Site 4), while Tl was detected only at Site 1, with a CF of 3.48, categorizing it as considerable contamination. Tl is a less commonly monitored metal, but its toxicity is well established. Its presence at this level could be indicative of industrial emissions, especially from coal combustion or cement production. The ranges of CFs for Co (0.74–2.41), Cr (0.40–1.13), and As (0.38–1.99) across the four sites indicated low contamination to moderate contamination. Ni exhibited the lowest CF values overall (0.26–0.80), falling into the low contamination category at all sites. The CF pattern revealed that Site 1 was the most contaminated, with elevated values across the majority of metals. In general, contamination factor (CF) values ranged from low to very high contamination levels, indicating a wide variation in pollution intensity across the study sites. These findings are consistent with those reported by Yawo & Akpan (2021), who also observed low to high levels of heavy metal contamination in the sediments of the Utibete River, Eastern Obolo, Akwa Ibom State. In contrast, Aigberua & Tarawou (2018) reported only moderate CF values for all heavy metals studied in the bottom sediments of the River Nun, Bayelsa State, Nigeria, suggesting that contamination levels may vary depending on regional environmental conditions and anthropogenic influences. Obadimu, et al. (2025c) utilized machine learning models to assess heavy metal contamination in the Nwaniba River, revealing notably significant levels of pollutants in the river’s sediments.

Cd exhibited the highest enrichment factor (EF) values among all the metals, ranging from 6.42 (Site 4) to 9.48 (Site 2), indicating significant enrichment (5 < EF < 20). The elevated enrichment factors observed for Cd suggest a potential ecological risk. Similar trends were reported by Chris & Anyanwu (2023). Such elevated EFs may result from substantial anthropogenic inputs (Jean-Lavenir et al., 2024; Tiabou et al., 2025b). Cadmium contamination in aquatic environments is often attributed to the use of fertilizers and pesticides (Cui, et al., 2019; Tiabou et al., 2024b, 2024c). EF values for As, Cr, Co, and Ni indicated absent to minimal enrichment (EF < 2) across all sites. Tl and Pb displayed EF values ranging from minimal enrichment (EF < 2) to moderate enrichment (2 < EF < 5). Notably, Tl was enriched only at Site 1 (EF = 2.14) and was effectively absent (EF = 0) at all other sites. In summary, enrichment factors for the studied metals ranged from no enrichment to significant enrichment, indicating varying anthropogenic influence across the sites (Yiika et al., 2024). The enrichment of these heavy metals along site 1 to site 4 may be attributed to rapid urbanization, biomass burning associated with agricultural practices, fossil fuel combustion, and the introduction of contaminants from agricultural activities (Jean-Lavenir, et al., 2020; Isotuk, et al., 2023; Yiika et al., 2023).

According to the sediment quality classification by Müller (1969), the geo-accumulation index (Igeo) values for Cr and Ni at all four sites indicated an uncontaminated status (Igeo < 0). In contrast, Cd exhibited the highest Igeo values across all sites (ranging from 1.94 to 3.08), placing the sediments in categories ranging from moderately contaminated (1 < Igeo ≤ 2), through moderately to heavily contaminated (2 < Igeo ≤ 3), to heavily contaminated (3 < Igeo ≤ 4). This pattern aligns with findings by Anyanwu, et al. (2023), who also reported higher Igeo values for Cd compared to other analyzed metals. For Tl, Igeo values ranged from non-detectable at Sites 2 to 4 to moderately contaminated levels (1 < Igeo ≤ 2) at Site 1. Pb, on the other hand, exhibited Igeo values spanning from uncontaminated (Igeo < 0), through uncontaminated to moderately contaminated (0 < Igeo ≤ 1), to moderately contaminated levels (1 < Igeo ≤ 2). The Igeo values for As and Co across the sites ranged from uncontaminated (Igeo < 0) to uncontaminated-to-moderately contaminated levels (0 < Igeo ≤ 1). Overall, the Igeo results from this study indicate a contamination spectrum from uncontaminated to heavily contaminated conditions. These findings differ from those of Olayinka-Olagunju (2021), who reported uncontaminated sediment quality (Igeo < 0) for the metals analyzed in the sediments of the Owena River, Ondo State. However, they are consistent with the observations of Audu, et al. (2022), who reported moderate to strong pollution levels in selected water bodies in the southern senatorial district of Niger State, Nigeria.

The Pollution Load Index (PLI) was used to assess the overall contamination at the four sampled sites. The Pollution Load Index (PLI) values, summarized in Table 4, provide insight into the overall contamination status of the sites. PLI values for Site 1 (1.94), Site 2 (1.92), and Site 4 (1.94) were all greater than 1, indicating pollution from multiple metals. In contrast, Site 3 recorded a PLI of 0.82, which is below 1, suggesting that the site is not polluted with respect to the analyzed metals. This interpretation aligns with the classification system proposed by Barakat, et al. (2020) and Onoyima, et al. (2021), where PLI > 1 indicates polluted conditions and PLI < 1 indicates unpolluted conditions. In summary, Sites 1, 2, and 4 show evidence of pollution, suggesting that a significant portion of the river is at risk from heavy metals. In line with the findings of this study, Pollution Load Index (PLI) values greater than 1 have also been reported in riverine sediments across the Niger Delta region of Nigeria by other researchers (Anyanwu, et al., 2023; Chris & Anyanwu, 2023; Chris & Davies, 2024), suggesting similar levels of anthropogenic influence.

Beyond pollution levels, the ecological impact of heavy metals was assessed using the Potential Ecological Risk Index (PERI), offering a more risk-oriented perspective. The ecological risk factor (Eri), which incorporates the toxic response factor of each metal, was employed to calculate the PERI. The Eri and PERI results obtained in this study are presented in Table 5. The ecological risk factor assessment in sediment samples across the study sites indicated low risks levels for As, Pb, Cr and Ni (Eri< 40). In contrast, Cd exhibited very high risk at sites 1 and 2 (Eri ≥ 320), and high risk at sites 3 and 4 (160 ≤ Eri < 320). The potential ecological risk index (PERI), calculated as the sum of the individual risk factors (Eri) for each metal per site, ranged from 187.00 to 406.00. Sites 1 and 2 demonstrated high ecological risk (300 ≤ PERI < 600), while sites 3 and 4 showed moderate ecological risk (150 ≤ PERI < 300). Environmental concentrations of Tl and Co were measured but excluded from the PERI assessment due to a lack of established toxic response factors in literature (Hakanson, 1980; Isotuk et al., 2023). As noted by Ma and Han (2020), PERI values are often elevated by heavy metals with high toxic response factors, such as Cd (30) and Hg (40). In the present study, Cd was the primary contributor to the high PERI values observed. Chris and Davies (2024) reported higher PERI values in sediments (589.5–3190.3) from a study conducted at three stations in the coastal marine wetlands of Rivers State, Nigeria. As evidenced by the data in this study, the PERI values point to a moderate to high ecological risk, reflecting the cumulative impact of multiple heavy metals.

Table 3.  Calculated CF, EF and Igeo Values for Heavy Metals in Sediments from the Sampling Sites

Heavy                                  CF                                             EF                                          Igeo

Metals               S1	S2	S3	S4	   S1	    S2	   S3	S4	      S1        S2        S3         S4

Tl	         3.48	 –	–	–	   2.14       –           –          –               1.21      –          –            –

As	         0.38        1.72     0.50    1.99             0.23     1.37    0.61    1.51             −1.98   0.19   −1.58     0.40

Pb	         3.12        2.05     1.36    3.22             1.92     1.64    1.65      2.45              1.06     0.45   −0.14     1.10

Cd	         12.71      11.84   5.77    8.44             7.81     9.48    6.99    6.42                3.08    2.98     1.94      2.49

Cr	         1.13        1.12     0.40    0.62            0.70    0.89     0.48    0.47               −0.41   −0.42   −1.92   −1.27

Co	         2.41        1.36     0.74    2.38             1.48    1.09     0.89    1.81                0.69   −0.14   −1.02      0.67

Ni	         0.73        0.80     0.26    0.67             0.45    0.64     0.31    0.51              −1.05   −0.90   −2.54   −1.16

KEY: S1 = SITE 1, S2 = SITE 2, S3 = SITE 3 and S4 = SITE 4. CF = Contamination Factor, EF = Enrichment Factor and Igeo = Geo-accumulation index.

*Tl was below detection limits for S2 to S4 (Table S2)

Table 4: Pollution Load Index (PLI) Results for Study Sites

Site 1		Site 2		Site 3		Site 4

1.94		1.92		0.82		1.94

Table 5: Eri and PERI of Heavy Metals in Sediments across Sampling Sites

Heavy

Metals 	    Eri 1		Eri 2		Eri 3		Eri 4

Tl	      –		 –	              –	               –

As	    3.80             17.15                 5.00                  19.90

Pb	   15.60	           10.25	           6.80	           16.10

Cd	   381.30          355.20              173.10               253.20

Cr	    2.26              2.24	           0.80                   1.24

Co	      –		 –		 –	              –

Ni	    3.63	            4.00	           1.30                   3.35

PERI 	   406.60          388.84	           187.00	           293.79

KEY: Eri1 –  Eri4 denotes Ecological risk factors from Sites 1–4, respectively. *Eri values for Tl and Co were not available due to the absence of corresponding toxic response factors. PERI = Potential Ecological Risk Index

3.3 Transfer Factors of Heavy Metals from Water to Sediment Across Sites

Table 6 presents the transfer factor (TF) results of the analyzed heavy metals from water to sediment at the studied sites. The TF values provide insight into the mobility of the metals from the water. Generally, the detectable transfer factors exceeded 1 across the sampling sites, implying strong metal accumulation (Davies et al., 2021). While the transfer factors in the current study are comparatively higher, they align with Obadimu et al. (2025a), who documented TF values above 1 for heavy metals detected (Cr, Cu, Mn, Ni, and Fe) at Ikoneto in the Nwaniba River system. The non-detectable transfer factors at the observed sites stem from the fact that the metal concentrations were either below detection limits in the water or in the sediment. The higher the TF, the greater the contamination in water and sediment, which could pose a health risk if metals enter the food chain (Obadimu et al., 2025a).

Table 6.   Heavy Metals Transfer Factor from Water to Sediment

Heavy

Metals		Site 1		Site 2		Site 3		Site 4

Tl	           50.21	              ND                  ND                  ND

As                  116.69	              ND                  ND                  541.01

Pb	             ND	            1157.46          4851.25             ND

Cd                   ND	              ND                 ND                    ND

Cr                  16704.26           21395.32         791.04                ND

Ni	            1406.75            3138.51            462.66                ND

3.4. Correlation Analysis

The Pearson correlation analysis of heavy metal concentrations in sediment (Table 7 and Figure 4) along the Nwaniba River reveals distinct relationships among the analyzed metals (Simou et al., 2024a; Djibril et al., 2024; Afahnwie et al., 2025b, 2025c, 2025d). The correlation matrix demonstrates strong positive associations among Pb, Cd, Cr, Co, and Ni, with correlation coefficients ranging between 0.50 and 0.99. This suggests that these metals may originate from similar sources, possibly linked to anthropogenic inputs such as agricultural runoff, industrial discharge, or natural mineral weathering (Ubong et al., 2023b;

Obadimu et al., 2025a; Tiabou et al., 2025a, 2025b, 2025c). The near-perfect correlation between Cd−Cr and Co−Pb (r = 0.99) indicates they behave similarly in sediment deposition or adsorption processes. Moderate correlations were also observed between As and the other metals, reflecting its potential partial association with the same contamination sources. These correlations provide valuable insights into the geochemical behavior, transport mechanisms, and potential common origins of heavy metals within the Nwaniba River ecosystem (Yiika et al., 2022; 2024).

Table 7.   Correlation Data of Heavy Metals in Sediment

As           Pb         Cd           Cr            Co           Ni

As     1.00

Pb     0.27       1.00

Cd     0.05       0.52      1.00

Cr      0.04       0.41      0.99       1.00

Co     0.22       0.99      0.55        0.44         1.00

Ni      0.50       0.66      0.89       0.86          0.66         1.00

*Raw data used for this calculation are provided in the supplementary material (Table S2)

Fig 4. Pearson Correlation Heatmap for Heavy Metals in Sediment

4. Conclusion

This study demonstrates that the Nwaniba River is subject to varying degrees of heavy metal contamination, with evidence of both natural and anthropogenic inputs. Pollution indices revealed that while some sites exhibited low contamination (e.g., Site 3; PLI = 0.82), others experienced significant pollution and ecological risk (Sites 1, 2, and 4; PLI = 1.92–1.94 and PERI = 187.00–406.60), particularly due to cadmium. The high-risk potential of Cd underscores its threat to ecological health. The consistent pattern of transfer factors (TF > 1) observed raises concerns about trophic transfer and potential long-term ecological impacts. These findings highlight the urgent need for improved waste management, pollution control measures, and ongoing monitoring to prevent further degradation of the Nwaniba river system. Strengthening environmental regulations and promoting sustainable agricultural and industrial practices within the catchment are essential to protect aquatic life and human health.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

Reference

Afahnwie N.A, , Fomekong, B.K, Marian, C.S., Azeh, B., Bate Tibang, E.E.,  Nkwanyang, L.T., Anoh, A.O., Yiika, L.P., Fombalag, F.H., Djibril, K.N.G. (2025a). Geochemical and petrographic characterization of heavy mineral fractions in stream sediments from Akum area, Northwest Cameroon: implications for sources, weathering and mineral exploration. Journal of the Cameroon Academy of Sciences. DOI: https://dx.doi.org/10.4314/jcas.v22i2.6

Afahnwie, N.A., Embui, V.F., Yiika, L.P., Kouankap, N.G.D., Fomekong, B.K. (2025b). Preliminary stream sediment geochemical exploration for base metals and other elements in terms of source apportionment and contamination status of Manjo and environs, Cameroon. Discover Chemistry. 2:93 | https://doi.org/10.1007/s44371-025-00183-2

Afahnwiea, N.A., Yiika, L.P., Marian, C.S., Kachoueiyan, F., Gupta N., Kouankap, N.G.D., Birnila, M.C., Fomekong, B.K., Simou, A., Wochoko P. (2025c).  Geospatial distribution of hazardous metals and other elements in stream sediments of Akum, Cameroon: Insights into mineral exploration and ecosystem health using indexical and multivariate approaches. Marine Pollution Bulletine. https://doi.org/10.1016/j.marpolbul.2025.118761

Afahnwie, NA.,  Yiika, L.P., Ngatcha , R.B.,  Simou, A., Sanad, H., Njolai, M.,  Fombalang, F.H., Kehding, F.B.,  Wotchoko, P., Djibril, KNG.,  Njoh   OA. (2025d). Multivariate analysis and risk assessment of trace metal contamination in sediments of Mesue-Tonkwen, Cameroon: Implications for mineral exploration and ecosystem health. China Geology. doi: 10.31035/cg2025178

Aigberua A.O., & Tarawou, T. (2018) Ecological Risk Assessment of Selected Elements in Sediments from Communities of The River Nun, Bayelsa State, Nigeria. J Environ Bio Res. 2(1), 1-5

Algül, F., & Beyhan, M. (2020). Concentrations and sources of heavy metals in shallow sediments in Lake Bafa, Turkey Scientific Reports, 10, 11782. doi.org/10.1038/s41598-020-68833-2

Aljahdali, M. O., & Alhassan, A. B. (2020). Ecological risk assessment of heavy metal contamination in mangrove habitats using biochemical markers and pollution indices: A case study of Avicennia marina L, in Rabigh Lagoon, Red Sea. Saudi Journal of Biological Sciences, 27, 1174-1184. Retrieved March 20, 2022

Anyanwu, E. D., Davies, C. I. & Adetunji, O. G. (2023). Assessment of heavy metals in sediments and associated ecological risks in Ikwu river, Umuahia, Nigeria. Journal Environmental Problems, 8(3), 167–177. DOI: https://doi.org/10.23939/ep2023.03.167

Audu, Y., Aliyu, A. D., & John Dadi-Mamud, N. (2022). Evaluation of heavy metals contamination in the sediments of some selected water of south senatorial district of Niger State, Nigeria. Science World Journal, 17(4), 487–494.

Awakama, J. J., Udo, G. J., Uwanta, E. J., Igwe, R., Etukudo, N. J., & Akpanudo, N. W. (2024). Comparative Analysis of Heavy Metal Contamination in Regular and Artisanal Petroleum Products in Akwa Ibom State, South-South Nigeria. Communication in Physical Sciences, 11(3), 559 - 568.

Barakat, A., Ennaji, W., Krimissa, S., & Bouzaid, M. (2020). Heavy metal contamination and ecological-health risk evaluation in peri-urban wastewater-irrigated soils of Beni-Mellal city (Morocco). International Journal of Environmental Health Research, 30(4), 372-387.

Chris, D. I. & Anyanwu, B. O. (2023). Pollution and Potential Ecological Risk Evaluation Associated with Toxic Metals in an Impacted Mangrove Swamp in Niger Delta, Nigeria. Toxics 2023, 11,6. https:// doi.org/10.3390/toxics11010006

Chris, D. I., & Davies, I. I. (2024). Geo-Ecological Risk Assessment of Heavy Metals in Sediment and Water from Coastal Marine Wetland in Rivers State, Nigeria. Pollution, 10 (4), 1103-1116. https://doi.org/10.22059/poll.2024.373993.2287

Cui, S., Zhang, F., Hu, P., Hough, R., Fu, Q., Zhang, Z., et al. (2019). Heavy Metals in Sediment from the Urban and Rural Rivers in Harbin City, Northeast China. International Journal of Environmental Research and Public Health, 16, 4313. https:// doi.org/10.3390/ijerph16224313

Davies, I. C., Agarin, O. J., & Onoja C. R. (2021). Study On Heavy Metals Levels and Some Physicochemical Parameters of a Polluted Creek Along the Tin Can Island in Lagos. International Journal of Environment and Pollution Research. 9(2), 25-39.

Djibril, K. N. G., Yiika, L. P., Etutu, M. E. M. M., Eric, B. E., Mengu, E. E., Jean-Lavenir, N. M., & Armstrong-Altrin, J. S. (2024). Geochemistry of sediments from the Mugheb River, Bamenda, Cameroon Volcanic Line: implications for provenance, paleoweathering and tectonic setting. Arabian Journal of Geosciences, 17(12), 331. https://doi.org/10.1007/s12517-024-12148-3

Fennel, K., Gehlen, M., Brasseur, P., Brown, C. W., Ciavatta, S., Cossarini, G., et al. (2019). and the GODAE Ocean View Marine Ecosystem Analysis and Prediction Task Team. Advancing Marine Biogeochemical and Ecosystem Re-analyses and Forecasts as Tools for Monitoring and managing Ecosystem Health – A review. Frontiers in Marine science, 8, Pp. 3-11

Hakanson, L. L. (1980). An ecological risk index aquatic pollution control, a sedimentological approach. Water Research, 14 (8), 975–1001.

Hamuna, B. & Wanimbo, E. (2021). Heavy Metal Contamination in Sediments and Its Potential Ecological Risks in Youtefa Bay, Papua Province, Indonesia. Journal of Ecological Engineering, 22(8), 209–222. doi.org/10.12911/22998993/139116.

Islam, M. S., Ahmed, M. K., Raknuzzaman, M., Habibullah, M., Al Mamum, M. H. & Islam, M. K. (2015). Heavy metal pollution in surface water and sediment: A preliminary assessment of an urban river in a developing country. Ecological Indicator, 48, 282-291.

Islam, M.S., Proshad, R. & Ahmed, S. (2017). Ecological risk of heavy metals in sediment of   an urban	river	in Bangladesh. Human and Ecological Risk Assessment: An International Journal. doi.org/10.1080/10807039.2017.1397 499

Isotuk, U. G., Etesin, U. M., Nsi, E. W., & Ukpong, E. J. (2023). Ecological and health risk assessment of heavy metals in sediments, surface waters and oysters (Crassostrea gasar) from Eastern Obolo marine ecosystems, Akwa Ibom State, Nigeria. Communication in Physical Sciences, 9(4), 545-571.

Jean-Lavenir, N. M., Cyrille, S., Omar, E. D., Yiika, L. P., & Junior, E. E. M., (2025). Geochemical and Gold-Ore Potential Assessment in Stream Sediments of Bindiba Gold District, Eastern Cameroon: Implications for Gold Exploration, Sediment Provenance, Paleoenvironment, and Tectonic Setting. Mining, Metallurgy & Exploration, 42(4), 2415–2439. https://doi.org/10.1007/s42461-025-01270-9

Jean-Lavenir, N. M., Kiki, T. N., Yiika, L. P., & Ndi, G. M. (2023). Contamination, sources and risk assessments of metals in stream sediments of Pouma area, Pan-African Fold Belt, Southern Cameroon. Water, Air, & Soil Pollution, 234(3), 160. https://doi.org/10.1007/s11270-023-06180-4

Jean-Lavenir, N. M., Sigué, C., Igor Fulbert, M.N., Nkongho, A.A.O., Yiika, L.P., Nga Essomba, T.P.E., Eseya Mengu, E.J. (2024). Potentially toxic metals contamination in stream sediments of Mbal area (Pan African fold belt, Cameroon). Discover  Geoscience 2:15. https://doi.org/10.1007/s44288-024-00017-1

Jean-Lavenir, N. M. J., Junior, E. E. M., & Monesperance, M. M. (2020) Assessment of vanadium in Stream Sediments from River Mbete, Loum Area (Pan-African Fold Belt, Cameroon): Implications for vanadium exploration. International Journal of innovative Science and Research Technology, 5, 1.

Kachoueiyan, F., Alipour Atmianlu, P., Rajabi, A., Ustaoğlu, F., Karbassi A. (2024a). Contamination source, health and ecological risk assessment of hazardous elements in the sediment of the Shahid Rajaee reservoir. Front Environmental Science.  12:1493547. doi: 10.3389/fenvs.2024.1493547

Kachoueiyan, F., Karbassi, A., Nasrabadi, T., Rashidiyan, M., De-la-Torre, G.E., (2024b). Speciation characteristics, ecological risk assessment, and source apportionment of heavy metals in the surface sediments of the Gomishan wetland. Marine Pollution Bulletin 198:115835.https://doi.org/https://doi.org/10.1016/j.marpolbul.2023.115835

Kolawole, T. O., Olatunji, A. S., Jimoh, M. T. & Fajemila, O. T. (2018). Heavy Metal Contamination and Ecological Risk Assessment in Soils and Sediments of an Industrial Area in South-Western Nigeria. Journal of Health and Pollution, 8(19): 180906

Ma, L., & Han, C. (2020) Water quality ecological risk assessment with sedimentological approach. In: Summers, K. (Ed). Water quality Science, Assessments and Policy. Intechopen.

McMaster, S. A., Noller, B. N., Humphrey, C. L., Trenfield, M. A & Hayford, A. J. (2020). Speciation and Partitioning of Uranium in Water bodies near Ranger Uranium Mine. Environ. Chemistry, 18(1), 12-19.

Moses, E. A., Akpan, D. J., & Shaibu, S. E. (2022). Risk Assessment of Pesticide Residues in Water Samples from Nwaniba River, Akwa Ibom State, Nigeria. Journal of Chemical Society of Nigeria, 47(6): 1321-1334

Muller, G. (1969).  Index of geo-accumulation in sediments of Rhine River, Geochemical Journal, 2: 108 – 118.

Obadimu, C. O., Shaibu, S. E., Ekwere, I. O., & Adelagun, R. O. A. (2024). Machine Learning-Based Forecasting of Bioaccumulation and Histopathological Effects in Aquatic Organisms. FUDMA Journal of Sciences (FJS), 8(6), 485 – 496. https://doi.org/10.33003/fjs-2024-0806-3002

Obadimu, C. O., Ekwere, I. O., Shaibu, S. E., Ben, U. U., Nnamso, D. I, Adelagun, R. O. A et al. (2025a). “Heavy metal distribution in Water, Sediment, and some benthic organisms in Nwaniba River of Akwaibom State”. International Journal of Development Research, 15, (04), 68134-68140.

Obadimu, C. O., Ekwere, I. O., Shaibu, S. E., Essien, U. B., Adelagun, R. O. A. & Adewusi, S. G. (2025b). Application of Machine Learning Framework on Heavy Metals Fate in the Coastal Environment. International Journal of Advanced Research in Chemical Science, 12(1), 21 -29. https://doi.org/10.20431/2349-0403.1201003.

Obadimu, C. O., Ekwere, I. O., Shaibu, S. E., & Adewusi, S. G. (2025c). “Heavy Metal Contamination in Nwaniba River: A Machine learning-driven Ecosystem Analysis”, Asian Journal of Science and Technology, 16, (05),13675 – 13681.

Okibe, F.G., Yahaya, I. A., Onoyima, C.  C. & Afolayan, M.O. (2020). Application of Multivariate Statistical Methods for Assessment of Sediment Quality in Selected Locations of the Flood Plain of River Kaduna in Niger State, Nigeria. The Pacific Journal of Science and Technology, 21(2), 360-370.

Okoye, E.A., Ezejiofor, A.N., Nwaogazie, I.L., Frazzoli, C., & Orisakwe, O. E. (2022). Heavy metals and arsenic in soil and vegetation of Niger Delta, Nigeria: Ecological risk assessment. Case Stud. Chem. Environ. Eng. 6, 100222.

Olayinka-Olagunju, J. O. (2021). Analysis And Potential Ecological Risk Assessment of Heavy Metals in Surface Water and Sediments of Owena River Ondo State. FUTA Journal of Life Sciences, 1(1), 38 – 52.

Onoyima, C. C. (2021). Assessment of pollution levels and risk of some heavy metals in soil and Mangifera indica around Nigeria Police Academy, Wudil, Kano State, Nigeria. Science World Journal, 16(3), 325-332.

Onoyima, C. C., Okibe, F. G., Ogah, E., & Dallatu, Y. A. (2021). Heavy metal pollution and ecological risk assessment in the sediments of river Kaduna, Nigeria. Journal of Research in Forestry, Wildlife and Environment, 13(4), 205-214.

Sanad, H., Moussadek, R., Mouhir, L., Lhaj, M.O., Zahidi, K., Dakak, H., Manhou, K., Zouahri, A. (2025a). Ecological and human health hazards evaluation of toxic metal contamination in agricultural lands using multi-index and geostatistical techniques across the mnasra area of Morocco’s gharb plain region. Journal of Hazardous Materials Advances, 18, 100724. doi: 10.1016/j.hazadv.2025.100724.

Sanad, H., Moussadek, R., Mouhir, L., Lhaj. M.O., Dakak, H., Zouahri, A. (2025b). Geospatial analysis of trace metal pollution and ecological risks in river sediments from agrochemical sources in Morocco’s Sebou basin. Scientific Reports, 15, 16701. doi: 10.1038/s41598-025-01199-5.

Sharma, R., Kumar, A., Singh, N., & Sharma, K. (2021). Impact of seasonal variation on water quality of Hindon River. Physicochemical and biological analysis. SN Appl. Sci, 3, 28.

Sigue, C., Yewong, N.E., Yiika, L.P., Tiabou, A.F., Simou, A., Liyouck, P.R., Ndema, M.J.L., Yen, N.V. (2025). Assessemnt of contamination, sources and health risks of potentially hazardous elements in surface sediment of Dibang, Cameroon. Discover Environment. 3:165 | https://doi.org/10.1007/s44274-025-00363-y

Simou, A., Mrabet, A., Abdelfattah, B., Bougrine, O., Khaddor, M., & Allali, N., (2024b). Distribution, ecological, and health risk assessment of trace elements in the surface seawater along the Littoral of Tangier Bay (Southwestern Mediterranean Sea). Marine Pollution Bulletin, 202, 116362. https://doi.org/10.1016/j.marpolbul.2024.116362

Simou, A., Sarti, O., Abdelfattah, B., Mrabet, A., Khaddor, M., Allali, N., (2024c). Assessing ecological and health risks of potentially toxic elements in marine and beach sediments of Tangier Bay, Southwestern Mediterranean sea. Marine Pollution Bulletin, 209, 117234. https://doi.org/10.1016/j.marpolbul.2024.117234

Simou, A.., Mansouri, F.E., Abdelfattah, B., Mrabet, A., Khaddor, M., Allali, N., (2024a). Assessment of ecological risk and metal contamination caused by the Lihoud River emissary along the Bay of Tangier Littoral in Morocco (Southwestern Mediterranean Sea). Environmental Quality Mgmt tqem. 22269. https://doi.org/10.1002/ tqem.22269

Singh, B.P., Choudhury, M., Samanta, P., Gaur, M., and Kumar, M. (2021). Ecological Risk Assessment of Heavy Metals in Adjoining Sediment of River Ecosystem. Sustainability, 13, 10330. https://doi.org/10.3390/ su131810330

Tiabou, A.F., Atabe, G.A.A., Sigue, C., Yiika, L.P., Kachoueiyan, F., Forchenallah, E.F., 2024b. Appraisal of pollution, ecological and health risk assessment of trace metals in soils of Logbadjeck quarrying area, Nyong Series, Southern Cameroon. Journal of Trace Elements and Minerals 10:100204. https://doi.org/10.1016/j.jtemin.2024.100204

Tiabou, A.F., Esukac, C.N., Mbowou Ngantche, I.F., Yiika, L.P., Wadgua, B.P., Agyingi, C.M., (2025a). Spatial distribution, contamination characteristics and eco-risk assessment of toxic metals in stream sediments of Njombe-Penja Banana plain, Cameroon Volcanic Line. Chemistry Africa.8, 619-639. https://doi.org/10.1007/s42250-024-01159-y,

Tiabou, A.F., Manka’a, F.C., Yiika, L.P., Mengu, E.E. (2025c). Potentially toxic metals contamination, pollution and ecological risks assessment in volcanic soils of Njombe-Penja Banana Plain, Cameroon Volcanic Line. Arabian Journal of Geosciences. 18:180. https://doi.org/10.1007/s12517-025-12317-y

Tiabou, A.F., Mboudou, G.M.M., Ghanyuymo, M.M., Yiika, L.P., Forchenallah, N.E., (2025b). Evaluation of surface and groundwater quality in Logbadjeck quarrying area: implications for trace metals pollution and health risk assessment. International Journal of Energy and Water Resources. https://doi.org/10.1007/s42108-024-00321-z

Tiabou, A.F., Takem-Agbor, A.A., Yiika, L.P., Eseya Mengu, E., Kachoueiyan, F., Agyingi, CM., (2024a). Distribution, source apportionment and ecological risk assessment of heavy metals in Limbe River sediments, Atlantic Coast, Cameroon Volcanic Line. Discover Water 4:62. https://doi.org/10.1007/s43832-024-00120-x

Tiabou. A.F., Tanyi, T.A.M., Yiika, L.P., Ayuk, M.M.A., (2024c). Spatial distribution, ecological and ecotoxicity evaluation of heavy metals in agricultural soils along Lala-Manjo Highway, Cameroon Volcanic Line. Discover Soil 1:7. https://doi.org/10.1007/s44378-024-00010-7

Turekian, K. K., & Wedepohl, K. H. (1961). Distribution of the elements in some major units of the earth’s crust. Geological Society of America Bulletin, 72, 175-192. Retrieved November 3, 2022.

Ubong, U. U., Ekwere, I. O. & Obadimu, C. O. (2023a). Human health Risk Assessment. A case study of Heavy Metals Accumulation in Tissues of Callinectus latimanus from Iko River, Eastern Obolo L. G. A., Akwa Ibom State. Researchers Journal of Science and Technology, 3(1), 29-41.

Ubong, U. U., Ekwere, I. O., Ikpe, E. E. & Obadimu, C. O. (2021). Assessment of Physicochemical Properties and Water Quality index of borehole water in Mkpat Enin Local Government Area, Akwa Ibom State, International Journal of Chemistry Studies, 5(2), 49 - 58

Ubong, U. U., Ekwere, I. O., Ikpe, E. E. & Obadimu, C. O. (2023b). Evaluation of Heavy Metals and Total Hydrocarbon Contents in CrassostreaSpp (Oysters) from Qua Iboe River, Ibeno LGA American Journal of Applied and Industrial Chemistry, 7 (1), 17 – 24

USEPA (2023). EPA Expobox: Exposure assessment tools by media-aquatic biota. United States Environmental Protection Agency.

Varol, M. (2011). Assessment of heavy metal contamination in sediments of the Tigris River (Turkey) using pollution indices and multivariate statistical techniques. Journal of Hazardous Materials, 195, 355-364

WHO (World Health Organisation) (2010).  Human health risk assessment toolkits: Chemical Hazard:	WHO/IPCS Harmonization project document, No 8.

Xie, Z., Zhu, G., Xu, M., Zhang, H., Yi, W., Jiang, Y., et al. (2022). Risk assessment of heavy metals in a typical mangrove ecosystem - A case study of Shankou Mangrove National Natural Reserve, southern China. Marine Pollution Bulletin, 178, 113642.

Yawo, O. J. & Akpan, I. O. (2021). Assessments of heavy metal concentration in sediments of Utibete River, Eastern Obolo, Southeastern Nigeria using particle induced X-ray emission (PIXE) technique. Researchers Journal of Science and Technology, 1, 17-36.

Yiika LP., Meniemoh AR., Mengu EE., Berinyuy CT., Kouankap G.D., (2025). Geochemistry and source area weathering of soils around Mount Bamboutos (Cameroon Volcanic Line). Advances in Analytic Science. 6(1): 3670. https://doi.org/10.54517/aas3670

Yiika, L.P., Ndema Mbongué, J.L., Mboudou, G.M.M., Eseya Mengu E., Bewah, E.B., (2022). Contamination and Risk Assessment of Heavy Metals in Stream Sediments of Bambui Area, western Cameroon, . International Journal of Research and Innovation in Applied Science.  7(11):64–75

Yiika, L.P., Ndema Mbongué, J.L., Suh, G.C., Nkemasong, J.A., Eseya Mengu, E.J., Fomekong, B.K., Sekem, S.G., Kouankap, G.D., (2024). Distribution, Sources, and Eco-toxicological Assessment of Potentially Toxic Metals in River Sediments of Nkwen Area (Cameroon Volcanic Line). Water Air Soil Pollution. 235-16. https://doi.org/10.1007/s11270-023-06830-7

Yiika, L.P., Tita, M.A., Suh, C.S., Mimba, M.E., Ndema Mbongué, J.L., (2023). Heavy Metal Speciation by Tessier Sequential Extraction Applied to Artisanal Gold Mine Tailings in Eastern Cameroon. Chemistry Africa 6:2705–2723. https://doi.org/10.1007/s42250-023-00652-0

Table S1.   Heavy metals concentration (mg/l) in water along Nwaniba River

Sampling	Tl           As            Pb        	Cd           Cr               Co	      Ni

Site

1	     0.0970	0.0423       ND	              ND          0.061         ND	     0.0351

2	       ND		ND	   0.0354	 ND        0.0047         ND	     0.0174

3	       ND		ND	   0.0056	 ND        0.0452         ND	     0.0379

4	       ND		0.0477	      ND		 ND	    ND          0.0442	       ND

ND = Not Detected

Table S2.  Heavy metal concentrations (mg/kg) in sediment along Nwaniba River and Reference Shale values

Sampling    	Tl	     As		Pb		Cd	          Cr	       Co 	Ni

Site

1	          4.870	     4.936      	62.414      	3.813	         101.896         45.847      49.377

2                	ND	     22.298    	40.974       	3.552	         100.558         25.795      54.610

3	            ND	     6.525     	27.167       	1.732	          35.755          14.026      17.535

4	            ND	     25.806   	64.302     	2.532	          55.853           45.191     45.597

shales*           1.4                13          	         20                   0.3                    90                 19             68

*source: Turekian and Wedepohl (1961)