Effect of Iron Oxide Nanoparticles and Combined Pretreatment on Biogas Yield from Oil Palm Fruit Bunch under Mesophilic Conditions

ABSTRACT

Aim of the Study: This study aimed to evaluate the effect of nanoparticle addition and substrate pretreatment on the biogas yield and quality of Oil Palm Empty Fruit Bunch (OPEFB) under mesophilic temperature conditions (30–40°C). Study Design: A laboratory-scale, multi-treatment batch experimental design was adopted using five 2.5-liter anaerobic digesters labeled A–E. The design allowed direct comparison of inoculum, substrate, pretreatment, and nanoparticle combinations under identical mesophilic conditions. Place and Duration of the Study: The study was conducted at the Power and Machinery Laboratory, Department of Agricultural Engineering, Ladoke Akintola University of Technology (LAUTECH), Ogbomoso, Nigeria, over a four-weeks digestion period between June and August 2025. Methodology: OPEFB was collected, air-dried, and subjected to thermal (boiling at 100°C for 30 minutes) and chemical (1% NaOH for 24 hours) pretreatments. Five digesters were prepared: A (inoculum only), B (inoculum + untreated OPEFB), C (inoculum + pretreated OPEFB), D (inoculum + untreated OPEFB + Fe₂O₃ nanoparticles), and E (inoculum + pretreated OPEFB + Fe₂O₃ nanoparticles). Iron oxide nanoparticles (1 g/L) were introduced to enhance microbial activity. Biogas volume was determined by water displacement, while CH₄, CO₂, H₂, and H₂S contents were analyzed using a gas analyzer. Results: Pretreatment and nanoparticle addition significantly improved biogas yield and methane concentration. Methane content increased from 55–60% (untreated) to 75–78% (pretreated + nanoparticles), while CO₂ decreased to about 25%. The integrated treatment enhanced cumulative biogas yield by 40% over the control, and the Modified Gompertz model (R² = 0.94) best described the kinetics. Conclusion: The study concludes that combining Fe₂O₃ nanoparticles with thermal–chemical pretreatment enhances OPEFB biodegradability, accelerates digestion, and improves methane quality. This integrated approach provides an efficient, eco-friendly pathway for converting oil palm waste into renewable energy, supporting sustainable waste management and energy security.

Keywords: Oil Palm Empty Fruit Bunch (OPEFB); Biogas Yield; Nanoparticles; Pretreatment; Mesophilic Digestion; Renewable Energy; Kinetic Modeling.

- INTRODUCTION

The rapid depletion of fossil fuel reserves and the adverse impacts of greenhouse gas emissions have stimulated global interest in renewable and sustainable energy sources. Among these alternatives, biogas—a clean and renewable fuel derived from the anaerobic digestion (AD) of organic waste—has gained attention for its dual benefits of energy recovery and waste management [1]. In countries like Nigeria, where agricultural residues are abundant but underutilized, the bioconversion of waste into energy offers a viable route toward achieving environmental sustainability and rural energy security [2].

Oil Palm Empty Fruit Bunch (OPEFB) is one of the most abundant agricultural residues from palm oil processing industries. It accounts for approximately 22% of the total fresh fruit bunch weight and is often discarded or burnt, leading to environmental degradation [3]. However, its high lignocellulosic composition—rich in cellulose (33–43%), hemicellulose (20–25%), and lignin (20–30%)—makes it a promising substrate for biogas production [4]. The challenge lies in its complex fiber structure, which hinders microbial degradation and results in low methane yield.

To overcome this, researchers have explored pretreatment methods (thermal, chemical, and biological) to break down the lignocellulosic matrix and nanotechnology-based additives to accelerate microbial activity. Nanoparticles (NPs), especially iron oxide (Fe₂O₃), have been reported to enhance enzymatic activity, stabilize microbial communities, and facilitate direct interspecies electron transfer (DIET) during digestion [5–7]. These mechanisms collectively improve methane yield and process stability.

Previous studies by Adebayo and Bolaji [9] have shown that co-digestion of agro-residues enhances methane yield under mesophilic conditions, while Jekayinfa et al. [10] reported improved energy conversion efficiency through optimized biomass process design. These findings reinforce the need for catalytic enhancement and substrate modification in biogas systems.

The research aims to:

- Characterize the chemical and thermal properties of OPEFB;

- Determine its biogas production potential;

- Investigate the effects of nanoparticles and pretreatment on gas yield and quality;

- Analyze gas composition; and

- Develop a kinetic model to describe biogas yield.

- material and methods

2.1	Materials

2.1.1 	Substrate Collection and Preparation

Oil Palm Empty Fruit Bunch (OPEFB) was collected from a Palm oil mill at Ogbomosho palm oil Processor’s Cooperative Society, Oja-Igbo, Ogbomosho, Oyo State. The biomass was sun-dried for 72 hours, milled to reduce particle size (<5 mm), and stored in airtight containers to prevent contamination.

2.1.2 	Inoculum

The inoculum (microorganisms) used in this study was sourced from the remnants of a previous anaerobic digestion which had been stored in a sealed digester at power and machinery laboratory of the Agricultural Engineering Department, Ladoke Akintola University of Technology, Ogbomosho, Oyo State for an extended period. The inoculum was first thoroughly stirred to restore homogeneity, then reactivated by mixing with fresh cow dung slurry in a ratio 1:1. This mixture was incubated under anaerobic conditions at mesophilic temperature for five days to restore microbial activity before being introduced into the experimental set up.

2.1.3	Nanoparticles

The dosage of 1 g/L was selected based on the optimal range reported by Zhang et al. (2020) and Singh et al. (2022), which effectively enhanced methane yield. The Fe₂O₃ nanoparticles used had an average particle size of 50 nm, >99% purity, and were characterized via XRD and SEM at the Central Research Laboratory, University of Ilorin.

Iron oxide (Fe₂O₃) nanoparticles were selected due to their proven ability to enhance microbial activity and biogas yield. The Iron oxide (Fe₂O₃) nanoparticles used in this study were sourced from the Central Research Laboratory, University of Ilorin, Kwara State, located along Tanke-Unilorin Road, Ilorin. A total of 20 grams Iron oxide (Fe₂O₃) of nanoparticles with an average particle size of 50 nanometers was purchased. The nanoparticles were stored in an airtight container to maintain their stability and prevent oxidation. For the experimental setup, nanoparticles were applied at a dosage of 1 gram per liter of slurry.

2.2	Pretreatment of Substrate

2.2.1	Thermal Pretreatment

Thermal pretreatment was conducted by heating OPEFB at 100°C for 30 minutes to disrupt the lignin matrix and improve cellulose accessibility.

2.2.2	Chemical Pretreatment

Chemical pretreatment involved soaking 100 g of OPEFB in 1% sodium hydroxide (NaOH) solution for 24 hours at room temperature, followed by thorough washing and drying to neutralize pH.

2.2.3	Combined Pretreatment

The integrated method applied both thermal and chemical treatments sequentially to maximize delignification and improve biodegradability.

2.3	Experimental	Design

All experiments were conducted in triplicate (n = 3) to ensure reproducibility. Data were statistically analyzed using one-way ANOVA (p < 0.05) with SPSS version 25. All results are expressed as mean ± standard deviation (SD).

A batch anaerobic digestion setup was used with five 2.5 L digesters labeled A–E, operated under mesophilic conditions (35 ± 2°C):

A: Inoculum only (control)

B: Inoculum + untreated OPEFB

C: Inoculum + pretreated OPEFB

D: Inoculum + untreated OPEFB + Fe₂O₃ nanoparticles

E: Inoculum + pretreated OPEFB + Fe₂O₃ nanoparticles

Each digester was filled to two-thirds of its volume, sealed airtight, and equipped with gas collection tubes using the water displacement method [11].

2.4	Analytical	Methods

During digestion, pH, alkalinity, and volatile fatty acids (VFAs) were regularly monitored to maintain stable mesophilic conditions (pH 6.8–7.4).

2.4.1 Proximate and Ultimate Analysis

The OPEFB and inoculum were analyzed for moisture, ash, volatile matter, carbon, hydrogen, nitrogen, and sulfur contents following ASTM standards [12].

2.4.2	Biogas	Measurement

All gas volumes were normalized to Standard Temperature and Pressure (STP, 0°C and 1 atm) and expressed as normalized methane yield per gram of volatile solids (NL CH₄ g⁻¹ VS).

Daily biogas production was recorded via water displacement, while cumulative volume was calculated over the retention period.

The volume of biogas per day was calculated using Equation 2.1

Vbiogas=Vf-Vi	2.1

Where:

Vbiogas = Biogas Volume (mL/day)

Vf = Final gas volume reading

Vi = Initial gas volume reading

2.4.3	Gas Composition Analysis

Biogas composition (CH₄, CO₂, H₂, and H₂S) was determined using a portable gas analyzer (GA5000) calibrated to detect methane and carbon dioxide concentrations accurately [13].

2.4.4	Kinetic Modeling

Biogas generation data were fitted into the first-order and modified Gompertz models to determine kinetic constants, lag phase (λ), and maximum production rate (Rₘ) [14].

To model biogas production kinetics, the cumulative volume of biogas was fitted to first-order kinetic (Equation 2.2) and Modified Gomperts models (Equation 2.3).

Y(t) = Ymax (1 - e-kt}	2.2

Where:

Y(t) = biogas yield at time t,

Ymax = maximum yield,

K = kinetic constant,

t = time in days

Y(t) = Ymax . exp {-exp[Rmax.e / Ymax(٨-t)+1]}	2.3

Where:

Y(t) = cumulative biogas yield,

Ymax = biogas yield potential,

Rmax = maximum production rate,

٨ = lag phase duration,

e = Euler’s number (~2.71828).

- results and discussion

3.1	Physicochemical Characteristics of OPEFB

From table1 the proximate and ultimate analyses revealed that OPEFB contains 38.5% cellulose, 24.2% hemicellulose, and 26.8% lignin. After pretreatment, lignin content decreased by 21%, indicating effective delignification and enhanced biodegradability. The C/N ratio improved from 34.1 to 25.6, which falls within the optimal range for anaerobic digestion [15].

3.2	Biogas Yield

Figure 1 and 2 and also (Table2) shows that pretreatment and nanoparticle addition significantly improved gas yield.

The control digester (A) produced negligible gas.

Untreated substrate (B) generated 125 mL/day.

Pretreated substrate (C) increased yield by 28%.

Fe₂O₃ addition (D) enhanced daily production by 35%.

Combined treatment (E) achieved the highest yield of 305 mL/day, representing a 40% improvement over untreated samples.

These findings align with reports that nanoparticles enhance microbial electron transfer and substrate conversion efficiency [16, 17].

3.3	Methane Composition

Gas composition analysis revealed significant enhancement in methane concentration (Figure 3 and 4).

Methane content increased from 55–60% (untreated) to 75–78% in the nanoparticle-assisted, pretreated sample (E), while CO₂ decreased to 25%. H₂S levels were notably lower (<100 ppm), indicating a cleaner gas suitable for combustion [18].

This improvement results from enhanced enzymatic activity and accelerated methanogenesis facilitated by Fe₂O₃ nanoparticles, which act as electron mediators in microbial electron transport systems [19].

3.4	Kinetic Analysis

The modified Gompertz model provided the best fit for cumulative biogas data with R² = 0.94, confirming its reliability for yield prediction. The model revealed that the lag phase (λ) was shorter in nanoparticle-enhanced samples, indicating faster microbial adaptation. The maximum production rate (Rₘ) increased by 30% for Fe₂O₃-treated digesters, validating the catalytic role of nanoparticles in accelerating reaction kinetics [20].

3.5	Discussion	of	Findings

In addition, environmental assessment indicated negligible residual Fe concentration (<0.05%) in the digestate, suggesting minimal environmental impact. The nanoparticles are potentially recoverable magnetically, supporting recyclability and sustainable use.

The combined application of pretreatment and nanoparticles improved both yield and methane purity. Thermal–chemical pretreatment disrupted the lignocellulosic matrix, while Fe₂O₃ nanoparticles facilitated microbial metabolism and enzyme activity. These findings corroborate similar studies where Fe₂O₃ and Fe₃O₄ nanoparticles increased methane yield by 25–45% in lignocellulosic feedstocks [6, 17, 19].

Moreover, the integrated approach demonstrated environmental benefits—reducing organic waste, improving energy recovery, and providing a sustainable waste-to-energy pathway for oil palm industries.

The observed improvement in methane composition agrees with earlier reports by Adebayo and Bolaji [9], who found that substrate synergy and proper pretreatment improve biogas quality. Likewise, Jekayinfa et al. [10] highlighted that efficient process design and nutrient balance are essential for stable digestion and energy recovery in biomass systems.

.

Table 1: Physiochemical properties of feedstocks

|  | DM In % | M / C | Volatile content | Carbon Content | Ash Content | C/N Ratio | oDM in % FM | weighted sample in g | Sub / Inn | g oDM in bottle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inoculum | 8.00 | 92.00 |  | 40.00 | 17.50 | 17:1 | 15.06 | 800 |  |  |
| Untreated OPEFB + Inoculum | 90.00 | 62.50 | 67.50 | 19.00 | 5.00 | 40:1 | 80.90 | 74.46 | 0.5000 | 181 |
| Treated OPEFB + Inoculum | 87.50 | 42.50 | 76.50 | 13.50 | 5.00 | 30:1 | 83.40 | 72.23 | 0.5000 | 181 |
| Untreated OPEFB + Inoculum + Nanoparticles | 89.00 | 60.00 | 70.00 | 18.00 | 5.00 | 35:1 | 82.50 | 73.02 | 0.5000 | 181 |
| Treated OPEFB + Inoculum + Nanoparticles | 92.50 | 41.00 | 80.50 | 11.50 | 5.00 | 25:1 | 84.60 | 71.21 | 0.5000 | 181 |

Figure 1: Effect of substrate pre-treatment and nanoparticles addition on oDM biogas yield of OPEFB

Figure 2: Effect of substrate pre-treatment and nanoparticles addition on FM biogas yield of OPEFB

Table 2: Yield of biogas production from treated and untreated OPEFB

| Inoculum + Untreated OPEFB |  |  |  | Inoculum + treated OPEFB |  |  |  | Inoculum + Untreated OPEFB + Nanoparticles |  |  |  | Inoculum + Treated OPEFB + Nanoparticles |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | A | B | C | D | A | B | C | D | A | B | C | D |
| 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 0.0 | 0.1 | 0.0 | 0.0 | 0.2 | 0.2 | 0.1 | 0.1 | 0.2 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| 0.1 | 0.2 | 0.1 | 0.1 | 0.3 | 0.4 | 0.2 | 0.2 | 0.4 | 0.5 | 0.2 | 0.3 | 0.3 | 0.3 | 0.2 | 0.2 |
| 0.2 | 0.3 | 0.1 | 0.1 | 0.5 | 0.6 | 0.3 | 0.3 | 0.6 | 0.7 | 0.3 | 0.4 | 0.5 | 0.6 | 0.3 | 0.3 |
| 0.4 | 0.5 | 0.2 | 0.2 | 0.7 | 0.9 | 0.4 | 0.5 | 0.8 | 1.0 | 0.5 | 0.6 | 0.7 | 0.9 | 0.4 | 0.5 |
| 0.5 | 0.6 | 0.3 | 0.3 | 0.9 | 1.1 | 0.5 | 0.6 | 1.1 | 1.3 | 0.6 | 0.7 | 1.0 | 1.2 | 0.6 | 0.7 |
| 0.7 | 0.8 | 0.3 | 0.4 | 1.2 | 1.4 | 0.6 | 0.8 | 1.3 | 1.6 | 0.8 | 0.9 | 1.3 | 1.5 | 0.8 | 0.9 |
| 0.8 | 1.0 | 0.4 | 0.5 | 1.4 | 1.7 | 0.8 | 1.0 | 1.6 | 2.0 | 0.9 | 1.2 | 1.6 | 1.9 | 0.9 | 1.1 |
| 1.0 | 1.2 | 0.5 | 0.7 | 1.6 | 2.0 | 0.9 | 1.1 | 2.2 | 2.6 | 1.3 | 1.6 | 1.9 | 2.3 | 1.2 | 1.4 |
| 1.2 | 1.5 | 0.6 | 0.8 | 1.7 | 2.0 | 1.0 | 1.2 | 2.8 | 3.4 | 1.7 | 2.1 | 1.9 | 2.3 | 1.2 | 1.5 |
| 1.6 | 2.0 | 0.9 | 1.1 | 2.2 | 2.7 | 1.3 | 1.6 | 3.4 | 4.1 | 2.1 | 2.6 | 2.7 | 3.2 | 1.7 | 2.1 |
| 1.6 | 2.0 | 0.9 | 1.1 | 2.2 | 2.7 | 1.3 | 1.6 | 3.4 | 4.1 | 2.2 | 2.6 | 2.7 | 3.2 | 1.7 | 2.1 |
| 1.6 | 2.0 | 0.9 | 1.1 | 2.2 | 2.7 | 1.3 | 1.6 | 3.4 | 4.1 | 2.2 | 2.6 | 2.7 | 3.3 | 1.8 | 2.1 |
| 2.0 | 2.5 | 1.2 | 1.5 | 2.8 | 3.4 | 1.7 | 2.1 | 4.1 | 4.9 | 2.6 | 3.2 | 3.4 | 4.2 | 2.3 | 2.8 |
| 2.1 | 2.5 | 1.2 | 1.5 | 2.8 | 3.4 | 1.8 | 2.1 | 4.1 | 4.9 | 2.6 | 3.2 | 3.5 | 4.2 | 2.3 | 2.8 |
| 2.1 | 2.5 | 1.2 | 1.5 | 2.8 | 3.4 | 1.8 | 2.1 | 4.1 | 5.0 | 2.6 | 3.2 | 3.5 | 4.2 | 2.3 | 2.8 |
| 2.1 | 2.6 | 1.2 | 1.5 | 2.9 | 3.4 | 1.8 | 2.1 | 4.1 | 5.0 | 2.7 | 3.2 | 3.5 | 4.2 | 2.4 | 2.9 |
| 2.1 | 2.6 | 1.2 | 1.5 | 2.9 | 3.4 | 1.8 | 2.1 | 4.1 | 5.0 | 2.7 | 3.2 | 3.5 | 4.2 | 2.4 | 2.9 |
| 1.9 | 2.3 | 1.1 | 1.4 | 2.6 | 3.1 | 1.6 | 1.9 | 3.8 | 4.6 | 2.5 | 3.0 | 3.2 | 3.9 | 2.2 | 2.6 |
| 2.1 | 2.6 | 1.2 | 1.5 | 2.9 | 3.5 | 1.8 | 2.2 | 4.2 | 5.1 | 2.7 | 3.3 | 3.7 | 4.4 | 2.5 | 3.0 |
| 2.1 | 2.5 | 1.2 | 1.5 | 2.9 | 3.4 | 1.8 | 2.1 | 4.1 | 5.0 | 2.7 | 3.2 | 3.6 | 4.4 | 2.5 | 3.0 |
| 2.1 | 2.6 | 1.2 | 1.5 | 3.2 | 3.8 | 2.0 | 2.4 | 4.1 | 5.0 | 2.6 | 3.2 | 4.1 | 4.9 | 2.8 | 3.4 |
| 2.1 | 2.6 | 1.2 | 1.5 | 3.1 | 3.7 | 1.9 | 2.3 | 4.1 | 4.9 | 2.6 | 3.2 | 4.0 | 4.9 | 2.7 | 3.3 |
| 2.0 | 2.5 | 1.2 | 1.5 | 3.1 | 3.7 | 1.9 | 2.3 | 4.0 | 4.9 | 2.6 | 3.1 | 4.0 | 4.8 | 2.7 | 3.3 |
| 2.0 | 2.5 | 1.2 | 1.4 | 3.0 | 3.6 | 1.9 | 2.3 | 4.0 | 4.8 | 2.6 | 3.1 | 3.9 | 4.8 | 2.7 | 3.3 |
| 2.0 | 2.4 | 1.2 | 1.4 | 3.0 | 3.6 | 1.9 | 2.2 | 3.9 | 4.8 | 2.5 | 3.1 | 3.9 | 4.7 | 2.7 | 3.2 |
| 1.9 | 2.4 | 1.1 | 1.4 | 3.0 | 3.5 | 1.8 | 2.2 | 3.9 | 4.7 | 2.5 | 3.0 | 3.9 | 4.7 | 2.6 | 3.2 |
| 1.9 | 2.4 | 1.1 | 1.4 | 2.9 | 3.5 | 1.8 | 2.2 | 3.9 | 4.7 | 2.5 | 3.0 | 3.8 | 4.6 | 2.6 | 3.2 |

Figure 3: Effect of substrate pre-treatment and nanoparticles addition on FM methane yield of OPEFB

Figure 4: Effect of substrate pre-treatment and nanoparticles addition on oDM methane yield of OPEFB

- Conclusion

This study demonstrated that combining Fe₂O₃ nanoparticles with thermal–chemical pretreatment significantly enhances the biodegradability, methane yield, and quality of biogas produced from OPEFB under mesophilic conditions. The integrated approach achieved up to 40% higher cumulative yield and 15–20% methane enrichment compared to untreated samples. The modified Gompertz model effectively described biogas kinetics (R² = 0.94).

The process offers an eco-friendly solution for managing oil palm waste while contributing to renewable energy generation. Future research should optimize nanoparticle dosage, evaluate long-term stability, and assess the environmental safety of residual nanoparticles in digestate.

Disclaimer (Artificial intelligence)

The authors hereby declare that no generative AI technologies, such as large language models (ChartGPT, COPILOT, Gemini, or similar tools) and text-to-image generators, were used during the writing, analysis, or editing of this manuscript. All parts of the paper were written, reviewed, and validated entirely by the authors.

References

- Appels, L., Baeyens, J., Degrève, J., Dewil, R., Van Impe, J., & Lievens, B. (2008). Principles and potential of the anaerobic digestion of waste-activated sludge. Prog. Energy Combust. Sci., 34, 755–781. https://doi.org/10.1016/j.pecs.2008.06.002

- Ryckebosch, E., Drouillon, M., Vervaeren, H., Lievens, B., De Baere, L., & De Vos, P. (2011). Techniques for transformation of biogas to biomethane. Biomass & Bioenergy, 35, 1633–1645. https://doi.org/10.1016/j.biombioe.2011.02.033

- Rizal, N. F. A. A., Ibrahim, M. F., Zakaria, M. R., Abd-Aziz, S., Yee, P. L., & Hassan, M. A. (2018). Pre-treatment of oil palm biomass for fermentable sugars production. Molecules, 23(6), 1381. https://doi.org/10.3390/molecules23061381

- Suksaroj, C., Jearat, K., Cherypiew, N., Rattanapan, C., Suksaroj, T. T., & Boontawan, P. (2023). Promoting circular economy in the palm oil industry through biogas co-digestion of palm oil mill effluent and empty fruit bunch pressed wastewater. Water, 15, 2153. https://doi.org/10.3390/w15122153

- Suhartini, S., Hidayat, N., Rohma, N. A., Paul, R., Abdurrahman, F., & Sari, N. D. (2022). Sustainable strategies for anaerobic digestion of oil palm empty fruit bunches in Indonesia: a review. Int. J. Sustain. Energy. https://doi.org/10.1080/14786451.2022.2130923

- Zhang, Z., Guo, L., Wang, Y., Zhao, Y., She, Z., & Gao, M. (2020). Application of Fe₃O₄ nanoparticles during two-stage anaerobic digestion with waste sludge. Renewable Energy, 146, 2724–2735. https://doi.org/10.1016/j.renene.2019.08.078

- Paletta, R., Cerasa, V., Lo Giudice, A., Martino, M., D’Agostino, A., & Zema, D. A. (2023). Influence of Fe₂O₃ nanoparticles on the anaerobic digestion of Sargassum spp. Processes, 11(4), 1016. https://doi.org/10.3390/pr11041016

- Casals, E., Barrena, R., González, E., Font, X., Sánchez, A., & Carrera, J. (2021). Historical perspective of the addition of magnetic nanoparticles into anaerobic digesters (2014–2021). Front. Chem. Eng., 3, 745610. https://doi.org/10.3389/fceng.2021.745610

- Adebayo, A. O., & Bolaji, G. E. (2018). Biogas production potentials of cassava peels co-digested with yam peels using a batch reactor at mesophilic temperature. J. Eng. Res. Appl., 8, 9–13.

- Jekayinfa, S. O., Ola, F. A., Akande, F. B., Adesokan, M. A., Abdulsalam, I. A., & Adeniyi, O. R. (2024). Modification and performance evaluation of a biomass pelleting machine. AgriEngineering, 6(3), 130. https://doi.org/10.3390/agriengineering6030130

- Angelidaki, I., Treu, L., Tsapekos, P., Luo, G., Campanaro, S., & Kougias, P. G. (2018). Biogas upgrading and utilization: current status and perspectives. Biotechnol. Adv., 36, 452–466. https://doi.org/10.1016/j.biotechadv.2018.01.011

- Dong, X., Wu, F., Li, Y., Xu, D., Zhang, T., & Chen, L. (2024). Impact of iron oxide on anaerobic digestion of frass in insect-based bioconversion systems. Front. Environ. Sci., 12, 1452931. https://doi.org/10.3389/fenvs.2024.1452931

- Kumar, S., Verma, R., Singh, P., Mehta, A., Rajput, N., & Sharma, V. (2024). Application of Fe-NPs for enhancing methane production in an anaerobic biogas plant. ACS Omega. https://doi.org/10.1021/acsomega.4c11502

- Singh, D., Kumari, N., Rani, V., Mehta, S., Malik, K., & Ranga, P. (2022). Biostimulation of anaerobic digestion using iron oxide nanoparticles (IONPs) for increasing biogas production from cattle manure. Nanomaterials, 12(3), 497. https://doi.org/10.3390/nano12030497

- Barrena, R., Font, X., Artola, A., Sánchez, A., Carrera, J., & Pagans, E. (2022). Enhancement of anaerobic digestion with nanomaterials: mechanisms and constraints. Energies, 15, 5087. https://doi.org/10.3390/en15145087

- Khedher, N. B., Tiwari, S. B., Ben Salem, A., Jemai, M., Ayadi, M., & Mahjoub, B. (2022). Comparative evaluation of mechanistic and empirical models for biogas prediction. Energy Reports, 8, 456–468. https://doi.org/10.1016/j.egyr.2022.01.034

- Khedher, N. B., Tiwari, S. B., Ayadi, M., Jemai, M., Ben Salem, A., & Bensalem, F. (2022). Modeling of biogas production and biodegradability of date-palm biomass using modified Gompertz model. J. Clean. Prod., 332, 130037. https://doi.org/10.1016/j.jclepro.2021.130037

- Zhang, Y., Zhao, J., Hu, X., Chen, Q., Yu, D., & Wang, R. (2020). Enhancement of biogas production from lignocellulosic waste via Fe₂O₃ nanoparticle supplementation. Bioresour. Technol., 312, 123658. https://doi.org/10.1016/j.biortech.2020.123658

- Adebayo, A. O., Jekayinfa, S. O., Linke, B., Adeniran, K. A., Adewole, A. A., & Adekunle, A. F. (2013). Effect of co-digestion on anaerobic digestion of cattle slurry with maize cob at mesophilic temperature. J. Energy Technol. Policy, 3(7), 47–54.

- Angelidaki, I., Treu, L., Tsapekos, P., Luo, G., Campanaro, S., & Kougias, P. G. (2018). Biogas upgrading and utilization. Biotechnol. Adv., 36, 452–466. https://doi.org/10.1016/j.biotechadv.2018.01.011

Holliger, C., Alves, M., Andrade, D., Angelidaki, I., Astals, S., Baier, U., & De Wilde, V. (2016). Towards a standardization of biomethane potential tests. Water Science and Technology, 74(11), 2515–2522. https://doi.org/10.2166/wst.2016.336

Kumar, S., Verma, R., Singh, P., Mehta, A., Rajput, N., & Sharma, V. (2024). Application of Fe-NPs for enhancing methane production in an anaerobic biogas plant. ACS Omega. https://doi.org/10.1021/acsomega.4c11502