Formulation and in-vitro Evaluation of Extended Release Metformin Hydrochloride Directly Compressible Tablets by Using ULTRAMODTM Polymer Premix

Abstract

Aims: Different types of drug release modifying polymers, their grades and quantities have to be optimised in the formulation considering solubility characteristics and biological half-life of the drugs. Further, formulations containing such discrete ingredients processed by conventional, multi-step manufacturing techniques can be a resource-intensive and challenging task for the formulators. This necessitated development of ready-to-use, co-processed polymer premixes.

Methodology: Present study proposed to develop an optimised composition of a polymer premix, ULTRAMODTM and evaluate its performance in modifying release of highly water-soluble model drug (BCS Class III) having short biological half-life, metformin hydrochloride. Blend micromeritics and tablet physical properties and performance (in-vitro drug release) were evaluated by pharmacopeial methods.

Results: Such formulations showed excellent flow and compressibility properties, tablet physical properties and drug release compiled to the pharmacopeial specifications. These quality attributes and release modifying performance of the polymer premix stored at accelerated stability conditions for 6 months also was unchanged.

Significance: The present research contributes to scientific formulation management by demonstrating how co-processed polymer premixes simplify extended-release tablet design, reduce formulation optimization trials, and enhance manufacturability. ULTRAMOD™ offers a scientifically robust and industrially scalable approach to developing matrix tablets for highly water-soluble drugs, minimizing batch variability and reducing production cost and time.

Conclusions: Ready-to-use, co-processed polymer premix proved to be a lucrative substitute to conventional polymers. This reduced formula optimisation challenges, manufacturing complexity and promises quick commercialisation.

Keywords: ULTRAMOD™, metformin hydrochloride, direct compression, extended release, matrix tablet, coprocess

1. Introduction

The development of oral drug delivery systems that optimize therapeutic efficacy while enhancing patient compliance has been a significant focus of pharmaceutical research. Among these, modified release formulations have substantial interest, particularly in the management of chronic conditions such as diabetes mellitus, where maintaining consistent drug plasma levels is crucial for effective treatment [1]. Oral drug delivery is the most widely utilized route of administration among all other routes that have been explored for systemic delivery of drugs. The oral route offers advantages such as ease of administration, patient acceptance, and cost-effective manufacturing process [2]. The objective of designing modified drug release delivery systems is to reduce the frequency of dosing or to increase the drug's effectiveness by localization at the site of the action, reducing the dose required, or providing uniform drug delivery [3]. The success of modified release formulations depends largely on the choice of polymer used in the tablet matrix. Polymers control the rate at which the drug is released from the formulation, and selecting the appropriate polymer is critical to achieving the desired release profile [4]. Traditional polymers like hydroxypropyl methylcellulose, ethyl cellulose, and carboxymethyl cellulose sodium have been widely used because of their predictable and reliable drug release modifying characteristics [5].

Metformin hydrochloride (Metformin HCl), which is basically used in the treatment of type 2 diabetes, is often formulated in such extended release forms to overcome the limitations such as gastrointestinal side effects, frequent dosing, shorter duration of action associated with its immediate-release counter parts [6]. Metformin HCl popularly formulated into a modified drug release system because it is a highly water-soluble active pharmaceutical ingredient with short half life. Such formulations ensure more stable blood sugar levels, and reduced dosing frequency also, which improves patient adherence [7]. While previous studies have modified the release of metformin HCl using wet granulation, this method presents significant formulation challenges, including drug-polymer incompatibility during aqueous processing, variability in granule particle size distribution, and prolonged processing time [8]. Wet granulation often necessitates additional steps such as binder optimization, drying control, and moisture-sensitive stability testing, which complicate scale-up. In contrast, our direct compression method eliminates these hurdles by avoiding water exposure, ensuring uniform drug-polymer blending, and streamlining manufacturing critical advantages for robust extended-release metformin HCl tablets [9]. This research explores the formulation and in vitro evaluation of extended release metformin hydrochloride tablets formulated with ready-to-use co-processed polymer premix.

ULTRAMOD™ is a proprietary polymer blend designed to achieve extended-release formulations. It comprises HPMC and carboxymethyl cellulose sodium, which work synergistically to control the drug release profile. It offers improved flow compressibility and hydration properties, making it suitable for direct compression tablet manufacturing. Its ability to form a consistent gel matrix that modulates drug release effectively. Development of ready-to-use co-processed excipients like ULTRAMOD™ marks a significant advancement in the field of pharmaceutical formulation development and their commercialization. This simplifies tablet manufacturing through its excellent micromeritics, eliminating the need for wet granulation [9]. Use of such excipient offers several advantages, including reduced processing time, lower production costs, and enhanced batch-to-batch consistency [10]. ULTRAMOD™ also combines the desirable properties of various traditional polymers, providing a unique matrix that ensures extended drug release while maintaining the mechanical integrity of the tablet. Considering short biological half-life of metformin hydrochloride is 1.5-4.5 hours and high water solubility (>300 mg/mL), it was formulated into matrix type extended-release tablets to reduce the dosage frequency and enhance patient compliance using such novel approach [11]. Our study, further proposes to introduce other grades of such co-processed excipient for modifying release of moderate to low soluble drug also.

- Material and Methods

2.1 Materials

ULTRAMOD™ polymer premix (release retarding agent) and HiCelTM LP200 MCC-based (binder) were manufactured at SIGACHI® Industries Limited, (Gujarat, India), Colloidal silicon dioxide (glidant) was purchased from Evonik Industries, (Germany). Metformin hydrochloride (API) was purchased from IOL Chemicals and Pharmaceutical Limited, (Punjab, India) and magnesium stearate (lubricant) was purchased from Sunshine Organics Private Limited, (Mumbai, India). Marketed tablet brand Cgmet-500 (metformin hydrochloride extended release tablet 500 mg, B. No. 22230E manufactured by CMG Biotech Private Limited, Himachal Pradesh, India) was purchased from local market. Other chemicals of analytical reagent grade used in this study were sourced locally.

2.2 Methods

2.2.1 Manufacturing of Metformin hydrochloride tablets using ULTRAMOD™

To evaluate the functionality of polymer matrix in controlling release of metformin HCl, initially five different formulations were prepared (entitled F1, F2, F3, F4, F5 and F6), given in table 1.

Accurately weighed quantities of metformin hydrochloride (previously sifted through a 30# ASTM), polymer premix and other tablet excipients (previously sifted through a 30# ASTM) were transferred into a powder blender of appropriate capacity (Make: Reva Pharma Machinery, India, Model: TRMIX-20). These materials were mixed for 10 minutes at 25 revolutions per minutes. To this premix, an accurately weighed required quantity of lubricant (previously sifted through a 60-mesh stainless steel sieve) was added and mixed for not more than 3 minutes at 25 revolutions per minutes to obtain a final blend. The final blend was characterized by angle of repose, untapped bulk density, tapped bulk density, Hausner's ratio, and compressibility index parameters. Then this blend was compressed into tablets by using a 12-station tablet compression machine (Make: Eliza Press, India, Model: EP-200) fitted with 15 mm diameter standard concave shaped “D” tooling sets. Different compositions polymer premix were formulated as shown in Table 1. Relative humidity and temperature were maintained below 45% and 25°C respectively.

Table 1

Various formulations of metformin hydrochloride extended release tablets

| Ingredients | Quantity/tablet (mg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | F1 | F2 | F3 | F4 | F5 | F6 |
| Metformin hydrochloride | 500.0 | 500.0 | 500.0 | 500.0 | 500.0 | 500.0 |
| ULTRAMOD™ | 50.0 | 100.0 | 150.0 | 200.0 | 250.0 | - |
| HiCelTM LP200 | 430.0 | 380.0 | 330.0 | 280.0 | 230.0 | 480.0 |
| Colloidal silicon dioxide | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 |
| Magnesium stearate | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 | 10.0 |
| Average Tablet Weight (mg) | 1000.0 | 1000.0 | 1000.0 | 1000.0 | 1000.0 | 1000.0 |

2.2 Excipients Compatibility studies

Molecular interactions as an indication of compatibility between ingredients were studied using Fourier transform infrared spectroscopy (FTIR) (Make: Shimadzu Japan, Model: IR Spirit-S), and the spectrum was recorded in the wavelength region from 4000 to 400 cm−1 by using the diffuse reflectance system (DRS) method. The procedure consisted of mixing a sample with potassium bromide and placing it into a sample disc [12].

2.3 Evaluation of physical properties of  a powder blend containing ULTRAMOD™

The powder blend was evaluated for bulk density (untapped and tapped), flow properties (Hausner’s ratio, Carr’s index), and angle of repose as per USP-NF 2024 General Chapters <616> (Method I) and <1174> (Method 1). For untapped bulk density, a 25 mL graduated cylinder was filled with powder, and the volume was recorded without compaction. Tapped bulk density was determined using a tapped density tester (500 taps) per USP <616>. Hausner’s ratio and Carr’s index were calculated from these densities to assess flowability. The angle of repose was measured using a fixed funnel method. All tests were performed in triplicate (n=3) using powder blends from three independent batches to ensure reproducibility [13].

Hausner’s Ratio =  Tapped Density Untapped Density

Compressibility Index (%) = Tapped Density−Untapped Density Tapped Density×100

2.4 Evaluation of Physical Properties of Metformin HCl extended release tablet

2.4.1 Weight Variation

Tablet weight variation was evaluated as per USP general chapter <905> (2024). Twenty tablets were randomly selected and individually weighed using an analytical balance. The average weight was calculated, and the percentage deviation of each tablet from the average weight was determined. Acceptance criteria for tablets weighing more than 324 mg, deviations up to ±5% was permitted [14].

2.4.2 Tablet hardness

Tablet hardness was determined using a calibrated hardness tester (Make: Labindia, India, Model: TH1050M) as per the USP general chapter <1217>. Ten tablets were randomly selected and individually placed in the tester, applying force diametrically until fracture was propagated. The mean hardness [in kp(kgf)] and standard deviation were calculated [15].

2.4.3 Thickness

Thickness was measured by using a digital tablet thickness tester machine (Make: Labindia India, Model: TH1050M).

2.4.4 Friability

The friability test was conducted using a friability tester (Make: Labindia India, Model: FT1020). A total of 10 uncoated tablets, each weighing approximately 1 g were accurately weighed to determine initial weight. These tablets were placed in the friabilator drum and subjected to 100 and 200 revolutions at 25±1 revolutions per minutes. After completion of the test, the tablets were removed, dedusted and reweighed to obtain final weight. The percentage friability was determined by method mentioned in United States Pharmacopoeia: general chapter on tablet friability <1216> [16].

2.4.5 In-vitro drug release

The dissolution profile of Metformin hydrochloride extended release tablet was performed using dissolution test apparatus (Make: Labindia India, Model: DS8000) by using the USP 44 compendial method, apparatus II (paddle), speed 100 revolutions per minutes for 10 hours in 1000 mL of phosphate buffer pH 6.8 at 37 ± 0.5°C medium temperature. Six tablets were placed in each dissolution vessel. Sample aliquots of 5 mL were withdrawn from each dissolution vessel at 1 hour, 3 hours, and 10 hours. After withdrawal, samples were filtered through Whatman filter paper (No. 42). 1 mL of filtrate from the beaker was taken and transferred into 10 mL of volumetric flask and diluted up to 10 mL with dissolution medium. The same procedure was repeated for all the remaining 5 tablets. Standard and sample absorbances were recorded using a UV visible spectrophotometer (Make: Shimadzu Japan, Model: UV-1900) at λ = 232 nm wavelength. The metformin hydrochloride released from the tablet formulations was calculated using the below-mentioned formula [17].

Amount of drug released (mg) = Concentration of released drug  ×  Dilution factor  ×  Volume of dissolution medium   1000

Drug released (%) =  Amount of  drug  released (mg) label claim (mg)×100

2.4.6 Calibration curve of metformin hydrochloride

Phosphate buffer pH 6.8 filtered through whatman filter paper used as a diluent for preparing the standard solution of the drug. About 10 mg of metformin hydrochloride was accurately weighed and transferred into a 100 mL volumetric flask, the solution was sonicated and the resulting solution was diluted with the diluent to obtain a primary stock solution to obtain 100 µg/mL metformin hydrochloride. From this solution, suitable standard solutions of different concentrations were prepared.

Linearity of the absorbance was determined by measuring standard dilution of metformin hydrochloride in the range of 10-40 µg/mL. The absorbance of metformin hydrochloride was monitored at 232 nm and the corresponding absorbances were obtained. From these absorbances, a graph of concentration and absorbance was plotted. The regression equation (Figure 3) obtained was used to estimate the amount of drugs present in tablet dosage formulations.

2.5 Stability studies

The ULTRAMOD™ was packed in double poly-lined paper bag and subjected to accelerated stability conditions at 40±2°C / 75±5% RH for 6 months as recommended by ICH Q1A (R2) guidelines (ICH, 2003) at 3 months intervals. The sample were examined for any physical changes (colour, texture), aggregation and its micromeritics were evaluated.

3. Results and discussion

Formulations were prepared by varying amounts of ULTRAMOD™ polymer premix in the tablet formulation to see the effect of their concentrations on drug release rate and powder micromeritics. These powder blends study were physically evaluated for following parameters.

3.1 Characterization of powder blends containing ULTRAMOD™ and metformin hydrochloride

3.1.1 Compatibility studies

FTIR spectra of metformin hydrochloride and the powder blend containing ULTRAMOD™ were found to be super imposed. The spectra of the formulations showed no additional peaks in the functional group and fingerprint regions. This indicated that there was no alteration in the molecular structure and functional groups in the blend. According to this investigation, metformin hydrochloride remained chemically unaltered when presents in polymer matrix tablets.

Figure 1 (a)  FTIR spectrum comparison of ULTRAMOD™ (       ) and mixture of

ULTRAMOD™ and metformin hydrochloride (      )

Figure 1 (b) FTIR spectrum of metformin hydrochloride

Figure 1 (c) FTIR spectrum comparison of ULTRAMOD™ (       ) and mixture of

ULTRAMOD™ and HiCelTM LP200 (      )

Figure 1 (d) FTIR spectrum comparison of ULTRAMOD™ (       ) and mixture of

ULTRAMOD™ and colloidal silicon dioxide (      )

Figure 1 (e) FTIR spectrum comparison of ULTRAMOD™ (       ) and mixture of

ULTRAMOD™ and magnesium stearate (      )

As the concentration of the polymer premix increased, a corresponding rise in both bulk and tapped densities was observed. This trend suggested that the powder blend became more compactable with higher polymer content. Specifically, the tapped density increased from 0.576 g/mL in formulation F1 to 0.711 g/mL in formulation F5, indicating improved particle packing at elevated concentrations of ULTRAMOD™, as presented in (Table 2). This may be attributed to the enhanced cohesiveness and better filling efficiency provided by the polymer matrix. Interestingly, formulation F1 exhibited superior flow properties compared to F5, as reflected by its lower Carr’s Index. Moreover, F1 showed slightly better compressibility, as evidenced by a lower Hausner’s ratio, further supporting the influence of polymer concentration on powder characteristics.

Table 2

Micromeritics of Metformin hydrochloride formulation F1-F6

| Formulation | Angle of Repose (°) | Bulk Density (g/mL) | Tapped Density (g/mL) | Carr’s Index (%) | Hausner's ratio | Pre-Compression (kN) | Main Compression (kN) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F1 | 35 | 0.442 | 0.576 | 23.26 | 1.30 | 3.8 | 15.6 |
| F2 | 36 | 0.459 | 0.604 | 24.00 | 1.32 | 4.2 | 16.2 |
| F3 | 36 | 0.474 | 0.625 | 24.16 | 1.32 | 4.5 | 16.5 |
| F4 | 37 | 0.502 | 0.673 | 25.41 | 1.34 | 4.8 | 17.1 |
| F5 | 39 | 0.526 | 0.711 | 26.02 | 1.35 | 5.4 | 17.8 |
| F6 | 34 | 0.376 | 0.482 | 21.99 | 1.28 | 3.1 | 13.1 |

Formulations with a greater polymer content (such as F4 and F5) showed higher angle of repose and Carr's index values of all specimens (Figure 2). This suggested poor flowability, which may make tablet manufacturing more difficult [18]. The compressibility of the powder blend significantly increased as the polymer content increased. According to this, powder mixes that contained more polymers are more compressible and less free-flowing.

Figure 2. Effect of ULTRAMODTM polymer premix concentration on flow properties

and compressibility of various formulations, F1-F6

3.1.2 Evaluation of physical parameters of metformin hydrochloride extended release tablet

In the present work, matrix type metformin HCl tablets were prepared by direct compression technique.

3.1.2.1 Properties of metformin hydrochloride extended release tablets

Their physical parameters such as weight variation, hardness, thickness and friability as essential indicators of quality and consistency were evaluated. Tablet formulations containing F3 showed better appearance, minimal visual defects and smooth edges, which were evaluated based on visual observation than the tablets containing F1, F2, F4, F5 and F6 formulations. All the tablets passed the weight variation test, as the percentage weight variation was within pharmacopoeial limits of ±5% of the weight as mentioned in table 3. Tablet hardness is an essential parameter, as it impacts on the drug's release profile and their mechanical stability. Formulation F1 exhibited the highest hardness 11.84 ± 0.14 [Kp(kgf)], while formulation F5 showed the least hardness 9.26 ± 1.91 [Kp(kgf)] as mentioned in Table 3. The decrease in hardness from F1 to F5 suggests that increased ULTRAMOD™ polymer matrix concentration might result lower the tablet hardness at applied compression forces. This trend could impact the release characteristics of the drug, as softer tablets tend to disintegrate more readily.

Table 3

Physical properties of Metformin hydrochloride extended release tablets of all formulation

| Formulation | Observations |  |  |  |
| --- | --- | --- | --- | --- |
|  | Weight variation (mg) | Hardness [Kp(kgf)] | Thickness (mm) | Friability (%) |
| F1 | 1001 ± 0.08 | 11.84 ± 0.14 | 4.82 ± 0.07 | 0.54 ± 0.04 |
| F2 | 1008 ± 0.36 | 11.36 ± 0.34 | 4.74 ± 0.34 | 0.57 ± 0.28 |
| F3 | 1003 ± 0.58 | 10.89 ± 0.29 | 4.80 ± 0.39 | 0.60 ± 0.19 |
| F4 | 1012 ± 0.66 | 10.12 ± 1.48 | 4.83 ± 0.47 | 0.62 ± 0.34 |
| F5 | 1017 ± 1.14 | 9.26 ± 1.91 | 4.77 ± 0.58 | 0.67 ± 0.45 |
| F6 | 998 ± 0.19 | 13.78 ± 0.81 | 4.81 ± 0.28 | 0.31 ± 0.32 |
| Marketed Tablet | 852 ± 1.01 | 14.85 ± 1.91 | 4.54 ± 0.58 | 0.12 ± 0.24 |

The thickness of the tablets ranged from 4.74 mm ± 0.34 to 4.83 mm ± 0.47 (Table 3). This consistency indicated uniform compression ability of ULTRAMOD™ across all formulations, despite differences in polymer concentration. The friability results within the acceptable limit of less than 1%. which indicated that the tablets containing ULTRAMOD™obtained by direct compression had reasonable strength.  ULTRAMOD™ through significantly influenced the modified-release behavior, the role of HiCel™, a microcrystalline cellulose-based filler, should not be overlooked. Varying concentrations of HiCel™ altered tablet porosity, hardness, and disintegration time, thereby affecting overall release performance. Hence, the observed tablet behavior is likely due to the combined effects of both ULTRAMOD™ and HiCel™.

3.1.3 In-vitro dissolution study of metformin hydrochloride extended release tablet

The drug release profile is the most critical factor in evaluating the extended release formulation, prepared with different concentrations of ULTRAMOD™ polymer premix (Table 4).

Table 4

Metformin hydrochloride extended release tablet drug release specification as per USP monograph [16]

| Time intervals | Amount drug released |
| --- | --- |
| 1 hour | Between 20% to 40% |
| 3 hours | Between 45% to 65% |
| 10 hours | Not less than 85% |

As mentioned in (Figure 4), the average percentage of drug release was assessed at intervals of 1 hour, 3 hours and 10 hours. The results of the drug release profile of Metformin hydrochloride extended release tablets in various formulations (F1–F6) are demonstrated in (Figure 4).

F6 which was formulated without the ULTRAMOD™ polymer premix, exhibited immediate release with over 96.18% ± 1.06 of the drug released within 1 hour as mentioned in (Figure 4). Formulation F1 showed drug release slightly above compendial specifications with values of 42.15% ± 0.98. F1 showed drug release percentages slightly above the specified range with values of 75.84% ± 1.46. This highlight the critical role of ULTRAMOD™ in sustained drug release and confirms its effectiveness in extended-release tablet formulations.

Figure 3. Standard calibration curve of Metformin hydrochloride

Figure 4.  In-vitro drug release profile of Metformin hydrochloride

extended release tablets

Formulation F2 showed drug release after 1 hour within the specified limit but, after 3 hours, drug release was slightly above the specified range with values of 68.93% ± 0.82 as mentioned in (Figure 4). Formulation F3 was superior in balancing drug release and physical properties. It offered extended release over 10 hours and closely matched the performance as per standard drug release specifications as well as the marketed tablet. Although the formulation demonstrated a release profile similar to the marketed product, it is important to acknowledge that batch-to-batch variability existed in commercial formulations. The optimal amount of ULTRAMOD™ polymer premix, about 15% w/w (150 mg/tab), ensured the desired release profile. Whereas tablets containing formulations F4 and F5 showed drug release after 10 hours less than 85%, as mentioned in (Figure 4). Amongst various formulations of Metformin HCl with ULTRAMOD™ the tablets containing 15% ULTRAMOD™ showed 26.93% drug release in 1 hour, 51.13% drug release in 3 hours and 93.18% drug release in 10 hours as specified by Metformin HCl extended release tablet USP-NF monograph [19]. Such performance was unchanged even after using ULTRAMOD™ which was stored at 40°C/75% RH for 6 months in simulated packing materials. Also critical material attributes of this polymer premix, such as density, flow properties, particle size distribution, viscosity, residue on ignition were found to be comparable to those during initial evaluation.

It was observed that crucial physico-chemical parameters of ULTRAMOD™ charged on accelerated stability conditions remained unchanged even after 6 months, as tabulated below. This indicated that ULTRAMOD™ polymer premix would continue to contribute to unchanged quality and performance characteristics of formulation during storage also.

Table 5

Stability study on Micromeritics of ULTRAMOD™ stored at 40±2°C/75±5% RH for 6 months

| Test Parameters | Observations |
| --- | --- |
| Description | A white to off white color powder |
| pH (1% solution) | 7.01 |
| Loss on drying (%) | 5.18% |
| Residue on ignition (%) | 6.70% |
| Viscosity (centipoise) | 7570 centipoise |

4. Conclusion

Metformin hydrochloride tablet formulation 'F3' with an excellent micromeritics (Bulk density > 0.6 g/mL) and improved processability (Hausner's ratio ~ 1.3 and Carr's index < 25%) could be produced by direct compression method. Drug release from such matrix tablets evaluated by compendial method showed that such polymer premix was effective at as low as 15% w/w concentration. Such performance was comparable to that of a well-known marketed formulation, which was proposed to have produced by conventional multi-step manufacturing process. This highlighted superiority of co-processed polymer premix in formula optimisation, simplifying manufacturing and increasing speed to market. Further, such formulation quality attributes and performance were unchanged when the polymer premix stored at accelerated conditions for 6 months was used. This provided a confidence to the formulators that such co-processed polymer premix has robust physico-chemical nature thus ensuring least variations in the product quality. The present study demonstrated a lucrative substitute to such conventional polymers by developing a co-processed premix containing time-tested polymers, like hypromellose and carmellose sodium in accurate quantities. Hence this ensured less tedious formulation development and quick manufacturability of extended release formulations.

Author contributions statement

Dr. Viraj Kulthe contributed to conceptualization and supervision of the study. Rauf Pathan carried out the experimental work and manuscript writing. Amit Raj Sinha contributed to data analysis and manuscript review. All authors read and approved the final manuscript.

Conflict of interest

The authors state and confirm no conflict of interest.

Disclaimer (Artificial intelligence)

The author confirms that no generative artificial intelligence (AI) tools were used in the preparation of this manuscript.

References

- Brahmankar, D.M. and Jaiswal, S.B. (2009) Biopharmaceutics and Pharmacokinetics. 2nd Edition, Vallabh Prakashan, Delhi, 399-401.

- Chein YW. Novel drug delivery systems. 2nd edition. New York: Marcel Dekker; 1992; 1-42.

[3]  Rajarao CH and Rohini P: Formulation design of empagliflozin and metformin hydrochloride extended release tablets: optimization of formulation using statistical experimental design. IJPSR 2020; 11(12): 6434-47.

[4]   Li H, Sun S, Zhao Y. Quality by Design (QbD) Approaches in developing extended-release tablets: Case studies and regulatory prospectives. AAPS PharmSciTech. 2021; 22(3): 88.

[5]   Pockle, R. D., Masareddy, R. S., Patil, A. S., & Patil, P. D. (2023). A comprehensive review on pharmaceutical excipients. Therapeutic Delivery, 14(7): 443-458.

[6]  Lalla JK. Introduction to controlled release and oral controlled drug delivery system. East Pharm. 1991;45: 25-8.

[7]   Modi SA, Gaikwad PD, Bankar VH, Pawar SP. Sustained release drug delivery system: a review. Int J Pharm Res Dev. 2011; 2 (12):147-60.

[8]   Roy H, Brahma CK, Nandi S, Parida KR. Formulation and design of sustained release matrix tablets of metformin hydrochloride: Influence of hypromellose and polyacrylate polymers. Int J Appl Basic Med Res. 2013 Jan; 3(1): 55-63.

[9]  Takasaki, H., et al. (2015). The importance of binder moisture content in metformin HCl granulation and tablet compactibility. International Journal of Pharmaceutical Sciences and Research, 6(1): 123–130.

[10] Rowe, R.C., et al. (2023) Handbook of Pharmaceutical Excipients (9th Edition). Pharmaceutical Press.

[11] Van der Merwe J, Steenekamp J, Steyn D, Hamman J. The Role of Functional Excipients in Solid Oral Dosage Forms to Overcome Poor Drug Dissolution and Bioavailability. Pharmaceutics, 2020 Apr 25;12(5): 393.

[12]  Sathoskar RS, Bhandarkar SD and Nirmala N. Pharmacology and pharmacotherapeutics. Popular

Prakashan; Mumbai, 2009; Re-Revised 21st edition: 895.

[13] Armin H, Gerhardt. Moisture effects on solid dosage forms formulation processing and stability. Journal of GXP Compliance, 2009; (13): 58-66.

[14] United States Pharmacopeia-National Formulary (USP-NF) (2024). General Chapters <616> Bulk Density and <1174> Powder Flow.

[15] United States Pharmacopeia-National Formulary (USP-NF) (2024). Uniformity of dosage units 	  <905>.

[16] United States Pharmacopeia-National Formulary (USP-NF) (2024). General Chapter <1217> Tablet Breaking Force.

[17] United States Pharmacopeia-National Formulary (USP-NF) (2024). General Chapter <1216> Tablet Friability.

[18] United States Pharmacopeia (USP) (2024). Metformin HCl extended release tablet.

[19] Karsten H, Katharina MP. Evaluation of a new Co-processed compound based on lactose and maize starch for tablet formulation. AAPS Pharm. Sci. Tech. 2004; 6: 1–12.