Modelling the Soil Wetting Pattern under Pulse and Continuous Drip Irrigation

Abstract:

Challenges in continuous drip irrigation systems in Gujarat, India, include emitter clogging due to narrow water passages and low discharge rates. In sandy soils, the wetting depth becomes much greater than the width, resulting in excessive deep percolation beyond the crop root zone. To address these issues, widening the wetted area using high-discharge emitters is necessary, which supports the use of pulse drip irrigation. Since understanding wetted depth and width is essential for proper drip system design, the lack of models for predicting wetting patterns under pulse and intermittent flow—beyond those developed for continuous flow—creates a significant gap. Therefore, this study aimed to develop a dimensional analysis model capable of estimating the wetted zone geometry across continuous, intermittent, and pulsed flow conditions.

A semi-empirical dimensional analysis approach was used to construct the model, and its predicted wetted depths and widths were compared with field observations. Experiments measured maximum wetting dimensions at 30, 60, and 120 minutes under different discharge rates (2, 4, and 8 LPH). Model performance was evaluated using root mean square error, mean error, and model efficiency, all of which indicated strong predictive accuracy. The results confirmed that the models can reliably estimate wetting patterns across all three irrigation regimes. Under pulse flow, reducing the operating duration for the same water volume increased horizontal spread while decreasing vertical penetration. Increasing pulse frequency (one to four pulses) further reduced deep percolation and enhanced lateral wetting, demonstrating the advantages of pulse irrigation in minimizing water loss while allowing the use of high-discharge emitters.

The dimensional analysis models developed in this study effectively predicted wetted depth and width under continuous, intermittent, and pulsed line-source irrigation. Validation against controlled laboratory data showed strong performance, with efficiencies of 98% and 97% for depth and width under continuous flow, 94% and 83% under intermittent flow, and 86% and 79% under pulsed flow. These results confirm that the models reliably represent wetting behavior across different irrigation regimes and are suitable for predicting wetting patterns in line-source applications.

Key words: Drip irrigation Dimensional analysis Soil wetting pattern Pulse flow Intermittent    flow Continuous flow

INTRODUCTION

Gujarat is located in a semi-arid to arid climate zone, where rainfall is unevenly distributed and often scarce, particularly in the northern and western regions. A large portion of the state experiences frequent droughts, and desert-like conditions prevail in areas such as Kutch. The main sources of fresh water for Gujarat include surface water from rivers such as the Narmada, Tapi, Mahi, and Sabarmati, along with limited groundwater resources. The Sardar Sarovar Project on the Narmada River has become a vital source for irrigation, drinking, and industrial water supply across the state (Government of Gujarat, 2023). In addition to this, groundwater reservoirs are used extensively, though overexploitation in several regions has led to declining water tables and salinity intrusion (Central Ground Water Board, 2023).

The total irrigated area in Gujarat is around 5.0 million hectares and is expected to increase further through improved irrigation efficiency and the adoption of modern irrigation techniques such as drip and sprinkler systems, particularly in water-scarce regions (Department of Agriculture, Farmers Welfare and Co-operation, Government of Gujarat, 2023). Meanwhile, the water demand continues to grow rapidly due to population increase, industrial expansion, and rising living standards. As a result, the per capita availability of water has been steadily declining and, in several regions, has approached the “water stress” or “water scarcity” threshold (NITI Aayog, 2022). Therefore, water shortage remains a critical challenge and one of the major constraints to achieving sustainable agricultural and economic development in the state.

Drip irrigation can significantly enhance water use efficiency, but only when the system is properly designed to match soil and crop requirements. Accurate knowledge of the wetted width and depth beneath emitters is essential for efficient system design and for ensuring precise placement of water and nutrients within the crop root zone. However, drip irrigation systems often face challenges such as emitter clogging, deep percolation losses, and limited horizontal water spread—especially in coarse-textured soils. Increasing emitter passage size is one strategy to reduce clogging, but this often leads to higher discharge rates, which in turn alter the wetting pattern in the soil.

To address these issues, intermittent or pulsed application of water has been explored as an alternative to continuous flow. Oron (1981) noted that continuous application tends to increase deep percolation below the root zone. The concept of pulsing, first introduced by Karmelli and Peri (1974), involves alternating phases of water application and rest. Although the instantaneous discharge during pulsing may be higher, the average discharge over the full cycle remains equal to that of a continuous system.

The effectiveness of pulse irrigation varies with soil type, emitter discharge, and the duration of operating and resting phases (Jackson and Kay, 1988). High pulsed discharge can lead to rapid surface ponding if it exceeds the soil’s infiltration capacity, potentially causing runoff, erosion, and distortion of the wetting pattern. While pulsing may help alleviate clogging issues, it may also increase system costs due to the need for larger pipes and automated valves. These expenses, however, can be compensated through reduced maintenance and longer emitter lifespan. Pumping costs typically remain unchanged, provided that the pulsed discharge is matched with continuous-flow requirements through proper selection of the number of laterals operating during each pulse.Mostaghimi and Mitchell (1983) found that pulsed drip irrigation significantly reduces deep percolation losses in sandy soils and enhances lateral water spread. Similarly, Levin and Van Rooyen (1979) reported that pulsing reduces percolation losses and that wetting patterns under pulsed and continuous flows are nearly identical at low discharges from point sources. Intermittent irrigation, based on discharge pulses followed by breaks, can improve water management and efficiency by aligning mean irrigation rates with soil hydraulic conductivity and minimizing deep percolation losses (Oron, 1996).

Schwartzman and Zur (1986) developed a simplified semi-empirical method to determine the geometry of wetted soil zones under line-source water application for continuous flow regimes. Their model, derived via dimensional analysis, predicts the wetting front position under surface drip irrigation as a function of applied water and basic soil properties such as saturated hydraulic conductivity. This approach simplifies the design process by eliminating the need for complex numerical or analytical models. While information on matric potential distribution is not always necessary, data on the width and depth of the wetted zone remain essential for field design (Zur, 1996).

However, there is a lack of predictive models for wetting patterns under pulsed and intermittent flow regimes, as most existing models apply only to continuous flow. Therefore, there is a strong need to develop a dimensional analysis model capable of predicting wetted soil depth and width under different flow regimes with line sources of water application.

MATERIALS AND METHODS

A field experiment was conducted during the winter season of 2021–22 at Muvaliya Farm, Anand Agricultural University, Dahod, located at 22°44′–22°58′ N latitude and 74°07′–74°27′ E longitude, with an elevation of 217 m above mean sea level. The region falls under the Middle Gujarat Agro-Climatic Zone-III and is characterized by a semi-arid climate, receiving 700–900 mm of annual rainfall, mostly during the southwest monsoon (June–September). Average temperatures range from 9.5 °C to 47 °C, with relative humidity between 45 % and 80 %. The experimental soil was sandy loam, well-drained, and moderately fertile with pH 7.3–7.5, low organic carbon, medium phosphorus, and high potassium content.

Emitter discharge characteristics and manufacturing variation were evaluated under operating pressures ranging from 0.8 to 1.7 kg/cm², following ASAE (1996) and ISO (1991) standards. The field was well-prepared using a tractor-drawn cultivator and rotavator to obtain fine tilth, ensuring uniform soil conditions for accurate assessment of wetting patterns under pulse and continuous flow drip irrigation systems.

Experimental setup for soil wetting geometry.

In the present study, the effects of pulse flow compared to continuous flow on soil wetting geometry and moisture distribution patterns were evaluated under varying discharge rates and pulse frequencies. Two experimental factors were considered: emitter discharge rate and the number of pulses applied. Three emitter discharge rates—2, 4, and 8 liters per hour (lph)—and four pulse combinations (1, 2, 3, and 4 pulses) were tested. The total volume of water applied was kept constant across all treatments, with continuous flow converted into equivalent pulsed intervals. For example, a 4 lph emitter operated continuously for 2 hours (delivering 8 liters) was pulsed into two 1-hour irrigation events separated by a 1-hour rest period. All treatment combinations followed a similar on–off pulsing schedule.

At the end of each irrigation event, the soil surrounding the emitter was excavated to expose the vertical soil profile and to measure the wetting front in both horizontal and vertical directions. A coordinate system was established with the origin at the soil surface directly above the emitter, and plastic sheets were used to minimize evaporation during observation. The maximum wetting distances in each direction were recorded, and soil samples were collected from specific coordinates within the wetted zone. The samples were analyzed to determine soil moisture content using the gravimetric method, providing data on the distribution and extent of moisture movement under different flow regimes.

Table 1: Details of Experimental Factors

| Main Factor: Discharge Rate | Sub Factor: Number of Pulse |
| --- | --- |
| Q1 = 2 LPH Q2 = 4 LPH Q3 = 8 LPH | P1 = Continues flow P2 = two pulse (total time of operation divide in two pulse) P3 = three pulse P4 = four pulse |

Table 2: Details of Various Flow and Pules Combination

| Q (lph) | Time (min) | Volume (lit.) | Number of pulse(min) |  |  |  |  |  |  |  |  |  |  | Total time for operation(min) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P1 | P2 |  | P3 |  |  | P4 |  |  |  | P1 |  | P2 | P3 | P4 |
| 2 | 30 | 1 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 2 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 4 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |
| 4 | 30 | 2 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 4 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 8 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |
| 8 | 30 | 4 | 30 | 15 | 15 | 10 | 10 | 10 | 7.5 | 7.5 | 7.5 | 7.5 | 30 |  | 45 | 50 | 52.5 |
|  | 60 | 8 | 60 | 30 | 30 | 20 | 20 | 20 | 15 | 15 | 15 | 15 | 60 |  | 90 | 100 | 105 |
|  | 120 | 16 | 120 | 60 | 60 | 40 | 40 | 40 | 30 | 30 | 30 | 30 | 120 |  | 180 | 200 | 210 |

Determination of Wetting Pattern Dimension

The laterals used for determination of wetting patterns were 16 mm having drippers of inline with 2 lph and 4 lph. The laterals were connected to sub
main line. The system was continuously operated for 2 hours and 1st time zone’s cock
valve was closed off after 30 minutes operating time, similarly it was repeated for all operating time zone i.e. 30, 60 and 120 minutes. After that system was turned off and wetting pattern dimension was measured.

Development of mathematical model

Schwartzman and Zur (1986) introduced a semi-empirical technique for estimating the wetted soil profile produced by surface-laid line-source emitters operating under continuous irrigation. Their concept suggests that the final dimensions of the wetted zone—both its lateral spread and its penetration depth—are governed by the soil’s inherent properties, the emitter discharge per unit length of the lateral line, and the initial moisture condition of the soil. In their formulation, soil characteristics are incorporated through the use of saturated hydraulic conductivity, which reflects how easily water moves through the soil matrix.

z = f1 (Qc, Ks, V)                                                        (1)

d = f2 (Qc, Ks, V)                                                        (2)

where f1 and f2 are function signs; 
z - wetted soil depth (cm).
d - wetted soil width (cm)
Qc - the discharge rate of continuous drip irrigation (cm3/ min).
Ks - the saturated hydraulic conductivity of soil (cm / min)

T Is irrigation time (min)

z = αz Ksa Qcb Vc                                                         (3)

d = αd Ksa Qcb Vc                                                        (4)

Where αz, αd, a, b and c are constant

Ismail et al. (2014) modified Schwartzman and Zur (1986) equation and introduce two term. they suggested that discharge rate is not same in pulse flow compare to continuous one, because on and off system of operation it was change. And second one was cycle ratio, which is ratio of system operating time with total time of one cycle. And they developed relationship of wetted soil depth and width.

Pulse and Intermittent Flow:The dimensions of the wetted pattern under pulse and intermittent drip irrigation are influenced by several key factors, including emitter discharge rate, soil saturated hydraulic conductivity, and the accumulated duration of irrigation cycles. For these flow regimes, two distinct functional relationships are used to describe wetting pattern geometry—one representing the wetted soil depth (z) and the other representing the wetted soil width (d). These relationships can be expressed as follows:

z = f1 (Qp, Qc, Ks, T)                                                   (5)

d = f2 (Qp, Qc, Ks, T)                                                   (6)

where f1 and f2 are function signs; 
z - wetted soil depth (cm).
d - wetted soil width (cm)
Qc - the discharge rate of continuous drip irrigation (cm3/ min).

Qp - the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ).
Ks Is the saturated hydraulic conductivity of soil (cm / min)

T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

n Is number of cycles.

The basic dimension of each variables can be expressed as follows:

z = L, d = L, Qp = L3T-1, Qc = L3T-1, Ks = LT-1 and T = T

The number of π terms (free variables) is equal to (n-m), where n=5 which is the total number of variables in the experiment (z, Qp, Qc, Ks, T) and m=2 which is the number of reference dimensions (L, T).

free variables = n-m = 5-2 =3

F (π1, π2, π3) = 0                                             (7)

The dependent variables (repeating variables) were selected as Ks and Qc, which involve all fundamental dimensions. The π term was formed by grouping one of the free variables with all dependent variables, starting with the free variable, z, the first π term can be formed by combining z with the dependent variables such that

π1z = Ksa1. Qcb1. Z                                           (8)

Estimating a1, b1 so that the combination is dimensionless

(LT-1) a1.( L3T-1) b1. (L) = L0T0

So: a1+3b1= 0 (for L)  and -a1-b1= 0 (for T)

Therefore: a1= 1/2 and b1= -1/2

The π term them becomes

π1z = zKsQc                                            (9)

Repeating the preceding steps, the second and third π term can be obtained as follows:

π2z = QpQc                                                     (10)

π3z = TKs3Qc                                               (11)

Now base on all π terms one final equation for z term become like:

π1z = F (π2z , π3z)                                      (12)

π1z = az (π2z)bz (π3z)cz                                    (13)

The values of unknowns az, bz, cz and were estimated using the data of the laboratory experiments with curve fitting techniques then Substituting all π term then the final equation which estimates the depth of wetted pattern can be written as follows:

z = αz Ksa Qpb Qcc Td                                               (14)

d = αd Ksa Qpb Qcc Td                                              (15)

Where αz, αd, a, b, c and d are constant

In case of pulse flow, Qc can be calculated as follow;

Qc = CR Qp (16)

where the cycle ratio is, the ratio between the on time (Ton) and the cycle time (Ton+Toff), cycle ratio can be calculated as follow CR = TonTon+Toff (17)

RESULTS AND DISCUSSION

Because the simulation of soil water movement is highly dependent on soil characteristics, which vary from place to place, any developed model must be validated under actual field conditions before it can be reliably used. In this study, different models describing soil moisture distribution from point-source irrigation were evaluated through simulation, and their outputs were compared to identify which model best represents the wetting behaviour of the sandy loam soils in the study area.

Ismail et al. (2014) modified Schwartzman and Zur (1986) equation and introduce two term. T and Qp which is the elapsed time (min) and the discharge rate of pulse drip irrigation which changes according to the pulse ration. And generalize equation z = f1 (Qp, Qc, Ks, T) and d = f2 (Qp, Qc, Ks, T) (3.8 and 3.9). for solving this with empirical method used 3 π term introduce. And make other generalize equation 3.10 (F (π1, π2, π3) = 0) by formulating problem and then solving this equation finalize empirical equation was developed which is discuss below.

Depth of Wetting Front

The generalized expression for radius of spread below emitting point can be expressed as:

z = αz Ksa Qpb Qcc Td

And all constant can be determined by The Buckingham Pi theorem for that we have to solved which is π1z = az (π2z)bz (π3z)cz based on experimental data finalize equation is

π1z = 1.94*π2z0.95 *π3z0.51                       (18)

zKsQc=1.94*QpQc0.95* TKs3Qc0.51            (19)

Z = 1.94  Ks0.265 T0.51 Qp0.95 Qc-0.71                   (20)

where z is the wetted soil depth (cm), Qc Is the discharge rate of continuous drip irrigation (cm3/ min), Qp Is the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ), Ks Is the saturated hydraulic conductivity of soil (cm / min) and T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

Table 3: Parameter Estimates of the Samir Ismail model for predicting Depth of wetting front.

| Parameter | Estimate | Std. Err | 95% Confidence Interv |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Bound | Upper Bound |
| az | 1.94 | 0.09 | 1.85 | 2.03 |
| bz | 0.95 | 0.11 | 0.85 | 1.06 |
| cz | 0.51 | 0.05 | 0.46 | 0.55 |

Table 4: Correlations of Parameter Estimates for Depth of wetting front.

| Variable | π2z | π3z | π1z |
| --- | --- | --- | --- |
| π2z | 1.000 | -0.008 | 0.516 |
| π3z | -0.008 | 1.000 | 0.716 |
| π1z | 0.516 | 0.716 | 1.000 |

The value of az, bz and cz for predicting wetted soil depth was calculated to be 1.94, 0.95 and 0.51. The standard error for the model was varying from 0.05 to 0.11. The lower bound and upper bound for az, bz and cz were also given in table 4. The correlation matrix for the parameter estimates for wetted soil depth is presented in table 4. The coefficient of determination was found to be 0.844.

The scatter plot of observed Vs predicted wetted soil depth front from surface was plotted in fig. 1. A good agreement between the observed and predicted values were obtained. The residual plot for various observed wetted soil depth values is also shown in fig. 2. The residues in respect of the observed values were within the tolerable limits.

Fig. 1 scatter plot of observed Vs predicted wetted soil depth.

Fig. 2: Residual plot for observed values of wetted soil depth.

Radius of Spread

The generalized expression for radius of spread below emitting point can be expressed as:

d = αd Ksa Qpb Qcc Td

And all constant can be determined by The Buckingham Pi theorem for that we have to solved which is π1d = ad (π2d)bd (π3d)cd based on experimental data finalize equation is

π1d = 2.06 *π2d-1.32 *π3z0.21                       (21)

dKsQc=2.06*QpQc-1.32* TKs3Qc0.21            (22)

d = 2.06 Ks-0.185 T0.21 Qp-1.32 Qc1.81                     (23)

where d is the wetted soil width (cm), Qc Is the discharge rate of continuous drip irrigation (cm3/ min), Qp Is the discharge rate of pulse drip irrigation which changes according to the pulse ration on/off, (cm3/min ), Ks Is the saturated hydraulic conductivity of soil (cm / min) and T Is the elapsed time (min), which is a multiple of cycle time = n. (Ton+Toff).

Table 5: Parameter Estimates of the Samir Ismail model for predicting Radius of spread.

| Parameter | Estimate | Std. Err | 95% Confidence Interv |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Bound | Upper Bound |
| ad | 2.06 | 0.31 | 1.76 | 2.37 |
| bd | -1.32 | 0.28 | -1.60 | -1.03 |
| cd | 0.21 | 0.07 | 0.14 | 0.28 |

Table 6: Correlations of Parameter Estimates for Radius of spread.

| Variable | π2d | π3d | π1d |
| --- | --- | --- | --- |
| π2d | 1.000 | -0.022 | -0.635 |
| π3d | -0.022 | 1.000 | 0.317 |
| π1d | -0.635 | 0.317 | 1.000 |

The value of az, bz and cz for predicting Radius of spread was calculated to be 2.06, -1.32 and 0.21. The standard error for the model was varying from 0.07 to 0.31. The lower bound and upper bound for az, bz and cz were also given in table 5. The correlation matrix for the parameter estimates for radius of spread is presented in table 6. The coefficient of determination was found to be 0.840.

The scatter plot of observed Vs predicted Radius of spread at surface was plotted in fig.3. A good agreement between the observed and predicted values were obtained. The residual plot for various observed Radius of spread values is also shown in fig. 4. The residues in respect of the observed values were within the tolerable limits.

Fig. 3 scatter plot of observed Vs predicted Radius of spread.

Fig. 4: Residual plot for observed values of Radius of spread.

(Boundary Conditions and Applicability of develop model are: Soil type: sandy loam, Operating time: up to 120 min, Dripper discharge: 2 LPH to 8 LPH)

CONCLUSIONS

Existing models for estimating wetting pattern dimensions are largely designed for continuous irrigation flow, and therefore do not adequately represent wetting behavior under pulsed or intermittent irrigation. As a result, there is a clear gap in predictive models for these alternative flow regimes.

Laboratory results on sandy soil indicated that, in pulse flow, the wetted width increased and wetted depth decreased as the on-time decreased for the same amount of applied water volume. This result shows the advantage of pulse flow, for reducing the deep percolation of water under the crop root zone, while obtaining a wide horizontal spread of wetting. This enables using a large emitter discharge with the same amount of water. Using large emitter discharge reduces the emitter clogging.

The dimensional analysis models developed in this study successfully estimated the wetted depth and width produced by line-source irrigation under continuous, intermittent, and pulsed flow conditions. Model performance was evaluated by comparing simulated outcomes with measurements obtained under controlled laboratory settings. Under continuous flow, the model demonstrated high reliability, achieving efficiencies of 98% for predicting wetted depth and 97% for wetted width. For intermittent flow, the efficiencies were slightly lower, at 94% and 83%, respectively. In the case of pulsed irrigation, the model predicted wetted depth and width with efficiencies of 86% and 79%. These results indicate that the proposed models are capable of representing wetting behavior across all three flow regimes and can be effectively applied for predicting wetting patterns under line-source water application.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

ASAE. (1996). Standards, Engineering Practices Data: EP405.1 – Design and installation of microirrigation systems. American Society of Agricultural Engineers.

Central Ground Water Board. (2023). Dynamic groundwater resources of India (2023 report). Ministry of Jal Shakti, Government of India.

Department of Agriculture, Farmers Welfare and Co-operation, Government of Gujarat. (2023). Annual report 2022–23. Government of Gujarat.

Government of Gujarat. (2023). Sardar Sarovar Narmada Nigam Limited: Water resources management report. Government of Gujarat.

ISO. (1991). Irrigation equipment—Emitters—Specifications and test methods (ISO 9261:1991). International Organization for Standardization.

Ismail, S. M., Moghazy, S. M., & El Marazky, M. S. A. (2014). Modified Schwartzman and Zur model for predicting wetting front under surface trickle irrigation. Soil and Water Research, 9(2), 64–71. https://doi.org/10.17221/7/2013-SWR

Jackson, R. D., & Kay, B. D. (1988). The influence of soil properties on water movement and distribution under trickle irrigation. Irrigation Science, 9(1), 1–12. https://doi.org/10.1007/BF00257940

Karmeli, D., & Peri, G. (1974). Basic principles of pulse irrigation. Journal of Irrigation and Drainage Division, 100(IR3), 309–319.

Levin, I., & Van Rooyen, P. C. (1979). The effect of discharge rate and intermittent water application by point-source irrigation on the soil moisture distribution pattern. Soil Science Society of America Journal, 43(1), 8–11. https://doi.org/10.2136/sssaj1979.03615995004300010003x

Mostaghimi, S., & Mitchell, J. K. (1983). Trickle irrigation: Effects of pulsed application on water and solute distribution. Transactions of the ASAE, 26(6), 1683–1688. https://doi.org/10.13031/2013.33833

NITI Aayog. (2022). Composite water management index (CWMI 2.0): Water resources management report. Government of India.

Oron G., 1981. Simulation of water flow in the soil under sub-surface trickle irrigation with water uptake by roots. Agric. Water Manage, 3: 179-193

Schwartzman, M., & Zur, B. (1986). Emitter spacing and geometry of wetted soil volume. Journal of Irrigation and Drainage Engineering, 112(3), 242–253. https://doi.org/10.1061/(ASCE)0733-9437(1986)112:3(242)

Zur, B. (1996). Wetted soil volume as a design objective in trickle irrigation. Irrigation Science, 16(3), 101-105.