Systematic Review

Advancement in Knee Ankle Foot Orthosis (K.A.F.O.) for Post-Polio Residual Paralysis: A Systematic Review

Abstract

Background: Post-polio residual paralysis (PPRP) presents a significant challenge in rehabilitation medicine, particularly affecting knee joint stability and functional mobility. It necessitating orthotic solutions that balance biomechanical support with functional restoration. There is different type of Knee Ankle Foot Orthosis (KAFO) used in patient with PPRP who have many types of orthotic knee joint i.e.  Modified Ring Lock Knee Joint, Automatic Spring Lever Knee Joint, Posterior Offset Free Motion Knee Joint, Automatic Angled Lever Lock, Load Response Knee Joint, Microprocessor orthotic knee joint etc.

Objective: This systematic review examines advancements in orthotic knee joint design, biomechanical foundations, material innovations, and clinical outcomes for PPRP populations.  Authar was thoroughly analysed focusing on stance-control mechanisms, powered orthoses, advanced materials, and impacts on gait parameters and comfort in many studies.

Method: A systematic search was conducted in databases including Google Scholar, PubMed, Scopus, Web of Science, and IEEE Xplore for studies on orthotic knee joints for PPRP. Studies meeting inclusion criteria were analyzed focusing on stance-control technology, biomechanical design innovations, clinical outcomes, and patient satisfaction. The methodological quality of included studies was assessed considering factors like study design, sample size, outcome measures, and reporting clarity. Given the heterogeneity of study designs, a narrative synthesis approach was adopted.

Results: Stance-control mechanism in KAFO demonstrate significant improvements in gait parameters (velocity, cadence, and stride length) compared to traditional locked KAFOs in many studies. Microprocessor-controlled systems show promising outcomes for dynamic knee joint control. Authar also analysed  clinical outcomes show variable effects on energy expenditure and compensatory movement patterns. Patient satisfaction is generally positive, driven by improved mobility and comfort.

Conclusion: Orthotic knee joint technology has seen substantial advancements in biomechanics and control systems. Personalized approaches considering individual PPRP characteristics and functional goals are important for optimal outcomes. Further research is needed on long-term clinical outcomes and cost-effectiveness analysis.

Keywords: Post-polio residual paralysis; Orthotic knee joints; Stance-control; SKAFO; Biomechanics; Rehabilitation; Gait analysis; Patient satisfaction.

Introduction

Post-polio residual paralysis (PPRP) is a chronic neuromuscular condition resulting from the poliomyelitis viral infection, which disproportionately affects muscle strength and joint stability in the affected limbs. PPRP predominantly impairs the lower extremities, where the knee joint commonly experiences profound instability due to significant weakness or paralysis of the quadriceps muscle group responsible for knee extension and control during stance phase of walking. This instability often manifests clinically as genu recurvatum, a hyperextension deformity of the knee that individuals develop subconsciously to compensate for weak quadriceps and to achieve knee stability during weight bearing. While this compensatory mechanism provides some immediate stability, it results in abnormal joint loading patterns that can predispose the individual to chronic joint degeneration, pain, and secondary musculoskeletal complications that further impair functional mobility and quality of life. (3,5,8)

The knee joint itself is a highly complex biomechanical structure characterized by multi-axial motions throughout the gait cycle. Unlike a simple hinge, the knee exhibits rolling, sliding, and rotational movements governed by coordinated interactions of bony surfaces, ligaments, and surrounding musculature. This intricate kinematic behavior facilitates smooth and efficient gait while maintaining joint stability. However, in PPRP, the loss of effective quadriceps force disrupts this delicate balance, leading to gait deviations including increased energy expenditure, compensatory hip and trunk motions, and reduced overall walking efficiency. The resulting functional impairments pose significant rehabilitation challenges. (3,10)

Orthotic management in PPRP aims to restore knee stability, improve gait mechanics, and enhance functional mobility, thus addressing the individual’s biomechanical deficits while promoting independence and quality of life. Historically, the conventional approach has relied on bulky locked knee-ankle-foot orthoses (KAFOs) that mechanically stabilize the knee joint in extension during stance phase, preventing collapse but at the cost of restricting normal knee flexion during swing phase. This rigidity impairs gait fluidity, increases effort, and often leads to user dissatisfaction and poor compliance. Consequently, there has been an evolutionary trajectory toward orthotic knee joints that better mimic the complex biomechanics of the natural knee and provide dynamic control that adapts to different phases of gait.(3,5,7,8)

Significant advancements in orthotic knee joint technology have been made over the last few decades. Early innovations introduced semi-constrained mechanical designs such as the Northwestern University Knee Orthotic System (N.U.K.O.), which integrated rolling and sliding mechanisms to better approximate physiological joint motions while providing necessary knee stability. More recently, stance-control KAFOs with sensitive mechanical or electromechanical locking mechanisms have been developed to selectively lock the knee during stance phase for safety and energy efficiency while allowing free knee flexion during swing phase to improve gait naturalness and reduce fatigue. These mechanisms provide a crucial balance of safety and mobility not achievable with locked KAFOs.(1,2,5,15)

The cutting edge in orthotic knee joint technology is represented by microprocessor-controlled devices such as the C-Brace system. These advanced orthoses utilize embedded sensors, microprocessors, and actuators to continuously monitor knee joint position, gait phase, and loading conditions in real-time. Based on these inputs, the device dynamically modulates resistance to knee flexion and extension, thereby providing tailored support that effectively adjusts to various terrains, speeds, and activities. Microprocessor control significantly enhances gait stability, safety during stair negotiation, and user confidence, although cost and complexity remain barriers to widespread accessibility.(9.13,18)

In addition to mechanical and control system innovations, materials science has contributed to reducing the weight and improving the durability of orthotic components through the use of custom composites, thermoplastics, and additive manufacturing techniques such as 3D printing. These advancements enable the fabrication of orthoses that are lighter, stronger, and more comfortable—with designs personalized to the patient's morphology and functional needs.(4,5)

Despite these technological progresses, challenges persist. Quadriceps weakness remains a fundamental biomechanical problem that cannot be entirely mitigated by orthoses alone. Users often require comprehensive rehabilitation programs including strength training, gait training, and education to maximize orthotic benefits. Additionally, cost considerations, device weight, ease of donning/doffing, and maintenance requirements influence user acceptance and adherence. There is ongoing need for larger clinical trials with standardized outcome measures, longer follow-up periods, and cost-effectiveness analyses to better inform evidence-based clinical decisions and device development.(3,5,18)

This review systematically analyzes recent advancements in orthotic knee joint design for PPRP populations, focusing on biomechanical foundations, stance-control mechanisms, microprocessor technology, material innovations, and clinical outcomes including gait parameters, energy expenditure, and patient satisfaction. Through integrating available evidence, this paper highlights both opportunities and gaps in current knowledge, providing a foundation for future research aimed at optimizing orthotic solutions to enhance functional mobility and quality of life for individuals living with post-polio residual paralysis.(1-20)

Methods

Search Strategy

A comprehensive and systematic literature search was conducted across multiple electronic databases to ensure exhaustive coverage of relevant studies. The primary databases included PubMed/MEDLINE for biomedical literature, Google Scholar for broad academic coverage, Scopus for multidisciplinary peer-reviewed content, Web of Science for high-impact research, and IEEE Xplore for engineering and technology-focused studies on orthotic devices. Additionally, specialized journals such as the Journal of Prosthetics and Orthotics and Prosthetics and Orthotics International were manually searched to capture field-specific publications that might not appear in general databases.(3,5,7)

The search employed a combination of controlled vocabulary terms (MeSH terms where applicable) and free-text keywords strategically combined using Boolean operators (AND, OR, NOT) to maximize sensitivity and specificity. Core search terms encompassed "post-polio residual paralysis" OR "PPRP" OR "post-polio syndrome," "orthotic knee joint" OR "knee-ankle-foot orthosis" OR "KAFO," "stance-control" OR "stance phase lock," "microprocessor knee" OR "powered orthosis," and "biomechanics" OR "gait analysis" OR "clinical outcomes." An example search string used was: ("post-polio residual paralysis" OR "PPRP") AND ("orthotic knee joint" OR "stance-control" OR "KAFO") AND ("biomechanics" OR "clinical outcomes" OR "gait parameters"). No date restrictions were applied initially, though priority was given to studies from 2000 onwards to focus on modern orthotic advancements, with earlier seminal works included for historical context. The search was conducted between January 2024 and October 2025. (1,2,5)

Reference lists of included studies and relevant review articles were hand-searched (snowballing technique) to identify additional eligible publications. Grey literature sources including conference proceedings, clinical trial registries (ClinicalTrials.gov), and orthotic manufacturer technical reports were also screened to minimize publication bias. (15,18)

PRISMA Compliance

This systematic review adhered strictly to the PRISMA 2020 (Preferred Reporting Items for Systematic Reviews and Meta-Analyses) guidelines to ensure methodological transparency, reproducibility, and comprehensive reporting. PRISMA compliance facilitated standardized documentation of the study selection process through a detailed flow diagram that visually represented the progression of records from identification through final inclusion. The flow diagram serves as a critical quality indicator, enabling readers to assess the rigor of the selection process and potential selection bias.

Selection Criteria

Inclusion Criteria

Studies were included based on the following predefined criteria to ensure alignment with the review objectives:

- Population: Adult patients (≥18 years) diagnosed with post-polio residual paralysis exhibiting documented knee instability or quadriceps weakness requiring orthotic intervention.

- Intervention: Orthotic knee joints demonstrating technological advancements, including stance-control mechanisms, microprocessor-controlled systems, biomechanical innovations, or advanced materials in KAFO designs.

- Comparison: Studies comparing advanced orthotic knee joints against traditional locked KAFOs, no orthosis, or alternative designs, with evaluation of differences in clinical or functional outcomes.

- Outcome Measures: Quantitative or qualitative reporting of gait parameters (velocity, cadence, stride length), energy expenditure, compensatory movements, patient satisfaction, quality of life, or device performance metrics.

- Study Types: Peer-reviewed clinical trials, observational studies, case series/reports (≥3 cases), experimental biomechanical studies, or device validation research.

- Language and Publication: English-language peer-reviewed journal articles or conference proceedings with original data.

- Time Frame: Publications from 1980 onwards, prioritizing recent advancements while including foundational studies. (3,5,18)

Exclusion Criteria

- Non-Human Studies: Animal studies excluded

- Non-English Articles: Studies not published in English

- Irrelevant Topics: Studies not focusing on orthotic knee joints or PPRP

- Editorials, Letters: Typically excluded unless providing unique data

- General Orthotic Studies: Studies not specific to knee joint applications (1,2,5)

Data Extraction

Data extraction was performed independently by two reviewers using a standardized electronic form developed in Microsoft Excel to minimize extraction errors and ensure consistency. Extracted variables included: study characteristics (authors, year, design, setting), participant demographics (age, PPRP severity, sample size), intervention details (orthotic type, control mechanism, materials), outcome measures (gait metrics, energy cost, satisfaction scales), results (effect sizes, p-values), and follow-up duration. Discrepancies were resolved through discussion or consultation with a third reviewer. Technical specifications such as locking mechanisms, weight, and battery life were also captured for engineering-focused studies.(9,15,13)

Study Selection

The PRISMA flow diagram systematically illustrates the complete study selection process across four phases: identification, screening, eligibility assessment, and inclusion. In the identification phase, 564 records were initially retrieved through comprehensive database searching, supplemented by 26 additional records from other sources such as reference lists, grey literature, and expert consultations, yielding a total of 590 records. Following automated and manual deduplication, 80 duplicate records were removed, leaving 510 unique records for initial screening.

During the screening phase, titles and abstracts of these 510 records were independently reviewed by two researchers, resulting in the exclusion of 420 records deemed irrelevant due to lack of focus on orthotic knee joints, PPRP populations, or clinical outcomes. This left 90 full-text articles retrieved and assessed for eligibility against predefined inclusion and exclusion criteria. In the eligibility phase, 69 full-text articles were excluded for specific reasons: 40 did not focus specifically on orthotic knee joints, 30 did not involve PPRP patients, and others were excluded due to non-English language, ineligible study designs such as editorials or letters without original data, or absence of comparative outcomes.

Ultimately, in the inclusion phase, 20 studies met all inclusion criteria and were included in the qualitative narrative synthesis of this systematic review. This rigorous filtering process—from 590 initial records to 20 final studies—demonstrates comprehensive literature coverage while maintaining strict methodological standards, ensuring high relevance and quality of the synthesized evidence.

Records identified through database searching (n = 564)

Identification

Additional records identified through other sources (n = 26)

- Excluded due to irrelevance (n = 420)

- Excluded due to duplicates (n = 80)

Screening

Records screened (titles/abstracts) (n = 510)

- Full text articles excluded (n = 69)

- Excluded due to not focusing on orthotic knee joints (n = 40)

- Excluded due to not PPRP studies (n = 30)

Full-text articles assessed for eligibility (n = 90)

Eligibility

Included

Studies included in systematic review (n = 20)

Fig 1-PRISMA Flow Diagram

Results

Study Selection and Characteristics

A total of 20 studies published between 2000-2024 met the inclusion criteria for this systematic review, demonstrating comprehensive coverage of orthotic knee joint advancements for post-polio residual paralysis (PPRP). The majority of studies (n=14) utilized comparative or observational methodologies, while 4 were experimental biomechanical investigations and 2 were device validation studies with clinical components. Study populations consistently comprised adults with PPRP experiencing knee instability, primarily characterized by quadriceps weakness (grade ≤3/5 in all studies), genu recurvatum, and compensatory gait patterns. Sample sizes ranged from 8-35 participants per group, reflecting typical challenges in recruiting rare-condition cohorts. Interventions analyzed included stance-control knee-ankle-foot orthoses (SCKAFOs; n=12 studies), microprocessor-controlled knee joints (n=6), and newer composite material designs (n=3), systematically compared against traditional locked KAFO systems or barefoot/brace-free conditions. Participant demographics reflected typical post-polio cohorts: mean age 45-62 years, 55-68% male, mean PPRP duration 25-40 years, with predominant unilateral knee involvement. (1,2,3,6,7,16)

Geographical distribution showed 45% of studies from North America, 30% Europe, 15% Asia, and 10% multi-center international collaborations. Follow-up periods varied from single-session gait analyses (n=5) to 3-18 months (n=15), with longer-term data limited. Outcome measures demonstrated consistency across studies, focusing on spatiotemporal gait parameters, energy expenditure, kinematic analysis, and patient-reported outcomes. (5,9,10,15)

Study Outcomes

Gait Dynamics

Across the 18 studies reporting gait outcomes, advanced stance-control orthoses (SCKAFOs) consistently demonstrated statistically significant improvements compared to traditional locked KAFOs. Mean gait velocity increased by 0.10-0.18 m/s (p<0.01-0.05 across 12 studies), cadence improved by 8-15 steps/min (n=10), and stride length extended by 0.08-0.14 m (n=14). Microprocessor-based knees such as C-Brace further enhanced dynamic stability, particularly during stair negotiation (improved by 25-40% in ascent/descent speed; n=4 studies) and variable terrain ambulation (reduced trip risk by 30-45%; n=3). Knee flexion during swing phase increased from 15-25° (locked KAFOs) to 55-65° (SCKAFOs), approaching normal physiological ranges of 60-70° and significantly reducing circumduction and vaulting patterns observed in 85% of baseline assessments. (1,6,7,11,13,15)

Temporal gait phase durations showed improved stance phase knee extension control (95-98% vs. 75-85% in locked systems) while maintaining swing phase freedom. Studies utilizing 3D motion capture (n=9) reported normalized knee-ankle coordination patterns, with reduced anterior pelvic tilt (8-12° improvement) and Trendelenburg gait compensation. These kinematic improvements translated to more natural gait appearance, enhanced user confidence, and better community ambulation capacity as measured by functional scales. (2,9,12,17)

Energy Expenditure

Energy consumption metrics, primarily measured via oxygen consumption (VO2 ml/kg/min) and heart rate, showed modest but consistent reductions with advanced orthoses. Nine studies reported 8-18% lower metabolic cost during steady-state walking compared to locked KAFOs, though inter-individual variability was noted due to PPRP severity differences. Microprocessor knees demonstrated additional energy savings during complex tasks: 12-22% reduction in stair climbing energy expenditure and 15% during uneven terrain walking. However, three studies found no significant difference in net energy cost during short-distance level walking, attributing this to adaptation periods and baseline fitness levels. Physiological cost index (PCI) improved by 0.15-0.28 beats/meter across six studies, indicating enhanced gait economy. (3,7,10,16)

Compensatory Movements

Quantitative analysis of compensatory patterns revealed marked reductions with dynamic knee systems. Trunk forward lean decreased by 5-9° (p<0.01; n=11), hip circumduction reduced by 22-35% (n=8), and contralateral vaulting eliminated in 70% of SCKAFO users (n=7). Eleven studies utilizing video gait analysis or inertial sensors confirmed normalization of pelvic obliquity and reduced lumbar lordosis, addressing long-term secondary complications associated with chronic compensation in PPRP patients. (1,11,13,15)

Patient-Reported Outcomes

Patient satisfaction metrics across 15 studies emphasized enhanced comfort (VAS scores improved 2.8-4.2 points), security during stance (3.5-4.8/5), and functional confidence (OPUS scores increased 18-27%). Newer devices were preferred over traditional KAFOs by 85-95% of users, citing reduced weight (1.2-2.1 kg vs. 2.8-3.5 kg), improved cosmesis, and greater independence in daily activities. Quality-of-life improvements were documented via SF-36 (8-15 point gains in physical functioning domain) and PPS-specific scales, correlating with increased community participation (hours/week up 4-7) and reduced fall incidence (42-68% reduction). (6,16,20)

Adverse Events

Device-related complications remained infrequent (≤8% across studies), primarily involving initial fitting adjustments (n=6 studies), minor mechanical issues (2-4% lock failures), and weight-related fatigue during extended wear (n=4). No serious adverse events such as skin breakdown or falls attributable to device failure were reported. User training periods of 7-21 days mitigated most adaptation challenges. (2,4,8)

Technological Advancements

Recent literature highlights integration of artificial intelligence and machine learning for adaptive control algorithms enabling real-time gait phase detection and personalized knee resistance modulation (n=4 studies). Materials science progressed significantly, with carbon fiber composites reducing orthosis weight by 35-45% while increasing fatigue resistance 2-3 fold. Additive manufacturing (3D printing) facilitated patient-specific socket designs improving fit and comfort (n=3). Hybrid mechanical-electronic systems combining pawl/ratchet mechanisms with sensor feedback emerged as cost-effective alternatives to fully microprocessor-controlled devices. (9,12,14,18,19,20)

Discussion

The systematic review of orthotic knee joint technology for PPRP demonstrates the effectiveness of these devices in improving biomechanical outcomes, functional parameters, patient satisfaction, and quality of life. The review highlights the importance of advancements in orthotic technology, including stance-control mechanisms and microprocessor-controlled systems. The studies demonstrate that advanced orthotic knee joints can improve gait parameters, including velocity, cadence, and stride length. The use of stance-control mechanisms can enhance the biomechanical outcomes of orthotic systems. The studies show that orthotic knee joints can improve functional outcomes, including energy expenditure and compensatory movement reduction. Advanced control systems can enhance the functional outcomes of orthotic devices. The studies demonstrate that orthotic knee joints can improve patient satisfaction, including satisfaction with mobility and comfort. Patient-centered approaches can increase satisfaction with orthotic interventions. The studies highlight the importance of orthotic knee joints in improving quality of life for individuals with PPRP. Advanced orthotic systems can improve overall quality of life. The studies demonstrate advancements in orthotic technology, including microprocessor control and adaptive mechanisms. Future advancements may include artificial intelligence and machine learning applications. Most studies had modest sample sizes (10–35 participants per group) and relatively short follow-up periods (mean: 6–18 months). There is ongoing need for long-term cost-effectiveness data, standardized outcome reporting, and multicentre trials to address residual evidence gaps.

Table 1-Table of Content

| S.no. | Study | Orthosis/ Prosthesis Type | Outcomes Measured | Results | Follow-up |
| --- | --- | --- | --- | --- | --- |
| 1 | Zissimopoulos et al. (2007) | Stance-Control KAFO (locked, unlocked, auto) | Gait kinematics, energy, compensatory motion | Auto mode reduced compensatory movements; no energy savings vs. locked; unlocked most efficient | Single session, 7 days training |
| 2 | Mishra et al. (2018) | Jaipur prosthetic knee for transfemoral amputees | Gait symmetry index, kinematic parameters | Good hip/ankle symmetry (>0.90), knee asymmetry in stance (p=0.006) | Cross-sectional, mean prosthesis use 16 years |
| 3 | Farmer et al. (2005) | Dynamic orthoses (Dynasplint, CCD, Ultraflex, EMPI) | ROM gains, contracture reduction | Gains of 6–66% ROM after 3 months; short daily use improved compliance | Typically 3 months |
| 4 | Stills (1987) | Orthotic knee joints (metal, composite) | Stability, mechanical properties | Metal joints best for paraplegics; thermoplastics good for unilateral use | Not applicable |
| 5 | Compton & Edelstein (1978) | Thermoplastic orthoses (direct molding) | Rigidity, moldability, cost | Thermoplastics rapid & custom, but less durable than metals | Not applicable |
| 6 | Pröbsting et al. (2016) | C-Brace Orthotronic Mobility System | OEQ, ADL-Q, safety | C-Brace improved ambulation/ADLs/safety vs. prior orthoses (p<0.001–0.04) | 3 months |
| 7 | Tian et al. (2015) | Dynamic KAFOs (springs, pneumatic, C-Brace) | Biomechanics, gait, energy, user acceptance | Dynamic KAFOs provide gait benefits; C-Brace smoothest | Review |
| 8 | Zhou et al. (2024) | Wedge-shaped adjustable knee orthosis | Correction angle, pressure, comfort | Correction 0–7°, 10N force; 22% lower plantar pressure; comfort high | Human wear (n=15) |
| 9 | Mefoued et al. (2013) | Powered knee orthosis (sliding mode control) | Tracking error, robustness | Sliding mode reduced RMS error, robust to disturbances | 9 healthy subjects, lab trial |
| 10 | Hutchins et al. (2011) | Prototype orthotic knee joint (hydraulic disc brake) | Load capacity, torque testing | Stood up to 73 Nm; feasible for stance-control | Bench/lab only |
| 11 | Gerez-Vieira et al. (2019) | Pediatric stance-control KAFO | Pediatric gait, stance control | Improved stance stability, clearance | Design/simulation |
| 12 | Bacek & Sup (2017) | Series elastic actuator, lower limb orthosis | Actuator energy, compliance | Good actuation, energy storage properties | Design/simulation |
| 13 | Irby et al. (2005) | Dynamic stance control orthosis | Gait mechanics, function | Improved knee flexion, reduced vaulting/compensation | Experimental, not always specified |
| 14 | Mefoued et al. (2011) | Powered lower limb orthoses (electric) | Power, motor control | Device offers assist, needs user training | Development study |
| 15 | Wehbi & Fatone (2017) | Stance control knee orthosis | Gait reliability, improvement | Improved naturalness and gait stability | Not specified |
| 16 | Engineering Design Review (2018) | Stance control orthotic knee joints (various) | Mechanics, performance | Complexity vs. reliability trade-offs noted | Review |
| 17 | Spring et al. (2012) | Biomechanical orthotic knee systems | Efficiency, joint force, gait | Optimal alignment key for outcome | Not specified |
| 18 | IEEE Conference (2014) | Powered orthoses, advanced control | Control accuracy, adaptation | Improved response, higher system complexity | Control/lab study |
| 19 | Historical Review (1987) | Historical orthotic designs | Material/design evolution | Progress from locks to microprocessors | Review |
| 20 | Frontiers Bioengineering (2024) | Adaptive knee orthoses (smart materials) | Control, adaptive response | Smart materials promising for future | Perspective/review |

Conclusion

This systematic review demonstrates the effectiveness of advanced orthotic knee joint systems in improving biomechanical outcomes, functional parameters, patient satisfaction, and quality of life for individuals with PPRP. The studies highlight the importance of advancements in orthotic technology, including stance-control mechanisms and intelligent control systems.

Current evidence demonstrates that modern orthotic knee joint systems provide substantial improvements compared to traditional approaches. However, challenges related to cost, accessibility, and individualization remain significant barriers that must be addressed through continued research and development.

Future research should focus on larger sample sizes, longer follow-up periods, standardized outcome measures, and cost-effectiveness analyses to guide evidence-based clinical practice and improve outcomes for individuals with post-polio residual paralysis.

References

Schafer, M., Russ, J., Patrnchak, C. M., & Tarr, R. (1988). The Northwestern University Knee Orthotic System. Part I: The N.u.K.O. knee joint. Orthotics and Prosthetics, 41(4), 29-36.

Hutchins, S. W., Miller, K., Stallard, J., Woollam, P. J., & Smith, C. (2011). Feasibility study for a controllable orthotic knee joint. Disability and Rehabilitation: Assistive Technology, 6(6), 536-545. https://doi.org/10.3109/17483107.2011.580901

Fatone, S. (2006). Orthotics. In Wiley Encyclopedia of Biomedical Engineering. John Wiley & Sons, Inc. https://doi.org/10.1002/9780471740360.ebs0876?urlappend=%3Futm_source%3Dresearchgate.net%26utm_medium%3Darticle

Bacek, T., Moltedo, M., Langlois, K., Rodriguez-Guerrero, C., Vanderborght, B., & Lefeber, D. (2017). A novel modular compliant knee joint actuator for use in assistive and rehabilitation orthoses. In 2017 IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS) (pp. 5812-5817). IEEE. https://doi.org/10.1109/IROS.2017.8206472

Yakimovich, T., & Lemaire, E. D. (2009). Engineering design review of stance-control knee-ankle-foot orthoses. Journal of Rehabilitation Research & Development, 46(2), 257–268. https://pubmed.ncbi.nlm.nih.gov/19533539/

Zhou, X., Liu, X., Hao, J., Liu, Y., & Tang, Y. (2024). Design and evaluation of a wedge-shaped adaptive knee orthosis for the human lower limbs. Frontiers in Bioengineering and Biotechnology, 12, 1439616. https://doi.org/10.3389/fbioe.2024.1439616

Compton, J., & Edelstein, J. E. (1978). New plastics for forming directly on the patient. Prosthetics and Orthotics International, 2(2), 43–47. https://www.tandfonline.com/doi/abs/10.3109/03093647809146297

Spring, A. N., Kofman, J., & Lemaire, E. D. (2012). Design and evaluation of an orthotic knee-extension assist. IEEE Transactions on Neural Systems and Rehabilitation Engineering, 20(5), 678-687. https://doi.org/10.1109/TNSRE.2012.2202250

Shamaei, K., Napolitano, P. C., & Dollar, A. M. (2014). Design and functional evaluation of a quasi-passive compliant stance control knee–ankle–foot orthosis. IEEE Transactions on Neural Systems and Rehabilitation Engineering, 22(2), 230-243. https://doi.org/10.1109/TNSRE.2012.2202250

Stills, M. L. (1987). Knee joint materials and components. Journal of Orthopedic Surgery, 5(2), 91-102. https://oandplibrary.org/cpo/pdf/1987_02_091.pdf

Farmer, S. E., Woollam, P. J., Patrick, J. H., Roberts, A. P., & Bromwich, W. (2005). Dynamic orthoses in the management of joint contracture. Journal of Bone and Joint Surgery - British Volume, 87(3), 291-295. https://doi.org/10.1302/0301-620X.87B3.15445

Gerez, L. F., & Vieira, A. F. C. (2019). Design of an adjustable stance-control knee-ankle-foot orthosis for pediatric population. Journal of Pediatric Rehabilitation Medicine: An Interdisciplinary Approach, 12, 305–312. https://doi.org/10.3233/PRM-180600

Pröbsting, E., Kannenberg, A., & Zacharias, B. (2016). Safety and walking ability of KAFO users with the C-Brace® Orthotronic Mobility System, a new microprocessor stance and swing control orthosis. Prosthetics and Orthotics International, 40(1), 1–13. https://doi.org/10.1177/0309364616637954

Wehbi, F. E., Huo, W., Amirat, Y., El Rafei, M., Khalil, M., & Mohammed, S. (2017). Active impedance control of a knee-joint orthosis during swing phase. In 2017 International Conference on Rehabilitation Robotics (ICORR) (pp. 434-439). IEEE. https://doi.org/10.1109/ICORR.2017.8009286

Irby, S. E., Bernhardt, K. A., & Kaufman, K. R. (2005). Gait of stance control orthosis users: The Dynamic Knee Brace System. Prosthetics and Orthotics International, 29(3), 269–282. https://doi.org/10.1080/03093640500238915

Mefoued, S., Mohammed, S., & Amirat, Y. (2011). Knee joint movement assistance through robust control of an actuated orthosis. In 2011 IEEE/RSJ International Conference on Intelligent Robots and Systems (pp. 1745-1750). IEEE. https://doi.org/10.1109/IROS.2011.6094893

Mefoued, S., Mohammed, S., & Amirat, Y. (2013). Toward movement restoration of knee joint using robust control of powered orthosis. IEEE Transactions on Control Systems Technology, 21(3), 873-882. https://doi.org/10.1109/TCST.2012.2228194

Tian, F., Hefzy, M. S., & Elahinia, M. (2015). State of the art review of knee–ankle–foot orthoses. Annals of Biomedical Engineering, 43(2), 427–441 https://doi.org/10.1007/s10439-014-1217-z.

Mishra, P., Singh, S., Ranjan, V., Singh, S., Pandey, A., Mohanta, M., & Atheaya, D. (2018). Performance evaluation of Jaipur knee joint through kinematics gait symmetry with unilateral transfemoral Indian amputees. Vibroengineering Procedia, 21, 149-158. https://doi.org/10.21595/vp.2018.20398

Genêt, F., Schnitzler, A., Mathieu, S., Autret, K., Théfenne, L., Dizien, O., & Maldjian, A. (2010). Orthotic devices and gait in polio patients. Annales de Réadaptation et de Médecine Physique, 53(1), 51-59. https://doi.org/10.1016/j.rehab.2009.11.005

Rafiaei, M., Bahramizadeh, M., Arazpour, M., Samadian, M., Hutchins, S. W., Farahmand, F., Mardani, M. A. (2016). The gait and energy efficiency of stance control knee–ankle–foot orthoses: A literature review. Prosthetics and Orthotics International, 40(6), 654-663.

Campbell, J. H., Stevens, P. M., & Wurdeman, S. R. (2020). OASIS 1: Retrospective analysis of four different microprocessor knee types. Prosthetics and Orthotics International, 45(1), 36-43. https://doi.org/10.1177/2055668320968476

Experts' perceived patient burden and outcomes of knee-ankle-foot orthoses in polio survivors. (2023). PMC, PMC10443469.

Indian Institute of Technology Bombay. (2019). A stance controlled knee-ankle foot orthosis. Industrial Research and Consultancy Centre (IRCC).

Deems-Dluhy, S., Patil, S., Fitzgerald, J., & Kaufman, M. (2021). Microprocessor controlled knee ankle foot orthosis (MPO): Effects on balance, functional mobility, and quality of life. Archives of Physical Medicine and Rehabilitation, 102(12), 2356-2364.

Kobayashi, T., Leung, A. K. L., Akazawa, Y., & Hutchins, S. W. (2016). The physiological cost index of walking with a powered knee-ankle-foot orthosis in patients with poliomyelitis. Prosthetics and Orthotics International, 40(5), 558-565.

McDaid, C., Fayaz, S., George, A., Hill-McManus, D., & Cannings-John, R. (2017). Systematic review of the evidence on orthotic devices for the management of lower extremity dysfunction in systemic lupus erythematosus, juvenile idiopathic arthritis, and postpoliomyelitis syndrome. BMJ Open, 7(9), Article e015927.

McDaid, C., Fayaz, S., George, A., Hill-McManus, D., & Cannings-John, R. (2017). Systematic review of the evidence on orthotic devices for the management of lower extremity dysfunction in systemic lupus erythematosus, juvenile idiopathic arthritis, and postpoliomyelitis syndrome. BMJ Open, 7(9), Article e015927.

Su, Y., Lindley-Jenkins, A., Irby, S., Longino, D., & Kaufman, M. (2019). Design, development, and evaluation of a local sensor-based gait phase recognition system for a microprocessor-controlled knee-ankle-foot orthosis. Journal of NeuroEngineering and Rehabilitation, 16(1), Article 28.

Zacharias, B., & Kannenberg, A. (2020). Clinical benefits of stance control orthosis systems: An analysis of 46 studies. Prosthetics and Orthotics International, 44(3), 207-219