Assessment of Physico-chemical, Heavy Metals and Microbial Characteristics of a Surface Water Resource: The Rumuosi Borrow Pit, Port Harcourt, Nigeria

ABSTRACT

Aim: Rumuosi borrow pit water quality was assessed to determine its physico-chemical, heavy metals, and microbial properties and fitness for human use.

Study Design: A Descriptive research design was used. One random sample point on each side of the study area was located and sampled following standard procedures.

Place and Duration Study: The Study was carried out on the Rumuosi borrow pit located at the intersection of the East-West Road with the New Port Harcourt Airport Road, Port Harcourt in southern Nigeria.

Methodology: Grab sampling technique was used for sample collection. Water samples were collected using clean and sterile bottles and labeled, placed in an ice-packed coolant bag, and taken to AnalConcept laboratory for physico-chemical, heavy metals, and microbial properties analyses.

Results: Water colour ranged from 217 Pt-Co to 298 Pt-Co, odour: unobjectionable, pH: 6.5 – 6.6, biochemical oxygen demand: 0.42 mg/L– 0.79 mg/L, dissolved oxygen: 4.7 – 5.3 mg/L, phosphate: 0.15 mg/L– 0.27 mg/L, nitrate: 0.3 mg/L– 0.7 mg/L, and total hydrocarbon content: 0.01 mg/L. Alkalinity: 40 mg/L - 100 mg/L, copper (Cu): 0.026 mg/L– 0.047 mg/L, mercury (Hg), lead (Pb): 0.001 mg/L, and total coliform and faecal coliform counts: ≥ 2,400 MPN/100 mL. Mercury (Hg), lead (Pb), and copper (Cu) levels and coliform counts were at a risk threshold.

Conclusion: Water physico-chemical properties were below the standard provisions and good for use as a fish pond and irrigation, but levels of copper (Cu), mercury (Hg), lead (Pb), total and faecal coliforms were at hazardous levels. Therefore, the status of Rumuosi borrow pit water is of bad quality and unfit to serve as a source of clean water supply for human use.

Keywords: Biochemical Oxygen Demand, dissolved oxygen, surface water body, Niger Delta, water pollution, water quality

INTRODUCTION

In most developing countries, development projects such as road construction, erosion control, building construction, etc., are common. These projects excavate acres of massive open pits on land, which are non-natural depressions called borrow pits (Lawal, 2011). In most cases, the borrow pits are abandoned after the closure of excavation, and these become unmanaged man-made earthen ponds or are used as waste dump pits as a means of reclaiming such degraded land by waste landfilling method.

In Nigeria, several such abandoned borrow pits exist nationwide (Nwachukwu and Osoro, 2013; Nwachukwu et al., 2017). One such abandoned borrow pit in Rivers State is at Rumuosi, a small town located on the outskirts of Port Harcourt city. This borrow pit was excavated over four decades ago during the construction of the Port Harcourt section of Nigeria’s East-West Road. After the completion of the road construction, the borrow pit was not put to any use. In shallow groundwater areas, borrow pits constantly receive and store water from run-off, precipitation, and recharged water through seepage from surrounding areas and groundwater, like earthen ponds (Van Steenbergen et al., 2021). In line with UN-SDG 6 of Agenda 2030, borrow pits can be managed towards achieving the sustainable supply of clean water for human use in aquaculture (Van Steenbergen, 2017). Ecologically, borrow pits provide habitat for diverse species of hydrophytes, amphibians, reptiles, and other animals and therefore support biodiversity conservation (Rein et al., 2013). However, despite the actual and potential importance of borrow pits, they are affected by the pollution of their water content. Pollution of borrow pit water damages its quality, which may be harmful for the ecosystem it supports.

A review of some literature showed that no study has been undertaken to assess and determine the water quality of the Rumuosi borrow pit water. Given this unknown water quality status of this borrow pit, this study was carried out to ascertain the background levels of its physico-chemical, heavy metals, and microbiological properties of its water using a multi-criteria ecological assessment and establish its suitability and safety for human use and conservation purposes.

MATERIALS AND METHODS

Study Area

This study was carried out in the Rumuosi borrow pit located beside the intersection of the East-West Road with Prof. Tam David-West Boulevard, popularly known as the New Airport Road in Port Harcourt (Figures 1 & 3) at Rumuosi. This area is located at 40 52’ N, 60 57’ E (Figure 2), in kilometre 5 of the East-West Road from Choba, Rivers State. The borrow pit has the dimensions of 634.3 m in length, 326.2 m in width, and 6.8 m deep.

The Rumuosi borrow pit contains water year-round. It has a church and residential buildings on its northern side; the East-West Road on its southern end; Prof. Tam David-West Boulevard on its eastern boundary, and a public recreation facility, plastic/scrap metals dump, and an automobile repairs workshop on the western boundary. The borrow pit floor is covered by vegetation of herbaceous hydrophytes and some trees and shrub plants. It also has some economic plants such as Bambusa vulgaris, Elais guineensis, Musanga cecropioides, Musa paradisiaca, Psidium guajava, etc. on the sides (Plate 1). Rumuosi town is beside Port Harcourt metropolis, which is in the southern region of Nigeria, a region with a warm-wet climate.

Figure 1: Aerial view of Obiri Ikwerre Roundabout/Flyover and the study area

Plate 1: Broad view of Rumuosi Borrow Pit

Sampling Design

Four random sample points: A, B, C, and D (Plate 1), located 1.5 m from the shoreline on each side of the borrow pit, were taken for sample collection. These points are marked in Figure 2, and their geographical positioning system (GPS) coordinates are given in Table 1.

Sample points

Sample points

Figure 2: Map of study area showing distribution of sample points

Table 1: Sample points coordinates

| Sample Point | Latitude | Longitude |
| --- | --- | --- |
| A | 4.877163oN | 6.953375oE |
| B | 4.877344oN | 6.954153oE |
| C | 4.877562oN | 6.953651oE |
| D | 4.877618oN | 6.952896oE |

Samples Collection

The Grab sampling method was used to collect water samples. Three (3) 500 mL capacity clean and sterile polyvinyl chloride (PVC) bottles were filled with water sample from sample point A and bulked together to form an integrated sample, which was labeled immediately after collection. Water sample for odour and biochemical oxygen demand (BOD) analyses was collected with a 350 mL amber colour BOD bottle following the standard procedure. The water samples collected and labeled immediately after collection and labeling were placed in a coolant bag containing ice blocks to prevent chemical status alteration. The same procedure as described above was followed to collect samples from the other sample points. After sample collection, the ice-packed coolant bag, with the water samples, was taken to AnalConcept Laboratory for analysis.

Variables of the Study

Water colour was determined through the Platinum-cobalt method by visual comparison of the water sample colour with standard solutions of potassium hexachloroplatinate and cobalt chloride solution. Water odour was determined by olfactory sensory test after the addition of potassium hydroxide (KOH) solution to the sample and heating over a water bath to 60 °C. Water pH was determined by the electrometric method with a digital pH meter (Hanna, HI 2210 model), and biochemical oxygen demand (BOD) was determined following the Winkler method. Total organic compound (TOC) was assessed using the method of Walkley-Black wet oxidation method with an eisco photocolorimeter (El 911 digital model). Phosphate content was determined using the Ascorbic Acid method by measuring the sample phosphate concentration absorbance with a spectrophotometer (GBC Flame AAS Avanta 41542 model), nitrate content was ascertained by the spectrophotometric method, while Dissolved Oxygen (DO) was determined using the Winkler method. Water turbidity was measured by the nephelometric method using a Tanco digital nephelometer (EE-027 model), and alkalinity was measured by the titrimetric method. Sample total hydrocarbon content, THC, was measured after extraction using hexane as extraction solvent, and the concentration was measured by gas chromatography-flame ionization detection, GC-FID, with a Gas Chromatograph, MAK 2191 model. Sample heavy metal (Cu, Hg, and Pb) contents were measured through graphite furnace atomization absorption spectroscopy (GF-AAS) using a spectrophotometer (Shimadzu AA-6800 model), while coliform bacteria count, i.e., total and faecal Coliform Counts, were determined using the Most Probable Number (MPN) test.

Data Presentation

Data obtained on the variables examined were compared with the DPR/FMEnv (2011) general provisions for surface waters and presented in tables and bar charts using the Microsoft Office Excel software 2016 programme.

RESULTS AND DISCUSSION

Water Physico-chemical Properties

Figure 3 displays water colour results. Sample points A – C had colour values of 298 Pt-Co, 275 Pt-Co, and 277 Pt-Co, respectively, while 217 Pt-Co was recorded in point D. The highest value (298 Pt-Co) was obtained in point A, while the least value (217 Pt-Co) was recorded in point D.  The Pt-Co scale is a measure colouration of water, specifically, yellow colour, expressed on a scale of 0 – 500 Pt-Co. The observations here were of the slight yellow colour region. The values observed here tend to suggest contamination; however, yellow water colouration does not automatically translate to pollution and toxicity, as natural, healthy surface water bodies have colouration and support ecosystems. This observation is in contrast with the findings reported by Ogbeibu and Oriabure (2023) from the assessment of water physico-chemical properties of the Benin River. According to DataStream (2021), the yellow colour observed in the samples may have resulted from colour impartation from the dissolution of organic matter or inorganic minerals such as oxides of iron, manganese, and aluminium oxides contained in benthic materials. The source of the yellow colour is recommended for investigation in another study.

Figure. 3: Rumuosi borrow pit water colour

Water odour results are presented in Table 2. Water samples from all the locations had an unobjectionable odour quality. This implies the absence of odoriferous gas-producing substances and bacteria in water. The water odour results correspond with the findings of Awoyemi et al. (2021); Raji et al. (2021); Ewulonu et al. (2019); Shittu et al. (2008) on surface water of South-West Nigeria. Water odour results from the activities of algae and bacteria on organic materials in water (Roy, 2022). This result shows that water had good oxidation.

Table 2: Water odour of Rumuosi borrow pit surface water

| Variable | Sample Location |  |  |  | DPR/FMEnv 2011 Standard |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D |  |
| Odour | unobjectionable | unobjectionable | Unobjectionable | unobjectionable | unobjectionable |

Figure 4 displayed water pH results and the DPR/FMEnv (2011) standard. From the results, sample points A, B, and C had pH = 6.5, while 6.6 was recorded in sample point D. These values are about the lower prescribed minimum standard limits (i.e., 6.5) but below the maximum prescribed standard limit (i.e., 8.5) for surface water bodies. A pH range of 6.5 to 8.5 is conducive to supporting aquatic organisms. Therefore, the water pH result, which falls within the weak acid range, is within the prescribed standard index for supporting wildlife and ecosystems. This result aligns with the findings of Ngah et al. (2016); Vaikosen et al. (2014) from assessments of the physico-chemical properties of surface water bodies of other borrow pits in Nigeria’s Niger Delta areas. Similar studies on some rivers outside the Niger Delta by Agorua et al. (2021); Okorie and Nwosu (2014); Adedeji et al. (2019) also reported water pH in the same range as obtained in this study.

Figure 4: pH of Rumuosi Borrow Pit water pH

Biochemical Oxygen Demand (BOD) of sample points showed a sinuous trend (Figure 5). The highest BOD value was recorded in sample point A, followed by points C and D, while point B had the least (Figure 5). All BOD results were below the standard limit, i.e., 3 mg/L by DPR/FMEnv (2011). Details of the BOD results are displayed in Figure 5. The low BOD result implies a low organic matter content in water, i.e., no organic pollution. This is in accordance with the report of Abowei and George (2009) from a study of Okpoka Creek, Niger Delta, Nigeria. According to the Department of Environment (2012), BOD level and state of water body classification (Table 3), the borrow pit water is in the “Not Contaminated” class.

Figure 5: Biochemical oxygen demand of Rumuosi borrow pit water

Table 3: Biochemical oxygen demand levels and state of water body

| S/No. | BOD Level (mg/L) | State of Water Body |
| --- | --- | --- |
| 1 | ˂ 1.0 | Not Contaminated |
| 2 | 1 - 4 | Slightly Contaminated |
| 3 | 4 - 6 | Moderately Contaminated |
| 4 | 6 - 12 | Contaminated |
| 5 | ˃ 12.0 | Highly Contaminated |

Source: Department of Environment (2012)

Total organic carbon (TOC) of all the sampled points was low. TOC results were generally below 10 mg/L, i.e., normal TOC concentration for rivers and lakes.  The details of the TOC result are shown in Figure 6.

Figure 6: Total organic carbon of Rumuosi borrow pit water

Figure 7 contains the phosphate results of the sample points. Water phosphate content showed a sinuous trend in the sample points, i.e., sample point A had 0.27 mg/L, followed by 0.25 mg/L in point C. Sample point B had 0.20 mg/L, while point D recorded the least value of 0.15 mg/L. All sample point results were below the prescribed standard (Figure 7).

Figure 7: Phosphate content of Rumuosi borrow pit water

The nitrate content result is shown in Figure 8. Nitrate contents in the sample locations were below the prescribed maximum standard limit. For example, sample point A had 0.7 mg/L of nitrate, while locations B and D had 0.3 mg/L, which are below 8 mg/L (the prescribed maximum standard limit). Nitrogen is an important nutrient that influences species and their populations in aquatic systems. Below standard index concentration of nitrate concurs with the findings of Seiyaboh and Izah (2017) out of an assessment of water quality properties of Epie Creek, River Nun, Igbedi Creek, and Efi Creek in Bayelsa State, Nigeria; however, it disagrees with Adesuyi et al. (2015), who reported high nitrate content in Nwaja Creek. This high nitrate content of Nwaja Creek is understood because it is located in a high population density and industrial area of the Port Harcourt metropolis; therefore, it is highly exposed to nutrient loading through waste disposal into it and consequently pollution.

Figure 8: Nitrate content of Rumuosi borrow pit water

Figure 9 displays dissolved oxygen (DO) content results. DO values obtained ranged from 4.7 to 5.3 mg/L. Sample point A had the least value (4.7 mg/L), while point C recorded 5.3 mg/L DO content. In general, the sample points DO contents were below the 10.0 mg/L minimum standard limit for wildlife, fishery, and other aquatic organisms and recreation water bodies. Dissolved oxygen (DO) is an important water quality parameter used in the assessment of the fitness of an aquatic system to support its biotic components. The DO result obtained here was low compared with the DPR/FMEnv (2011). This result is incongruous with the findings of Onwugbuta-Enyi et al. (2008); Adesuyi et al. (2015), who investigated the dissolved oxygen level of Nwaja Creek surface water in Port Harcourt. Low DO content in surface water bodies results from high water temperatures, excessive organic matter content, nutrient pollution, low or absence of autotrophs in water, and reduced water flow, etc. (USEPA, 2024). The low DO result obtained from this assessment can be postulated to have resulted due to the limiting effect of high environmental temperature on the dissolution of oxygen in water, as sample collection was done after midday during the hot-dry period of the year at which time the ambient surface water temperature of the study area may have risen significantly high to limit the solubility and enhanced loss of oxygen from water.

Figure 9: Dissolved oxygen content of Rumuosi borrow pit water

Figure 10 contains the total hydrocarbon content (THC) results of the study area. The THC levels of the sample points were generally 0.01 mg/L. This value is significantly below the reference 10 mg/L, i.e., the maximum permissible concentration prescribed by the DPR/FMEnv (2011) and the DPR (2002) for wildlife, fisheries, and ecosystem conservation. As shown in the variable result Figure, the low THC implies no hydrocarbon contamination, which shows that there was no extraneous input of THC into the borrow pit water. This observation is in contrast with Wokoma (2008); Dumka and Kingdom (2018); Paul et al. (2022) from the examination of THC in surface waters of the Imo River in Oyibo Local Government Area, Otamiri River in Etche Local Government Area Kua/Kinabere Creek in Ogoni, respectively.

Figure 10: Total hydrocarbon content of Rumuosi borrow pit water

Total Alkalinity of point A was 100 mg/L CaCO3, B and C had 50 mg/L CaCO3 (Figure 11), while D recorded 40 mg/L CaCO3. From this result, sample location A had the highest alkalinity, while location D had the least. The low alkalinity of the borrow pit water corresponds with the findings of Seiyaboh and Izah (2017); Ehiagbonare and Ogunrinde (2020) from analysis of concrete and earthen fish ponds water in Edo State. These corresponding observations suggest that low alkalinity may be natural in lentic freshwater ecosystems.

Figure 11: Total alkalinity of Rumuosi borrow pit water

Water turbidity results were of sample points were generally higher than the standard provision (Figure 12). The least turbidity value, 22 NTU, was recorded in point D, followed by 23 NTU in point C, while points A and B had 28 NTU and 30 NTU, respectively (Figure 12). The turbidity result obtained is classified to have moderate turbidity (DataStream Initiative, 2021). This result implies a moderate presence of suspended nutrients or organic matter, or microbial population suspension in water, as both natural and human factors affect the turbidity levels of a water body. This observation is in line with the findings of Rim-Rukeh (2013) from a study of physico-chemical characteristics of stagnant surface water bodies (ponds and lakes) in the Niger Delta, Nigeria.

Figure 12: Turbidity of Rumuosi borrow pit water

Heavy Metals

Table 4 shows the composite heavy metals concentration results. The concentrations of copper (Cu) fluctuated between the sample locations.  Sample points A and C had 0.026 mg/L of Cu, while points B and D recorded 0.069 mg/L and 0.047 mg/L, respectively (Table 4). The levels of the metal in the study area were significantly higher than the DPR/FMEnv (2011) prescribed standard level of copper in water. Mercury (Hg) content of sample locations was of equal concentration (i.e., 0.001 mg/L). This concentration level is at the threshold of the prescribed maximum concentration. A high concentration of mercury in the environment is hazardous to cells and living systems. The concentrations of lead (Pb) in the sampled points were also of equal concentration and at the DPR/FMEnv (2011) maximum concentration limit. Lead (Pb) occurs naturally in the environment. However, high concentration of copper in any environmental media often results from human activities.

The high concentration of copper (Cu) and mercury (Hg) means that the borrow pit water is polluted with the metals and therefore hazardous. This is in agreement with the findings of Ehiagbonare and Ogunrinde (2020), who reported high copper content from analysis of earthen fish ponds in Okada, Edo State, Nigeria. High concentrations of the metals are dangerous to plants and other biotic components of the study area and, by extension, to humans who may consume them, as the metals may have bioaccumulated to dangerous levels in their tissues. Mercury is a toxic metal and extremely persistent in the environment. In aquatic systems, it is converted to methylmercury in water by bacteria, in which form it is easily absorbed by aquatic organisms such as fish. Through trophic interactions, methylmercury can bioaccumulate to dangerous levels in tissues of fish and other organisms over time, and if consumed by humans, can cause serious health problems. High concentrations of heavy metals in natural bodies of water could result from natural sources (Borgohain et al., 2024). However, human activities, which include input from industrial activities (e.g., mining and manufacturing) and improper disposal of heavy metal-containing wastes, etc., are in many cases responsible for high levels of heavy metal contamination of the natural environment. The actual source of the heavy metals contamination in the borrow pit surface water is unclear. However, the likely source could be the release of the metals from the bottom sediment through redox reactions into the water. This can be investigated by analysis of the pit sediment. Regular monitoring is recommended to determine the safety status of water from the ecosystem for any human use or harvest of its biotic resources for human consumption.

Table 4: Heavy metal content of the Rumuosi borrow pit water

| Metal | Sample location |  |  |  | DPR/FMEnv (2011) Standard |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D |  |
| Copper, Cu (mg/L) | 0.026 | 0.069 | 0.026 | 0.047 | 0.001 |
| Mercury, Hg (mg/L) | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |
| Lead, Pb (mg/L) | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 |

Microbiological Properties

Total and faecal coliform counts are presented in Table 5. Total coliform count (TCC) was greater than 2,400 MPN/100 ml at the sample points. Similarly, faecal coliform count (FCC) was greater than 2,400 MPN/100 ml. Both TCC and FCC results were significantly higher than their standard limits. The result highlights the proliferation of total and faecal coliform bacterial species in the Rumuosi borrow pit water.

The presence of coliform bacteria counts shows water contamination. High total and faecal coliform counts in water result from the discharge of organic wastes, including faecal matter, into water. This microbial result corresponds with the findings of Rim-Rukeh (2013) from investigations of biological characteristics of the Ikarama, Tenmako, Karabodone, and Adiegbe lakes in the Niger Delta area. A similar finding was also reported by Dutta et al. (2021) from the evaluation of the sustainability of shrimp farm ponds. The high coliform counts recorded in this study may have resulted from the release of faecal matter by herds of cattle, which are usually taken into the borrow to graze and have water. Studies with a focus on the impact of cattle grazing near surface resources and microbial load are recommended to ascertain the cause of the observed high microbial load. This will enable informed management for conservation purposes.

Table 5: Microbial properties of Rumuosi borrow pit water

| Microbiological Variable | Sample location |  |  |  | DPR/FMEnv 2011 Standard |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D |  |
| Total Coliform Count (MPN/100 ml) | ≥ 2,400 | ≥ 2,400 | ≥ 2,400 | ≥ 2,400 | 20 |
| Faecal Coliform Count (MPN/100 ml) | ≥ 2,400 | ≥ 2,400 | ≥ 2,400 | ≥ 2,400 | 0 |

CONCLUSION

This study surveyed the background levels of the physico-chemical, heavy metals, and microbial characteristics of Rumuosi borrow pit surface water to ascertain its quality and address the gap in knowledge on its quality and safety for human use. The findings show that physico-chemical properties were below standard provisions and good for the support of wildlife, fisheries, and ecosystem conservation. The levels of copper (Cu), mercury (Hg), lead (Pb), total and faecal coliforms were at hazardous levels. Although the present findings are based on the results obtained, the number of sampling locations and the non-inclusion of sediment in the study as limitations of this study that should be addressed in future research. Despite these limitations, the present study has contributed to the knowledge of the water quality status of the borrow pit. We hope that this research will stimulate further investigation of this important resource. It is suggested that dredging the borrow pit will remediate the high levels of heavy metal content. This will support the realization of the UN-SDG 6 for the benefit of the nearby residents of the Rumuosi borrow pit.

References

Abowei, J.F.N. & George, A.D.I. (2009). Some Physical and Chemical Characteristics in Okpoka Creek, Niger Delta, Nigeria. Research Journal of Environmental and Earth Sciences 1: 45–53.

Adedeji, H. A., Idowu, T. A., Usman, M. A., & Sogbesan, O. A. (2019). Seasonal variations in the physico-chemical parameters of Lake Ribadu, Adamawa state Nigeria. International Journal of Fisheries and Aquatic Studies, 7(1), 79-82. https://www.fisheriesjournal.com/archives/2019/vol7issue1/PartB/7-1-10-100.pdf

Adesuyi, A., A., Nnodu, V.C., Njoku, K.L. & Anuoluwapo, J. (2015). Nitrate and Phosphate Pollution in Surface Water of Nwaja Creek, Port Harcourt, Niger Delta, Nigeria. International Journal of Geology, Agriculture and Environmental Sciences 3: 14 – 20.

## Agorua, U. N., Sikoki, F. D., & Vincent-Akpu, I. (2021). Physico-Chemical Parameters of Oguta Lake, Imo State, Nigeria. International Journal of Scientific Engineering and Applied Science, 7, 207-222. https://www.ijseas.com/volume7,issue6/ijseas-0706018.pdf

Awoyemi, M. O., Ajama, O. D., Adekola, S. A., Arogundade, A. B., Fashina, C. D., Akinlade, G. O., & Oyekunle, J. A. O. (2021). Water and sub-soil contamination in the coastal aquifers of Arogbo, Ondo State, Nigeria. Journal of Hydrology: Regional Studies, 38, 100944. https://doi.org/10.1016/j.ejrh.2021.100944

Borgohain, D., Lanong, S., & Jaishi, H.P. (2024). Heavy metal contamination and health risks in ground water at Byrnihat industrial area: urgent need for remediation and public health safeguards. Proceedings of the Indian National Science Academy, 90, 931–942. https://doi.org/10.1007/s43538-024-00273-2

DataStream Initiative. (2021). A Water Monitor’s Guide to Water Quality. https://datastream.org/en-ca/guidebook

Department of Environment, Malaysia. (2012). *Malaysia Environmental Quality Report*. Ministry of Science, Technology and the Environment. https://www.doe.gov.my/portalv1/en/info-data/environmental-quality-report

Department of Petroleum Resources, DPR. (2002). Environmental guidelines and standards for the petroleum industry in Nigeria (EGASPIN).

National Environmental Standards and Regulations Enforcement Agency (NESREA). (2011). *National Environmental (Surface and Groundwater Quality Control) Regulations, S. I. No. 22*. Federal Republic of Nigeria Official Gazette.

Dumka, N.J. & Kingdom, A. (2018). Total Hydrocarbon Concentrations (THC) in surface water, sediments and biota from Otamiri River, Rivers State, Nigeria. International Journal of Chemical Studies 6: 2743 - 2748.

Dutta, P., Sinha, M., Pramanick, P., Zaman, S., & Mitra, A. (2021). Sustainability of Shrimp farming in coastal West Bengal in the backdrop of microbial load. Parana Journal of Science and Education. https://sites.google.com/site/pjsciencea/v-7-n-6-july

Ehiagbonare, J. E., & Ogunrinde, Y. O. (2020). Physico-chemical analysis of fish pond water in Okada and its environs, Nigeria. *African Journal of Fisheries Science*, *8*(5), 001–007. https://www.internationalscholarsjournals.org/journals/african-journal-of-fisheries-science/abstract/physico-chemical-analysis-of-fish-pond-water-in-okada-and-its-environs-nigeria

Ewulonu, C. C., Obire, O., & Akani, N. P. (2019). Microbiological and Physiochemical Quality of Freshwater in Isiokpo Community, Rivers State, Nigeria. South Asian Journal of Research in Microbiology, 3(1), 1-8. https://doi.org/10.9734/sajrm/2019/v3i130078

Lawal, P. O. (2011). Effects of Sand/Gravel Mining in Minna Emirate Area of Nigeria on Stakeholders. Journal of Sustainable Development, 4(1), 193-199. https://doi.org/10.5539/jsd.v4n1p193

Ngah, S.A., Ubong, I.U. & Ozoekwe, V.E. (2016). Impact of Municipal Wastes on the Water Quality around Rumuola Borrow Pit, Rivers State, Nigeria.  International Journal of Engineering and Applied Sciences 3:84 - 89.

Nwachukwu, M. A., & Osoro, E. (2013). Assessment of environmental hazards: Linking borrow pits, gully erosion, and road failure. Journal of Civil Engineering and Construction Technology, 4(6), 199-210. https://doi.org/10.5897/2013.0270

Nwachukwu, M. A., Nwachukwu, M. I., Ahiarakwem, C. O., & Nwankwo, G. I. (2017). Abandoned borrow pits; risk factors and reclamation procedure. MOJ Civil Engineering, 2(3), 88–95. https://doi.org/10.15406/mojce.2017.02.00033

Ogbeibu, A. E., & Oriabure, P. A. (2023). Assessment of the Water Quality of the Benin River, Southern Nigeria, Prior to the Seaport Development Project. Journal of Water Resource and Protection, 15, 597-610. https://doi.org/10.4236/jwarp.2023.1511033

Onwugbuta-Enyi, J., Zabbey, N., & Erondu, E.S. (2008). Water Quality of Bodo Creek in the Lower Niger Delta Basin. Advances in Environmental Biology, 2(3), 132-136. http://www.aensiweb.com/AEB/

Okorie, D.O. & Nwosu, P.O. (2014). Seasonal Variations in Physico-Chemical Properties of Imo River. Journal of Pharmacy and Biological Sciences 9: 07 - 09.

Paul, S., Owhoeke, E., & Ehiwario, J. N. (2022). Total Hydrocarbon Contents: Spatial Variations in Aquatic Environment of Oyigbo Communities, Rivers State. International Journal of Environmental Protection and Policy, 10(1), 1-5. https://doi.org/10.11648/j.ijepp.20221001.11

Raji, M., Adeleye, A.O., Amoo, F.K., Saadatu, A.Y., Isah, S.F., & Amoo, A.O. (2021). Water quality assessment: Bacteriological and physicochemical evidences from River Ngalda, Northeast Nigeria. Nigerian Research Journal of Chemical Sciences, 9, 322-333. http://www.unn.edu.ng/nigerian-research-journal-of-chemical-sciences/

Reine, K., Clarke, D., Ray, G., & Dickerson, C. (2013). Fishery resource utilization of a restored estuarine borrow pit: A beneficial use of dredged material case study. Marine Pollution Bulletin, 73(1), 115-128. https://doi.org/10.1016/j.marpolbul.2013.05.031

Rim-Rukeh, A. (2013). Physico-Chemical and Biological Characteristics of Stagnant Surface Water Bodies (Ponds and Lakes) Used for Drinking and Domestic Purposes in Niger Delta, Nigeria. Journal of Environmental Protection, 4, 920-928. https://doi.org/10.4236/jep.2013.49106

Roy, S. (2022). Sources and Treatment of Taste and Odour in Water – A Review. Water and Energy International, 65(7), 12-16 https://bbdu.ac.in/sources-and-treatment-of-taste-and-odour-in-water-a-review/

Seiyaboh, E. I., & Izah, S. C. (2017). Review of Impact of Anthropogenic Activities in Surface Water Resources in the Niger Delta Region of Nigeria: A Case of Bayelsa State. International Journal of Ecotoxicology and Ecobiology, 2(2), 61-73. https://doi.org/10.11648/j.ijee.20170202.12

Shittu, O. B., Olaitan, J. O., & Amusa, T. S. (2008). Physico-Chemical and Bacteriological Analyses of Water Used for Drinking and Swimming Purposes in Abeokuta, Nigeria. *African Journal of Biomedical Research*, *11*(3), 285–290. https://doi.org/10.4314/ajbr.v11i3.44000

## van Steenbergen, F., Arroyo-Arroyo, F., Rao, K., Hulluka, T. A., Woldearegay, K., & Deligianni, A. (2021). Green Roads for Water: Guidelines for Road Infrastructure in Support of Water Management and Climate Resilience. International Development in Focus. Washington, DC: World Bank. https://doi.org/10.1596/978-1-4648-1677-2

## van Steenbergen, F. V. (2017). Road-Side Borrow Pits as Ponds for Off-Season Small-Scale Irrigation. Hamburg University of Applied Sciences, Hamburg, Germany.

United States Environmental Protection Agency. (1986). Ambient Water Quality Criteria for Dissolved Oxygen. U.S. Environmental Protection Agency, Office of Water, Washington, DC. https://www.epa.gov/wqc/ambient-water-quality-criteria-dissolved-oxygen

Vaikosen, E. N., Ebeshi, B. U., & Airhihen, B. (2014). Bioaccumulation of Heavy Metals and Hydrocarbons in Hemichromis Fasciatus Exposed to Surface Water in Borrow Pits Located Within an Onshore Oil Exploration and Production Area. Environment and Pollution, 3(3), 38–55. https://doi.org/10.5539/ep.v3n3p38

Wokoma, O. A. F. (2014). Levels of total hydrocarbon in water and sediment of a polluted tidal creek, Bonny River, Niger Delta, Nigeria. International Journal of Scientific & Technology Research, 3(12), 351-354. https://www.researchgate.net/publication/317009000_Levels_Of_Total_Hydrocarbon_In_Water_And_Sediment_Of_A_Polluted_Tidal_Creek_Bonny_River_Niger_Delta_Nigeria