# Original Research Article

# SEASONAL VARIATION IN ALGAL COMMUNITY STRUCTURE IN KOPILI RIVER, ASSAM, INDIA

ABSTRACT

Aim: This pilot study was conducted to document algal species diversity and to evaluate influence of environmental and anthropogenic factors on algal community structure in Kopili River.

Methodology: Algal samples were collected from three selected study sites : Panimur, Kheroni and Chaparmukh along with the key water quality parameters such as pH, remperature, conductivity., etc. for a period of one year ( June 2023- May 2024).Standard procedures were followed for the identification, enumeration and diversity analysis.

Results: A total of 64 ( Sixty four ) algal species were recorded, with Bacillariophyceae to be the most prevalent group, followed by Chlorophyceae, Cyanophyceae and Zygnematophyceae. Seasonal variation was evident with maximum diversity observed during the pre- monsoon season and reduced diversity observed during the monsoon, largely influenced by high sedimentation and discharge.

Interpretation: Significant seasonal variation and the impact of anthropogenic activities from sand mining and domestic discharge on the algal community were observed in this study. Findings establish algae as effective bioindicators and highlight the need for sustainable watershed management for preserving the river ecosystem health.

Keywords : Algal Diversity;Bioindicators; Kopili River; Seasonal Variation; Water Quality.

Graphical Abstract

INTRODUCTION

Freshwater Algae are essential to aquatic ecosystems, serving as a primary producers at the base of the food web. Apart from sustaining the diverse food webs, they significantly contribute to primary productivity, oxygen production, and nutrient cycling through photosynthesis .(Koller et al., 2014.,Joshi et al., 2018;; Krichen et al., 2019; Sharma et al., 2019,). They are also a reliable indicator of ecological health and water quality due to their rapid response ability to environmental changes (Bellinger and Sigee, 2010; Gökçe, 2016; Khalil et al., 2021). Shift in algal community structure often reflect delicate or cumulative changes in physico-chemical parameters, including temperature, light availability, flow regime, nutrient levels, light availability, flow regime, and human-caused pollution (Dokulil, 2003; Omar, 2010). In tropical monsoon climatic regions with pronounced seasonal variability, the structure, composition and abundance of algal communities differ significantly across the seasons in response to changing river flow, fluctuating concentrations level of pollutants and monsoonal inputs (Branco et al., 2008; França et al., 2011; John and Kumar, 2021; Mamun et al., 2021).

Rivers in North- East India , though teeming with life are facing escalating threats from anthropogenic  activities. One such prime example is the the Kopili River, tributary of the river Brahmaputra.It starts in the lush hills of Meghalaya and winds through diverse landscapes, from the rugged terrain of Dima Hasao to the fertile plains of Nagaon. This natural diversification makes it an ideal location for studying aquatic life. However, from the past two decades, the Kopili River has been under growing pressure from human activities, including activities like unregulated sand mining (Hackney et al., 2020; Koehnken et al., 2020; Rentier and Cammeraat, 2022), direct discharge of untreated domestic wastewater (Bhat and Qayoom, 2021; Xie et al., 2022; Samaraweera et al., 2024;), and the construction of hydropower infrastructure (Baumgartner et al., 2021; Liu et al., 2021). These factors have disrupted the river’s natural flow and chemical balance, posing significant risks to aquatic life and the overall health of the ecosystem (Lekomo et al., 2021; Lwanga et al., 2022; Park, 2024). River ecosystem are equally susceptible to these disturbances according to studies and they can result in reduction in ecosystem system and biodiversity (Alonso et al., 2021; Dudgeon, 2010; Overed-Sayer et al., 2025; Yang et al., 2020; Zúñiga-Sarango et al., 2020). Despite the socio-economic and ecological significance of the Kopili River, there's not much scientific study specifically focused on its biological communities, especially concerning the diversity of algae and the environmental factors influencing them. However due to the lack of baseline data, it is challenging to accurately evaluate the ecological health of the river and identify early indicators of environmental deterioration. Studies on other Indian rivers have shown that bioindicators such as Bacillariophyceae, cholorophyceae, and Cyanophyceae can be used to accurately measure ecological status, changes in habitat, contamination level (Abdullahi and Inbrahim, 2018; Buragohain and Yasmin, 2018; Kadam et al, 2020; Khalil et al., 2021 ).Yet, the Kopili river remains largely unexamined in this regard.  To address this gap, the current study aims to represents the first thorough effort to document and analyze algal diversity in the Kopili river across both different locations and time periods.

MATERIALS AND METHODS

Study Area

The Kopili river has its source in the Meghalaya plateau and flows through the states of Assam and Meghalaya before merging with the Brahmaputra River.The Basin exhibits a varied topography, characterized by steep hills, dense forests, and fertile agricultural plains. The area is marked by a monsoonal climate, featuring clear wet and dry seasons that greatly affect the hydrological patterns of the rier and its water quality. Water is a basic resource that is essential to both aquatic environment health and human population survival. Essential Geographical and environmental characteristics for each sampling site are highlighted to provide a thorough overview of the study area’s geography (Figure 1 and Table 1).

Figure 1: Map of Kopili River with Sampling Location.

| Table 1: Geographical and environmental characteristics of the selected three Study sites of River Kopili of Assam, India |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site | Location | Co-ordinates | Description | Anthropogenic influence |
| 1 | Panimur | 25.7148o N and 92.8234oE | Hilly, scenic | Picnic spot |
| 2 | Kheroni | 25. 8374o N and 92.8859o E | Urban, Midstream | Domestic discharge |
| 3 | Chaparmukh | 26.2031o N and 92.5118o E | Floodplain | Sand mining |

Sample Collection and Analysis :

To thoroughly study the algal communities; water and algal samples were collected each month from June 2023 to May 2024.  On site measurement were made of the water temperature, pH, Water current and conductivity. Dissolved oxygen, Biological Oxygen Demand, phosphate, nitrate, nitrite and magnesium were analysed following the standard protocols (APHA, 2012). Phytoplankton was collected from the water surface using a plankton net (45µm). Periphytic algae were carefully removed from various surfaces like , rocks, stones,  pebbles, decaying leaves, and sediments using a scalpel and toothbrush. The algal samples were preserved with 4% formaldehyde and transported to the USTM laboratory for further analysis. A trinocular microscope was used to observe and photograph the algal specimens. Identification of algal species was conducted using standard taxonomic guides and monographs (Prescott (1951), Tiffany and Britton (1952), Desikachary (1959), Randhawa (1959), Scott and Prescott (1961), Philipose (1967), Croasdale et al. (1994), Bey and Ector (2013), Karthick et al. (2013), etc and taxonomy was updated based on the Algae Base online database.

Data Analysis:

Species diversity was calculated using the Shannon–Wiener index. Spearman correlation coefficient was also calculated to determine the relationship between different variables. All statistical analyses were performed using PAST software (Version 4.03).

RESULT AND DISCUSSION

## Physico-chemical characteristics of Water:

## The Physico-chemical parameters of the Kopili river exhibited significant seasonal and spatial variation (Figure 2).Water temperatures averages, ranging from 23°C to 25oC showed the characteristics of monsoonal river systems. The pH, varying between 5.46 and 7.91 showed moderate condition of mildly acidic to slightly alkaline. The highest pH was observed at Site 3 during the pre-monsoon, while the lowest was recorded at Site 1. Studies showed that spatial and temporal variations in physico-chemical parameters are typical in rivers and can be influenced by local geological, land uses and anthropogenic activities (Rodrigues et al.,2017;Rahman et al., 2021;Kirui et al., 2019; Mishra and Singh., 2020; Aweng et al., 2021).

## Electrical Conductivity peaked during the monsoon season at Site 3, with values ranging from 52. 46 µS/cm to 152µS/cm. This could be attributed due to ionic concentrations from the surface run off that frequently happens during heavy monsoonal rainfall carrying dissolved solids into river systems (Ouyang et al., 2006). Meanwhile, Lower conductivity values observed during pre-monsoon at Site 1 and Site 2 indicate dilution effects from reduced run off. Parameters like Chloride, nitrate and nitrite also differed significantly across seasons and sites influenced by agriculture run off and wastewater discharge (Liu et al., 2020). Nitrate levels, for instance peaked at Site 2 during the monsoon, likely due to these anthropogenic inputs. Total Hardness, Magnesium, Phosphate  also showed temporal and spatial fluctuations, which may have been caused by human activity and by natural hydro- metereorolical variables ( P.U et al., 2017; Rostami et al., 2019).Dissolved Oxygen ranged from 4.6 mg/L to 8.2 mg/L, with the highest concentration observed at Site 1 during Pre- Monsoon , indicating favourable conditions for aquatic life. The lower Dissolved Oxygen concentration recorded at Site 3 during the monsoon could be attributed to the increased decomposition of organic matter, a common phenomenon seen in tropical river during monsoon ( Hamdhani et al., 2023). Meanwhile the Biological Oxygen demand (BOD) of the Kopili river ranged from 2.0 mg/L to 2.4 mg/L.Though these BOD values indicate some organic pollution, this values  also indicate the self purification capacity within the river. Monsoonal rainfall influence the water current speed seasonally impacting Dissolved oxygen level and consequently aquatic biota ( Rostami et al., 2019)

## The  inter- relationship between these water parameters are further elucidated by the Spearman correlation coefficients presented (Table 2).  A significant positive correlation was found between total hardness, temperature and magnesium. On the other hand, temperature showed a moderate negative correlation with dissolved oxygen and a weak negative correlation with pH, implying that higher temperature results in somewhat lower Ph and lower DO levels. This is a crucial findings, as reduced DO level at higher temperature can significantly impact the aquatic life ( Pramanik et al., 2020). The strong negative correlation  between conductivity and both DO and pH imply that as conductivity increases, likely due to higher dissolved solids or salinity, both DO and pH tend to decrease. Studies by Ouyang et al., 2006 showed that there is a direct connection between nutrient enrichment and organic pollution , often associated with agricultural run off and wastewater , as evidenced by the moderately positive association between nitrate levels and both BOD and other nutrients. Phosphate also showed a moderate positive correlation with Magnesium and a weaker positive correlation with nitrate, further demonstrating the interdependence of nutrient dynamics. Such correlation are essential for evaluating temporal variations of river pollution due to natural or anthropogenic inputs ( Ouyang et al., 2006).

Figure 2 :    Seasonal Variation of Water Quality Parameters  of the Kopili River  along with standard error.

Figure 3 :    Seasonal Variation of Water Quality Parameters  of the Kopili River  along with standard error.

Table 2 : Spearman’s rank correlation for different water quality parameters of the Kopili River.

| Variables | Temperature | Conductivity | pH | DO | BOD | chloride | total hardness | Nitrate | Nirite | Magnesium | Phosphate | Water Current |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Conductivity | 0.119176 | 1 |  |  |  |  |  |  |  |  |  |  |
| pH | 0.0819 | 0.860828 | 1 |  |  |  |  |  |  |  |  |  |
| DO | 0.158811 | -0.78994 | -0.74043 | 1 |  |  |  |  |  |  |  |  |
| BOD | 0.039829 | 0.181956 | 0.109687 | 0.109221 | 1 |  |  |  |  |  |  |  |
| chloride | 0.306452 | -0.83333 | -0.70892 | 0.756329 | -0.20795 | 1 |  |  |  |  |  |  |
| total hardness | 0.602626 | -0.53009 | -0.52385 | 0.836331 | 0.217797 | 0.718185 | 1 |  |  |  |  |  |
| Nitrate | -0.06466 | 0.295382 | 0.307692 | 0.042554 | 0.500173 | -0.23631 | -0.11256 | 1 |  |  |  |  |
| Nirite | 0.286957 | 0.36604 | 0.288804 | -0.14164 | 0.716926 | -0.33199 | 0.004367 | 0.51295 | 1 |  |  |  |
| Magnesium | 0.119176 | 0.9 | 0.886147 | -0.9244 | 0.043323 | -0.73333 | -0.61559 | 0.033758 | 0.29794 | 1 |  |  |
| Phosphate | -0.06957 | 0.859769 | 0.698302 | -0.6052 | 0.451398 | -0.68952 | -0.43669 | 0.465535 | 0.443478 | 0.715055 | 1 | - |
| Water Current | -0.17951 | -0.3431 | -0.1017 | 0.430384 | 0.4742 | 0.200839 | 0.145956 | 0.686465 | 0.440235 | -0.40168 | -0.11968 | 1 |

## Algal Community Composition and Diversity:

## During the study period from June 2023 to May 2024, a total of 64 (Sixty four) algal species were identified from eight major classes in the Kopili River, with Bacillaruiphyceae being the most dominant group. This highlight the significant aquatic biodiversity of this freshwater ecosystem.  Due to their high species richness and ability to thrive across a wide range of aquatic conditions, diatom dominance is align with the already establish  ecological knowledge, reinforcing the understanding that diatoms play a central role in structuring freshwater communities  (Rondon and Aragon., 2018; Obiuto et al., 2022). Diatoms role and adaptability as primary producers make them crucial components of freshwater ecosystems (Franca   et al., 2011).

## Different seasonal patterns were observed within the algal communities during the study period. Studies shows that many species displayed a specific seasonal occurrence rather than a year round presence. For instance, several Bacillariophceae species such as Fragillaria biceps, Meridion circularae, Cyclotella sp., and Achnanthidium exiguum were primarily recorded during the pre-monsoon period. This strong seasonal presence of some types of diatoms appear more strongly in certain seasons could be attributed to favourable conditions  such as increased light penetration and specific algal nutrient availability as indicated by the study ‘s correlation  between algal distribution and environmental factors like phosphate, nitrate and temperature.  On the other hand,  some species , such as Plerosigma sp. and Melosira varians ( both Bacillariophyceae) were not found in winter. Similarly, the green algae Desmidium coaractum (chlorophyceae) was missing in winter but occurred during other seasons.  some species, such as Melosira varians and Pleurosigma sp. (both Bacillariophyceae), were not found in winter. Similarly, the green algae Desmidium coaractum (Chlorophyceae) was missing in winter but occurred during other seasons.Cyanophycea species such as Nostoc sp. and Anabaena constricta ., were present exclusively during the Monsoon period, indicating their presence for conditions prevalent in this season. These seasonal shifts observed are consisitent with patterns reported in other tropical riverine systems, where monsoonal inputs and changing regimes of flow significantly influence algal abundance and composition ( Branco et al., 2008;Chong et al.,2020; Mamun et al.,2021). While some studies suggest periphytic algal diversity and richness can be independent of season, the distinct seasonal variations in the Kopili river emphasize the strong influence of local environmental drivers.

## Table 3 : Checklist of algal Species observed during the study period in the Kopili River , Assam, India.

| SR.NO | ALGAL SPECIES | CLASS | MONSOON | AUTUMN | WINTER | PRE-MONSOON |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Fragillaria biceps | BACILLARIOPHYCEAE | (+) | – | + | + |
| 2 | Meridion circularae |  | (+) | – | + | + |
| 3 | Achnanthidium exiguum |  | (+) | + | – | + |
| 4 | Achnanthidium reimeri |  | (+) | – | + | + |
| 5 | Cymbella reinhadtii |  | (+) | + | + | + |
| 6 | Cyclotella sp. |  | (+) | + | + | + |
| 7 | Gomphonema lanceolatum |  | (+) | + | + | + |
| 8 | Gomphonema parvulum |  | (+) | – | – | + |
| 9 | Melosira granulate |  | (+) | – | – | - |
| 10 | Pinnularia scythica |  | - | + | – | + |
| 11 | Surirella brebissonii |  | (+) | – | – | + |
| 12 | Navicula producta |  | (+) | – | – | + |
| 13 | Nitzchia obtuse |  | - | + | + | + |
| 14 | Melosira juergensii |  | - | + | + | + |
| 15 | Eunotia flexuasa |  | - | + | – | + |
| 16 | Melosira granulate |  | (+) | – | – | + |
| 17 | Surirella splendid |  | (+) | – | – | + |
| 18 | Surirella elegans |  | (+) | – | – | + |
| 19 | Sellaphora bacillum |  | (+) | – | – | + |
| 20 | Gyrosigma scalproides |  | (+) | + | + | + |
| 21 | Fragillaria ulna |  | - | + | + | + |
| 22 | Melosira varians |  | - | + | – | + |
| 23 | Pleurosigma sp. |  | (+) | – | – | + |
| 24 | Gyrosigma sp. | BACILLARIOPHYCEAE | - | – | – | + |
| 25 | Ulnaria sp. |  | (+) | + | – | + |
| 26 | Nitzchia sp. |  | (+) | – | – | + |
| 27 | Chlorococcum minutum | CHOLOROPHYCEAE | + | + | (±) | + |
| 28 | Chlorococcus minutas |  | + | + | – | + |
| 29 | Golenkinia radiata |  | + | + | – | + |
| 30 | Desmidium coaractum |  | + | + | (±) | + |
| 31 | Monoraphidium sp. |  | + | + | – | + |
| 32 | Coelastrum cambricum |  | + | + | (±) | + |
| 33 | Oedogonium sp. |  | - | - | + | + |
| 34 | Pediastrum angulosum |  | + | + | + | + |
| 35 | Scenedesmus quadricauda |  | - | + | + | + |
| 36 | Scenedesmus armatus |  | + | + | + | + |
| 37 | Scenedesmus acuminatus |  | + | + | + | + |
| 38 | Scenedesmus abundans |  | - | + | + | + |
| 39 | Monoraphidium griffithi |  | - | - | – | + |
| 40 | Ulothrix tenuissima |  | + | - | (±) | + |
| 41 | Ulothrix sp. |  | + | + | (±) | + |
| 42 | Zygnema sp. | ZYGENEMATOPHYCEAE | + | + | + | - |
| 43 | Closterium purvulum |  | + | - | + | + |
| 44 | Closterium calosporum |  | + | + | + | + |
| 45 | Closterium acutum |  | + | + | + | + |
| 46 | Cosmarium moniliformae |  | - | + | - | + |
| 47 | Cosmarium diplosporum |  | - | + | + | + |
| 48 | Euastrum denticulatum |  | + | + | + | + |
| 49 | Spirogyra subsala | ZYGNEMATOPHYCEAE | + | + | - | + |
| 50 | Euastrum binale |  | - | + | + | + |
| 51 | Anabaena constricta | CYANOPHYCEAE | + | (+) | (+) | + |
| 52 | Coelosphaerium dubium |  | + | (+) | (+) | + |
| 53 | Spirulina megeghiniana |  | + | (+) | (+) | + |
| 54 | Merismopedia glauca |  | + | (+) | (+) | – |
| 55 | Nostoc sp. |  | - | (+) | (+) | + |
| 56 | Gloeocapsa punctata |  | + | (+) | (+) | – |
| 57 | Phormidium favorum |  | - | (+) | (+) | – |
| 58 | Phormidium sp. |  | - | (+) | (+) | – |
| 59 | Oscillatoria sp. |  | + | (+) | (+) | – |
| 60 | Phacus pleuronectes | EUGLENOPHYCEAE | – | + | + | + |
| 61 | Trachelomonas abrupt |  | – | + | + | + |
| 62 | Euglena pascheri |  | – | + | + | + |
| 63 | Chlorella vulgaris | TREBOUXIOPHYCEAE | + | + | + | + |
| 64 | Ulothrix variabilis | ULVOPHYCEAE | + | + | + | + |

Note: The symbols used to indicate the seasonal occurrence of algal species represent their relative abundance and presence. A plus sign (+) denotes species that are abundant and frequently observed during the respective season. A plus sign in parentheses (+) indicates a moderate or occasional presence. The symbol (+) is used for species that are observed rarely or in low numbers, representing occasional or less abundant occurrence. A minus sign (-) signifies that the species is absent or extremely rare during that season.

Beyond temporal variations, notable spatial differences were noted among the three sampling sites of the Kopili river reflecting a range of anthropogenic influences. Site 1- Panimur , representing the less disturbed upper reaches of the river , consistently exhibited greater algal diversity , particularly during the winter and pre-monsoon periods. This suggests that less impacted area serve as important refugia for a greater variety of species ( Khatri et al., 2021 ).  Meanhwhile, Site 2- Kheroni, largely affected ny discharge of wastewater from homes had a higher number of species that could witstand pollution and a lower level of algae. This observation underscore the utility of algal communities as sensitive bioindicators of water quality degradation ( Omar., 2010;Khalil et al., 2021;Das.,2022).

Untreated or Partially treated sewage introduces high level of nutrients like nitrogen and phosphorus leading to eutrophication and significant shifts in aquatic biodiversity (Rao et al., 2019; Bhat and Qayoom., 2021; Hsieh et al., 2021; Katam et al., 2022). Studies showed that periphytic algae that are exposed to sewage effluents have similarly shown changes in species composition with an increase in pollution tolerant species  ( Abe et al., 1996; Chamber and Prepas., 1994; Hidayah et al., 2022).Changes in Algal composition at Site 3- Chaparmukh  influenced by sand mining and household domestic wastes further highlight how direct human interventions can disrupt aquatic ecosystems. River sand mining is known to degrade the water quality by disrupting or altering the physico- chemical parameters through sediment accumulation, erosion,changes in river channel dynamics posing serious environmental problems (Lekomo et al., 2021; Alhafizoh et al., 2022; Damseth et al., 2024;Pant et al., 2025).  These findings align with the previous research demonstrating the vulnerability of riverine ecosystems to such disturbances, which can lead to significant declines in biodiversity and ecosystem function.

Figure 4 :Shannon Diversity index across the three study sites of the Kopili River.

The Shannon diversity index of the algal community showed significant seasonal variation, reaching its minimum during the monsoon season and its maximum during winter (Figure 4). The monsoon’s increased water turbulence and sediment load likely contributed to lower diversity by rendering the environment less stable for various species (Rasal et al., 2019).This pattern indicate that colder less turbulent months support a more diverse algal community. Seasonal fluctuations in these parameters, driven by climatic cycles and human activities probably cause the noted changes in species dominace and composition. Algae are widely recognized as effective bioindicators due to their rapid response to pollutanta and their role in indicating eutrophication (Bellinger and Sigee., 2010; Buragohain and Yasmin., 2014; Hariyati and Putro., 2019; Kadam et al., 2020; Khalil et al., 2012; Jodhani et al., 2025).

CONCLUSION

This pilot study provides crucial baseline data on the algal diversity and spatio- seasonal dynamics of the Kopili River. The results show how anthropogenic activities directly affect water biodiversity in addition to establishing the distinctive algae assemblage of this little studied area. Knowing which species are sensitive and tolerant  can help build tailored bioindicators for continuous water quality monitoring. This foundational knowledge is essential for informing effective watershed management strategies and supporting conservation efforts in ecologically vital, yet increasingly threatened, freshwater ecosystems in Northeast India.

Research Content : The research content of the manuscript is original and has not been submitted elsewhere.

Ethical Approval : Not Applicable.

Conflict of Interests : The author declare that there are no conflict of interests.

Data Availability : Not Applicable.

Consent to Publish :  All authors agree to publish the paper in the journal of Environmental Biology.

REFERENCES

Abdullahi, U., & Ibrahim, S. M. (2018). An assessment of pollution in aquatic environment using bioindicators: a review [Review of An assessment of pollution in aquatic environment using bioindicators: a review]. Bayero Journal of Pure and Applied Sciences, 10(2), 64. African Journals OnLine. https://doi.org/10.4314/bajopas.v10i2.11

Abe, S., Kato, T., ITO, Y., YOKOBAYASI, K., & Miyazaki, K. (1996). Effects of Effluent from Sewage Treatment Plant on Periphytic Algae of Rivers. Japanese Journal of Water Treatment Biology, 32(1), 51. https://doi.org/10.2521/jswtb.32.51

Alhafizoh, F., Widiastuti, E. L., Tugiyono, Susanto, G. N., & Mumtazah, D. F. (2022). Heavy metals concentration in green macroalgae Spirogyra sp. of Way Ratai River Pesawaran Regency Lampung. IOP Conference Series Earth and Environmental Science, 1027(1), 12031. https://doi.org/10.1088/1755-1315/1027/1/012031

Alsabah, N. J. (2017). Impact of Sewage Discharge on Coral Reefs. The International Journal of Engineering and Science, 6(6), 9. https://doi.org/10.9790/1813-0606020913

Annang, T., Yirenya-Tawiah, R., Clerk, G., & Smith, T. E. (2014). Some aspects of the ecology of freshwater algae in the Densu River and two tributaries in Southern Ghana. International Journal of Environment, 3(2), 246. https://doi.org/10.3126/ije.v3i2.10637

Aweng, E. R., Aisyah, S., Salam, M., & Mior, I. B. (2021). Correlation between physico-chemical water quality and river ecosystems in Malaysia rivers with different land uses. IOP Conference Series Earth and Environmental Science, 842(1), 12041. https://doi.org/10.1088/1755-1315/842/1/012041

Bellinger, E. G., & Sigee, D. C. (2010). Freshwater Algae. https://doi.org/10.1002/9780470689554

Bhat, S. U., & Qayoom, U. (2021). Implications of Sewage Discharge on Freshwater Ecosystems. In IntechOpen eBooks. IntechOpen. https://doi.org/10.5772/intechopen.100770

Branco, C. C. Z., Krupek, RA., & Peres, C. K. (2008). Seasonality of macroalgal communities in a subtropical drainage basin in Paraná state, southern Brazil. Brazilian Journal of Biology, 68(4), 741. https://doi.org/10.1590/s1519-69842008000400008

Buragohain, B. B., & Yasmin, F. (2014). Biomonitoring of Pollution by Microalgae Community in Aquatic System with Special Reference to Water Quality of River Kolong, Nagaon, Assam, India. International Journal of Applied Sciences and Biotechnology, 2(1), 45. https://doi.org/10.3126/ijasbt.v2i1.9345

Chambers, P. A., & Prepas, E. E. (1994). Nutrient dynamics in riverbeds: The impact of sewage effluent and aquatic macrophytes. Water Research, 28(2), 453. https://doi.org/10.1016/0043-1354(94)90283-6

Chong, B. W. K., Leong, S. C. Y., Kuwahara, V. S., & Yoshida, T. (2020). Monsoonal variation of the marine phytoplankton community in Kota Kinabalu, Sabah. Regional Studies in Marine Science, 37, 101326. https://doi.org/10.1016/j.rsma.2020.101326

Damseth, S., Thakur, K., Kumar, R., Kumar, S., Mahajan, D., Kumari, H., Sharma, D., & Sharma, A. K. (2024). Assessing the impacts of river bed mining on aquatic ecosystems: A critical review of effects on water quality and biodiversity [Review of Assessing the impacts of river bed mining on aquatic ecosystems: A critical review of effects on water quality and biodiversity]. HydroResearch, 7, 122. Elsevier BV. https://doi.org/10.1016/j.hydres.2024.01.004

Das, B. K. (2022). Macrobenthic Pollution Bioindicator for Ecological Monitoring in Riverine Ecosystem. International Journal of Environmental Sciences & Natural Resources, 29(5). https://doi.org/10.19080/ijesnr.2022.29.556273

França, R. C. S., Lopes, M. R. M., & Ferragut, C. (2011). Structural and successional variability of periphytic algal community in a Amazonian lake during the dry and rainy season (Rio Branco, Acre). Acta Amazonica, 41(2), 257. https://doi.org/10.1590/s0044-59672011000200010

Hamdhani, H., Eppehimer, D. E., Quanrud, D. M., & Bogan, M. T. (2023). Seasonal and longitudinal water quality dynamics in three effluent-dependent rivers in Arizona. PeerJ, 11. https://doi.org/10.7717/peerj.15069

Hariyati, R., & Putro, S. P. (2019). Bioindicator for environmental water quality based on saprobic and diversity indices of planktonic microalgae: a study case at Rawapening lake, Semarang district, Central Java, Indonesia. Journal of Physics Conference Series, 1217(1), 12130. https://doi.org/10.1088/1742-6596/1217/1/012130

Hidayah, E. N., Cahyonugroho, O. H., Sulistyo, E. N., & Karnaningroem, N. (2022). Using molecular weight–based fluorescent detector to characterize dissolved effluent organic matter in oxidation ditch with algae. Environmental Science and Pollution Research, 29(44), 67418. https://doi.org/10.1007/s11356-022-22464-4

Jodhani, K. H., Gupta, N., Dadia, S., Patel, H., Patel, D., Jamjareegulgarn, P., Singh, S. K., & Rathnayake, U. (2025). Sustainable groundwater management through water quality index and geochemical insights in Valsad India. Scientific Reports, 15(1). https://doi.org/10.1038/s41598-025-92053-1

Kadam, A. D., Kishore, G., Mishra, D., & Arunachalam, A. (2020). Microalgal diversity as an indicator of the state of the environment of water bodies of Doon valley in Western Himalaya, India. Ecological Indicators, 112, 106077. https://doi.org/10.1016/j.ecolind.2020.106077

Katam, K., Ananthula, R., Anumala, S., Sriariyanun, M., & Bhattacharyya, D. (2022). The impact of light intensity and wavelength on the performance of algal-bacterial culture treating domestic wastewater. E3S Web of Conferences, 355, 2003. https://doi.org/10.1051/e3sconf/202235502003

Khalil, S., Mahnashi, M. H., Hussain, M., Zafar, N., Waqar-Un-Nisa, Khan, F. S., Afzal, U., Shah, G. M., Niazi, U. M., Awais, M., & Irfan, M. (2021). Exploration and determination of algal role as Bioindicator to evaluate water quality – Probing fresh water algae. Saudi Journal of Biological Sciences, 28(10), 5728. https://doi.org/10.1016/j.sjbs.2021.06.004

Khatri, N., Raval, K., & Jha, A. K. (2021). Integrated water quality monitoring of Mahi river using benthic macroinvertebrates and comparison of its biodiversity among various stretches. Applied Water Science, 11(8). https://doi.org/10.1007/s13201-021-01451-z

Kirui, S., Gershom, N. A., Maingi, J. M., & Kebira, A. (2019). Effects of Seasonality on Physico-Chemical Properties and Selected Ligands on Rivers in Sigor Division, West Pokot County, Kenya. Aquademia, 3(1). https://doi.org/10.29333/aquademia/8393

Lau, R. Y. S., Park, E., Tran, D. D., & Wang, J. (2023). Recent intensification of riverbed mining in the Mekong Delta revealed by extensive bathymetric surveying. Journal of Hydrology, 626, 130174. https://doi.org/10.1016/j.jhydrol.2023.130174

Lekomo, Y. K., Ekengoue, C. M., Douola, A., Lele, R. F., Suh, G. C., Obiri, S., & Dongmo, A. K. (2021). Assessing impacts of sand mining on water quality in Toutsang locality and design of waste water purification system. Cleaner Engineering and Technology, 2, 100045. https://doi.org/10.1016/j.clet.2021.100045

Liu, Y., Li, H., Cui, G., & Cao, Y. (2020). Water quality attribution and simulation of non-point source pollution load flux in the Hulan River basin. Scientific Reports, 10(1). https://doi.org/10.1038/s41598-020-59980-7

Lwanga, A. I., Tsingalia, H. M., Agevi, H., & Shilenje, Z. W. (2022). Effects of Sand-Harvesting on River Water Quality and Riparian Soil Physico-Chemical Properties. Open Journal of Ecology, 12(8), 570. https://doi.org/10.4236/oje.2022.128032

Mamun, M., Atique, U., Kim, J. Y., & An, K. (2021). Seasonal Water Quality and Algal Responses to Monsoon-Mediated Nutrient Enrichment, Flow Regime, Drought, and Flood in a Drinking Water Reservoir. International Journal of Environmental Research and Public Health, 18(20), 10714. https://doi.org/10.3390/ijerph182010714

Mishra, R., & Singh, D. (2020). Impact of Pollution on Kelo River of Raigarh District. Materials Today Proceedings, 29, 310. https://doi.org/10.1016/j.matpr.2020.07.280

Narchonai, G., Arutselvan, C., LewisOscar, F., & Thajuddin, N. (2019). Deciphering the microalgal diversity and water quality assessment of two urban temple ponds in Pondicherry, India. Biocatalysis and Agricultural Biotechnology, 22, 101427. https://doi.org/10.1016/j.bcab.2019.101427

Obiuto, N. J., Anyaele, U., & Flourizel, I. (2022). Seasonal variation and plankton physico-chemical characteristics of Omeremaduche River, Abia state, Niger delta, Nigeria. International Journal of Fisheries and Aquatic Studies, 10(2), 17. https://doi.org/10.22271/fish.2022.v10.i2a.2645

Omar, W. M. W. (2010). Perspectives on the use of algae as biological indicators for monitoring and protecting aquatic environments, with special reference to malaysian freshwater ecosystems. PubMed. https://pubmed.ncbi.nlm.nih.gov/24575199

Ouyang, Y., Nkedi‐Kizza, P., Wu, Q., Shinde, D., & Huang, C. H. (2006). Assessment of seasonal variations in surface water quality. Water Research, 40(20), 3800. https://doi.org/10.1016/j.watres.2006.08.030

Pant, R. R., Varol, M., Phuyal, S., Bhattarai, S., Awasthi, M. P., Thakur, T. K., Bohara, R., Basnet, N., Sitaula, S., & Afandi, G. E. (2025). How Sand Mining is Shaping the Trishuli River in the Himalayas of South Asia. Earth Systems and Environment. https://doi.org/10.1007/s41748-025-00569-3

Pramanik, A. K., Majumdar, D., & Chatterjee, A. (2020). Factors affecting lean, wet-season water quality of Tilaiya reservoir in Koderma District, India during 2013–2017. Water Science, 34(1), 85. https://doi.org/10.1080/11104929.2020.1765451

P.U., I., C.C., C., F.C., I., I.F., F., & C.A., O. (2017). A Review of Environmental Effects of Surface Water Pollution [Review of A Review of Environmental Effects of Surface Water Pollution]. International Journal of Advanced Engineering Research and Science, 4(12), 128. AI Publications. https://doi.org/10.22161/ijaers.4.12.21

Rahman, A., Jahanara, I., & Jolly, Y. N. (2021). Assessment of physicochemical properties of water and their seasonal variation in an urban river in Bangladesh. Water Science and Engineering, 14(2), 139. https://doi.org/10.1016/j.wse.2021.06.006

Rao, S. M., Anthony, P., & Mogili, N. V. (2019). Biochemical Indicators of Algal Bloom in Sewage-Contaminated Lakes. Journal of Hazardous Toxic and Radioactive Waste, 23(4). https://doi.org/10.1061/(asce)hz.2153-5515.0000459

Rasal, V. M., Yadre, S. G., Shukla, S. P., Ravi, P. M., Ravi, P. M., Mishra, M., Munilkumar, S., Pal, A. K., Lakra, W. S., & Dasgupta, S. (2019). Microalgae Distribution and Diversity in the Narmada River Basin around Chutka, Madhya Pradesh, India. International Journal of Current Microbiology and Applied Sciences, 8(9), 1488. https://doi.org/10.20546/ijcmas.2019.809.171

Razak, A., Takaijudin, H., Osman, M., Yusof, K. W., Ali, W., & Ishak, S. (2021). Assessment on biochemical oxygen demand (BOD) removal using treatment train system for agricultural run-off. IOP Conference Series Earth and Environmental Science, 847(1), 12020. https://doi.org/10.1088/1755-1315/847/1/012020

Rodrigues, V. A., Estrany, J., Ranzini, M., Cicco, V. de, Martín‐Benito, J. M. T., Hedo, J., & Lucas‐Borja, M. E. (2017). Effects of land use and seasonality on stream water quality in a small tropical catchment: The headwater of Córrego Água Limpa, São Paulo (Brazil). The Science of The Total Environment, 1553. https://doi.org/10.1016/j.scitotenv.2017.10.028

Rostami, S., He, J., & Hassan, Q. K. (2019). Water quality response to river flow regime at three major rivers in Alberta. Water Quality Research Journal, 55(1), 79. https://doi.org/10.2166/wqrj.2019.033

Thitanuwat, B., & Wongsoonthornchai, M. (2021). Estimation of Nitrogen Loading to Surface Water from Agriculture Based Area and Its Application for Water Pollution Mitigation. Environment and Natural Resources Journal, 19(3), 230. https://doi.org/10.32526/ennrj/19/2020239