Comparative Assessment of Emerging Contaminants Loads in Urban and Rural Wastewater Effluents: A Review

Abstract

Emerging contaminants (ECs), encompassing pharmaceuticals, personal care products (PPCPs), endocrine-disrupting compounds (EDCs), per- and polyfluoroalkyl substances (PFAS), microplastics, and antibiotic resistance genes (ARGs), are increasingly detected in wastewater effluents worldwide. This systematic review provides a comparative assessment of EC loads in urban and rural wastewater systems, highlighting global patterns, regional disparities, and the specific context of Nigeria. Relevant peer-reviewed studies from 2021–2026 were synthesized to evaluate concentration ranges, treatment  efficiency, and environmental implications. Results reveal that urban wastewater effluents generally exhibit higher EC loads, driven by higher population density, industrialization, and lifestyle-related chemical usage. However, rural effluents increasingly contribute non-negligible EC loads due to agricultural runoff, decentralized sanitation systems, and limited treatment infrastructure. The persistence of PFAS, hormones, and antibiotics underscores gaps in conventional treatment technologies, particularly in low- and middle-income countries. A synthesis from global to local scales demonstrates a clear gradient in data availability and monitoring sophistication, with Africa—especially Nigeria—representing a critical data-sparse region. Strengthening analytical capacity, promoting advanced oxidation and biological hybrid technologies, and integrating EC monitoring into national wastewater management policies are essential for sustainable control. This review provides a roadmap for comparative EC assessment, offering insights for future research, policy harmonization, and the protection of aquatic ecosystems in urban and rural contexts.

Keywords: Emerging contaminants, wastewater effluents, urban–rural comparison, systematic review, Nigeria, pharmaceuticals, PFAS.

1. Introduction

1.1. Background and Rationale

Emerging contaminants (ECs) are synthetic or naturally occurring chemicals not routinely monitored in the environment but known to pose potential risks to ecosystems and human health [1, 2]. These substances include pharmaceuticals, personal care products (PPCPs), endocrine-disrupting chemicals (EDCs), per- and polyfluoroalkyl substances (PFAS), microplastics, and antibiotic resistance genes (ARGs). Their ubiquitous presence in water, sediments, and biota has raised global concern, especially as conventional wastewater treatment plants (WWTPs) are not designed to completely remove them [3, 4].

Urban and rural areas differ significantly in terms of contaminant sources, infrastructure, and treatment technologies.  Urban wastewater usually comes from densely populated areas with high domestic, hospital, and industrial inputs, leading to complex mixtures of ECs [5]. Rural wastewater, conversely, often derives from agricultural runoff, livestock operations, and decentralized sanitation systems, resulting in distinct contaminant profiles [6]. Despite these differences, both urban and rural effluents serve as major pathways for EC release into receiving water bodies, threatening aquatic life and potentially entering the human food chain [7].

1.2. Global Significance of Emerging Contaminants

Globally, over 4,000 chemical compounds have been identified as emerging contaminants, many of which have been detected in wastewater effluents at concentrations ranging from nanograms to micrograms per liter [8]. Studies from Europe, North America, and East Asia consistently report high loads of pharmaceuticals—such as carbamazepine, diclofenac, and sulfamethoxazole—in urban effluents [9]. PFAS and microplastics have also emerged as contaminants of concern due to their persistence and bioaccumulative nature [8].

The persistence of ECs in treated wastewater reflects limitations in conventional treatment technologies, such as activated sludge systems and trickling filters, which were originally designed for the removal of biodegradable organic matter and nutrients, not trace synthetic chemicals (Matesun et al. 2024b). As a result, ECs frequently bypass treatment plants and enter aquatic systems, leading to chronic exposure and ecosystem disruption. [9]. In high-income countries, advanced treatment technologies—such as ozonation, membrane bioreactors (MBRs), and activated carbon filtration—have shown promise in EC removal. However, their high operational costs limit widespread adoption, particularly in developing regions [11].

1.3. Emerging Contaminants in Urban Wastewater Effluents

Urban wastewater effluents serve as concentrated sources of ECs due to intensive anthropogenic activities. Hospitals and households are key contributors of PPCPs and antibiotics, while industrial discharges introduce complex mixtures of solvents, surfactants, and heavy organics [12].

Reported EC concentrations in urban effluents often exceed 1 µg/L for compounds such as sulfamethoxazole, diclofenac, and caffeine [13]. Temporal studies indicate that EC loads fluctuate seasonally, influenced by rainfall, water usage, and treatment efficiency [14]. Urban effluents in megacities like Beijing, London, and São Paulo exhibit some of the highest EC diversity and concentrations globally, highlighting the influence of urbanization intensity [8].

The ecological and health implications are profound. Chronic exposure to low concentrations of EDCs like bisphenol A and estradiol has been linked to endocrine disruption in fish, reproductive toxicity, and altered microbial community structures [15]. Moreover, wastewater-borne ARGs promote antimicrobial resistance in natural water systems, an escalating global health threat (Zhao et al. 2023).

1.4. Emerging Contaminants in Rural Wastewater Effluents

Rural wastewater effluents, though less studied, represent a growing concern. The intensification of agriculture, extensive pesticide use, and inadequate sanitation contribute to diverse EC profiles [17]. In many rural and peri-urban settings, untreated or intermittently treated effluents are often discharged directly to surface waters because wastewater infrastructure is limited. Several studies from South Africa and other parts of Africa have detected veterinary-related antibiotics (notably sulfonamides) in streams and waterways impacted by such discharges, although direct, side-by-side comparisons showing rural loads are consistently comparable to urban discharges remain limited[18].

Furthermore, microplastics originating from plastic mulch, fertilizers, and synthetic textiles are increasingly found in rural effluents [19]. These particles can adsorb other ECs, facilitating their transport and persistence in aquatic environments [20]. The absence of centralized wastewater treatment systems exacerbates the accumulation of ECs in rural catchments, where natural attenuation processes are insufficient.

1.5. The African and Nigerian Context

In Africa, research on ECs remains limited compared to other regions, though recent studies reveal growing contamination across urban and peri-urban waterways [21]. South Africa, Egypt, and Nigeria have reported detectable levels of PPCPs, PFAS, and microplastics in wastewater and surface waters [22].

Nigeria, the most populous country in Africa, faces rapid urbanization, inadequate wastewater infrastructure, and weak regulatory capacity, resulting in frequent discharge of untreated or poorly treated effluents to the environment [23]. Only a fraction of municipal wastewater is treated, and most rural communities rely on pit latrines or open defecation systems. Consequently, both urban and rural wastewater streams act as unregulated sinks and sources for ECs. Recent studies have detected antibiotics, hormones, and plastics in Nigerian water bodies [24]. However, systematic data on comparative EC loads between urban and rural effluents remain scarce, underscoring the need for integrated national monitoring.

To illustrate the major contaminant sources, transport pathways, and environmental fate processes discussed in the introductory sections, Figure 1 presents a conceptual framework summarizing EC dynamics in both urban and rural wastewater systems.

Figure 1. Conceptual framework showing sources, pathways, and fate of emerging contaminants

1.6. Objectives and Scope of the Review

This systematic review aims to provide a comprehensive comparative assessment of emerging contaminant loads in urban and rural wastewater effluents using a top-down geographical framework—from global to Nigerian contexts. The specific objectives are to:

1. Identify and classify the major ECs reported in urban and rural wastewater effluents worldwide.

2. Compare concentration ranges, removal efficiencies, and ecological implications across regions.

3. Highlight existing data gaps and methodological inconsistencies.

4. Discuss the state of knowledge and policy gaps in Nigeria, with recommendations for future monitoring and control strategies.

By adopting this approach, this review synthesizes recent findings from peer-reviewed studies to establish a robust comparative understanding of EC dynamics, emphasizing the need for harmonized analytical methods and management policies in low- and middle-income regions.

in urban and rural wastewater systems

2. Methodology

2.1. Review Protocol

The methodology comprised three main stages : (i) literature identification, (ii) screening and eligibility assessment, and (iii) data extraction and synthesis. The process was designed to capture recent and relevant studies  addressing emerging contaminants (ECs) in urban and rural wastewater effluents.

A comprehensive literature search was conducted across major scientific databases including Scopus, Web of Science, ScienceDirect (Elsevier), and PubMed. Search strings combined key terms such as “emerging contaminants”, “urban wastewater”, “rural wastewater”, “pharmaceuticals”, “PFAS”, “microplastics”, and “antibiotic resistance genes”. Boolean operators and wildcard truncations (e.g., contaminant AND effluent AND urban OR rural) were applied to refine the scope.

2.2. Inclusion and Exclusion Criteria

The selection criteria were defined to ensure the inclusion of studies that:

1. Reported concentrations or loads of ECs in wastewater effluents (influent and/or treated).

2. Provided comparative or contextual data for urban and rural settings.

3. Contained explicit analytical details (instrumentation, detection limits, quality assurance).

Exclusion criteria included non-English publications, reviews without primary data, studies limited to industrial effluents, and reports lacking quantitative EC data.

3. Review and Discussion

3.1. Global Overview of Emerging Contaminants in Wastewater Effluents

Emerging contaminants (ECs) have transitioned from localized pollutants to global environmental priorities due to their persistence, bioactivity, and ubiquity [25, 26]. Studies across Europe, Asia, and North America reveal widespread detection of pharmaceuticals, endocrine disruptors, per- and polyfluoroalkyl substances (PFAS), and microplastics in both raw and treated wastewater effluents. Concentrations of PPCPs such as carbamazepine, sulfamethoxazole, and diclofenac often range between 50 and 2,000 ng/L in secondary effluents  [9]. Similarly, PFAS compounds such as perfluorooctanoic acid (PFOA) and perfluorooctanesulfonic acid (PFOS) have been reported at median levels of 80–300 ng/L, even in advanced treatment systems [27].

The persistence of ECs reflects both their structural stability and the inadequacy of conventional treatment processes. Primary and secondary wastewater treatment systems are designed primarily to remove suspended solids, organic carbon, and nutrients but not micro-pollutants at trace levels [28]. Advanced oxidation processes (AOPs), ozonation, membrane bioreactors (MBRs), and granular activated carbon (GAC) filtration have demonstrated improved EC removal but remain cost-intensive [29].

Recent meta-analyses [16, 30] highlight that even in regions with advanced technologies, over 30% of targeted ECs persist post-treatment. These residuals accumulate in surface waters, sediments, and biota, contributing to chronic exposure risks. Moreover, the co-occurrence of multiple ECs amplifies synergistic toxicity, an area still underexplored [31].

Urban wastewater systems dominate global EC studies due to higher analytical data availability. However, rural wastewater effluents, though representing smaller volumetric discharges, often exhibit comparable or higher per capita EC loads when normalized by wastewater volume [32]. The lack of treatment facilities in rural settings results in direct discharge, elevating contaminant persistence in soils and aquatic ecosystems [33].

3.2. Comparative Assessment: Urban versus Rural Wastewater Effluents

Urban effluents contain complex mixtures of chemicals originating from domestic sewage, hospital wastewater, and industrial discharges [34]. PPCPs and EDCs dominate urban EC profiles, particularly non-steroidal anti-inflammatory drugs (NSAIDs), beta-blockers, and synthetic hormones such as ethinylestradiol. Temporal monitoring in European and Asian megacities has shown that EC loads in urban wastewater are significantly correlated with population density, per capita income, and healthcare intensity [35].

In contrast, rural effluents are typically dominated by agricultural and veterinary pharmaceuticals, pesticides, and manure-related contaminants [19, 36]. The absence of centralized treatment plants in rural areas exacerbates pollutant dispersion. A study in Spain reported that rural effluents contained higher relative concentrations of tetracyclines and sulfonamides compared to nearby urban discharges [37]. Similar findings were reported in India and China, where decentralized treatment systems exhibited limited removal efficiency for antibiotics and PFAS [38]. To visualize the concentration differences reported across multiple studies, Figure 2 compares selected emerging contaminant levels in urban (WWTP) versus rural (on-site sanitation) wastewater effluents.

Figure 2: Comparative concentrations of selected emerging contaminants (ECs) in urban (WWTP) versus rural (on-site sanitation) wastewater effluents. Data adapted from [21, 39–41]

From a mechanistic standpoint, urban ECs often undergo partial transformation during secondary treatment, producing byproducts that may be more persistent or toxic than parent compounds [42]. Rural wastewater systems—often relying on septic tanks, lagoons, or stabilization ponds—tend to facilitate natural attenuation but are vulnerable to leaching and groundwater contamination [43].

Antibiotic resistance genes (ARGs) are a particular concern in both contexts. Urban effluents serve as hotspots for horizontal gene transfer due to dense microbial communities, while rural effluents from livestock operations provide selective pressure via sustained antibiotic residues [16]. The convergence of these sources enhances the environmental resistome, contributing to antimicrobial resistance spread.

3.3. Regional Overview: Emerging Contaminants in Africa

Compared to Europe and North America, Africa remains underrepresented in EC research. However, emerging data indicate significant contamination in both urban and rural effluents [44, 45]. In South Africa, monitoring studies also confirm the presence of pharmaceuticals in wastewater and receiving waters, including carbamazepine and sulfamethoxazole among more than 100 compounds detected across various regions [44]. A critical review of emerging pollutant occurrence in the Middle East and North Africa (MENA) region reports that pharmaceuticals, including anti-inflammatories and antibiotics, as well as microplastics, are commonly detected in water bodies and treated wastewater systems [46]. In East Africa, emerging contaminant studies document the occurrence of both pharmaceutical compounds and PFAS in water resources, including PPCPs and PFAS detected in Kenyan wastewater and surface waters [47]. Table 1 presents representative data on emerging contaminants reported in wastewater effluents across Africa between 2021 and 2026, summarizing key contaminant classes, measured concentrations, and source studies.

Table 1: Selected emerging contaminants in wastewater effluents across Africa (2021–2026)

| Contaminant class | Example compounds / markers | Representative concentrations / findings | Country / region (study) | Ref |
| --- | --- | --- | --- | --- |
| Pharmaceuticals (general) | Analgesics, NSAIDs, antiretrovirals, antibiotics (e.g., ibuprofen, diclofenac, carbamazepine, ciprofloxacin) | Reported concentrations span ng/L → µg/L in wastewater and receiving waters; >100 pharmaceuticals detected across South Africa (review synthesis). | South Africa (national review, many WWTPs & rivers). | [44] |
| Antibiotics (sewage sludge / wastewater) | Fluoroquinolones, macrolides, sulfonamides (e.g., ciprofloxacin, ofloxacin, trimethoprim) | Antibiotics detected in hospital WWTP sludge; ciprofloxacin up to 674 ng·g⁻¹ (dry weight) in Nigerian hospital sludge; all 14 target antibiotics detected at least once. | Nigeria (two hospital WWTPs — Ibadan & Lagos). | [48] |
| PFAS (per- and poly-fluoroalkyl substances) | PFOS, PFOA, PFPeA, fluorotelomer sulfonates | Mean PFAS detected across sampled treatment/drinking-water plants; province maxima reported up to ~740 ng·L⁻¹ (varied by PFAS and site/season). | South Africa (multi-province sampling across drinking-water / treatment plants). | [49] |
| Pesticides (river/WWTP influence) | Triazines, neonicotinoids, organophosphates (e.g., atrazine, imidacloprid, chlorpyrifos, terbuthylazine) | Passive-sampling campaign found year-round pesticide detections; EQS exceedances recorded (e.g., imidacloprid, chlorpyrifos, terbuthylazine, spiroxamine) — indicates WWTPs can be a pesticide source in agricultural catchments. | Western Cape agricultural catchments, South Africa (Grabouw, Piketberg, Hex River Valley). | [50] |
| Microplastics (MPs) | Fibres & fragments; common polymers: PE, PP, PET, PES | Review synthesis: MPs reported in South African freshwater and treated waters; freshwater concentrations ~0.33–56 particles·L⁻¹ (varies by site/method); WWTPs identified as important MP sources and show limited removal for many MP size classes. | South Africa (multiple river, reservoir, WWTP and drinking-water studies compiled). | [51] |

Rural communities in Africa often lack centralized wastewater collection, resulting in direct discharge or infiltration of sewage into surface waters and groundwater [52]. For example, in Ethiopia, sulfamethoxazole and other antibiotics have been detected in rivers and groundwater, reflecting limited treatment and disposal infrastructure [53]. These findings illustrate that rural effluents, while less voluminous, exert disproportionately large ecological impacts.

Moreover, the analytical capacity for detecting such contaminants is constrained by limited access to advanced instrumentation like LC–MS/MS and GC–MS in many African laboratories, leading to underestimation of contaminant occurrence [40].

3.4. West African Perspective

West Africa presents a complex interplay between rapid urbanization, weak wastewater infrastructure, and agricultural intensification. In Ghana, Nigeria, and Benin, EC loads are driven by informal settlements, industrial clusters, and mixed domestic–agricultural discharges. In Ghana, irrigation and poultry-farm-related water systems have been shown to contain antibiotic residues and analgesics, including ibuprofen and caffeine [54]. High levels of antimicrobial usage in both commercial and domestic poultry farms have been documented [55].

In Benin and other parts of West Africa, untreated or poorly managed on-site sanitation systems — including pit latrines and septic tanks — increasingly contribute to the leaching of contaminants of emerging concern such as pharmaceuticals and endocrine-disrupting chemicals into shallow aquifers and peri-urban wetlands. Evidence from a recent systematic global review of on-site sanitation–groundwater interactions confirms that leakage and contaminant transport risks are strongly influenced by local hydrogeology, system age, and maintenance quality, and that regulatory oversight remains limited in many low- and middle-income settings (Mbae et al. 2024). Across West Africa, a comprehensive regional review reports widespread detection of pharmaceuticals and personal care products across wastewater, surface water, and groundwater systems, highlighting concentration ranges in the ng/L to low-µg/L scale and noting extensive monitoring gaps in countries such as Benin, Ghana, and Nigeria [45]. For PFAS and endocrine-disrupting chemicals, country-specific evaluations show that surveillance capacity remains extremely limited; for example, a recent assessment for Nigeria identifies substantial analytical and regulatory gaps that hinder accurate ecotoxicological and human-health risk assessment, a situation representative of the broader West African region [56]

3.5. The Nigerian Context

Nigeria exemplifies the dual challenge of urban expansion and rural underdevelopment in wastewater management. Despite its population exceeding 220 million, less than 10% of municipal wastewater undergoes any form of treatment [48]. Urban centers such as Lagos, Abuja, and Ibadan discharge large volumes of untreated or partially treated sewage directly into surface waters. Conversely, rural communities rely predominantly on pit latrines, septic tanks, or open defecation systems, with no formal effluent treatment.

In Nigeria, recent monitoring reveals substantial contamination of surface waters by pharmaceuticals: in Lagos State 26 different pharmaceuticals — including antibiotics — were detected in wastewater-impacted rivers and sewage effluent, with several compounds reaching up to 129 µg/L [57]. Meanwhile, studies on sediments from rivers in Southwestern Nigeria, such as the Ogun River basin, confirm microplastic pollution in receiving waters [58].

These data show that emerging contaminants — pharmaceuticals and microplastics — are present in Nigerian aquatic systems. However, data gaps remain for other classes of ECs (e.g. PFAS, bisphenol A, EDCs) in many regions.

In Nigeria, wastewater-related pollution and contaminated groundwater present important environmental health risks. Studies from rural areas show that boreholes and wells — common water sources for communities — are contaminated with antibiotics and other emerging pollutants, likely as a result of inadequate sanitation and waste-handling infrastructure [59]. Meanwhile, investigations of sewage sludge from hospital wastewater treatment plants have revealed accumulation of multiple antibiotics, demonstrating that even treated wastewater streams can contribute to environmental EC loads via sludge disposal [48]. Regulatory oversight remains limited: Nigeria’s principal sanitation and waste-control regulation (2009) does not specify standards for trace organics or emerging contaminants, exposing a regulatory gap in control of ECs.

These findings suggest that both rural groundwater use and wastewater treatment practices in Nigeria may contribute to environmental exposure to ECs. However, the literature is still sparse — data remain fragmentary, and there are no recent peer-reviewed studies that comprehensively compare urban vs rural wastewater EC profiles across multiple contaminant classes. This underscores an urgent need for systematic monitoring, regulatory updates, and research on decentralized or low-cost treatment systems appropriate for Nigeria’s varied urban and rural contexts.

3.6. Treatment Efficiency and Policy Disparities

Treatment efficiency for ECs varies widely across technologies and geographies. Advanced systems (MBR, ozonation, GAC) achieve >80% removal for most PPCPs but remain rare in low-income contexts (Kumar et al., 2023). Biological treatments, such as activated sludge and wetlands, exhibit variable removal efficiencies—typically 30–70%, depending on contaminant class and operational parameters [60].

In Africa, the implementation of advanced treatment technologies for emerging contaminants (ECs) is constrained by persistent economic, infrastructural, and governance challenges. Peer-reviewed assessments highlight that many water and sanitation systems across the continent continue to operate below design capacity due to chronic underfunding, limited maintenance, and institutional fragmentation [61]. These systemic constraints impede the adoption of higher-level treatment processes capable of removing pharmaceuticals, PFAS, and other micropollutants.

At the global level, regulatory frameworks are increasingly shifting toward integrated monitoring of emerging contaminants (ECs) and the use of ecotoxicological risk assessment to inform policy. In the European Union, recent developments in water-policy legislation reflect this trend: scientific evaluations of the EU’s updated Water Framework Directive proposals indicate that pharmaceuticals, PFAS, and other emerging pollutants are now being prioritized for more systematic monitoring and regulatory attention [62]. Complementary assessments further highlight that these substances are increasingly ranked and selected for inclusion in EU monitoring schemes through structured prioritization tools designed to support Watch-List decision-making[63]. Scientific analyses of these reforms emphasize that coupling chemical monitoring with ecological-risk evaluation is becoming an international benchmark for modern water-quality regulation [62].

4. Future Perspectives and Challenges

Despite significant progress in understanding emerging contaminants (ECs), multiple challenges persist in achieving comprehensive management, particularly in low- and middle-income regions. The future of EC control depends on four interconnected dimensions—analytical advancement, policy integration, treatment innovation, and public awareness.

4.1. Analytical and Monitoring Gaps

Accurate quantification of emerging contaminants (ECs) is still constrained by technological and financial limitations, particularly in low- and middle-income regions. Recent global reviews emphasize that advanced analytical platforms such as LC–MS/MS and high-resolution mass spectrometry are essential for sensitive EC detection, yet their high cost, technical complexity, and lack of standardized methods limit widespread deployment and routine monitoring [9]. In Africa, these challenges are magnified by shortages of mass spectrometry infrastructure and trained personnel: a recent review on PFAS detection explicitly highlights the limited availability of MS-based methods as a major barrier to reliable contaminant monitoring on the continent [21]. Regional assessments of contaminants of emerging concern in African wastewater effluents similarly note that research and monitoring capacity remain concentrated in high-income regions, with most African countries only beginning to build analytical capability [40].

To close these gaps, future EC-monitoring frameworks should prioritize capacity building through regional reference laboratories, investment in advanced mass-spectrometric instrumentation, and the development of harmonized detection protocols. In parallel, suspect and non-target screening approaches based on high-resolution mass spectrometry—already widely promoted in Europe through NORMAN network guidance and regulatory-oriented applications—offer powerful tools to expand detection beyond a predefined target list and should be adapted for use in resource-limited settings [64, 65].

4.2. Technological and Treatment Innovation

The development of cost-effective treatment technologies is critical. Biochar-amended constructed wetlands, microalgae-based systems, and photocatalytic nanomaterials show promise for EC removal [66–68]. Integration of advanced oxidation processes with biological treatment improves degradation of persistent compounds such as pharmaceuticals and some PFAS precursors [69]. Scalability depends on energy efficiency, maintenance simplicity, and local material availability; solar-driven hybrid reactors and solar-assisted photocatalytic systems are promising for tropical regions [70].

4.3. Policy and Regulatory Evolution

At the policy level, governance of emerging contaminants (ECs) remains fragmented, with only a few countries worldwide adopting formal EC monitoring guidelines, and none in sub-Saharan Africa establishing enforceable effluent standards. Incorporating EC parameters into national water-quality indices and effluent discharge permits would represent substantial progress. A risk-based monitoring model—similar to approaches promoted in recent EU water-policy reforms, which emphasize prioritization of pharmaceuticals, PFAS, and other ECs for systematic surveillance—could be adapted for African regulatory contexts [62]

5. Conclusion

This systematic review provides a comprehensive comparative synthesis of emerging contaminant (EC) loads in urban and rural wastewater effluents, with a geographical progression from global to Nigerian contexts. Across all scales, urban effluents consistently exhibit higher EC concentrations and diversity, primarily driven by intensive domestic, medical, and industrial activities. However, rural effluents, characterized by limited treatment and extensive agricultural inputs, increasingly contribute significant pollutant loads to surface and groundwater systems.

Persistent compounds—such as PFAS, EDCs, and antibiotics—remain inadequately removed by conventional wastewater treatment systems. The global disparity in monitoring infrastructure and policy attention underscores the need for harmonized frameworks, especially in Africa and Nigeria, where data scarcity and weak regulation prevail. Nigeria exemplifies a dual challenge: rapidly expanding urban centers lacking adequate treatment facilities and rural communities dependent on decentralized sanitation systems.

Future management should focus on (i) strengthening national monitoring and laboratory capacity, (ii) developing affordable hybrid treatment systems suited to local contexts, and (iii) integrating ECs into environmental policy and education programs. Bridging the urban-rural gap requires recognizing both the technological and socio-economic dimensions of wastewater governance.

Ultimately, controlling EC pollution demands a holistic, multi-scalar strategy that aligns scientific innovation with environmental justice—ensuring that all communities, regardless of geography or infrastructure, are protected from the long-term risks posed by emerging contaminants.

Competing Interests

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

[1]	Wydro, U., Wołejko, E., Luarasi, L., Puto, K., Tarasevičienė, Ž., & Jabłońska-Trypuć, A. (2024). A Review on Pharmaceuticals and Personal Care Products Residues in the Aquatic Environment and Possibilities for Their Remediation. Sustainability, 16(1), 169. https://doi.org/10.3390/su16010169

[2]	Nishmitha, P. S., Akhilghosh, K. A., Aiswriya, V. P., Ramesh, A., Muthuchamy, M., & Muthukumar, A. (2025). Understanding Emerging Contaminants in Water and Wastewater: A Comprehensive Review on Detection, Impacts, and Solutions. Journal of Hazardous Materials Advances. https://doi.org/10.1016/j.hazadv.2025.100755

[3]	Sanzana, S., Fenti, A., Iovino, P., & Panico, A. (2025). A Review of PFAS Remediation: Separation and Degradation Technologies for Water and Wastewater Treatment. Journal of Water Process Engineering, 74, 107793. https://doi.org/10.1016/j.jwpe.2025.107793

[4]	Kumar, M., Sridharan, S., Sawarkar, A. D., Shakeel, A., Anerao, P., Mannina, G., Sharma, P., & Pandey, A. (2023). Current research trends on emerging contaminants pharmaceutical and personal care products (PPCPs): A comprehensive review. Science of The Total Environment, 859, 160031. https://doi.org/10.1016/j.scitotenv.2022.160031

[5]	Yang, W.; Bu, Q.; Shi, Q.; Zhao, R.; Huang, H.; Yang, L.; Tang, J.; Ma, Y. Emerging Contaminants in the Effluent of Wastewater Should Be Regulated: Which and to What Extent? Toxics, 2024, 12 (5), 309. https://doi.org/10.3390/toxics12050309.

[6]	Naeem, M.; Gill, R.; Gill, S. S.; Singh, K.; Sofo, A.; Tuteja, N. Editorial: Emerging Contaminants and Their Effect on Agricultural Crops. Front Plant Sci, 2023, 14. https://doi.org/10.3389/fpls.2023.1296252.

[7]	Wang, F.; Xiang, L.; Sze-Yin Leung, K.; Elsner, M.; Zhang, Y.; Guo, Y.; Pan, B.; Sun, H.; An, T.; Ying, G.; et al. Emerging Contaminants: A One Health Perspective. The Innovation, 2024, 5 (4), 100612. https://doi.org/10.1016/j.xinn.2024.100612.

[8]	Jiang, T.; Wu, W.; Ma, M.; Hu, Y.; Li, R. Occurrence and Distribution of Emerging Contaminants in Wastewater Treatment Plants: A Globally Review over the Past Two Decades. Science of The Total Environment, 2024, 951, 175664. https://doi.org/10.1016/j.scitotenv.2024.175664.

[9]	Li, X.; Shen, X.; Jiang, W.; Xi, Y.; Li, S. Comprehensive Review of Emerging Contaminants: Detection Technologies, Environmental Impact, and Management Strategies. Ecotoxicol Environ Saf, 2024, 278, 116420. https://doi.org/10.1016/j.ecoenv.2024.116420.

[10]	Matesun, J.; Petrik, L.; Musvoto, E.; Ayinde, W.; Ikumi, D. Limitations of Wastewater Treatment Plants in Removing Trace Anthropogenic Biomarkers and Future Directions: A Review. Ecotoxicol Environ Saf, 2024, 281, 116610. https://doi.org/10.1016/j.ecoenv.2024.116610.

[11]	Wang, H.; Gao, X.; Zuo, Y. Research and Application of Water Treatment Technologies for Emerging Contaminants (ECs): A Pathway to Solving Water Environment Challenges. Water (Basel), 2024, 16 (13), 1837. https://doi.org/10.3390/w16131837.

[12]	Verlicchi, P.; Galletti, A.; Petrovic, M.; Barceló, D. Hospital Effluents as a Source of Emerging Pollutants: An Overview of Micropollutants and Sustainable Treatment Options. J Hydrol (Amst), 2010, 389 (3–4), 416–428. https://doi.org/10.1016/j.jhydrol.2010.06.005.

[13]	Netshithothole, R.; Managa, M.; Botha, T.; Madikizela, L. Occurrence of Selected Pharmaceuticals in Wastewater and Sludge Samples from Wastewater Treatment Plants in Eastern Cape Province of South Africa. South African Journal of Chemistry, 2024, 78, 7–14. https://doi.org/10.17159/0379-4350/2024/v78a02.

[14]	Paíga, P.; Correia-Sá, L.; Correia, M.; Figueiredo, S.; Vieira, J.; Jorge, S.; Silva, J. G.; Delerue-Matos, C. Temporal Analysis of Pharmaceuticals as Emerging Contaminants in Surface Water and Wastewater Samples: A Case Study. J Xenobiot, 2024, 14 (3), 873–892. https://doi.org/10.3390/jox14030048.

[15]	Dasmahapatra, A. K.; Williams, C. B.; Myla, A.; Tiwary, S. K.; Tchounwou, Paul. B. A Systematic Review of the Evaluation of Endocrine-Disrupting Chemicals in the Japanese Medaka (Oryzias Latipes) Fish. Frontiers in Toxicology, 2023, 5. https://doi.org/10.3389/ftox.2023.1272368.

[16]	Zhao, F.; Yu, Q.; Zhang, X.-X. A Mini-Review of Antibiotic Resistance Drivers in Urban Wastewater Treatment Plants: Environmental Concentrations, Mechanism and Perspectives. Water (Basel), 2023, 15 (17), 3165. https://doi.org/10.3390/w15173165.

[17]	Khmaissa, M.; Zouari-Mechichi, H.; Sciara, G.; Record, E.; Mechichi, T. Pollution from Livestock Farming Antibiotics an Emerging Environmental and Human Health Concern: A Review. Journal of Hazardous Materials Advances, 2024, 13, 100410. https://doi.org/10.1016/j.hazadv.2024.100410.

[18]	Ncube, S.; Nuapia, Y. B.; Chimuka, L.; Madikizela, L. M.; Etale, A. Trace Detection and Quantitation of Antibiotics in a South African Stream Receiving Wastewater Effluents and Municipal Dumpsite Leachates. Front Environ Sci, 2021, 9. https://doi.org/10.3389/fenvs.2021.733065.

[19]	Sa’adu, I.; Farsang, A. Plastic Contamination in Agricultural Soils: A Review. Environ Sci Eur, 2023, 35 (1), 13. https://doi.org/10.1186/s12302-023-00720-9.

[20]	Cobos, A. G. Z.; Ochoa, K. P.; Portilla, F.; Duque, P. Emerging Contaminants in Rural Water: Microplastic Pollution and Its Association with Agricultural, Livestock, and Industrial Activities in Ecuador. Environ Monit Assess, 2025, 197 (12), 1337. https://doi.org/10.1007/s10661-025-14761-5.

[21]	Necibi, M. C.; Dhiba, D.; El Hajjaji, S. Contaminants of Emerging Concern in African Wastewater Effluents: Occurrence, Impact and Removal Technologies. Sustainability, 2021, 13 (3), 1125. https://doi.org/10.3390/su13031125.

[22]	Okwuosa, R.; Nomngongo, P. N.; Olatunji, O. S.; Okonkwo, O. J. Legacy and Emerging Poly and Perfluoroalkyl Substances (PFAS) in Surface Water, Sediments, and Treated Effluent: A Case Study in Pretoria (South Africa). Water Air Soil Pollut, 2025, 236 (5), 326. https://doi.org/10.1007/s11270-025-07984-2.

[23]	Onu, M. A.; Ayeleru, O. O.; Oboirien, B.; Olubambi, P. A. Challenges of Wastewater Generation and Management in Sub-Saharan Africa: A Review. Environmental Challenges, 2023, 11, 100686. https://doi.org/10.1016/j.envc.2023.100686.

[24]	Wada, O. Z.; Olawade, D. B. Recent Occurrence of Pharmaceuticals in Freshwater, Emerging Treatment Technologies, and Future Considerations: A Review. Chemosphere, 2025, 374, 144153. https://doi.org/10.1016/j.chemosphere.2025.144153.

[25]	Boahen, E.; Owusu, L.; Adjei-Anim, S. O. A Comprehensive Review of Emerging Environmental Contaminants of Global Concern. Discover Environment, 2025, 3 (1), 144. https://doi.org/10.1007/s44274-025-00259-x.

[26]	Zhang, H.; Shen, N.; Li, Y.; Hu, C.; Yuan, P. Source, Transport, and Toxicity of Emerging Contaminants in Aquatic Environments: A Review on Recent Studies. Environmental Science and Pollution Research, 2023, 30 (58), 121420–121437. https://doi.org/10.1007/s11356-023-30869-y.

[27]	Li, Y.; Li, Y.; Zhang, S.; Gao, T.; Gao, Z.; Lai, C. W.; Xiang, P.; Yang, F. Global Distribution, Ecotoxicity, and Treatment Technologies of Emerging Contaminants in Aquatic Environments: A Recent Five-Year Review. Toxics, 2025, 13 (8), 616. https://doi.org/10.3390/toxics13080616.

[28]	Falås, P.; Wick, A.; Castronovo, S.; Habermacher, J.; Ternes, T. A.; Joss, A. Tracing the Limits of Organic Micropollutant Removal in Biological Wastewater Treatment. Water Res, 2016, 95, 240–249. https://doi.org/10.1016/j.watres.2016.03.009.

[29]	Mukherjee, J.; Lodh, B. K.; Sharma, R.; Mahata, N.; Shah, M. P.; Mandal, S.; Ghanta, S.; Bhunia, B. Advanced Oxidation Process for the Treatment of Industrial Wastewater: A Review on Strategies, Mechanisms, Bottlenecks and Prospects. Chemosphere, 2023, 345, 140473. https://doi.org/10.1016/j.chemosphere.2023.140473.

[30]	Melvin, S. D.; Leusch, F. D. L. Removal of Trace Organic Contaminants from Domestic Wastewater: A Meta-Analysis Comparison of Sewage Treatment Technologies. Environ Int, 2016, 92–93, 183–188. https://doi.org/10.1016/j.envint.2016.03.031.

[31]	Cheng, Y.; Ding, J.; Davila Arenas, C. E.; Brinkmann, M.; Ji, X. A Brief Review on the Assessment of Potential Joint Effects of Complex Mixtures of Contaminants in the Environment. Environmental Science: Advances, 2024, 3 (5), 661–675. https://doi.org/10.1039/D4VA00014E.

[32]	Delgado, N.; Orozco, J.; Zambrano, S.; Casas-Zapata, J. C.; Marino, D. Veterinary Pharmaceutical as Emerging Contaminants in Wastewater and Surface Water: An Overview. J Hazard Mater, 2023, 460, 132431. https://doi.org/10.1016/j.jhazmat.2023.132431.

[33]	Undeman, E.; Rasmusson, K.; Kokorite, I.; Leppänen, M. T.; Larsen, M. M.; Pazdro, K.; Siedlewicz, G. Micropollutants in Urban Wastewater: Large-Scale Emission Estimates and Analysis of Measured Concentrations in the Baltic Sea Catchment. Mar Pollut Bull, 2022, 178, 113559. https://doi.org/10.1016/j.marpolbul.2022.113559.

[34]	Verlicchi, P.; Al Aukidy, M.; Zambello, E. Occurrence of Pharmaceutical Compounds in Urban Wastewater: Removal, Mass Load and Environmental Risk after a Secondary Treatment—A Review. Science of The Total Environment, 2012, 429, 123–155. https://doi.org/10.1016/j.scitotenv.2012.04.028.

[35]	Rodriguez-Mozaz, S.; Vaz-Moreira, I.; Varela Della Giustina, S.; Llorca, M.; Barceló, D.; Schubert, S.; Berendonk, T. U.; Michael-Kordatou, I.; Fatta-Kassinos, D.; Martinez, J. L.; et al. Antibiotic Residues in Final Effluents of European Wastewater Treatment Plants and Their Impact on the Aquatic Environment. Environ Int, 2020, 140, 105733. https://doi.org/10.1016/j.envint.2020.105733.

[36]	Mutuku, C.; Gazdag, Z.; Melegh, S. Occurrence of Antibiotics and Bacterial Resistance Genes in Wastewater: Resistance Mechanisms and Antimicrobial Resistance Control Approaches. World J Microbiol Biotechnol, 2022, 38 (9), 152. https://doi.org/10.1007/s11274-022-03334-0.

[37]	Conde-Cid, M.; Núñez-Delgado, A.; Fernández-Sanjurjo, M.; Álvarez-Rodríguez, E.; Fernández-Calviño, D.; Arias-Estévez, M. Tetracycline and Sulfonamide Antibiotics in Soils: Presence, Fate and Environmental Risks. Processes, 2020, 8 (11), 1479. https://doi.org/10.3390/pr8111479.

[38]	Sharma, M.; Yadav, A.; Dubey, K. K.; Tipple, J.; Das, D. B. Decentralized Systems for the Treatment of Antimicrobial Compounds Released from Hospital Aquatic Wastes. Science of The Total Environment, 2022, 840, 156569. https://doi.org/10.1016/j.scitotenv.2022.156569.

[39]	Mbae, M.; Hansen, P.; Way, C.; Mills, F.; Willetts, J.; Foster, T.; Evans, B. Onsite Sanitation Systems and Contamination of Groundwater: A Systematic Review of the Evidence for Risk Using the Source-Pathway-Receptor Model. PLOS Water, 2024, 3 (7), e0000167. https://doi.org/10.1371/journal.pwat.0000167.

[40]	Aborode, A. T.; Adesola, R. O.; Idris, I.; Sakariyau Adio, W.; Olapade, S.; Oluwafisayo, G.; Onifade, I. A.; Fakorede, S.; Bakare-Abidola, T.; Olaoye, J.; et al. Challenges Associated With PFAS Detection Method in Africa. Environ Health Insights, 2025, 19. https://doi.org/10.1177/11786302241310430.

[41]	Kodom, K.; Attiogbe, F.; Kuranchie, F. A. Assessment of Removal Efficiency of Pharmaceutical Products from Wastewater in Sewage Treatment Plants: A Case of the Sewerage Systems Ghana Limited, Accra. Heliyon, 2021, 7 (11), e08385. https://doi.org/10.1016/j.heliyon.2021.e08385.

[42]	Matesun, J.; Petrik, L.; Musvoto, E.; Ayinde, W.; Ikumi, D. Limitations of Wastewater Treatment Plants in Removing Trace Anthropogenic Biomarkers and Future Directions: A Review. Ecotoxicol Environ Saf, 2024, 281, 116610. https://doi.org/10.1016/j.ecoenv.2024.116610.

[43]	Kofi Yeboah Adjei; Abdulmumin Aliyu; Emmanuel Owhe-Ureghe; Enosata Anita Oriakhi; Taiwo Bakare-Abidola; Jelil Olaoye; Yetunde Oyinkansola Adepoju. Biological Treatment of Emerging Organic Micropollutants in Wastewater: Recent Advances and Perspectives. World Journal of Biology Pharmacy and Health Sciences, 2025, 21 (3), 488–505. https://doi.org/10.30574/wjbphs.2025.21.3.0181.

[44]	Munzhelele, E. P.; Mudzielwana, R.; Ayinde, W. B.; Gitari, W. M. Pharmaceutical Contaminants in Wastewater and Receiving Water Bodies of South Africa: A Review of Sources, Pathways, Occurrence, Effects, and Geographical Distribution. Water (Basel), 2024, 16 (6), 796. https://doi.org/10.3390/w16060796.

[45]	Cangola, J.; Abagale, F. K.; Cobbina, S. J. A Systematic Review of Pharmaceutical and Personal Care Products as Emerging Contaminants in Waters: The Panorama of West Africa. Science of The Total Environment, 2024, 911, 168633. https://doi.org/10.1016/j.scitotenv.2023.168633.

[46]	Haddaoui, I.; Mateo-Sagasta, J. A Review on Occurrence of Emerging Pollutants in Waters of the MENA Region. Environmental Science and Pollution Research, 2021, 28 (48), 68090–68110. https://doi.org/10.1007/s11356-021-16558-8.

[47]	Shikuku, V. O.; Ngeno, E. C.; Njewa, J. B.; Ssebugere, P. Pharmaceutical and Personal Care Products (PPCPs) and per- and Polyfluoroalkyl Substances (PFAS) in East African Water Resources: Progress, Challenges, and Future. Physical Sciences Reviews, 2024, 9 (1), 291–308. https://doi.org/10.1515/psr-2022-0124.

[48]	Ajibola, A. S.; Zwiener, C. Occurrence and Risk Assessment of Antibiotic Residues in Sewage Sludge of Two Nigerian Hospital Wastewater Treatment Plants. Water Air Soil Pollut, 2022, 233 (10), 405. https://doi.org/10.1007/s11270-022-05875-4.

[49]	Rapoo, S. M.; Thaoge-Zwane, M. L.; Okonkwo, J. O. Distribution Pattern, Source Apportionment, and Health Risk Assessment of per- and Polyfluoroalkyl Substances in Drinking Water Treatment Plants in South Africa. Environ Toxicol Chem, 2025, 44 (8), 2133–2142. https://doi.org/10.1093/etojnl/vgaf136.

[50]	Davies, E.; Stamm, C.; Fuhrimann, S.; Chow, R. Mixed Pesticide Sources Identified by Using Wastewater Tracers in Rivers of South African Agricultural Catchments. Science of The Total Environment, 2024, 956, 177206. https://doi.org/10.1016/j.scitotenv.2024.177206.

[51]	Mokgalaka-Fleischmann, N. S.; Melato, F. A.; Netshiongolwe, K.; Izevbekhai, O. U.; Lepule, S. P.; Motsepe, K.; Edokpayi, J. N. Microplastic Occurrence and Fate in the South African Environment: A Review. Environmental Systems Research, 2024, 13 (1), 59. https://doi.org/10.1186/s40068-024-00389-w.

[52]	Tegegne, A. A.; Mekasha, Y. T.; Ayu, A. A.; Hasen, G.; Suleman, S. A Review on Emerging Pharmaceutical Residues in Ethiopia: Occurrence, Ecotoxicological Aspects, and Regulatory Concerns. Front Microbiol, 2024, 15. https://doi.org/10.3389/fmicb.2024.1499487.

[53]	Abafe, O. A.; Lawal, M. A.; Chokwe, T. B. Non-Targeted Screening of Emerging Contaminants in South African Surface and Wastewater. Emerg Contam, 2023, 9 (4), 100246. https://doi.org/10.1016/j.emcon.2023.100246.

[54]	Otoo, B. A.; Amoabeng, I. A.; Darko, G.; Borquaye, L. S. Antibiotic and Analgesic Residues in the Environment – Occurrence and Ecological Risk Study from the Sunyani Municipality, Ghana. Toxicol Rep, 2022, 9, 1491–1500. https://doi.org/10.1016/j.toxrep.2022.07.003.

[55]	Paintsil, E. K.; Ofori, L. A.; Akenten, C. W.; Fosu, D.; Ofori, S.; Lamshöft, M.; May, J.; Danso, K. O.; Krumkamp, R.; Dekker, D. Antimicrobial Usage in Commercial and Domestic Poultry Farming in Two Communities in the Ashanti Region of Ghana. Antibiotics, 2021, 10 (7), 800. https://doi.org/10.3390/antibiotics10070800.

[56]	Kikanme, K. N.; Dennis, N. M.; Orikpete, O. F.; Ewim, D. R. E. PFAS in Nigeria: Identifying Data Gaps That Hinder Assessments of Ecotoxicological and Human Health Impacts. Heliyon, 2024, 10 (9), e29922. https://doi.org/10.1016/j.heliyon.2024.e29922.

[57]	Ogunbanwo, O. M.; Kay, P.; Boxall, A. B.; Wilkinson, J.; Sinclair, C. J.; Shabi, R. A.; Fasasi, A. E.; Lewis, G. A.; Amoda, O. A.; Brown, L. E. High Concentrations of Pharmaceuticals in a Nigerian River Catchment. Environ Toxicol Chem, 2022, 41 (3), 551–558. https://doi.org/10.1002/etc.4879.

[58]	Shokunbi, O. S.; Idowu, G. A.; Davidson, C. M.; Aiyesanmi, A. F. Investigation of Microplastics and Potentially Toxic Elements (PTEs) in Sediments of Two Rivers in Southwestern Nigeria. Environ Monit Assess, 2024, 196 (10), 947. https://doi.org/10.1007/s10661-024-13090-3.

[59]	Akintobi, O. A.; Adedeji, A. O.; Ogunlaja, A.; Ladokun, O. A.; Ogunlaja, O. O. Occurrence and Exposure Assessment of Antibiotics, Parabens, and Potentially Toxic Elements in Groundwater Sources from Two Rural Communities in Nigeria. Environ Toxicol Chem, 2025, 44 (11), 3336–3348. https://doi.org/10.1093/etojnl/vgaf179.

[60]	Oluwole, A. O.; Omotola, E. O.; Olatunji, O. S. Pharmaceuticals and Personal Care Products in Water and Wastewater: A Review of Treatment Processes and Use of Photocatalyst Immobilized on Functionalized Carbon in AOP Degradation. BMC Chem, 2020, 14 (1), 62. https://doi.org/10.1186/s13065-020-00714-1.

[61]	Mutschinski, K.; Coles, N. A. The African Water Vision 2025: Its Influence on Water Governance in the Development of Africa’s Water Sector, with an Emphasis on Rural Communities in Kenya: A Review. Water Policy, 2021. https://doi.org/10.2166/wp.2021.032.

[62]	Backhaus, T. Commentary on the EU Commission’s Proposal for Amending the Water Framework Directive, the Groundwater Directive, and the Directive on Environmental Quality Standards. Environ Sci Eur, 2023, 35 (1), 22. https://doi.org/10.1186/s12302-023-00726-3.

[63]	Yulikayani, P. Y., De Witte, B., Ali, A., Barber, J., Bellas, J., Briant, N., Brooks, S., Bruvold, A. S., French, M. A., Hylland, K., Kaberi, H., León, V. M., Martins, S., Mauffret, A., Molinari, F., Reid, M., Sturve, J., Van Poucke, C., Valdersnes, S., Walgraeve, C., Demeestere, K., & Wennberg, A. C. (2025). CONTRAST prioritisation tool: filtering and ranking contaminants of emerging concern in the marine environment using hazard-based approaches. *Environmental Sciences Europe*, *37*(1), 203. https://doi.org/10.1186/s12302-025-01257-9

[64]	Hollender, J.; Schymanski, E. L.; Ahrens, L.; Alygizakis, N.; Béen, F.; Bijlsma, L.; Brunner, A. M.; Celma, A.; Fildier, A.; Fu, Q.; et al. NORMAN Guidance on Suspect and Non-Target Screening in Environmental Monitoring. Environ Sci Eur, 2023, 35 (1), 75. https://doi.org/10.1186/s12302-023-00779-4.

[65]	Hollender, J.; van Bavel, B.; Dulio, V.; Farmen, E.; Furtmann, K.; Koschorreck, J.; Kunkel, U.; Krauss, M.; Munthe, J.; Schlabach, M.; et al. High Resolution Mass Spectrometry-Based Non-Target Screening Can Support Regulatory Environmental Monitoring and Chemicals Management. Environ Sci Eur, 2019, 31 (1), 42. https://doi.org/10.1186/s12302-019-0225-x.

[66]	Zhang, X.; Xiong, S.; Sathiyaseelan, A.; Zhang, L.; Lu, Y.; Chen, Y.; Jin, T.; Wang, M.-H. Recent Advances in Photocatalytic Nanomaterials for Environmental Remediation: Strategies, Mechanisms, and Future Directions. Chemosphere, 2024, 364, 143142. https://doi.org/10.1016/j.chemosphere.2024.143142.

[67]	Pereira, A.; de Morais, E. G.; Silva, L.; Pena, A.; Freitas, A.; Teixeira, M. R.; Varela, J.; Barreira, L. Pharmaceuticals Removal from Wastewater with Microalgae: A Pilot Study. Applied Sciences, 2023, 13 (11), 6414. https://doi.org/10.3390/app13116414.

[68]	Karki, B. K. Amended Biochar in Constructed Wetlands: Roles, Challenges, and Future Directions Removing Pharmaceuticals and Personal Care Products. Heliyon, 2024, 10 (21), e39848. https://doi.org/10.1016/j.heliyon.2024.e39848.

[69]	Babu Ponnusami, A.; Sinha, S.; Ashokan, H.; V Paul, M.; Hariharan, S. P.; Arun, J.; Gopinath, K. P.; Hoang Le, Q.; Pugazhendhi, A. Advanced Oxidation Process (AOP) Combined Biological Process for Wastewater Treatment: A Review on Advancements, Feasibility and Practicability of Combined Techniques. Environ Res, 2023, 237, 116944. https://doi.org/10.1016/j.envres.2023.116944.

[70]	Constantin, M. A.; Constantin, L. A.; Ionescu, I. A.; Nicolescu, C. M.; Bumbac, M.; Tiron, O. Performance of a Solar-Driven Photocatalytic Membrane Reactor for Municipal Wastewater Treatment. Processes, 2024, 12 (3), 617. https://doi.org/10.3390/pr12030617.