Growth Performance and Water Quality Dynamics of Common Carp (Cyprinus carpio) Under a fabricated Three-Tier Perforated Sheet Aerator System

ABSTRACT Aims: This study was designed to explore how a newly fabricated three-tier perforated sheet aerator could enhance the growth of Common Carp (Cyprinus carpio) and maintain better water quality during culture. Study Design: A comparative experimental approach was followed to evaluate differences between aerated culture systems and traditional non-aerated systems. Place and Duration of Study: The work was carried out at the Research and Instructional Fish Farm, College of Fisheries, Mangaluru, using six cement ponds over a complete culture period. Methodology: A total of 300 fingerlings were stocked uniformly across six ponds (50 fish per pond). The aerator was operated for 3 hours daily to increase oxygen exposure during water circulation. Fish growth was monitored every two weeks by measuring length and weight, while major water quality parameters such as dissolved oxygen, pH, temperature, nitrogenous compounds, alkalinity, and hardness were evaluated following standard procedures. Data were statistically analyzed using an independent t-test at p < 0.05. Growth Performance and Water Qu… Results: Fish in aerated ponds showed noticeably better performance with higher weight gain (61.77 ± 0.54 g), faster growth rates (SGR: 1.94 ± 0.03%/day; ADG: 0.46 ± 0.02 g/day), improved feed efficiency (FCR: 2.02), and slightly higher survival (87.50%) compared to the control group. Dissolved oxygen increased significantly under aeration (8.74 ± 1.06 mg/L; p = 0.000), along with improvements in nitrite, nitrate, and alkalinity levels. Conclusion: The results clearly indicate that this aerator model can strengthen fish growth and overall pond health, offering a simple and economical tool for sustainable aquaculture development.

Keywords: Common carp; Perforated sheet aerator; Growth performance; Water quality; Dissolved oxygen; Sustainable aquaculture

1. INTRODUCTION

Aquaculture has emerged as the fastest-growing food production sector worldwide, contributing significantly to food and nutritional security. In 2022, global aquaculture production reached 130.9 million tonnes valued at USD 313 billion, with aquatic animals accounting for 94.4 million tonnes of this output (FAO, 2024). As the global population is projected to reach 9.73 billion by 2050 (Anon, 2019), the demand for sustainable protein sources continues to rise. Notably, aquaculture provides nearly 20% of the animal protein consumed by 3.2 billion people (FAO, 2018). India plays a vital role in this sector, contributing 7.1% to global aquaculture production, where freshwater carp culture remains the primary driver of inland fisheries development (Ayyappan and Diwan, 2006).

However, the rapid intensification of aquaculture brings challenges, including increased disease vulnerability, escalating production costs, and deteriorating water quality. Among the various environmental parameters, dissolved oxygen (DO) stands out as the most crucial factor influencing fish health, survival, and growth efficiency (Boyd, 1998; Kemker, 2013). Oxygen depletion causes severe stress, reduced feeding, lowered immunity, and potential mortality (Roy et al., 2021), while supersaturated oxygen conditions may induce gas bubble disease (Kemker, 2013). DO dynamics are further regulated by environmental factors such as temperature, salinity, and altitude (Khani and Rajaee, 2017; Moon et al., 2003; Zaker, 2007; Paz et al., 2020).

Artificial aeration is widely adopted to maintain optimum DO levels in intensive aquaculture systems. Conventional aerators including paddle wheels and diffusers are effective but require high energy input and add substantial production cost (Boyd, 1998). The perforated tray aerator has gained recognition as a cost-efficient and eco-friendly alternative by enhancing atmospheric oxygen absorption through gravity-driven droplet formation across multiple layers (Eltawil and ElSbaay, 2016; Boyd and Watten, 1989). While moderate aeration improves productivity and economic returns (Avinash et al., 2013), aeration alone can still contribute over 15% of total operational costs in fish farms (Kumar et al., 2013).

Therefore, the present study focuses on the development and evaluation of a fabricated three-tier perforated sheet aerator designed to enhance oxygen transfer efficiency and improve culture conditions. The study assesses its effects on the growth performance and water quality dynamics of Common Carp (Cyprinus carpio), aiming to contribute toward more sustainable and economically viable aquaculture technologies.

2. MATERIALS AND METHODS

2.1 EXPERIMENTAL SETUP

The study was conducted at the Research and Instructional Fish Farm of the College of Fisheries, Mangaluru. A custom-built three-tier horizontal perforated sheet aerator model was developed using galvanized iron perforated sheets measuring 85 × 85 × 5 cm. The aerator featured three vertical tiers mounted on a wooden stand and was powered by a one-horsepower pump. The experimental design utilized six uniform-sized cement ponds divided into two groups: three aerated ponds (T1a, T1b, T1c) equipped with the aerator system and three non-aerated control ponds (T2a, T2b, T2c) (Fig. 1). Aeration was applied for three hours daily; water was pumped through the perforated trays (Fig. 2), breaking into fine particles to enhance oxygen transfer by contacting atmospheric air before entering the pond water.

Fig.1 Fabricated Three-tier perforated sheet Aerator model used in the study

Fig.2 Experimental design with Aerator model

2.2 FISH STOCKING AND CULTURE

Common Carp (Cyprinus carpio) fingerlings of uniform size were stocked at a density of 10,000 fingerlings per hectare, 50 fingerlings per pond (total 300 fish). The ponds were prepared following standard aquaculture protocols. Throughout the culture period, fish were fed a formulated diet containing tapioca, fish meal, rice bran, and groundnut oil cake.

2.3 WATER QUALITY ANALYZES

Water samples were collected fortnightly early in the morning using polythene containers. Parameters measured included temperature, pH, dissolved oxygen, ammonia, nitrite, nitrate, alkalinity, and hardness. Dissolved oxygen was determined in the field using the Winkler method with oxygen fixing bottles (BOD bottles). Water temperature was recorded using a mercury-in-glass thermometer, and pH was measured via digital pH meter (WTW pH 320). Ammonia was assessed by the phenol-hypochlorite method, while nitrite and nitrate levels were estimated as per standard procedures. Alkalinity was evaluated by titrimetric method following APHA (1998) protocols.

2.4 GROWTH PERFORMANCE EVALUATION

Fish sampling occurred every 2 weeks, and total length and weight were recorded using a centimetre scale and an electronic balance, respectively. At least 50% of the stock was sampled cautiously and returned to the pond. Growth and survival indices were calculated as follows:

- Weight Gain (g) = Final weight - Initial weight

- Percentage Weight Gain (%) = [(Mean final weight – Mean initial weight) / Mean initial weight] × 100

- Average Daily Gain (g day−1) = (Final weight – Initial weight) / Culture period (days)

- Specific Growth Rate (%) = [(ln final weight – ln initial weight) / Number of days] × 100

- Survival Rate (%) = (Number of fish survived / Initial number stocked) × 100

- Feed Conversion Ratio (FCR) = Total feed given / Animal weight gain

3. STATISTICAL ANALYSIS

Data on growth performance and water quality parameters between aerated and non-aerated ponds were analyzed using the independent samples ‘t’ test for significance (p < 0.05) employing IBM SPSS version 26 software.

4. RESULTS AND DISCUSSION

This study focused on improving the growth of common carp by using a specially designed aerator model. Aeration is known to play an important role in supporting the growth and well-being of aquatic animals. The results showed that fish reared in tanks with the aerator grew better than those in tanks without it. Overall, the study suggests that proper aeration can be an effective way to boost the growth and survival of common carp.

4.1 GROWTH PARAMETERS

The growth performance of common carp was significantly better (p<0.05) in tanks equipped with the fabricated three-tier horizontal perforated sheet aerator compared to the control tanks without aeration. This improvement was evident through higher values of weight gain, percentage weight gain, specific growth rate, and a more efficient feed conversion ratio, all contributing to increased production. In this study, various growth parameters were analyzed, including feed conversion ratio (FCR), specific growth rate (SGR), average daily gain (ADG), percentage weight gain (%WG), weight gain (WG), and survival rate (%). The fish reared in tanks with the aerator model showed consistently better growth performance, as indicated by the mean ± standard error values presented in Table 1.

| Table 1: Growth performance of Common carp in tanks (without and with fabricated aerator) during the study |  |  |
| --- | --- | --- |
| Growth Parameter | Aerated Tanks | Non-aerated Tanks |
| FCR | 2.02 | 2.12 |
| SGR (%/day) | 1.94 ± 0.03 | 1.54 ± 0.02 |
| ADG (g/day) | 0.46 ± 0.02 | 0.30 ± 0.01 |
| Survival Rate (%) | 87.50 | 82.00 |
| Weight Gain | 61.77 ± 0.54 | 40.69 ± 0.28 |
| Length Gain | 8.81± 0.17 | 6.92± 0.07 |

The overall length gain in the aerator-equipped tanks was determined to be 8.81± 0.17%, whereas the length gain in the non-aerator tanks was 6.92± 0.07% (Fig.3). Comparably, for aerated tanks and non-aerated tanks, the percentage weight gain is 61.77 ± 0.54% and 40.69 ± 0.28%, respectively (Fig. 4).

The fishes in the tanks with and without an aerator had average daily gains of 0.46 ± 0.02and 0.30 ± 0.01, respectively (Fig. 5). The specific growth rate (SGR) was 1.94 ± 0.03and 1.54 ± 0.02 for tanks without aerators and with aerators, respectively (Fig. 6).

Magnoni et al. (2018), in their research, observed that a decrease in DO concentrations had significantly affected the feed intake of rainbow trout. The feed conversion ratio (FCR) is observed to be higher for the fish in tanks without an aerator (2.12) than that of the fish in tanks with an aerator (2.02) (Fig. 7). Fish consistently demonstrated good feed efficiency and low FCR when fed at the required DO in water (Duan et al., 2011, Sultana et al., 2017, and Sarmam et al., 2018). In addition, the survival rates were observed to be higher in the tanks with aerators, i.e., 87.50%, than in the control tanks, i.e., 82.00% (Fig. 8). The average survival rates in aerated and non-aerated tanks were 64% and 21% for rohu, mrigal, silver carp, and grass carp (Qayyum et al., 2005), and 92% and 40% for channel catfish (Hollerman and Boyd, 1980), respectively.

Fig.3 Comparison between the Average length gain of fishes in aerated and non-aerated tanks

Fig.4 Comparison between the Average weight gain of fishes in aerated and non-aerated tanks

Fig.5 Comparison between the average daily gain of fishes in aerated and non-aerated tanks

Fig.6 Comparison between the specific growth rate of fishes in aerated and non-aerated tanks

Fig.7 Comparison between the feed conversion ratio of fishes in aerated and non-aerated tanks

Fig.8 Comparison between the survival rate of fishes in aerated and non-aerated tanks

4.2 WATER QUALITY

| Treatment | Temp (°C) | pH | CO2 (mg/L) | DO (mg/L) | Ammonia (mg/l) | Nitrite (mg/l) | Nitrate (mg/l) | Alkalinity (mg/l) | Hardness (mg/l) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arated Tanks | 27.95 ± 1.49 | 7.65 ± 0.36 | 0.44 ± 0.57 | 8.74 ± 1.06 | 0.0099 ± 0.0062 | 0.0016 ± 0.0005 | 0.0053 ± 0.0006 | 87.89 ± 13.77 | 78.63 ± 16.19 |
| Non-Aerated tanks | 27.81 ± 0.85 | 7.89 ± 0.42 | 0.53 ± 0.79 | 6.66 ± 0.70 | 0.0159 ± 0.0094 | 0.0023 ± 0.0006 | 0.0075 ± 0.0020 | 71.44 ± 12.00 | 72.37 ± 10.56 |
| p - value | 0.810 | 0.212 | 0.786 | 0.000 | 0.132 | 0.016 | 0.011 | 0.015 | 0.348 |

The physico-chemical characteristics of the water, such as dissolved oxygen, pH, temperature, hardness, and turbidity, are all impacted by the aeration system in addition to the fish's growth. Moses (1992) made similar conclusions, stating that a variety of physical, chemical, and biological factors affect water quality, which in turn affects how soluble it is for fish and other aquatic animal production and dispersion. The water quality was assessed using a number of factors that affect fish health and are represented as the mean ± standard error (Table 2).

The recorded temperature values were 27.95 ± 1.49 °C in the tanks without an aerator and 27.81 ± 0.85 °C in those equipped with an aeration system. Aerators play a vital role in minimizing thermal stratification by enhancing the interface between air and water. This process facilitates better oxygen diffusion and promotes continuous water circulation, contributing to a stable aquatic environment (Boyd and Martinson, 1984). In the present study, CO2 values were 0.53 ± 0.79 tanks without aeration and 0.44 ± 0.57 those with aeration. Earlier studies by Boyd (1982) and Hollerman and Boyd (1980) also showed that aerated ponds tend to maintain lower CO₂ levels compared to stagnant ponds, where carbon dioxide accumulates more easily. The dissolved oxygen (DO) concentration was notably higher in aerated tanks (8.74±1.06 mg/L) compared to non-aerated tanks (6.66±0.70 mg/L). The three-tier horizontal perforated sheet aerator demonstrated superior efficiency in improving pond oxygenation relative to control systems (Mohan et al., 2022).

In the present study, pH values were 7.89±0.42 for tanks without aeration and 7.65±0.36for those with aeration. Generally, DO and temperature maintain an inverse relationship; as temperature increases, DO tends to decrease. The observed rise in DO, accompanied by a slight reduction in temperature and pH, may be explained by aeration-induced degasification processes (USEPA, 1998). Ammonia levels were 0.0159±0.0094mg/L in non-aerated tanks and 0.0099±0.0062mg/L in aerated tanks. Similarly, nitrite concentrations were 0.0023±0.0006mg/L and 0.0016±0.0005mg/L, while nitrate levels were 0.00533±0.0006mg/L and 0.0075±0.0020mg/L, in tanks without and with aeration, respectively. The total concentration of nitrogenous compounds namely ammonia, nitrite, and nitrate remained within acceptable limits, with aeration effectively reducing their accumulation.

Organic residues such as uneaten feed, manure, and metabolic waste can decompose incompletely, producing harmful byproducts that consume large amounts of oxygen. Excessive accumulation of toxic compounds like hydrogen sulphide, ammonia, and nitrite poses a serious risk to cultured fish (Chen et al., 2013). In the present study, alkalinity levels were 71.44±12.00mg/L in non-aerated tanks and 87.89±13.77mg/L in aerated tanks. Correspondingly, hardness levels were 72.37±10.56mg/L and 78.63±16.19mg/L, both within the recommended range. For freshwater systems, alkalinity between 5 and 500 mg/L is considered desirable (Boyd, 1982; Lawson, 1995). Additionally, an alkalinity level above 20 mg/L has been reported to enhance productivity in fertilized and tilapia culture ponds (Boyd, 1982; Lucas and Southgate, 2012).

Overall, the three-tier perforated sheet aerator proved highly effective in elevating dissolved oxygen concentrations a key determinant of water quality and fish growth in aquaculture systems. DO influences not only the physiological functioning of the cultured organisms but also various physicochemical and biological parameters of the water. By sustaining adequate oxygen levels and promoting stable environmental conditions, the aeration system supports optimal fish health, efficient nutrient cycling, and improved overall pond productivity (Fouda et al., 2023).

5. Conclusion:

The present study evaluated the effectiveness of a fabricated three-tier perforated sheet aerator in enhancing the growth performance of common carp (Cyprinus carpio) relative to non-aerated tanks, recognizing the central role of aeration in aquaculture productivity. The results indicated that the aerator model not only improved key physico-chemical properties of the water but also exerted a significant positive effect on fish growth and survival. Future investigations could compare the oxygen transfer efficiency of different aerator configurations to identify more effective designs. In addition, economic analyses that account for the costs of design, fabrication, operation, and maintenance are warranted to assess the practical applicability of this aerator model in commercial aquaculture.

References:

American Public Health Association & American Water Works Association. (1925). Standard methods of the examination of water and sewage (6th ed.)..

United Nations, Department of Economic and Social Affairs, Population Division. (2019). World Population Prospects 2019: Highlights. https://population.un.org/wpp/

Ayyappan, S., & Diwan, A. D. (2006). Fisheries research and development in India. Fishing Chimes, 26(1), 19-23. http://eprints.cmfri.org.in/id/eprint/7424

Boyd, C. E. (1998). Pond water aeration systems. Aquacultural Engineering, 18, 9–40. https://doi.org/10.1016/S0144-8609(98)00019-3

Boyd, C. E., & Martinson, D. J. (1984). Evaluation of propeller-aspirator-pump aerators. Aquaculture, 36(3), 283-292. https://doi.org/10.1016/0044-8486(84)90243-6

Boyd, C.E. and Watten, B.J. (1989), Aeration systems in aquaculture. Rev. Aquat. Sci., 1: 425 – 472.

Chen, X. J., Zhu, Y. G., & Zhao, Z. M. (2013). The Influence of Micropore Oxygen Aeration on the Pond Water Quality Environment. Advance Journal of Food Science and Technology, 5(11), 1478-1482. https://doi.org/10.19026/rjaset.5.3370

Duan, Y., Dong, X., Zhang, X., & Miao, Z. (2011). Effects of dissolved oxygen concentration and stocking density on the growth, energy budget and body composition of juvenile Japanese flounder, Paralichthys olivaceus (Temminck et Schlegel). Aquaculture Research, 42(3), 407-416. https://doi.org/10.1111/j.1365-2109.2010.02635.x

Eltawil, M. A., & ElSbaay, A. M. (2016). Utilisation of solar photovoltaic pumping for aeration systems in aquaculture ponds. International Journal of Sustainable Energy, 35(7), 629–644. https://doi.org/10.1080/14786451.2014.928295

FAO. (2018). The State of World Fisheries and Aquaculture 2018 - Meeting the sustainable development goals. Food and Agriculture Organization of the United Nations. https://www.fao.org/3/I9540EN/i9540en.pdf

FAO. (2024). The State of World Fisheries and Aquaculture 2024. Blue Transformation in action. Food and Agriculture Organization of the United Nations. https://doi.org/10.4060/cd0683en

Fouda, T., Elrayes, A. E., & Elhanafy, A. E. (2023). The Effect of Aeration Method on Nile Tilapia Growth, Water Quality Indicators and Environmental Impact. Scientific Papers Series Management, Economic Engineering in Agriculture & Rural Development, 23(1).

Hollerman, W. D., & Boyd, C. E. (1980). Nightly aeration to increase production of channel catfish. Transactions of the American Fisheries Society, 109(4), 446-452. https://doi.org/10.1577/1548-8659(1980)109<446:NATIPO>2.0.CO;2

Kemker, C. (2013). Dissolved Oxygen. In *Fundamentals of Environmental Measurements*. Fondriest Environmental, Inc. https://www.fondriest.com/environmental-measurements/parameters/water-quality/dissolved-oxygen/

Khani, S., & Rajaee, T. (2017). Modeling of dissolved oxygen concentration and its hysteresis behavior in rivers using wavelet transform‐based hybrid models. CLEAN–Soil, Air, Water, 45(2). https://doi.org/10.1002/clen.201500395

Kumar, A., Moulick, S., & Mal, B.C. (2013). Selection of aerators for intensive aquacultural pond. Aquacultural Engineering, 56, 71-78. https://doi.org/10.1016/j.aquaeng.2013.05.003 .

Lawson, T. B. (1995). Recirculating aquaculture systems. In Fundamentals of Aquacultural Engineering (pp. 192-247). Boston, MA: Springer US. https://doi.org/10.1007/978-1-4613-0479-1

Lucas, J. S., & Southgate, P. C. (2012). Reproduction, life cycles and growth. In Aquaculture: Farming Aquatic Animals and Plants, 2nd Edition (pp. 126-137). Wiley-Blackwell Publishing Inc. https://doi.org/10.1002/9781118687932

Magnoni, L. J., Eding, E., Leguen, I., Prunet, P., Geurden, I., Ozório, R. O., & Schrama, J. W. (2018). Hypoxia, but not an electrolyte-imbalanced diet, reduces feed intake, growth and oxygen consumption in rainbow trout (Oncorhynchus mykiss). Scientific Reports, 8(1), 4965. https://doi.org/10.1038/s41598-018-23352-z

Mohan, M., Prasad, G. S., Dash, R. K., Prakash, K. V., Mollah, N. M., Barik, P., & Varadaraju, S. (2022). Fabricated aerators influence on oxygen dissolution in pond water during day and nighttime. The Pharma Innovation. https://www.thepharmajournal.com/archives/2022/vol11issue10/PartSP/11-10-100-900.pdf

Moon, B. H., Seo, G. T., Lee, T. S., Kim, S. S., & Yoon, C. H. (2003). Effects of salt concentration on floc characteristics and pollutants removal efficiencies in treatment of seafood wastewater by SBR. Water Science and Technology, 47(1), 65–70. https://doi.org/10.2166/wst.2003.0017

Moses, B. S. (1992). Introduction to tropical fisheries (No. 1). Ibadan University Press.

Paz, M. V. G., Sánchez, J. F. M., Burggren, W., & Martínez, J. L. A. G. (2020). Metabolic rate and hypoxia tolerance in Girardinichthys multiradiatus (Pisces: Goodeidae), an endemic fish at high altitude in tropical Mexico. Comparative Biochemistry and Physiology Part A: Molecular and Integrative Physiology, 239, 110576. https://doi.org/10.1016/j.cbpa.2019.110576

Qayyum, A., Ayub, M., & Tabinda, A. B. (2005). Effect of aeration on water quality, fish growth and survival in aquaculture ponds. Pakistan Journal of Zoology, 37(1), 75-80. https://zsp.com.pk/pdf/75-80%20(20)%20P.J.Z.Vol.%2037(1)%202005.pdf

U.S. Environmental Protection Agency, Office of Research and Development, Center for Environmental Research Information, Risk Reduction Engineering Laboratory. (1989). Design Manual Fine Pore Aeration Systems. https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=200012210.PDF

Roy, S. M., Jayraj, P., Machavaram, R., Pareek, C. M., & Mal, B. C. (2021). Diversified aeration facilities for effective aquaculture systems—a comprehensive review. Aquaculture International, 29, 1181-1217. https://doi.org/10.1007/s10499-021-00685-7

Sarma, K., Rahman, A., & Dey, A. (2018). Impact of solar operated aerator on dissolved oxygen and fish growth. Journal of Experimental Zoology, India, 21(2), 1041–1046. https://www.connectjournals.com/jez

Zaker, N. H. (2007). Characteristics and Seasonal Variations of Dissolved Oxygen. International Journal of Environmental Research, 1(4), 296-301. https://doi.org/10.22059/IJER.2007.1.4.296