Evaluation of Excess Lifetime Cancer Risk Radiation from Background Ionizing Radiation in Eligbam and Government Residential Area (Gra Phase II)

.

ABSTRACT

A study of evaluation of excess lifetime cancer risk was carried out in Eligbam and government residential area phase II (GRA PHASE II) in Obio/Akpor local government area, the study was successfully caried out using the GMC-300 Giger Muller counter Radiation Detector to measure background radiation, The geographical coordinates of the various sample points were recorded using the geographical positioning systems (GPS).The measured radiation levels in Eligbam location ranged from 0.007 mR/hr to 0.016 mR/hr with an overall average of 0.012±0.06 mR/hr. The detected radiation levels in GRA phase II varied from 0. 011mR/hr to 0.034mR/hr with an overall mean of 0.0139±0.04mR/hr. Also, the computed absorbed dose from the background radiation levels in Eligbam location varied from 60.9nGy/hr to 139.2nGy/hr with an overall mean of 104.98±20.04nGy/hr. While the calculated absorbed dose from the measured radiation levels in GRA Phase II ranged from 95.7nGy/hr to147.9nGy/hr with an overall average of 120.64±36.96nGy/hr. The annual effective dose equivalent (AEDE) computed for Eligbam varied from 0.09mSv/yr to 0.21mSv/yr with an overall mean of 0.16±0.03mSv/yr. Similarly, the computed annual effective dose rate for GRA Phase II varied from 0.13mSv/yr to 0.45mSv/yr with an overall average of 0.18±0.06mSv/yr. The excess lifetime cancer risk calculated for Eligbam varied from 0.33×10-3 to 0.45×10-3 with an overall mean of 0.56 ×10-3 . The computed excess lifetime cancer risk for GRA Phase II ranged from 0.47×10-3 to 1.57×10-3 with an overall average of 0.66 ×10-3 which is quite higher than the world average. The range of values for the background radiation are all within the acceptable safe limit by the International Commission on radiological Protection (ICRP, 2000). However, the computed values for the absorbed dose, annual effective dose equivalent and the excess lifetime cancer risk are all higher than the ICRP safe limit of 89.0nGy/hr, 1.0mSV/yr and 0.29×10-3 respectively. Although, there is presently no reported health hazard associated to exposure due to ionizing radiation, but adequate caution need to be apply in case of impending danger.

Keywords: Background Ionizing Radiation, Radiation exposure, Excess Lifetime Cancer Risk, ICRP, UNSCEAR

1. INTRODUCTION

Background radiation measurement is essential for assessing its potential impact on human health, particularly in urban environments characterized by diverse geological and environmental conditions. Evaluating gamma radiation exposure is vital in determining its contribution to the global external dose from natural background radiation. Ionizing radiation, which possesses sufficient energy to ionize atoms and molecules, arises from both natural and anthropogenic sources. Gamma radiation, a form of ionizing radiation, is primarily emitted during the radioactive decay of unstable atomic nuclei. Prolonged exposure to gamma radiation has been linked to adverse health effects, including chromosomal damage, cancer induction, and disorders of the circulatory system (ICRP, 2007; UNSCEAR, 2021; Kareem et al, 2024).

There are two types of background radiation, the ionizing radiation and the non-ionizing radiation.The non-ionizing radiation are the type of radiations which do not possess enough energy to ionize living cell it comes in contact with or it is exposed to. While the ionizing radiation are those radiations which have enough energy to ionize any living cell that may be exposed to it. Radiations are usually emitted from different sources such as natural and Man-made sources.

The natural sources are the radiations from elements that have existed on the earth and in outer space. The Man-made sources are the radiations which results from human activities which contribute to the amount of radiation humans are exposed to on earth. They can be found in building materials, including decorative rocks, stone and bricks (Farai et al. 2000; ICRP,2000).

Heavy Radioactive elements such as uranium and thorium occur naturally in rocks in the ground. Uranium decays into other radioactive elements such as Thorium, Radon and its progenies, alpha-particles, Beta-particles etc.

These naturally occurring radiations can be detected   in various geological formations such as soils, rocks, plants, water and air (Jwanbot et al. 2014; Taskin et al, 2009). Higher amount of natural background radiations is mainly from rock disintegration through natural processes. They are transported to the soil by run-off by the rain (Taskin et al, 2009), making soil radioactivity one of the main determinants of natural background radiation.

Also, some human activities such as mining, milling, dredging greatly increase the background ionizing radiation in our environment (Ononugbo and Komolafe, 2016). Some of these radiations may be man-made such as medical imaging, X-ray diagnostic centres, radiotherapy equipment, security screening equipment and smoke detectors, nuclear weapons (Ononugbo and Komolafe, 2016).

Some of the sources which can make significant contribution to the background radiation involves radon gas, rocks, buildings, food, drinks and cosmic rays.

Figure 1: Map of the Study Area Showing the Locations

2. material and methods

The GQ Nuclear radiation detector (GMC-300) was used for the measurement of the background ionizing radiation. It is a hand-held meter, which was raised to height of about 1m above the ground level to measure the background Ionizing radiation (BIR) at the selected sample locations.  A total of 30 sampling points were taken; at each sampling point three distinct  readings was taken and the mean values were recorded. The measuring tape was used to measure the distances from one sample point to the other while the Global Positioning System (GPS) was used to record the geographical co-ordinates of the sample points. The following radiological health parameters were computed.

2.2.1 Average value of BIR and Absorbed Dose

The mean value was computed using the expression;

Mean value (µx)=inxin        …………………….. (1)

Where,

n = number of reading(s)

x = total number of readings (average)

1μR⁄ℎ = 8.7 nGy⁄ℎ =8.7 × 10 μGy/ (18760y)                 (2)

1μR⁄ℎ = 876.212 μGy/y

2.1.2 ANNUAL EFFECTIVE DOSE EQUIVALENT (AEDE)

Absorbed dose is a term used to describe the potential biochemical changes in specific tissues in the human body (Orlunta and Agbonifo, 2025). Dose conversion factor of 0.7 Sv/Gy as recommended by UNSCEAR for the conversion coefficient from the absorbed dose in air to the effective dose received by adults (Agbalagba, 2017) and an occupancy factor of 0.25 for outdoor exposure was used (Olanrewaju and Avwiri, 2017). The annual effective dose equivalent was estimated using Equation 3.

AEDE (outdoor)(mSv/y ) = Absorbed dose(nGy/h) × 8760 × 0.7Sv/Gy × 0.25          (3)

    - EXCESS LIFETIME CANCER RISK (ELCR)

The excess lifetime cancer risks (ELCR) describes the potential Carcinogenic effects, calculated based on the probability of cancer-induced incidence in a population. This is a result of exposure to radiation or the intake of harmful chemical substances for a lifetime. In other words, the ELCR indicates the chances of contracting a cancer from the exposure from radiation or toxic chemical substances for a specific lifetime. According to (ICRP) the excess lifetime is calculated from the equation

ELCR= AEDE× (DL) × (RF)                                                       (4)

Where,

AEDE =Annual effective dose equivalent

DL = Average Duration of Life (70 years)

RF = Risk Factor 0.05×10ˉ⁵ per Sievert (Orlunta and Sokari, 2023)

  - Statistical Analysis

The data were analysed with Ms-Word 2019, for plotting of bar charts, SPSS-20, for computing mean and standard deviation

3. results and discussion

The measured background radiation levels and their corresponding geographical coordinates are presented in tables 1.0 and 2.0 for Eligbam and GRA Phase II respectively. Also, the average radiation levels and their corresponding health risk parameters are presented in Tables 3.0 and 4.0 for both sample locations respectively. The respective overall averages of the background radiation levels for both Eligbam and GRA Phase II with their respective overall averages of the health risk parameters, and their corresponding safe limits are presented in table 5.0. The contour and the surface maps for Eligbam are presented in Figures 2.0 and 3.0. The contour and surface maps of GRA Phase II location are presented in figures 4.0 and 5.0 respectively. The comparism of the exposure rate with world average is presented in Figures 6.0 and 7.0 for Eligbam and GRA Phase II respectively. Also, the comparism of the annual effective dose equivalent with the world average is presented in figure 8.

Table 1.0 In situ measurements of exposure rate of Eligbam, in Obio/Akpor LGA

| S/N |  | Geographical Coordinate |  |  |  |  | 1st Reading (mR/hr) |  | 2nd Reading (mR/hr) |  | 3rd Reading (mR/hr) |  | Average Reading |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CODE |  | Latitude |  |  | Longitude |  |  |  |  |  |  |  | (mR/hr) |  |
| ELB-1 | 4049’59.406" |  |  | 6059’44.952" |  |  |  | 0.014 |  | 0.016 |  | 0.011 |  | 0.014 |  |
| ELB-2 | 4049’55.188" |  |  | 6059’41.669" |  |  |  | 0.010 |  | 0.014 |  | 0.012 |  | 0.012 |  |
| ELB-3 | 4049’51.202" |  |  | 6059’40.266" |  |  |  | 0.0014 |  | 0.008 |  | 0.012 |  | 0.007 |  |
| ELB-4 | 4049’43.618" |  |  | 6059’35.673" |  |  |  | 0.013 |  | 0.014 |  | 0.010 |  | 0.012 |  |
| ELB-5 | 4049’46.899" |  |  | 6059’24.999" |  |  |  | 0.010 |  | 0.016 |  | 0.013 |  | 0.013 |  |
| ELB-6 | 4049’41.942" |  |  | 6059’26.152" |  |  |  | 0.009 |  | 0.010 |  | 0.012 |  | 0.010 |  |
| ELB-7 | 4049’34.213" |  |  | 6059’25. 632" |  |  |  | 0.017 |  | 0.013 |  | 0.011 |  | 0.014 |  |
| ELB-8 | 4049’30.472" |  |  | 6059’25.581" |  |  |  | 0.012 |  | 0.013 |  | 0.010 |  | 0.012 |  |
| ELB-9 | 4049’30.136" |  |  | 6059’33.731" |  |  |  | 0.017 |  | 0.014 |  | 0.007 |  | 0.013 |  |
| ELB-10 | 4049’29.924" |  |  | 6059’45.169" |  |  |  | 0.016 |  | 0.012 |  | 0.021 |  | 0.016 |  |
| ELB-11 | 4049’26.605" |  |  | 6059’41177" |  |  |  | 0.018 |  | 0.018 |  | 0.011 |  | 0.016 |  |
| ELB-12 | 4049’18.899" |  |  | 6059’41.704" |  |  |  | 0.014 |  | 0.014 |  | 0.015 |  | 0.014 |  |
| ELB-13 | 4049’17.693" |  |  | 6059’38.135" |  |  |  | 0.016 |  | 0.015 |  | 0.010 |  | 0.014 |  |
| ELB-14 | 4049’13.166" |  |  | 6059’39.901" |  |  |  | 0.011 |  | 0.011 |  | 0.012 |  | 0.011 |  |
| ELB-15 | 4049’6.784" |  |  | 6059’41.868" |  |  |  | 0.012 |  | 0.010 |  | 0.010 |  | 0.011 |  |
| ELB-16 | 4049’11.948" |  |  | 6059’42.209" |  |  |  | 0.007 |  | 0.010 |  | 0.011 |  | 0.009 |  |
| ELB-17 | 4048’58.289" |  |  | 6059’42.306" |  |  |  | 0.009 |  | 0.013 |  | 0.009 |  | 0.010 |  |
| ELB-18 | 4048’47.293" |  |  | 6059’42.813" |  |  |  | 0.010 |  | 0.012 |  | 0.013 |  | 0.012 |  |
| ELB-19 | 4048’47.293" |  |  | 6059’43.638" |  |  |  | 0.013 |  | 0.010 |  | 0.009 |  | 0.011 |  |
| ELB-20 | 4048’48.903" |  |  | 6059’58.483" |  |  |  | 0.011 |  | 0.010 |  | 0.010 |  | 0.010 |  |
| ELB-21 | 4048’52.413" |  |  | 6059’47.938" |  |  |  | 0.014 |  | 0.013 |  | 0.010 |  | 0.012 |  |
| ELB-22 | 4049’1.723" |  |  | 6059’47.295" |  |  |  | 0.014 |  | 0.010 |  | 0.016 |  | 0.013 |  |
| ELB-23 | 4049’8.879" |  |  | 6059’45.777" |  |  |  | 0.010 |  | 0.009 |  | 0.014 |  | 0.011 |  |
| ELB-24 | 4049’8.879" |  |  | 6059’43.039" |  |  |  | 0.008 |  | 0.011 |  | 0.012 |  | 0.010 |  |
| ELB-25 | 4049’14.160" |  |  | 6059’43.399" |  |  |  | 0.010 |  | 0.008 |  | 0.007 |  | 0.008 |  |
| ELB-26 | 4049’40.220" |  |  | 6059’47.078" |  |  |  | 0.012 |  | 0.007 |  | 0.010 |  | 0.010 |  |
| ELB-27 | 4049’44.843" |  |  | 6059’50.328" |  |  |  | 0.013 |  | 0.020 |  | 0.010 |  | 0.014 |  |
| ELB-28 | 4049’46.517" |  |  | 6059’49.563" |  |  |  | 0.016 |  | 0.018 |  | 0.011 |  | 0.015 |  |
| ELB-29 | 4049’51.630" |  |  | 6059’45.701" |  |  |  | 0.017 |  | 0.020 |  | 0.012 |  | 0.016 |  |
| ELB-30 | 4047’53.209" |  |  | 6059’46.651" |  |  |  | 0.014 |  | 0.008 |  | 0.014 |  | 0.012 |  |
|  |  |  |  |  |  | Average BIR 0.0121±0.002 |  |  |  |  |  |  |  |  |

Figure 2: Contour map of Background ionization radiation in Eligbam

Figure 3: Surface map of Background ionization radiation in Eligbam

Table 2: In situ measurements of exposure rate of GRA Phase II in Obio/Akpor LGA

| S/N |  | Geographical Coordinate |  |  |  |  | 1st Reading (mR/hr) |  | 2nd Reading (mR/hr) |  | 3rd Reading (mR/hr) |  | Average Reading |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CODE |  | Latitude |  |  | Longitude |  |  |  |  |  |  |  | (mR/hr) |  |
| GRA-1 | 4°49'44.94" |  |  | 6°59'21.66" |  |  |  | 0.011 |  | 0.015 |  | 0.013 |  | 0.013 |  |
| GRA-2 | 4°49'39.72" |  |  | 6°59'23.86" |  |  |  | 0.011 |  | 0.012 |  | 0.011 |  | 0.011 |  |
| GRA-3 | 4°49'43.90" |  |  | 6°59'17.28" |  |  |  | 0.010 |  | 0.010 |  | 0.017 |  | 0.012 |  |
| GRA-4 | 4°49'39.11" |  |  | 6°59'21.02" |  |  |  | 0.010 |  | 0.009 |  | 0.011 |  | 0.010 |  |
| GRA-5 | 4°49'46.54" |  |  | 6°59'13.54" |  |  |  | 0.012 |  | 0.011 |  | 0.011 |  | 0.011 |  |
| GRA-6 | 4°49'34.88" |  |  | 6°59'26.27" |  |  |  | 0.012 |  | 0.010 |  | 0.013 |  | 0.012 |  |
| GRA-7 | 4°49'39.76" |  |  | 6°59'15.55" |  |  |  | 0.014 |  | 0.008 |  | 0.013 |  | 0.012 |  |
| GRA-8 | 4°49'41.26" |  |  | 6°59'11.54" |  |  |  | 0.012 |  | 0.013 |  | 0.014 |  | 0.013 |  |
| GRA-9 | 4°49'40.19" |  |  | 6°59'9.450" |  |  |  | 0.010 |  | 0.010 |  | 0.010 |  | 0.010 |  |
| GRA-10 | 4°49'34.69" |  |  | 6°59'20.95" |  |  |  | 0.011 |  | 0.014 |  | 0.017 |  | 0.014 |  |
| GRA-11 | 4°49'31.81" |  |  | 6°59'12.32" |  |  |  | 0.014 |  | 0.012 |  | 0.017 |  | 0.014 |  |
| GRA-12 | 4°49'38.20" |  |  | 6°59'9.090" |  |  |  | 0.014 |  | 0.014 |  | 0.017 |  | 0.015 |  |
| GRA-13 | 4°49'32.43" |  |  | 6°59'20.16" |  |  |  | 0.013 |  | 0.016 |  | 0.017 |  | 0.015 |  |
| GRA-14 | 4°49'38.36" |  |  | 6°59'24.58" |  |  |  | 0.016 |  | 0.009 |  | 0.017 |  | 0.014 |  |
| GRA-15 | 4°49'37.48" |  |  | 6°59'18.61" |  |  |  | 0.008 |  | 0.010 |  | 0.012 |  | 0.010 |  |
| GRA-16 | 4°49'41.57" |  |  | 6°59'21.39" |  |  |  | 0.014 |  | 0.080 |  | 0.009 |  | 0.034 |  |
| GRA-17 | 4°49'42.50" |  |  | 6°59'15.67" |  |  |  | 0.011 |  | 0.014 |  | 0.012 |  | 0.012 |  |
| GRA-18 | 4°49'32.83" |  |  | 6°59'24.45" |  |  |  | 0.011 |  | 0.012 |  | 0.012 |  | 0.012 |  |
| GRA-19 | 4°49'42.98" |  |  | 6°59'24.00" |  |  |  | 0.012 |  | 0.018 |  | 0.012 |  | 0.014 |  |
| GRA-20 | 4°49'43.95" |  |  | 6°59'9.330" |  |  |  | 0.015 |  | 0.018 |  | 0.016 |  | 0.016 |  |
| GRA-21 | 4°49'45.17" |  |  | 6°59'14.14" |  |  |  | 0.021 |  | 0.012 |  | 0.017 |  | 0.017 |  |
| GRA-22 | 4°49'43.88" |  |  | 6°59'21.71" |  |  |  | 0.021 |  | 0.014 |  | 0.013 |  | 0.016 |  |
| GRA-23 | 4°49'35.13" |  |  | 6°59'18.81" |  |  |  | 0.015 |  | 0.013 |  | 0.011 |  | 0.013 |  |
| GRA-24 | 4°49'36.55" |  |  | 6°59'24.55" |  |  |  | 0.015 |  | 0.017 |  | 0.011 |  | 0.014 |  |
| GRA-25 | 4°49'41.13" |  |  | 6°59'24.81" |  |  |  | 0.010 |  | 0.014 |  | 0.018 |  | 0.014 |  |
| GRA-26 | 4°49'35.28" |  |  | 6°59'23.04" |  |  |  | 0.017 |  | 0.018 |  | 0.013 |  | 0.016 |  |
| GRA-27 | 4°49'32.89" |  |  | 6°59'17.11" |  |  |  | 0.017 |  | 0.012 |  | 0.013 |  | 0.013 |  |
| GRA-28 | 4°49'31.77" |  |  | 6°59'18.54" |  |  |  | 0.014 |  | 0.016 |  | 0.015 |  | 0.015 |  |
| GRA-29 | 4°49'32.51" |  |  | 6°59'23.19" |  |  |  | 0.010 |  | 0.010 |  | 0.014 |  | 0.011 |  |
| GRA-30 | 4°49'41.37" |  |  | 6°59'18.65" |  |  |  | 0.013 |  | 0.010 |  | 0.015 |  | 0.013 |  |
|  |  |  |  |  |  | Average BIR 0.0139±0.004 |  |  |  |  |  |  |  |  |

Figure 4: Contour map of Background ionization radiation in GRA Phase II

Figure 5: Surface map of Background ionization radiation in GRA Phase II

Table 3: Radiological Parameters of Eligbam, in Obio/Akpor LGA.

| S/N CODE | Average Radiation level (mR/hr) | Absorbed dose (nGy/hr) | AEDE (mSv/y) | ELCR x 10¯³ |
| --- | --- | --- | --- | --- |
| ELB-1 | 0.014 | 121.8 | 0.19 | 0.65 |
| ELB-2 | 0.012 | 104.4 | 0.16 | 0.56 |
| ELB-3 | 0.007 | 60.9 | 0.09 | 0.33 |
| ELB-4 | 0.012 | 104.4 | 0.16 | 0.56 |
| ELB-5 | 0.013 | 113.1 | 0.17 | 0.61 |
| ELB-6 | 0.010 | 87.0 | 0.13 | 0.47 |
| ELB-7 | 0.014 | 121.8 | 0.19 | 0.65 |
| ELB-8 | 0.012 | 104.4 | 0.16 | 0.56 |
| ELB-9 | 0.013 | 113.1 | 0.17 | 0.61 |
| ELB-10 | 0.016 | 139.2 | 0.21 | 0.75 |
| ELB-11 | 0.016 | 139.2 | 0.21 | 0.75 |
| ELB-12 | 0.014 | 121.8 | 0.19 | 0.65 |
| ELB-13 | 0.014 | 121.8 | 0.19 | 0.65 |
| ELB-14 | 0.011 | 95.7 | 0.15 | 0.51 |
| ELB-15 | 0.011 | 95.7 | 0.15 | 0.51 |
| ELB-16 | 0.009 | 78.3 | 0.12 | 0.42 |
| ELB-17 | 0.010 | 87.0 | 0.13 | 0.47 |
| ELB-18 | 0.012 | 104.4 | 0.16 | 0.56 |
| ELB-19 | 0.011 | 95.7 | 0.15 | 0.51 |
| ELB-20 | 0.010 | 87.0 | 0.13 | 0.47 |
| ELB-21 | 0.012 | 104.4 | 0.16 | 0.56 |
| ELB-22 | 0.013 | 113.1 | 0.17 | 0.61 |
| ELB-23 | 0.011 | 95.7 | 0.15 | 0.51 |
| ELB-24 | 0.010 | 87.0 | 0.13 | 0.47 |
| ELB-25 | 0.008 | 69.6 | 0.11 | 0.37 |
| ELB-26 | 0.010 | 87.0 | 0.13 | 0.47 |
| ELB-27 | 0.014 | 121.8 | 0.19 | 0.65 |
| ELB-28 | 0.015 | 130.5 | 0.20 | 0.70 |
| ELB-29 | 0.016 | 139.2 | 0.21 | 0.75 |
| ELB-30 | 0.012 | 104.4 | 0.16 | 0.56 |
| Mean | 0.0121±0.0062 | 104.98±20.04 | 0.16±0.03 | 0.56±0.11 |
| Acceptable limits | 0.013 | 59.0 | 1.0 | 0.29 |

Table 4: Radiological Parameters of  GRA PhaseII , in Obio/Akpor LGA.

| S/N CODE | Average Radiation level (mR/hr) | Absorbed dose (nGy/hr) | AEDE (mSv/y) | ELCR x 10¯³ |
| --- | --- | --- | --- | --- |
| GRA-1 | 0.013 | 113.1 | 0.17 | 0.61 |
| GRA-2 | 0.011 | 95.7 | 0.15 | 0.51 |
| GRA-3 | 0.012 | 104.4 | 0.16 | 0.56 |
| GRA-4 | 0.010 | 87.0 | 0.13 | 0.47 |
| GRA-5 | 0.011 | 95.7 | 0.15 | 0.51 |
| GRA-6 | 0.012 | 104.4 | 0.16 | 0.56 |
| GRA-7 | 0.012 | 104.4 | 0.16 | 0.56 |
| GRA-8 | 0.013 | 113.1 | 0.17 | 0.61 |
| GRA-9 | 0.010 | 87.0 | 0.13 | 0.47 |
| GRA-10 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-11 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-12 | 0.015 | 130.5 | 0.20 | 0.70 |
| GRA-13 | 0.015 | 130.5 | 0.20 | 0.70 |
| GRA-14 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-15 | 0.010 | 87.0 | 0.13 | 0.47 |
| GRA-16 | 0.034 | 295.8 | 0.45 | 1.59 |
| GRA-17 | 0.012 | 104.4 | 0.16 | 0.56 |
| GRA-18 | 0.012 | 104.4 | 0.16 | 0.56 |
| GRA-19 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-20 | 0.016 | 139.2 | 0.21 | 0.75 |
| GRA-21 | 0.017 | 147.9 | 0.23 | 0.79 |
| GRA-22 | 0.016 | 139.2 | 0.21 | 0.75 |
| GRA-23 | 0.013 | 113.1 | 0.17 | 0.61 |
| GRA-24 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-25 | 0.014 | 121.8 | 0.19 | 0.65 |
| GRA-26 | 0.016 | 139.2 | 0.21 | 0.75 |
| GRA-27 | 0.013 | 113.1 | 0.17 | 0.61 |
| GRA-28 | 0.015 | 130.5 | 0.20 | 0.70 |
| GRA-29 | 0.011 | 95.7 | 0.15 | 0.51 |
| GRA-30 | 0.013 | 113.1 | 0.17 | 0.61 |
| Mean | 0.0139±0.04 | 120.64±36.96 | 0.18±0.06 | 0.66±0.2 |

Table 5: Radiological Parameters of  Eligbam and GRA Phase II , in Obio/Akpor LGA

| S/N CODE | Average Radiation level (mR/hr) | Absorbed dose (nGy/hr) | AEDE (mSv/y) | ELCR x 10¯³ |
| --- | --- | --- | --- | --- |
| Eligbam | 0.0121±0.06 | 104.98±20.04 | 0.16±0.03 | 0.56±0.11 |
| GRA | 0.0139±0.04 | 120.64±36.96 | 0.18±0.06 | 0.66±0.2 |
| Acceptable limits | 0.013 | 89.0 | 1.0 | 0.29 |

Figure 6: Comparison of exposure rate with world average., UNSCEAR, 2000

Figure 7: Comparison of Absorbed Dose with World Average., UNSCEAR, 2000

Figure 8: Comparison of AEDE with world average., UNSCEAR, 2000

Figure 9: Comparison of ELCR with world average value., UNSCEAR, 2000

The contour map and surface map for Eligbam are plotted in Figure 2 and 3 respectively.

From Figure 2, the areas with deep orange colour on the contour map show places with higher radiation level, while the places with deep blue colour indicate areas of lower radiation levels. Also, from the surface map of Figure 3, the regions of higher elevation indicate areas of higher radiation level in Eligbam village.

Figure 4, is the contour map of G.R. A. Phase 1.2, the region light purple colour shows places of higher radiation levels, while the region of black colour on the contour map is the places of lower radiation level. Also, Figure 4, the surface map, the region of high elevation shows places higher radiation level while the region with no elevation show places of lower radiation level.

The contour map and surface map for G.R.A are plotted in Figure 3 and 4 respectively. The deep purple colour on the contour map represents the area with high radiation level, while the deep blue colour on the surface map represents the area with high radiation level.

Also, the values of Ilie background ionizing radiation exposure rate, annual absorbed dose rate, annual effective dose equivalent and excess lifetime cancer risk are plotted in the bar-chart of Figures 5, 6, 7 and 8 respectively. The result of the BIR in Eligbam village and GRA are presented in Tables 1 and 2. The result of the health effect parameters are presented in Tables 3 and 4.

A total of 30 sample points were measured in each of the locations, Eligbam and GRA Phase II. The results of the BIR for both locations varied from 0.008mR/hr - 0.016mR/h and 0.010mR/hr-0.034mR/hr respectively with an overall average of 0.0121 ± 0.002mRh for Eligbam village and 0.0139 ± 0.04mR/hr for GRA Phase II.

An earlier work by Shahbazi-Gahrouei (2003) on background ionizing radiation in Charmahal and Bakhtiari province, obtained an average radiation level of 28.4mR/hr which is higher than the results in this work. However. The results obtained in this work are all within the safe limit of 0.013mR/hr recommended by International Commission on Radiological Protection (ICRP, 2000).

The absorbed dose rate for both locations ranged from 69.6nGy/hr - 139.2 nGy/h and 87.0nGy/hr- 295.8nGy/h respectively with an overall average of 104.98 ± 20.04nGy/hr for Eligbam village and 120.64 ± 36.96nGy/h for GRA Phase II

An earlier work by Ahmet Bozkurt et al (2007) on measurements of outdoor gamma radiation in air in Sanliurfa province of South eastern Turkey, the result obtained was 60.9 nGy/h which is lower than the results obtained in this work.

The results of the absorbed dose obtained for this work are all higher than the safe limit of 89.0 nGy/hr recommended by the International Commission on Radiological Protection (ICRP, 2000).

The annual effective dose equivalent (AEDR) for both locations ranged from 0.11-0 21 mSv/y and 0.13-0.21 mSv/y respectively with an overall average of 0.16 ± 0.03mSv/yr for Eligbam and 0.18 ± 0.06mSv/yr for GRA Phase II.

A previous work by Faghihi et al. (2011) on comprehensive study of natural radioactivity in building materials used in Iran, show the result obtained to be 0.37 mSv/y which is higher than the results in this work. However, the results obtained in this work are all than the safe limit of 1.0 mSv/yr recommended by the international Commission on Radiological Protection (ICRP, 2000).

The excess lifetime cancer risk for both locations ranged from 0.33×10-3 -0.75×10-3 and 0.47×10-3- 0.79×10-3 respectively with an overall of 0.56 ± 0.11×10-3 for Eligbam and 0.66± 0.2×10-3  for GRA Phase II and are all higher than the safe limit of 0.2×10-3 recommended by (ICRP, 2000).

An earlier work by Ortunta and Sylvester (2023) on terrestrial radiation of 30 selected sample points within Port Harcourt showed the result obtained to be within the range of 1.54×10-3 which is higher than the ICRP recommended safe limit of 0.29×10-3. Also, another work by Ortunta and Sokari on the Evaluation of excess lifetime cancer risk in some selected diagnostics centres in Port Harcourt, the result ranged from 0.60×10-3 - 0.94×10-3. These results are also higher than the ICRP recommended of 0.29×10-3.

4. Conclusion

The result of the excess lifetime cancer risk of Eligbam and GRA Phase II, computed from background ionizing radiation were higher than the recommended safe limits of 0.29×10-3 of the International Commission on Radiological Protection. However, there is presently no immediate threat of any health hazard among the populace of the sample locations.

Recommendation:

It is recommended that routine monitoring of the sample locations should be carried out on quarterly basis in order to assess the prevailing average radiation exposure of the populace, in case of impending danger.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Ahmet B., Nuri Y., Erol K., Gursel K. & Ahmet E.O (2007): Assessment Of Environmental Radioactivity for Sanliurfa region of Southeastern Turkey Journal of radiation measurement 42 (8), 1387-1391.

Faghihi, R., Mehdizadeh, S., & Sina, S. (2011). Natural and artificial radioactivity distribution in soil of Fars Province, Iran. Radiation Protection Dosimetry, 145(1), 66–74.

Farai, I.P., & Jibri, N.N., (2000). Baseline Studies of Terrestrial Outdoor Gamma Dose Rate Levels in Nigeria. Radiation and Protection Dosimetry, 88(3), 121-123.

International Commission on Radiological Protection (ICRP). (2007). The 2007 Recommendations of the International Commission on Radiological Protection. ICRP Publication 103. Annals of the ICRP, 37,2-4.

International Commission on Radiological Protection (ICRP). (2000). Protection of the Public in Situations of Prolonged Radiation Exposure (ICRP Publication 82). Annals of the ICRP, 29(1-2).

Jwanbot, D.I., Izam, M. M., Nyarn, G.G., & Yusuf, M. (2014). Indoor and Outdoor Gamma Dose Rate Exposure Levels in Major Commercial Building Materials Distribution Outlets in Jos, Plateau State-Nigeria, Asian Review of Environmental and Earth Sciences, 1(l), 5-7.

Kareem, A. L., Fiza, A., Nasir, N.A., Yakubu, A. &Fatima, K. (2024), Assessment Dose Associated with Background radioactivity in a Quarry Soil at Dawakin-Kudu Local Government Area, Kano, Nigeria. Nigerian Journal of Physics, 33(1), 126-136.

Ononugbo, C. P. & Komolafe, E. (2016), Dosimetry of some Selected Industrial Sites at Onne Oil and Gas Free Trade Zone. Journal of Environmental and Engineering Technology, 4(1), 2-9.

Orlunta, A. N., & Agbonifo, E. M. (2025).  Evaluation of Background Ionizing Radiation in 20 Selected Locations within Rumuosi and Choba Villages, Port-Harcourt, Rivers State, Nigeria. Asian Journal of Advanced Research and Reports 19(3), 433-444

Orlunta, A.N. & Sokari, S.A (2023) evaluation of excess cancer risks in some selected x-ray diagnostics center in Port Harcourt Rivers State. Asian Journal of advanced research and report 17(4) 26-33.

Orlunta, A.N. & Sokari, S.A (2023) terrestrial radiation of 30 selected sample points within Port Harcourt. Asian Journal of research and review in physics. 7(1), 24-23.

Shahbazi-Gahrouei, D. (2003). Natural background radiation dosimetry in the highest altitude region of Iran. Journal of Radiation Research, 44(3), 285-287.

Taskin, H., Karavus, M., Ay, P., Topuzoglu, A., Hidiroglu, S., and Karahan, G. (2009). Radionuclide concentration in soil and lifetime cancer risk due to the gamma radioactivity in Kirklareli, Turkey. Journal of Environmental Radioactivity. 100,49-53.

United Nations Scientific Committee on the Effects of Atomic Radiation (UNSCEAR). (2021). Sources, Effects and Risks of Ionizing Radiation: UNSCEAR 2020/2021 Report to the General Assembly, with Scientific Annexes. Volume I. United Nations.