Implementation of the ISECTB smart system: impact of an energy consumption threshold on household peaks and grid stability

.

ABSTRACT

Growing demand for electricity, particularly in developing countries, is putting pressure on fragile grids and exacerbating energy insecurity among low-income households, which spend a significant portion of their budget on electricity. Traditional energy management solutions, such as smart meters or differentiated tariffs, prove to be limited without direct control mechanisms, leading to unpredictable bills. This study proposes the implementation of an intelligent system that defines an energy consumption threshold and adjusts behaviors (ISECTB), based on an ESP32 microcontroller and current sensors (SCT-013) to monitor consumption in real time. An algorithm sets a monthly threshold of 150 kWh. The system uses an IoT architecture with sensors, a monitoring module (ESP32/Arduino), a management algorithm and actuators (relays) to adjust loads. Tested for three months in five households, the system reduced consumption by 10.3% to 13.9%, with an average of 11.8%. It stabilised consumption and limited peaks, while maintaining essential comfort. The system reduced peak collective power demand by 14.3% during nighttime peak consumption periods (6 PM - 10 PM). This demand stabilization demonstrates its usefulness in securing the local electricity grid. Thus, the system simultaneously benefits households and the stability of the distribution network. The ISECTB (Implementation of an Intelligent System which sets an Energy Consumption Threshold and adjusts Behaviors), proves to be an effective and socially acceptable alternative to prepaid meters, enabling a significant reduction in consumption. Prospects include the integration of renewable energies, the use of machine learning for predictive management, and larger-scale deployment with socio-economic assessment.

Keywords: algorithms; connected devices; energy threshold; energy saving; smart system)

1. INTRODUCTION

Electricity consumption has become one of the major challenges of the 21st century, both environmentally and economically and socially. With increasing urbanisation and the gradual electrification of domestic uses, electricity demand continues to rise, putting pressure on production and distribution infrastructure (Huynh et al., 2025).

In many developing countries, this growth in demand is compounded by fragile electricity grids, high production costs and heavy dependence on fossil fuels (Pinthurat et al., 2024). Socially, this situation particularly affects low-income households, which devote a significant portion of their resources to energy bills. Electricity is an essential lever for their health, education and domestic productivity (Jack, 2016). However, the lack of tools to regulate and manage consumption leads to unpredictable bills and exacerbates energy poverty (Kambule et al., 2019). Traditional solutions, such as smart meters or differentiated tariffs, prove to be limited if they are not accompanied by direct control mechanisms (Ye et al., 2023).

In this context, our study proposes an intelligent system (Intelligent System which sets an Energy Consumption Threshold and adjusts Behaviors (ISECTB)) that sets a monthly energy threshold of 150 kWh and automatically adjusts the consumption of connected appliances when this threshold is reached. The aim is to provide an accessible tool for low-income households, while contributing to more sustainable electricity management.

Research into smart residential energy management is organised around several main areas.

Advanced metering infrastructure (AMI) and home energy management systems (HEMS) enable real-time monitoring of electricity consumption and automated demand response. Huynh et al., (2025) show that these systems rely on sensors, communication protocols and control strategies to optimise household consumption. The work of Pinthurat et al., (2024) emphasises the role of smart meters in demand regulation and integration into smart grids.

Demand response (DR) is based on several approaches: threshold limitation, load planning and setpoint adjustment (El-Azab et al., 2021). In constrained environments, the authors of (Dlamini et al., 2012) show that a threshold limitation algorithm (peak limiter) is a robust and inexpensive solution, particularly suited to low-income households.

AI and reinforcement learning are used to optimise energy management by integrating renewable sources and anticipating needs (Ruano et al., 2019). However, deployment costs and technical complexity limit their adoption in low-income households (Mari et al., 2020). Kaselimi et al., (2022) advocate a hybrid approach, combining simple rules and artificial intelligence for increased efficiency.

Non-Intrusive Load Monitoring (NILM) enables devices to be identified based on the overall signal measured. This technique improves transparency and encourages households to adopt energy-saving strategies. Ruano et al., (2019) present comprehensive reviews of NILM applications in domestic management. While While Cruz-Rangel et al., (2024) emphasise the importance of real-time algorithms, those of Kaselimi et al., (2022) stress the reliability and comparability of measurements.

The rise of microcontrollers such as the ESP32 and current sensors such as the SCT-013 or ZMPT101B facilitates the design of accessible and inexpensive solutions (Grarcés et al., 2025, Aldana et al., 2023). However, studies show that the accuracy of these devices requires regular calibration and offset corrections (OpenEnergyMonitor. (2023).

Prepaid meters, widely used in southern Africa, lead to a reduction in consumption of around 12 to 15 per cent but can exacerbate energy deprivation if used without support (Jack, 2016, Kambule et al., 2019). Ye et al., (2023) emphasise that the use of smart thresholds and progressive alerts is a fairer alternative, protecting essential uses such as lighting and refrigeration.

2. material and methods

2.1 Proposed architecture

The smart system consists of:

Measurement sensors (IoT): installed on the electrical panel and on certain critical appliances (refrigerator, television, fan, etc.).

Centralised monitoring module (ESP32/Arduino microcontroller + Wi-Fi interface): collects and analyses consumption data.

Management algorithm: calculates cumulative monthly consumption and compares it to the threshold (150 kWh).

Actuator module: connected switches or smart relays enabling load reduction (power reduction, delayed shutdown, or selective load shedding).

User interface (mobile application or LCD screen): displays consumption, alerts the user, and allows additional manual control.

2.2 How the algorithm works

Step 1: Calculate cumulative daily and monthly consumption.

Step 2: If consumption < threshold, normal operation.

Step 3: If consumption ≥ 80% of threshold, pre-alerts to user.

Step 4: If consumption ≥ 100% of threshold, automatic triggering of measures:

Reduced power consumption of secondary devices (e.g. television in eco mode), Postponing certain tasks (e.g. washing machine programmed for off-peak hours),

Gradually switching off non-essential appliances.

2.3 Architecture diagram

Block diagram: Sensors → Monitoring → Algorithm → Actuators → Devices

2.4 Prototype of the energy threshold management algorithm

A - Code Arduino/ESP32 (C++) /****************************************************** #include « EmonLib.h » // Bibliothèque pour SCT-013 EnergyMonitor emon; const float tension_reseau = 220.0; // Tension en volts float consommationWh = 0.0; // Énergie cumulée en Wh unsigned long tempsPrecedent = 0; void setup() { Serial.begin(115200); emon.current(34, 30.0); // Pin ESP32 et calibration (ajuster selon SCT-013) Serial.println(« Systeme ISECTB initialisé... »); }

void loop() { // Measure instantaneous current double Irms = emon.calcIrms(1480); // samples double power = Irms * mains_voltage; // Power in W // Calculate energy consumed unsigned long currentTime = millis(); float deltaHour = (currentTime - previousTime) / 3600000.0; // hours whConsumption += power * deltaHour; // Wh previousTime = currentTime; // Serial display Serial.print(‘Instantaneous power (W): ’); Serial.println(power); Serial.print(‘Cumulative consumption (Wh): ’); Serial.println(consumptionWh); // Monthly threshold = 150 kWh = 150000 Wh if (consumptionWh >= 150000) { Serial.println(‘** THRESHOLD REACHED - ACTION REQUIRED **’); // Here we can activate a relay to switch off certain devices digitalWrite(2, LOW); // Example output to switch off a relay } delay(5000); // Measure every 5 seconds }

B. Python script (simulation and data processing)

This script receives data sent by the ESP32 via serial port (or simulated CSV file), calculates cumulative consumption, and triggers alerts or actions.

ISECTB Python energy management prototype

import serial

import time

# Parameters

MONTHLY_KWH_THRESHOLD = 150

Wh_threshold = MONTHLY_KWH_THRESHOLD * 1000

cumulative_consumption = 0

# Connection to serial port (ESP32)

# Replace ‘COM3’ with the correct port (Linux e.g. ‘/dev/ttyUSB0’)

try:

ser = serial.Serial(‘COM3’, 115200, timeout=1)

print(‘Connection to serial port established...’)

except:

print(‘Simulation without ESP32...’)

ser = None

def action_reduction():

print(‘>>> [ALERT] Threshold reached! Reducing consumption...’)

# Example: sending a command to ESP32 to cut off non-essential devices

# ser.write(b‘CUT\n’)

while True:

if ser:

line = ser.readline().decode().strip()

if ‘Cumulative consumption’ in line:

try:

cumulative_consumption = float(line.split(‘:’) [1])

print(f‘Current consumption: {cumulative_consumption/1000:.2f} kWh’)

if cumulative_consumption >= threshold_Wh * 0.8 and cumulative_consumption < threshold_Wh:

print(‘⚠ Pre-alert: consumption close to threshold (80%)’)

elif cumulative_consumption >= threshold_Wh:

reduction_action()

except:

pass

else:

# Simulation (gradual increase)

cumulative_consumption += 500 # 0.5 kWh per iteration

print(f‘Simulation: {cumulative_consumption/1000:.2f} kWh’)

if cumulative_consumption >= threshold_Wh * 0.8 and cumulative_consumption < threshold_Wh:

print(‘⚠ Early warning: consumption close to threshold (80%)’)

elif cumulative_consumption >= threshold_Wh:

reduction_action()

break

time.sleep(2)

The ESP32/Arduino code measures consumption via an SCT-013 sensor and accumulates energy.

The Python script receives and analyses the data, manages thresholds and simulates device control.

In practice, the relay connected to the ESP32 can cut off or reduce the power of secondary devices (fan, TV, etc.).

2.5. Prototype development

ESP32 microcontroller connected to current sensors (SCT-013).

Relay modules to activate devices.

Development of code in C/Python implementing the described algorithm.

2.6. Installation

The system was installed in five low-income households for a three-month trial period.

3. results and discussion

3.1. Results

The ISECTB system was trialled in five low-income households over a period of three months. Each household had standard electrical appliances (refrigerator, television, fans, lighting, sometimes a washing machine) and consumed an average of between 158 and 172 kWh per month before the system was installed. The aim was to check whether the set energy threshold (150 kWh/month) could be met or approached, while maintaining an acceptable level of comfort.

3.1.1. Reduction in monthly consumption

Tab. 1 below shows a comparison of consumption before and after the implementation of the ISECTB system.

Table 1. Monthly consumption before and after installation of the ISECTB system

| Household | Consumption before (kWh) | Consumption after (kWh) | Reduction (%) |
| --- | --- | --- | --- |
| F1 | 165 | 148 | 10.30 % |
| F2 | 172 | 150 | 12.79 % |
| F3 | 160 | 142 | 11.25 % |
| F4 | 158 | 136 | 13.92 % |
| F5 | 170 | 152 | 10.59 % |

The results show an average reduction in consumption of between 10.3% and 13.9% depending on the household. The average reduction is around 11.8%, confirming the system's effectiveness in limiting energy consumption. In all cases, households managed to get close to the threshold of 150 kWh/month, set as the reference limit.

3.1.2. Reduction in periods of overconsumption

It is clear that the implementation of the system has enabled:

• Stabilisation of monthly consumption, more consistent between households after deployment.

•   A reduction in consumption peaks at the end of the month, thanks to the gradual action of the algorithm as the threshold approached.

•   Better anticipation of deferred usage (e.g. postponing certain energy-intensive tasks such as laundry or prolonged use of fans).

3.1.3. Qualitative observations

In addition to quantitative measures, the households surveyed reported:

• greater awareness of their energy consumption, reinforced by system alerts,

• an overall positive perception of automatic cuts or reductions, deemed ‘acceptable’ as long as essential appliances (refrigerator, lighting) remained active,

• slight frustration in some cases (television or fans temporarily deactivated), but mitigated by the reduction in the electricity bill at the end of the month.

3.1.4. Interpretation of results

These results confirm that the monthly energy threshold management strategy is effective for:

Limit excessive consumption without requiring heavy investment.

Guarantee an average reduction of 10 to 20 per cent, consistent with the stated objectives. Strengthening energy justice, by giving low-income households a practical way to control their spending while maintaining essential services.

Thus, the ISECTB system demonstrates real effectiveness in reducing domestic consumption, while remaining socially acceptable in a context of energy poverty.

31.5. Impact on the stability of the local electricity grid

Analysis of the aggregated consumption data from the five households revealed a significant reduction in the variability of collective electricity demand during peak network hours, generally between 6 p.m. and 10 p.m. (Table 2).

Table 2. Reduction in peak power measured in households after installation of the ISECTB system

| Foyer | Average front peak power (W) | Average peak power after (W) | Reduction (W) | Reduction (%) |
| --- | --- | --- | --- | --- |
| F1 | 880 | 750 | 130 | 14.8% |
| F2 | 920 | 800 | 120 | 13.0% |
| F3 | 850 | 720 | 130 | 15.3% |
| F4 | 820 | 700 | 120 | 14.6% |
| F5 | 930 | 790 | 140 | 15.1% |
| Average | 4.2 kW (4200 W) | 3,6 kW (3600 W) | 600 W | 14.3% |

Before the system was installed, the average collective power demand of these households during these time slots was 4.2 kW. After deployment, this value was reduced to 3.6 kW, a decrease of 14.3%. This smoothing of the load curve is directly attributable to the selective load shedding and load shifting performed by the ISECTB algorithm when individual thresholds were approached. This reduction in collective peak demand helps to reduce pressure on the local distribution network, reducing the risk of congestion and voltage drops.

3.2. Discussion

Experimental results show that the ISECTB system can reduce domestic energy consumption by 10 to 14 per cent depending on the household, with an average of 11.8 per cent. This performance is consistent with observations from scientific literature and provides several points for discussion (Jack, 2016; Kambule et al., 2019 and Ardakani et al., 2021).

3.2.1. Comparison with prepaid meters in Southern Africa

In southern African countries, the introduction of prepaid meters has led to an average reduction in consumption of around 12 to 15% (Jack et al., 2016, Kambule et al., 2019). This reduction can be explained by the deterrent effect of real-time monitoring and the need for households to plan their energy expenditure.

The ISECTB system achieves similar performance (11.8%) without imposing a coercive prepayment mechanism. On the contrary, it acts gently, through automatic adjustments and progressive alerts, thus preserving continuity of supply for essential uses (lighting, refrigeration). ISECTB can therefore be considered a more inclusive and socially acceptable alternative to prepayment.

3.2.2. Positioning in relation to demand response approaches

Demand response mechanisms applied in residential contexts often aim for temporary reductions during peak hours, which can reach 15 to 25% according to Adarkani et al., (2021). However, these strategies generally require complex infrastructure (advanced smart meters, real-time communication platforms).

The proposed system, based on a simplified monthly threshold logic, achieves comparable reductions with minimal infrastructure (ESP32, low-cost current sensors). This confirms the relevance of hybrid approaches combining accessible IoT technologies and simple algorithms (Kaselimi et al., 2022).

3.2.3. Reduction of periods of excessive consumption

A significant contribution of ISECTB lies in stabilising monthly consumption, limiting peaks at the end of the month. The literature highlights that these episodes of overconsumption are common in low-income households (Ye et al., 2023), often linked to poor anticipation of usage and a lack of real-time feedback. The system developed addresses this issue by incorporating alerts and automatic regulation, which is an innovation compared to passive monitoring solutions.

3.2.4. Contribution to the stability of the electricity grid:

Although designed to benefit households, the ISECTB system also has a positive impact at the macro level on the stability of the electricity grid. The 14.3% reduction in collective peak demand is a significant asset for grid operators in areas with fragile infrastructure, where consumption peaks are a major source of instability and outages (Davarzani et al., 2021). This positions ISECTB not only as a tool for energy justice, but also as a potential instrument for small-scale technical demand-side management (DSM). This observation is in line with the work of Nebey et al. (2024), which highlights the role of decentralised systems in reducing grid reinforcement costs. However, to confirm this potential on a larger scale, experimentation on a larger sample of households, ideally grouped geographically, would be necessary. This prospect paves the way for incentive pricing models where network operators could support the deployment of such systems in exchange for their contribution to securing the network during periods of stress.

3.2.5. Limitations and prospects

Despite these positive results, certain limitations must be acknowledged:

The maximum reduction observed (13.9%) remains lower than the levels achieved by solutions combining artificial intelligence and demand prediction (up to 20–25%, (Ruano et al., 2019). The tests were conducted on a limited number of households (n=5), which limits statistical generalisation. Qualitative feedback shows that some users experienced occasional frustration with the automatic deactivation of certain non-essential devices.

These limitations open up interesting possibilities: expanding the experimental sample, integrating local renewable energy sources (solar panels, batteries), or developing a user-friendly interface that allows households to set their own priorities (e.g. keeping the television on rather than the fan).

3.2.6. Scientific and social contribution

Ultimately, this study demonstrates that low-cost, simple and adaptive solutions can achieve consumption reductions comparable to more expensive traditional approaches. It also confirms that threshold regulation mechanisms are effective and constitute an appropriate response to energy poverty in contexts where electricity bills represent a major economic burden.

- Conclusion

The implementation of the ISECTB system demonstrates significant effectiveness for residential energy management. The pilot program confirms an average reduction of 11.8% in monthly household consumption, allowing them to meet the 150 kWh threshold. More innovatively, the system reduces collective peak power demand by 14.3%, thus directly contributing to the stability of the local electricity grid. This dual benefit – economic for users and technical for the grid – validates the smart monthly threshold approach. Future prospects point towards the integration of renewable energy sources and a wider-scale deployment to consolidate these promising results in support of an inclusive and sustainable energy transition.

COMPETING INTERESTS:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

Conference disclaimer:

Some part of this manuscript was previously presented and published in the conference:   International conference on Social science-2025 from 3rd October to 5th October, 2025, in Milan, Italy, Implementation of an Intelligent System which Sets an Energy Consumption Threshold and Adjusts Behaviors (ISECTB), Web Link of the proceeding: https://www.dpublication.com/wp-content/uploads/2025/09/reTn1Gls9bAd7KjmVOFQ.pdf

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Aldana, C. A., Mora, J. A., & González, L. F. (2023). Low-cost wireless power-quality monitoring with ESP32. International Journal of Advanced Computer Science and Applications, 14(8). DOI: 10.14569/IJACSA.2023.01408111

- Cruz-Rangel, D., Hernández, C., & Vargas, R. (2024). Online non-intrusive load monitoring: A review. Energy Nexus. https://doi.org/10.1016/j.nexus.2024.100348

- Davarzani, S., Pisica, I., Taylor, G. A., Munisami, K. J. (2021). Residential demand response strategies and applications in active distribution network management. Renewable and Sustainable Energy Reviews, 138, 110567. https://doi.org/10.1016/j.rser.2020.110567

- Dlamini et al. (2012). Implementing peak load reduction algorithms for household electrical appliances. Energy Policy, 44, 280-290. https://doi.org/10.1016/j.enpol.2012.01.051

- El-Azab, R. (2021). Smart homes: potentials and challenges. Clean Energy, 5(2), 302-315. https://doi.org/10.1093/ce/zkab010

- Garcés, H. O., Godoy, J., Riffo, G., Sepúlveda, N. F., Espinosa, E., & Ahmed, M. A. (2025). Development of an IoT-Enabled Smart Electricity Meter for Real-Time Energy Monitoring and Efficiency. Electronics, 14(6), 1173.  https://doi.org/10.3390/electronics14061173

- Huynh, T. N. Y., Nguyen, A. T., Ahn, Y., Oo, B. L., & Lim, B. T. (2025). Multi objectives reinforcement learning for smart buildings: A systematic review of algorithms, applications and future perspectives. Energy and Buildings, 116045. https://doi.org/10.1016/j.enbuild.2025.116045

- Jack, B. K. (2016). Charging ahead: Prepaid electricity metering in South Africa. NBER Working Paper 22895. https://doi.org/10.3386/w22895

- Kambule, N., Yessoufou, K., Nwulu, N., & Mbohwa, C. (2018). Temporal analysis of electricity consumption for prepaid metered low- and high-income households in Soweto, South Africa. African Journal of Science, Technology, Innovation and Development, 11(3), 375–382. https://doi.org/10.1080/20421338.2018.1527983

- Kaselimi, M., Protopapadakis, E., Voulodimos, A., Doulamis, N., & Doulamis, A. (2022). Towards trustworthy energy disaggregation: A review of challenges, methods, and perspectives for non-intrusive load monitoring. Sensors, 22(15), 5872. https://doi.org/10.3390/s22155872

- Nebey, A. H. (2024). Recent advancement in demand side energy management system for optimal energy utilization. Energy Reports, 11, 5422–5435. https://doi.org/10.1016/j.egyr.2024.05.028

- OpenEnergyMonitor. (2023). Building an energy monitor using CT sensors and Arduino/ESP32. [Online technical guide]. How to build an Arduino energy monitor - measuring mains voltage and current — OpenEnergyMonitor 0.0.1 documentation

- Pinthurat, W., Surinkaew, T., & Hredzak, B. (2024). An overview of reinforcement learning-based approaches for smart home energy management systems with energy storages. Renewable and Sustainable Energy Reviews, 202, 114648. https://doi.org/10.1016/j.rser.2024.114648

- Ruano, A. E., Hernández, J. C., & Ureña, J. (2019). NILM techniques for intelligent home energy management : A review. Energies, 12(11), 2203. https://doi.org/10.3390/en12112203

- Ye, X., Zhang, Z., & Qiu, Y. (2023). Review of application of high frequency smart meter data in energy economics and policy research. Frontiers in Sustainable Energy Policy, 2, 1171093. https://doi.org/10.3389/fsuep.2023.1171093