Original Research Article

Investigating Water Storage Losses from Dugout Type

Farm Ponds on Farmer’s Field in Vidarbha Region of Maharashtra

.

ABSTRACT

Water is one of the most vital requirements for economic and social development. Human population of Indian subcontinent is ever increasing thereby increasing demand for water for domestic, agricultural and industrial use. To satisfy this demand, fresh resources of water supply must be tapped, and serious thought is being given towards adoption of different methods of conservation of water. Farm ponds are the most common technique of rainwater harvesting and recycling in rolling topography areas. In Maharashtra the total cultivable land is 18.5 M ha and gross cropped area is 22.66 M ha out of which 4.04 M ha is the gross irrigated area and the percentage of gross irrigated area to gross cropped area comes as 17.8, which is quite below the national average of 27 percent. Rainwater is the most vital and a critical input for agricultural production in rainfed farming areas in Maharashtra state. At Ghusar village in Akola taluka of Akola district in saline tract area, dugout type farm ponds water level losses recorded in four consecutive years (2019-20 to 2022-23). It was observed that the storage losses due to evaporation from dugout type farm ponds with bund in Akola taluka, having inlet and outlet closed by net or growing barrier of cotton crop around the farm pond reduces in the range of 23.67 to 27.97 per cent compared to pond without bund and without any vegetative barrier.

Keywords: Cultivable land, Farm Pond, Harvesting, Production

1. INTRODUCTION

For the region like Vidarbha in Maharashtra State of India, where precipitation is very uncertain and nearly 89% of the cultivated area is under rainfed farming there is need for development in the micro-watershed area. (Dongardive et al., 2018). Water has become a scarce resource here not only due to deficient rainfall but also due to over-exploitation of groundwater. (Ramamohan Reddy et al., 2013). In this region sustainable crop production and water resources development planning is must (Patode et al., 2016). The success mainly depends on the evenly distributed rainfall during crop growing period. The root zone soil moisture is utilized for transpiration, when the rainfall becomes insufficient to meet the potential needs to transpiration. This causes depletion in soil moisture storage and a situation which may be designated as agricultural drought (Patode et al, 2017). We must make maximum practice of the easily accessible, normally wasted, locally renewable source of water, that is rainwater. Once the soils get saturated then sub-surface as well as overland flow of rainwater started and this runoff should be stored with the help of reservoirs. At the time of heavy rainfall, infiltration is less, and rainwater started flowing from surface. If it is taken through drain lines to the storage structures, then maximum collection of rainwater is possible. If proper drain lines are not constructed then the surface water flows randomly and its collection at one place becomes difficult. Since rainwater collection and its judicious use is the need of the day, everybody should adopt the technical things that are important for proper implementation and construction of farm ponds (Sharma et al., 2010).  With the aim of maintenance of soil fertility and plant nutrient supply to an optimum level for sustaining the desired crop productivity the in-situ resource management of soil is important (Chandel et al., 2017). To improve the drought prone situation in drought affected districts of Vidarbha, large number of dugout type ponds (water storage structures) for harvesting of excess rain water are constructed on farmer’s field. The availability of harvested water in dug out type farm pond during rabi season depends on losses from pond during water storage period and information on this aspect is not available for Vidarbha region in Maharashtra state. To generate this information the study was undertaken to quantify evaporation and seepage losses (storage losses) from dugout type farm ponds in deep clay soils on farmer’s field. Farm pond is the most common technique of rainwater harvesting and recycling in rolling topography area. Rain water is the most vital and a critical input for agricultural production in rainfed farming areas. To minimize the evaporation losses from such small water harvesting structures in semi-arid regions scientist world over, have tried several types of anti-evaporants such as floating sheet cover of plastic membrane, polystyrene sheet, foamed wax blocks, plant residue, oil mulches, polyethylene oxides, gum mixtures and fatty alcohols (Dhruva Narayan et al., 1997). There was not much success with these attempts. As yet, an efficient, economical and durable evaporation suppressant has not emerged from the research which can be used widely. Catchment area can be envisioned based on the landscape/ topography and is the idea for designing soil and water conservation formations for water resources management (Kurothe et al., 2014; Patode et al. 2017). Supervising agricultural field crops throughout the growing season is gradually more important to adjust the supervision and to specify knowledge for prediction yield at harvest time (Moharir et al. 2021). These outcomes of in-situ soil and water conservation measures were used for impact evaluation (Shinde, 2006; Pendke, 2009).

Considering this a field experiment was carried out to study the evapotranspiration losses with following objectives as quantification of storage losses from dugout type farm ponds in farmer’s field and preventive measures for reduction of evaporation losses in dugout type farm ponds.

2. material and methods

The field experiment was undertaken during 2019 to 2023 under Department of Soil and Water Conservation Engineering Dr.PDKV, Akola. Ghusar village in Akola taluka of Akola district in saline tract area which falls under assured rainfall zone of Vidarbha region of Maharashtra was selected for the study. It is located at 2007’N latitude, 7707’ E longitude and at an altitude of 282 m above mean sea level. The soils are mainly black cotton soils with depth varying from 4 m to 10 m having clayey texture. The total average annual rainfall in Akola taluka is 751.52 mm.

Four ponds were selected in this study. The size of selected farm ponds was 30m x30. The water level fluctuations in these dugout type ponds were recorded with reference to 60 cm long scale fasten over a peg which was fixed vertically on two opposite sides of the pond in such a way that scales upper surface just touches the water level in the pond. The peg positions were maintained just below the previous one to continue measurement of water level fluctuations during subsequent period. The comparison is made. The results were obtained from these comparisons and storage losses were worked out and results were interpreted.

3. results and discussion

Storage losses from farm pond without bund

At Ghusar village in Akola taluka of Akola district in saline tract area, dugout type farm ponds water level losses were recorded in four consecutive years (2019-20 to 2022-23).  Water level recorded in observation ponds for the year 2019-20 are given in Table 1. From the data in Table 1 it was observed that the losses were more in farm pond no.1. and found lowest in farm pond no. 4. Overall, 24.97% less water losses had occurred in farm pond no.4 as compared to farm pond no. 1.

Table 1. Comparison of water storage losses in different dugout type farm ponds

on farmer’s field during 2019-20

| Date | Days | Dugout type farm pond without bund |  | Dugout type farm pond with bund |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without any vegetative cover | Cotton crop around farm pond | With Inlet outlet opening | With Inlet outlet opening protected by green net |
|  |  | storage losses Pond No. 1, cm | storage losses Pond No. 2, cm | storage losses Pond No. 3, cm | storage losses Pond No.4, cm |
| 07.11.2019 |  | Starting date of observation |  |  |  |
| 14.11.2019 | 7 | 4.8 | 4.0 | 4.1 | 3.8 |
| 22.11.2019 | 8 | 2.8 | 2.5 | 2.1 | 2.3 |
| 01.12.2019 | 8 | 5.4 | 3.4 | 3.9 | 3.5 |
| 07.12.2019 | 6 | 3.3 | 2.9 | 2.7 | 2.8 |
| 13.12.2019 | 6 | 2.7 | 2.4 | 2.1 | 2.4 |
| 22.12.2019 | 9 | 3.5 | 3.3 | 3.0 | 2.8 |
| 01.01.2020 | 10 | 5.4 | 3.7 | 3.6 | 3.4 |
| 10.01.2020 | 9 | 3.6 | 3.0 | 2.9 | 2.9 |
| 18.01.2020 | 8 | 5.7 | 4.1 | 4.1 | 3.4 |
| 25.01.2020 | 7 | 4.8 | 3.2 | 3.5 | 3.1 |
| 31.01.2020 | 6 | 4.9 | 3.4 | 3.7 | 3.2 |
| 11.02.2020 | 11 | 6.2 | 5.3 | 5.0 | 5.4 |
| 21.02.2020 | 10 | 5.7 | 4.3 | 4.8 | 4.2 |
| 29.02.2020 | 8 | 5.2 | 4.6 | 4.4 | 4.3 |
| 07.03.2020 | 7 | 3.4 | 3.0 | 2.8 | 2.9 |
| 16.03.2020 | 9 | 5.4 | 4.3 | 3.8 | 4.4 |
| Total | 129 | 72.6 | 57.1 | 56.5 | 54.5 |
| Percent saving in water storage loss in 129 days period |  | -- | 21.36 | 22.1 | 24.97 |

Water level recorded in observation ponds for the year 2020-21 are given in Table 2. From the data in Table 2 it was observed that the losses were more in farm pond no.1. and found lowest in farm pond no. 4. Overall, 29.95% less water losses had occurred in farm pond no.4 as compared to farm pond no. 1.

Table 2. Comparison of water storage losses in different dugout type farm ponds

on farmer’s field 2020-21

| Date | Days | Dugout type farm pond without bund |  | Dugout type farm pond with bund |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without any vegetative cover | Cotton crop around farm pond | With Inlet outlet opening | With Inlet outlet opening protected by green net |
|  |  | storage losses Pond No. 1, cm | storage losses Pond No. 2, cm | storage losses Pond No. 3, cm | storage losses Pond No.4, cm |
| 04.10.2020 | - | Starting date of observation |  |  |  |
| 13.10.2020 | 9 | 4.7 | 4.0 | 3.7 | 3.9 |
| 22.10.2020 | 9 | 4.2 | 3.1 | 3.2 | 3.0 |
| 30.10.2020 | 8 | 5.0 | 3.5 | 4.0 | 3.4 |
| 11.11.2020 | 11 | 4.5 | 4.1 | 3.4 | 4.0 |
| 23.11.2020 | 12 | 4.4 | 4.3 | 3.6 | 4.0 |
| 03.12.2020 | 10 | 4.1 | 3.6 | 3.2 | 3.6 |
| 20.12.2020 | 17 | 7.3 | 6.0 | 5.6 | 5.7 |
| 6.01.2021 | 16 | 7.8 | 6.1 | 5.5 | 5.5 |
| 17.01.2021 | 11 | 6.7 | 3.5 | 4.3 | 3.4 |
| 31.01.2021 | 14 | 7.4 | 3.5 | 4.5 | 3.2 |
| Total | 117 | 56.1 | 41.7 | 40.88 | 39.3 |
| Percent saving in water storage loss in 117 days period |  | -- | 25.76 | 27.12 | 29.95 |

Water level recorded in observation ponds for the year 2021-22 are given in Table 3. From the data in Table 3 it was observed that the losses were more in farm pond no.1. and found lowest in farm pond no. 4. Overall, 28.89% less water losses had occurred in farm pond no.4 as compared to farm pond no. 1.

Table 3. Comparison of water storage losses in different dugout type farm ponds

on farmer’s field 2021-22

| Date | Days | Dugout type farm pond without bund |  | Dugout type farm pond with bund |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without any vegetative cover | Cotton crop around farm pond | With Inlet outlet opening without protection | With Inlet outlet opening protect by green net |
|  |  | storage losses Pond No. 1, cm | storage losses Pond No. 2, cm | storage losses Pond No. 3, cm | storage losses Pond No.4, cm |
| 10.10.2021 | - | Starting date of observation |  |  |  |
| 19.10.2021 | 9 | 4.9 | 3.9 | 3.9 | 3.6 |
| 28.10.2021 | 9 | 4.1 | 2.9 | 2.9 | 2.7 |
| 08.11.2021 | 11 | 5.4 | 3.4 | 3.4 | 3.3 |
| 20.11.2021 | 12 | 3.2 | 3.1 | 3.1 | 2.9 |
| 28.11.2021 | 8 | 3.1 | 2.7 | 2.6 | 2.5 |
| 07.12.2021 | 9 | 3.5 | 3.2 | 3.0 | 2.9 |
| 15.12.2021 | 8 | 5.6 | 3.7 | 3.6 | 3.5 |
| 26.12.2021 | 9 | 3.7 | 3.2 | 3.1 | 3.1 |
| 05.01.2022 | 10 | 5.7 | 4.1 | 4.0 | 3.9 |
| 16.01.2022 | 11 | 4.8 | 3.2 | 3.1 | 3.1 |
| 29.01.2022 | 13 | 5.1 | 3.6 | 3.4 | 3.4 |
| 07.02.2022 | 9 | 6.2 | 5.2 | 5.0 | 5.0 |
| 16.02.2022 | 9 | 6.2 | 4.3 | 4.2 | 4.0 |
| Total | 127 | 61.2 | 46.2 | 45.3 | 43.5 |
| Percent saving in water storage loss in 127 days period |  | -- | 24.44 | 25.98 | 28.89 |

Water level recorded in observation ponds for the year 2022-23 are given in Table 4. From the data in Table 4 it was observed that the losses were more in farm pond no.1. and found lowest in farm pond no. 4. Overall, 28.07% less water losses had occurred in farm pond no.4 as compared to farm pond no. 1.

Table 4. Comparison of water storage losses in different dugout type farm ponds

on farmer’s field 2022-23.

| Date | Days | Dugout type farm pond without bund |  | Dugout type farm pond with bund |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without any vegetative cover | Cotton crop around farm pond | With Inlet outlet opening | With Inlet outlet opening protected by green net |
|  |  | storage losses Pond No. 1, cm | storage losses Pond No. 2, cm | storage losses Pond No. 3, cm | storage losses Pond No.4, cm |
| 12.10.2022 |  | Starting date of observation |  |  |  |
| 20.10.2022 | 8 | 4.3 | 3.9 | 3.7 | 3.6 |
| 29.10.2022 | 9 | 4.2 | 3.3 | 3.5 | 3.3 |
| 08.11.2022 | 9 | 4.8 | 3.4 | 3.7 | 3.3 |
| 18.11.2022 | 10 | 4.3 | 3.2 | 3.4 | 3.5 |
| 25.11.2022 | 7 | 3.9 | 2.7 | 2.6 | 2.4 |
| 06.12.2022 | 11 | 4.5 | 3.2 | 3.1 | 3.2 |
| 17.12.2022 | 11 | 4.9 | 3.7 | 3.7 | 3.5 |
| 29.12.2022 | 12 | 5.4 | 3.2 | 3.3 | 3.6 |
| 08.01.2023 | 10 | 5.9 | 4.2 | 4.1 | 3.9 |
| 20.01.2023 | 12 | 5.4 | 4.8 | 4.4 | 4.0 |
| 02.02.2023 | 12 | 5.7 | 5.4 | 4.3 | 4.5 |
| Total | 111 | 53.2 | 40.9 | 39.5 | 38.3 |
| Percent saving in water storage loss in 24 days period |  | -- | 23.13 | 25.78 | 28.07 |

The average water level observations four years pooled data (2019-20 to 2022-23) in selected dugout type farm ponds were presented in Table 5,  and indicates maximum average losses (60.78 cm) from pond no.1 without bund and without any vegetative cover around it followed by pond no. 3 bund type dugout type pond with inlet-outlet opening without protection (45.50 cm), pond no. 2 without bund having cotton crop of 1.3 to 1.5 m height on all the four sides as cover crop (46.48 cm) and minimum average losses (43.88 cm) from pond No. 4 of bund type dugout type pond with inlet-outlet opening protect by green net. The average percent saving of 27.97 % in dugout type farm pond with bund having inlet-outlet opening protect by green net and 23.67 % average saving in dugout type farm pond without bund with 1.3 to 1.5 m high barrier cotton crop on all the four sides was observed which was mainly due to the retardation of wind velocity over pond water surface in both pond no. 4 and 2 respectively as compare to without bund and without any vegetative cover around it (Pond No. 1) and bund type with inlet and outlet opening without protection (pond No. 3). Because the recorded losses are mainly due to reduction of evaporation loss component since, the seepage losses from 3 m deep dugout type farm ponds in saline track areas mostly get stabilized after 2 to 3 months of water storage and are negligible which the finding of earlier study was conducted in same village.

Table 5. Comparison of water storage losses in different dugout type farm ponds

on farmer’s field during 2019-20 to 2022-23.

| Year | Observation Days | Dugout type farm pond without bund |  | Dugout type farm pond with bund |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without any vegetative cover | Cotton crop around farm pond | With Inlet outlet opening without protection | With Inlet outlet opening protect by green net |
|  |  | storage losses Pond No. 1, cm | storage losses Pond No. 2, cm | storage losses Pond No. 3, cm | storage losses Pond No.4, cm |
| 2019 | 129 | 72.6 | 57.1 | 56.50 | 54.5 |
| Percent saving in water storage loss |  | -- | 21.36 | 22.16 | 24.97 |
| 2020 | 117 | 56.1 | 41.7 | 40.88 | 39.3 |
| Percent saving in water storage loss |  | -- | 25.76 | 27.12 | 29.95 |
| 2021 | 127 | 61.2 | 46.2 | 45.30 | 43.5 |
| Percent saving in water storage loss |  | -- | 24.44 | 25.98 | 28.89 |
| 2022 | 111 | 53.2 | 40.9 | 39.50 | 38.2 |
| Percent saving in water storage loss |  | -- | 23.13 | 25.78 | 28.07 |
| overall average water storage loss (2019-20 to 2022-23) |  | 60.78 | 46.48 | 45.50 | 43.88 |
| Percent savings on water storage loss |  | -- | 23.67 | 25.26 | 27.97 |

Hence, the recorded differences in water storage losses from the farm ponds are considered as evaporation loss components. Based on result shown in Table 5, it can be inferred that reducing the wind velocity over water surface in farm pond by protecting the inlet and outlet in bund type farm pond by green net is helpful in reducing evaporation component of water storage losses. It is recommended that the dugout type farm pond constructed on farmers field should have the existing bund (1.5 m height) or grown the vegetative barrier (1.3 -1.5 m height) around the farm pond to save the storage losses up to 27.97 percent.

4. Conclusion

1. The storage losses observed from dugout type farm pond with bund (height 1.5 m) having inlet-outlet opening protected by green net was 43.88 cm (27.97 per cent less compared to pond without bund and without any vegetative barrier).

2. The storage losses observed from dugout type farm pond without bund with vegetative barrier of cotton crop (with 1.3 to 1.5 m) on all sides was 46.48 cm (23.67 per cent less) compared to pond without bund and without any vegetative barrier.

3. The storage losses due to evaporation from dugout type farm ponds with bund in Akola taluka, having inlet and outlet closed by net or growing barrier of cotton crop around the farm pond reduces in the range of 23.67 to 27.97 per cent compared to pond without bund and without any vegetative barrier.

Disclaimer (Artificial intelligence)

Author(s) hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Chandel Ashwini, V. V. Gabhane, M. B. Nagdeve, A. B. Turkhede and R. S. Patode (2017). Effect of INM on soil fertility, productivity and economics of cotton- green gram intercropping system in vertisols. Int. J. Curr. Microbiol. App. Sci.6(11): 3738-3743. https://doi.org/10.20546/ijcmas.2017.611.437

- Dongardive, M. B., Patode, R. S., Nagdeve, M. B., Gabhane, V. V., & Pande, C. B. (2018). Water resources planning for the micro watersheds using geospatial techniques, International Journal of Chemical Studies, 6(5), 2950-2955. https://www.chemijournal.com/archives/2018/vol6issue5/6-5-10-100.pdf

- Dhruva Narayana, G. Sastry and U. S. Patnaik (1990). Watershed management: Published by ICAR, New Delhi (INDIA). Google Book of 176 pages.

- Kurothe, R. S., Vishwakarma, A. K., Sena, D. R., Gopal Kumar, Rao, B. K. and Pande, V. C. (2014). Decision support system for contour trenching. Indian J. of Soil Conservation, 42(2): 143-153.

- Moharir K. N., C. B. Pande, R. S. Patode, M. B. Nagdeve and A.M. Varade (2021). Prioritization of sub-watersheds based on morphometric analysis using geospatial technology. Water Management and Water Governance: Hydrological Modeling. Springer International Publishing:19-33.

- Patode, R. S., Nagdeve, M. B., & Pande, C. B. (2016). Groundwater level monitoring of Kajaleshwar-Warkhed watershed Tq. Bashitakli Distt. Akola. Advances in Life Sciences, 5(24), 11207-11210. https://www.researchgate.net/publication/321937373

- Patode, R. S., M. B. Nagdeve, M. M. Ganvir and V. V. Gabhane (2017). "Evaluation of In-situ Moisture Conservation Practices for Sustainable Productivity of Major Crops in Vidarbha Region." Int. J. Curr. Microbiol. App. Sci, 6(10): 261-268. https://doi.org/10.20546/ijcmas.2017.610.033

- Pendke, M. S. (2009). Qualitative evaluation of soil and water conservation structures in Daregaon watershed, Journal of Soil and Water Conservation, 8(1): 9-13.

- Ramamohan Reddy, K. and R. S. Patode (2013). Assessment of groundwater quality- A case study of Kondapur mandal, Medak district, Andhra Pradesh. Current World Environment, 8(2), 267-273. https://doi.org/10.12944/CWE.8.2.12

- Sharma, B.R., Rao, K.V., Vittal, K.P.R., Ramakrishna,Y.S. and Amarasinghe, U. (2010). Estimating the potential of rainfed agriculture in India, Prospects for water productivity improvements. Agricultural water Management 97:23-30. https://doi.org/10.1016/j.agwat.2009.08.002

- Shinde, M., Smout, I. and Gorantiwar, S. (2006). Assessment of water harvesting and groundwater recharge through continuous contour trenches. Recharge systems for protecting and enhancing groundwater resources, UNESCO, IHP-VI, series on Groundwater,13:229-235.