Determination of Interfacial Tension of binary n mixtures of nalkanes+CH4 and n-alkanes+CO2 using Density gradient theory and Peng-Robinson Equation of State at elevated temperatures and pressures

Abstract: The interfacial tensions of alkanes in the presence of CH4 and CO2 were predicted using the model taken as a combination of Density Gradient theory and Peng-Robinson equation of state and compared with existing simulation data. The main aim of this work was to make a progress in modeling the IFTs between diverse n-alkanes and CH4 and CO2 natural gases. Several statistical measures and graphical descriptions were employed to aid the accuracy analysis of the proposed model. The statistical measures indicate a complex relationship among thermal conditions, model selection, and performance metrics, which can be observed by examining the tables presented in this work. Overall, the results indicate that the Peng-Robinson equation effectively predicted the interfacial tensions.

Keywords: Density gradient theory; Interfacial tension; n-alkanes; Natural gases and Statistical errors.

1. Introduction

The interfacial tension is an important physical property in several industrial applications because the performance of many engineering processes depends on the behavior of fluid–fluid and solid–fluid interfaces. Numerous experimental, theoretical, and simulation investigations have been conducted on the interfacial characteristics of binary mixtures of n-alkane with natural gases such as CH4 and CO2 in different ranges of temperature and pressure [Mahdaviara et al. 2021; Li et al. 2017; Menand et al., 2020; Mique et al.,2008; Carey et al., 1980; Ameli et al., 2018; Laffite et al., 2006; Amezquita et al., 2010; Choudhary et al., 2019; Mirzaie et al., 2020, Amar et al., 2021].Several authors have sought to estimate this parameter to circumvent the complexities and expenses linked to laboratory measurements. In recent decades, various mathematical and thermodynamic models have emerged as promising approaches for interfacial tension (IFT) estimation, including the Parachor [Macleod et al., 1923; Sugden et al., 1924], Scaling Law Method [Mique et al.,(2008); Corresponding State Method [Mique et al., 2008; Zuo et al., 1996; Linear Gradient Theory (DGT) [Mique et al., 2008; Pereira et al., 2016] and Density Gradient Theory (DGT) [Mique et al., 2008; Aahrafizadeh et al., 2011]. Additionally, several researchers have utilized combinations of these theoretical models along with different Equations of State (EOSs) to concurrently clarify various interfacial properties of fluid systems in the presence of different gases. Some simulation methods, such as Molecular Dynamic (MD) simulations [Choudhary et al., 2021], have been also found as useful approaches for addressing the IFT of the diverse systems. To our knowledge, only a limited number of experimental investigations [Choudhary et al., 2021; Mejia et al., 2014; Jaeger et al., 2010; Miquez et al., 2014; Mirzaie et al., 2020; Mahdaviara et al., 2022; Zolghadr et al., 2013] examining the interfacial tension of (n-alkanes + CH4) and (n-alkanes + CO2) systems have been conducted at the pressure conditions considered in this study. In the paper they [Choudhary et al., 2021] have utilized volume translated Peng-Robinson equation of state (VT-PR EOS) for prediction of interfacial tension at these pressures. The van der Waals theory is the foundation of density gradient theory for inhomogeneous fluids. It was rediscovered by Cahn and Hilliard [Cahn & Hilliard 1958] and represents an extension of the Helmholtz free energy density around a step density profile that is truncated after the second term. The local free energy density is calculated using an equation of state, such as the Peng-Robinson equation.

Numerous experimental investigations have been conducted on (CO₂ + n-alkanet) and (CH₄ + n-alkanet) systems across a wide range of temperature and pressure to create a comprehensive interfacial tension database. Measuring interfacial tension in the laboratory is both labor-intensive and costly. Accurate determination of interfacial tension is crucial in petroleum and chemical research and engineering. It plays a vital role in various processes, including separation, emulsion formation and stability, fluid transportation, and reservoir engineering tasks such as gas condensate recovery, near-critical fluid recovery (enhanced oil recovery, EOR), and improved oil recovery techniques. The oil and gas industry relies on enhanced oil recovery methods to meet the growing demand for oil, which remains a key energy source. Notable techniques for miscible enhanced oil recovery include carbon dioxide injection and hydrocarbon injection (Methane). Carbon dioxide injection is a well- established and widely used method in this field. Due to limitations in the availability of carbon dioxide as a gas source, there is growing interest in using hydrocarbon gas flooding to enhance oil recovery. The accurate prediction of surface tension (or liquid-vapor interfacial tensions) under the thermodynamic conditions present in petroleum reservoirs is essential for maximizing oil recovery. To prevent labor-intensive and expensive measurements, there is a need for reliable theoretical estimations of surface tensions or interfacial tensions.

2. Density gradient theory

The density gradient theory (DGT) [Cahn & Hilliard et al., 1958; Li et al., 2008; Mejia et al., 2014; Carey et al., 1980; Mique et al., 2004] is based on a mean field approximation that describes the uniform evolution of Helmholtz energy density across the interface. This approach aids in determining interfacial parameters, such as the density profile and interfacial tension. Confirmed the DGT to be more efficient and precise than other methods. This technique primarily uses the equation of state (EOS) to analyze interfacial characteristics and phase equilibrium. The DGT has been combined with various equations of state. Cahn and Hilliard developed the well-known equation, which describes the separation of phases in binary alloys. Since its inception, the CH equation has been applied across numerous fields, including science, chemistry, physics, and biology. Consequently, understanding how this equation operates within different models is essential. Currently, density gradient theory serves as an effective method for determining the interfacial tension of mixtures. Understanding the influence parameter is essential for computation. Assume the interface between the liquid and vapor bulk phases is planar. The interface thickness is represented by z, which denotes the distance normal to this interface. According to the density gradient theory, in the absence of external potentials, the differential equation governing the density distributions ρi(z) through a planar interface is given by equation (1) Ameli et al., 2018; Blas et al., 2001]. In this context, nc represents the number of components in the mixture, and  ρ represents the grand thermodynamic potential.

j=1ncddzcijdρjdz-12j=1nck=1nc∂cjk∂ρidρjdzdρkdz=∂ρ∂ρi                                                                    (1)

The information regarding the intermolecular geometry of the interface is encapsulated within the influence parameter. This parameter also establishes a link between the extent of deviation in the Helmholtz energy density and the chemical potential. The grand thermodynamic potential, represented by the symbol ∆ ρ, is defined as follows:

∆ρ=f0ρ-iρiμiB+P                                                                                         2

Where, f0ρ is the local Helmholtz free energy of homogeneous fluid at interfacial region of density ρi(z) whereas μiB the chemical potential of component i  in the bulk phase. In equation (1), the second component of the left-hand side (L.H.S.) will not be present when considering that the influence parameter cjk are independent of composition. This allows for the simplification of the governing equation (1) for the planar interface to

j=1nccijd2ρidz2=μiρ-μiB                                                                                                     (3)

Where, the μiρ is the chemical potential at the interfacial region where the density equals to ρ. Multiplying equation(3) by dρidz, summing over i and integrating gives

ij12cijdρidzdρjdz=∆ρ-B                                                                                     (4)

Equation (4) describes the difference between the grand thermodynamic potential ∆ ρ=f0ρ-iρiμiB at local composition and its corresponding value in the bulk phases, which is denoted as  B=-P, where P represents the vapor-liquid equilibrium pressure. The differential equation (3) is governed by the following boundary condition                                                            z→+∞,ρ→ρL

z→-∞,ρ→ρV

Where, the ρL and ρV are the liquid and vapor phase densities. Using gradient theory, the interfacial tension is computed as:

ρVρL2(ρ-Bijcijdρidρdρjdρdρ                                                                     (5)

The equilibrium pressure (P), Helmholtz free energy density (f0), and chemical potential (μi) at the interface can be calculated. Now, the density profiles can be obtained from the following expression

z=z0+ρVρLijcijdρidρdρjdρ2∆(ρ)dρ

Where, z0 is an arbitrarily chosen position.

Here we have utilized the Peng-Robinson equation of state [Mejia et al., 2014; Choudhary et al., 2021] to calculate the densities ρL and ρV in the liquid and vapor phase in equilibrium.

P=RTv-b-a(T)vv+b+b(v-b)

Carey and co workers [Carey et al., 1980] were the first to offer such a combination of the gradient theory and PR-EOS, and since then, it has been adopted on multiple times in the literature.

This study employs the classical van der Waals mixing rules for the co-volume parameter b  and the temperature-dependent energy parameter a(T) of the mixture:

b=ixibi                                                                                                                         (6)

a=ijxixj(1-kij)aiaj                                                                                       (7)

Where, xi are the mole fractions of component i in each phase and are the binary interaction parameters. The parameter of pure fluids are defined by classical equations

bi=0.07780RTciPci                                                                                                               (8)

aiT=0.45724R2Tci2PciαTri                                                                                              (10)

with

αTri=1+mi1-Tri2                                                                                            (11)

Table 7 gives the critical properties and acentric factor for alkanes and gases used in equations (8) to (11).

3. Influence parameter

According to the mixing rule, the pure component influence parameters ci and cj are connected to the cross influence parameters:

cij=1-βijcicj                                                                                                         (12)

Where,  βij are binary interaction coefficients. For thermodynamic stability of the interface, the values  βij must be constrained between 0 and 1. The mixing rule is simplified to the geometric mean. Previous literature indicates that the geometric mixing rule for influence parameters provides the most efficient estimation for hydrocarbon/hydrocarbon pairs and gas (carbon dioxide, methane, or nitrogen) / hydrocarbon pairs. The expression for the influence parameter applicable to non-polar pure fluids is determined when the gradient theory is combined with the PR-EOS. It describes the variation of the influence parameter with the reduced temperature ti=1-TTci as follows:

ciaibi2/3=Aiti+Bi

Where, biis the co-volume and ai is the energy parameter in the PR-EOS. The coefficients Ai and Bi are only associated with the acentric factor of the component i by the relationships:

Ai=-10-161.2326+1.377ωi                                                                                   13

Bi=-10-160.9051+1.5410ωi                                                                                 (14)

To compute interfacial tension values using equation (5), the density distribution of each component (dρidz) is estimated. The profile of each component may be computed by applying the Helmholtz free energy minimization criterion to planar surfaces and solving the set of nc non-linear differential equations in a finite domain [0,L] defined as:

icijd2ρjdz2=μiρiz,……..ρnz-μiB for i,j=1…nc                                                  (15)

With ρiz=0=bulk vapor=ρiVand ρiz=L=bulk liquid=ρiL.

4. Result and Discussion

The effect of temperatures and pressures on the IFT of (n-alkane+CH4) and (-alkane+CO2) has been studied. Figure 1-3, the simulated density data [Choudhary et al., 2021] have been compared with the chosen model of DGT+EOS for the ( -alkane+CH4) and ( n-alkane+CO2) systems on two isotherms at 323K and 343K and pressure up to 30 MPa. The simulated density data have been fitted as a function of pressure for correlating DGT+EOS for each isotherm with a maximum absolute percentile deviation of 4.0% and average absolute deviation of 1.5%. So, a good agreement between the theory and experiment was found. The simulated IFT data were compared with the predictions using DGT + PR EOS at two isotherms 323K and 343K for pressure up to 30 MPa. Figures 4 – 6 give the comparison results of calculated and simulated IFTs data [26] as a function of pressure. It can be seen that the calculated results are in good accordance with the simulated data. The IFTs in these Figures show an opposite dependence on the temperature at lower pressures and positive at higher pressure. The general trend can be observed in each of the plotted graphs.

Error analysis: Interfacial tension determination of three (n-alkane+CH4) and three (-alkane+CO2) systems have been performed for temperatures at 323K and at 343K  and pressure upto 30 MPa for each system. The available simulated data were utilized to meticulously assess the performance of the proposed model by statistical error functions [38], namely Coefficient of Determination  R2 , Chi-square,  Residual Sum of Squares Error (ERRSQ/SSE),  Average Relative Error (ARE), Standard Deviation (SD), Mean Square Error (MSE), Root Mean Square Error (RMSE), Percentage Average Relative Deviation (AAD%) and Percentage Absolute Average Relative Deviation (AARD%). The error analysis equations are conveyed as follows:

- Determination Coefficient, a well-known statistical measure that is defined as follows:

R2=1-i=1NIFTiexp-IFTipred2i=1NIFTiexp-IFTiexp2

- Chi-square, it is defined as follows:

Chi-square=1-i=1NIFTiexp-IFTipred2IFTiexp

- ERRSQ/SSE, Residual Sum of Square Error is also defined as:

ERRSQ/SSE=i=1NIFTiexp-IFTipred2

- ARD, the Average Relative Deviation is defined as:

ARE=1Ni=1NIFTiexp-IFTipredIFTiexp

- ARD%, the Percentage Average Relative Deviation is defined as:

ARD%=100Ni=1NIFTiexp-IFTipredIFTiexp

- Standard Deviation, a measure indicating the distribution of the values:

SD=1N-1i=1NIFTiexp-IFTipredIFTiexp2

- Mean Square of Error, the average of the squares of discrepancies between the calculated values and the target:

MSE=1Ni=1NIFTiexp-IFTipred2

- Root Men Square of Error, the second root of the MSE

RMSE=1Ni=1NIFTiexp-IFTipred2

- Absolute Average Relative Deviation, the absolute average relative AARD:

AARD%=100Ni=1NIFTiexp-IFTipredIFTiexp

The lower values of the SD, AAD, MSE, RMSE, and AARD, the higher would be the accuracy of the model. The reverse is true in case of the R2 parameter. R2:  From Table 1-6, Determination coefficient has more value at 443K then at 323K for C7+CH4 while for C7+ CO2 it is nearly constant. The same was noted for mixtures of all alkanes and natural gases used in this study. So, at higher temperature the accuracy of the model is good.

The lower values of standard deviation (SD), average absolute deviation (AAD), mean squared error (MSE), root mean squared error (RMSE), and average absolute relative deviation (AARD) indicate greater accuracy in the model. The opposite is true for the R2parameter. According to Table 1-6, the coefficient of determination(R2) is higher at 443K than at 323K; however, it remains relatively constant at these temperatures. The mixtures of all alkanes and natural gases examined in this study exhibited similar trends at these temperatures. At the elevated temperature of 443K, the model's accuracy is deemed acceptable. Regarding the chi-square analysis, the accuracy of the model is more reliable at 323K compared to 443K, as indicated by the lower value. The same observation has also been noted in the mixtures of all alkanes and natural gases utilized in this study. For the standard deviation (SD), the values are higher at 323K than at 443K. Therefore, the selected model and mixture show better performance at higher temperatures compared to lower temperatures. In contrast, the percentage average relative deviation (ARD%) indicates that the selected model and mixture perform better at lower temperatures. The findings from the study on alkane and natural gas performance reveal a nuanced relationship between temperature and efficiency for these mixtures and the model considered. The performance disparities became more pronounced with temperature variations. Notably, the increased standard deviation at 323K compared to 443K suggests that higher temperatures yield more consistent performance outcomes, which could be advantageous for applications requiring reliability. Conversely, the ARD% metric underscores that these mixtures demonstrate improved efficiency at lower temperatures, highlighting a complex interplay between thermal conditions, model selected and performance metrics. The prime objective of this research was to make progress in the modelling the IFTs between diverse n-alkanes and CH4 and CO2 natural gases. This research involved several statistical measures and graphical descriptions to aid the accuracy analysis of the proposed model. For IFT predictions, the parachor method (and its derivatives) is the most commonly employed in the petroleum industry due to its ease of application. Unfortunately, when used predictively, it is often inefficient. Indeed, the parachors are often modified to match experimental findings. As a result, accurate estimates of the surface tension of petroleum mixtures remain uncertain. We use theoretical models such as cubic plus association (CPA), simplified perturbed chain statistical associating fluid theory (Spc - SAFT), SAFT VR-Mie, perturbed chain SAFT (PC SAFT) equation of state, and GT, LGT, and DGT combined with an equation of state for predicting interfacial tension because they offer superior accuracy and more comprehensive in accounting for complex physical phenomena, and have a wider range of applicability than traditional, often empirical methods. This paper is a part of systematic application of the gradient theory of fluid mixtures. In this paper we gave very good estimates of the interfacial tension made of natural gases and n-alkanes. These gradient theory and equation of state-based interfacial tension models have the following main advantages:

(1) By establishing the density profile of the constituents, the adsorption behavior of the constituents may be ascertained.

(2) They may cover liquid gas, liquid-liquid, and liquid-supercritical regions in systems like natural gases and alkanes because they can be used with both low- and high-pressure data.

Figure 1 Comparison of predicted density with simulated data [Choudhary et al., 2021] versus pressure P for C7+ CH4 (Figure 1(a) and Figure 1(b)) and C7+ CO2 (Figure 1(c) and Figure 1(d)) at 323K and 443K respectively [Choudhary et al., 2021].

Figure 2 Comparison of predicted density with simulated data [Choudhary et al., 2021] versus pressure P for C12+ CH4 (Figure 2(a) and Figure 2(b)) and C12+ CO2 (Figure 2(c) and Figure 2(d)) at 323K and 443K respectively [Choudhary et al., 2021].

.

Figure 3 Comparison of predicted density with simulated data [Choudhary et al., 2021] versus pressure P for C19+ CH4 (Figure 3(a) and Figure 3(b)) and C19+ CO2(Figure 3(c) and Figure 3(d)) at 323K and 443K respectively [Choudhary et al., 2021].

Figure 4: Plot of IFT versus Pressure (a) (C7+CH4) (b) (C7+CH4) at  323K and 443K, respectively;

(c) (C7+CO2) (d) (C7 + CO2) at T=323K and 443K,  respectively [Choudhary et al., 2021].

Figure 5: Plot of IFT versus Pressure (a) (C12 + CH4), (b) (C12 + CH4) at  323K and 443K,  respectively ;

(c)  (C12 + CO2), (d) (C12 + CO2) at T=323K and 443K, respectively [Choudhary et al., 2021].

Figure 6: Plot of IFT versus Pressure (a) (C19+CH4),(b)(C19+CH4) at 323K and 443K,  respectively;

(c) (C19+CO2), (d) (C19 + CO2) at T=323K and 443K,  respectively [Choudhary et al., 2021].

Table 1: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C7 + CH4) at 323K and 443K.

| C7+CH4 323K |  |  |  |  | C7+CH4 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 1.953 | 14.78 | 14.87 | -0.09 | 0.0081 | 2.010 | 4.62 | 5.36 | -0.74 | 0.5476 |
| 3.166 | 12.27 | 12.98 | -0.71 | 0.5041 | 2.813 | 4.95 | 4.59 | 0.36 | 0.1296 |
| 4.972 | 11.05 | 10.54 | 0.51 | 0.2601 | 4.804 | 3.78 | 3.18 | 0.60 | 0.3600 |
| 8.103 | 7.96 | 7.38 | 0.58 | 0.3364 | 7.349 | 1.78 | 1.96 | -0.18 | 0.0324 |
| 11.553 | 4.65 | 4.94 | -0.29 | 0.0841 |  |  |  |  |  |
| R2 0.99997 | Chi.Sq 0.126 | ERSSQ or SSE 1.193 | ARD -0.1460 | SD 12.5311 | R2 0.99948 | Chi.Sq 0.258 | ERSSQ or SSE 1.069 | ARD -0.7459 | SD 4.6168 |
| IFT (Expt.) 10.1420 | MSE 0.239 | RMSE 9.7950 | ARD% 914.054 | AARD% 914.054 | IFT (Expt.) 3.7825 | MSE 0.2674 | RMSE 3.0517 | ARD% 277.504 | AARD% 277.504 |

Table 2: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C7 + CO2) at 323K and 443K

| C7+CO2 323K |  |  |  |  | C7+CO2 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 2.031 | 13.10 | 13.51 | -0.41 | 0.1681 | 2.081 | 5.14 | 5.86 | -0.72 | 0.5184 |
| 2.931 | 11.10 | 10.62 | 0.48 | 0.2304 | 2.968 | 4.13 | 4.65 | -0.52 | 0.2704 |
| 3.998 | 8.89 | 7.93 | 0.96 | 0.9216 | 4.084 | 4.04 | 3.49 | 0.55 | 0.3025 |
| 5.167 | 6.53 | 5.71 | 0.82 | 0.6724 | 5.870 | 1.61 | 2.22 | -0.61 | 0.3721 |
| R2 0.99987 | Chi.Sq 0.240 | ERSSQ or SSE 1.992 | ARD 6.1376 | SD 13.0737 | R2 0.99922 | Chi.Sq 0.472 | ERSSQ or SSE 1.463 | ARD -12.7182 | SD 4.4517 |
| IFT (Expt.) 9.9050 | MSE 0.4980 | RMSE 9.2800 | ARD% 896.637 | AARD% 896.637 | IFT (Expt.) 3.7300 | MSE 0.3658 | RMSE 2.9650 | ARD% 260.281 | AARD% 260.281 |

Table 3: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C12 + CH4) at 323K and 443K.

| C12+CH4 323K |  |  |  |  | C12+CH4 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 2.132 | 19.07 | 19.91 | -0.84 | 0.7073 | 2.114 | 11.28 | 12.25 | -0.97 | 0.9409 |
| 4.784 | 16.22 | 16.11 | 0.11 | 0.0121 | 4.686 | 9.13 | 9.69 | -0.56 | 0.3136 |
| 9.916 | 11.12 | 10.79 | 0.33 | 0.1089 | 10.002 | 6.29 | 6.02 | 0.27 | 0.0729 |
| 14.761 | 7.12 | 7.41 | -0.29 | 0.0841 | 15.674 | 3.66 | 3.62 | 0.04 | 0.0016 |
| 19.934 | 4.54 | 4.89 | -0.35 | 0.1225 | 19.881 | 3.68 | 2.47 | 1.21 | 1.4641 |
| 25.167 | 3.29 | 3.25 | 0.04 | 0.0016 | 25.012 | 1.54 | 1.56 | -0.02 | 0.0004 |
| R2 0.99998 | Chi.Sq 0.059 | ERSSQ or SSE 1.036 | ARD -1.8885 | SD 12.0940 | R2 0.99991 | Chi.Sq 0.129 | ERSSQ or SSE 2.793 | ARD 3.7057 | SD 6.8209 |
| IFT (Expt.) 10.2265 | MSE 0.1727 | RMSE 10.9088 | ARD% 920.761 | AARD% 920.761 | IFT (Expt.) 5.9300 | MSE 0.4655 | RMSE 5.9711 | ARD% 496.705 | AARD% 496.705 |

Table 4: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C12 + CO2) at 323K and 443K.

| C12+CO2 323K |  |  |  |  |  |  | C12+CO2 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa | IFT Expt. mNm-1 |  | IFT Calc. mNm-1 | Residual |  | Residual2 | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 2.023 | 17.46 |  | 19.42 | -1.96 |  | 3.8416 | 2.280 | 10.70 | 12.08 | -1.38 | 1.9044 |
| 3.290 | 14.55 |  | 14.96 | -0.41 |  | 0.1681 | 3.792 | 9.84 | 10.07 | -0.23 | 0.0529 |
| 3.925 | 13.49 |  | 13.16 | 0.33 |  | 0.1089 | 6.298 | 8.64 | 7.49 | 1.15 | 1.3225 |
| 6.266 | 8.37 |  | 8.18 | 0.19 |  | 0.0361 | 7.899 | 6.11 | 6.17 | -0.06 | 0.0036 |
| 7.233 | 5.93 |  | 6.73 | -0.8 |  | 0.6400 | 10.055 | 4.22 | 4.78 | -0.56 | 0.3136 |
|  |  |  |  |  |  |  | 11.898 | 3.34 | 3.79 | -0.45 | 0.2025 |
| R2 0.99991 |  | Chi.Sq 0.243 | ERSSQor SSE 4.794 |  | ARD -4.5635 | SD 14.8212 | R2 0.99991 | Chi.Sq 0.974 | ERSSQ or SSE 0.337 | ARD -4.9416 | SD 8.3533 |
| IFT (Expt.) 11.9600 |  | MSE 0.9589 | RMSE 11.6991 |  | ARD% 1091.436 | AARD% 1091.436 | IFT (Expt.) 7.1416 | MSE 0.6332 | RMSE 6.7102 | ARD% 609.225 | AARD% 609.225 |

Table 5: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C19 + CH4) at 323K and 443K.

| C19+CH4 323K |  |  |  |  |  | C19+CH4 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa | IFT Expt. mNm-1 |  | IFT Calc. mNm-1 | Residual | Residual2 | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 2.012 | 22.73 |  | 23.76 | -1.03 | 1.061 | 2.125 | 16.04 | 16.72 | -0.68 | 0.462 |
| 4.911 | 19.08 |  | 19.68 | -0.60 | 0.360 | 4.802 | 13.42 | 14.12 | -0.70 | 0.490 |
| 10.004 | 14.87 |  | 14.01 | 0.86 | 0.739 | 10.382 | 10.25 | 10.08 | 0.17 | 0.029 |
| 14.570 | 10.84 |  | 10.38 | 0.46 | 0.211 | 14.959 | 7.62 | 7.67 | -0.05 | 0.0025 |
| 18.988 | 7.50 |  | 7.75 | -0.25 | 0.062 | 20.738 | 6.82 | 5.46 | 1.36 | 1.850 |
| 24.915 | 5.05 |  | 5.22 | -0.17 | 0.029 | 25.339 | 4.27 | 4.06 | 0.21 | 0.044 |
|  |  |  |  |  |  | 30.243 | 2.68 | 3.05 | -0.37 | 0.137 |
| R2 0.99998 |  | Chi.Sq 0.134 | ERSSQ or SSE 2.463 | ARD -0.7247 | SD 15.8951 | R2 0.99997 | Chi.Sq 0.068 | ERSSQ or SSE 3.014 | ARD 0.3714 | SD 9.9939 |
| IFT (Expt.) 13.3450 |  | MSE 0.4105 | RMSE 13.8176 | ARD% 1233.775 | AARD% 1233.775 | IFT (Expt.) 8.7285 | MSE 0.4306 | RMSE 8.9212 | ARD% 773.228 | AARD% 773.228 |

Table 6: Pressure with simulated [Choudhary et al., 2021] and calculated IFT Data (this work) and performance evaluation of DGT + EOS for the binary mixture (C19+CO2) at 323K and 443K.

| C19+CO2 323K |  |  |  |  |  |  |  | C19+CO2 443K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pressure in MPa |  | IFT Expt. mNm-1 |  | IFT Calc. mNm-1 | Residual | Residual2 |  | Pressure in MPa | IFT Expt. mNm-1 | IFT Calc. mNm-1 | Residual | Residual2 |
| 1.995 | 21.41 |  |  | 22.38 | -0.97 | 0.9409 |  | 2.197 | 14.38 | 15.34 | -0.96 | 0.9216 |
| 3.006 | 18.41 |  |  | 18.38 | 0.03 | 0.0009 |  | 3.661 | 13.98 | 13.78 | 0.20 | 0.0400 |
| 3.95 | 16.01 |  |  | 15.36 | 0.65 | 0.4225 |  | 6.115 | 12.02 | 11.51 | 0.51 | 0.2601 |
| 5.739 | 11.21 |  |  | 10.81 | 0.40 | 0.1600 |  | 8.227 | 10.04 | 9.92 | 0.12 | 0.0144 |
| 6.906 | 8.13 |  |  | 8.62 | -0.49 | 0.2401 |  | 9.980 | 8.29 | 8.74 | -0.45 | 0.2025 |
|  |  |  |  |  |  |  |  | 11.795 | 7.87 | 7.65 | 0.22 | 0.0484 |
| R2 0.99998 |  |  | Chi.Sq 0.084 | ERSSQor SSE 1.764 | ARD -0.5532 |  | SD 18.6976 | R2 0.99998 | Chi.Sq 0.090 | ERSSQ or SSE 1.487 | ARD -0.4066 | SD 13.2013 |
| IFT (Expt.) 15.0340 |  |  | MSE 0.3528 | RMSE 14.8280 | ARD% 1402.846 |  | AARD% 1402.846 | IFT (Expt.) 11.0967 | MSE 0.2478 | RMSE 10.4111 | ARD% 1009.260 | AARD% 1009.260 |

Table 7: Critical properties and acentric factor for alkanes and gases

| Component | Tc (K) | Pc (MPa) | Acentric Factor |
| --- | --- | --- | --- |
| C7 | 540.2 | 2.74 | 0.350 |
| C12 | 658.0 | 1.82 | 0.576 |
| C19 | 773.8 | 1.24 | 0.853 |
| CH4 | 190.6 | 4.56 | 0.011 |
| CO2 | 304.2 | 7.29 | 0.239 |

5. Conclusion:

This work aimed to enhance the prediction of interfacial tension (IFT) between various n-alkanes and the natural gases methane and carbon dioxide using density gradient theory combined with the Peng Robinson Equation of state (PR-EOS). The interfacial tension of the CH₄/CO₂ -alkane systems is known to diminish with rising pressure. In the case of the IFT- temperature relationship, however, a contradictory trend is observed, in which the IFT decreases or increases with temperature in low and high pressures, respectively. While studying model accuracy analysis, it has been observed that the performance differences increased with temperature. The greater standard deviation at 323K compared to 443K shows that higher temperatures produce more consistent performance, which may benefit reliability-critical applications. The ARD% statistic shows that these combinations are more efficient at lower temperatures, demonstrating a complicated relationship between thermal circumstances, model selection, and performance metrics. Accurate IFT prediction is essential for understanding phase behavior in enhanced oil recovery, carbon dioxide sequestration, and reservoir simulation. By applying density gradient theory together with the Peng–Robinson EOS, the work provides a practical framework that can be used by researchers and engineers working with complex fluid interfaces. Thus, this work contributes useful theoretical insight by validating the DGT+PR-EOS model against simulation data across a wide range of temperatures and pressures. The analysis of statistical error metrics further strengthens understanding of the model’s performance and limitations.

6. Limitations and Future work

The findings presented do not guarantee that gradient theory is effective for calculating the interfacial tension of binary mixtures under reservoir conditions. There are three main limitations associated with its applications: (1) Some model require an adjustable coefficient for the influence parameter, which leads to a lack of predictive capability.

(2) The application of density gradient theory does not allow for the accurate determination of the density profile, thus impeding the evaluation of adsorption behavior.

(3) The binary interaction parameter of the equation of state, known as, is determined based on interfacial tensions. This paper presents significant theoretical predictions, outlining error measures and deviations based on simulated data. However, several aspects concerning experimental data, alternative simulation methods, diverse models, and various equations of state for the fluid mixtures examined require further investigation.

Disclaimer (Artificial Intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc) and NO text-to-image generators have been used during writing or editing of this manuscript.

Competing Interests

Authors have declared that no competing interests exist.

Acknowledgements

The authors acknowledge the University department of Physics, Ranchi University, Ranchi, Jharkhand, India for providing technical support during the analytical work. No external funding agency influenced the study design, data collection, analysis, interpretation, or manuscript preparation.

References:

- Ameli, F.,Hemmati - Sarapardeh, A., Schaffie, M., Husein, M. M. (2018). Modeling interfacial tension in N2/n alkane systems using corresponding state theory: Application to gas injection processes. Fuel 222 779–791. https://doi.org/10.1016/j.fuel.2018.02.067.

- Ashrafizadeh, S.N., Ameri, G.A. (2011). An investigation on the applicability of Parachor model for the prediction of MMP using five equations of state. Chem. Eng. Res. Des. 89, (6), 690–696. https//:doi:10.1016/j.cherd.2010.10.015.

- Amar,  M., N., Ghriga, M. A., Ben, S.E.A. (2021). Predicting Solubility of Nitrous Oxide in Ionic Liquids Using Machine Learning Techniques and Gene Expression Programming. J. Taiwan Inst. Chem. Eng., 128, 156– 168. https://doi.org/10.1016/j.jtice.2021.08.042.

- Amezquita, O.G.N., Enders, S., Jaeger, P.T., Eggers, R. (2010). Evaluation of Interfacial Tension by Image Processing of the Shape Drops J Supercritical Fluids. 55 (2) 724–734.  http://dx.doi.org/10.1016/j.supflu.2010.09.040.

- Blas, F. J., Del Rıo,E. M., De Miguel, E., and Jackson G. (2001). An examination of the vapour-liquid interface of associating fluids using a SAFT-DFT approach. Mol. Phys. 99(22), 1851– 1865. htpps//:doi.org/:10.1080/00268970110075176.

- Behvandi, R., Mirzaie, M. (2022). A novel correlation for modeling interfacial tension in binary mixtures of CH₄, CO₂, and N₂+ normal alkanes systems application to gas injection EOR process. Fuel. 1(325), 124622.  https://doi.org/10.1016/j.fuel.2022.124622 .

- Choudhary, N., NarayananNair, A.K., Che Ruslan,M.F.A., Sun,S. (2019). Bulk and interfacial properties of decane in the presence of carbon dioxide, methane, and their mixture. Sci. Rep. 9. (1), 19784. https://doi:10.1038/s41598-019-56378-y.

- Choudhary, N., Ruslan, M.F.A.C., Nair, A.K.N. & Sun, S. (2021). Bulk and Interfacial Properties of Alkanes in the Presence of Carbon Dioxide, Methane, and Their Mixture. Ind.Eng. Chem.Res. 60,729-738. https://doi.org/10.1021/acs.iecr.0c04843.

- Cachadiña, I., Hernández, A., Mulero, Á. (2024). A New General Correlation for the Influence Parameter in Density Gradient Theory and Peng–Robinson Equation of State for n-Alkanes. Molecules. 28, 29(23). 5643. https://doi.org/10.3390/molecules29235643.

- Cachadiña, G. I., Hernández, S.A.G., Mulero, D.Á.A., (2024). A new general correlation for the influence parameter in density gradient theory and Peng–Robinson equation of state for n-Alkanes.  29(23). 5643. https://doi.org/10.3390/molecules29235643.

- Cahn, J.W., Hilliard, J.E. (1958). Free energy of a non-uniform system “I” Interfacial free energy. J. Chem. Phys. 28, 258-267. https://doi.org/10.1063/1.1744102.

- Carey, B.S., Scriven, L.E., Davis, H.T. (1980). Semi-empirical theory of surface tension of binary systems. AIChE,  26, 5, 705-711. https://doi.org/10.1002/aic.690260502.

- Guggenheim, E.A., The Principle of Corresponding States. J.Chem. Phys. 13 (7), 253–261(1945).

  - https://doi.org/10.1063/1.1724033.

- Hemmati-Sarapardeh, A., Mohagheghian, E.,(2017). Modeling interfacial tension and minimum miscibility pressure in paraffin-nitrogen systems: Application to gas injection processes. Fuel, 205 80–89. https://doi.org/10.1016/j.fuel.2017.05.035.

- Jaeger, P.T., Alotaibi, M.B., Nasr-El-Din, H. A.(2010). Influence of compressed carbon dioxide on the capillarity of the gas− crude oil− reservoir water system.J. Chem. Eng.  Data 55, (11), 5246–5251. https://doi:10.1021/je100825b.

- Khaghani, A., Moghaddam, M.S., Kheshti, M.F. (2025). Investigation of the liquid-vapor equilibrium in the mixture of carbon dioxide and normal alkanes in binary systems using the Peng-Robinson equation of state modified with mixing rules. Chemical Thermodynamics and Thermal Analysis. 1(17), 100161.  https://doi.org/10.1016/j.ctta.2025.100161

- Li, X.-S., Liu, J. -M,. Fu, D.(2008). Investigation of Interfacial Tensions for Carbon Dioxide Aqueous Solutions by Perturbed-Chain Statistical Associating Fluid Theory Combined with Density Gradient Theory. Indust. & Engin.Chem Rese. 47, 8911–8917. https://doi.org/10.1021/ie800959h.

- Li N., Zhang, C., Ma, Q,L., Jiang L,Y.,  Xu, Y, X., Chen, C., Sun,Y. L., Yang Y. (2017). Interfacial tension measurement and calculation of (Carbon Dioxide+n-Alkane) binary mixtures. J. Chem. Eng. Data, 62(9), pp.2861-2871. https://doi.org/10.1021/acs.jced.7b00159

- Lafitte, T., Bessieres, D., Pineiro, M. M., and Daridon, Jean-L.(2006). Simultaneous estimation of phase behavior and second-derivative properties using the statistical associating fluid theory with variable range approach. J. of Chem. Phys, 124, (2), 4509–4524. https://doi.org/10.1063/1.2140276.

- Llovell, F., Mac Dowell, N., Blas, F. J., Galindo, A., & Jackson, G.(2012). Application of the SAFT-VR density functional theory to the prediction of the interfacial properties of mixtures of relevance to reservoir engineering. Fluid Phase Equilibria. 336, 137 – 150. https://doi.org/10.1016/j.fluid.2012.07.033.

- Mahdaviara, M., Amar, N., Sarapardeh, A. H., Dai,Z., Zhang C., Xiao, T., Zhang, X. (2021). Toward smart schemes for modeling CO2 solubility in crude oil: Application to carbon dioxide enhanced oil recovery, Fuel 285. 119147. https://doi.org/10.1016/j.fuel.2020.119147.

- Mahdaviara, M., Amar, M.N., Ostadhassan, M., Hemmati-Sarapardeh, A. (2022).  On the evaluation of the interfacial tension of immiscible binary systems of methane, carbon dioxide, and nitrogen-alkanes using robust data - driven approaches, Alexandria Engineering Journal. 61(12), 11601 - 11614. https://doi.org/10.1016/j.aej.2022.04.049.

- Menad, N.A., Noureddine, Z., Taiwan, J. (2019). An efficient methodology for multi-objective optimization of water alternating CO2 EOR process. Inst. Chem. Eng. 99, 154–165. https://doi.org/10.1016/j.jtice.2019.03.016.

- Mirzaie, M., Tatar, A. (2020). Modeling of interfacial tension in binary mixtures of CH4, CO2, and N2 - alkanes using gene expression programming and equation of state.J.Mol.Liq.320, 114454. https://doi.org/10.1016/j.molliq.2020.114454.

- Macleod, D. B. (1923). On a relation between surface tension and density. Trans. Faraday Soc. 19, (July), 38–41. https://doi.org/10.1021/ie50406a015.

- Mique, C., Mendiboure, B., Graciaa, A., Lachaise, J.(2008). Petroleum mixtures: An efficient predictive method for surface tension estimations at reservoir conditions. Fuel 87, 612-621. https://doi.org/10.1016/j.fuel.2007.05.049.

- Miqueu, C., Mendiboure, B., Graciaa, C., & Lachaise, J.(2004). Modelling of the surface tension of binary and ternary mixtures with the gradient theory of fluid interfaces. Fluid Phase Equilibria, 218, (2), 189–203. https://doi.org/10.1016/j.fluid.2003.12.008.

- Miguez,J.M.,Garrido,,J.,M.,Blas,F.J.,Segura,H.,Mejia,A.,Pineiro,M.M.(2014).Comprehensive characterization of interfacial behavior for the mixture CO2+H2O+CH4: Comparison between atomistic and coarse grained molecular simulation models and density gradient theory. The Journal of Physical Chemistry C, 18, 24504–24519. https://doi.org/10.1021/jp507107a.

- Mejıa, A., Segura, H., and Cartes, M. (2011). Measurement and theoretical prediction of the vapor–liquid equilibrium, densities and interfacial tensions of the system hexane + 2-methoxy- 2-methylbutane. Fluid Phase Equilibria, 308, (1-2), 15–24. https//doi.org/:10.1016/j.fluid.2011.06.007.

- Mejıa, A., Cartes, M., Segura, H., Mu¨ller, E.A., (2014), Use of Equations of State and Coarse Grained Simulations to Complement Experiments: Describing the Interfacial Properties of Carbon Dioxide plus Decane and Carbon Dioxide plus Eicosane Mixtures. J. Chem. Eng. Data 59, (10), 2928–2941. https//:doi:10.1021/JE5000764.

- NaitAmar, M., (2020).Modeling solubility of sulfur in pure hydrogen sulfide and sour gas mixtures using rigorous machine learning methods. Int. J. Hydrogen Energy 45 (58) 33274–33287. https://doi.org/10.1016/j.ijhydene.2020.09.145.

- NaitAmar, M., Ghriga, M. A., Ouaer, H. (2021). On the evaluation of solubility of hydrogen sulfide in ionic liquids using advanced committee machine intelligent systems. J. Taiwan Inst. Chem.Eng. 118, 159–168. https//:doi.org:/ 118 (2021) 159168.

- Pereira, L.M.C., Chapoy, A., Burgass, R., Oliveira, M.B., Coutinho, J.A.P., Tohidi, B. (2016). Study of the impact of high temperatures and pressures on the equilibrium densities and interfacial tension of the carbon dioxide/water system. J. Chem. Thermodyn. 93, 404–415.  https://doi.org/10.1016/j.jct.2015.05.005.

- Sugden, S. J.(1924). The variation of surface tension with temperature and some related functions. J. chem. Soc. Trans.125, (0), 32–41. https://doi.org/10.1039/CT9242500032.

- Sarapardeh, A. H., Varamesh, A., Husein, M.M., Karan, K. (2018). On the evaluation of the viscosity of nanofluid systems: Modeling and data assessment,"  Sustainable Energy Reviews, Elsevier, vol. 81(P1), pages 313-329. https://libkey.io/10.1016/j.rser.2017.07.049.

- Warsito, B., Santoso, R., Suparti., Yasin, H.(2018). Cascade Forward Neural Network for Time Series Prediction. J Phys. Conf. Ser.1025, 012097. https//: doi.org/10.1088/1742-6596/1025/1/012097.

- Zolghadr, A., Escrochi, M., Ayatollahi, S. (2013). Temperature and Composition Effect on CO2 Miscibility by Interfacial Tension Measurement. J. Chem. Eng.Data 58, (5), 1168–1175. https://doi.org/10.1021/je301283e.

- Zuo,Y.-X., Stenby, E. H. (1996). Calculation of Surface Tensions of Polar Mixtures with a Simplified Gradient Theory Model. J. Chem. Eng. Jpn. 29, (1), 159–165. https://doi.org/10.1252/jcej.29.159.