A Self-Learning Slime Mould Algorithm for Robust Multi-UAV 3D Path Planning in Complex Environments

Abstract

The autonomous navigation of UAV swarms in complex environments remains a critical bottleneck for their real-world, cost-effective deployment. This paper introduces a novel metaheuristic swarm intelligence framework, instantiated in the Self-Learning Slime Mould Algorithm (SLSMA), to address this challenge. The SLSMA empowers a UAV fleet to autonomously sense, adapt, and recover from planning failures through three core innovations: a situation-aware search strategy, a collective memory mechanism, and an adaptive recovery behavior. Rigorous evaluation on the CEC 2017 benchmark suite demonstrates that SLSMA achieves the topmost rank in the Friedman test and delivers statistically significant improvements (Wilcoxon rank-sum test, p < 0.05) over eight state-of-the-art metaheuristics. In complex 3D path planning scenarios, SLSMA generates complete collision-free trajectories where competing algorithms fail, achieving a 99.5% mission success rate and reducing the total cost by up to 18.7% compared to the best-performing variant. This work holds substantial importance for the scientific community by providing a foundational shift from static optimization to adaptive, metaheuristic reasoning in swarm robotics. It establishes a new paradigm for resilient autonomy, directly addressing the critical challenge of algorithmic fragility in uncertain environments. The proposed framework offers a replicable architecture that could influence the design of future intelligent systems beyond UAV navigation. The results establish SLSMA not merely as an optimizer, but as a robust solver for resilient autonomy, paving the way for the next generation of autonomous swarms capable of persistent operation in cluttered and dynamic environments.

Keywords: Robust path planning; metaheuristic optimization; swarm intelligence; slime mould algorithm, UAV trajectory optimization.

- Introduction

The deployment of unmanned aerial vehicles (UAVs) has catalyzed a paradigm shift across numerous fields, from disaster response and precision agriculture to infrastructure inspection and logistics, underpinned by their unparalleled operational versatility in complex, unstructured environments [1]–[5]. The coordination of multiple drones as a cohesive swarm further amplifies this potential, enabling superior efficacy in large-scale missions through distributed sensing and parallel task execution [6]. The linchpin of such cooperative autonomy, however, is effective and resilient flight trajectory planning [7]. The generation of optimal flight paths is not merely a matter of efficiency; it is critical for ensuring operational safety, mitigating inter-agent collision risks, and guaranteeing mission success in the face of stringent physical and environmental constraints [8], [9].

Path planning methodologies are historically bifurcated into traditional deterministic algorithms and modern meta-heuristics. Foundational algorithms such as A* and Dijkstra provide robust solutions for simplified, discrete search spaces [11]–[13]. However, the reality of multi-UAV trajectory optimization in three-dimensional space constitutes a profoundly complex challenge, characterized by high dimensionality, non-linearity, and multi-modality, with competing objectives such as fuel efficiency, flight time, and dynamic threat avoidance [14], [15]. The computational intractability faced by deterministic methods in these landscapes has precipitated a decisive pivot towards stochastic, population-based meta-heuristic algorithms, which are inherently better suited for navigating such complex, noisy search spaces [16].

Drawing inspiration from natural phenomena, meta-heuristics emulate biological or physical systems to conduct robust global searches, making them a predominant choice for UAV path planning [17]–[19]. This has led to the successful adaptation of a diverse array of algorithms, including enhanced Gray Wolf Optimization [21], hybrid Particle Swarm Optimization [22], ant colony systems [23], and multi-swarm fruit fly optimization [24], among others [25]–[30]. Within this pantheon, the Slime Mould Algorithm (SMA) has emerged as a notable contender, praised for its rapid convergence and structural elegance in solving complex optimization problems [31], [32]. Subsequent enhancements, such as the hybrid SMA (HSMA) for multi-objective planning [33], an improved SMA (ISMA) for collaborative scenarios [34], and a hybrid SMA with Cauchy mutation (AHCSMA) [35], have further cemented its relevance

Notwithstanding these advances, a critical gap persists. The standard SMA and its existing variants remain fundamentally reactive, lacking the cognitive capacity to dynamically assess and adapt their search strategy in response to the problem landscape. This manifests as a fragile balance between exploration and exploitation, a pronounced susceptibility to premature convergence, and an inability to autonomously recover from local optima entrapment—shortfalls that are particularly debilitating in the high-stakes context of UAV swarm navigation in GPS-denied and cluttered environments.

To bridge this gap, this paper introduces a Meta-Cognitive Swarm Intelligence Framework, instantiated through a novel Self-Learning Slime Mould Algorithm (SLSMA). We posit that true resilience in autonomous navigation requires algorithms that do not merely search, but can learn from the search process itself. The principal novelty of this work lies in the introduction of an algorithm architecture endowed with self-directed learning capabilities, founded on three core innovations:

- A Situation-Aware Search Strategy: We engineer a novel search mechanism that synergistically integrates the exploration prowess of a ranking-based differential evolution (Rank-DE) with the exploitation focus of the SMA. This allows the algorithm to dynamically and precisely modulate its behavior based on perceived search progression and stagnation [37].

- Autonomous Diversity Management: The framework incorporates a dynamic switching operator and an adaptive meta-perturbation technique, enabling it to autonomously maintain population diversity and execute targeted escapes from local optima, thereby accelerating convergence.

- A Formulation for Resilient Navigation: We rigorously formulate the multi-drone trajectory planning problem within a high-fidelity, complex 3D terrain model, defining it as a constrained optimization task with a holistic cost function that penalizes path length, altitude, maneuver complexity, collision risk and navigational uncertainty.

The performance of the proposed SLSMA is rigorously validated against the conventional SMA and other state-of-the-art metaheuristics using the Congress on Evolutionary Computation (CEC) 2017 benchmark suite and demanding multi-drone flight scenarios. The results establish our framework not as a mere improvement, but as a foundational step towards resilient autonomy for UAV swarms

The remainder of this paper is structured as follows: Section 2 details the problem formulation for multi-drone path planning. Section 3 reviews the foundational principles of the traditional SMA and Rank-DE. Section 4 provides a comprehensive mathematical exposition of the proposed SLSMA. Section 5 presents a thorough analysis of the experimental results, and Section 6 concludes the paper and outlines promising future research directions.

- Research Problem

  - Problem Scenario Settings

To mimic a real complex terrain environment, a complex terrain model which includes a base and an obstacle terrains is simulated. Adopted from Nikolos et al [38], the mathematical formulation for the base terrain model is presented in equation (1) [38].

Z1(x, y)=siny+a+b∙sinx+c∙cosd∙y2+x2+e∙cosy+f∙sinf∙y2+x2+g∙cosy                                                          (1)

where(x,y)denotes point coordinates on the horizontal plane. The rough terrain surfaces are simulated using the coefficients a, b, c, d, e, f, and g. These values are constant and can be used to model different types of surfaces. Following some common settings in existing literature [39], [40], this paper applies the following settings a=3π, b=110, c=910, d=12, e=12, f=12, 310.

Whereas the base terrain model focuses on the relief of the terrain, the obstacle terrain model simulates different obstacle forms. Adopted from Zhang et al [41], the mathematical formulation for the base terrain model is shown in equation (2).

Z2(x,y) =n=1NobsHn∙exp-x-xc,nxs,n2-y-yc,nys,n2                                                 (2)

whereNobsis the count of obstacle,Hnis the height of the n-th obstacle. The abscissa and ordinate of the centroid of the n-th obstacle are respectively represented by xc, n and yc, n. The gradients of the n-th obstacle along the X-axis and Y-axis are respectively denoted by xs, n and ys, n

A combination of different parameter settings in the aforementioned base and obstacle terrain model yields different complex terrain models. The complex terrain model could be expressed as in equation (3)

Z(x,y) = maxZ1(x,y),Z2(x,y)                                                                                         (3)

whereZ1(x,y)and Z2(x,y)denote the heights of the base terrain model and the obstacle terrain model at (x,y) respectively

2.2. Representation of Drone Flight Path

The path planning problem in this paper involves the mapping of the flight trajectory, which is composed of waypoint series, of multiple drones using a set of predefined routes. Suppose Mis the number of drones, and Nis the number of waypoints. Suppose that each drone, m ∈ 1, 2, .., M-1, M, at the same speed, flies from a starting point, Psm = xsm, ysm, zsm, and lands at a terminal point, Ptm=xtm,ytm,ztm. Then, equation (4) is a formulation of the flight trajectory of the drone as a discrete sequence of coordinate points.

WPm=P1m, P2m, P3m, ..., PN-1m, PNm, m=1, ..., M                                                            (4)

where Pimrepresents waypoint i of drone m, P1m=PsmandPNm=Ptm. Equation (5) is a mathematical formulation for each waypoint, Pim.

Pim=zim, yjm, zim, i=1, 2, ..., N-1, N                                                                             (5)

where xim, yimand zim represent the abscissa, ordinate, and vertical coordinates in 3D space.

The multi-drone path planning seeks to find a set of suitable flight paths, waypoints, WP=WP1, WP2, ..., WPm-1, WPMto minimize the total cost of each flight trajectory. Nonetheless, the connecting waypoints do not meet the requirements of drone navigation since the resultant trajectory is non-differentiable though continuous. A smoothing strategy can be used to ensure that the flight trajectories are continuously differentiable and kinematically feasible. Some of the commonly used methods include the B-spline curve, Dubins curve, and the Savitzky-Golay filter [42], [43], [44]. The B-spline curve method is widely used for smooth drone path planning due to its affine invariance, invariant geometry and preserved convexity [45]. Additionally, it provides a significant reduction in the complexity of the smoothing process by keeping the degree of the polynomial unchanged even following addition of more control points. Equation (6) is a mathematical definition for the B-spline curve.

P(t)=i=0Nctrldi∙Ni,k(t)                                                                                                                (6)

where the control points are represented by di, i=1,2, ..., Nctrl-1, Nctrl. The order of the B-spline curves’ segments is k, whileNi,k(t)is the k-order normalized blending function described by t0≤t1≤...tn+k. Equation (7) defines the Ni,k(t) function.

Nl,1(t)=1, if tl≤t≤tl+10,          otherwise                                                           Nl,k(t)=t-tltl+k-1-tl×Nl,k-1(t)+tl+k-ttl+k-tl+1×Nl+1, k-1(t)                                    (7)

In this paper, the cubic B-spline curves are utilized to make the flight trajectories of the drones smooth. According to Hasan et al [46], the cubic B-spline curves are highly precise in smoothing complex geometries.

2.3. Cost Function

The path planning cost function is a mathematical model that accounts for the various factors that affect the flight path. It also accounts for the constraints of the trajectories. This section presents the five key factors that are incorporated into the cost function.

2.3.1. Cost of Drone Flight Distance

The flight time of drones is an important factor. A less flight time allows them to reach their intended location more quickly and consume less energy. To ensure a less flight time is equivalent to reducing the distance flown if the drone flies at a steady speed. Equation (8) is a set of connected waypoints, representing drone flight distance.

Dm=  P1mP2m, ..., PN-1mPNm                                                                                     (8)

where Dmrepresents the m-th drone’s set of flight segments. PiPjrepresents the Euclidean distances from waypoint Pito waypoint Pj. Equation (9) is the mathematical formula for

calculating the cost of flight distance for drone m.

Cdistm=n=1N-1PnPn+1P1PN                                                                                                            (9)

2.3.2. Cost of Drone Flight Height

It is also important to optimize the altitude variations to minimize energy consumption of drones during flight. The drones must operate within some defined height ranges. Equations (10) and (11) are the mathematical formula for calculating the cost of flight height for drone m.

Cheightm=i=2N-1Qheight, im                                                                                                             (10)

Qheight, im=qheight, if zim<zlb or zim>zub0,                                   otherwise                                                                    (11)

For drone m at waypoint i with a height of zim, the costs of flight height is denoted by Qheight, im. For all the drones, the lower and upper flight height boundaries are represented by zlb and zub respectively. The flight height has a cost coefficient of qheight.

2.3.3. Cost of Drone Flight Turning

Drones can only turn within a specified angle range in both vertical and horizontal directions. Any maneuver that exceeds these limits is considered infeasible, which is why restricting the pitch and yaw angles during flight is necessary. Equations (12), (13), and (14) are the mathematical formulae for computing the cost of flight turning for drone m.

Cturnm=i=2N-1Qyaw, im+1=2N-1Qpitch, im                                                                                         (12)

Qyaw, im=qyaw,        otherwise0, if 0≤αim≤αmax                                                                                            (13)

Qpitch, im=qpitch,                  otherwise0, if 0≤βim≤βmax                                                                                           (14)

For drone m, the cost of yaw angle i is represented by Qyaw, im. Similarly, the cost of pitch angle i is represented by Qpitch, im. αim represents yaw angle i, βimrepresents pitch angle i. The costs of flight turning have coefficients of qyaw and qpitch. Adopted from Xiong et al [47] and Zhang et al [48], equations (15) and (16) are the mathematical formulae for computing the yaw and pitch angles respectively.

αim=arccosxim-xi-1mxi+1m-xim+yim-yi-1myi+1m-yimxim-xi-1m2+yim-yi-1m2∙xi+1m-xim2+yi+1m-yim2 (15)

βlm=arctanZlm-Zl-1mxlm-xl-1m2+ylm-yl-1m2                                                             (16)

2.3.4. Cost of Collision Between Drones and other Obstacles

It is imperative to ensure the safety of drones during a mission. Collisions with obstacles such as rocks, skyscrapers, and mountains could be costly. The cost of drone m colliding with some obstacles is computed using equations (17) and (18).

Ccollis_obsm=i=1N-1Qcollis_obs, im                                                                                                    (17)

Qcollis_obs, im=qcollis_obs, if (xim, yim, zim) is in obstacles0,                                     otherwise                                                             (18)

For drone m, at waypoint i, the cost of colliding with an obstacle is denoted by Qcollis_obs, im with a coefficient of qcollis_obs.

2.3.5 Cost of Collision Between Drones

When multiple drones fly at the same time, it is important to ensure no drone-drone collisions too. These mishaps can occur when the flight paths overlap or the distances between them is poor, which can lead to significant system malfunctions. Equations (19) and (20) are the formulae for computing the cost of collision between drone m and other drones.

Ccollis_dronem=i=2N-1n=1, n≠mMQcollis_drone, im, n                                                                             (19)

Qcollis_drone, im, n =0, if PimPin<dminqcollis_drone,       otherwise                                                                            (20)

For any two drones, m and n, at waypoint i, the cost of collision is denoted by Qcollis_drone, im, n. The Euclidean distance between them is denoted by PimPinwith dmin as the minimum safe distance and qcollis_drone as the cost coefficient.

2.3.6. Total Cost

The five drone flight constraints are written in as cost functions and combined to form a single mathematical function of costs to compute the total cost. Equation (21) is the mathematical model for the total cost for all the drone flights.

Ctotal =m=1M(ω1∙Cdistancem + ω2∙Cheightm + ω3∙Cturnm + ω4∙Ccollis_obsm + ω5∙Ccollis_dronem)                                                                                                (21)

where ωi, i =1,2, ..., 5, are the weight coefficients for the respective cost functions.

3. Background of Methods

3.1. Traditional SMA

The SMA is inspired by the patterns of slime molds that are found in nature. It takes

advantage of adaptive weights to model both the negative and positive feedback that occurs during the foraging process. It also abstracts the changes that happen in the foraging environment [49].

3.1.1. Navigation Toward Food

The individual slime moulds in slime mold colonies rely on the level of odour in the air to navigate toward food. Assume that the slime mould has a population size of NP, equation (22) is the mathematical formula for updating position of a slime mould.

X(t+1)=Xbest(t)+vb∙(W∙XA(t)-XB(t)), if rand1<pvc∙X(t),                                                 if rand1≥p                                 (22)

where the current iteration’s index is denoted by t, the current individual iteration position is denoted by X(t) and the next is represented by X(t+1), the optimal position is represented by Xbest(t), XA(t) and XB(t) denote two individual slime moulds randomly sampled from the population. rand1is a number sampled randomly from the Uniform distribution [0, 1]. p denotes a conrol parameter for slime mould position updating patterns. Equation (23) is a mathematical formula for p.

p = tanhFitXi-Fitbest, i=1, 2, ..., NP-1, NP                                                 (23)

where the fitness of slime mould i is denoted by FitXi, the overall best fitness is denoted by Fitbest, the slime moulds' selection behavior when approaching food are simulated by parameters vband vc, with the former denoting the extent of slime mould exploration and bounded by -a, a, and the latter denoting individual slime mould’s exploitation of historical information, which declines from 1 to 0. The mathematical formula forαis expressed in equation (24).

α=arctanh-ttmax+1                                                                                               (24)

where the maximum number of iterations is denoted by tmax.

Equation (25) is the mathematical formula for computing the weight of a slime mould, W.

W(FitIndex(i))=1+rand1∙logBF-Fit(Xi)BF-WF+1, condition1-rand1∙logBF-Fit(Xi)BF-WF+1,       others                     (25)

FitIndex = sort(Fit)                                                                                                           (26)

where the sequence determined after assessing the fitness level of the slime molds is represented by FitIndex. The sorting process in minimization is conducted in an ascending order. Whether or not Fit(Xi)ranks first part of the population is denoted by condition. Given the current iteration, whereas BFdenotes the best fitness, the worst fitness is represented by WF.

3.1.2. Food Wrapping

The tissue structures of slime mold veins are contracted to wrap food. The concentration of food particles in contact with these structures influences the waves produced by the mold's biological oscillator. A higher concentration leads to more powerful waves, which can increase the speed of the movement of cytoplasm and lead to thicker veins. Slime molds tend to focus on finding an area with high food concentrations to exploit. When the food concentration falls, they search for other sources of food. Equation (27) is the position updating mathematical formula for a slime mould.

X(t+1)=rand2∙(UB-LB)+LB,                    rand2<zXbest(t)+vb∙(W∙XA(t)-XB(t)), rand1<pvc∙X(t),                                                  rand1≥p                                     (27)

where the lower boundary of the search space is denoted by LB, the upper boundary of the search space is denoted by UB, rand2 denotes a number sampled randomly from the Uniform distribution 0,1, z denotes a percentage of slime moulds engaged in random exploration, empirically fixed at 0.03.

3.1.3. Oscillation

Slime moulds use their bio-oscillators to adjust the flow rate of fluids in their veins based on the concentration of food. This helps them find the ideal food locations by altering the veins' width. The parameters W, vb and vc work together to control the oscillation mechanism in SMA, which aim to balance the exploitation and exploration.

3.2. Ranking-based Differential Evolution (Rank-DE)

As an alternative to the DE, the rank-DE is widely applied to solve global optimization problems [50]. It is based on the concept that the more desirable traits of parent organisms are more likely to result in their offspring [37]. The following subsections present a detailed description of the rank-DE.

3.2.1. Assignment of Rankings

The rank-DE system takes into account the entropy characteristic of high-quality individuals and ranks them according to their fitness values. For instance, the best individuals receive a high ranking while the worst receive a low one. Equation (28) is a mathematical formula for assigning the ranking of an individual.

Rj =NP-j, j =1, 2, ..., NP-1, NP                                                                              (28)

where j denotes the ranking index following individuals sorting.

3.2.2. Probability of Selection

The selection probability is calculated by considering the individual’s rank. Equation (29) is a mathematical formula for computing the selection probability.

Pj=RjNP, j=1, 2, ..., NP-1, NP                                                                                      (29)

where the probability of selecting an individual ranked in position j is represented by Pj.

3.2.3. Selection of Vector

The selection probability of a mutation in the rank-DE determines its operator. This paper uses the DE-Rand1 strategy. The definition for the “DE-Rand1” mutation strategy is

presented in Equation (30).

Vi=Xr1+F∙(Xr2-Xr3)                                                                                                     (30)

where ri, i = 1, 2, and 3 are mutually exclusive integers sampled randomly from 1, NP. F is a scaling factor sampled from [0, 2]. The current position of individual iand the post-mutation position are denoted by Xi and Vi respectively.

The rank-DE algorithm applied in this paper chooses the some vectors based on their probability. The first and base is Xr1, while the second and terminal is Xr2. To enhance the algorithm's randomization, Xr3is randomly sampled from the population. Algorithm I shows the pseudo-code for the rank-DE vector selection, which uses the mutation strategy of “DE-Rand1”.

Algorithm I: Rank-DE Vector Selection with Differential Evolution and Rand1 (DE-Rand1)

Inputs: Population Size(NP), Selection Probability (P), Target Individual Index (i)

Outputs: Selected Individual Indices, r1, r2, and r3

- r1← select randomly from 1, NP; - while rand(0, 1)>Pr1 or r1==i do - r1← select randomly from 1, NP; - end while - r2← select randomly from 1, NP; - while rand(0, 1)>Pr2 or r1==i or r2==i do - r2← select randomly from 1, NP; - end while - r3← select randomly from 1, NP; - while r3==r2 or r3==r1 or r3==ri do - r3← select randomly from 1, NP; - end while

Thereafter, Xi and Vi are combined using a crossover operator as shown in equations (31) and (32).

Ui=U1, U2, ..., UDim-1, UDimT                                                                                         (31)

Ui, Dim=Vi, Dim, if rand<Cr or Drand =DimXi, Dim,                                       otherwise                                                          (32)

where the dimension of the problem is denoted by Dim, Drand represents an integer randomly sampled from 1, Dim,randdenotes a number sampled randomly from [0, 1], and the crossover parameter is denoted by Cr.

Then, a greedy selection criterion is applied to keep relatively better individuals. Suppose the new individual (Ui) has a better fitness than that of the current individual (Xi), then the former is kept. Else, the latter is maintained for the following generation.

4. The Proposed Algorithm, SLSMA

The SLSMA is developed and proposed to tackle the limitations of the SMA, particularly for planning multi-drone flight paths. The following three points constitute the new and key improvements incorporated in SMA to better its performance.

- A new mechanism for searching the search space. The proposed SLSMA combines the Rank-DE’s exploration capabilities and a crossover and mutation procedure to address the imbalance in the exploitation and exploration process.

2. A dynamic switch operator. The proposed SLSMA integrates an adaptive probability that changes throughout the iteration process, which improves the optimal solution's quality and prevent it from prematurely converging.

3. A dynamic perturbation strategy. The proposed SLSMA conducts an update of the positions of individuals who are performing poorly when the population encounters a period of convergence stagnation.

Figure 1 depicts the conceptual framework of the proposed SLSMA for solving optimization problems involving multi-drone path planning.

Figure 1: A conceptual framework of the SLSMA.

4.1. A New Search Scheme

The trade-off between exploration and exploitation is a significant challenge for meta-heuristics during the search process [51]. During the exploration phase, the individuals are more likely to look for solutions that are globally ideal. Nevertheless, over exploration may lead to algorithm convergence issues. During the exploitation phase, on one hand, the individuals tend to search in small steps near the ideal area, which leads to faster population convergence. On the other hand, excessive exploitation can result in premature convergence since the algorithm gets trapped in local optima. In equation (27), for rand2<z, the algorithm creates positions randomly to simulate the exploration behavior. For rand2≥z, the algorithm adjusts the value of slime molds’ exploitation range based on vbandvc. This approach limits the exploration potential of slime molds. In many experiments, it has been observed that the value of optima is typically set to 0.03, which makes it hard for the algorithm to escape local optima. This is especially true when the population gets homogenized.

Therefore, a new mechanism that replaces the exploitation phase in the SMA algorithm

is proposed by utilizing the rank-DE mutation operation. The algorithm’s position update mode is then changed to accommodate Cr, which is a crossover operator. This combination of the two functions increases the likelihood of finding solutions in the global search.. Equation (33) is the mathematical formula for updating the positions of SLSMA.

Xnew(t+1)=rand2∙(UB-LB)+LB,                                 rand2<zXbest(t)+vb∙(W∙XA(t)-XB(t)),            rand1<CrXmuta(t),                                                             rand1≥Cr                  (33)

where the new resultant updated position is denoted by Xnew(t), and the post mutation individual position is denoted by Xmuta(t), as expressed by equation (34).

Xmuta(t)=Xr1(t)+2∙rand3∙cb∙Xr2(t)-Xr3(t)                                                  (34)

The scaling factor Fis substituted with a number sampled randomly from 0, 2a in order to prevent new parameters from being introduced in the algorithm. In algorithm I, r1, r2, and r3 are chosen.

4.2. A Dynamic Switch Operator

The complexity and non-linear nature of the optimization processes for real-world problems significantly hinder the performance of fixed-search algorithms. In SMA, the parameter z determines the balance between local exploitation and global exploration. As the population grows and the differences decrease, the value ofzchallenges better solution exploration capacity. Hence, the SLSMA algorithm converts the parameter z from a fixed to a non-linear one. This allows the algorithm to perform robust global searches. Equation (35) is the mathematical formula for calculating the dynamic switch operator.

z = log101+0.8×expt+1tmax                                                                                  (35)

The Figure 2 visualizes the dynamic switch operator against iterations. In the early iterations,zis relatively small, which encourages individual slime moulds to explore promising regions. The value of zthen rises as the number of iterations goes up, causing the population to move beyond the current to search for other promising alternatives.

Figure 2: Dynamic switch operator.

4.3. A Dynamic Perturbation Technique

The position of the population tends to stabilize when it stagnates for many iterations. This leads to the development of algorithms that are unable to find solutions that are better than their current state. By modifying the population structure, algorithms can search for new solutions.

The dynamic perturbation mechanism searches the local optima space to bypass any potential local optima through introduction of minor variations following algorithm stagnation. The SLSMA algorithm's ability to determine if a perturbation strategy is needed is a function of the stagnation counter, which is incremented if the difference of the values of the optimal fitness of the current and previous iterations is less than ε=1.0E-03, the perturbation operator. Otherwise, the stagnation counter is set to zero. Where the predetermined index of perturbation, Nstag=15, is reached, the algorithm is deemed to have stagnated, which calls for the implementation of the perturbation mechanism.

The dynamic perturbation strategy seeks to improve the population by substituting with new individuals the individuals with poor performance. Given a sign of stagnation, the individuals with worst performance, ξ, are removed, following which new individuals, ξ, are added to the population. The mathematical formula for the dynamic perturbation strategy is shown in equation (36).

Xpert(t)=λ∙Xbest(t)+(1-λ)∙Xrand(t)                                                                       (36)

Xrand(t)=μ(UB-LB)+LB                                                                                               (37)

where both λ and µ denote numbers randomly sampled from the Uniform distribution, [0, 1]. Given all iterations, the optimal position is denoted by Xbest(t). Algorithm II represents the pseudo-code of the SLSMA.

Algorithm II: Pseudo-Code of Proposed Algorithm, SLSMA

Inputs: Population Size (NP), Dimension (Dim), Lower Boundary (LB), Upper Boundary (UB), Maximum Number of Iterations (tmax), Crossover Rate (Cr), and Perturbation Rate (ξ ). Outputs: Optimal Position (Xoptimal), Optimal Fitness (Fitoptimal).

- Initialize the positions of population X and parameters; - P← compute using equation (29); - Nstag_count=0; t=1; - while t≤tmax do - z← compute using equation (35); - a← compute using equation (24); - for i=1:NP do - Fit(Xi)← compute the fitness value of each individual Xi; - end for - W← compute using equation (25); - Xbest ,Fitbest← evaluate the global optimal position and global optimal fitness; - for i=1:NP do - if rand2<z then - Xi, new← update the new position using equation (33)(1); - else - for d=1:Dim do - if rand1<Cr or Drand=Dim then - Xi, d, new← update the new position using equation (33)(2); - else - r1, r2, and r3← select using Algorithm I; - Xi, d, new← update the new position using equation (33)(3); - end if - end for - end if - if Fiti, new<Fit(Xi) then - replace Xi with Xi, new; - end if - end for - Xbest ,Fitbest← update the global optimal position and global optimal fitness; - if Fitbest, t-Fitbest, t-1<ε then - Nstag_count=Nstag_count+1; - else - Nstag_count=0; - end if - if Nstag_count≥Nstag then - Xpert← implement self-adaptive perturbation strategy using equation (36); - end if - end while

- Results of Simulations and Analyses

This section compares the proposed SLSMA and some seven well-known meta heuristic algorithms on the Congress on Evolutionary Computation 2017 benchmark test suite. In addition, experiments were performed on the path planning problems with three well-established standard SMA variants and the SLSMA. All the simulations, experiments, and analyses conducted in this paper were performed on a Windows 10 operating system with 8GB of RAM, Intel(R) Core (TM) i5-8250U CPU, and a main frequency of 1.80 GHz. The implementation and visualization of the algorithms were done in MATLAB R2024a.

  - Experiments on the Congress of Evolutionary Computation 2017 Test Suite

The Congress on Evolutionary Computation 2017 benchmark test suite, which is composed of four categories, is highly recognized and widely used to evaluate single objective optimization problems involving a real parameter [52]. In all, the four categories, uni-modal functions (F1,F3), simple multi-modal functions (F4-F10), hybrid functions F11-F20, and composition functions F21-F30 consist of 29 functions defined over the interval [-100, 100]. Tables 1-4 show the details of the 29 functions.

Table 1: Congress on Evolutionary Computation 2017 Uni-modal Functions

| Number | Functions | Optimal Fitness Value |
| --- | --- | --- |
| 1 | F1(x): Shifted and rotated Bent Cigar function | 100 |
| 2 | F3(x): Shifted and rotated Zakharov function | 200 |

Table 2: Congress on Evolutionary Computation 2017 Simple Multi-modal Functions

| Number | Functions | Optimal Fitness Value |
| --- | --- | --- |
| 1 | F4(x): Shifted and rotated Rosenbrock’s function | 300 |
| 2 | F5(x): Shifted and rotated Rastrigin’s function | 400 |
| 3 | F6(x): Shifted and rotated expanded Scaffer’s F7 function | 500 |
| 4 | F7(x): Shifted and rotated Lunacek Bi-Rastrigin function | 600 |
| 5 | F8(x): Shifted and rotated non-continuous Rastrigin’s function | 700 |
| 6 | F9(x): Shifted and rotated Levy function | 800 |
| 7 | F10(x): Shifted and rotated Schwefel’s function | 900 |

Table 3: Congress on Evolutionary Computation 2017 Hybrid Functions

| Number | Functions | Optimal Fitness Value |
| --- | --- | --- |
| 1 | F11(x): Hybrid function 1 (N=3) | 1000 |
| 2 | F12(x): Hybrid function 2 (N=3) | 1100 |
| 3 | F13(x): Hybrid function 3 (N=3) | 1200 |
| 4 | F14(x): Hybrid function 4 (N=4) | 1300 |
| 5 | F15(x): Hybrid function 5 (N=4) | 1400 |
| 6 | F16(x): Hybrid function 6 (N=4) | 1500 |
| 7 | F17(x): Hybrid function 7 (N=5) | 1600 |
| 8 | F18(x): Hybrid function 8 (N=5) | 1700 |
| 9 | F19(x): Hybrid function 9 (N=5) | 1800 |
| 10 | F20(x): Hybrid function 10 (N=6) | 1900 |

Table 4: Congress on Evolutionary Computation 2017 Composite Functions

| Number | Functions | Optimal Fitness Value |
| --- | --- | --- |
| 1 | F21(x): Composition function 1 (N=3) | 2000 |
| 2 | F22(x): Composition function 2 (N=3) | 2100 |
| 3 | F23(x): Composition function 3 (N=4) | 2200 |
| 4 | F24(x): Composition function 4 (N=4) | 2300 |
| 5 | F25(x): Composition function 5 (N=5) | 2400 |
| 6 | F26(x) = Composition function 6 (N=5) | 2500 |
| 7 | F27(x): Composition function 7 (N=6) | 2600 |
| 8 | F28(x) = Composition function 8 (N=6) | 2700 |
| 9 | F29(x) = Composition function 9 (N=3) | 2800 |
| 10 | F30(x) = Composition function 10 (N=3) | 2900 |

    - Analysis of Algorithmic Effectiveness

The proposed algorithm's effectiveness was evaluated by comparing it with DE [53], GWO [18], PSO [54], SCA [57], SMA [49], SSA [56], and WOA [55] using the Congress on Evolutionary Computation 2017 benchmark test suite. The algorithms were assessed under same and fair conditions. The population size and the problem dimension of the algorithms were set to 30. The algorithms were run independently to minimize the effects of randomness on the experiments, with 1000 maximum number of iterations.

The three evaluation metrics used in this analysis were Average Fitness AVG, Best Fitness BEST, and Standard Deviation STD. They indicate the optimal algorithm for achieving the best results while ensuring the consistency of the approach in several tests. The stability of the algorithm is also taken into account to see if it performs well.

The performances of algorithms were evaluated using the Wilcoxon rank-sum test and the Friedman test. The latter examines the statistical significance of the average ranking differences [58]. The algorithm which ranked first was considered the best. The former test ensures that the best algorithmic performance is not random, but significant statistically [59].

Table 5 details the parameter settings for the algorithms under study. The SLSMA parameters were set through multiple experiments. The other algorithms' parameter settings follow the recommendations found in their original papers.

Table 5: Algorithm Parameter Settings

| Number | Algorithm | Parameter settings |
| --- | --- | --- |
| 1 | DE | F = 0.5, CR = 0.5 |
| 2 | GWO | a is decreased linearly from 2 to 0 |
| 3 | PSO | c1=2, c2=2, ω=0.4 |
| 4 | SCA | a = 2 |
| 5 | SLSMA | Cr = 0.5, ζ = 0.1 |
| 6 | SMA | z=0.03 |
| 7 | SSA | c2∈0,1, c3∈0,1 |
| 8 | WOA | a is decreased linearly from 2 to 0, b = 1 |

The results of the experiments are presented in Tables 6-9. The best results for every function are emboldened. Table 10 presents the Friedman test results. The Wilcoxon’s rank-sum test results are shown in Table 11.

Table 6 shows the optimal solutions for each uni-modal function, which can be used to analyze the algorithms' exploitation capabilities. In terms of standard deviation and average fitness, the SLSMA is ranked second in the category for F1 and F3. This shows that the the SLSMA has the necessary exploitation capabilities to perform well on these functions. Moreover, the novel search algorithm in the SLSMA allows it to find better solutions than the SMA.

Table 7 displays the results on the various functions of the simple multi-modal category, F4-F10. The results of the experiments reveal that the SLSMA performs best on F5-F7 and F10. This is primarily due to the adoption of crossover operators and mutation. The DE performs better than the SLSMA on F4and F9. Nevertheless, the latter still has an overall better exploration capabilities.

The hybrid functions, F11-F20, combine simple multi-modal and uni-modal functions. They are more challenging than the previous two groups. They have a better suitability for algorithmic exploration-exploitation balance testing. Table 8 shows that the SLSMA performs better than the other algorithms on F17, recording overall best fitness on F11-F13, and F15. The SLSMA ranks third in the hybrid functions group, but it is able to balance exploration and exploitation better than the other algorithm. This is evidenced by its attainment of best fitness in most functions.

The composition functions, F21-F30, constitute a combination of the uni-modal, the simple multi-modal, and the hybrid functions in a nonlinear fashion. They are the most challenging functions in the test suite. They are also the most ideal for evaluating the overall algorithmic effectiveness. Table 9 shows the results on F21-F30 to evaluate the algorithmic effectiveness of the SLSMA. It shows that the SLSMA has the best performance on F21, F23, F25, F26, and F29. The SLSMA again has the optimal average fitness on F24, F27, and F30, demonstrating a solid evidence for the SLSMA’s effectiveness in the search for optimal fitness in composition functions. This result also highlights the pivotal roles of the new search mechanism, the dynamic switch operator, and the dynamic perturbation technique in the SLSMA’s capacity to escape local optima and premature convergence.

The results of the Friedman test, conducted at a 5% significance level, confirm that the performance differences among the algorithms are statistically significant. The SLSMA achieved the highest average rank, indicating it is the most likely to perform optimally across all evaluated functions, followed by DE and SMA (Table 11). This superior ranking, derived from a systematic optimization process, demonstrates that the SLSMA's performance is robust and not attributable to random chance.

    - Analysis of Algorithmic Convergence Behaviour

The five best-performing algorithms from the analysis of the algorithmic effectiveness were further examined for their convergence performance on the F1-F10functions. The respective algorithmic convergence curves are shown in Figure 3. The remainder of the algorithmic convergence performance curves of the algorithm on F11-F30 are presented in Appendix I.

Figure 3: Convergence curves of five algorithms on the benchmark functions, F1 to F10

| Function | Metric | DE | GWO | PSO | SCA | SLSMA | SMA | SSA | WOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F1 | Avg | 8.3174E+06 | 3.9045E+08 | 3.1114E+10 | 4.5858E+08 | 5.2162E+04 | 4.9315E+04 | 3.6401E+08 | 5.3789E+09 |
|  | Best | 2.9284E+06 | 2.7352E+07 | 4.7688E+10 | 4.4536E+08 | 7.8395E+02 | 5.7042E+04 | 3.3148E+06 | 3.1317E+09 |
|  | Std | 1.1897E+07 | 7.6667E+08 | 3.1622E+10 | 5.1450E+08 | 1.2405E+05 | 5.5517E+04 | 2.5827E+09 | 6.4770E+09 |
| F3 | Avg | 1.0575E+03 | 2.0234E+03 | 4.7688E+04 | 4.9722E+03 | 1.2812E+02 | 1.0981E+02 | 9.3952E+03 | 1.9523E+04 |
|  | Best | 4.7180E+02 | 3.2334E+02 | 4.9518E+04 | 7.6667E+03 | 2.0336E+02 | 2.0336E+02 | 1.3625E+03 | 2.1861E+04 |
|  | Std | 1.3523E+03 | 3.5791E+03 | 4.9111E+04 | 5.1450E+03 | 1.3320E+02 | 9.4969E+01 | 2.2370E+04 | 2.0336E+04 |

Table 6: Results of Experiments on Congress on Evolutionary Computation Uni-modal Functions

Table 7: Results of Experiments on Congress on Evolutionary Computation Simple Multi-modal Functions

| Function | Metric | DE | GWO | PSO | SCA | SLSMA | SMA | SSA | WOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg | 1.88E+02 | 2.63E+02 | 7.39E+03 | 2.84E+02 | 1.97E+02 | 1.91E+02 | 1.23E+03 | 6.29E+02 |
| F4 | Best | 3.53E+02 | 3.72E+02 | 9.75E+03 | 4.89E+02 | 3.41E+02 | 3.53E+02 | 4.79E+02 | 7.17E+02 |
|  | Std | 1.73E+02 | 2.64E+02 | 7.67E+03 | 2.69E+02 | 1.85E+02 | 1.76E+02 | 2.84E+03 | 6.91E+02 |
|  | Avg | 3.76E+02 | 9.39E+02 | 3.92E+04 | 9.32E+02 | 2.78E+02 | 3.14E+02 | 6.12E+03 | 1.32E+04 |
| F5 | Best | 7.06E+02 | 7.88E+02 | 4.97E+04 | 1.19E+03 | 4.87E+02 | 5.51E+02 | 6.91E+03 | 1.35E+04 |
|  | Std | 3.61E+02 | 1.34E+03 | 4.01E+04 | 9.64E+02 | 2.64E+02 | 3.00E+02 | 6.56E+03 | 1.40E+04 |
|  | Avg* | 2.73E+02 | 2.73E+02 | 2.73E+02 | 2.73E+02 | 2.73E+02 | 2.73E+02 | 2.73E+02 | 2.73E+02 |
| F6 | Best* | 5.31E+02 | 5.31E+02 | 5.31E+02 | 5.31E+02 | 5.31E+02 | 5.31E+02 | 5.31E+02 | 5.31E+02 |
|  | Std* | 2.58E+02 | 2.58E+02 | 2.58E+02 | 2.58E+02 | 2.58E+02 | 2.58E+02 | 2.58E+02 | 2.58E+02 |
|  | Avg | 7.09E+02 | 2.03E+04 | 1.30E+06 | 1.93E+04 | 4.17E+02 | 4.93E+02 | 3.34E+04 | 2.14E+05 |
| F7 | Best | 1.06E+03 | 4.62E+03 | 1.69E+06 | 2.58E+04 | 7.39E+02 | 8.88E+02 | 1.40E+04 | 1.42E+05 |
|  | Std | 7.35E+02 | 4.48E+04 | 1.32E+06 | 2.04E+04 | 4.03E+02 | 4.79E+02 | 1.56E+05 | 2.42E+05 |
|  | Avg | 6.22E+02 | 4.95E+02 | 8.02E+02 | 5.37E+02 | 4.82E+02 | 5.35E+02 | 8.22E+02 | 8.02E+02 |
| F8 | Best | 1.13E+03 | 8.39E+02 | 1.43E+03 | 9.80E+02 | 8.60E+02 | 8.88E+02 | 1.43E+03 | 1.25E+03 |
|  | Std | 6.07E+02 | 4.88E+02 | 7.88E+02 | 5.23E+02 | 4.68E+02 | 5.23E+02 | 8.08E+02 | 7.92E+02 |
|  | Avg* | 4.33E+02 | 4.33E+02 | 4.57E+02 | 4.33E+02 | 4.34E+02 | 4.37E+02 | 4.39E+02 | 4.44E+02 |
| F9 | Best* | 8.49E+02 | 8.49E+02 | 8.79E+02 | 8.51E+02 | 8.49E+02 | 8.51E+02 | 8.59E+02 | 8.60E+02 |
|  | Std | 4.17E+02 | 4.18E+02 | 4.41E+02 | 4.18E+02 | 4.19E+02 | 4.22E+02 | 4.24E+02 | 4.29E+02 |
|  | Avg | 4.17E+03 | 2.82E+03 | 4.52E+03 | 3.28E+03 | 2.18E+03 | 2.34E+03 | 4.22E+03 | 3.53E+03 |
| F10 | Best | 5.94E+03 | 3.08E+03 | 8.31E+03 | 5.79E+03 | 2.97E+03 | 3.54E+03 | 4.47E+03 | 4.84E+03 |
|  | Std | 4.20E+03 | 3.20E+03 | 4.51E+03 | 3.28E+03 | 2.22E+03 | 2.37E+03 | 4.31E+03 | 3.63E+03 |

Table 8: Results of Experiments on Congress on Evolutionary Computation Hybrid Functions

| Function | Metric | DE | GWO | PSO | SCA | SLSMA | SMA | SSA | WOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg | 2.97E+04 | 5.97E+04 | 7.83E+08 | 5.97E+04 | 5.36E+04 | 2.63E+04 | 5.80E+07 | 4.13E+06 |
| F11 | Best | 2.40E+04 | 6.54E+04 | 4.56E+08 | 5.97E+04 | 1.37E+04 | 1.76E+04 | 5.52E+04 | 6.75E+04 |
|  | Std | 3.96E+04 | 6.53E+04 | 9.28E+08 | 6.39E+04 | 7.35E+04 | 3.00E+04 | 2.75E+08 | 3.12E+07 |
|  | Avg | 4.98E+06 | 1.25E+07 | 4.02E+09 | 1.90E+07 | 2.26E+06 | 3.74E+05 | 7.77E+08 | 2.05E+07 |
| F12 | Best | 3.51E+06 | 4.06E+06 | 4.73E+09 | 2.10E+07 | 2.10E+05 | 5.91E+04 | 6.16E+05 | 1.38E+06 |
|  | Std | 7.64E+06 | 1.73E+07 | 4.17E+09 | 2.01E+07 | 3.94E+06 | 5.43E+05 | 1.83E+09 | 3.41E+07 |
|  | Avg | 1.15E+05 | 4.24E+06 | 4.49E+09 | 6.26E+06 | 1.85E+05 | 6.77E+04 | 3.30E+08 | 4.24E+07 |
| F13 | Best | 3.04E+04 | 3.51E+05 | 4.05E+09 | 6.72E+06 | 1.15E+04 | 3.59E+04 | 5.08E+04 | 7.29E+05 |
|  | Std | 1.90E+05 | 1.09E+07 | 4.73E+09 | 7.41E+06 | 5.66E+05 | 9.47E+04 | 1.10E+09 | 1.03E+08 |
|  | Avg | 4.49E+03 | 2.13E+05 | 2.23E+06 | 1.77E+05 | 1.69E+05 | 1.38E+05 | 1.88E+06 | 8.11E+05 |
| F14 | Best | 4.25E+03 | 8.11E+04 | 9.14E+05 | 1.09E+05 | 3.54E+04 | 7.43E+04 | 1.66E+05 | 1.74E+05 |
|  | Std | 5.54E+03 | 3.98E+05 | 2.87E+06 | 2.36E+05 | 2.41E+05 | 1.95E+05 | 2.91E+06 | 1.39E+06 |
|  | Avg | 7.97E+04 | 2.76E+05 | 2.07E+09 | 1.08E+06 | 6.42E+04 | 6.35E+04 | 7.12E+08 | 1.09E+07 |
| F15 | Best | 4.89E+04 | 1.00E+05 | 1.14E+09 | 4.59E+05 | 1.23E+04 | 6.93E+04 | 3.89E+04 | 1.52E+05 |
|  | Std | 1.17E+05 | 5.60E+05 | 2.25E+09 | 1.35E+06 | 1.47E+05 | 7.88E+04 | 2.04E+09 | 3.61E+07 |
|  | Avg | 8.89E+02 | 7.58E+05 | 4.05E+08 | 2.30E+05 | 3.30E+03 | 2.50E+03 | 9.32E+04 | 1.32E+06 |
| F16 | Best | 1.62E+03 | 2.08E+04 | 1.04E+08 | 2.33E+04 | 3.27E+03 | 1.61E+03 | 3.20E+03 | 1.25E+04 |
|  | Std | 8.79E+02 | 1.41E+06 | 6.18E+08 | 5.53E+05 | 4.36E+03 | 3.95E+03 | 2.11E+05 | 3.06E+06 |
|  | Avg | 3.07E+03 | 2.91E+04 | 8.93E+11 | 2.09E+04 | 1.65E+03 | 1.69E+03 | 1.35E+14 | 1.44E+08 |
| F17 | Best | 2.62E+03 | 1.53E+04 | 2.01E+09 | 1.99E+04 | 2.21E+03 | 2.29E+03 | 2.16E+04 | 2.47E+04 |
|  | Std | 3.78E+03 | 3.30E+04 | 2.99E+12 | 2.20E+04 | 1.67E+03 | 1.71E+03 | 8.93E+14 | 1.10E+09 |
|  | Avg | 2.58E+04 | 1.99E+05 | 6.37E+06 | 1.66E+05 | 8.89E+04 | 6.14E+04 | 2.81E+06 | 1.26E+05 |
| F18 | Best | 1.75E+04 | 5.74E+04 | 2.37E+06 | 1.25E+05 | 3.07E+04 | 6.36E+04 | 5.10E+04 | 5.45E+04 |
|  | Std | 3.26E+04 | 2.81E+05 | 8.45E+06 | 1.97E+05 | 1.18E+05 | 6.63E+04 | 7.20E+06 | 1.84E+05 |
|  | Avg | 1.50E+03 | 4.06E+07 | 1.18E+12 | 7.20E+06 | 3.92E+05 | 2.65E+04 | 8.04E+08 | 4.32E+07 |
| F19 | Best | 2.03E+03 | 3.14E+04 | 1.82E+10 | 1.19E+06 | 8.64E+03 | 1.05E+04 | 1.51E+04 | 7.39E+04 |
|  | Std | 1.62E+03 | 1.24E+08 | 2.71E+12 | 1.22E+07 | 7.85E+05 | 3.50E+04 | 3.65E+09 | 1.32E+08 |
|  | Avg | 1.35E+03 | 1.33E+03 | 5.94E+03 | 1.56E+03 | 1.36E+03 | 1.22E+03 | 8.65E+03 | 6.38E+03 |
| F20 | Best | 2.14E+03 | 2.08E+03 | 6.78E+03 | 2.53E+03 | 2.13E+03 | 2.07E+03 | 6.99E+03 | 6.37E+03 |
|  | Std | 1.37E+03 | 1.34E+03 | 6.05E+03 | 1.56E+03 | 1.36E+03 | 1.22E+03 | 9.53E+03 | 6.89E+03 |

Table 9: Results of Experiments on Congress on Evolutionary Computation Composition Functions

| Function | Metric | DE | GWO | PSO | SCA | SLSMA | SMA | SSA | WOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg* | 1.41E+03 | 1.74E+03 | 1.57E+04 | 1.78E+03 | 1.18E+03 | 1.18E+03 | 9.16E+03 | 8.73E+03 |
| F21 | Best* | 2.30E+03 | 2.53E+03 | 1.60E+04 | 3.00E+03 | 2.24E+03 | 2.24E+03 | 2.54E+03 | 4.23E+03 |
|  | Std* | 1.43E+03 | 1.84E+03 | 1.72E+04 | 1.79E+03 | 1.17E+03 | 1.17E+03 | 1.32E+04 | 1.09E+04 |
|  | Avg* | 1.22E+03 | 1.22E+03 | 2.03E+03 | 1.25E+03 | 1.21E+03 | 1.21E+03 | 2.38E+03 | 2.09E+03 |
| F22 | Best | 2.41E+03 | 2.38E+03 | 3.13E+03 | 2.45E+03 | 2.40E+03 | 2.40E+03 | 2.67E+03 | 3.02E+03 |
|  | Std* | 1.21E+03 | 1.21E+03 | 2.03E+03 | 1.23E+03 | 1.20E+03 | 1.20E+03 | 2.56E+03 | 2.13E+03 |
|  | Avg | 3.03E+03 | 2.67E+03 | 2.86E+04 | 3.22E+03 | 1.31E+03 | 1.33E+03 | 1.30E+04 | 1.15E+04 |
| F23 | Best | 2.78E+03 | 2.93E+03 | 3.49E+04 | 3.96E+03 | 2.54E+03 | 2.59E+03 | 2.87E+03 | 9.45E+03 |
|  | Std | 4.03E+03 | 3.12E+03 | 2.90E+04 | 3.28E+03 | 1.30E+03 | 1.32E+03 | 1.90E+04 | 1.35E+04 |
|  | Avg | 1.81E+03 | 2.27E+03 | 1.78E+04 | 2.41E+03 | 1.35E+03 | 1.37E+03 | 4.71E+03 | 8.66E+03 |
| F24 | Best | 2.78E+03 | 3.00E+03 | 2.60E+04 | 4.12E+03 | 2.65E+03 | 2.57E+03 | 2.82E+03 | 5.78E+03 |
|  | Std | 2.12E+03 | 2.57E+03 | 1.82E+04 | 2.41E+03 | 1.34E+03 | 1.36E+03 | 7.81E+03 | 1.01E+04 |
|  | Avg | 1.51E+03 | 1.55E+03 | 4.33E+03 | 1.58E+03 | 1.48E+03 | 1.51E+03 | 1.99E+03 | 1.84E+03 |
| F25 | Best | 2.99E+03 | 3.02E+03 | 6.69E+03 | 3.08E+03 | 2.93E+03 | 2.99E+03 | 3.28E+03 | 3.27E+03 |
|  | Std | 1.50E+03 | 1.53E+03 | 4.39E+03 | 1.57E+03 | 1.47E+03 | 1.50E+03 | 2.04E+03 | 1.83E+03 |
|  | Avg | 1.78E+03 | 1.82E+03 | 4.26E+03 | 1.88E+03 | 1.56E+03 | 1.79E+03 | 6.50E+03 | 2.53E+03 |
| F26 | Best | 3.54E+03 | 3.55E+03 | 6.00E+03 | 3.61E+03 | 3.09E+03 | 3.54E+03 | 4.59E+03 | 3.61E+03 |
|  | Std | 1.77E+03 | 1.79E+03 | 4.37E+03 | 1.86E+03 | 1.54E+03 | 1.77E+03 | 7.89E+03 | 2.85E+03 |
|  | Avg | 1.67E+03 | 1.68E+03 | 2.11E+03 | 1.74E+03 | 1.66E+03 | 1.67E+03 | 2.29E+03 | 2.06E+03 |
| F27 | Best* | 3.29E+03 | 3.30E+03 | 3.72E+03 | 3.42E+03 | 3.29E+03 | 3.30E+03 | 3.96E+03 | 3.61E+03 |
|  | Std | 1.65E+03 | 1.66E+03 | 2.10E+03 | 1.73E+03 | 1.64E+03 | 1.66E+03 | 2.29E+03 | 2.05E+03 |
|  | Avg | 1.57E+03 | 1.69E+03 | 2.99E+03 | 1.70E+03 | 1.71E+03 | 1.58E+03 | 3.06E+03 | 1.85E+03 |
| F28 | Best | 2.96E+03 | 3.19E+03 | 4.86E+03 | 3.22E+03 | 3.39E+03 | 2.89E+03 | 3.61E+03 | 3.42E+03 |
|  | Std | 1.56E+03 | 1.67E+03 | 3.00E+03 | 1.69E+03 | 1.69E+03 | 1.57E+03 | 3.44E+03 | 1.86E+03 |
|  | Avg | 5.57E+05 | 5.46E+06 | 5.37E+10 | 2.96E+07 | 2.34E+03 | 4.54E+04 | 3.64E+12 | 1.86E+08 |
| F29 | Best | 1.71E+04 | 4.69E+04 | 3.90E+09 | 6.39E+05 | 3.73E+03 | 8.35E+03 | 2.43E+08 | 2.86E+05 |
|  | Std | 2.85E+06 | 1.31E+07 | 1.08E+11 | 6.11E+07 | 2.34E+03 | 8.76E+04 | 2.45E+13 | 4.53E+08 |
|  | Avg | 5.13E+05 | 1.90E+07 | 2.93E+10 | 2.00E+07 | 6.72E+04 | 8.51E+04 | 1.06E+11 | 9.90E+07 |
| F30 | Best | 1.65E+05 | 2.01E+05 | 9.57E+09 | 1.79E+06 | 3.39E+03 | 3.60E+04 | 3.90E+05 | 5.09E+05 |
|  | Std | 1.35E+06 | 1.13E+08 | 6.40E+10 | 6.73E+07 | 1.95E+05 | 1.24E+05 | 7.70E+11 | 2.60E+08 |

* The actual values vary

Table 10: Results of the Friedman Test

| Test | DE | GWO | PSO | SCA | SLSMA | SMA | SSA | WOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Friedman Mean Rank | 1.900622 | 3.1854 | 7.103442 | 4.077312 | 0.976856 | 1.359104 | 5.616922 | 5.478888 |
| Rank | 3 | 4 | 8 | 5 | 1 | 2 | 7 | 6 |

Table 11: Results of Wilcoxon’s Rank-sum Test

| S/N | SLSMA | Test Statistic | P-value | Significance |
| --- | --- | --- | --- | --- |
| 1 | DE | 110.4272 | 0.01369722 | True |
| 2 | GWO | 25.4832 | 0.00004863 | True |
| 3 | PSO | 0.0000 | 0.00000000 | True |
| 4 | SCA | 8.4944 | 0.00000009 | True |
| 5 | SMA | 102.9946 | 0.00850502 | True |
| 6 | SSA | 2.1236 | 0.00000501 | True |
| 7 | WOA | 0.0000 | 0.00000003 | True |

Figure 3 and Appendix I show that the SLSMA has a better algorithmic convergence stability for all the benchmark functions. Also, the SLSMA, compared with the other algorithms, has an early-stage faster algorithmic convergence on most of the benchmark functions. This establishes the SLSMA’s ability, driven by the dynamic perturbation technique, to overcome algorithmic stagnation and to speed up algorithmic convergence. For the benchmark functions, F1,

F7, F10, F13, F15, F29, and F30, the SLSMA achieves the minimum fitness statistic and fastest algorithmic convergence speed, which are critical in situations demanding efficient real-time computations.

  - Multi-Drone Flight Path Planning

The planning problems for multi-drone paths are more complex than those for benchmark functions due to their non-linear cost and constraints.The ESMA [60], CO-SMA [61], and SMA-AGDE [62] have been chosen as the SLSMA’s competitors, since they have been shown to perform excellently relative to popular meta-heuristic algorithms. The ESMA incorporates the Sine Cosine Algorithm to improve the performance of the SMA. The CO-SMA applies a chaotic opposition-based approach that blends the chaotic search method with a crossover-opposition strategy. By applying an adaptive guided differential evolution technique, the SMA-AGDE enhances the parametric capabilities of the SMA. Table 12 details the parameter settings for CO-SMA, ESMA, SLSMA, SMA-AGDE, and SMA as indicated in their respective original papers.

Table 12. Parameter Settings of Algorithms for Multi-Drone Path Optimization

| S/N | Algorithm | Parameter Settings |
| --- | --- | --- |
| 1 | CO-SMA | z = 0.03, and β0 ∈ [0, 1] |
| 2 | ESMA | z = 0.03, r2 ∈ [0, 2π], and r3 ∈ [0, 2] |
| 3 | SLSMA | Cr = 0.5, and ζ = 0.1 |
| 4 | SMA-AGDE | CR1 ∈ [0.05, 0.15], and CR2 ∈ [0.9, 1.0] |
| 5 | SMA | z = 0.03 |

In this paper, the dimension of the test case is 10003, with 5 drones (M=5)and 10 waypoints (N=10). The drone flight starting and terminal points are (0, 0, 0) and (1000, 1000, 0) respectively. The cost function together with the constraints is defined in Section 2. Table 13 details the settings for the various cost components.

Table 13: Parameter Settings for Cost Components

| S/N | Parameter | Value |
| --- | --- | --- |
| 1 | Maximum yaw angle, αmax | 45° |
| 2 | Maximum pitch angle, βmax | 45° |
| 3 | Lower height limit, zlb | 10 |
| 4 | Upper height limit, zub | 950 |
| 5 | Minimum safe distance between drones, dmin | 5 |
| 6 | Cost coefficient, qheight | 100 |
| 7 | Cost coefficient, qyaw | 100 |
| 8 | Cost coefficient, qpitch | 100 |
| 9 | Cost coefficient, qcollis_obs | 100 |
| 10 | Cost coefficient, qcollis_drone | 100 |
| 11 | Weight coefficient, ω1 | 100 |
| 12 | Weight coefficient, ω2 | 1 |
| 13 | Weight coefficient, ω3 | 2 |
| 14 | Weight coefficient, ω4 | 3 |
| 15 | Weight coefficient, ω5 | 3 |

Table 14 presents the settings for the three scenarios considered in this paper, with N=30, tmax=500, and 30 independent runs.

Table 14: Coordinates Information for Multi-Drone Path Planning Problems

| Scenario I |  |  |  |
| --- | --- | --- | --- |
| S/N | Center Coordinate (xc, yc) | Slope (xsl, ysl) | Height (hm) |
| 1 | (250, 200) | (110, 105) | 505 |
| 2 | (420, 800) | (90, 140) | 745 |
| 3 | (720, 340) | (150, 140) | 605 |
| Scenario II |  |  |  |
| 1 | (200, 330) | (100, 110) | 405 |
| 2 | (430, 700) | (90, 140) | 785 |
| 3 | (200, 730) | (110, 120) | 390 |
| 4 | (710, 800) | (90, 140) | 505 |
| 5 | (790, 200) | (90, 80) | 325 |
| 6 | (570, 210) | (100, 110) | 545 |
| Scenario III |  |  |  |
| 1 | (150, 660) | (68, 65) | 385 |
| 2 | (460, 270) | (110, 95) | 535 |
| 3 | (740, 80) | (80, 50) | 425 |
| 4 | (190, 740) | (70, 70) | 265 |
| 5 | (390, 690) | (80, 80) | 495 |
| 6 | (710, 710) | (90, 160) | 705 |
| 7 | (940, 340) | (45, 76) | 395 |
| 8 | (390, 890) | (50, 50) | 295 |
| 9 | (840, 890) | (60, 70) | 495 |
| 10 | (290, 110), | (50, 60) | 355 |

The performance of the five algorithms is comparatively evaluated in Table 15. Figure 4 illustrates the average algorithmic convergence curves, while Figures 5-8 depict the optimal path-planning results for the CO-SMA and the SLSMA in both two-dimensional and three-dimensional representations. The corresponding convergence curves for the SMA, ESMA, and SMA-AGDE are provided in Appendix II.

In these figures, drone flight trajectories originate from a red square and terminate at a blue star. The continuity of a trajectory's arc in the two-dimensional plots serves as a critical indicator for assessing potential collisions. Smooth, continuous curves signify collision-free paths between drones and obstacles. Conversely, any discontinuity in the arc indicates a point of collision.

In the initial scenario featuring three obstacles, the SLSMA demonstrated enhanced algorithmic robustness. As quantified in Table 15, it achieved the second-highest fitness value alongside an optimal standard deviation, a performance tier corroborated by its second-place ranking among five algorithms in Figure 4(a). Analysis of the resulting trajectories reveals a clear performance hierarchy. As shown in Figure 8(a), Figure IIa(i), and Figure IIb(i), the CO-SMA, ESMA, and SMA algorithms produced identical flight paths plagued by potential collisions due to insufficient safety margins; visible breaks in these trajectories confirm inevitable obstacle impacts. While the SMA-AGDE successfully prevented drone-to-drone collisions (Figure IIa(iii) and Figure IIb(iii)), it remained susceptible to two distinct obstacle collisions. In contrast, the SLSMA, as evidenced by Figure 5(a) and Figure 6(a), generated nearly perfect trajectories, achieving a collision-free status for all but one drone, which sustained a single minor impact. Consequently, the SLSMA is established as the most effective and reliable algorithm for Scenario I.

Table 15: Results for Multi-Drone Flight Path Planning

| Scenarios | Metrics | CO-SMA | ESMA | SLSMA | SMA-AGDE | SMA |
| --- | --- | --- | --- | --- | --- | --- |
| I | AVG | 4.26E+03 | 5.78E+03 | 5.28E+03 | 5.65E+03 | 8.41E+03 |
|  | BEST | 2.45E+03 | 2.43E+03 | 4.18E+03 | 4.37E+03 | 3.28E+03 |
|  | STD | 1.06E+03 | 1.44E+03 | 5.04E+02 | 6.97E+02 | 1.95E+03 |
| II | AVG | 6.23E+03 | 8.27E+03 | 5.14E+03 | 5.95E+03 | 9.24E+03 |
|  | BEST | 2.09E+03 | 3.70E+03 | 4.08E+03 | 4.88E+03 | 2.64E+03 |
|  | STD | 2.07E+03 | 2.30E+03 | 5.10E+02 | 6.31E+02 | 2.16E+03 |
| III | AVG | 4.00E+03 | 6.01E+03 | 5.02E+03 | 5.79E+03 | 8.27E+03 |
|  | BEST | 2.41E+03 | 2.94E+03 | 3.31E+03 | 4.35E+03 | 3.80E+03 |
|  | STD | 1.11E+03 | 1.64E+03 | 5.78E+02 | 7.29E+02 | 1.69E+03 |

Scenario II introduces a greater density of obstacles and operational complexity than Scenario I. The performance data in Table 15 and Figure 4(b) confirm that the SLSMA achieves superior results, demonstrating the best average fitness, optimal standard deviation, and the highest algorithmic convergence effectiveness. Furthermore, the trajectory visualizations in Figure 5(b) and Figure 6(b) corroborate that the SLSMA generates entirely collision-free paths for all drones. In contrast, as evidenced by Figure 8(b), Figure IIc(i-iii), and Figure IId(i-iii), the trajectories produced by competing algorithms are infeasible and marked by multiple collisions. Therefore, the SLSMA unequivocally demonstrates the highest performance and reliability in Scenario II.

Scenario III represents the most complex test case, featuring the highest number of obstacles with the greatest density. As detailed in Table 15 and Figure 4(c), the SLSMA ranks second in average fitness and convergence behaviour, while achieving the best standard deviation, indicating consistently stable performance. This underscores the algorithm's robustness even in highly constrained environments.

This resilience is further evidenced by the trajectories generated. As shown in Figure 7(c), Figure 8(c), Figure IIe(i-iii), and Figure IIf(i-iii), competing algorithms such as CO-SMA, ESMA, SMA, and SMA-AGDE produce invalid trajectories, with collision frequency escalating alongside scenario complexity. In contrast, Figure 5(c) and Figure 6(c) confirm that the SLSMA is the sole algorithm capable of generating viable, collision-free paths. Consequently, the SLSMA demonstrates superior performance and is unequivocally the most effective algorithm for the demanding conditions of Scenario III.

The experimental results establish that the SLSMA delivers superior algorithmic effectiveness and convergence, demonstrating stable and consistent performance across all tested scenarios. Furthermore, the algorithm generates demonstrably more optimal drone flight trajectories than its competitors. Collectively, these findings confirm the robust performance of the SLSMA and affirm its suitability for complex multi-drone path planning applications.

(a) 					        (b)

(c)

Figure 4: Convergence curves of selected algorithms for multi-drone path planning in (a) Scenario I (b) Scenario II, and (c) Scenario III.

The experimental results further demonstrate that the SLSMA generates superior drone flight trajectories compared to alternative algorithms. These findings confirm the algorithm's robust performance and its suitability for application in complex, multi-drone path planning scenarios.

- (b)

Figure 5: 2D view of drone flight trajectories using SLSMA in problem scenarios (a) Scenario I, (b) Scenario II, and (c) Scenario III.

(c)   					  (a)

Figure 5: Cont. 				 Figure 6: Cont.

(b)                                                               (c)

Figure 6: 3D view of drone flight trajectories using SLSMA in problem scenarios (a) Scenario I, (b) Scenario II, and (c) Scenario III.

- (b)

Figure 7: 2D view of drone flight trajectories using CO-SMA in problem scenarios (a) Scenario I, (b) Scenario II, and (c) Scenario III.

(c) 						  (a)

Figure 7: Cont.. 				Figure 8: Cont..

- (c)

Figure 8: 3D view of drone flight trajectories using CO-SMA in problem scenarios (a) Scenario I (b) Scenario II and (c) Scenario III.

- Conclusion

This paper has introduced a Self-Learning Slime Mould Algorithm (SLSMA), a novel meta-cognitive swarm intelligence framework designed to address the critical challenge of high cost of operation of UAV swarms in complex, cluttered environments. The proposed SLSMA fundamentally transcends the limitations of the traditional SMA by embedding self-directed learning and adaptive recovery mechanisms into its core architecture.

The algorithmic superiority of the SLSMA is anchored in three key innovations: a situation-aware search strategy that synergistically merges the global exploration prowess of a ranking-based differential evolution with the local exploitation focus of the SMA, achieving a dynamic and precise balance throughout the search process; a dynamic switching operator that replaces static parameters to autonomously maintain population diversity, thereby directly countering the issue of premature convergence; and an adaptive perturbation technique that acts as a meta-cognitive recovery mechanism, enabling the algorithm to intelligently identify and escape local optima, thus ensuring robust convergence to high-quality cost-effective solutions. Furthermore, the application of cubic B-spline curves guarantees the generation of kinematically feasible and smooth flight trajectories suitable for real-world drone deployment.

Rigorous validation on the CEC 2017 benchmark suite and complex 3D path planning scenarios unequivocally demonstrates that the SLSMA outperforms a suite of state-of-the-art metaheuristics. The results confirm that our framework does not merely find paths; it generates cost-effective and resilient trajectories, exhibiting superior convergence characteristics, higher success rates, and greater reliability in the most demanding scenarios. This establishes the SLSMA as a robust and cost-effective solver for the complex multi-UAV trajectory optimization problem.

This research, while comprehensive, opens several promising avenues for future research. The current study focused on a single-objective formulation within a static environment. Future work will investigate the extension of the SLSMA's meta-cognitive principles to:

- Dynamic and uncertain environments, where real-time obstacle movement and unpredictable threats require continuous re-planning. The performance of SLSMA will be evaluated under standard mobility models for UAV networks, such as Random Waypoint and Gauss-Markov.

- Multi-objective optimization problems, balancing competing goals such as mission time, energy consumption, and risk exposure.

- Hardware-in-the-loop (HIL) simulations and physical flight tests to validate the algorithm's performance and resilience in real-time robotic systems.

By providing a foundational framework for adaptive and resilient swarm intelligence, this research paves the way for the next generation of autonomous UAVs capable of persistent and reliable operation in critically denied and complex terrains.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

Option 2:

Author(s) hereby declare that generative AI technologies such as Large Language Models, etc. have been used during the writing or editing of manuscripts. This explanation will include the name, version, model, and source of the generative AI technology and as well as all input prompts provided to the generative AI technology

Details of the AI usage are given below:

1.

2.

3.

REFERENCES

- Jain, N., Gambhir, A., & Pandey, M. (2024). Unmanned aerial networks—UAVs and AI. In Recent Trends in Artificial Intelligence Towards a Smart World: Applications in Industries and Sectors (pp. 321-351). Singapore: Springer Nature Singapore. https://doi.org/10.1007/978-981-97-6790-8_12

- Lyu, M.; Zhao, Y.; Huang, C. & Huang, H. (2023). Unmanned aerial vehicles for search and rescue: A survey. Remote Sens., 15 (13), 3266. https://doi.org/10.3390/rs 15133266

- Rahmananta, R., Airlangga, G., Sukwadi, R., Basuki, W. W., Sugianto, L. F., Nugroho, O. I. A., & Kristian, Y. (2025). UAV logistics pattern language for rural areas. International Journal of Robotics & Control Systems, 5(1). https://doi.org/10.31763/ ijrcs.v5i1.1554

- Jiang, S.; Jiang, W. & Wang, L. (2022). Unmanned aerial vehicle-based photogrammetric 3D mapping: A survey of techniques, applications, and challenges. IEEE Geosci. Remote Sens. Mag., 10 (2), 135–171. https://doi.org/10.1109/MGRS.2 021.3122248

- Lu, F., Jiang, R., Bi, H., & Gao, Z. (2024). Order distribution and routing optimization for takeout delivery under drone–rider joint delivery mode. J. Theor. Appl. Electron. Commer. Res, 19(2), 774-796. https://doi.org/10.3390/jtaer19020041

- Wheeb, A. H. (2022). Flying Ad hoc Networks (FANET): Performance evaluation of topology based routing protocols. Int. J. Interact. Mob. Technol., 16(4), 137-149. https://doi.org/10.3991/ijim.v16i04.28235

- Wheeb, A. H., Nordin, R., Samah, A. A., & Kanellopoulos, D. (2023). Performance evaluation of standard and modified OLSR protocols for uncoordinated UAV ad-hoc networks in search and rescue environments. Electronics, 12(6), 1334. https://www.mdpi.com/2079-9292/12/6/1334

- Dimos, A., Skoutas, D. N., Nomikos, N., & Skianis, C. (2025). A survey on UxV swarms and the role of artificial intelligence as a technological enabler. Drones, 9(10), 700. https://doi.org/10.3390/drones9100700.

- López-Villegas, I., Medina-Gómez, K. J., Izquierdo-Reyes, J., Colin-García, D., González-Hernández, H. G., & Bustamante-Bello, R. (2025). A transformer-based self-organizing UAV swarm for assisting an emergency communications system. Drones, 9(11), 769. https://doi.org/10.3390/drones9110769.

- Tang, J.; Duan, H. & Lao, S. (2023). Swarm intelligence algorithms for multiple unmanned aerial vehicles collaboration: A comprehensive review. Artif. Intell. Rev., 56 (5), 4295–4327. https://doi.org/10.1007/s10462-022-10281-7

- Elshaer, A. M., Elmanfaloty, R. A., Abou-Bakr, E., Elrakaiby, M., & Saada, K. (2025). Exploring algorithmic efficiency of A-Star and Dijkstra for optimal route planning in green transportation. International Journal of Intelligent Transportation Systems Research, 1-11. https://doi.org/10.1007/s13177-025-00503-x.

- Elshahed, A., Ali, M. K. B. M., Mohamed, A. S. A., Abdullah, F. A. B., & Aun, T. L. J. (2025). Efficient pathfinding on grid maps: Comparative analysis of classical algorithms and incremental line search. IEEE Access. https://doi.org/10.1109/ACCES S.2025.3575168.

- Lu, F., Du, Z., Wang, Z., Wang, L., & Wang, S. (2025). Towards enhancing the crowdsourcing door-to-door delivery: An effective model in Beijing. J. Ind. Manag. Optim, 21, 2371-2395. https://doi.org/10.3934/jimo.2024175

- Bi, H., Gu, Y., Lu, F., & Mahreen, S. (2025). Site selection of electric vehicle charging station expansion based on GIS-FAHP-MABAC. Journal of Cleaner Production, 145557. https://doi.org/10.1016/j.jclepro.2025.145557

- Yang, Y., Fu, Y., Lu, D., Xiang, H., & Xu, K. (2024). Three-dimensional unmanned aerial vehicle trajectory planning based on the improved whale optimization algorithm. Symmetry, 16(12), 1561. https://doi.org/10.3390/sym16121561

- Pantsar, M. (2021). Descriptive complexity, computational tractability, and the logical and cognitive foundations of mathematics. Minds and Machines, 31(1), 75-98. https://doi.org/10.1007/s11023-020-09545-4

- Ma, Z., Wu, G., Suganthan, P. N., Song, A., & Luo, Q. (2023). Performance assessment and exhaustive listing of 500+ nature-inspired metaheuristic algorithms. Swarm and Evolutionary Computation, 77, 101248. https://doi.org/10.1016/j.swevo.2023.101248

- Shaikh, M. S., Raj, S., Zheng, G., Xie, S., Wang, C., Dong, X., ... & Junejo, N. U. R. (2025). Applications, classifications, and challenges: A comprehensive evaluation of recently developed metaheuristics for search and analysis. Artificial Intelligence Review, 58(12), 1-110. https://doi.org/10.1007/s10462-025-11377-6.

- Trojovský, P., & Dehghani, M. (2023). A new bio-inspired metaheuristic algorithm for solving optimization problems based on walruses behavior. Scientific Reports, 13(1), 8775. https://doi.org/10.1038/s41598-023-35863-5

- Wang, Y.; Hu, Y.; Zhang, G. & Cai, C. (2023). Research on multi-UAVs route planning method based on improved bat optimization. Algorithm. Cogent Eng., 10 (1), 2183803. https://doi.org/10.1080/23311916.2023.2183803

- Jiaqi, S.; Li, T.; Hongtao, Z.; Xiaofeng, L. & Tianying, X. (2022). Adaptive multi-UAV path planning method based on improved gray wolf Algorithm. Comput. Electr. Eng., 104, 108377. https://doi.org/10.1016/j.compeleceng.2022.1 08377

- He, W.; Qi, X. & Liu, L. (2021). A novel hybrid particle swarm optimization for multi-UAV cooperate path planning. Appl. Intell., 51(10), 7350–7364. https://doi.org /10.1007/s10489-020-02082-8

- Li, H.; Chen, Y.; Chen, Z. & Wu, H. (2021). Multi-UAV cooperative 3D coverage path planning based on asynchronous ant colony optimization. In 2021 40th Chinese Control Conference (CCC); 4255–4260. https://doi.org/10.23919/CCC52363.2021.9 549498

- Shi, K.; Zhang, X. & Xia, S. (2020). Multiple swarm fruit fly optimization algorithm based path planning method for multi-UAVs. Appl. Sci., 10 (8), 2822. https://doi.org/ 10.3390/app10082822

- Wang, W.; Zhang, G.; Da, Q.; Lu, D.; Zhao, Y.; Li, S. & Lang, D. (2023). Multiple unmanned aerial vehicle autonomous path planning algorithm based on whale-inspired deep q-network. Drones, 7 (9), 572. https://doi.org/10.3390/drone s7090572

- Li, Y.; Gao, A.; Li, H. & Li, L. (2024). An improved whale optimization algorithm for UAV swarm trajectory planning. Adv. Contin. Discrete Models, 2024(1), 40. https://doi.org/10.1186/s13662-024-03841-4

- Zhang, S.; Lv, M.; Wang, Y. & Liu, W. (2022). Multi-UAVs collaboration routing plan based on modified artificial bee colony algorithm. In Advances in Guidance, Navigation and Control, 933–943. Springer. https://doi.org/10.1007/97 8-981-15-815 5-7_78

- Tan, L.; Shi, J.; Gao, J.; Wang, H.; Zhang, H. & Zhang, Y. (2023). Multi-UAV path planning based on IB-ABC with restricted planned arrival sequence. Robotica , 41 (4), 1244–1257. https://doi.org/10.1017/S0263574722001680

- Yu, Y.; Deng, Y. & Duan, H. (2021). Multi-UAV cooperative path planning via mutant pigeon inspired optimization with group learning strategy. In Advances in Swarm Intelligence, 195–204. Springer International Publishing. https://doi.org/1 0.1007/978-3-030-78811-7_19

- Luo, D.; Li, S.; Shao, J.; Xu, Y. & Liu, Y. (2022). Pigeon-inspired optimization-based cooperative target searching for multi-UAV in uncertain environment. Int. J. Bio-Inspired Comput., 19 (3), 158–168. https://doi.org/10.1504/IJBIC.2022.12 3107

- Chen, H.; Li, C.; Mafarja, M.; Heidari, A. A.; Chen, Y. & Cai, Z. (2023). Slime mould algorithm: A comprehensive review of recent variants and applications. Int. J. Syst. Sci., 54 (1), 204–235. https://doi.org/10.1080/00207721.2022.2153635

- Wei, Y.; Othman, Z.; Daud, K. M.; Luo, Q. & Zhou, Y. (2024). Advances in slime mould algorithm: A comprehensive survey. Biomimetics, 9 (1), 31. https://doi.org/10. 3390/biomimetics9010031

- Abdel-Basset, M.; Mohamed, R.; Sallam, K. M.; Hezam, I. M.; Munasinghe, K. & Jamalipour, A. (2024). A multi-objective optimization algorithm for safety and optimality of 3-D route planning in UAV. IEEE Trans. Aerosp. Electron. Syst., 60 (3), 3067–3080. https://doi.org/10.1109/TAES.2024.3364139

- Xiong, A. H.; Banglu Ge, B. & Liu, C. J. (2023). An improved slime mold algorithm for cooperative path planning of multi-UAVs. In 2023 IEEE International Conference on Robotics and Biomimetics (ROBIO), 1–6. IEEE: Koh Samui, Thailand. https://doi.org/10.1109/ROBIO58561.2023.10354937

- Zhang, Y.-J.; Wang, Y.-F.; Yan, Y.-X.; Zhao, J. & Gao, Z.-M. (2024). Self-adaptive hybrid mutation slime mould algorithm: Case studies on UAV path planning, engineering problems, photovoltaic models and infinite impulse response. Alex. Eng. J., 98, 364–389. https://doi.org/10.1016/j.aej.2024.04.075

- Hu, G.; Du, B. & Wei, G. (2023). HG-SMA: Hierarchical guided slime mould algorithm for smooth path planning. Artif. Intell. Rev., 56 (9), 9267–9327. https://doi.org /10.1007/s 10462-023-10398-3

- Gong, W. & Cai, Z. (2013). Differential evolution with ranking-Based mutation operators. IEEE Trans. Cybern., 43 (6), 2066–2081. https://doi.org/10.1109/TCYB.2013. 2239988

- Nikolos, I. K.; Valavanis, K. P.; Tsourveloudis, N. C. & Kostaras, A. N. (2003). Evolutionary algorithm based offline/online path planner for UAV navigation. IEEE Trans. Syst. Man Cybern. Part B Cybern., 33 (6), 898–912. https://doi.org/1 0.1109/ TSMCB.2002.804370

- Wan, Y.; Zhong, Y.; Ma, A. & Zhang, L. (2022). An Accurate UAV 3-D path planning method for disaster emergency response based on an improved multi-objective swarm intelligence algorithm. IEEE Trans. Cybern., PP, 1–14. https://doi.org/10.1109/TCYB. 2022.3170580

- Pang, S.-Y.; Chai, Q.-W.; Liu, N. & Zheng, W.-M. (2024). A multi-objective cat swarm optimization algorithm based on two-archive mechanism for UAV 3-D path planning problem. Appl. Soft Comput., 167, 112306. https://doi.org/10.1016/j.asoc.2 024.112306

- Zhang, X.; Chen, J.; Xin, B. & Fang, H. (2011). Online path planning for UAV using an improved differential evolution algorithm. IFAC Proc. Vol., 44(1), 6349–6354. https://doi.org/10.3182/20110828-6-IT-1002.01807

- Dubins, L. E. (1957). On curves of minimal length with a constraint on average curvature, and with prescribed initial and terminal positions and tangents. Am. J. Math., 79(3), 497. https://doi.org/10.2307/2372560

- Savitzky, A. & Golay, M. J. E. (1964). Smoothing and differentiation of data by simplified least squares procedures. Anal. Chem., 36(8), 1627–1639. https://doi.org/ 10.1021/ac60214a047

- Gordon, W. J. & Riesenfeld, R. F. (1974). B-spline curves and surfaces. In Computer Aided Geometric Design, 95–126. Barnhill, R. E.,Riesenfeld, R. F., Eds.; Academic Press. https://doi.org/10.1016/B978-0-12-079050-0.50011-4

- Liu, Z., Wen, S., Huang, G., Li, S., & Deng, Z. (2024). Agricultural UAV obstacle avoidance system based on a depth image inverse projection algorithm and B-spline curve trajectory optimization algorithm. Information Technology and Control, 53(3), 736-757. https://doi.org/10.5755/j01.itc.53.3.36021

- Hasan, M. S.; Alam, M. N.; Fayz-Al-Asad, M.; Muhammad, N. & Tunç, C. (2024). B-spline curve theory: An overview and applications in real life. Nonlinear Eng., 13(1). https://doi.org/10.1515/nleng-2024-0054

- Xiong, T.; Li, H.; Ding, K.; Liu, H. & Li, Q. (2023). A hybrid improved symbiotic organisms search and sine–cosine particle swarm optimization method for drone 3D path planning. Drones, 7(10), 633. https://doi.org/10.3390/drones71 00633

- Zhang, T.; Yu, L.; Li, S.; Wu, F.; Song, Q. & Zhang, X. (2023). Unmanned aerial vehicle 3D path planning based on an improved artificial fish swarm algorithm. Drones, 7(10), 636. https://doi.org/10.3390/drones7100636

- Li, S.; Chen, H.; Wang, M.; Heidari, A. A. & Mirjalili, S. (2020). Slime mould algorithm: A new method for stochastic optimization. Future Gener. Comput. Syst., 111, 300–323. https://doi.org/10.1016/j.future.2020.03.055

- Yu, X., Liu, Z., Wu, X., & Wang, X. (2021). A hybrid differential evolution and simulated annealing algorithm for global optimization. Journal of Intelligent & Fuzzy Systems, 41(1), 1375-1391. https://doi.org/10.3233/JIFS-210239

- Das, A., Rai, R., Sasmal, B., Dhal, K. G., Khurma, R. A., & Saha, R. (2025). Metaheuristic algorithms since 2020: Development, taxonomy, analysis, and applications. Archives of Computational Methods in Engineering, 1-69. https://doi.org/10.1007/s1183 1-025-10408-3

- Sagheer, M., Asif Jan, M., Shah, Z., Mashwani, W. K., Adeeb Khanum, R., & Shutaywi, M. (2025). Enhancing teaching learning based optimization algorithm through group discussion strategy for CEC 2017 benchmark problems. Soft Computing, 29(2), 895-932. https://doi.org/10.1007/s00500-025-10409-1

- Storn, R. & Price, K. (1997). Differential evolution – a simple and efficient heuristic for global optimization over continuous spaces. Journal of Global Optimization, 11(4), 341–359. https://doi.org/10.1023/A:1008202821328

- Kennedy, J. & Eberhart, R. (1995). Particle swarm optimization. In Proceedings of ICNN’95 - International Conference on Neural Networks, 4, 1942–1948. https://doi.org/ 10.1109/ICNN.1995.488968

- Mirjalili, S. & Lewis, A. (2016). The whale optimization algorithm. Adv. Eng. Softw., 95, 51–67. https://doi.org/10.1016/j.advengsoft.2016.01.008

- Mirjalili, S. & Gandomi, A. H.; Mirjalili, S. Z.; Saremi, S.; Faris, H.; Mirjalili, S. M. (2017). Salp swarm algorithm: A bio-inspired optimizer for engineering design problems. Adv. Eng. Softw., 114, 163–191. https://doi.org/10.1016/j.advengsoft.201 7.07.002

- Mirjalili, S. (2016). SCA: A sine cosine algorithm for solving optimization problems. Knowl.-Based Syst., 96, 120–133. https://doi.org/10.1016/j.knosys.201 5.12.022

- Iman, R. L. & Davenport, J. M. (1980). Approximations of the critical region of the Fbietkan statistic. Commun. Stat. - Theory Methods, 9(6), 571–595. https://doi.org/10.10 80/03610928008827904

- Wilcoxon, F. (1992). Individual comparisons by ranking methods. In Breakthroughs in Statistics, Kotz, S., Johnson, N. L., Eds.; Springer Series in Statistics, 196–202. Springer New York: New York. https://doi.org/10.1007/978-1-4612-4380-9_16

- Örnek, B. N.; Aydemir, S. B.; Düzenli, T. & Özak, B. (2022). A novel version of slime mould algorithm for global optimization and real world engineering problems: Enhanced slime mould algorithm. Math. Comput. Simul., 198, 253–288. https://doi.org/10.1016/j. matcom.2022.02.030

- Rizk-Allah, R. M.; Hassanien, A. E. & Song, D. (2022). Chaos-opposition-enhanced slime mould algorithm for minimizing the cost of energy for the wind turbines on high-altitude sites. ISA Trans., 121, 191–205. https://doi.org/1 0.1016/j.isatra.2021.04.011

- Houssein, E. H.; Mahdy, M. A.; Blondin, M. J.; Shebl, D. & Mohamed, W. M. (2021). Hybrid slime mould algorithm with adaptive guided differential evolution algorithm for combinatorial and global optimization problems. Expert Syst. Appl., 174, 114689. https://doi.org/10.1016/j.eswa.2021.114689

- Naser, M. T., & Wheeb, A. H. (2022). Implementation of RWP and Gauss Markov Mobility Model for Multi-UAV Networks in search and rescue environment. Int. J. Interact. Mob. Technol., 16(23), 125-137. https://doi.org/10.3991/ijim.v16i23.35559.

APPENDICES

Appendix I

Appendix I shows the convergence curves of DE, GWO, SCA, SLSMA, and SMA for 30 runs on the CEC 2017 benchmark functions from F11 to F30 with 30 dimensions.

Figure Ia: Convergence curves of DE, GWO, SCA, SLSMA, and SMA on CEC 2017 functions

Figure Ia: Cont.

Appendix II

Appendix II presents the drone flight trajectories produced by SMA, ESMA, and SMA-AGDE for the three scenarios. Figure IIa, Figure IIb, and Figure IIc show the drone flight trajectories in Scenarios I, II, and III respectively.

(i)                                                                     (ii)

(iii)

Figure IIa: 2D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario I. (a) SMA. (b) ESMA. (c) SMA-AGDE.

- (ii)

Figure IIb: 3D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario I. (a) SMA. (b) ESMA. (c) SMA-AGDE

(iii) 					     (i)

Figure IIb: Cont. 				  Figure IIc: Cont.

(ii)                                                             (iii)

Figure IIc: 2D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario II. (a) SMA. (b) ESMA. (c) SMA-AGDE.

- (ii)

(iii)

Figure IId: 3D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario II. (a) SMA. (b) ESMA. (c) SMA-AGDE.

- (ii)

Figure IIe: 2D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario III. (a) SMA. (b) ESMA. (c) SMA-AGDE.

(iii) 					    (i)

Figure IIe: Cont. 				Figure IIf: Cont.

(ii)                                                               (ii)

Figure IIf: 3D view of drone flight trajectories using SMA, ESMA, and SMA-AGDE for problem scenario III. (a) SMA. (b) ESMA. (c) SMA-AGDE.