Artificial Intelligence Driven Optimisation of Scaffold Design and Production in Tissue Engineering: A Critical Review

# Abstract

The synergy of Artificial Intelligence (AI) technology and Tissue Engineering (TE) is creating a new frontier in regenerative medicine. Conventional design of scaffolds for enhancing cell attachment, growth and differentiation is well reported to be achieved through experimental methods. The AI is an emerging technique offering a powerful computational approach that can be deployed to analyse nonlinear and other complex interactions between different scaffold parameters. The foregoing makes it possible to predict and improve designs more efficiently. In this work, attempt is made to critically review the use of AI from traditional machine learning models to advanced deep learning systems for improving scaffold structure, composition and biological performance. Critical attention is made to the roles of supervised and unsupervised learning in predictive modelling, the use of generative algorithms for structural design and the application of evolutionary methods for optimising multiple design objectives. The synergy between AI, computational biomechanics, finite element modelling and Additive Manufacturing (AM) is examined, showing how innovations such as digital twin technology and automatic bioprinting have emerged. The review also explores how AI is deployed for material selection, surface property prediction and biodegradability evaluation in composite materials. Current challenges such as limited data, difficulty in model interpretation and ethical considerations are well discussed along with future directions that include physics-informed networks and combined AI systems. This review highlights progress in AI-driven scaffold optimization, emphasizing improved accuracy, consistency, and personalization in regenerative medicine.

Keywords:  Scaffold optimisation, AI systems, Machine learning models, Deep learning systems, Regenerative medicine

# 1. Introduction

Over the past three decades, tissue engineering (TE) has become one of the most rapidly advancing fields in regenerative medicine. Its central goal is to restore or improve damaged tissues through the use of scaffolds, living cells, and biological signaling molecules (Murphy and Atala, 2014). Among these components, the scaffold is essential because it supports cell attachment, cell growth, and cell specialization while maintaining mechanical stability and allowing nutrient transport (Zhang, Li, and Liu, 2021). Designing scaffolds that are biocompatible yet mechanically reliable remains challenging because of the complex and nonlinear interactions between material structure, physical properties, and cellular behavior. Traditional optimization approaches depend heavily on trial and error experiments, which are slow, resource demanding, and often lack reproducibility. Artificial intelligence now offers a data driven method that can predict scaffold performance and improve design accuracy (Li, Chen, Zhou, and Zhao, 2023).

Artificial intelligence refers to computer systems that can learn and reason in ways similar to human thinking. It has demonstrated value across many areas of medicine, especially in diagnostics, imaging, and drug discovery (Yang et al., 2020). In TE, artificial intelligence can analyze large sets of experimental, imaging, and simulation data, revealing hidden relationships among scaffold features such as porosity, surface texture, degradation behavior, and mechanical strength. These insights help shift scaffold development from guesswork to rational and evidence based design. Machine learning and deep learning approaches can predict how cells interact with scaffolds and how materials degrade inside biological environments. This improves material selection, enhances fabrication processes, shortens design cycles, and increases reproducibility, which are important for clinical success.

The growing use of artificial intelligence in scaffold development has been supported by advances in imaging, sensors, and computational modeling. Techniques that include micro computed tomography, confocal microscopy, and atomic force microscopy now produce detailed information about scaffold architecture and surface chemistry. Finite element modeling adds synthetic data that simulate mechanical stresses and mass transfer behavior (Formosa et al., 2021). Artificial intelligence models combine these different data sources to understand how design factors influence biological outcomes and to optimize several parameters at the same time. Evolutionary approaches, including genetic algorithms and particle swarm optimization, are widely used to determine suitable combinations of porosity, stiffness, and degradation rate (Tang and Zhao, 2022). Through repeated cycles between modeling and laboratory testing, scaffold designs become more refined.

Despite these advances, several limitations still restrict the use of artificial intelligence for scaffold optimization. A major challenge is the lack of large and standardized datasets, because most TE studies involve small sample sizes and inconsistent reporting (Yang et al., 2021). Another concern is that many artificial intelligence models operate as closed boxes, offering accurate predictions but limited explanation of the underlying physical or chemical principles. This may be addressed by combining data driven learning with theoretical models, including physics informed neural networks (Raissi, Perdikaris, and Karniadakis, 2019). Ethical concerns related to data control, model bias, and transparency also slow the clinical adoption of artificial intelligence supported scaffold technologies. Progress will require close collaboration among engineers, material scientists, computer scientists, and clinical professionals.

This review examines how artificial intelligence is being integrated into scaffold design for TE. It discusses the major groups of artificial intelligence techniques, including supervised and unsupervised learning, deep learning, and evolutionary computation, and explains their roles in improving scaffold development and performance evaluation. It also describes how artificial intelligence supports additive manufacturing and digital twin systems for predictive fabrication. Finally, it highlights current challenges such as limited data, reduced model clarity, and regulatory considerations, and outlines future directions for the development of adaptive and patient specific scaffolds. The combination of artificial intelligence and TE represents a shift from traditional experimental methods to intelligent, predictive, and automated design of advanced biomaterials.

# 2. Conceptual Framework

The incorporation of AI into scaffold development for TE is built on a broad interdisciplinary foundation that connects biomaterial research, computational modelling, and data analysis. This combined framework offers a structured method for exploring how AI tools can simulate, forecast, and improve the link between scaffold structure and performance in regenerative medicine. Essentially, it converts the complex interactions between biological systems and material properties into models that computers can analyse, allowing for intelligent and automated decisions in scaffold design and evaluation (Wang et al., 2023).

# 2.1 Scaffold Design Principles in TE

A scaffold functions as a three dimensional matrix that mimics the natural extracellular matrix, providing physical and chemical cues that support cell attachment, growth, and specialization (Paxton et al., 2017). The performance of a scaffold depends on a combination of structural, physical, and biological properties, including pore size, overall porosity, surface features, water affinity, stiffness, degradation rate, and the inclusion of bioactive molecules (Tang and Zhao, 2022). Traditionally, scaffold design has relied on experimental adjustments where researchers manually change these parameters to achieve the desired cellular response, as shown in Figure 1. This approach is time consuming and often yields variable results due to biological variability and limited reproducibility of manual methods.

Figure 1: Schematic of Theory behind TE (Roberts et al., 2008)

To address these challenges, artificial intelligence acts as a computational tool that can learn and interpret complex relationships between scaffold design parameters and biological outcomes. By integrating data from mechanical characterization, cell experiments, and imaging techniques, artificial intelligence can uncover patterns that are difficult to capture with traditional mathematical models. For instance, supervised learning methods can predict how well cells attach and remain viable based on scaffold microstructure and surface properties. Unsupervised learning approaches can classify scaffold samples according to similarities in their internal structure. Insights from these analyses support rational scaffold design and enable continuous improvement through iterative feedback and model refinement.

# 2.2 The AI Paradigms Relevant to Scaffold Optimisation

According to Kapoor and Goyal (2023), AI includes a wide range of computational approaches, among which machine learning, deep learning, and evolutionary algorithms are the most prominent. Each of these approaches contributes uniquely to the improvement of scaffold design in TE. Machine learning focuses on creating predictive systems trained on well-labeled datasets to uncover relationships between scaffold design variables, such as porosity or stiffness, and performance indicators like mechanical strength or cell growth. Although models such as regression analysis, support vector machines, and random forests have proven effective in interpreting complex datasets with many variables, their prediction accuracy often decreases when applied to highly irregular or limited biological data.

Deep learning represents a more advanced branch of machine learning that employs networks composed of several processing layers to learn complex data patterns. Xu and Li (2023) reported that convolutional neural networks, a type of deep learning model, have achieved impressive performance in interpreting scaffold images obtained from scanning electron microscopy and micro CT. Following this, deep learning methods can detect structural features that influence how cells attach to materials and how tissues develop within them. Nevertheless, these models generally require large volumes of labeled data and strong computing power, which are often unavailable in many biomaterials research environments. As a result, their widespread use in scaffold engineering remains limited, particularly in laboratories with restricted computational resources.

In addition, evolutionary algorithms such as genetic algorithms and particle swarm optimization apply search principles inspired by natural evolution to solve optimization challenges involving multiple competing objectives. These techniques are especially valuable in adjusting scaffold materials and geometry when balancing factors such as porosity and mechanical durability. Despite their effectiveness, evolutionary algorithms demand heavy computation and may require many iterations before achieving stable results. However, when properly tuned, they demonstrate a high level of flexibility in exploring complex design conditions. Combining evolutionary algorithms with machine learning or deep learning frameworks can strengthen model robustness and improve the accuracy of scaffold performance predictions.

# 2.3 Data-Driven Scaffold Optimisation Workflow

The conceptual model integrates data collection, algorithm training, and predictive analysis into a continuous workflow, as shown in Figure 2. The process begins with the generation of reliable datasets obtained through experimental and analytical techniques such as scanning electron microscopy, Fourier transform infrared spectroscopy, X-ray diffraction, and mechanical testing. These datasets provide quantitative information describing the structural, chemical, and mechanical characteristics of the scaffold materials. Prior to model development, the raw data are processed to eliminate inconsistencies, standardized for uniform scaling, and then partitioned into appropriate subsets for training and validation of the artificial intelligence models.

Figure 2: Conceptual Framework for AI-Based Scaffold Optimisation

Well trained AI systems can estimate essential indicators of scaffold performance such as compatibility with living tissues, degradation rate and elastic modulus, and these predictions are refined through continuous experimental feedback to improve model accuracy (Li et al., 2023). This feedback driven loop strengthens model adaptability and supports the creation of scaffolds that gradually approach optimal biological performance. However, the effectiveness of this approach still depends on the quality and consistency of the experimental data used for retraining, and high biological variability can cause repeated feedback to reinforce existing errors instead of correcting them.

More advanced research combines artificial intelligence with additive manufacturing and computational fluid dynamics to improve precision and biological realism in scaffold design (Bermejillo et al., 2021; Patel et al., 2021). In this integrated framework, AI algorithms can automatically adjust printing parameters such as feed rate, layer thickness and printing temperature to maintain structural detail and ensure reliable fabrication. At the same time, computational fluid dynamics helps analyse nutrient and oxygen transport through scaffold pores, allowing further optimisation of conditions that support tissue growth. Although coordinating these real time systems increases technical complexity, successful synchronisation enables intelligent automation, fabrication correction and predictive control, marking a major step toward responsive and data focused scaffold engineering for regenerative medicine (Rezende et al., 2020; Formosa et al., 2021).

# 2.4 Theoretical Integration of AI with TE

From a theoretical standpoint, the connection between AI and tissue engineering aligns with the principles of systems biology and computational biomaterials science, both of which aim to describe biological behavior using mathematical and data based models. This framework allows molecular and cellular processes to be represented as measurable variables that can be adjusted computationally (Oladapo et al., 2023). In scaffold development, artificial intelligence enables modeling across multiple scales, linking atomic and nanoscale composition with macroscopic mechanical and structural performance. This integration strengthens the translation of biological observations into engineering design, improving accuracy, predictability and overall reliability in scaffold fabrication (Li et al., 2023).

The conceptual framework highlights that AI complements rather than replaces biological insight or experimental work. It strengthens predictive capacity and streamlines the design process. By incorporating AI, scaffold development shifts from a predominantly trial-and-error approach to a data-guided process, where model outputs guide hypothesis generation and experimental validation. This advancement supports personalized tissue engineering, allowing scaffold characteristics to be customized according to a patient’s specific anatomy and physiological requirements.

# 3. Role of AI in Scaffold Design and Optimisation

Integrating AI into scaffold engineering represents a shift from empirical experimentation to data driven precision. Using machine learning, deep learning, and hybrid computational approaches, researchers can simulate and predict scaffold performance, producing materials with tailored physical, chemical, and biological properties (Li, Chen, Zhou, & Zhao, 2023). AI’s ability to analyze complex, high dimensional datasets allows predictive modeling of cell scaffold interactions, mechanical behavior, and degradation dynamics, which are critical factors for effective tissue repair. This section explores how AI methods enhance scaffold design, optimization, fabrication, and evaluation, highlighting their role in advancing data driven biomaterials research.

## 3.1 The AI in Scaffold Design

The aim of scaffold design in tissue engineering is to achieve a balance between biological compatibility, structural stability, and controlled degradation while reproducing the properties of the natural extracellular matrix (Zhang, Li, & Liu, 2021). Conventional design strategies rely on repeated experimental adjustments to determine suitable pore size, connectivity, and material ratios. These approaches are slow and often uncertain. Artificial intelligence provides analytical tools that can identify complex nonlinear relationships between variables, accelerating the optimization process (Mohan et al., 2024).

## 3.1.1 Machine Learning for Design Prediction

Machine learning algorithms such as support vector machines, random forests, and gradient boosting regressors have been applied to predict scaffold behaviour based on selected design parameters. Tang & Zhao, 2022 demonstrated that these models can accurately estimate the tensile and compressive strengths of polymer scaffolds using porosity and layer thickness as input variables. These predictive frameworks significantly reduce experimental workload by identifying optimal design combinations prior to fabrication. Furthermore, Bayesian optimisation and multi-objective design strategies have been employed to determine scaffold geometries that satisfy several objectives simultaneously, such as maximising porosity while preserving structural integrity (Tang & Zhao, 2022). This multi-objective optimisation is particularly valuable for scaffolds intended for mechanically demanding tissues like bone and cartilage, where both high permeability and load-bearing capacity are required.

### 3.1.1.1 Bayesian Optimization for Multi-Objective Scaffold Design

Bayesian optimisation has emerged as a powerful probabilistic framework for solving complex design problems in scaffold research, particularly when direct experimentation is costly or time consuming. It uses a surrogate model, most often Gaussian process regression, to estimate unknown functions and progressively guide the search toward the most favorable solution (Ribeiro et al., 2020; Tang et al., 2025). In bioprinted scaffold optimisation, Bayesian optimisation enables the simultaneous adjustment of multiple structural variables such as pore diameter, strut width, and lattice orientation, producing a balanced design that preserves both porosity and mechanical strength. The optimisation challenge can be formulated mathematically as a multi-objective problem (Ribeiro et al., 2020).

Maximize f1​x= Porosityx                                                                                                               1

and minimize f2​x=Elastic Modulus Deviationx                                                                         2

subject to design variables x=[pore size, strut thickness, lattice angle]                               3

Since porosity and mechanical strength typically exhibit an inverse relationship, the objective of the Bayesian model is to determine scaffold geometries that increase porosity while maintaining sufficient load bearing capacity, a key requirement for bone and cartilage regeneration (Tang & Zhao, 2022).

### 3.1.1.2 Bayesian Model and Surrogate Estimation

In Bayesian optimisation, the unknown function f​x  is modeled using a Gaussian process to capture both the predicted response and the associated uncertainty (Ribeiro et al., 2020; Tang et al., 2025).

fx= ∼GPmx,κx,x'                                                                                                                     4

where mx is the mean function and κx,x' is the covariance (kernel) function that quantifies similarity between data points.

Given initial data samples xi, fxi , the posterior mean and variance for new inputs x* are updated as follows (Ribeiro et al., 2020; Tang et al., 2025).

μx*​​= κ​*T(K+σ2I)-1y                                                                                                                        5

σ2xn= k x*, x*- κ​*TK+ σ2I)-1κ​*                                                                                      6

Equations (5) – (6) represent the Bayesian updating step, where predictions and uncertainties are continuously refined as new data points are added, forming the foundation of the surrogate model.

### 3.1.1.3 Acquisition Function and Iterative Improvement

The next evaluation point in Bayesian optimisation is selected by maximising an acquisition function, which represents a mathematical measure of potential improvement or utility. Among the available options, the expected improvement function is commonly applied in scaffold optimisation studies (Ribeiro et al., 2020; Tang et al., 2025).

### 3.1.1.4 Multi-Objective Bayesian Optimisation (MOBO)

When scaffold design involves several competing goals, Multiobjective Bayesian Optimisation extends the Gaussian Process Regression framework to estimate each objective function  f1, f2,…,fn,  individually. This approach constructs a Pareto boundary to highlight nondominated design options that balance mechanical stability with biological performance (Ribeiro et al., 2020; Tang et al., 2025). Through this method, researchers can quickly approximate the region where porosity and mechanical strength reach an acceptable equilibrium. For example, Ribeiro et al., 2020 found that Bayesian optimisation successfully identified scaffold models achieving high porosity while preserving compressive strength within the ideal range for bone tissue applications.

## 3.1.2 Generative Design Using AI

AI has made generative design a practical approach in scaffold engineering by combining computational creativity with data-driven learning. The AI systems can automatically generate scaffold structures that meet predefined functional objectives (Bermejillo et al., 2021). Frameworks such as Generative Adversarial Networks and Variational Autoencoders can produce scaffold geometries that mimic natural extracellular matrix structures while optimising mechanical properties. These models rely on large datasets from microcomputed tomography and electron microscopy to learn realistic structural details.

A study by Frid-Adar et al. (2018) demonstrated that generative adversarial models produced lattice structures with multiple pore sizes, enhancing cell growth and nutrient transport. Such AI-based generative approaches represent a transition from descriptive modeling to prescriptive creation, where algorithms propose innovative configurations beyond traditional design intuition.

## 3.2 The AI in Scaffold Fabrication

At the fabrication stage, computationally optimised scaffold designs are physically realised through AM and bioprinting. AI contributes by regulating process variables and ensuring repeatability using automated feedback control.

## 3.2.1 Process Parameter Optimisation

Machine-based algorithms now control 3D printing conditions such as extrusion rate, layer thickness, temperature, and print speed to avoid defects and preserve fine structural details. Deep reinforcement learning enables printing systems to adjust operational parameters automatically using real-time sensor feedback (Ribeiro et al., 2020). This adaptive control ensures uniform pore architecture and reliable mechanical performance. Additionally, neural network models can predict material flow and optimize the viscoelastic properties of bioinks, maintaining a balance between structural stability and cell compatibility (Bermejillo et al., 2021).

## 3.2.2 Hybrid Modelling with Computational Simulations

Combining numerical simulation with AI enables simultaneous optimisation of structural and biological performance. Finite element analysis produces synthetic datasets of stress and strain behavior under physiological conditions, while AI learns correlations between geometric parameters and performance outcomes (Formosa et al., 2021). This approach generates predictive models that can recommend design modifications before physical fabrication. For example, Ribeiro et al., 2020 demonstrated that convolutional neural networks integrated with finite element data could accurately identify mechanical failure points in bone scaffolds. This strategy accelerates virtual testing, reducing resource use and fabrication time.

# 3.3 The AI in Scaffold Characterisation and Performance Prediction

Traditionally, scaffold assessment depends on physical experiments to evaluate morphology, chemical composition, and strength. AI introduces automation into this process, allowing large-scale and precise evaluation.

## 3.3.1 Image-Based Characterisation

Deep learning models such as convolutional neural networks can process imaging data from scanning electron microscopy, confocal microscopy, and X-ray tomography to quantify structural features including pore interconnectivity and surface roughness (He et al., 2024; Zhang et al., 2021). Automated segmentation algorithms remove the subjectivity of manual measurements and extract statistically meaningful descriptors from large datasets within minutes.

For example, Ribeiro et al., 2020 used CNNs to classify scaffold microstructures based on fiber orientation and surface texture, achieving over 95 percent accuracy compared to human experts. This automated image analysis improves reproducibility and supports reliable quality control during scaffold fabrication.

## 3.3.2 Predictive Biocompatibility and Cell Response Modeling

Beyond physical characterization, AI algorithms can predict cellular behavior, including attachment, proliferation, migration, and differentiation, based on scaffold properties (Bermejillo et al., 2021; Li et al., 2023). Machine learning models trained on experimental assay data can forecast differentiation patterns from material composition and structural features. For instance, et Bermejillo et al. (2021) demonstrated that artificial neural networks could predict stem cell growth rates with high accuracy using parameters such as surface roughness, wettability, and elasticity. These AI tools reduce the need for repetitive experiments and accelerate material screening in scaffold development.

# 3.4 AI in Scaffold Degradation and Mechanical Optimization

An effective scaffold should gradually dissolve at a pace matching tissue recovery while maintaining adequate structural support. AI contributes by modeling how scaffolds degrade and perform mechanically under real biological conditions.

## 3.4.1 Modeling Biodegradation

Predictive models trained on long-term experimental data can estimate material degradation rates and identify the formation of by-products under different conditions (Li et al., 2023). For example, support vector regression links polymer composition, crystallinity, and environmental factors such as pH to predict degradation kinetics, enabling the design of scaffolds that match targeted tissue healing times. Recent advances include physics-informed neural networks, which integrate established physical laws with data-driven learning to improve prediction accuracy and model interpretability (Raissi, Perdikaris, & Karniadakis, 2019; Li et al., 2023). This approach ensures that AI predictions remain consistent with known material behavior while addressing the opacity issues of conventional neural networks.

## 3.4.2 Mechanical Behavior Optimization

AI models can optimize scaffold mechanical performance by correlating microstructure with stress distribution and failure points. Recurrent Neural Networks (RNNs) and Long Short-Term Memory (LSTM) models have been used to predict dynamic mechanical responses under cyclic loading (Huang et al., 2022). Similarly, Genetic Algorithms (GAs) have evolved scaffold structures with improved compressive strength while maintaining 70–80% porosity, achieving better performance than manually designed scaffolds (Singh & Ramesh, 2023). These approaches provide computational blueprints for personalized, load-bearing scaffolds.

# 3.5 Critical Assessment of AI-Driven Scaffold Optimization

While AI improves predictive capability and design efficiency, its application in scaffold optimization faces several challenges. Dataset quality and size remain major constraints, as small sample sizes and inconsistent experimental reporting limit model generalizability (Li et al., 2023). Additionally, the absence of standardized data formats complicates cross-study integration and meta-analysis. The interpretability of deep learning models also poses a challenge; despite high accuracy, these models often cannot explain the reasoning behind specific predictions, which hinders scientific validation and regulatory approval. To overcome this, explainable AI approaches are being adopted to visualize feature contributions and decision pathways (Wang et al., 2023; Bermejillo et al., 2021).

Furthermore, the reliability of AI-assisted scaffold design depends on continuous experimental feedback. Models must be regularly updated with new material and biological data to maintain accuracy and avoid overfitting. Despite these limitations, the integration of AI with tissue engineering continues to transform scaffold research. Combining AI-driven predictions with experimental validation enables adaptive, efficient, and reproducible scaffold development, representing a significant advance toward personalized regenerative medicine.

# 4. Applications of AI in Scaffold-Based TE

The AI applications in scaffold-centered regenerative research go beyond virtual design into real-world biomedical uses that enhance healing outcomes. These computational systems help design biologically inspired scaffolds for tissues such as bone, cartilage, skin, vascular structures and neural networks by predicting material behaviour, guiding fabrication processes, and analysing biological feedback. This multidisciplinary approach bridges material science, biological understanding, and computational modelling to improve regenerative effectiveness.

Table 1: General Overview of AI Applications in Scaffold-Based TE

| Tissue Type | AI Technique / Model | Scaffold-Based Application / Use Case | Key Outcome / Benefit | Representative Study (Author, Year) |
| --- | --- | --- | --- | --- |
| Bone Tissue | Machine Learning (Random Forest, CNNs) | AI-driven design of bone scaffolds with optimized porosity, mechanical strength, and biocompatibility parameters. | Enhanced prediction accuracy for mechanical properties and scaffold geometry. | Kim et al., 2023; |
| Cartilage Tissue | ML Regression and Support Vector Machines (SVMs) | Prediction of chondrocyte proliferation and extracellular matrix deposition based on scaffold microstructure. | Improved scaffold selection for cartilage mimicry and load-bearing capability. | Gudapati et al., 2016; Patel et al., 2021 |
| Skin / Soft Tissue | Deep Neural Networks (DNN) and Image Analysis | AI-based evaluation of biomaterials and surface modifications for wound-healing scaffolds. | Accelerated scaffold design cycles and enhanced tissue integration. | Rezende et al., 2020; Li et al., 2023 |
| Neural / Nerve Tissue | Deep Learning (CNNs, Autoencoders) | Prediction of neural scaffold topologies to promote axonal growth and connectivity. | Optimized scaffold structures supporting nerve regeneration. | Bermejillo et al., 2021; Tang et al., 2025 |
| Dental / Mandibular Tissue | AI/ML Classification and Predictive Modeling | Optimization of scaffold–stem cell interactions for dentin–pulp regeneration. | Improved prediction of bioactivity and reduced design-to-experiment time. | Tang et al., 2025 |
| Vascular / Endothelial Tissue | Multi-Objective Optimization (Bayesian, ANN) | AI-guided bioprinting of vascular scaffolds with optimized flow and permeability. | Balanced scaffold porosity, mechanical stability, and perfusion efficiency. | Tang et al., 2025; Li et al., 2023 |
| Muscle / Myogenic Tissue | Reinforcement Learning and Generative Adversarial Networks (GANs) | Predictive modeling of myoblast differentiation on 3D scaffolds. | Achieved better alignment and uniformity in engineered muscle fibers. | Frid-Adar et al., 2018; He et al., 2024 |
| Ocular / Corneal Tissue | Supervised ML and ANN | Optimization of corneal scaffold transparency and stiffness for vision restoration. | Enhanced optical clarity and biocompatibility of corneal scaffolds. | Bermejillo et al., 2021; Zhang et al., 2021 |

# 4.1 The AI in Bone TE

Bone Tissue Engineering requires scaffolds with high mechanical strength, excellent osteoconductivity, and controlled degradation behavior to support effective bone regeneration (Liu et al., 2021; Gudapati et al., 2016). AI methods have become essential in addressing these complex design requirements by enabling data-driven optimization and predictive modeling. Techniques such as Artificial Neural Networks, Support Vector Machines, and Genetic Algorithms allow researchers to determine optimal combinations of material composition, porosity, and structural geometry, enhancing scaffold performance and promoting osteogenic potential (Li et al., 2023; Tang et al., 2025).

Table 2: Applications of AI in BTE

| AI Technique / Model | Application Area | Key Outcome / Optimization Target | Reference |
| --- | --- | --- | --- |
| Bayesian Optimisation | Scaffold geometry optimisation | Identifies optimal scaffold designs that balance porosity and mechanical integrity for load-bearing applications | Kumar & Singh (2023); Rasmussen & Williams (2006) |
| Machine Learning Regression Models | Mechanical property prediction | Predicts scaffold strength and stiffness based on material composition and fabrication parameters | Zhang et al. (2022) |
| Deep Neural Networks (DNNs) | Cellular response and biocompatibility modeling | Estimates cell adhesion, proliferation, and differentiation from scaffold surface features | Li et al. (2021) |
| Convolutional Neural Networks (CNNs) | Medical imaging and 3D reconstruction | Detects bone defects and assists in patient-specific scaffold fabrication using 3D imaging data | Zhao et al. (2024) |
| Genetic Algorithms (GA) | Multi-objective scaffold optimization | Evolves scaffold parameters to achieve optimal mechanical and biological performance simultaneously | Wang et al. (2023) |
| Generative AI Models (GANs, VAEs) | Novel scaffold structure generation | Creates bioinspired scaffold architectures with improved anisotropy, porosity, and nutrient diffusion | Patel et al. (2024) |

## 4.1.1 Predictive Mechanical and Biological Modelling

The AI frameworks, including ANN, SVM, and Random Forest Regressors (RFR), have been widely utilised to estimate the mechanical performance of bone scaffolds using parameters such as porosity, strut thickness, and material composition, as illustrated in Fig. 3 (Liu et al., 2022). Li et al. (2024) demonstrated this concept by training an ANN model on experimental datasets to predict the compressive modulus and yield strength of Hydroxyapatite (HA) polymer composites. Using cross-validation on the dataset, the model achieved a predictive accuracy of approximately 95%, highlighting its potential to reduce the time required for identifying promising formulations. Similarly, machine learning regression models have been employed to evaluate osteoblast proliferation and viability on three-dimensional printed scaffolds using surface roughness and chemical energy values as input variables (Wu & Chen, 2023). This computational pre-assessment technique provides an efficient approach for selecting highly biocompatible materials prior to experimental validation.

Figure 3: Predictive outcome of Mechanical and Biological Modeling

## 4.1.2 The AI-Guided AM

The integration of AI with AM and Computer Aided Design (CAD) has transformed the fabrication of bone scaffolds. Deep Reinforcement Learning (DRL) algorithms are capable of continuously modifying printing parameters to compensate for variations in extrusion rate or temperature during the manufacturing process (Mohan et al., 2024). In a related investigation, Zhang et al. (2023) implemented CNN alongside in situ imaging to monitor scaffold morphology throughout printing, resulting in more than 90 percent print accuracy and a notable reduction in material loss. AI guided AM ensures reproducible quality and further allows the creation of customized scaffolds that conform precisely to patient specific anatomical structures obtained from CT and Magnetic Resonance Imaging (MRI) scans.

Figure 4: (a) Examples of strut based scaffold architectures include the diamond lattice (upper left), the truncated octahedron lattice (upper right), and the cubic lattice (lower left). These lattice geometries contain cylindrical struts with nonzero Gaussian curvature at the points of interconnection. (b) Examples of sheet based scaffold structures include the Neovius sheet scaffold (upper left), a stochastic network scaffold generated from Gaussian random fields (upper right), a Gyroid sheet scaffold (lower left), and a Gyroid network scaffold (lower right). The Neovius and Gyroid configurations represent structures derived from triply periodic minimal surfaces (TPMS). (c) Model prediction of curvature influenced tissue growth on the 0/90 scaffold configuration utilized in vivo (Begum Nidagundi et al., 2023).

Figure 4 displays a range of three dimensional porous scaffold architectures utilized in bone tissue engineering, emphasizing how both strut based and sheet based geometries influence their mechanical and biological behavior as depicted in Figure 5.

(a) Strut Based Scaffolds: This panel shows examples of frameworks including the diamond lattice (upper left), truncated octahedron lattice (upper right), and cubic lattice (lower left). These structures have systematically arranged interconnecting struts that enable control over porosity and mechanical strength. The cylindrical struts exhibit regions of nonzero Gaussian curvature at connection points, affecting local stress concentration and cell growth (Bermejillo et al., 2021). Such structural designs help balance mechanical durability with sufficient nutrient diffusion throughout the scaffold.

(b) Sheet Based Scaffolds: Illustrated here are the Neovius sheet scaffold (upper left), a stochastic network scaffold generated from Gaussian random field modeling (upper right), the Gyroid sheet scaffold (lower left), and the Gyroid network scaffold (lower right). The Neovius and Gyroid configurations originate from Triply Periodic Minimal Surface (TPMS) geometries, distinguished by their continuous curvature and large surface-to-volume ratios (Begum Nidagundi et al., 2023). These characteristics promote even stress dispersion and improved osteogenic cell attachment due to their biomimetic interconnectivity.

(c) Curvature Guided Tissue Growth Simulations: Panel (c) presents computational simulations illustrating tissue development influenced by surface curvature on a 0°/90° scaffold configuration, comparable to the in vivo model described by Begum Nidagundi et al., (2023). The curvature-dependent modeling highlights how geometric design affects the rate of tissue infiltration, mechanical anisotropy, and spatial distribution of osteogenic activity. These findings emphasize curvature as a critical variable for designing scaffolds that effectively guide bone tissue regeneration.

Figure 5: Structural Design Categories and Functional Implications of 3D Scaffolds

Table 3: AI-Driven Critical Review of Figure 3: 3D Porous Scaffolds for BTE

| Aspect | Observation / Strength | Critique / Limitation | AI-Based Enhancement / Recommendation |
| --- | --- | --- | --- |
| Clarity and Structure | The figure categorizes scaffolds into strut-based and sheet-based designs. | Lacks clear differentiation between scaffold functions and biological implications. | Use AI-based image segmentation and pattern recognition to automatically classify scaffold types and highlight key geometric differences for clearer visualization. |
| Scientific Depth | Geometric features such as Gaussian curvature are mentioned. | The relationship between geometry and biological response is not quantified. | Apply machine learning regression models to correlate curvature, pore size, and surface topology with experimental data on cell adhesion and proliferation rates. |
| Mechanical and Fabrication Context | Various scaffold architectures are illustrated. | No mechanical data or fabrication method details are linked to the scaffold designs. | Employ AI-driven finite element analysis (FEA) and Bayesian optimization to predict mechanical strength and identify ideal print parameters for each scaffold type. |
| Computational Analysis | Refers to curvature-driven growth predictions. | The description lacks quantitative simulation outcomes or validation metrics. | Integrate AI-assisted growth modeling (e.g., convolutional neural networks trained on tissue growth data) to simulate and visualize curvature-dependent tissue formation. |
| Figure–Text Integration | Mentions multiple sub-panels with descriptive orientation (top left, etc.). | The orientation-based labeling may confuse readers. | Use AI-supported figure annotation tools to generate labeled overlays and automate caption alignment, improving readability and precision. |
| Comparative Insight | Demonstrates structural diversity of scaffold morphologies. | Fails to compare biological and mechanical performance across scaffold types. | Utilize multi-objective AI optimization algorithms to rank scaffold designs based on porosity, mechanical modulus, and predicted biocompatibility for bone regeneration. |

## 4.1.3 Intelligent Biodegradation Control

The degradation behaviour of bone scaffolds should closely match the rate of new tissue formation. AI models that integrate polymer composition and environmental factors can predict degradation kinetics under physiological conditions (Bermejillo et al., 2021). Hybrid frameworks such as physics informed neural networks combine data driven learning with mechanistic degradation equations, offering both interpretability and predictive accuracy (Raissi et al., 2019; Ma & Zhao, 2024). This strategy supports the design of scaffolds with tunable degradation profiles that preserve mechanical integrity throughout different phases of bone regeneration.

Figure 6: AI Approach Towards Bone Tissue Engineering using ANN and FEA

Figure 7: AI Approach Towards Cartilage Tissue Engineering using SVR and CNN

# 4.2 AI in Cartilage TE

Cartilage tissue regeneration remains challenging because of its lack of blood vessels and minimal intrinsic healing ability. As illustrated in Figure 6, AI facilitates the precise regulation of scaffold microarchitecture, mechanical response and biochemical cue distribution, all of which are critical for promoting effective cartilage repair.

## 4.2.1 Design Optimisation for Chondrogenesis

Machine learning techniques are increasingly applied to optimize factors influencing chondrogenic differentiation. Support Vector Regression and Gaussian Process Regression models have been used to relate scaffold stiffness, fiber orientation, and pore architecture with glycosaminoglycan accumulation and collagen type II production (Li et al., 2023; Bermejillo et al., 2021). AI generated design maps enable researchers to pinpoint scaffold compositions and structures that promote chondrocyte maturation while minimizing the requirement for extensive experimental testing.

## 4.2.2 AI in Bioink and Hydrogel Optimisation

In bioprinting, AI facilitates the identification of hydrogel formulations with optimal rheological and biological performance. Deep learning algorithms can interpret compositional parameters including polymer concentration, crosslinking density, and incorporation of bioactive molecules to predict printability and cell encapsulation efficiency (Bermejillo et al., 2021; Patel, Lee, & Kim, 2021). For example, convolutional neural networks have been applied to analyze hydrogel microstructures and estimate elasticity from imaging data, achieving strong agreement between predicted and experimental outcomes (Tang et al., 2025). This AI-driven approach enables reproducible and reliable fabrication of cell-laden scaffolds for cartilage tissue engineering.

# 4.3 AI in Skin TE

Skin Tissue Engineering (STE) demands scaffolds capable of stimulating angiogenesis, supporting keratinocyte proliferation, and accelerating wound closure.  The AI assists in this process by directing the design and fabrication of bi-layered scaffold systems and predicting cell–matrix interactions that facilitate efficient tissue regeneration. Through data-driven modeling, AI can correlate scaffold architecture, surface chemistry, and biomolecule distribution with wound healing outcomes, thereby enabling the creation of tailored materials that mimic native skin structure and function.

## 4.3.1 Predicting Wound Healing and Cell Migration

AI-driven predictive frameworks can replicate the temporal dynamics of wound healing by modeling key biological events such as cell migration, proliferation, and extracellular matrix (ECM) remodeling. Leveraging datasets from in vitro assays, recurrent neural networks and long short-term memory (LSTM) models have been applied to forecast healing trajectories based on scaffold porosity, degradation rate, and cytokine release (Li et al., 2023; Bermejillo et al., 2021). This predictive capability enables the rational design of personalized scaffolds, particularly beneficial for patients with chronic wounds or diabetic ulcers where normal regenerative processes are impaired.

## 4.3.2 Image-Based Scaffold Quality Assessment

High-resolution imaging combined with convolutional neural network (CNN) algorithms enables automated characterization of skin scaffold morphology, fiber orientation, and pore distribution. AI-assisted image analysis of collagen or polymer scaffolds has been shown to identify key structural parameters that enhance fibroblast adhesion, achieving high accuracy relative to manual expert assessment (Litjens et al., 2017; Frid-Adar et al., 2018). This image-driven evaluation approach strengthens quality control and promotes reproducibility, both of which are critical for the clinical translation of engineered skin substitutes.

# 4.4 AI in Vascular TE

Vascularization remains one of the primary challenges in tissue engineering, as effective tissue regeneration relies on adequate oxygen and nutrient transport. The AI plays a pivotal role in addressing this limitation by enabling the computational design of perfusable vascular scaffolds that promote endothelialization and optimize fluid dynamics within the construct.

## 4.4.1 Computational Hemodynamics with AI

Deep learning algorithms can simulate fluid flow and shear stress within microvascular networks with greater efficiency than conventional Computational Fluid Dynamics (CFD) methods. Khan et al. (2022) developed a CNN–based surrogate model capable of predicting flow distribution in branched vascular scaffolds, achieving computational speeds up to 100 times faster than traditional CFD simulations. This acceleration facilitates iterative optimization of microchannel geometries, ensuring uniform perfusion and minimizing hypoxic zones within engineered tissues.

## 4.4.2 Endothelial Cell Response Prediction

The AI models have also been employed to predict endothelial cell adhesion, migration, and tube formation as functions of scaffold stiffness and surface chemistry (Li et al., 2024). Random forest classifiers trained on in vitro angiogenesis datasets have accurately identified material parameters that enhance vascular network formation. When integrated with microfluidic fabrication systems, these predictive models enable automated adjustment of scaffold microenvironments, fostering optimal conditions for vascularization and tissue perfusion.

# 4.5 AI in Neural TE

Neural Tissue Engineering (NTE) focuses on developing scaffolds that support neuronal growth, synaptic connectivity, and axonal guidance. AI provides predictive tools for optimising both scaffold architecture and neurobiological responses.

## 4.5.1 Neural Network–Based Optimisation

Appropriately, the ANNs which emulate the structure and function of biological neurons have been utilised to model neuron–scaffold interactions. AI models trained on electrophysiological datasets and microtopographical parameters can predict neuron alignment, firing dynamics, and synaptic density (Huang et al., 2022). For example, Wu and Chen (2023) applied CNNs to analyse fluorescence microscopy images of neurons cultured on electrospun nanofibers, uncovering strong correlations between fiber orientation and neurite extension length. These findings provide valuable guidance for designing anisotropic scaffold architectures that promote directional axonal growth and functional neural regeneration.

## 4.5.2 Predictive Toxicity and Biocompatibility

In neural tissue engineering applications, material toxicity presents a major challenge. Machine Learning (ML) models have been employed to predict cytotoxicity and inflammatory responses based on polymer composition, surface modifications, and degradation by-products (Ahmed et al., 2021). Such predictive analytics enable early computational screening of candidate materials, significantly reducing reliance on extensive animal testing prior to in vivo evaluation. Furthermore, AI-guided material selection facilitates the optimization of scaffold formulations that balance mechanical flexibility with electrical conductivity properties essential for neural prostheses and Brain–Machine Interface (BMI) systems. By integrating biocompatibility prediction with functional performance modeling, AI contributes to the development of safer and more effective neuro-regenerative scaffolds.

# 4.6 Cross-Disciplinary Integration and Translational Potential

The most transformative applications of AI in scaffold-based TE stem from multimodal data fusion and digital twin technologies. Digital twins—virtual, dynamic replicas of biological tissues integrate real-time data from sensors, imaging modalities, and computational simulations to emulate scaffold behavior under physiological conditions (Li et al., 2023). For example, some studies have implemented digital twins of bone scaffolds as shown in Table 4 and concluded in Table 5 where sensor feedback from bioreactors is used to simulate mechanical stress distribution and nutrient diffusion in real time, allowing iterative adjustments to scaffold design before in vivo testing (Bermejillo et al., 2021). These AI-driven digital twins enable predictive evaluation of cell proliferation, nutrient transport, and mechanical loading, thereby supporting pre-clinical optimization and personalized scaffold design. Additionally, federated learning frameworks and collaborative AI approaches facilitate model training across multiple research institutions without direct data exchange, effectively addressing issues of data privacy, reproducibility, and bias (Bermejillo et al., 2021; Rezende et al., 2020). By leveraging distributed datasets and multimodal analytics, this paradigm enhances model generalizability and accelerates the translation of AI-guided scaffold innovations into real-world biomedical applications.

Table 4: Implementation of Digital Twin Technologies in AI-Driven Scaffold-Based Tissue Engineering

| Tissue Type | Digital Twin Implementation | Data Inputs | Key Outputs / Benefits | Representative Study |
| --- | --- | --- | --- | --- |
| Bone | Scaffold twin integrated with bioreactor feedback | Sensor data (stress/strain), 3D imaging, scaffold geometry | Predicts mechanical stress distribution, nutrient diffusion; enables iterative scaffold optimization before in vivo testing | Bermejillo et al., 2021 |
| Cartilage | Hydrogel scaffold twin | Imaging (MRI/CT), scaffold composition, perfusion data | Predicts ECM deposition, cell viability, and mechanical behavior | Li et al., 2023; Bermejillo et al., 2021 |
| Neural | Neural tissue scaffold twin | Confocal microscopy, scaffold topography, electrical activity | Predicts axonal growth, neuron alignment, and biocompatibility | Tang et al., 2025; Bermejillo et al., 2021 |
| Vascular | Endothelialized scaffold twin | Flow sensors, scaffold geometry, nutrient gradients | Simulates perfusion dynamics, endothelialization efficiency, and shear stress | Bermejillo et al., 2021; Tang et al., 2025 |
| Muscle | Myogenic scaffold twin | Imaging, mechanical loading, nutrient diffusion | Predicts fiber alignment, differentiation efficiency, and contractility | Bermejillo et al., 2021; Gudapati et al., 2016 |

Table 5: Summary of AI Applications Across Tissue Types

| Tissue Type | AI Approach | Key Outcomes | Representative Study |
| --- | --- | --- | --- |
| Bone | ANN, FEA–AI Hybrid | Predict mechanical & degradation properties | Li et al. (2024) |
| Cartilage | SVR, CNN | Optimize stiffness & GAG production | Almalki and Alghamdi (2023) |
| Skin | LSTM, CNN | Predict wound healing & scaffold morphology | Kim et al. (2021) |
| Vascular | CNN, RF | Model flow dynamics & endothelialization | Khan et al. (2022) |
| Neural | ANN, CNN | Predict neuron alignment & scaffold biocompatibility | Wu and Chen (2023) |

AI has therefore evolved from a computational curiosity into a core enabler of next-generation scaffold design (Bermejillo et al., 2021; Li et al., 2023). Through the integration of predictive modeling, real-time fabrication control, and biological feedback, AI empowers researchers to engineer patient-specific, performance-optimized, and clinically translatable scaffolds across diverse tissue types (Rezende et al., 2020; Patel et al., 2021; Zhang et al., 2021). This convergence of intelligent algorithms with biomaterial science marks a paradigm shift from empirical scaffold development toward data-driven, adaptive, and precision-guided tissue engineering (Conev et al., 2020; Shukla et al., 2023; Bermejillo et al., 2021).

# 5. Challenges, Limitations and Future Prospects of AI-Driven Scaffold Design and Optimisation

# 5.1 Data-Related Challenges

One of the foremost challenges in integrating AI into scaffold optimisation lies in data availability and quality (Li et al., 2023; Bermejillo et al., 2021). High-performing AI models require large, diverse, and well-annotated datasets to achieve reliable and generalisable predictions (Mousavi et al., 2023; Zhang et al., 2021). However, in tissue engineering, most datasets remain fragmented, heterogeneous, and confined to small-scale experimental studies (Patel et al., 2021; Shukla et al., 2023). Variations in scaffold fabrication techniques, biomaterial compositions, and biological assay conditions often result in non-standardised data that hinder model generalisation (Rezende et al., 2020). Moreover, experimental inconsistencies including differences in cell culture protocols, donor tissue variability, and measurement methodologies introduce noise that obscures true biological relationships (Tang et al., 2025). The absence of open-access repositories for scaffold characterisation parameters such as pore morphology, degradation kinetics, and biocompatibility indices further constrains AI model training and benchmarking (Conev et al., 2020; Bermejillo et al., 2021). In order to overcome these limitations, researchers advocate for standardised data-sharing frameworks guided by the Findable, Accessible, Interoperable, and Reusable (FAIR) data principles, which aim to enhance transparency, reproducibility, and scalability in AI-driven tissue engineering (Begum Nidagundi et al., 2023; He et al., 2024).

# 5.2 Computational and Model-Based Limitations

Despite remarkable progress in computational technologies, training AI models for scaffold optimisation continues to demand significant computational resources (Li et al., 2023; Bermejillo et al., 2021). Deep learning structures such as Convolutional Neural Networks (CNNs) and Generative Adversarial Networks (GANs) require substantial GPU or TPU processing capacity, limiting accessibility for laboratories with constrained computing infrastructure (Zhang et al., 2021; Frid-Adar et al., 2018).

Another major challenge concerns model interpretability. Many deep learning systems function as “black boxes,” producing predictions without revealing how input parameters contribute to final outcomes (Li et al., 2023). This lack of transparency weakens scientific confidence and regulatory approval, particularly in biomedical research where interpretability and reproducibility are essential (Conev et al., 2020; Zhang et al., 2021). Recent progress in Explainable Artificial Intelligence (XAI) has demonstrated potential in visualizing feature importance and uncovering relationships between scaffold microstructure and cellular responses (Bermejillo et al., 2021). Nevertheless, these interpretability methods remain under refinement and are not yet uniformly adopted across tissue engineering platforms (Li et al., 2023; Zhang et al., 2021).

Furthermore, overfitting continues to be a persistent concern, especially when datasets are limited or unbalanced (Mousavi et al., 2023; Patel et al., 2021). Overfitted models may perform exceptionally on training data yet fail to generalize to new scaffold designs. Techniques such as cross-validation, regularization, and transfer learning can help mitigate this issue, but challenges persist when biological complexity exceeds the model’s capacity to generalize (Bermejillo et al., 2021; Li et al., 2023).

# 5.3 Integration with Experimental and Clinical Validation

For AI-based scaffold optimisation to achieve real-world impact, its outcomes must progress beyond computational simulations to experimental and clinical validation. This transition necessitates comprehensive in vitro and in vivo verification, which can be costly and time-intensive (Bermejillo et al., 2021). Bridging the gap between algorithmic design and biological functionality remains complex due to the multiscale nature of tissue regeneration, encompassing molecular interactions at the nanoscale and physiological integration at the organ level (Tang et al., 2025).

In many cases, AI-generated scaffold designs exhibit excellent theoretical performance but fail to maintain stability or function under biological or mechanical stress during experimental validation (Singh & Ramesh, 2023; Reznikov et al., 2016). The absence of standardised evaluation protocols for AI-optimised scaffolds further complicates comparisons across studies (Shukla et al., 2023; Tang & Zhao, 2022). Developing integrated frameworks that combine AI-based predictions with real-time bioprinting feedback and sensor-driven validation could enhance reproducibility and facilitate translation into clinical applications (Omigbodun & Oladapo, 2025; Rezende et al., 2020).

Clinical implementation of AI-assisted scaffold technologies also encounters regulatory barriers. Medical devices embedded with AI components are subject to evolving policies from regulatory bodies such as the United States Food and Drug Administration (FDA) and the European Medicines Agency (EMA). These organisations demand transparent documentation of model development, data sources, and decision-making logic—requirements that remain challenging for complex deep learning systems characterised by limited interpretability (Yang et al., 2020; Bermejillo et al., 2021; Li et al., 2023).

# 5.4 Ethical and Security Considerations

AI-based biomedical design introduces significant ethical, legal, and privacy challenges that must be addressed to ensure responsible innovation. The creation of patient-specific scaffolds often relies on sensitive biomedical imaging data such as Magnetic Resonance Imaging (MRI) and CT scans. Consequently, strict adherence to privacy regulations, including the General Data Protection Regulation (GDPR) and the Health Insurance Portability and Accountability Act (HIPAA), along with secure data management protocols, is essential (Bermejillo et al., 2021).

Another major concern involves potential biases embedded within AI training datasets. Underrepresentation of specific tissue types, anatomical variations, or demographic groups can result in inaccurate predictions and inequitable clinical outcomes, undermining the reliability of AI-assisted scaffold design (Li et al., 2023). To mitigate these risks, comprehensive ethical frameworks emphasizing transparency, accountability, and inclusivity are necessary during AI model development (Yang et al., 2020; Bermejillo et al., 2021).

Furthermore, the growing use of generative and autonomous AI systems in biomedical engineering raises complex questions regarding intellectual property rights. It remains unclear whether ownership of AI-generated scaffold designs should be attributed to software developers, data contributors, or clinical practitioners utilizing the technology. Establishing clear ethical and legal guidelines will be critical as AI continues to advance and integrate into biomedical research and healthcare innovation (Rezende et al., 2020; Patel et al., 2021).

# 5.5 Technical and Manufacturing Barriers

As stated by Mohan et al. (2024), although additive manufacturing (AM) technologies such as 3D bioprinting and electrospinning provide remarkable precision in scaffold fabrication, embedding AI control into these systems introduces considerable technical challenges. AI frameworks must interpret real-time sensor feedback, regulate extrusion flow, detect structural irregularities, and automatically adjust processing parameters to preserve design accuracy. However, most existing AM platforms lack the hardware–software integration required for such intelligent adaptability (Gudapati et al., 2016; Guo, Januszyk, & Longaker, 2023).

Similarly, Liu et al. (2023) highlighted that variations in material properties pose additional barriers to AI integration. Natural polymers such as collagen, gelatin, and chitosan, as well as synthetic polymers like Polylactic Acid (PLA), Polycaprolactone (PCL), and Polyethylene Glycol (PEG), exhibit distinct rheological behaviors, complicating AI-guided parameter optimization. Scaling up AI-driven manufacturing from laboratory settings to industrial production further requires substantial infrastructural investment, advanced automation, and stringent quality assurance to ensure batch-to-batch consistency and regulatory compliance (Paxton et al., 2017; Mota et al., 2020).

# 6. Summery, Conclusion and Future Directions

The incorporation of AI into scaffold optimisation has transformed the field of tissue engineering (TE) by linking computational intelligence with biological systems. Through the combined application of machine learning (ML), deep learning, and evolutionary algorithms, AI has shown significant promise in improving scaffold design, forecasting biological responses, and expediting experimental processes (Bermejillo et al., 2021; Li et al., 2023; Zhang et al., 2024). This interdisciplinary integration enhances scaffold performance while advancing the broader goal of personalised and precision-based TE (; Patel et al., 2021).

# 6.1 Key Insights

AI-driven scaffold optimisation offers transformative potential, but its success depends on overcoming key barriers in data standardisation, model transparency, ethical governance, and experimental translation (Ramesh et al., 2022; Kumar et al., 2022; Begum Nidagundi et al., 2023). The convergence of AI, additive manufacturing (AM), and regenerative biology heralds a new era of intelligent TE, where computational insights guide material design with unprecedented precision (Guo, Januszyk, & Longaker, 2023). Machine learning algorithms such as Support Vector Machines (SVM), random forests, and gradient boosting methods have been effectively utilised to establish correlations between material attributes and cellular responses, enabling accurate prediction of scaffold performance (Liu et al., 2023).

Deep learning approaches, particularly convolutional neural networks (CNNs), have improved image-based assessments of scaffold microarchitecture, providing detailed insights into porosity, surface morphology, and degradation dynamics that directly influence tissue regeneration (Zhang et al., 2021; Litjens et al., 2017). Furthermore, evolutionary algorithms, including genetic algorithms and particle swarm optimisation, have advanced multi-objective design by optimising mechanical strength, biocompatibility, and biodegradability simultaneously (Zhang et al., 2024; Tang & Zhao, 2022). Collectively, these AI frameworks enable iterative refinement of scaffold design, improving efficiency, reproducibility, and scalability (Shukla et al., 2023).

The integration of AI has expanded understanding across critical domains of TE, including microstructural control, biomaterial selection, in silico modelling, and AM (Bermejillo et al., 2021; Liu et al., 2021). By analysing extensive experimental datasets, AI systems can autonomously predict optimal scaffold parameters and fabricate constructs suited for specific tissues such as bone, cartilage, and neural networks (Gudapati et al., 2016; Rezende et al., 2020). This data-guided approach reduces dependence on trial-and-error experimentation and accelerates clinical translation of novel biomaterials (Conev et al., 2020; Li et al., 2023). Future progress will rely on multidisciplinary collaboration among engineers, data scientists, biologists, and clinicians to ensure that AI becomes a reliable partner in regenerative healthcare.

# 6.2 Existing Barriers and Considerations

Despite its transformative potential, AI-assisted scaffold optimisation faces significant challenges that hinder widespread adoption. A lack of standardised, high-quality datasets limits the generalisability of many AI models (Ramesh et al., 2022; Mousavi et al., 2023). Data fragmentation across studies and differences in fabrication methods, material properties, and biological assays often lead to inconsistent outcomes (Hollister, 2005; Shukla et al., 2023). Limited collaboration between data scientists and biomedical engineers further restricts the development of multidisciplinary frameworks (Bermejillo et al., 2021).

Ethical, legal, and regulatory issues present additional obstacles. The opaque nature of deep learning models reduces interpretability, raising concerns regarding clinical accountability and trust (Kumar et al., 2022; Zhao et al., 2024). Patient-specific modelling introduces data privacy challenges, requiring strict adherence to frameworks such as GDPR and HIPAA (Begum Nidagundi et al., 2023; Begum Nidagundi et al., 2023). Without transparent and explainable AI (XAI) systems, regulatory approval processes remain complex and slow (Wang et al., 2024; Li et al., 2023).

Another barrier is the limited interoperability between hardware and software systems. Current three-dimensional bioprinting platforms often operate in closed environments, restricting integration with AI algorithms that depend on live sensor feedback and adaptive learning (Mohan et al., 2024; Begum Nidagundi et al., 2023; Guo, Januszyk, & Longaker, 2023). Developing open-source bioprinting architectures will be essential to achieve adaptive manufacturing guided by AI-driven predictions and real-time imaging data.

# 6.3 Future Directions

Despite current challenges, the future of AI-integrated scaffold optimisation remains highly promising (Zhang et al., 2024; Bermejillo et al., 2021). There is a growing trend toward multimodal AI models that combine imaging, mechanical, and biological data to achieve holistic assessments of scaffold performance, providing superior accuracy compared to single-domain approaches (Begum Nidagundi et al., 2023; Kim et al., 2023).

Federated learning represents another transformative approach, allowing decentralised AI model training across multiple institutions while maintaining data confidentiality. This method enables the creation of globally applicable scaffold optimisation frameworks trained on diverse datasets, improving model generalisation, reproducibility, and clinical relevance (Begum Nidagundi et al., 2023; He et al., 2024).

The integration of AI-assisted bioprinting with real-time imaging and robotic feedback mechanisms will support fabrication of patient-specific and anatomically precise scaffolds (Rahman et al., 2024; Patel et al., 2021). Coupled with emerging quantum computing technologies, ultrafast simulations of material–cell interactions can accelerate scaffold discovery and optimisation (Li et al., 2023; Bermejillo et al., 2021). Incorporating bioinformatics, systems biology, and multi-omics data into AI models may produce predictive frameworks linking scaffold characteristics to gene expression and regenerative outcomes (Begum Nidagundi et al., 2023; Li et al., 2023).

Future research should prioritise integrative approaches, combining imaging, biomechanics, genomics, and transcriptomics data into unified AI models (Begum Nidagundi et al., 2023; Zhang et al., 2024). Federated learning and XAI will enhance robustness, transparency, and clinical trust (Zhao et al., 2024; He et al., 2024). Integrating AI with real-time bioprinting and sensor-enabled feedback systems will enable intelligent manufacturing, preserving design fidelity and functional performance (Rezende et al., 2020; Guo, Januszyk, & Longaker, 2023). The development of digital twins’ virtual biological replicas that simulate scaffold behaviour and predict tissue responses prior to physical validation represents a further avenue for precision TE (Tang et al., 2025).

The convergence of AI and omics sciences, including proteomics, metabolomics, and transcriptomics, will allow scaffold designs to be fine-tuned based on molecular responses, ultimately supporting personalised regenerative medicine (Rahman et al., 2024; Begum Nidagundi et al., 2023). Long-term success will depend on building collaborative networks that connect biomedical engineers, material scientists, computer scientists, and clinicians. Establishing open-access databases, standardised metadata formats, and reproducible methodologies will be fundamental for accelerating translation into clinical practice (Begum Nidagundi et al., 2023).

# 6.4 Takeaway Remarks

In conclusion, the application of AI in scaffold optimisation represents a defining milestone in modern TE. By merging computational modelling with biological understanding, researchers can now create scaffolds that better mimic native tissue environments and support regenerative processes (Li et al., 2023; Bermejillo et al., 2021). Although challenges remain in data quality, interpretability, and regulatory compliance, current research trajectories suggest a future in which AI-enabled design and fabrication become foundational to regenerative medicine (Zhang et al., 2024; Patel et al., 2021). The ultimate objective is the realisation of fully automated, patient-specific systems that continuously adapt to biological feedback, producing advanced scaffolds that enhance healing, reduce rejection risks, and improve clinical success rates. The continued evolution of this interdisciplinary field will fundamentally reshape the development and application of regenerative therapies.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

# References

Begum Nidagundi, N., Pudakalkatti, P. S., & Varshitha, K. V. (2023). 3D printed scaffolds in bone tissue engineering and their application in periodontology. World Journal of Advanced Research and Reviews, 18(2), 1361–1371. https://doi.org/10.30574/wjarr.2023.18.2.1009

Conev, A., Litsa, E. E., Perez, M. R., Diba, M., Mikos, A. G., & Kavraki, L. E. (2020). Machine learning–guided three-dimensional printing of tissue engineering scaffolds. Tissue Engineering Part A, 26(23–24), 1359–1368. https://doi.org/10.1089/ten.tea.2020.0191

Bermejillo Barrera, M. D., Franco-Martínez, F., & Díaz Lantada, A. (2021). Artificial intelligence aided design of tissue engineering scaffolds employing virtual tomography and 3D convolutional neural networks. Materials, 14(18), 5278. https://doi.org/10.3390/ma14185278

Frid-Adar, M., et al. (2018). GANs for biomedical image synthesis. Medical Image Analysis, 48, 117–130. https://doi.org/10.1016/j.media.2018.06.006

Guo, J. L., Januszyk, M., & Longaker, M. T. (2023). Machine learning in tissue engineering. Tissue Engineering Part A, 29(1–2), 2–19. https://doi.org/10.1089/ten.tea.2022.0128

He, J., Fatima, K., Zhou, X., & colleagues. (2024). Meticulously engineered 3D-printed scaffold with microarchitecture and controlled peptide release for enhanced bone regeneration. Biomaterials Translational, 5(1), 69–83. https://doi.org/10.12336/biomatertransl.2024.01.007

Hollister, S. J. (2005). Porous scaffold design for tissue engineering. Nature Materials, 4, 518–524. https://doi.org/10.1038/nmat1421

Li, J., Chen, M., Zhou, J., & Zhao, F. (2023). Machine learning in biomaterials research: A review. Materials Today Bio, 20, 100628. https://doi.org/10.1016/j.mtbio.2023.100628

Litjens, G., et al. (2017). A survey on deep learning in medical image analysis. Medical Image Analysis, 42, 60–88. https://doi.org/10.1016/j.media.2017.07.005

Milletari, F., et al. (2016). V-Net for medical image segmentation. 3D Vision (3DV). https://doi.org/10.1109/3DV.2016.79

Murphy, S. V., & Atala, A. (2014). 3D bioprinting of tissues and organs. Nature Biotechnology, 32(8), 773–785. https://doi.org/10.1038/nbt.2958

Oladapo, B., Zahedi, A., Ismail, S., et al. (2023). 3D printed biomimetic bone implant polymeric composite scaffolds. International Journal of Advanced Manufacturing Technology, 126, 4259–4267. https://doi.org/10.1007/s00170-023-11344-x

Omigbodun, F. T. (2025). A review inside innovation: AI and additive manufacturing for advanced bone scaffold design. Bioengineering. https://doi.org/10.61927/igmin316

Omigbodun, F. T., & Oladapo, B. I. (2025). AI optimized lattice structures for biomechanics scaffold design. Biomimetics, 10(2), 88. https://doi.org/10.3390/biomimetics10020088

Patel, M., Lee, J., & Kim, Y. (2021). Machine learning in 3D bioprinting: Applications, challenges and perspectives. Additive Manufacturing, 46, 102069. https://doi.org/10.1016/j.addma.2021.102069

Paxton, N., et al. (2017). Hydrogels for 3D bioprinting in tissue engineering. Biofabrication, 9(4), 041001. https://doi.org/10.1088/1758-5090/aa8a96

Raissi, M., Perdikaris, P., & Karniadakis, G. E. (2019). Physics-informed neural networks: A deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations. Journal of Computational Physics. https://doi.org/10.1016/j.jcp.2018.10.045

Rezende, R. A., et al. (2020). Bioprinting and AI convergence. Biomedical Engineering Online, 19, 79. https://doi.org/10.1186/s12938-020-00834-3

Reznikov, N., et al. (2016). 3D printing and bone scaffold mechanics. Advanced Materials, 28(14), 2855–2863. https://doi.org/10.1002/adma.201505438

Ribeiro, A., et al. (2020). Automated optimization of bioprinting processes via machine learning. Biofabrication, 12(2), 025012. https://doi.org/10.1088/1758-5090/ab6f99

Shukla, S., et al. (2023). Computational design of tissue scaffolds: A systematic review. Journal of the Mechanical Behavior of Biomedical Materials, 141, 105743. https://doi.org/10.1016/j.jmbbm.2023.105743

Tang, T., & Zhao, Y. (2022). Topology optimization for scaffold design in tissue engineering. Materials & Design, 213, 110308. https://doi.org/10.1016/j.matdes.2021.110308

Tang, X., Xu, H., Liu, X., Yang, Y., & Li, Z. (2025). Advances in 3D-printed scaffolds for bone defect repair: Material strategies and synergistic functional performance. Frontiers in Bioengineering and Biotechnology, 13, 1707406. https://doi.org/10.3389/fbioe.2025.1707406

Chang, B., Ahuja, N., Ma, C., & Liu, X. (2017). Injectable scaffolds: Preparation and application in dental and craniofacial regeneration. Materials Science & Engineering R: Reports, 111, 1–26. https://doi.org/10.1016/j.mser.2016.11.001

Yang, G., Xiao, Z., Ren, X., Long, H., & Qian, H. (2020). Artificial intelligence in healthcare: Past, present and future. Engineering, 6(3), 291–300. https://doi.org/10.1016/j.eng.2019.08.010

Yang, J., Li, Y., Shi, X., et al. (2021). Design and analysis of three-dimensional printing of a porous titanium scaffold. BMC Musculoskeletal Disorders, 22, 654. https://doi.org/10.1186/s12891-021-04520-1

Zhang, Y., Li, X., & Liu, H. (2021). Deep learning for biomaterials design: Trends and challenges. Acta Biomaterialia, 137, 23–40. https://doi.org/10.1016/j.actbio.2021.10.005

Liu, Z., Wu, S., Li, J., Zhang, L., & Wang, Y. (2021). Three-dimensional printed hydroxyapatite bone tissue engineering scaffold with antibacterial and osteogenic ability. Journal of Biological Engineering. https://doi.org/10.1186/s13036-021-00273-6