Seasonal Variability and Risk Assessment of Heavy Metals in Shallow Groundwater of Selected Areas in Oyo Town, Southwestern Nigeria

ABSTRACT

Groundwater remains the main source of potable water in many urban and peri-urban communities in Nigeria, particularly where municipal supply is inadequate. This study investigates the seasonal variability and potential health risks of heavy metals in shallow groundwater from selected areas of Oyo Town, Southwestern Nigeria. Forty groundwater samples (20 per season) were collected from hand-dug wells in both wet and dry seasons and analyzed for twelve heavy metals (Al, Cd, Fe, B, Ba, Co, Pb, Mn, Cr, Zn, Cu, Ni) using Inductively Coupled Plasma Mass Spectrometry (ICP-MS). Data were assessed using descriptive statistics, ANOVA, Pearson correlation, and U.S. EPA health risk assessment models. Results revealed that aluminum (Al), lead (Pb), iron (Fe), manganese (Mn), and cadmium (Cd) exceeded WHO guideline values in multiple locations. Seven metals (Al, Cd, B, Ba, Mn, Cr, and Cu) showed statistically significant seasonal variations (p < 0.05): cadmium (Cd) and chromium (Cr) were elevated in the wet season, while aluminum (Al) and manganese (Mn) were higher in the dry season. Hazard Index (HI) values indicated greater non-carcinogenic risk in the wet season, with all locations exceeding the safety threshold (HI > 1) for both adults and children; children consistently demonstrated greater vulnerability. These findings underscore the urgent need for systematic groundwater monitoring, stricter regulation of waste disposal and agricultural practices, and the provision of alternative safe water sources, in order to safeguard public health and promote sustainable groundwater use in Oyo Town.

Keywords: Groundwater quality, Heavy metals, Seasonal variation, Oyo Town, Water contamination, Nigeria.

1.0 INTRODUCTION

Groundwater is a vital freshwater resource for billions of people globally, particularly in countries where surface water is rare or unreliable (Ali & Mubarak, 2017). In Nigeria, shallow hand-dug wells serve as a primary source of drinking water for urban and peri-urban populations. In Oyo Town, rapid urbanization and inadequate municipal water infrastructure have intensified dependence on groundwater (Akanbi et al., 2023; David, 2013). However, groundwater quality is increasingly threatened by contamination from both natural and anthropogenic sources. Natural contamination may arise from water–rock interactions, infiltration of poor-quality surface water (streams, rivers, lakes), seawater intrusion, or geothermal influences (Jolaosho et al., 2024). Heavy metals are of particular concern because of their toxicity, environmental persistence, and tendency to bioaccumulate (WHO, 2022; Hameed et al., 2020). Their sources include weathering of mineralized rock, leachates from domestic waste, agricultural runoff, industrial effluents, and vehicular emissions (Mmonwuba et al., 2023). In urban centers such as Oyo Town, risks are inscreases during the rainy season due to increased infiltration and leaching of contaminants from waste dumpsites, markets, automobile repair workshops, and agricultural fields (Jolaosho et al., 2024; Ojekunle et al., 2022).

Previous studies in southwestern Nigeria have described elevated concentrations of heavy metals such as cadmium (Cd), lead (Pb), manganese (Mn), and iron (Fe) in groundwater, often linked to the geochemistry of Basement Complex rocks and poor waste management practices (Agaku et al., 2025; Ojekunle et al., 2022). However, slight attention has been given to the seasonal dynamics of heavy metal contamination in shallow aquifers, which are particularly vulnerable to surface pollution and direct recharge. Understanding these seasonal variations is crucial for identifying contamination pathways, assessing potential health risks, and guiding groundwater protection policies. Seasonal changes and natural climatic variability play a critical role in regulating groundwater recharge, runoff, and discharge dynamics. Variations in precipitation intensity across seasons strongly influence the availability and quality of groundwater resources (Sakakibara et al., 2017).

The consumption of contaminated water can lead to numerous and diverse health risks, which are especially pronounced in developing countries due to extensive human-induced impacts on groundwater quality(Sheng et al., 2022). Risk assessment processes are essential for evaluating uncertainties regarding the magnitude and relationships of human health risks associated with exposure to heavy metals. The main pathways for human exposure to heavy metals in groundwater are oral ingestion and skin contact (U.S. EPA., 2004).

Therefore, this study investigates the seasonal variability of heavy metal concentrations in shallow groundwater of Oyo Town and evaluates the associated non-carcinogenic health risks for adults and children. The findings are intended to provide scientific evidence for groundwater quality management, public health protection, and sustainable water use in rapidly urbanizing environments.

1.1 Study Area

Oyo Town lies within the tropical savannah climatic zone of southwestern Nigeria, with distinct wet (April–October) and dry (November–March) seasons. It is located between latitudes 7°47'N and 7°62'N, and longitudes 3°52'E and 4°00'E. The geology comprises Precambrian Basement Complex rocks, predominantly granites, gneisses, and migmatites, which influence groundwater chemistry through weathering and mineral dissolution. Recharge is mainly from direct rainfall infiltration and subsurface flow. Shallow wells are the main water sources due to limited municipal supply. Table 1 provides their corresponding geographic coordinates while Figure 1 shows the geological map of the sampling locations.

Table 1: Sample location and their respective coordinates (Source: Author, 2025)

| Sample Location | Latitude (°N) | Longitude (°E) |
| --- | --- | --- |
| Cele | N7° 50' 35.02" | E3° 54' 31.58'' |
| Isokun road | N7° 52' 49.12'' | E3° 55' 46.12'' |
| Alayande | N7° 52' 24.92'' | E3° 54' 41.76'' |
| Araromi 1 | N7° 49' 37.23'' | E3° 55' 48.45' |
| Araromi 2 | N7° 49' 45.26'' | E3° 55' 31.48'' |
| Dacamca | N7° 48' 38.00" | E3° 55' 60.00'' |
| Mobolaje 1 | N7° 50' 26.08'' | E3° 56' 27.92'' |
| Mobolaje 2 | N7° 50' 31.40'' | E3° 56' 14.46'' |
| Bola | N7° 50' 50.28'' | E3° 55' 28.39'' |
| Adesina | N7° 53' 30.00" | E3° 55' 42.50" |
| Oke -Afin | N7° 51' 12.00" | E3° 55' 59.00" |
| Sakutu | N7° 51' 26.00" | E3° 56' 20.00" |
| Oke Alapo | N7° 51' 40.00" | E3° 50' 56.00" |
| Lagbodoko | N7° 51' 17.70" | E3° 56' 05.40" |
| Iyalamu | N7° 51' 27.00" | E3° 56' 14.00" |
| Oke ebo 1 | N7° 50' 53.00" | E3° 56' 40.00" |
| Oke ebo 2 | N7° 51' 50.00" | E3° 56' 53.00" |
| Baago | N7° 50' 39.00" | E3° 56' 33.00" |
| Oluajo | N7° 51' 08.00" | E3° 55' 57.00" |
| BodeThomas | N7° 51' 21.00" | E3° 58' 28.00" |

Figure 1: Geological map of the study area (modified after NGSA 2016, Source: NGSA)

2.0 MATERIAL AND METHODS

2.1 Groundwater Sampling and Analysis

Groundwater samples were collected from 20 hand-dug wells during both the wet (April–October) and dry (November–March) seasons, giving a total of 40 samples. The water samples were collected from the same wells during each sampling event. These wells were properly marked during the first sampling season to ensure easy identification and consistency throughout the subsequent sampling periods. The wells, commonly used for domestic water supply, ranged between 3–12 m in depth (typical of shallow aquifers in the area). (Figure 1). Samples were collected in acid-washed, pre-cleaned 1 L polyethylene bottles, immediately acidified to pH < 2 using ultrapure nitric acid, stored in ice chests, and transported to the laboratory to minimize changes in chemical composition.

2.2 Laboratory Analysis

Twelve heavy metals (Al, Cd, Fe, B, Ba, Co, Pb, Mn, Cr, Zn, Cu, and Ni) were analyzed using Inductively Coupled Plasma Mass Spectrometry (ICP-MS) following standard APHA (2017) procedures. Instrumental detection limits ranged from 0.0001–0.005 mg/L, depending on the element. Quality Assurance/Quality Control (QA/QC) measures involved the use of method blanks, duplicate samples, and certified reference materials (CRM) to verify accuracy and precision. Recovery rates for CRMs were within 90–110%.2.3 Data Analysis.

2.3 Data Analysis

Descriptive statistics and one-way Analysis of Variance (ANOVA) were conducted using SPSS v26, with a 95% confidence level (p < 0.05) set for statistical significance. Post-hoc comparisons were applied to identify specific seasonal differences where applicable. Pearson correlation analysis was also performed to assess potential relationships and sources of heavy metals.

2.4 Health Risk Assessment

The potential non-carcinogenic health risks associated with the consumption of groundwater containing heavy metals were evaluated using the U.S. EPA human health risk assessment model. The assessment involved the calculation of the Chronic Daily Intake (CDI) for each metal, followed by the determination of the Hazard Quotient (HQ) and Hazard Index (HI). The HQ provides an indication of the likelihood of adverse health effects occurring as a result of long-term exposure to a specific metal, while the HI represents the cumulative risk from exposure to multiple metals. Only the oral ingestion pathway was considered in this study.

The CDI of each metal was calculated using the formula:

CDI=Cw×IR×EF×EDBW×AT
where:

- Cw= metal concentration in water (mg/L)

- IR= ingestion rate (2 L/day for adults; 1 L/day for children)

- EF= exposure frequency (365 days/year)

- ED= exposure duration (30 years for adults; 6 years for children)

- BW= body weight (70 kg adults; 15 kg children)

- AT= averaging time (ED × 365 days)

The Hazard Quotient (HQ) for each metal was calculated as:

HQ=CDIRfD
where RfD is the oral Reference Dose (mg/kg·day) for the metal. The Hazard Index (HI) was then calculated as the sum of HQs across all metals. An HQ or HI value greater than 1 indicates a potential health risk.

The RfD values used were obtained primarily from the U.S. EPA Integrated Risk Information System (IRIS) (U.S. EPA, 1984; U.S. EPA, 2004). For metals without officially established EPA RfDs, values were taken from reliable toxicological literature (Thongyuan et al., 2021; Tian and Wu 2019) and the World Health Organization (WHO, 2003). This ensures reproducibility of the HQ and HI calculations. The oral RfD values used in this study are summarized in Table 2

| Metal | RfD (mg/kg·day) | Source |
| --- | --- | --- |
| Aluminum (Al) | — | No official EPA RfD available. |
| Cadmium (Cd) | 0.0005 | EPA IRIS (U.S. EPA, 2004). |
| Iron (Fe) | 0.007 | Thongyuan et al., 2021 |
| Boron (B) | 0.2 | EPA Health Effects Support Document (U.S. EPA, 1984). |
| Barium (Ba) | 0.07 | U.S. EPA (1984). |
| Cobalt (Co) | — | Provisional values reported in toxicology literature |
| Lead (Pb) | 1×10⁻⁷ | value adapted from WHO guidelines (WHO, 2003). |
| Manganese (Mn) | 0.14 | EPA IRIS. |
| Chromium (Cr VI) | 0.003 | Tian and Wu 2019 |
| Zinc (Zn) | 0.3 | Toxicology literature. |
| Copper (Cu) | 0.004 | Tian and Wu 2019 |
| Nickel (Ni) | 0.02 | Toxicology literature; no EPA RfD established. |

Table 2: Reference Dose (RfD) Values Used for Health Risk Assessment

3.0 RESULTS ANALYSIS AND DISCUSSION

3.1 Heavy Metal Concentrations

Table 3 presents the seasonal concentrations of heavy metals in groundwater. The results reveal two dominant patterns. Aluminum (Al) concentrations were notably higher during the dry season, with a mean value of 3.92 mg/L and a maximum concentration of 42.652 mg/L. These values far exceed the WHO aesthetic guideline of 0.2 mg/L (WHO, 2022), suggesting intense mineral weathering but may also reflect anthropogenic inputs from refuse dumpsites and effluents from small-scale aluminum and metal workshops common in Oyo Town. The wet season showed a lower mean of 0.478 mg/L, though still above the recommended limit.

Cadmium (Cd) during the dry season had a mean of 0.00071 mg/L—within the WHO limit of 0.003 mg/L. However, the wet season value rose to 0.0237 mg/L, clearly surpassing the guideline. This elevation may be attributed to agricultural runoff, use of phosphate fertilizers, and leaching from industrial waste (Jolaosho, et al., 2024; Adimalla & Qian, 2021). In Oyo Town, intensive vegetable farming around peri-urban wells likely contributes to cadmium enrichment during the rainy season through fertilizer application and soil leaching.

Iron (Fe) was also present in elevated concentrations across both seasons. The dry season recorded a mean of 2.15 mg/L, while the wet season had 0.510 mg/L—both exceeding the WHO aesthetic limit of 0.3 mg/L (WHO, 2022). The higher levels in the dry season are likely due to oxidation and accumulation under low recharge conditions, as groundwater tends to concentrate solutes under limited recharge and enhanced evaporative effects (Aladejana et al., 2020). However, localized waste dumpsites and corroded roofing sheets discarded in residential areas may further contribute to iron enrichment in shallow wells.

Table 3:  Seasonal concentrations of heavy metals in groundwater

| Parameter (mg/L) | Season | Min | Max | Mean | Median | Std. Dev | WHO (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aluminum (Al) | Dry | 0.491 | 42.652 | 3.92 | 1.018 | 9.50 | 0.2 |
|  | Wet | 0.066 | 1.952 | 0.478 | 0.134 | 0.604 |  |
| Cadmium (Cd) | Dry | 0.0001 | 0.0025 | 0.00071 | 0.00025 | 0.00088 | 0.003 |
|  | Wet | 0.0228 | 0.0240 | 0.0237 | 0.0239 | 0.00036 |  |
| Iron (Fe) | Dry | 0.075 | 22.770 | 2.15 | 0.387 | 5.19 | 0.3 |
|  | Wet | 0.020 | 4.524 | 0.510 | 0.031 | 1.129 |  |
| Boron (B) | Dry | 0.006 | 0.101 | 0.025 | 0.012 | 0.027 | 2.4 |
|  | Wet | 0.036 | 0.439 | 0.123 | 0.119 | 0.082 |  |
| Barium (Ba) | Dry | 0.111 | 1.600 | 0.438 | 0.279 | 0.447 | 0.7 |
|  | Wet | 0.031 | 0.261 | 0.156 | 0.156 | 0.055 |  |
| Cobalt (Co) | Dry | 0.0001 | 0.125 | 0.014 | 0.0030 | 0.031 |  |
|  | Wet | 0.0208 | 0.0271 | 0.0216 | 0.0210 | 0.0016 |  |
| Lead (Pb) | Dry | 0.001 | 0.225 | 0.0189 | 0.003 | 0.0517 | 0.01 |
|  | Wet | 0.029 | 0.053 | 0.038 | 0.036 | 0.006 |  |
| Manganese (Mn) | Dry | 0.009 | 7.123 | 1.28 | 0.283 | 2.11 | 0.4 |
|  | Wet | 0.049 | 0.639 | 0.146 | 0.069 | 0.179 |  |
| Chromium (Cr) | Dry | 0.001 | 0.027 | 0.0043 | 0.001 | 0.0074 | 0.05 |
|  | Wet | 0.028 | 0.033 | 0.030 | 0.0288 | 0.0017 |  |
| Zinc (Zn) | Dry | 0.014 | 2.415 | 0.290 | 0.097 | 0.541 | 3.0 |
|  | Wet | 0.009 | 0.809 | 0.156 | 0.105 | 0.182 |  |
| Copper (Cu) | Dry | 0.001 | 0.123 | 0.0129 | 0.0035 | 0.0275 | 2.0 |
|  | Wet | 0.011 | 0.072 | 0.050 | 0.054 | 0.015 |  |
| Nickel (Ni) | Dry | 0.002 | 0.061 | 0.0105 | 0.006 | 0.0145 | 0.07 |
|  | Wet | 0.0032 | 0.0062 | 0.0050 | 0.0052 | 0.00089 |  |

(Source: Author, 2025)

Boron (B) remained well within the WHO maximum permissible limit of 2.4 mg/L in both seasons, with mean concentrations of 0.025 mg/L (dry) and 0.123 mg/L (wet), indicating minimal health risk.

Barium (Ba) concentrations averaged 0.438 mg/L (dry) and 0.156 mg/L (wet), both below the WHO health-based guideline of 0.7 mg/L. This suggests that the groundwater is generally safe regarding barium toxicity (WHO, 2022).

Cobalt (Co) showed a seasonal increase from 0.014 mg/L in the dry season to 0.0216 mg/L in the wet. Although WHO has not set a guideline for cobalt, elevated levels may point to anthropogenic influences such as battery and metal waste (ATSDR, 2020).

Lead (Pb) exceeded the WHO limit of 0.01 mg/L in both seasons. The dry season mean was 0.0189 mg/L, and the wet season was even higher at 0.038 mg/L. This is a significant concern, as lead is highly toxic and can cause neurological and developmental damage, especially in children (WHO, 2022). The high lead levels can be linked to anthropogenic activities such as emissions from high traffic density, improper disposal of batteries, and waste burning in Oyo Town. Such sources intensify during the wet season when leaching and infiltration mobilize lead into groundwater.

Manganese (Mn) averaged 1.28 mg/L in the dry season—well above the WHO limit of 0.4 mg/L. However, wet season levels (0.146 mg/L) remained within acceptable range. Elevated manganese can cause staining, taste issues, and potential neurological effects at higher exposures (WHO, 2022). Given the geology of the Basement Complex terrain, natural geogenic weathering is expected; however, improper waste management and runoff from open markets may also enhance Mn loading in some locations.

Chromium (Cr) levels in both seasons were within the WHO limit of 0.05 mg/L. The wet season had a higher mean (0.030 mg/L) compared to the dry (0.0043 mg/L), possibly due to leaching from surface sources during rainfall.

Zinc (Zn) concentrations were safely within the WHO guideline of 3.0 mg/L, with 0.290 mg/L (dry) and 0.156 mg/L (wet). No health risk is expected from zinc at these levels.

Copper (Cu) values were far below the WHO limit of 2.0 mg/L in both seasons, with means of 0.0129 mg/L (dry) and 0.050 mg/L (wet), indicating safe levels.

Nickel (Ni) concentrations also remained within WHO limits (0.07 mg/L), with 0.0105 mg/L in the dry season and 0.0050 mg/L in the wet season.

3.2 Seasonal Variability (ANOVA)

Statistically evaluate the seasonal differences in heavy metal concentrations within groundwater from the study area, a one-way Analysis of Variance (ANOVA) was conducted for each parameter. Table 4 show comparison of the average concentration of the different heavy metals for both season of the year. The results show that seven (7) of the twelve (12) analyzed heavy metals exhibit statistically significant seasonal variation (p < 0.05), indicating that their concentrations differ meaningfully between wet and dry periods. Specifically, aluminum (Al) showed a highly significant difference (p = 0.0003), with concentrations markedly higher in the dry season. This pattern suggests reduced dilution during the dry period and possible geogenic contributions from weathered alumino-silicate minerals. Similarly, cadmium (Cd) presented a significant seasonal variation (p < 0.0001), with elevated wet season levels possibly linked to leaching from agricultural inputs and surface contamination. Other metals that showed significant seasonal effects include boron (B), barium (Ba), manganese (Mn), chromium (Cr), and copper (Cu). These variations may result from differences in groundwater recharge, oxidation conditions, and land use dynamics between seasons.

Table 4: Comparison of average concentration of heavy metals in both season

| Parameter | Dry Mean (mg/L) | Wet Mean (mg/L) | P-Value |
| --- | --- | --- | --- |
| Al | 3.9200 | 0.4780 | 0.0003 |
| Cd | 0.0007 | 0.0237 | <0.0001 |
| Fe | 2.1500 | 0.5100 | 0.3854 |
| B | 0.0250 | 0.1230 | <0.0001 |
| Ba | 0.4380 | 0.1560 | 0.0015 |
| Co | 0.0140 | 0.0216 | 0.4295 |
| Pb | 0.0189 | 0.0380 | 0.2573 |
| Mn | 1.2800 | 0.1460 | 0.0391 |
| Cr | 0.0043 | 0.0300 | <0.0001 |
| Zn | 0.2900 | 0.1560 | 0.2395 |
| Cu | 0.0129 | 0.0500 | 0.0059 |
| Ni | 0.0105 | 0.0050 | 0.1783 |

(Source: Author, 2025)

In contrast, iron (Fe), lead (Pb), cobalt (Co), zinc (Zn), and nickel (Ni) did not exhibit statistically significant differences across seasons (p > 0.05). This may indicate that their sources are more stable and less influenced by seasonal hydrological processes or that background levels persist irrespective of seasonal change.

3.3 Correlation Analysis

The correlation analysis suggests common geochemical origins for some metals but also indicates anthropogenic influences. Table 5 shows the Pearson correlation matrix for the dry season; it revealed several strong positive associations among the heavy metals. Aluminum (Al) exhibited very strong positive correlations with iron (Fe, r = 0.93), zinc (Zn, r = 0.91), copper (Cu, r = 0.93), nickel (Ni, r = 0.90), and lead (Pb, r = 0.94), indicating common geochemical sources or similar geochemical behavior in groundwater. Such patterns often reflect lithogenic origins from weathering of aluminosilicate and sulfide minerals, as well as possible anthropogenic contributions (Jolaosho et al., 2024; Aladejana et al., 2020).

Iron (Fe) also demonstrated high correlations with zinc (r = 0.98) and copper (r = 0.95), suggesting mobilization through related hydrogeochemical processes such as mineral dissolution or contamination from agricultural inputs.

Cobalt (Co) correlated strongly with barium (Ba, r = 0.87), while manganese (Mn) showed moderate to strong associations with aluminum (r = 0.59), barium (r = 0.64), and cobalt (r = 0.76), which may indicate co-occurrence from ferromanganese oxides and hydroxides. In contrast, boron (B) displayed generally weak and negative correlations with most metals, implying a different source or geochemical pathway, possibly linked to geogenic background rather than anthropogenic influx.

|  | Al ### mg/L | Cd ### mg/L | Fe ### mg/L | B ### mg/L | Ba ### mg/L | Co ### mg/L | Pb mg/L | Mn mg/L | Cr mg/L | Zn mg/L | Cu mg/L | Ni mg/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### Al mg/L | ### 1 | ### 0.52 | ### 0.93 | ### -0.07 | ### 0.52 | ### 0.80 | 0.94 | 0.59 | 0.69 | 0.91 | 0.93 | 0.89 |
| ### Cd mg/L | ### 0.52 | ### 1 | ### 0.93 | ### -0.07 | ### 0.61 | ### 0.51 | 0.60 | 0.42 | 0.37 | 0.57 | 0.56 | 0.2 |
| ### Fe mg/L | ### 0.93 | ### 0.57 | ### 1 | ### 0.15 | ### 0.50 | ### 0.79 | 0.98 | 0.64 | 0.71 | 0.98 | 0.95 | 0.92 |
| ### B mg/L | ### -0.07 | ### -0.07 | ### 0.15 | ### 1 | ### -0.17 | ### -0.11 | 0.02 | 0.12 | -0.02 | 0.09 | -0.02 | 0.11 |
| ### Ba mg/L | ### 0.52 | ### 0.61 | ### 0.50 | ### -0.17 | ### 1 | ### 0.87 | 0.51 | 0.64 | 0.29 | 0.49 | 0.49 | 0.66 |
| ### Co mg/L | ### 0.80 | ### 0.51 | ### 0.79 | ### -0.11 | ### 0.87 | ### 1 | 0.79 | 0.76 | 0.56 | 0.78 | 0.96 | 0.94 |
| ### Pb mg/L | ### 0.94 | ### 0.60 | ### 0.98 | ### 0.02 | ### 0.51 | ### 0.79 | 1 | 0.62 | 0.74 | 0.98 | 0.96 | 0.94 |
| ### Mn mg/L | ### 0.59 | ### 0.42 | ### 0.64 | ### 0.12 | ### 0.64 | ### 0.76 | 0.62 | 1 | 0.38 | 0.64 | 0.59 | 0.58 |
| ### Cr mg/L | ### 0.69 | ### 0.37 | ### 0.71 | ### -0.02 | ### 0.29 | ### 0.56 | 0.74 | 0.38 | 1 | 0.69 | 0.74 | 0.63 |
| ### Zn mg/L | ### 0.91 | ### 0.57 | ### 0.98 | ### 0.09 | ### 0.49 | ### 0.78 | 0.98 | 0.64 | 0.69 | 1 | 0.98 | 0.93 |
| ### Cu mg/L | ### 0.93 | ### 0.56 | ### 0.95 | ### -0.02 | ### 0.49 | ### 0.79 | 0.96 | 0.59 | 0.74 | 0.98 | 1 | 0.92 |
| ### Ni mg/L | ### 0.89 | ### 0.74 | ### 0.92 | ### -0.11 | ### 0.66 | ### 0.82 | 0.94 | 0.58 | 0.63 | 0.93 | 0.92 | 1 |

Table 5: Correlation Analysis for Dry season

### (Source: Author, 2025)

### The wet season correlation matrix (Table 6) showed a different interaction pattern. Cobalt (Co) exhibited a very strong correlation with iron (Fe, r = 0.99) and manganese (Mn, r = 0.91), highlighting their possible joint mobilization during wetter periods, likely due to redox changes or increased leaching. Lead (Pb) correlated strongly with chromium (Cr, r = 0.84) and zinc (Zn, r = 0.74), suggesting lithogenic sources or similar anthropogenic influences such as industrial effluents and vehicular emissions (Krishna & Mohan, 2014). Cadmium (Cd) showed a high correlation with barium (Ba, r = 0.78), implying shared origins, possibly from mineral dissolution or fertilizer residues. Aluminium (Al) maintained strong correlations with chromium (r = 0.79) and moderate correlations with lead (r = 0.56) and zinc (r = 0.59), pointing towards aluminosilicate weathering and associated trace metal release. Conversely Copper (Cu), however, displayed negative correlations with some metals, especially cadmium (r = –0.49) and nickel (r = –0.48), indicating it may be influenced by localized contamination sources such as plumbing corrosion or specific industrial discharges (Ojekunle et al., 2022). For instance, strong associations of Pb with Cr and Zn in the wet season (Table 6) imply shared contamination pathways from vehicular emissions, roadside workshops, and waste burning, all of which are prevalent land-use activities in Oyo town. Similarly, Cd’s strong correlation with Ba points toward fertilizer residues in agricultural zones

### Table 6: Correlation Analysis for wet season

|  | ### Al mg/L | ### Cd mg/L | ### Fe mg/L | ### B mg/L | ### Ba mg/L | ### Co mg/L | Pb mg/L | Mn mg/L | Cr mg/L | Zn mg/L | Cu mg/L | Ni mg/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### Al mg/L | ### 1 | ### 0.19 | ### 0.35 | ### -0.11 | ### 0.49 | ### 0.28 | 0.56 | 0.28 | 0.79 | 0.59 | -0.06 | 0.49 |
| ### Cdmg/L | ### 0.19 | ### 1 | ### -0.15 | ### 0.39 | ### 0.78 | ### -0.21 | 0.33 | -0.09 | 0.18 | 0.23 | -0.49 | 0.51 |
| ### Femg/L | ### 0.35 | ### -0.15 | ### 1 | ### -0.13 | ### 0.09 | ### 0.99 | 0.67 | 0.88 | 0.70 | 0.49 | -0.13 | 0.43 |
| ### B mg/L | ### -0.11 | ### 0.39 | ### -0.13 | ### 1 | ### 0.23 | ### -0.13 | -0.06 | -0.08 | -0.10 | -0.14 | -0.02 | -0.09 |
| ### Bamg/L | ### 0.49 | ### 0.78 | ### 0.09 | ### 0.23 | ### 1 | ### 0.04 | 0.52 | 0.19 | 0.46 | 0.31 | -0.36 | 0.54 |
| ### Comg/L | ### 0.28 | ### -0.21 | ### 0.99 | ### -0.13 | ### 0.04 | ### 1 | 0.65 | 0.91 | 0.65 | 0.47 | -0.08 | 0.36 |
| ### Pbmg/L | ### 0.56 | ### 0.33 | ### 0.67 | ### -0.06 | ### 0.52 | ### 0.65 | 1 | 0.77 | 0.84 | 0.74 | -0.34 | 0.56 |
| ### Mnmg/L | ### 0.28 | ### -0.09 | ### 0.88 | ### -0.08 | ### 0.19 | ### 0.91 | 0.77 | 1 | 0.69 | 0.48 | -0.12 | 0.36 |
| ### Crmg/L | ### 0.79 | ### 0.18 | ### 0.70 | ### -0.10 | ### 0.46 | ### 0.65 | 0.84 | 0.69 | 1 | 0.62 | -0.17 | 0.67 |
| ### Znmg/L | ### 0.59 | ### 0.23 | ### 0.49 | ### -0.14 | ### 0.31 | ### 0.47 | 0.74 | 0.48 | 0.62 | 1 | -0.36 | 0.47 |
| ### Cumg/L | ### -0.06 | ### -0.49 | ### -0.13 | ### -0.02 | ### -0.36 | ### -0.08 | -0.34 | -0.12 | -0.17 | -0.36 | 1 | -0.48 |
| ### Nimg/L | ### 0.49 | ### 0.51 | ### 0.43 | ### -0.09 | ### 0.54 | ### 0.36 | 0.56 | 0.36 | 0.67 | 0.47 | -0.48 | 1 |

(Source: Author, 2025)

3.4. Health Risk Assessment

The Hazard Index (HI) values for the twenty groundwater sampling locations indicates notable seasonal and demographic variations in potential non-carcinogenic health risks. Table (7) presents the Hazard Index values for adults and children during the dry season, while Figure (2) illustrates the same data in bar chart format. The results show that HI values for adults exceeded the safety threshold of unity (HI > 1) in only three locations (15%), with a maximum HI of 3.16, primarily attributable to manganese (Mn) and chromium (Cr) exposure. In contrast, children exhibited higher susceptibility, with HI exceedances in eight locations (40%) and a maximum HI reaching 7.37. Manganese remained the dominant risk driver for children in this period, consistent with its neurotoxic potential at elevated exposure levels (ATSDR, 2022).

Table (8) presents the Hazard Index values for adults and children during the wet season, while Figure (3) provides a bar chart visualization of the same data. The results reveal a more critical scenario compared to the dry season, as HI values exceeded unity (HI > 1) in all sampling locations (100%) for both adults and children. For adults, HI values reached up to 4.85, with cadmium (Cd) and chromium (Cr) identified as the major contributors. Children were significantly more vulnerable, with HI values ranging as high as 11.42, predominantly driven by Cd and Cr exposure. The widespread exceedances during the wet season underscore the influence of land-use activities such as agricultural runoff, leaching from waste disposal sites, and infiltration of contaminated surface water, which intensify during periods of high rainfall (Guo et al., 2018; Pakoksung et al., 2025; Parvin et al., 2021).

Table 7:  Hazard Index for Dry season Adult and Children

| Location | Hazard Index for Adult | Hazard Index for Children |
| --- | --- | --- |
| Cele | 0.254392 | 0.593582 |
| Isokun road | 0.097637 | 0.22782 |
| Alayande road | 0.056297 | 0.13136 |
| Araromi 1 | 0.063916 | 0.149138 |
| Araromi 2 | 0.076719 | 0.179011 |
| Dankanka | 1.16429 | 2.716677 |
| Mobolaje 1 | 0.559705 | 1.305979 |
| Mobolaje 2 | 0.186841 | 0.435963 |
| Bola | 0.295249 | 0.688915 |
| Adesina | 0.500138 | 1.166989 |
| Oke -Afin | 0.146968 | 0.342926 |
| Sakutu | 0.489127 | 1.141296 |
| Oke Alapo | 0.809177 | 1.888079 |
| Lagbodoko | 0.077467 | 0.180757 |
| Iyalamu | 0.323859 | 0.755672 |
| Oke ebo 1 | 0.193247 | 0.45091 |
| Oke ebo 2 | 1.472726 | 3.43636 |
| Baago | 3.15915 | 7.371349 |
| Oluajo | 0.775934 | 1.810513 |
| Bode Thomas | 0.241957 | 0.564566 |

(Source: Author, 2025)

Table 8: Hazard Index for Wet Adult and Child

| Location | Hazard for Adults | Hazard for Children |
| --- | --- | --- |
| Cele | 2.409653 | 5.622523 |
| Isokun road | 2.500779 | 5.835151 |
| Alayande road | 2.513313 | 5.864398 |
| Araromi 1 | 2.449875 | 5.716375 |
| Araromi 2 | 2.640124 | 6.16029 |
| Dankanka | 2.406545 | 5.615271 |
| Mobolaje 1 | 2.518302 | 5.876038 |
| Mobolaje 2 | 2.393023 | 5.58372 |
| Bola | 2.426247 | 5.661243 |
| Adesina | 2.288819 | 5.340577 |
| Oke -Afin | 2.408878 | 5.620715 |
| Sakutu | 2.379800 | 5.552866 |
| Oke Alapo | 2.624184 | 6.123097 |
| Lagbodoko | 2.390945 | 5.578873 |
| Iyalamu | 2.562134 | 5.978314 |
| Oke ebo 1 | 2.291726 | 5.347361 |
| Oke ebo 2 | 2.410479 | 5.624451 |
| Baago | 2.417179 | 5.640084 |
| Oluajo | 2.403118 | 5.607276 |
| Bode Thomas | 2.641133 | 6.162643 |

(Source: Author, 2025)

Figure 2: Hazard Index (HI) values for adults and children during the dry season across groundwater sampling locations in Oyo Town. The red dashed line represents the safety threshold (HI = 1). Children consistently show higher HI values than adults, with exceedances observed in 40% of locations. (Source: Author, 2025)

Figure 3: Hazard Index (HI) values for adults and children during the wet season across groundwater sampling locations in Oyo Town. All locations recorded HI values greater than unity, with significantly higher risks for children compared to adults. (Source: Author, 2025)

A seasonal comparison is shown in Figure 4, where the contrast between wet and dry seasons highlights the pronounced vulnerability of children and the widespread exceedance of HI in the wet season, this is likely due to enhanced mobilization of metals into groundwater through surface runoff, leaching of contaminated soils, and intensive land-use activities in Oyo Town, particularly poorly managed waste dumpsites, roadside mechanic workshops, open markets, and agricultural fields. (Agaku et al., 2025; Iqbal et al., 2023). The higher vulnerability of children compared to adults can be attributed to their lower body weight, higher water intake per body mass unit, and greater physiological sensitivity to toxicants (USEPA, 2011).

Figure 4: The plot highlights the marked increase in HI values during the wet season, with children showing consistently greater vulnerability across all locations. (Source: Author, 2025)

4.0 CONCLUSION

This study assessed the seasonal variability of heavy metals in shallow groundwater from selected areas of Oyo Town, Southwestern Nigeria, and their associated non-carcinogenic health risks. The findings revealed that aluminum (Al), cadmium (Cd), lead (Pb), iron (Fe), and manganese (Mn) frequently exceeded WHO guideline values across both wet and dry seasons. Seasonal variations were evident: geogenic-driven metals such as Al, Fe, and Mn were elevated during the dry season due to reduced dilution, mineral weathering, and redox conditions, while anthropogenic-driven metals including Cd, Cr, Pb, B, and Cu were more pronounced in the wet season, influenced by fertilizer application, leaching from dumpsites, and surface runoff. Health risk assessment indicated that children were more vulnerable than adults, with Hazard Index (HI) values exceeding the safe threshold in 40% of locations during the dry season and in all locations during the wet season, where Cd and Cr were the dominant risk drivers.

These results highlight serious health implications. Elevated levels of Pb, Cd, and Mn pose risks of neurological, renal, and developmental disorders, particularly in children, while the widespread exceedance of HI during the wet season underscores the vulnerability of shallow aquifers to both natural and anthropogenic contamination. The findings demonstrate that Oyo Town, like many rapidly urbanizing Nigerian communities, faces significant water quality challenges that threaten the sustainability of groundwater as a safe drinking water source.

To mitigate these risks, urgent interventions are required. Regular groundwater quality monitoring should be institutionalized to detect contamination trends early. Strict regulation of waste management and agricultural practices is needed to minimize leaching of toxic metals. Communities should be sensitized to the health risks associated with contaminated groundwater and encouraged to adopt household-level treatment methods such as filtration or boiling where feasible. In the long term, provision of alternative safe water sources, improved sanitation, and enforcement of environmental protection laws are essential to safeguard public health and ensure sustainable groundwater use in Oyo Town.

Disclaimer (Artificial intelligence)

Author(s) hereby declares that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

- Adimalla, N., and Qian, H. (2021). Groundwater quality evaluation using water quality index (WQI) for drinking purposes and human health risk (HHR) assessment in an agricultural region of Nanganur, South India. Ecotoxicology and Environmental Safety, 207, 111277. https://doi.org/10.1016/j.ecoenv.2020.111277

- Agaku, R.M., Ikyumbur, J.T., Amanyi M.I., and Onwoke, E.E. (2025). Assessment of Heavy Metals Concentration in Ground Water in Gboko, Nigeria. Nigerian Journal of Physics, 34(1), 122-133. https://doi.org/10.62292/njp.v34i1.2025.390.

- Agency for Toxic Substances and Disease Registry (ATSDR). (2020). Toxicological profile for lead. U.S. Department of Health and Human Services, Public Health Service. https://www.atsdr.cdc.gov/toxprofiles/tp13.pdf

- Agency for Toxic Substances and Disease Registry. (2022). Toxicological profiles. U.S. Department of Health and Human Services. https://ncbi.nlm.nih.gov/books/NBK153665/

- Akanbi, O. A., Falana, O. A., and Akande, I. O. (2023). Groundwater-Bearing Potential of the permanent site of Ajayi Crowther University, Offa-Meta, Oyo, southwestern Nigeria. Ajayi Crowther Journal of Pure and Applied Sciences. https://acjpas.acu.edu.ng/index.php/acjpas/article/view/82

- Aladejana, J.A., Kalin, R.M., Sentenac, P., and Hassan, I. (2020). Assessing the impact of climate change on groundwater quality of the shallow coastal aquifer of Eastern Dahomey Basin, Southwestern Nigeria. Water, 12(1), 224. https://doi.org/10.3390/w12010224

- APHA (2017). Standard Methods for the Examination of Water and Wastewater. 23rd Edition. American Public Health Association

- David, O.O. (2013). Sustainable Groundwater Exploitation in Nigeria. Journal of Water Resources and Ocean Science, 2(2), 9-14. https://doi.org/10.11648/j.wros.20130202.11

- Guo, X., Wang, C., Liu, Y., Yang, S., Dong, J., and Shi, G. (2018). Seasonal and spatial variability of anthropogenic and natural pollution in groundwater under monsoonal climate conditions. International Journal of Environmental Research and Public Health, 15(2), 279. https://doi.org/10.3390/ijerph15020279

- Hameed, M., Khurshid, Z., Bhat, R.A., and Qayoom, I. (2020). Concerns and threats of heavy metals' contamination on aquatic ecosystem. In V. S. Sharma & A. K. Varma (Eds.), Bioremediation and biotechnology (Vol. 4): Techniques for noxious substances remediation (pp. 1–21). Springer. https://doi.org/10.1007/978-3-030-48690-7_1

- Jolaosho, T. L., Mustapha, A. A., and Hundeyin, S. T. (2024). Hydrogeochemical evolution and heavy metal characterization of groundwater from southwestern Nigeria: An integrated assessment using spatial, indexical, irrigation, chemometric, and health risk models. Heliyon, 10(19), e38364. https://doi.org/10.1016/j.heliyon.2024.e38364

- Krishna, A.K., and Mohan, K.R. (2014) Risk assessment of heavy metals and their source distribution in waters of a contaminated industrial site. Environ Sci Pollut Res 21, 3653–3669. https://doi.org/10.1007/s11356-013-2359-5

- Mmonwuba, N. C., Francis, O., Somtochukwu, O., Chinweike, M. P., Ezenwafor, K., and Ifunanya, G. E. (2023). Effects of solid wastes (leachates) on the quality of underground water. Journal of Engineering Research and Reports, 24(3), 20-31. https://doi.org/10.9734/jerr/2023/v24i3805

- Nigerian Geological Survey Agency. (2016). Geological map of Nigeria. Nigerian Geological Survey Agency.

- Ojekunle, O.Z., Rasaki, A., Taiwo, A.M., Adegoke, K.A., Balogun, M.A., Ojekunle, O.O., Anumah, A.O., Ibrahim, A.O., and Adeyemi, A. (2022). Health risk assessment of heavy metals in drinking water leaching through improperly managed dumpsite waste in Kurata, Ijoko, Sango area of Ogun State, Nigeria. Groundwater for Sustainable Development, 18, 100792. https://doi.org/10.1016/j.gsd.2022.100792

- Pakoksung, K., Shimoyama, Y., Michishita, R., and Gunawan, H. (2025). Seasonal dynamics of water quality in response to land-use change: runoff-driven pollutant loading during the wet season. Scientific Reports, 15, 91820. https://doi.org/10.1038/s41598-025-91820-4

- Parvin, F., Hasan, M., and Chowdhury, M. S. (2021). Impact of landfill leachate contamination on surface and groundwater quality: a case study. Applied Water Science, 11, 95. https://doi.org/10.1007/s13201-021-01431-3

- Sakakibara, K., Tsujimura, M., Song, X., and Zhang, J. (2017). Spatiotemporal variation of the surface water effect on groundwater recharge in a low-precipitation region: Application of a multi-tracer approach to the Taihang Mountains, North China. Journal of Hydrology, 545, 132–144. https://doi.org/10.1016/j.jhydrol.2016.12.030

- Sheng, D., Meng, X., Wen, X., Wu, J., Yu, H., & Wu, M. (2022). Contamination Characteristics, Source Identification, and Source-Specific Health Risks of Heavy Metal (loid)s in Groundwater of an Arid Oasis Region in Northwest China. Science of the Total Environment, 841, Article ID: 156733. https://doi.org/10.1016/j.scitotenv.2022.156733

- Thongyuan, S., Khantamoon, T., Aendo, P., Binot, A., and Tulayakul, P. (2021). Ecological and Health Risk Assessment, Carcinogenic and Non-Carcinogenic Effects of Heavy Metals Contamination in the Soil from Municipal Solid Waste Landfill in Central, Thailand. Human and Ecological Risk Assessment: An International Journal, 27, 876-897. https://doi.org/10.1080/10807039.2020.1786666

- Tian, R., and Wu, J. (2019). Groundwater Quality Appraisal by Improved Set Pair Analysis with Game Theory Weightage and Health Risk Estimation of Contaminants for Xuecha Drinking Water Source in a Loess Area in Northwest China. Human and Ecological Risk Assessment: An International Journal, 25, 132-157. https://doi.org/10.1080/10807039.2019.1573035

- U.S. Environmental Protection Agency (U.S. EPA). (1984). Health Effects Assessment for Boron and Compounds. Environmental Criteria and Assessment Office.

- U.S. Environmental Protection Agency (U.S. EPA). (2004). Integrated Risk Information System (IRIS): Cadmium; CASRN 7440-43-9. https://iris.epa.gov/

- United States Environmental Protection Agency (USEPA). (2023). Integrated Risk Information System (IRIS). https://www.epa.gov/iris

- Wali, S.U., Usman, A.A., Danladi, P., Usman, H.M., and Aliyu, S. (2025). Assessment of heavy metals (As, Ba, Co and Ni), pollution in rural drinking water sources, using correlation and principal component analysis in Western Sokoto Basin, Nigeria. Int J Hydro. 2025;9(2):60‒72.DOI: 10.15406/ijh.2025.09.00404

- World Health Organization (WHO). (2003). Lead in Drinking-water: Background document for development of WHO Guidelines for Drinking-water Quality. WHO Press.

- World Health Organization (WHO). (2022). Guidelines for drinking-water quality: Fourth edition incorporating the first and second addenda. Geneva: World Health Organization

- Ali, M., & Mubarak, S. (2017). Approaches and methods of quantifying natural groundwater recharge–a review. Asian Journal of Environment & Ecology, 5(1), 1-27. DOI:10.9734/AJEE/2017/36987