Original Research Article

Assessment of some Heavy metals level in farmland along OPM- Aluu dumpsite in Ikwerre LGA, Rivers State, Nigeria

Abstract

The assessment of some heavy metal levels in farmland along OPM –ALUU Dumpsite near PortHarcourt was carried out using soil auger and Atomic Absorption spectrophotometer (AAS). The samples collected from site Allu 1 and Aluu 2 representing farmland near dumpsite and control soil respectively. The results obtained showed lead (Pb) 97.5mg/kg, Arsenic (As) 29.5mg/kg, cadmium (Cd) 0.82mglkg, Copper (Cu) 37.2mg/kg, Zinc (Zu) 64.8mg/kg, Chromium(Cu) 108.5mg/kg, Mangenese 58.3mg/kg and Nickel (Ni) 31.0mg/kg. These results are all higher than world Health organization (WHO) limit apart from Nickel (Ni) that is lower than the limit. Similarly, all the results were higher above the control soil apart from cadmium (Cd) and Nickel (Ni) that were not detected. Base on this result we concluded that the site is polluted with heavy metals, and should be abandoned for farming activities. Similarly people already farming in this area should be discouraged and made to go for routine medical exercise to ascertain the level of heavy metals in their body system. Finally, further studies using other analytical equipment should be used apart from atomic Absorption spectrophotometer, to authenticate our report.

Keywords: Assessment, farmland, dumpsites, heavy metals, AAS.

Introduction

Soil is very essential for plant sustenance, animal lives, sinking and recycling of both solid and liquid wastes as well as production of qualitative foods.

In Nigeria as a developing country, there are inadequate disposal facilities and waste management regulation enforcement, causing uncontrolled or indiscriminate dumping of wastes near urban cities, residential buildings, roadsides, farmlands and open spaces (Afolayan. 2018). According to Amusan et al., (2005), heavy metals are been introduced into the soil through waste generation from anthropogenic activities. These wastes resulting from human activities may include plastics, food waste, sewage, papers, batteries and metals.

Municipal wastes may contain metals like Arsenic (As), Cadmium(Cd), Cobalt (Co), Iron(Fe), Nickel (Ni), Zinc(Zn), manganese (Mn), mercury(Hg), Lead (Pb) copper(Cu) and chromium (Cr), which may be housed in the soil as the sink when they are leached out from the dump sites. Heavy metals are described as having densities higher or more than 5g/cm3. Some heavy metals like Iron (Fc) Nickel (N) are very essential to all forms of life in their low concentrations (Johnny and Lead, 2014). But the likes of Lead(Pb), Cadmium(Cd) and Mercury (Hg) are toxic and harmful to living organisms even in their low concentration, as such can Cause abnormalities in metabolic functions especially in higher Concentration  (Manaham, 2001).

Oyedele et al., (2008) reported that the presence of Heavy metals in soil through fertilizers, pesticides, industrial discharge, animal manures and other waste can raise concern because they are non-biodegradable.

Again, in their higher concentrations, physical, chemical and biological functionalities of soil are hampered causing toxicity in human beings, animals and plants (Shokr et al., 2016). However, according to Ebong et al., (2018), leachates from dumpsites may contain heavy metals that could scale up soil pollution due to the fact that metal depositions are caused by anthropogenic activities, they tend to be more mobile in soil.

Ayeimi et al., (2017) conducted an analysis of heavy metals in soil, Plants and herbivorous insects within Ado Ekiti dumpsite. The results showed presence of heavy metal contamination.

Adefemi and Awokunmi (2013) implicated the consumption of crops contamination by heavy metals to leading cause of health dysfunctional, including psychosocial faculties, intrauterine growth retardation, low immunological deficiencies, upper gastrointestinal cancer, kidney failure and stomatitis. This is why it is necessary to monitor the contamination level of soil constantly.

MATERIALS AND METHODS

STUDY SITE

Aluu is a community in Ikwere Local Government Area of Rivers State. It is geographical located at 405611uN - 60561 58uE. It is a residential and suburban area known for its proximity to university of Port Harcourt.

The people of Aluu are predominantly farmers. Some are traders, while few are fishermen owing to the Choba River that run across the community. The major crops of Aluu people are palm trees, cassava, maize, plantain, banana vegetables, yams etc as their food as well as for their income earnings.

MAP OF THE STUDY AREA

Fig 1: Map showing study locations of the work

SAMPLE COLLECTION

Soil samples were collected from farmlands (Lat. 4.907605, Long. 6.963782) very close to the waste dumpsite using  soil auger and a spatula, firstly at a depth of 0 – 15cm and secondly at 15 – 30cm. These two samples were mixed to form a composite sample from which a laboratory sample was obtained by grab method and labeled Aluu1. Another soil away from the dump site 4.9323305, 6.9494600, using the same procedure and labeled as Aluu2 serves as a control. These samples Aluu1 and Aluu2 were air dried and stored in labeled polythene bags and transported to the laboratory for analysis.

SAMPLE PREPARATIOM

The soil samples were oven dried at 95-105℃ to a constant weight. All glass wares were thoroughly washed with distilled water, soaked in diluted Nitric acid for 24 hours and then rinsed with distilled water to make sure is free from impurities.

DRY ASH OF THE SOIL SAMPLES

The dry soil Samples were crushed gently and sieved to collect 63gm gram size. 2.0g of the Sample was weighed into a pre-weighed dry crucible. The crucible was covered and transferred into a muffle furnace and the temperature was gradually increased to about 5000c for 7200 minutes. The samples were removed and allowed to cool to room temperature in a desiccator and then placed in acid-washed glass beakers.

DIGESTION OF THE SOIL SAMPLES

The digestion was carried out based on the analytical methods for Atomic absorption spectrometry. Here, the samples were digested by the addition of 10ml of 0.25ml HNO3 heated to dryness and thereafter 10ml of 0.25m HNO3 and 3ml of HClO3 were added. The solution was then heated in a fume chamber. Sample solution was obtained by leaching the residues with 4.0ml of HCl and thereafter filtered and diluted with distilled water to 100 ml mark.

HEAVY METAL ANALYSIS OF THE SOIL SAMPLES

The concentrations of the selected heavy metals in the soil samples after digestion were analyzed using atomic absorption spectrometer (Burk Scientific model 2010).

RESULTS AND DISCUSSION

Table 1 Results of heavy metals concentration in farmland close to OPM – Aluu Waste Dumpsite

| S/N | HEAVY METAL | ALUU1 (mg/kg) | ALUU2 (mg/kg) | WHO LIMIT |
| --- | --- | --- | --- | --- |
|  | Lead (Pb) | 97.5 | 20.2 | 85 |
|  | Arsenic (As) | 29.5 | 4.8 | 20 |
|  | Cadmium (Cd) | 0.82 | ND | 0.8 |
|  | Copper (Cu) | 37.2 | 12.1 | 36 |
|  | Zinc (Zn) | 64.8 | 15.0 | 50 |
|  | Chromium (Cr) | 102.5 | 10.8 | 100 |
|  | Manganese (Mn) | 585.3 | 50.7 | 500 |
|  | Nickel (Ni) | 31.0 | ND | 35 |

ND:  NOT Detectable

Fig 2: A plot of Heavy Metal Concentration against WHO limit

Fig.3: A plot of Heavy Metal Concentration against control

The results of heavy metal concentrations in soil from farmland around OPM – Aluu Road dumpsite and the control soil are presented in table1 and illustrated in figures 2 and 3 respectively.

The concentrations of these heavy metals showed Lead (Pb) 975mg/kg, Arsenic (As) 29.5 mg/kg, Cadmium (Cd) 0.82mg/kg, Copper (Cu) 30.2mg/kg, Zinc (Zn) 64.8 mg/kg, Chromium (Cr) 103.5 mg/kg, Manganese (Mn) 585.3 mg/kg and Nickel (Ni) 31.0 mg/kg. Generally, the results obtained showed that the concentrations of heavy metals from farmland around OPM – Aluu road dumpsite were higher than the World Health Organization (WHO) limit except Nickel (Ni) 31.0 mg/kg which has fair less concentrations below its WHO limit of 35 mg/kg. All the Heavy metals from the farmland near the dumpsite were all higher than their control soil, and this is in order with the results obtained from (Amusan et al., 2005) and (Ite, 2023) on the characteristics of soils and crops uptake of heavy metals municipal waste dump site in Nigeria. This could be traced to the availability of metal-containing wastes in the dumpsite that may have leached into the underlying soil.

The presence of Lead (Pb) in this agricultural soil sample may be implicated through pesticides. Improper dumping of old lead batteries and other lead compounds. At a low concentration, lead can be hazardous in soil and may trigger physical systems when human are exposed to it constantly. Lead can equally endanger the food chain and thereby causing adverse human health effect when contaminated farm products are consumed. This work reported Lead (Pb) concentration of 97.5 mg/kg exceed the 85 mg/kg WHO limit and 20.2 mg/kg of the control soil sample.

The presence of Arsenic (As) in agricultural soil is as a result of anthropogenic activities. Chopra et al., (2007) and ASTDR, (2005) reported that inorganic forms of arsenic are considered more harmful than the organic form. High concentration of arsenic could be deadly even in short periods, but long term exposure to low concentration of arsenic may not be deadly. High concentrations of arsenic can cause damage to plant roots and stunted growth (Mizra et al., 2014).

The concentration of cadmium (Cd) in this work is 0.82 mg/kg which is slightly higher than 0.8 mg/kg WHO limit. Its concentration was not detected by the AAs in the control soil sample. It is reported that when Cd concentration in soil exceeds 8.0 mg/kg or the bio-available Cd becomes < 0.001 mg/kg, it causes physiological, structural and chemical changes in plant (Karaca et al., 2010).

The copper (Cu) concentration from the farmland dumpsite is recorded as 37.2 mg/kg. This is higher than 36.mg/kg WHO limit and 12.1 mg/kg control soil. Researches had implicated high concentration of copper to plant toxicity, leading to impaired growth, oxidative stress and damage to cellular structures and functions. These conditions may disrupt photosynthesis and nutrient uptake and leads to plant death.

Presence of copper in this work may be traced to copper metals and wires deposited several years that may have been eroded ad leached into the soil.  (Luisa and Miguel 2006, Arit et al., 2016)

Zinc (Zn) from the farmland near the dumpsite was recorded as 64.8 mg/kg against 50 mg/kg and 15 mg/kg of WHO and control soil respectively. The concentration of zinc in this work is higher than the acceptable limits. This may have come from some scrap metals, rods and steels in the ground. Soil disruption can be implicated as a result of high zinc concentrations in soil, impacting negatively on the microbial community that are crucial for nutrient cycling, thereby affecting soil health and plant growth. (Dupley et al., 2014) (Mir et al., 2021)

Chromium (Cr) recorded in this work has a concentration of 108.5 mg/kg. This value is higher than 100 mg/kg WHO limit and 10.8 mg/kg from the control soil. The introduction of chromium into the environment is mainly through sewage and fertilizers (Duruibe et al., 2018). High concentrations of chromium can delay seed germination and seedling growth by interfering with enzymes. It can be toxic to plant and damage plant cellular processes (Dupley et al., 2014).

Manganese (Mn) concentration reported in this work is 585.3 mg/kg. This value is high than the WHO limit of 500 mg/kg and control soil sample of 50.7 mg/kg. High concentration of manganese can be very toxic to plant causing necrotic brown spots on leaves and chlorosis in young plant leaves. At high level of manganese photosystem can be damaged, it could interfere with plants ability to absorb and translocate other essential minerals like phosphorus, calcium among others.

Manganese could be implicate here to have result from the use of certain fertilizers and pesticides, combustion of fossil fuels and gasoline containers and even waste materials disposal of solid waste can contribute to the enhancement of manganese (Emir et al., 2018).

The concentration of Nickel (Ni) from this work is 31 mg/kg. This concentration was lower that the WHO limit of 35 mg/kg. The Atomic absorption spectrophotometer did not detect any Nickel presence in the control soil. Although we have a lot of problems associated with Nickel at high concentration, this work did not implicate Nickel hence its concentration is below the WHO Limit.

Heavy metal pollution not only result in adverse effects on parameter relating to plant quality and yield but also cause changes in the size, composition and activity of microbial community (Yao et al., 2003).

The plant uptake of heavy metals from contaminated soils may results in a great health risk considering food chain implications. Utilization of food crops contaminated with heavy metals is a major food chain route for human exposure. Uptake of heavy metals by crops and subsequent accumulation along the food chain is a potential threat to animal and man’s health (Sprynskyy et al., 2007).

The result of this work showed that there was heavy metal pollution in the soil. The increasing sequence of occurrence of the metals in the farmland is Mn > Cr > Pb > Zn > Cu > Ni > As > Cd

CONCLUSION

Municipal waste dump has been found to enhance the concentration of heavy metals in the environment. The concentration of lead (Pb), Arsenic (As), Cadmium (Cd), Copper (Cu), Zinc (Zn), Chromium (Cr), Manganese (Mn) and Nickel (Ni) was studied. The result showed that the concentration of heavy metals from Aluu1 has significance difference from that of Aluu2. This showed that the location termed Aluu1 had been impacted with heavy metals which may have leached through the waste dump site to the soil, from maybe old batteries, pigments, plastics, plating, phosphates from fertilizers, eroded metals and others.

RECOMMENDATION

- Heavy metal concentration evaluated were higher than the World Health Organization limit and even the control soil values, we recommended that the land should be abandoned for any farming activities until it is well remediated

- The residents leaving within this site should be properly educated on the danger of heavy metals pollution.

- Those eating from agricultural produce from this area should be mandated to go for routinely medical checkup from time to time to ascertain the level of heavy metal intake in their blood and general system.

- The government should be advised to pay compensation to the owners of these farms used for waste dump since they cannot farm on these heavy metals polluted lands.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

Adefemi, O. S., & Awokunmi, E. E. (2013). Uptake of Heavy Metals by Tomato (Lycopersicum esculentus) Grown on Soil Collected from Dumpsites in Ekiti State, South West, Nigeria. International Journal of Chemistry, 5(3), 70-75. https://doi.org/10.5539/ijc.v5n3p70

Afolayan, A. O. (2018). Accumulation of Heavy Metals from Battery Waste in Topsoil, Surface Water, and Garden Grown Maize at Omilende Area, Olodo, Nigeria. Global Challenges, 2(3), 1700090. https://doi.org/10.1002/gch2.201700090

Amusan, A. A., Ige, D. V., & Olawale, R. (2005). Characteristics of soils and crops' uptake of metals in municipal waste dump sites in Nigeria. Journal of Human Ecology, 17(3), 167-171. https://doi.org/10.1080/09709274.2005.11905775

Agency for Toxic Substances and Disease Registry. (2000). Toxicological profile for arsenic (Update), 2000. U.S. Department of Health and Human Services. https://ntrl.ntis.gov/NTRL/dashboard/searchResults/titleDetail/PB2000108021.xhtml

Ayeni, K., Ajayi, O., & Odeyemi, D. (2017). Studies on Heavy Metal Accumulation in Plant, Insect and Soil in a Public Dumpsite in Ado-Ekiti, Ekiti State. International Journal of Sciences: Basic and Applied Research, 36(2), 18-26. https://www.ijsbar.com/vol36/no2/v36n2a2.pdf

Chopra, B. K., Bhat, S., Mikheenko, I. P., Xu, Z., Yang, Y., Luo, X., Chen, H., van Zwieten, L., Lilley, R. M. C., & Zhang, R. (2007). The characteristics of rhizosphere microbes associated with plants in arsenic-contaminated soils from cattle dip sites. Science of the Total Environment, 378(3), 331-342. https://doi.org/10.1016/j.scitotenv.2007.02.036

Duplay, J., Semhi, K., Errais, E., Imfeld, G., Babcsanyi, I., & Perrone, T. (2014). Copper, zinc, lead and cadmium bioavailability and retention in vineyard soils (Rouffach, France): The impact of cultural practices. Geoderma, 230-231, 318-328. https://doi.org/10.1016/j.geoderma.2014.04.022

Duruibe, J. O., Ogwuegbu, M. O. C., & Egwurugwu, J. N. (2007). Heavy metal pollution and human biotoxic effects. International Journal of Physical Sciences, 2(5), 112-118. https://doi.org/10.5897/IJPS.9000289

Ebong, G. A., Akpan, M. M., & Mkpenie, V. N. (2008). Heavy metal contents of municipal and rural dumpsite soils and rate of accumulation by Carica papaya and Talinum triangulare in Uyo, Nigeria. E-Journal of Chemistry, 5(2), 281–290. http://www.e-journals.org/ejchem/vol_5_no_2/281-290.pdf

Šahinović, E., Čivić, H., & Murtić, S. (2018). Manganese pollution in agricultural soils with implications for food safety. Hrana u zdravlju i bolesti, 7(2), 31-36. https://hrcak.srce.hr/220429

Ite B.M (2023)·Assessment of Heavy metals Level in farmland around Bo-Biara  Dump site in B-Dere Gokana Local Government Ares, Rivers State. A Final year project Submitted to [CCE] KENPOLY Bori.

Karaca, A., Cetin, S. C., Turgay, O. C., & Kizilkaya, R. (2010). Effects of heavy metals on soil enzyme activities. In I. Sherameti & A. Varma (Eds.), *Soil heavy metals* (pp. 237-262). Springer. https://doi.org/10.1007/978-3-642-02436-8_11.

Cortez, L. A. S., & Ching, J. A. (2014). Heavy metal concentration of dumpsite soil and accumulation in Zea mays (corn) growing in a closed dumpsite in Manila, Philippines. International Journal of Environmental Science and Development, 5(1), 77-80. https://doi.org/10.18178/ijesd.2014.5.1.454

Luisa L, M and Miguel P.M (2006). Effect of excess copper on tomato plants. Growth parameters, enzyme activities chlorophyll, and mineral content. Journal of plant Nutrition https: //doi.org/10/.1080/0191160600972845.

Manahan, S. E. (2001). Fundamentals of environmental chemistry (2nd ed.). CRC Press. https://doi.org/10.1201/9781420056716

Oyedele, D. J., Gasu, M. B., & Awotoye, O. O. (2008). Changes in soil properties and plant uptake of heavy metals on selected municipal solid waste dump sites in Ile-Ife, Nigeria. African Journal of Environmental Science and Technology, 3(5), 107-115. http://www.academicjournals.org/AJEST

Shokr, M. S., El Baroudy, A. A., Fullen, M. A., El-beshbeshy, T. R., Ramadan, A. R., Abd El Halim, A. M., Guerra, A. J. T., & Jorge, M. C. O. (2016). Spatial distribution of heavy metals in the middle nile delta of Egypt. International Soil and Water Conservation Research, 4, 293-303. https://doi.org/10.1016/J.ISWCR.2016.10.003

Sprynskyy, M., Kosobucki, P., Kowalkowski, T., & Buszewski, B. (2007). Influence of clinoptilolite rock on chemical speciation of selected heavy metals in sewage sludge. Journal of Hazardous Materials, 149(2), 310-316. https://doi.org/10.1016/j.jhazmat.2007.04.001

Yao, H., Xu, J., & Huang, C. (2003). Substrate utilization pattern, biomass and activity of microbial communities in a sequence of heavy metal-polluted paddy soils. Geoderma, 115(1-2), 139-148. https://doi.org/10.1016/S0016-7061(03)00083-1