The effect of refining process on the oxidative stability, thermal stability, and fatty acid composition of shea olein

Abstract

This study aimed to examine how the refining process conditions influence the oxidative and thermal stability and fatty acid composition of shea olein. A chemical refining method was applied to shea olein, which included the removal of mucilaginous compounds, free fatty acids, and metals, thereby enhancing its acceptability in the food industry. The refining losses due to degumming, neutralization, and bleaching were 8.80%, 6.56%, and 7.56%, respectively. A 47% and 87.6% reduction in PV and CD, respectively, was observed, while ρ-AV and TBARS increased by a factor of 2 compared to the starting material. Thermal stability decreased in the order: NSO~BSO>DSOB>CSB. Refining improved the oleic acid content of shea olein by 10.48% and decreased arachidic acid content by 13.37%, respectively. These findings revealed that refining shea olein chemically provided a laudable benefits of stability enhancenent , but with drawbacks of  secondary products accumulation and increased trans-fatty acid. Hence, a need to optimize a chemical refining process for shea olein to improve  its  stability and utilization in the food industry.

Keywords: Shea olein, refining, oxidative stability, thermal stability, fatty acid composition

1.Introduction

Shea butter, derived from the nuts of the shea tree (Vitellaria paradoxa), is a versatile fat widely used in the food, cosmetic, and pharmaceutical industries due to its unique physicochemical properties (Ky-Dembele et al., 2021). Native to sub-Saharan Africa, the shea tree produces nuts that are processed to extract butter, which consists primarily of triglycerides rich in stearic and oleic acids, along with a high unsaponifiable fraction including tocopherols, phytosterols, and phenolic compounds (Abdel-Razek et al., 2023). These components contribute to its emollient, antioxidant, and anti-inflammatory attributes, making it a valuable ingredient in products ranging from bakery/chocolate formulations to skincare creams. According to the Mordor Intelligence report, the global shea butter market is projected to reach USD 3.75 billion by 2030, driven by increasing demand for natural and sustainable ingredients in the beauty and food sectors, resulting in significant growth (Mordor Intelligence, 2025) . However, if the food sector will benefit immensely from this growth, attention must be paid to the potential of the shea olein fraction of shea butter, which is composed of above 50% oleic acid in its fatty acid profile, in addition to the current benefits obtained from shea stearin, which is used as a cocoa butter substitute in the food confectionery industry (Korede et al., 2024; Abdel-Razek et al, 2023; Hang et al., 2017).

Shea olein, a liquid fraction of shea butter obtained after the complete removal of shea stearin, a hard fraction, is an emerging food oil widely used in the cosmetic industry but ignored in the food industry due to its instability and unpleasant sensory notes (Korede et al., 2024). Shea olein is a golden-yellow liquid characterized by its unique fatty acid composition and bioactive compounds, which are similar to those found in olive oil (Korede et al., 2024; Zhang et al., 2017). It contains the natural antioxidant alpha tocopherol, one of the advantages of oils considered for cooking. Studies have found that the alpha tocopherol content of shea olein is about 3 times higher than that of refined palm olein and that its fatty acids are highly resistant to oxidation (Korede et al., 2024; Abdel-razek et al., 2023; Zhang et al., 2017). Although various vegetable oils such as flaxseed oil, sunflower oil, canola oil, and soybean oil have been previously refined and the effects of refining steps on different oxidative indices measured (Jing et al., 2025; Gharby, 2022; Nagy et al., 2024). There is a paucity of findings on refining shea olein and the possible outcomes for its stability during the process. If shea olein is considered an emerging food oil with great potential, there is a need to assess its stability during the refining process. This will elucidate its stability potential and identify areas of necessary intervention during the process.

Refining is the process of removing unwanted and detrimental components to prepare healthy and acceptable food products (Squeo et al., 2022). Various methods have been established for refining vegetable oil: chemical and physical methods. The choice of refining method is highly dependent on the quality and composition of the crude oil (free fatty acids), economic considerations (yield and energy consumption), and the desired final product quality (tocopherol retention and flavor) (Gharby, 2022; Wen et al., 2023). However, studies have shown that refining generally improves safety and removes harmful substances (oxidation products), enhances sensory qualities, increases stability and shelf life, and preserves essential fatty acids, among other benefits (Zufarov et al., 2024; Mavlanov et al., 2024; Oey et al., 2022). The process also contributes to the loss of beneficial minor components such as tocopherol, phytosterol, polyphenols, and carotenoids. It could also result in potential formation of undesirable compounds, reduced nutritional profile and environmental drawbacks (Gharby, 2022; Oey et al., 2022).

It is therefore important, to weigh the pros and cons of refining shea olein, if it will become an added benefit to the food industry. Hence, this work aims to measure the effect of refining on the stability and fatty acid composition of shea olein

2. Materials

Shea kernels utilized to produce shea butter were procured at Kishi market of Irepodun local government area of Oyo state.  Chemicals and reagents used for this study were of analytical grade

3. Methods

Refining of shea olein

Crude shea butter (CSB) was prepared according to the method described by Korede et al. (2024). The refining process was conducted according to the method described by Tavakoli et al. (2022). To about 1 kg of melted CSB at 65 ℃, 4 mL (0.4% w/v shea olein) of 85% ortho phosphoric acid was added, and the mixture was homogenized with a high-speed hand mixer (BOSCH, Ergo Mixx, Slovenia) for two minutes. The mixture was separated using a cold centrifuge (5430R, Eppendorf, Germany) at 4000 revolutions per minute (rpm) for 30 min at 20 ℃ to remove gums and other phospholipid materials. After degumming, CSB was fractionated into degummed shea olein and degummed shea stearin according to Hwang et al. (2021). A 500 mL sample of degummed shea olein at 50 ℃ was further mixed with 2.5 mL of 0.8N NaOH for five minutes. The emulsion was transferred into a glass beaker and melted in a water bath (WBE601, LAB CON, Maraisburg) at 90 ℃ for 10 minutes. The soapstock was separated from the oil using an Eppendorf cold centrifuge (5430R, Eppendorf, Germany) at 4000 rpm for 30 min at 20 ℃. The neutralized oil was washed with 1% sodium chloride at a volume 3% larger than the free fatty acid content of shea olein. This was repeated to ensure effective removal of soapstock by centrifugation. The resulting shea olein was placed in an oven at 60 ℃ to reduce its moisture content. Further refining was carried out by bleaching shea olein at 85 ℃ under vacuum with the aid of bentonite for 30 min. A 2% bentonite was added to shea olein to ensure complete removal of color bodies. The resulting mixture was centrifuged at 4000 rpm for 20 minutes to separate refined bleached shea olein from the bleaching earth. The refined bleached shea olein and other fractions were further kept at -20 ℃ for further application and analysis.

Determination of peroxide value

A 5 g sample of oil was weighed into a conical flask. The content of the flask was dissolved in a 30 mL mixture of acetic acid and chloroform (2:1), and 0.5 mL of a saturated potassium iodide solution was added. The mixture was then shaken and allowed to stand for 1 min. After which, 30 mL of distilled water and 0.5 mL starch indicator were added, and the content was titrated against 0.01 N sodium thiosulphate. The blank test was conducted (AOAC, 2012). Peroxide value was calculated as ml equivalent of active oxygen /kg of sample as indicated in Eqn. 3.

Peroxide value =       				                          1

S = Titre value of sample – Titre value of blank, N = Molarity of the thiosulphate

Determination of ρ-anisidine value

The p-anisidine value (AV) was determined according to the AOCS Official Method Cd 18-90, with modification using a UV-visible microplate reader (PowerWave HT microplate reader, BioTek) at a wavelength of 350 nm. The 0.25% p-anisidine reagent was prepared every working day. 0.25 grams of p-anisidine was dissolved in 100 mL of 100 % acetic acid, and the absorbance was measured to ensure a value below Abs = 0.2. To analyze the samples, 0.1 g (100 mg) of oil was weighed directly in test tubes and dissolved in 5 mL of iso-octane. 2.5mL of sample was transferred to a cuvette, and the absorbance was measured at 350 nm against pure iso-octane as a blank. Then, 0.5 mL of p-anisidine reagent was added, and the cuvette was shaken by hand. The cuvette was kept in the dark for 10 min before the second absorbance measurement was made. The p-AV is given by the formula:

ρ-AV=D x [1.2 x AS2-AB2-(AS1-AB1)/m]                                      2

Where, D is the volume of iso-octane used to dissolve oil sample (1 mL), AS1= Absorbance of the oil solution before reaction with the p-anisidine reagent, AS2 = Absorbance of the oil solution after reaction with the p-anisidine reagent, AB1= Absorbance of iso-octane before reaction with the p-anisidine reagent, AS2 = Absorbance of iso-octane after reaction with the p-anisidine reagent, and m = mass of the sample (20 mg).

Determination of conjugated dienes

Conjugated dienes were determined by the Kūka et al. (2018) method. 100 mg of each sample was weighed into a test tube, and 5 mL of iso-octane was added to dissolve it. About 300 µL of each sample was transferred into a 96-well microplate. The absorbance reading of each sample solution was taken at 234 nm using a microplate reader (PowerWave HT microplate reader, BioTek). Absorbance was taken against 1% methyl stearate in iso-octane as a blank.

CD=A/εl								          3

Where

CD is the concentration of conjugated dienes in the sample

A is the sample absorbance in iso-octane, which varies across the samples

l is the path length, which equals 1 cm

ε is the molar absorptivity of 1% solution, which is 25250 M-1 ·cm -1 – molar absorptivity of linoleic acid hydroperoxide

The conjugated diene value (CD value) was calculated as follows:

CD value=[CD]×5×104m                                                                                      4

where

5 x104 – a factor that encompasses the volume of iso-octane (50 mL) used to dissolve the oil sample for the determination of CD concentration in µmol, m – sample weight

Determination of thiobarbituric acid reactive substance (TBARS)

TBARS was measured with a little modification in the method described by Zeb and Ullah (2016). Briefly, 50 mg of the oil sample was homogenized in 1 mL of 50% glacial acetic acid solution at 50 °C for 2 min. The mixture was allowed to cool to room temperature. A 57.66 mg thiobarbituric acid (TBA) in 100 mL glacial acetic acid (4.0 mM) was prepared, and exactly 150 µL each of the sample solution and TBA were placed in microplate wells and heated in an oven (60 °C) for 60 min to develop a pink color. The blank sample was prepared by adding 150 µL 50% acetic acid + 150 µL TBA in a microplate well. The calibration standard (1mM) was prepared by dissolving 31.35 mg of malondialdehyde tetrabutylammonium salt (MDA) in 100 mL of glacial acetic acid. This was diluted to give 0.2, 0.4, 0.6, and 0.8 mM. 150 µL of TBA was added to the same volume of MDA in the microplate well. The samples were measured at 532 nm in a microplate reader (PowerWave HT microplate reader, BioTek). A calibration curve of absorbance was plotted against the concentration, and the concentration of the samples was determined and expressed as mmol MDA equivalents/g oil.

Determination of thermal stability

The thermal stability of the shea fractions was assessed using the method described by Islam et al. (2023). An empty sealed pan was used as a reference, and lipid samples (6–8 mg) were firmly sealed in a 100 µL aluminium pan. At a rate of 20 °C per min, these samples were heated isothermally from 25 °C to 140 °C, and they were kept in the Differential Scanning Calorimeter (DSC) furnace for 90 min. The profiles were analyzed using the DSC system software (estar, Mettler Tonado, Shelton, CT, USA). The temperature for the isothermal program was set at 140 °C.

Determination of fatty acid composition

The following two procedures were used to prepare the fatty acid methyl esters (FAMEs): 10 mL screw-capped tubes containing 40 μL of treated shea olein were filled with 0.7 mL of potassium hydroxide (10 M) solution and 5.3 mL of methanol. For 1.5 hours, the reaction was carried out at 55 °C, with 5 seconds of mixing every 20 min. 0.58 mL of sulfuric acid (10 M) solution was added after the mixture had cooled to room temperature. The reaction was then maintained at 55 °C for 1.5 hours, with 5 s of mixing every 20 min. A 3 mL of n-hexane was added and stirred for five minutes after the mixture had cooled to room temperature. The tubes were then centrifuged for five minutes, and the extracts were placed in the vial for analysis (Wang et al., 2015).

A gas chromatography flame ionization detector (GC/FID) analysis using a Shimadzu instrument was conducted. The separation was performed on an RTX-5 capillary column, measuring 30 × 0.25 mm in diameter with a film thickness of 0.25 μm. Here are the operating conditions: the carrier gas pressure was set at 100 kPa, and we used nitrogen with a split ratio of 1:100. The injection temperature was 250 °C, and the oven temperature was programmed to start at 50 °C for 2 minutes, then increase to 174 °C at a rate of 50 °C/min, and finally rise to 215 °C at 2 °C/min until the end of the analysis. We identified and quantified the individual fatty acids by comparing their retention times with the external FAME 37 standards (Korede et al., 2024).

3.1 Statistical Analysis

Samples were prepared in two duplicates, and analyses were carried out twice. All of the experimental results, which were presented as means and standard deviation, were assessed using one-way analysis of variance (ANOVA). Using IBM SPSS Statistics 27 software (SAS Institute, Inc., Cary, NC, USA), the Tukey HSD test found significant differences (p<0.05) across the examined samples.

4. Result and discussion

Refining loss

The refining steps (degumming, neutralization, bleaching) resulted in a measurable oil loss as shown in Figure 1: degumming (8.8%), neutralization (6.56%), and bleaching (7.56%). These losses are typical and arose from the removal of gums, free fatty acids soaps, and adsorbates, respectively. According to Hwang et al. (2021), degumming losses could arise from the removal of gummy materials in crude shea butter. Degumming removes phospholipids (gums), metal-bound complexes, and other hydratable impurities. When gums are hydrated and separated, some oil is entrained in the gum phase, producing about 8–9% loss as reported by Abdel-Razek et al. (2023). Our result aligns with that of Méndez et al. (1991), who reported a degumming loss from shea butter refining of between 5.6% and 7.6%. The observed degumming loss correlates with the amount of unsaponifiable fraction of shea butter as reported by Hwang et al. (2021).

The neutralization loss observed during refining could be a result of soapstock formation and oil entrainment. In this step, free fatty acids (FFA) are converted into soap, which is separated as a soapstock. This step inherently removes the FFA plus entrained neutral oil, which explains the 6.5% loss observed in Figure 1. Gharby et al. (2022) identified possible causes of increased loss of oil to include long-mix neutralization and inadequate phase separation. These losses could be minimized with modern refiners who utilize optimized alkali dosing, centrifugation, and counter-current washing. The observed result aligns with Kumar and Krishna (2015), who reported a 30% loss of neutral oil during alkaline neutralization of palm oil, but a lesser loss (12%) was observed when ethanol was used for neutralization.

Bleaching removes pigments and polar minor components by adsorption (and with them some oil). Adsorbent earth removes coloured compounds, trace metals, and oxidation precursors, but they also physically retain small amounts of oil on the clay surface. Adsorption or entrainment of oil explains the 7–8% loss observed during bleaching. The extent of loss depends on bleaching earth type, dosage, contact time, and decolourization temperature.

Peroxide value

Table 1 presents the oxidative indices of super shea olein during refining. The peroxide values were significantly different (p˂0.05) during refining, ranging from 0.24 meq O2/kg to 1.59 meq O2/kg. PV decreased from 1.59 meq O2/kg in crude shea butter (CSB) to 0.32 meq O2/kg in degummed shea olein (DSOB) and further to 0.26 meq O2/kg in neutralized shea olein (NSO), indicating that the degumming and neutralization processes were effective in reducing primary oxidation products. The observation indicates the removal of phosphatides, pro-oxidant metals, and hydroperoxides during the processes (Karim et al., 2022; Wen et al., 2023). However, the notable observation in this case was that bleaching (BSO) reduced PV even further to 0.24 meq O2/kg, suggesting the adsorptive character of bleaching earth and activated clays for the removal of hydroperoxides, pigments, and trace metals (Almoselhy, 2021). This observation agrees with the findings of   Icyer and Durak (2018) when assessing ultrasound-assisted bleaching of canola oil. Thus, bleaching provided the best improvement in PV stability.

ρ-Anisidine value

The anisidine value, which measures secondary oxidation products (aldehydes, ketones), decreased significantly from 11.51 in CSB to 7.49 in DSOB and 7.28 in NSO. This revealed that early refining steps helped decrease volatile aldehydes by removing phospholipids and free fatty acids, which catalyze their formation. Bleaching, nevertheless, caused AV to rise to 9.30. This could probably be because bleaching conditions promoted some breakdown of hydroperoxides to aldehydes on heating and contact with adsorbents. This is in agreement with the report of Shahidi and Zhong (2020), that bleaching, while effective in reducing pigments and peroxides, may catalyze the breakdown of hydroperoxides to secondary products and enhance ρ-AV. Hence, the need for optimization of bleaching process.

Conjugated dienes

CD decreased stepwise during refining from 5.00 mmol/g (CSB) to 0.59 mmol/g (DSOB) to 0.47 mmol/g (NSO). This downward trend reflects that neutralization is highly

effective for the removal of dienic hydroperoxides. Bleaching, however, brought about a slight increase in CD to 0.62 mmol/g. the slight increase could be attributed to the partial isomerization or oxidation of residual unsaturated lipids during bleaching. Increases in CD after bleaching have been observed for palm oil and shea oil and are attributable to light oxidation catalyzed by bleaching earth (Abdel-Razek et al., 2023).

Thiobarbituric acid reactive substance (TBARS)

TBARS, which measure aldehydic breakdown products (mainly malondialdehyde), ranged between 0.10 mmol MDA/g to 0.26 mmol MDA/g. TBARS were relatively low in CSB (0.14 mmol MDA/g), and were not affected by degumming (0.10 mmol MDA/g) and neutralization (0.16 mmol MDA/g), but were elevated by bleaching (0.26 mmol MDA/g). This showed that while early refining steps did not worsen aldehydic breakdown, bleaching promoted thermal decomposition of polyunsaturated lipids and hydroperoxides to TBARS-active species. This phenomenon was observed in food oils, where bleaching could encourage the formation of secondary oxidation products if not properly controlled (Rohm et al., 2020; Durand et al., 2024).

Refining significantly improved the oxidative profile of shea olein by reducing hydroperoxides (PV) and conjugated dienes (CD), especially through degumming and neutralization.  However, the drawbacks of the refining process resulted in increased p-AV and TBARS during bleaching process. This indicates that bleaching conditions (temperature, time, and adsorbent dosage) must be well optimized to improve oxidative stability without inducing hydroperoxide decomposition to aldehydes.

Effects of refining on thermal stability

The thermal stability of shea olein during refining DSC curve, is shown in Figure 2. The thermogram illustrated the thermal properties (Onset temperature, Peak temperature and Enthalpy) of shea olein as presented in Table 2 during refining.

The onset temperature, a sensitive indicator of thermal stability representing the initial melting point, ranged from 130.14 °C to 138.18 °C across the samples. Crude shea butter (CSB) exhibited the lowest onset temperature (130.135 °C), reflecting the presence of pro-oxidant impurities such as free fatty acids, phospholipids, peroxides, and unsaponifiable matter that facilitate premature melting and reduce overall thermal stability. Degumming markedly enhanced thermal stability, as evidenced by a substantially reduced exothermic drop at the beginning of heating and a comparatively flat baseline near 0 mW as shown in Figure 2. This improvement is attributed to the removal of phospholipids, free fatty acids, and initial oxidation products, which exacerbate thermal decomposition in crude shea butter. The degumming step achieved reductions of 79.87%–88.20% in primary oxidation products (peroxide value, PV) and 28.50%–31.36% in secondary oxidation products (p-anisidine value, p-AV), consistent with Abdel-Razek et al. (2023), who reported a 89.3% reduction in PV (from 5.91 to 0.63 meq O₂/kg) during degumming of shea olein.

DSOB, NSO, and BSO demonstrated higher onset temperatures (135.56 °C–138.18 °C), although the differences were not statistically significant. The highest onset was observed in neutralized shea olein (NSO) at 138.18 °C, while bleached shea olein (BSO) showed a slight decrease to 137.92 °C, possibly due to partial loss of natural antioxidants such as tocopherols during bleaching, which may limit further stability gains despite the removal of oxidation catalysts. This is consistent with the finding of Abedi et al. (2025), who reported a tocopherol reduction of 16.8 to 37.8% in sunflower oil with corresponding decreases in oxidative resistance due to diminished radical scavenging. Neutralization, involving alkali treatment to remove free fatty acids (FFAs), further enhanced oxidative stability, as indicated by a small exothermic transition (−1.41 mW) followed by a stable plateau at baseline. This pattern reflects minimized hydroperoxide decomposition and reduced propagation of secondary oxidation products, consistent with the removal of FFAs that catalyze thermal instability (Islam et al., 2023).

Peak temperatures remained highly consistent across all samples, ranging between 139.35 °C and 140.20 °C (a narrow range of ~0.85 °C), with statistically significant differences (p < 0.05) observed during refining. This range signaled stability and suggests that the dominant triglyceride species responsible for the main melting event, which are primarily Arachidic -Oleic-Arachidic (A-O-A) and related high melting triacylglycerides are largely preserved throughout  refining process. This is consistent with our finding in Table 3, where arachidic acid is maitained throughout refining process. The slight upward shift in temperature form CSB (139.35 ), DSOB (140.02 ), NSO (140.06 ) and BSO (140.20 ) during refining indicates a modest sharpening of the melting profile, likely due to selective removal of high-melting components and enrichment in more uniform crystalline forms. This is consistent with our findings on fatty acid composition in Table 3, the lower the content of arachidic acid in the refined shea olein , the higher the peak temperature, the better the thermal stability. Zhang et al. (2017) and Gao et al. (2022) related improved stability and sharper DSC melting transition to reduction in in long-chain saturated fatty acids (e.g., arachidic, behenic). this is because a more homogeneous crystal networks is formed with unsaturated triglycerides. In another study on shea butter thermal behaviuor by Goumbri et al. (2024), he reported that refining led to a shift  toward higher and sharper peak temperatures in olein fractions, attributed to the preferential removal of high-melting saturated species and retention of a more uniform oleic-rich profile

Enthalpy values, representing the heat absorbed during melting, decreased from CSB (1647.11 J/g) to DSOB (1342.705 J/g), indicating a reduction in the total energy required for melting after degumming. NSO exhibited a high enthalpy (1700.125 J/g), comparable to CSB, possibly due to neutralization preserving more crystalline structure. Bleached shea olein (BSO) showed an intermediate value (1507.59 J/g), suggesting partial loss of crystalline material during the removal of pigments, peroxides, and natural antioxidants.

In a nutshell, neutralization shea olein is considered most stable among the sample as a result of high onset temperature and enthalpy values. Bleached shea olein is moderately stable due to its high peak temperature but could not match-up with NSO due to loss of natural antioxidants during process. Yet, the process enhanced thermal stability of shea olein as compared with crude shea butter. Hence, these improvements render refined shea olein more resistant to thermal degradation and oxidative deterioration, making it highly suitable for food applications such as frying oils, specialty fats, and margarine.

Effects of refining on fatty acid composition

Fatty acid composition during refining is presented in Table 3. The composition of fatty acids varied significantly (p˂0.05) in all samples. The refining sequence (CSB → DSOB → NSO → BSO) produced clear, systematic shifts in individual fatty acids: oleic acid rose markedly (47.65 → 58.13%), arachidic (C20:0) and other very-long-chain saturated fatty acids declined (C20:0: 39.05 → 26.42%; C22:0: 1.18 → 0.52%), palmitic (C16:0) was essentially stable (≈3.2–3.4%), elaidic (trans-C18:1) increased (6.87 → 9.52%), and minor PUFAs (linoleic, C18:2; DGLA, C20:3 n6) showed small rises in the final bleached oil.

The significant variations (p<0.05) in fatty acid composition across the refining sequence (CSB → DSOB → NSO → BSO) highlight the progressive purification and minor redistributions induced by each step, with oleic acid (C18:1) exhibiting a marked increase from 47.65% to 58.13%. This elevation in monounsaturated fatty acids (MUFAs) like oleic is typical in refined olein fractions, as refining processes such as degumming, neutralization, and bleaching selectively remove impurities while preserving or slightly enhancing the liquid, unsaturated components through incidental fractionation effects during heating and filtration (Azad et al., 2021; Tavakoli et al., 2022). This is consistent with the study conducted by Abdel-Razek et al. (2023) on crude shea butter and refined shea olein. He observed a similar trend in oleic content in refined olein to 56.04% from 45.77% in crude, attributed to the migration of oleic-rich triglycerides into the liquid phase, aligning closely with the observed trend in Table 2. Similarly, Jing et al. (2025) observed an increase in oleic acid of flaxseed oil during refining.

Conversely, the decline in very-long-chain saturated fatty acids, such as arachidic (C20:0) from 39.05% to 26.42% and behenic (C22:0) from 1.18% to 0.52%, reflects the preferential removal of saturated chains during refining steps, particularly neutralization and bleaching, which can induce partial crystallization and separation of high-melting-point saturates. This pattern underscores saturation reduction in refined shea olein for improved fluidity (Khan et al., 2024). This result aligns with the observation of Abdel-Razek et al. (2023), who obtained a TSFA of 35.40% on refined shea olein while investigating the fatty acid component of shea butter during refining. With reduced TSFA, olein becomes more liquid and transparent at room temperature, with improved cold stability (Tavakoli et al., 2022; Khan et al., 2024). High arachidic acid value in this composition is worthy of note, as it represents a switch in the composition of stearic acid common to shea butter.  This could result from different varieties of shea trees in the south-western part of Nigeria.

The increase in elaidic acid (trans-C18:1) from 6.87% to 9.52% suggests potential isomerization during thermal steps, such as bleaching, where elevated temperatures (typically 90–110°C) can promote cis-to-trans conversions in unsaturated chains. However, this is less common in mild refining and warrants control to minimize trans fats. Minor polyunsaturated fatty acids (PUFAs) like linoleic (C18:2) (0.49% to 0.73%), and dihomo-gamma-linolenic acid (DGLA, C20:3 n-6) (0.34% to 0.52%), showing small rises in bleached oil align with enrichment in the liquid fraction, as PUFAs partition into olein enhancing nutritional value but requiring antioxidants to counter oxidation susceptibility (Azad et al., 2021; Fadda et al., 2022)

Conclusion

This study demonstrated that refining improved oxidative indices, thermal stability, and nutritional profile of shea olein. Among refined samples, NSO demonstrated increased thermal stability over BSO, which resulted from the elimination of pigments, trace metals,, peroxides and natural antioxidant through bleaching.  BSO also revealed fatty acid composition with the highest oleic acid and monounsaturated fatty acid  with highest trans-fatty acid content as compared to other refined samples. Oxidative indices revealed NSO as most stable against the formation of primary and secondary oxidation products, while the formation of secondary oxidation products (ρ-AV and TBARs) are enhanced during bleaching process. Future research should focus on optimizing the refining process to improve its stability against the formation of secondary oxidation products and trans fatty acids, thereby providing another healthy vegetable oil source for food formulations.

Competing Interests

Authours have declared that no competing interest exist.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Abdel-Razek, A. G., Abo-Elwafa, G. A., Al-Amrousi, E. F., Badr, A. N., Hassanein, M. M. M., Qian, Y., Siger, A., Grygier, A., Radziejewska-Kubzdela, E.,and Rudzińska, M. (2023). Effect of refining and fractionation processes on minor components, fatty acids, antioxidant and antimicrobial activities of shea butter. Foods, 12(8), 1626.

- Almoselhy, R. I. (2022). Application of nanotechnology for optimization of bleaching efficiency of edible oils. Open Access J Sci, 5(1), 29-30.

- AOAC. (2012). Peroxide Value of Oils and Fats, in Official Methods of Analysis of AOAC International, 19th Ed., AOAC International, Gaithersburg, MD.

- AOCS. (1995). Official methods and recommended practices of the American Oil Chemists' Society (Method Cd 18-90). AOCS Press.

- Azad, M., et al. (2021). "Impact of fractionation on fatty acids composition, phenolic compounds, antioxidant characteristics of olein and super olein fractions of flaxseed oil." Journal of Food Processing and Preservation 45(4): e15369.

- Fadda, A., Sanna, D., Sakar, E. H., Gharby, S., Mulas, M., Medda, S., Yesilcubuk, N.S., Karaca, A.C., Gozukirmizi, C.K., Lucarini, M.and Durazzo, A. (2022). Innovative and sustainable technologies to enhance the oxidative stability of vegetable oils. Sustainability, 14(2), 849

- Gharby, S. (2022). Refining vegetable oils: Chemical and physical refining. The Scientific World Journal, 2022(1), 6627013.

- Goumbri, B. W., Kouassi, A. K., Djang’eing’a, R. M., Semde, R., Mouithys-Mickalad, A., Sakira, A. K., ... & Danthine, S. (2024). Quality characteristics and thermal behavior diversity of traditional crude shea (Vitellaria paradoxa Gaertn) butter from Burkina Faso. Food Biophysics, 19(3), 609-626.

- Guo, T., Wan, C., Huang, F., Wei, C., & Xiang, X. (2021). Process optimization and characterization of arachidonic acid oil degumming using ultrasound-assisted enzymatic method. Ultrasonics Sonochemistry, 78, 105720.

- Hwang, J., Jun, H., Roh, S., Lee, S. J., Mun, J. M., Kim, S. W., Chung, M.-Y., Kim, I.-H., and Kim, B. H. (2021). Preparation of low-diacylglycerol cocoa butter equivalents by hexane fractionation of palm stearin and shea butter. Molecules, 26(11), 3231.

- Icyer, N. C. and M. Z. Durak (2018). Ultrasound-assisted bleaching of canola oil: Improve the bleaching process by central composite design. Lwt 97, 640-647.

- Islam, M., Kaczmarek, A., & Tomaszewska-Gras, J. (2023). Differential scanning calorimetry as a tool to assess the oxidation state of cold-pressed oils during shelf-life. Journal of Food Measurement and Characterization, 17(6), 6639-6651.

- Jing, Y., Song, Z., Li, H., Wei, X., Xie, J., & Shen, M. (2025). The quality and oxidative stability of flaxseed oils of varying degrees of refinement. Grain & Oil Science and Technology.

- Karim, N. A. A., Othman, N. H., & Yusoff, M. M. (2022). Vegetable oil refining. In Recent advances in edible fats and oils technology: processing, health implications, economic and environmental impact (pp. 101-140). Singapore: Springer Singapore.

- Khan A, Nadeem M., Ullah R., Gulzar N., Al-Asmari F., Imran M., Rahim M.A., Zongo E., Hussain I., Tayyab M., and Almalki R.S. (2024). Fatty Acid Composition, Phenolic Compounds, Phytosterols, and Lipid Oxidation of Single- and Double-Fractionated Olein of Safflower Oil Produced by Low-Temperature Crystallization. ACS Omega, 9(6), 6787-6796.

- Korede, A., Adeboyejo, F., Fapojuwo, O., and Ogunwole, O. (2024). Quality variation in shea butter and olein from Irepodun Local Government Area, Oyo State, Nigeria. African Journal of Food Science, 18(2), 28-36.

- Kūka, M., Čakste, I., & Kūka, P. (2018). Inhibition of formation of conjugated dienes in linseed oil. Proceedings of the Latvian Academy of Sciences (Vol. 72, p. 80): De Gruyter Poland.

- Kumar, P. P. and A. G. Krishna (2015). "Effect of different deacidification methods on phytonutrients retention in deacidified fractionated palm oil." Journal of the American Oil Chemists' Society, 92, 645-658.

- Ky-Dembele, C., Bayala, J., Boffa, J. M., Kalinganire, A., & Minang, P. A. (2021). Shea tree crop management in West Africa. Tree Commodities and Resilient Green Economies in Africa. Nairobi, Kenya: World Agroforestry (ICRAF).

- Mavlanov, U., Czaja, T. P., Nuriddinov, S., Dalimova, D., Dragsted, L. O., Engelsen, S. B., & Khakimov, B. (2024). The effects of industrial processing and home cooking practices on trans-fatty acid profiles of vegetable oils. Food Chemistry, 142571.

- Méndez, M. R., Lope, J.H., Pereda, J., & Dobaganes M.C. (1991). "Assays for physical refining of shea butter." Grasas y Aceites 42(2), 121-126.

- Mordor Intelligence (2025). Shea Butter Market Size, Share & Trends Analysis Report. 		Https://www.mordorintelligence.com/industry-reports/shea-butter-market.

- Nagy, K., Iacob, B.C., Bodoki, E., and Oprean, R. (2024). Investigating the thermal stability of omega fatty acid-enriched vegetable oils. Foods, 13(18), 2961

- Oey, S. B., van der Fels-Klerx, H. J., Fogliano, V., & van Leeuwen, S. P. (2022). Chemical refining methods effectively mitigate 2-MCPD esters, 3-MCPD esters, and glycidyl esters formation in refined vegetable oils. Food Research International, 156, 111137.

- Squeo, G., Silletti, R., Napoletano, G., Greco Miani, M., Difonzo, G., Pasqualone, A., & Caponio, F. (2022). Characterization and effect of refining on the oil extracted from durum wheat by-products. Foods, 11(5), 683.

- Tavakoli, A., Sahari, M. A., Barzegar, M., Ahmadi Gavlighi, H., Marzocchi, S., Marziali, S., and Caboni, M. F. (2022). The effect of refining process on the volatile compounds, oxidation stability and fatty acids profile of soybean oil using an electrostatic field. Journal of Food Processing and Preservation, 46(1), e16160.

- Tavakoli, A., Sahari, M. A., Barzegar, M., Ahmadi Gavlighi, H., Marzocchi, S., Marziali, S., and Caboni, M. F. (2022). The effect of refining process on the volatile compounds, oxidation stability and fatty acids profile of soybean oil using an electrostatic field. Journal of Food Processing and Preservation, 46(1), e16160.

- Wang, J., Wu, W., Wang, X., Wang, M., and Wu, F. (2015). An effective GC method for the determination of the fatty acid composition in silkworm pupae oil using a two-step methylation process. Journal of the Serbian Chemical Society, 80(1), 9-20

- Wen, C., Shen, M., Liu, G., Liu, X., Liang, L., Li, Y., Zhang, J. & Xu, X. (2023). Edible vegetable oils from oil crops: Preparation, refining, authenticity identification and application. Process Biochemistry, 124, 168-179.

- Zeb, A., & Ullah, F. (2016). A simple spectrophotometric method for the determination of thiobarbituric acid reactive substances in fried fast foods. Journal of analytical methods in chemistry, 2016(1), 9412767.

- Zhang, Z., Ma, X., Huang, H., & Wang, Y. (2017). Shea olein based specialty fats: Preparation, characterization and potential application. LWT, 86, 492-500.

- Zufarov, O., Serkayev, K., & Isroilova, S. (2024). Influence of Chemical and physical refining processes on phospholipid and total Tocopherol content in sunflower oil. Am. J. Polym. Sci, 13, 1-5.

Figure 1: % loss exhibited during refining of shea olein

| Table 1: Oxidative stability of shea olein during refining |  |  |  |  |
| --- | --- | --- | --- | --- |
| SAMPLES | PV (meq/kg) | p-AV | CD (mmol/g) | TBARS (mmol/g) |
| CSB | 1.59 ± 0.00a | 11.51±0.02a | 5.00 ± 0.25a | 0.14 ± 0.22b |
| DSOB | 0.32 ± 0.01b | 7.49 ±0.85c | 0.59 ± 0.00b | 0.10±0.03b |
| NSO | 0.26 ± 0.1c | 7.28 ± 0.05c | 0.47 ±0.30c | 0.16±0.07b |
| BSO | 0.24 ± 0.03c | 9.30 ± 0.01b | 0.62 ± 0.07b | 0.26±0.05a |

CSB-crude shea butter; DSOB-degummed shea olein @ -5 ℃; NSO-neutralized shea olein; BSO bleached shea olein. Results are mean values of two determinations ± standard deviation.  Values in each column bearing the same superscripts are not significantly different (p < 0.05).

Figure 2: DSC isothermal measurement during refining of shea olein.

| Table 2: Thermal properties of shea olein during refining |  |  |  |
| --- | --- | --- | --- |
| Samples | Onset Temp (℃) | Peak Temp. (℃) | Enthalpy (J/g) |
| CSB | 130.135±10.35a | 139.345±0.16b | 1647.11±70.64a |
| DSOB | 135.555±5.98a | 140.02±0.01ab | 1342.71±24.33a |
| NSO | 138.175±2.34a | 140.06±0.32ab | 1700.13±7.16a |
| BSO | 137.915±2.77a | 140.20±0.18a | 1507.59±72.63a |

Values are reported as means ± SD of two replicate analyses (n = 2);

| Table 3: % Fatty acid composition during refining super shea olein |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CSB | DSOB | NSO | BSO |
| Palmitic acid (C16:0) | 3.23± 0.00a | 3.29 ± 0.00a | 3.25± 0.01a | 3.42± 0.01a |
| Arachidic acid (C20:0) | 39.05± 0.00a | 29.26± 0.01c | 30.26± 0.12b | 26.42± 0.01d |
| Behenic acid (C22:0) | 1.18 ± 0.02a | 1.02 ± 0.01b | 0.50 ± 0.04c | 0.52 ± 0.02c |
| Elaidic acid (C18:1 n9t) | 6.87 ± 0.07d | 9.02 ± 0.03b | 8.64 ± 0.03c | 9.52 ± 0.00a |
| Oleic acid (C18:1 n9c) | 47.65± 0.09d | 56.23± 0.05b | 55.27± 0.09c | 58.13± 0.00a |
| Linoleic acid (C18:2 n9c) | 0.49 ± 0.01c | 0.64 ± 0.00b | 0.66 ± 0.01b | 0.73 ± 0.00a |
| dihomo-gamma-linolenic acid (C20:3 n6) | 0.34 ± 0.00c | 0.41 ± 0.01b | 0.5 ± 0.04a | 0.52 ± 0.02a |
| Eicosanoic acid (C20:3 n3) | 1.19 ± 0.01a | ND | ND | ND |
| SFA | 43.45± 0.02a | 33.7 ± 0.01c | 34.2 ± 0.16b | 30.46± 0.03d |
| MUFA | 54.53± 0.02d | 65.25± 0.02b | 63.9 ± 0.12c | 67.66± 0.00a |
| PUFA | 2.02 ± 0.01a | 1.05 ± 0.01d | 1.16 ± 0.03c | 1.25 ± 0.02b |
| TUFA | 56.55± 0.02d | 66.3 ± 0.01b | 65.07 ± 0.1c | 68.9 ± 0.02a |

Values are reported as means ± SD of two replicate analyses (n = 2); CSB: crude shea butter; DSOB: Degummed shea olein; NSO: neutralized shea olein; BSO: Bleached shea olein TSFA: Total saturated fatty acids; TMUFA: total monounsaturated fatty acids; TPUFA: Total polyunsaturated fatty acids; TUFA: Total unsaturated fatty acids; ND: not detected