A Statistical analysis of organic carbon of olive mill waste water and olive mill solid waste and its effects on soil properties

Abstract- Olive oil production is the main activity of Mediterranean countries. Olive oil production is a new emerging sector in India. Olive oil refinery, Lunkaransar has been producing olive oil since 2015. After oil extraction, huge amount of waste is generated-Olive mill waste water and olive mill solid waste. These wastes are dumped in soil which affects soil environment. Olive oil waste main component is total organic carbon. In our research paper organic carbon of olive mill waste effects on soil properties are evaluated. Soil parameters such as pH, EC, Bulk Density, water holding capacity and TOC are analysed. The present study aimed to statistical Analysis of soil parameters showed that soil properties pH, bulk density was reduced and water holding capacity was increased. Olive mill waste improved soil quality but it also contaminated organic carbon in soil. Our research study provides an idea to recycle olive mill solid waste as an organic fertiliser or making compost to manage olive mill waste disposal.

Key words- Bulk density, Olive mill waste, Total organic carbon, Water holding capacity.

INTRODUCTION

Waste are the unwanted and undesirable products which are generated after main primary activity. Agriculture industrial waste is the left-over materials produced by manufacturing process. Many Agriculture processing industries like food processing, diaries industries, oil processing mills, sugar mills are generated waste in large scale. These crop residues are full of nutrients rich compounds, which can be recycles or reuse. Generally, these agro wastes are used as compost or animal feeding. In India, olive oil production mill is located at Lunkaransar, Bikaner, Rajasthan state. In refinery, Olive oil is produced by two phase centrifugation technique. In this process, olive fruits are washed, crushed, malaxation and centrifuged. Olive oil is separated and mixture of solid residue- olive mill solid waste and effluent water- olive mill waste water is left over. These waste products are dumped into open land area behind the refinery. The waste products are generated in large amount in a short period of time. The wastes are collected every year.

Olive mill waste water is an acidic dark brown liquid effluent with high electric conductance [1]. It is a polluting effluent because of consists of organic compounds, especially polyphenols. Earlier investigation showed that this waste water is rich in organic matter with high BOD and COD value [2]. Our previous research study investigated that Olive mill waste water contains high concentration of macronutrients potassium and notable level of nitrogen and micronutrients iron, copper, zinc [3].

Olive mill solid waste is a semi solid waste which called olive pomace or alperujo [4]. It consists olive pulp, olive skin and olive stones. According to earlier researcher, Olive mill solid waste has high toxic organic loads, high COD and BOD and low pH [5]. Previous studies shown that olive mill waste water is mainly constituent of organic carbon, macronutrients Nitrogen and potassium, micronutrients- iron, copper, zinc and manganese [6]. Our previous research study showed that olive mill solid waste is a nutrient rich compound- specially potassium, organic carbon and micronutrients [7].

Earlier research author reported in their studies that long term application of olive mill solid pomace in soil, improved soil chemical, physical and biological properties [8]. The work of previous researcher indicated that olive mill solid waste’s compost significantly increased availability of macronutrients and organic matter in soil [9]. The similar result was found in previous study that wet pomace of olive mill solid waste improved soil fertility [10]. Research author investigated that soil treated with olive mill waste water showed increase of availability of potassium, it enhanced soil fertility no negative effects were observed [11]. Unlike previous studies, some researchers reported that untreated olive mill waste water increased soil salinity and reduced plant growth, while treated olive mill waste water improved plant growth but soil pH was decreased [12].

In recent years, composting has become useful approach to recycle and manage the olive mill agricultural waste disposal [13]. Although olive mill waste reuse offers significant benefits, but no previous studies have investigated of olive mill waste utilization and management in India. No prior studies have been conducted on the waste disposal of olive mill refinery Lunkaransar. The main component of olive mill waste is total organic carbon [7]. For recycling and reuse this waste as organic fertilizer or compost, it is important to know the effect of organic carbon of olive mill solid waste on soil properties. Therefore, the aim of present study is to report and analyse olive mill waste water and olive mill solid waste organic carbon and its effects on soil properties.

Main objectives of our study are-

- Analysis of total organic carbon of olive mill solid waste.

- Analysis of total organic carbon of olive mill waste water.

- Analysis of soil parameters at two different sites–

Soil parameters- pH, EC, Bulk density, water holding capacity and Total organic carbon.

Two different sites of soil- soil at waste disposal site and soil at Nearby from disposal site.

- Statistical Analysis- Effect of total organic carbon of OMWW and OMSW on soil parameters.

MATERIAL AND METHODS

Sampling of Olive mill waste water- Olive mill waste water samples were collected from olive oil refinery, Lunkaransar. In refinery, waste products are dumped into open land area. Samples were collected from waste water disposal site in sterile and labelled plastic bottles. Samples were immediately transported to laboratory and stored in refrigerator at -40C. Total 12 samples were collected for analysis.

- Sampling of olive mill solid waste- Olive mill solid waste samples were collected from olive oil refinery, Lunkaransar. Samples were collected from disposal site at depth of 10-15 am using auger. The samples were carried out in sterile and labelled polythene bags. Samples were immediately transported to laboratory and stored in refrigerator at -40C. Total 12 samples were collected for analysis.

- Sampling of soil- Soil samples were collected from two different site- at waste disposal site and nearby from waste disposal site. All samples were collected at the depth of 10-15 cm using shovel. Samples were placed in sterile and labelled polythene bags and transported to laboratory. Total 24 soil samples were collected, 12 from waste disposal site and remaining 12 were taken from nearby disposal site.

- Experimental Methodology of Analysis of OMWW, OMSW and Soil –

- OMWW- Total organic carbon – TOC of olive mill waste water was determined by Dry Combustion Method (APHA, section 5310B).

- OMSW- Total Organic carbon- Olive mill solid waste total organic carbon was analyzed by Walkely Black Method. (IS 2720, part 22, 1972)

- Soil Analysis-

- pH- Soil pH was determined by potentiometric method. (IS 2720, part 26:1987)

- Electrical Conductance – Soil EC was determined by conductimetry method using EC meter. (IS 14767:2000)

- Bulk Density – Soil bulk density was measured by cylinder method. (IS:2720-28:1974)

- Water Holding Capacity- Soil WHC was analyzed by Gravimetric saturation drainage method. (IS 2720, part 21:1977)

- Total organic carbon- Soil TOC was determined by Walkley Black method. (IS 2720, part 22:1972)

RESULT and DISSCUSION

Table 1: Analysis of total organic carbon of olive mill waste water and olive mill solid waste

| Sample no. | TOC (OMWW) | TOC (OMSW) |
| --- | --- | --- |
|  | 21.20 | 57.80 |
|  | 21.40 | 59.20 |
|  | 21.00 | 54.23 |
|  | 20.80 | 55.75 |
|  | 19.20 | 57.20 |
|  | 20.40 | 57.59 |
|  | 18.50 | 50.77 |
|  | 20.10 | 52.55 |
|  | 20.10 | 49.49 |
|  | 18.40 | 48.70 |
|  | 18.30 | 51.84 |
|  | 19.50 | 50.84 |

Table 2: Physicochemical Analysis of Soil at disposal site

| Sample No. | pH | EC (mS/cm) | BD (gm/cc) | WHC (%) | TOC (%) |
| --- | --- | --- | --- | --- | --- |
|  | 8.01 | 0.2000 | 1.23 | 55 | 0.80 |
|  | 8.01 | 0.2100 | 1.21 | 54 | 0.89 |
|  | 8.01 | 0.1855 | 1.27 | 55 | 0.85 |
|  | 8.02 | 0.2700 | 1.28 | 55 | 0.80 |
|  | 8.01 | 0.2200 | 1.30 | 52 | 0.80 |
|  | 8.03 | 0.2500 | 1.30 | 55 | 0.88 |
|  | 8.04 | 0.2100 | 1.30 | 52 | 0.75 |
|  | 8.04 | 0.2100 | 1.34 | 51 | 0.70 |
|  | 8.10 | 0.1850 | 1.33 | 51 | 0.75 |
|  | 8.10 | 0.2500 | 1.34 | 50 | 0.70 |
|  | 8.07 | 0.2100 | 1.32 | 52 | 0.70 |
|  | 8.04 | 0.2150 | 1.30 | 50 | 0.70 |

Table 3: Physicochemical Analysis of Soil at Nearby from disposal site

| Sample No. | pH | EC (mS/cm) | BD (gm/cc) | WHC (%) | TOC (%) |
| --- | --- | --- | --- | --- | --- |
|  | 8.25 | 0.1304 | 1.53 | 47 | 0.27 |
|  | 8.20 | 0.1300 | 1.50 | 46 | 0.21 |
|  | 8.20 | 0.1280 | 1.51 | 45 | 0.30 |
|  | 8.20 | 0.1500 | 1.51 | 45 | 0.28 |
|  | 8.25 | 0.1300 | 1.50 | 44 | 0.28 |
|  | 8.25 | 0.1500 | 1.50 | 42 | 0.28 |
|  | 8.20 | 0.1380 | 1.50 | 40 | 0.29 |
|  | 8.20 | 0.1340 | 1.50 | 40 | 0.21 |
|  | 8.20 | 0.1200 | 1.52 | 35 | 0.29 |
|  | 8.20 | 0.1600 | 1.5 | 35 | 0.28 |
|  | 8.25 | 0.1400 | 1.51 | 38 | 0.28 |
|  | 8.25 | 0.1400 | 1.50 | 40 | 0.27 |

| Mean value TOC (OMWW) | Mean value TOC (OMSW) |
| --- | --- |
| 19.9 | 53.83 |

Table 4: Mean value of TOC

| Sr. No. | Parameter | Soil at disposal site | Soil at nearby disposal site |
| --- | --- | --- | --- |
|  | pH | 8.04 | 8.22 |
|  | EC (mS/cm) | 0.2179 | 0.1375 |
|  | Bulk Density (gm/cc) | 1.29 | 1.50 |
|  | Water holding capacity (%) | 52.66 | 41.41 |
|  | Organic carbon (%) | 0.77 | 0.27 |

Table 5: Mean value of soil parameters

Total organic Carbon of OMWW and OMSW-From table.1 represents, total organic carbon of olive mill waste water and olive mill solid waste. The range of olive mill waste water TOC was 18.30-21.40 g/l and olive mill solid waste was 48.70-59.20 %. Total organic carbon is very high in waste water and olive pomace according to the standard parameter for de oiled cake, FCO-1985. The mean value of organic carbon was 19.9 g/l in waste water and 53.83 % in solid waste (table 4). The high concentration of organic compounds can pollute soil environment. Research author also reported that olive mill waste major component was organic carbon [14].

The presence of high amount of total organic carbon makes this waste useful for preparing compost and it becomes good organic fertiliser. But without proper treatment it will show harmful effect on soil.

Physicochemical Analysis of Soil at disposal site and nearby disposal site-

Table 2 and table.3, reveals that-

- pH- The result data indicated that Soil pH range at disposal site was 8.01-8.10. Soil pH at nearby site was in the range of 8.20-8.25. The mean value of soil pH at disposal site was 8.04 and at nearby site was 8.22 (table 5). Soil pH of disposal site was slightly low compared to nearby site. It is due to the disposal of olive mill solid waste. Olive mill waste water and solid waste are acidic in nature [3,7]. Our experimental analysis data showed that the major component of olive mill waste was organic carbon. This high level of organic compounds lowered soil pH at disposal site. Soil has buffering capacity, so soil pH was not much lowered. Researcher also found olive mill waste water decreased soil pH [15].

According to FAO guidelines for agriculture soil, soil pH mean value of both sites are in the alkaline zone.[16]

- EC- Analysis revealed that soil EC at disposal site and nearby site was in the range of 0.1850-0.2700 mS/cm and 0.1200-0.1600 mS/cm respectively. Mean value of EC at disposal site was 0.2179 mS/cm and nearby site was 0.1375 mS/cm (table 5). A marked variation was noted between both sites. Soil at disposal site has higher EC value. Olive mill waste disposal increased soil EC at disposal site. Olive mill waste contains high concentration of soluble and suspended salts. These waste water and solid waste increased soil salinity.

According to FAO guidelines for agriculture soil, EC of both sites are in the normal zone. [16]

- Bulk Density- The result data showed that soil bulk density at disposal site and nearby site was in the range of 1.21-1.34 gm/cc and 1.50-1.53 gm/cc respectively. The average value of bulk density at disposal site was 1.29gm/cc and nearby site was 1.50 gm/cc (table 5). A significant decrease in bulk density was observed at disposal site soil. Due to the disposal of olive mill waste, soil bulk density was decreased. This result suggests that olive mill wastes improved soil bulk density. Such results are consistent with earlier researcher study [17].

- Water holding capacity-Study data showed that soil water holding capacity at disposal site was in the range of 50-55 % and soil at nearby site water holding capacity range was 35-47%. The mean value of WHC of soil at disposal site and nearby site was 52.66 % and 41.41% respectively (table.5). The analysis revealed that soil water holding capacity was higher at disposal site compare to nearby site. The observation data suggested that olive mill waste improved soil water holding capacity. This result findings are similar with the finding of previous research literature [18].

- Total Organic carbon- It was observed that soil total organic carbon at disposal site was in the range of 0.70-0.89 % and nearby site was in the range of 0.21-0.30%. The average value of soil total organic carbon at disposal site was 0.77% and nearby site was 0.27%. A significant increase in soil total organic carbon was observed at waste disposal site. Soil organic carbon of disposal site was very high compare to nearby site. Olive mill solid waste disposal improved soil carbon content. But this high level of organic carbon pollutes soil at disposal site.

According to FAO guidelines for agriculture soil, Soil organic carbon at disposal site is in high and nearby site is in low carbon content category zone. [16]

We can recycle olive mill waste as organic fertilizer to enhance soil fertility. By composting, utilization of this waste can be solved its disposal problem also. Olive mill waste compost improved soil properties.

STATASTICAL ANALYSIS

Multiple Regression analysis and linear regression analysis are performed to show combined and individual effect of olive mill waste water and solid waste on soil properties. The statistical analysis was performed using by MS excel Toolpak. “Three regression models are developed”-

Table 6: Multiple regression analysis of OMWW and OMSW (TOC) v/s Soil parameters

| Statistical parameter | Soil pH | Bulk density | Water holding capacity |
| --- | --- | --- | --- |
| Multiple R | 0.831971 | 0.870537 | 0.842043 |
| R Square | 0.692176 | 0.757835 | 0.709037 |
| Significance F | 0.004982 | 0.001692 | 0.003866 |
| Pearson r for OMWW | -0.59907 | -0.70568 | 0.716627 |
| P value for OMWW | 0.809792 | 0.248674 | 0.190232 |
| Pearson r for OMSW | -0.83071 | -0.8467 | 0.802588 |
| P value for OMSW | 0.012287 | 0.012566 | 0.036219 |
| Intercept | 8.469026 | 1.87998 | 23.60723 |
| Cofficient TOC of OMWW | -0.00187 | -0.00972 | 0.6066 |
| Coefficient TOC of OMSW | -0.00728 | -0.00736 | 0.3164 |

- “Effect of TOC of Olive mill waste water and olive mill solid waste on soil pH at disposal site”-

Regression equation-

Soil pH=8.469-0.00187(TOCOMWW)-0.00728(TOCOMSW)

- Multiple regression analysis is performed on olive mill waste water and solid waste total organic carbon and soil pH. The analysis result showed that a significant negative decreasing effect of olive mill waste water and solid waste total organic carbon on soil pH is observed. Soil pH was decreased as increase in waste total organic carbon. An inverse relationship is analyzed in this model.

- Significant F was found <.05(Combined effect of OMWW and OMSW TOC). For olive mill waste water TOC, P value >0.05 and for olive mill solid waste TOC p value <0.05, showed that olive mill solid waste TOC is highly significant, it truly affects soil pH more than olive mill waste water TOC. R2=0.6921 showed that 69.21% of pH variance of disposal site is explained by olive mill waste water and solid waste TOC.

- Pearson coefficient r for olive mill waste water was found -0.59907 negative relationship was found.

- Pearson coefficient r for olive mill solid waste was found -0.83071 negative relationship was found.

- The linear regression analysis for separately olive mill waste water TOC and soil pH was –

Regression equation- SoilpH= -0.0179(TOCOMWW)+8.3958

It showed negative relationship of soil pH and TOC of olive mill waste water. R2=0.3589, only 35.89% of soil pH variation is explained by TOC of OMWW.

- The linear regression analysis for separately olive mill solid waste TOC and soil pH was-

Regression equation- SoilpH=-0.0077(TOCOMSW)+8.453

It showed negative relationship of soil pH and TOC of OMSW. R2=0.6901, 65.01% soil pH variation is explained by OMSW TOC.

Fig :1, linear regression graph of                          Fig :2, linear regression graph of

omww TOC v/s soil pH                                          omsw TOC v/s soil pH

- “Effect of TOC of Olive mill waste water and olive mill solid waste on soil Bulk Density”-

Regression equation-

SoilBD=1.87998-0.00972(TOComww)-0.00736(TOComsw)

- Multiple regression analysis is performed on olive mill waste water and solid waste total organic carbon and soil bulk density. The analysis result showed that a significant negative decreasing effect of olive mill waste water and solid waste total organic carbon on soil bulk density is observed. Soil bulk density was decreased as increasing in waste total organic carbon. An inverse relationship is analyzed in this model.

- Significant F was found <.05(Combined effect of OMWW and OMSW TOC). For olive mill waste water TOC, P value >0.05 and For olive mill solid waste TOC p value <0.05, showed that olive mill solid waste TOC is highly significant, it truly affects soil bulk density more than olive mill waste water TOC. R2=0.7578 showed that 75.78% of disposal site soil bulk density variance is explained by olive mill waste water and solid waste TOC.

- Pearson coefficient r for olive mill waste water was found -0.70568 negative relationship was observed.

- Pearson coefficient r for olive mill solid waste was found -0.8467 negative relationship was observed. It is highly significant effect of OMSW is observed on soil bulk density.

- The linear regression analysis for separately olive mill waste water TOC and soil bulk density was –

Regression equation- SoilBD= -0.0258(TOCOMWW)+1.8079

It showed negative relationship of soil bulk density and TOC of olive mill waste water. R2=0.5014, only 50.14% of soil bulk density variation is explained by TOC of OMWW.

- The linear regression analysis for separately olive mill solid waste TOC and soil bulk density was-

Regression equation- SoilBD=-0.0087(TOCOMSW)+1.7594

It showed negative relationship of soil Bulk density and TOC of OMSW. R2=0.5868, 58.68% soil bulk density variation is explained by OMSW TOC.

Fig :3, linear regression graph of omww             Fig :4, linear regression graph of omsw

TOC v/s soil bulk density                                   TOC v/s soil bulk density

3. “Effect of TOC of Olive mill waste water and olive mill solid waste on soil Water holding capacity”-

SoilWHC=23.607+0.6066(TOCOMWW) +0.3164(TOCOMSW)

- Multiple regression analysis is performed on olive mill waste water and solid waste total organic carbon and soil water holding capacity of disposal site. The analysis result showed that a significant positive effect of olive mill waste water and solid waste total organic carbon on soil WHC is observed. Soil WHC is increased as increase in waste total organic carbon. Soil WHC is directly proportional to waste TOC.

- Significant F was found <.005(Combined effect of OMWW and OMSW TOC). Model is highly significant.

- For olive mill waste water TOC, P value >0.05 and For olive mill solid waste TOC p value <0.05, showed that olive mill solid waste TOC is highly significant, it truly affects soil water holding capacity more than olive mill waste water TOC. R2=0.7090 showed that 70.90 % of WHC variance of disposal site is explained by olive mill waste water and solid waste TOC.

- Pearson coefficient r for olive mill waste water was 0.7166, positive relationship was observed.

- Pearson coefficient r for olive mill solid waste was found 0.8025, positive relationship was observed.

- The linear regression analysis for separately olive mill waste water TOC and soil water holding capcaity was –

Regression equation- SoilWHC= 1.2935(TOCOMWW)+26.914

It showed positive relationship of soil WHC and TOC of olive mill waste water. R2=0.5136, only 51.36% of soil at disposal site water holding capacity variation is explained by TOC of OMWW.

- The linear regression analysis for separately olive mill solid waste TOC and soil water holding capacity was-

Regression equation- SoilWHC= 0.4388(TOCOMSW)+29.044

It showed positive relationship of soil WHC and TOC of OMSW. R2=0.6165, 61.65% of soil at disposal site water holding capacity variation is explained by OMSW TOC.

Fig :5, linear regression graph of                    Fig :6, linear regression graph of

omww TOC v/s WHC                                   omsw TOC v/s WHC

From the statistical analysis model, it is proved that soil bulk density and water holding capacity was improved at disposal site. It is due to the disposal of olive mill waste. Soil bulk density and WHC is highly influenced from olive mill solid waste, rather than OMWW. Soil pH was decreased, but it is not much altered.

CONCLUSION

The Experimental study showed that olive mill waste disposal improved soil at disposal site. Soil pH was slightly decreased, soil EC was increased but it lied in normal limit. Soil bulk density was decreased and water holding capacity was increased at disposal site. Soil of nearby site has low carbon content. Olive mill waste increased high soil organic carbon. It pollutes soil environment. The statistical analysis result showed that olive mill solid waste is highly influenced soil pH, bulk density and water holding capacity compare to olive mill waste water. Soil bulk density and water holding capacity is improved by olive mill waste at disposal site. Our previous study [3,7] and current study suggested that olive mill solid waste has potential to be an organic fertilizer. Through composting, we can recycle it in valuable compound. We cannot direct use of olive mill solid waste because organic carbon contamination in soil is observed. After making compost, olive mill solid waste negative effects will decrease. The soil area of study site is alkaline and poor in organic carbon. The highly organic carbon contain olive mill solid waste compost will effective in agriculture use in this area. By recycling this organic agro-industrial waste may provide a solution for the management of this waste.

This study concluded that these wastes positively impacted soil properties. However, heavy metals did not analyse, these waste negative effects cannot be mentioned.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFERENCES

- Marco Ruggeri, Fabrizio DeLuca, Amedeo Ungolo, Barbara Vigani, Alejandro J. Pardes, Eleonora Russo, Maria Grazia Bottone, Eleonora Bianchi, Franca Ferrari, Silvia Rossi, Olive mill wastewater: From by-product to smart antioxidant material, InternationalJournalofPharmaceutics,8,100301,(2024). https://doi.org/10.1016/j.ijpx.2024.100301

- A. Elabdouni, K. Haboubi, I. Merimi, M.S.M El Youbi, Olive mill wastewater (OMW) production in the province of Al-Hoceima (Morocco) and their physico-chemical characterization by mill types, Journal of Materials today, proceedings, 27(4),3145-3150, (2020). https://doi.org/10.1016/j.matpr.2020.03.806

- Arti Aseri, Sushma Jain, Anuradha Aseri, A study of physicochemical parameters of waste water of olive oil refinery, Lunkaransar, Bikaner, Journal of Pollution Research, 43(3-4),320-322, (2024).http://doi.org/10.53550/PR.2024.v43i03-04.020

- J.A Alburquerque, J Gonzalvez, D Garcia, J Cegarra,Agrochemical characterisation of “alperujo”, a solid by-product of the two-phase centrifugation method for olive oil extraction,JournalofBioresourceTechnology,91(2),195-200,(2004). https://doi.org/10.1016/S0960-8524(03)00177-9

- S. Dermeche, M. Nadour, C. Larroche, F. MoultiMati, P. Michaud, Olive mill wastes: Biochemical characterizations and valorization strategies, Journal of Process Biochemistry,48(10),1532-1552,(2013). https://doi.org/10.1016/j.procbio.2013.07.010

- Manuel Tejada, Concepcion Benitez, Effects of different organic wastes on soil biochemical properties and yield in an olive grove, Journal of Applied Soil Ecology, 146,103371, (2020).  https://doi.org/10.1016/j.apsoil.2019.103371

- Arti Aseri, Sushma jain, Physicochemical analysis of olive mill solid waste of olive oil refinery, Lunkaransar (Bikaner), IJARESM, 12(9),310-313, (2024).

- Roberto Garcia-Ruiz, M Victoria Ochoa, M Belen Hinojosa and Beatriz Gomez-Munoz, Improved soil quality after 16 years of olive mill pomace application in olive oil groves, Journal of Agronomy for sustainable Development, 32,803-810,(2012). DOI:10.1007/s13593-011-0080-7.

- Antonia Fernandez Hernandez, Asuncion roig, Nuria Serramia, Concepcion Garcia Ortiz civantos, Miguel A. Sanchez Monedero, Application of compost of two-phase olive mill waste on olive grove: Effects on soil, olive fruit and olive oil quality, Journal ofWasteManagement,34(7),1139-1147,(2014). https://doi.org/10.1016/j.wasman.2014.03.027

- Giovanna Cucci, Giovanni Lacolla and Leonardo Caranfa, Improvement of soil properties by application of olive oil waste, Journal of Agronomy for sustainable Development,28,521-526,(2008).

- K. Chartzoulakis, G. Psarras, M. Moutsopoulou, E. Stefanoudaki,AgricultureApplication of olive mill wastewater to a Cretan olive orchard: Effects on soil properties, plant performance and the environment, Journal ofEcosystemsandEnvironment,138(3-4),293-298,(2010). https://doi.org/10.1016/j.agee.2010.05.014

- Munir J. M. Rusan, Ammar A. Albalasmeh and Hanan I. Malkawi, Treated Olive Mill Wastewater Effects on Soil Properties and Plant Growth, Journal of Water, Air and Soil Pollution, 227,135,(2016). DOI:10.1007/s11270-016-2837-8

- Ana Garcia-Randez, Silvia Sanchez Mendez, Luciano Orden, Francisco Javier Andreu-Rodriguez, Miguel Angel Mira-Urios, Jose A. Saez-Tovar, Encarnacion Martinez-Sabater, Maria Angeles Bustamante, Maria Dolores Perez-Murcia and Raul Mora, Evaluation of Olive Mill Waste Compost as a Sustainable Alternative to Conventional Fertilizers in Wheat Cultivation, Journal of Agriculture, 15,1543, (2025). https://doi.org/10.3390/agriculture15141543

- Youness Bouhia, Yedir Ouhdouch, Karim Lyamlouli, Olive mill waste sludge: From permanent pollution to a highly beneficial organic biofertilizer: A critical review and future perspectives, Journal of Ecotoxicology and Environmental safety,259,114997, (2023). 10.1016/j.ecoenv.2023.114997

- Jehan Khalil, Abd Al Karim Jaafar, Hassan Habib, Sirine Bouguerra, VeronicaNogueira, Andres Rodriguez-Seijo, The impact of olive mill wastewater on soil properties, nutrient and heavy metal availability - A study case from Syrian vertisols,JournalofEnvironmentalManagement,351,119861,(2024). https://doi.org/10.1016/j.jenvman.2023.119861

- Muhr et al., FAO soils Bulletin, 38/1, Soil and plant testing and analysis: Modified Rating chart for soil test data, (1965).

- Luis Parras alcantara, Beatriz Lozano Garcia, Saskia Keesstra, Artemi Cerda, Eric C. Brevik, Long-term effects of soil management on ecosystem services and soil loss estimation in olive grove top soils, Journal of Science of the total Environment, 571,498-506, (2016). https://doi.org/10.1016/j.scitotenv.2016.07.016

- Ghaida Abu Rumman, Effect of Olive Mill Solid Waste on Soil Physical Properties, InternationalJournalofSoilScience,11(3)94-101(2016). DOI: 10.3923/ijss.2016.94.101

- Muezzinoglu, A., Future trends in olive industry waste management: A literature review. Wastewater from Olive Oil Production: Environmental Impacts, Treatment and Valorisation, 221-297, (2023). DOI:10.1007/978-3-031-23449-1_10

- F. Atia, K. Elkarrach, S. Biyada, O. Laidi, A. Omor, M. Benlemlih, M. Merzouki, Treatment of olive mill wastewater by filtration through fly ash: an efficient and economic strategy for purification, International Journal of Environmental Science & Technology,22, 4511–4520, (2025). DOI:10.1007/s13762-024-06162-x

- Layla Moustafa Fleyfel, Joseph Matta, Nicole Fakhoury Sayegh, Audrey Allavena, Nathalie Karpel Vel Leitner, Marie Deborde &Nasma Hamdi El Najjar, Olive mill wastewater treatment using coagulation/flocculation and filtration processes, Journal of heliyon, 10,22, (2024).  DOI:10.1016/j.heliyon.2024.e40348