Machine Learning-based Automatic Grading of Mangoes

Abstract

Automation in agriculture can improve product quality and consistency while reducing labor and time costs. Manual mango grading is often slow, subjective, and error-prone; therefore, an automated grading approach is desirable. In this study, we propose an image-based mango grading framework that classifies mangoes into four grades (green, semi-ripe, ripe, and rotten) using five models: Artificial Neural Network (ANN), Decision Tree (DT), k-Nearest Neighbors (KNN), Support Vector Machine (SVM), and Convolutional Neural Network (CNN). We collected 4,207 mango images from multiple markets and locations and used a 70%/30% train–test split. Performance was evaluated using Accuracy, Precision, Recall, and F1-score across five repeated runs. CNN achieved the best overall performance (mean accuracy ≈ 0.99) and consistently outperformed the classical machine-learning baselines, indicating that deep learning can provide a reliable solution for automated mango grading and can be extended to other agricultural products.

Keywords: Mango grading; Computer vision; Image classification; Machine learning; Deep learning; Convolutional Neural Network (CNN); Fruit quality assessment.

- Introduction

Bangladesh is an agricultural country, and a large portion of its population depends directly on agricultural production, which contributes significantly to the national GDP. Among various agricultural products, mango is one of the most important and commercially valuable fruits. Mango is primarily produced in South and Southeast Asia and is widely known as the “King of Fruits” due to its high demand and nutritional value, particularly vitamins A and C. In recent years, mango production has increased to meet growing local demand, and several varieties such as Himsagar and Khirshapati are highly popular in both domestic and international markets. Consequently, proper grading of mangoes is essential to ensure consistent quality and to enhance export value.

Mango grading typically depends on multiple attributes, including shape, size, color, maturity, and weight. At present, sorting and grading are mostly performed manually; however, manual grading is time-consuming, labor-intensive, and often inconsistent, as grading accuracy can vary from one person to another. To overcome these limitations, there is an increasing need for an automated grading system using computer vision. Digital image processing and computer vision have advanced significantly and are widely applied in areas such as medical imaging, intelligent sensing, remote sensing, and industrial inspection. In agriculture, these technologies enable automation in precision farming, robotics, non-destructive inspection, quality control, and automated sorting and grading.

Previous studies have explored automated grading methods for various fruits and vegetables, including tomatoes, dates, apples, strawberries, peaches, and tamarind. Inspired by these developments, this study aims to develop an automated mango grading system and to evaluate the performance of multiple classification models, including Artificial Neural Network (ANN), Decision Tree (DT), k-Nearest Neighbors (KNN), Support Vector Machine (SVM), and Convolutional Neural Network (CNN). Compared to the conventional manual approach—which often requires skilled labor, higher costs, supervision, and extensive documentation—an automated system can be faster, more cost-effective, and more consistent in performance. Therefore, this study proposes a machine learning and deep learning-based mango grading framework. his study develops an automated mango grading system and compares five classification models—ANN, DT, KNN, SVM, and CNN—to evaluate their effectiveness for four-grade mango classification. The specific objectives of the work are listed below:

- To collect and label mango images to build a four-class dataset (Green, Semi-ripe, Ripe, Rotten).

- To preprocess images to reduce noise and standardize the input for model training.

- To train four classical machine-learning models (ANN, DT, KNN, SVM) and one deep-learning model (CNN) for mango grading.

- To evaluate and compare model performance using Accuracy, Precision, Recall, and F1-score across multiple runs.

- Literature Review

In this section, we review related work on computer-vision-based sorting, grading and quality assessment. For example, the authors in [1] classified mangoes into three size categories (big, semi-large and small) and used a geometry-based approach to estimate mass from the projected area, reporting 97% accuracy. Seema and colleagues [2] presented an automatic mango grading system using machine learning, where SVM and fuzzy logic were applied and the fuzzy approach achieved lower accuracy. Physical properties such as size, shape, surface area and color were analyzed in [3], including image acquisition, segmentation and property computation. Yossy et al. [4] developed a mango grading system based on color detection (ripe vs. unripe) using ANN. Support Vector Machines and fuzzy logic were combined in [5] to determine grade based on shape and color, and a fuzzy expert system was used in [6] to classify mango diseases with 98.88% overall accuracy. Fuzzy-logic-based mango grading using color and size was also reported in [7], and a hybrid system combining ANN with Fourier descriptors and spatial-domain analysis was evaluated in [8].
Beyond mangoes, several studies have explored automated sorting using sensors, vision and learning models. A microcontroller-based fruit sorting system considering dimension and color was presented in [9]. Internal defect detection in mangoes using soft X-rays was studied in [10], while volume estimation from 2D images was developed in [11]. Color and size feature extraction for mango grading was proposed in [12], and volume estimation for apples and oranges using image processing was investigated in [13]. Mango size and defect analysis using image analysis and nearest-neighbor methods was reported in [14]. Similar maturity/defect classification approaches were explored for oranges and other fruits in [15]–[19]. A comparative discussion of ANN, SVM, DT and KNN was provided in [20], and a machine-vision-based automatic mango sorting and grading system was developed in [21]. These studies motivate our image-based mango grading approach and our comparison of multiple learning models.

- Methodology

This study presents an image-based mango grading methodology using five classification algorithms: Artificial Neural Network (ANN), Decision Tree (DT), K-Nearest Neighbor (KNN), Support Vector Machine (SVM), and Convolutional Neural Network (CNN). The collected mango images are grouped into four grades—green, semi-ripe, ripe and rotten—and are used to train and evaluate the models.

3.1	Proposed System

A block diagram and an algorithm are used to describe the proposed system (Fig. 1). First, the collected mango images are resized and filtered, and additional preprocessing steps are applied to reduce noise and standardize the input. Next, relevant features are extracted. Finally, the machine learning models are trained to grade the mango images. After training, the models are evaluated using the test images and the overall performance is computed.

Figure 1: Block diagram of the proposed system

The steps of the proposed approach are summarized in Algorithm 1.

| Algorithm 1: Mango Grading |  |
| --- | --- |
| 1 | Read training mango images from the dataset |
| 2 | Split the dataset into training (70%) and testing (30%) sets. |
| 3 | Preprocess images and extract features (for classical ML models). |
| 4 | Train the classifiers (ANN, DT, KNN, SVM, and CNN) using the training set. |
| 5 | Predict grades for the test images using each trained model. |
| 6 7 | Compute performance metrics (Accuracy, Precision, Recall, F1-score). Repeat Steps 2–6 for multiple runs and report the average performance. |

3.2	Dataset Description

In this work, we collected mango images from multiple markets and locations. The images were categorized into four classes—green, semi-ripe, ripe, and rotten—as illustrated in Fig. 2. Each class contains more than one thousand images (Table 1). The dataset was randomly split into training (70%) and testing (30%) sets.

Figure 2: Four types of mango grading

Images were captured using a digital camera under natural lighting with varied backgrounds. The camera-to-object distance was approximately 60 cm, and all images were resized to 256 × 256 pixels for training and evaluation. Different mango varieties such as Surma, Fozli, Arsina, Amrapali, Mollika, and Zinuk were included.

Table 1: Summary of the mango image dataset

| Class | Grade | Number of images |
| --- | --- | --- |
| Class a | Green | 1012 |
| Class b | Ripe | 1035 |
| Class c | Semi-ripe | 1151 |
| Class d | Rotten | 1009 |

3.3 Image Pre-processing

In this work, we resized images using MATLAB’s imresize() function and applied imfilter() for basic noise reduction and enhancement. We used rgb2gray() to convert RGB images to grayscale where necessary, and graycomatrix() to compute texture features.

3.4 Feature Extraction

For the classical machine learning models (ANN, DT, KNN, and SVM), handcrafted image descriptors (e.g., color and texture features) were used as input features, while CNN learned features automatically from the images. Feature extraction (or feature learning) refers to deriving measurable characteristics from an image that help differentiate one class from another. The goal is to obtain a compact set of discriminative features that supports accurate classification. In our experiments, CNN learns image features automatically. Fig. 3 illustrates the feature learning process used in the model.

Figure 3: Automatic feature extraction using CNN

3.5 Machine Learning Models

Machine learning is an important branch of artificial intelligence. In this study, five different algorithms are trained and applied for mango grading: Artificial Neural Network (ANN), Decision Tree (DT), K-Nearest Neighbor (KNN), Support Vector Machine (SVM) and Convolutional Neural Network (CNN). ANN can learn complex non-linear relationships by imitating the behavior of neurons. Decision trees classify data step-by-step using feature-based rules that are easy to interpret. KNN is a distance-based approach in which a new sample is assigned the class of its nearest neighbors and often works well on smaller datasets. SVM finds an optimal hyperplane that maximizes separation between classes, enabling high-accuracy classification. CNN is a deep learning model particularly suited for image data; it can automatically learn hierarchical features and usually performs best when sufficient training images are available [22], [23], [24].

  - Model Performance Evaluation

For the four-class classification task, Precision, Recall, and F1-score were computed per class and then averaged across classes to report overall performance. The performance of the proposed approach is evaluated using four standard metrics: Accuracy, Precision, Recall and F1-score. These metrics help assess how reliably the models classify mangoes into different grading categories [25], [26].

Accuracy: Overall correctness of predictions, defined as (TP + TN) / (TP + TN + FP + FN). For the four-class classification task, Precision, Recall, and F1-score were computed per class and then averaged across classes to report overall performance.

where TP, TN, FP and FN denote true positive, true negative, false positive and false negative, respectively.

Precision: Proportion of correct positive predictions, defined as TP / (TP + FP).

Recall: Ability to detect all positive samples, defined as TP / (TP + FN).

F1-score: Harmonic mean of Precision and Recall, defined as 2 × (Precision × Recall) / (Precision + Recall).

- Results and Discussion

This section presents the experimental results of five machine learning models for mango grading: ANN, DT, KNN, SVM and CNN. For each experiment, 70% of the images were used for training and 30% for testing. The performance of each model was evaluated and compared quantitatively.

Classification Result of ANN: We evaluated ANN using the test set and summarized performance using its confusion matrix. Across repeated tests, the model achieved an accuracy above 80% for the four-class mango grading task (Fig. 4).

Figure 4: Confusion matrix for ANN                             Figure 5: Confusion matrix for DT

Classification Result of DT: The Decision Tree (DT) model was evaluated using a confusion matrix on the test set. Mango grading accuracy was computed as the number of correct predictions divided by the total number of test samples. DT achieved an accuracy above 80% in our experiments (Fig. 5).

Figure 6: Confusion matrix for KNN                     Figure 7: Confusion matrix for SVM

KNN Classification Result: The KNN model produced slightly different outputs across test samples; therefore, we report average performance across multiple evaluations. KNN achieved an accuracy above 70% for mango grading in our experiments (Fig. 6).

SVM Classification Result: SVM achieved an accuracy of 72.7% in classifying the test images (Fig. 7). The ripe class was identified with higher accuracy than the semi-ripe class, which showed relatively lower classification performance.

CNN Classification Result: For CNN, we trained the model using a large set of images divided into four grading categories. CNN achieved an accuracy above 90% (Fig. 8), indicating superior performance for image-based mango grading when sufficient data are available.

Figure 8: Confusion matrix for CNN

To assess reliability, each model was executed five times. In each run, Accuracy, Precision, Recall, and F1-score were computed, and average values were reported to reduce random variation. We then calculated the average values to reduce the effect of random variation and to make the comparison more reliable. The performance results are presented in Table 2, and the comparative bar chart is shown in Fig. 9.

Table 2: Performance comparison of different models

| Run | Models | Accuracy | Precision | Recall | F1-score |
| --- | --- | --- | --- | --- | --- |
| 1 | ANN | 0.813 | 0.796 | 0.796 | 0.796 |
|  | KNN | 0.779 | 0.7634 | 0.763 | 0.7634 |
|  | SVM | 0.727 | 0.7116 | 0.712 | 0.7116 |
|  | DT | 0.841 | 0.8242 | 0.824 | 0.8242 |
|  | CNN | 0.991 | 0.9812 | 0.981 | 0.9812 |
| 2 | ANN | 0.782 | 0.7664 | 0.766 | 0.7664 |
|  | KNN | 0.753 | 0.7379 | 0.738 | 0.7379 |
|  | SVM | 0.726 | 0.7115 | 0.712 | 0.7115 |
|  | DT | 0.811 | 0.7938 | 0.794 | 0.7938 |
|  | CNN | 0.991 | 0.9812 | 0.981 | 0.9812 |
| 3 | ANN | 0.791 | 0.7752 | 0.775 | 0.7752 |
|  | KNN | 0.771 | 0.7566 | 0.757 | 0.7566 |
|  | SVM | 0.728 | 0.7134 | 0.713 | 0.7134 |
|  | DT | 0.82 | 0.8036 | 0.804 | 0.8036 |
|  | CNN | 0.987 | 0.9673 | 0.967 | 0.9673 |
| 4 | ANN | 0.744 | 0.7291 | 0.729 | 0.7291 |
|  | KNN | 0.787 | 0.7713 | 0.771 | 0.7713 |
|  | SVM | 0.757 | 0.7419 | 0.742 | 0.7419 |
|  | DT | 0.846 | 0.8291 | 0.829 | 0.8291 |
|  | CNN | 0.992 | 0.9822 | 0.982 | 0.9822 |
| 5 | ANN | 0.773 | 0.7576 | 0.758 | 0.7576 |
|  | KNN | 0.777 | 0.7615 | 0.762 | 0.7615 |
|  | SVM | 0.724 | 0.7095 | 0.71 | 0.7095 |
|  | DT | 0.812 | 0.7957 | 0.796 | 0.7957 |
|  | CNN | 0.992 | 0.9822 | 0.982 | 0.9822 |

Table 2 summarizes the performance of ANN, DT, KNN, SVM and CNN using Accuracy, Precision, Recall and F1-score. To reduce the effect of randomness in data splitting and training, each model was evaluated over multiple runs, and the results show consistent trends. CNN delivers the highest and most stable performance across all metrics, indicating strong robustness when a large training dataset is available. Among the classical machine learning models, DT performs best overall, suggesting that rule-based feature splits remain effective for this grading task. ANN and KNN provide moderate performance, whereas SVM performs comparatively lower in this dataset. Overall, the results indicate that CNN is the most suitable model for automated mango grading, with DT serving as the strongest traditional baseline.

Figure 9: Performance comparison among the models

As shown in Fig. 9–10, Precision and Recall for each model in Run 5 are almost equal, indicating balanced behavior in terms of false positives and false negatives. CNN achieved the highest Precision/Recall (≈ 0.982), followed by DT (≈ 0.796). ANN and KNN achieved moderate scores (≈ 0.76), while SVM achieved the lowest performance (≈ 0.71).

Figure 10: Precision–Recall bar chart

As shown in Table 1 and Fig. 9–10, model performance is reported using four widely used metrics: Accuracy, Precision, Recall and F1-score. Accuracy measures the overall proportion of correct predictions. Precision measures how often predicted samples of a grade are correct, while Recall measures how many true samples of a grade are successfully retrieved. F1-score provides a balanced summary of Precision and Recall. To ensure that the comparison is not dominated by a single random split, results are computed across multiple runs and then compared. Fig. 9–10 show that CNN achieves the best performance across all runs, with consistently high Accuracy and Precision/Recall values, and only minor variation between runs. This indicates that CNN effectively learns discriminative visual patterns for mango grading when sufficient training data are available. Among the traditional methods, DT is the best-performing baseline, but it remains noticeably below CNN in overall performance. In addition, Precision and Recall are close to each other for most models, suggesting a balanced trade-off between false positives and false negatives. Overall, the results confirm that CNN is the most effective model for this task, while DT provides the strongest classical alternative.

In this study, five machine learning models—ANN, DT, KNN, SVM, and CNN—were tested and compared to develop an image-based automated mango grading system. The dataset was divided into four classes (Green, Semi-ripe, Ripe, Rotten) for training and testing, allowing for realistic comparison of the models’ performance. The analysis of the results shows that the CNN model consistently performed the highest. The main reason for this is that CNN can automatically learn the subtle patterns of color, texture, and shape within an image, and with sufficient data, these learned features are able to generalize well across different environments/variations. As a result, CNN performs very well in classes with relatively clear visual differences such as “Green vs. Ripe” or “Ripe vs. Rotten”. On the other hand, Decision Tree (DT) performed relatively well among the classical models. The advantage of Decision Tree is that it is interpretable; that is, the decision made based on which feature/characteristic can be explained in the form of a rule. In a real-world application (e.g. market/packing house), if a decision maker wants to know the answer to the question “Why is this mango marked as Semi-ripe?”, DT can provide a relatively easy explanation. ANN and KNN gave mediocre results. ANN can generally perform well, but its performance is often highly dependent on the amount of data, network architecture, and tuning. In the case of KNN, performance and computational cost can be limited when the data is large and the class boundaries are complex. SVM performed relatively poorly—this is an important observation. In practice, the “Semi-ripe” class can be the most challenging, because the color/texture changes slowly in the Semi-ripe stage and in many cases there is visual overlap with “Green” or “Ripe”. As a result, class separation becomes complicated and it can be relatively difficult for SVM to create a stable decision boundary, especially if the feature set is not optimally designed. Overall, the results of this study indicate that CNN-based methods are the most effective and realistic for mango grading when large and diverse image datasets are available, and Decision Tree can be considered a strong baseline in terms of interpretability and ease of implementation.

Potential Impact

- Time and labor savings: Automated grading can reduce manual effort, speed up sorting, and lower operational costs in packing houses and processing centers.

- Improved consistency: Unlike human grading, automated systems can apply consistent criteria, reducing subjective variation and improving standardization for large markets and exports.

- Higher market value and reduced rejection: Reliable grading supports transparent pricing and packaging decisions, which can improve customer satisfaction and reduce rejection rates.

- Conclusion

Automated fruit classification introduces artificial intelligence into the agriculture sector by enabling fast, consistent and non-destructive quality assessment. In this paper, we developed an image-based mango grading system and compared five machine learning models (ANN, DT, KNN, SVM and CNN). Experimental results demonstrate that CNN performs best when a large number of training images are available, achieving the highest accuracy and the most stable performance across runs. Therefore, CNN is a suitable model for automating mango grading and can help overcome the limitations of conventional manual grading by providing faster, more accurate and potentially lower-cost operation.

DISCLAIMER (ARTIFICIAL INTELLIGENCE): The authors used generative AI tools only for language polishing and readability improvement. All scientific content, results, and interpretations were verified and approved by the authors.

References

[1]	M. A. Momin, M. T. Rahman, M. S. Sultana, C. Igathinathane, A. T. M. Ziauddin, and T. E. Grift, “Geometry-based mass grading of mango fruits using image processing,” Information Processing in Agriculture, vol. 4, no. 2, pp. 150–160, 2017, doi: 10.1016/j.inpa.2017.03.003.

[2]	C. Engineering, A. Kumar, and G. S. Gill, “Automatic Fruit Grading and Classification System Using Computer Vision : A Review,” no. I, pp. 598–603, 2015, doi: 10.1109/ICACCE.2015.15.

[3]	Panitnat. Yimyam, Chalidabhongse. Thanarat, and Suwanee. Sirisomboon, Panmanas. Boonmung, “Physical Properties Analysis of Mango using Computer Vision,” Iccas2005, p. 5, 2005.

[4]	E. H. Yossy, J. Pranata, T. Wijaya, H. Hermawan, and W. Budiharto, “Mango Fruit Sortation System using Neural Network and Computer Vision,” Procedia Computer Science, vol. 116, pp. 596–603, 2017, doi: 10.1016/j.procs.2017.10.013.

[5]	N. B. A. Mustafa, S. K. Ahmed, Z. Ali, W. B. Yit, A. A. Z. Abidin, and Z. A. Md Sharrif, “Agricultural produce sorting and grading using support vector machines and fuzzy logic,” in ICSIPA09 - 2009 IEEE International Conference on Signal and Image Processing Applications, Conference Proceedings, 2009, pp. 391–396. doi: 10.1109/ICSIPA.2009.5478684.

[6]	R. Pandey, N. Gamit, and S. Naik, “A novel non-destructive grading method for Mango (Mangifera Indica L.) using fuzzy expert system,” Proceedings of the 2014 International Conference on Advances in Computing, Communications and Informatics, ICACCI 2014, pp. 1087–1094, 2014, doi: 10.1109/ICACCI.2014.6968366.

[7]	H. G. Naganur, S. S. Sannakki, V. S. Rajpurohit, and R. Arunkumar, “Fruits Sorting and Grading using Fuzzy Logic,” vol. 1, no. 6, pp. 117–122, 2012.

[8]	A. M. Aibinu, M. J. E. Salami, A. A. Shafie, N. Hazali, and N. Termidzi, “Automatic Fruits Identification System Using Hybrid Technique,” 2011, doi: 10.1109/DELTA.2011.47.

[9]	D. Jhala, A. Bagade, S. Naik, S. Lad, and B. Shah, “Microcontroller Based Fruit Sorting Using Image Processing In Matlab,” IOSR Journal of Engineering (IOSRJEN), vol. 4, pp. 43–46, 2018.

[10]	K. A. & K. Veena. T, D. V. Chidanand, “Quality Analysis of Mango Fruit with Fruit Fly Insect by Non-Destructive Soft X-Ray Method,” International Journal of Agricultural Science and Research (IJASR), vol. 5, no. 3, pp. 37–46, 2015.

[11]	K. A. Forbes and G. M. Tattersfield, “Estimating fruit volume from digital images,” IEEE AFRICON Conference, vol. 1, pp. 107–112, 1999, doi: 10.1109/afrcon.1999.820775.

[12]	A. MVyas, B. Talati, and S. Naik, “Quality Inspection and Classification of Mangoes using Color and Size Features,” International Journal of Computer Applications, vol. 98, no. 1, pp. 1–5, 2014, doi: 10.5120/17144-7161.

[13]	D. Kalantari, “Size determination of apple and orange fruits using the image processing technique.,” Advances in Agriculture, Sciences and Engineering Research, vol. 4, no. 2, pp. 1537–1543, 2014.

[14]	T. U. Ganiron Jr, “Size properties of mangoes using image analysis,” International Journal of Bio-Science and Bio-Technology, vol. 6, no. 2, pp. 31–42, 2014.

[15]	C. P. Diana Carolina and N. T. Deivis David, “Classification of oranges by maturity, using image processing techniques,” 2014 3rd International Congress of Engineering Mechatronics and Automation, CIIMA 2014 - Conference Proceedings, 2014, doi: 10.1109/CIIMA.2014.6983466.

[16]	C. K. Shetty, S. B. Kulkarni, U. P. Kulkarni, K. Ramesh, and R. Hegadi, “Defect Recognition of Fruit using Statistical Approach,” vol. 22, no. 7, pp. 331–334, 2015.

[17]	C. Science and M. Studies, “Weight Based Fruits Sorting System,” pp. 308–313, 2015.

[18]	M. F. Ibrahim, F. S. Ahmad Sa’ad, A. Zakaria, and A. Y. Md Shakaff, “In-Line Sorting of Harumanis Mango Based on External Quality Using Visible Imaging,” Sensors, vol. 16, no. 11, p. 1753, 2016.

[19]	. S. J., “Grading of Tomatoes Using Digital Image Processing on the Basis of Size,” International Journal of Research in Engineering and Technology, vol. 05, no. 10, pp. 109–111, 2016, doi: 10.15623/ijret.2016.0510020.

[20]	R. Gaur and V. S. Chouhan, “Classifiers in Image processing,” International Journal on Future Revolution in Computer Science & Communication Engineering IJFRCSCE, pp. 22–24, 2017.

[21]	C. S. Nandi, B. Tudu, and C. Koley, “Machine vision based techniques for automatic mango fruit sorting and grading based on maturity level and size,” in Sensing Technology: Current Status and Future Trends II, vol. 8, 2014, pp. 27–46. doi: 10.1007/978-3-319-02315-1_2.

[22]	S. S. T. Gontumukkala, Y. S. V. Godavarthi, B. R. R. T. Gonugunta, R. Subramani, and K. Murali, “Analysis of image classification using SVM,” in 2021 12th International Conference on Computing Communication and Networking Technologies (ICCCNT), 2021, pp. 1–6.

[23]	S. Y. Chaganti, I. Nanda, K. R. Pandi, T. G. Prudhvith, and N. Kumar, “Image Classification using SVM and CNN,” in 2020 International conference on computer science, engineering and applications (ICCSEA), 2020, pp. 1–5.

[24]	H. Manzoor, F. Khursheed, and A. M. Hafiz, “Finger vein recognition using an ensemble of KNN classifiers based on robust image features,” Signal Image Video Process, vol. 19, no. 10, Oct. 2025, doi: 10.1007/s11760-025-04369-0.

[25]	S. M. A. Al Shuaeb, A. Hossen, M. A. H. Dinar, and U. K. Roy, “Analyzing Different Architectures of Convolutional Neural Networks for Tomato Grading System,” Asian Journal of Research in Computer Science, vol. 17, no. 12, pp. 137–147, Dec. 2024, doi: 10.9734/ajrcos/2024/v17i12534.

[26]	B. Hossain, U. K. Roy, Shamsunnaher, Md. S. A. A. Kawser, and S. M. A. Al Shuaeb, “Image-based Pest Identification Using Support Vector Machine for Agricultural Crop Protection,” Asian J Res Crop Sci, vol. 10, no. 3, pp. 13–22, Jun. 2025, doi: 10.9734/ajrcs/2025/v10i3368.