Biodegradation and Detoxification of Distillery Spent Wash by Aspergillus niger and Pseudomonas fluorescens

ABSTRACT

Distillery spent wash (DSW) is a highly polluted industrial effluent requiring effective and eco-friendly treatment strategies. An experiment was conducted during 2024–25 at the Soil and Water Management Research Unit, Navsari Agricultural University (NAU), Navsari, to evaluate the efficiency of microbial inoculants in the degradation and detoxification of distillery spent wash using a Completely Randomized Design with Factorial Concepts along with three microbial treatments-M0 (no inoculation), M1 (Aspergillus niger at 1 g/L) and M2 (Pseudomonas fluorescens at 10 mL/L) and five replications. Results indicated that microbial inoculation significantly reduced organic pollution parameters and treatment efficiency increased with incubation time. Among the treatments, M2 recorded the lowest mean values of chemical oxygen demand, biochemical oxygen demand, total solids and total dissolved solids. The interaction M2D3 (30 days incubation) was the most effective, achieved maximum reduction in chemical oxygen demand, biochemical oxygen demand and total solids contents, indicated a strong synergistic effect of microbial inoculation and extended incubation. M2 also showed the highest reduction in optical density across all tested distillery spent wash concentrations, even at 10th day of incubation. In contrast, M1 was more effective in removal of micronutrient, particularly iron, manganese, zinc and copper, with the lowest concentrations observed under the M1D1 interaction. Overall, Pseudomonas fluorescens with extended incubation was optimal for organic pollution reduction, while Aspergillus niger was more effective for micronutrient removal, demonstrated that an integrated microbial approach is an efficient and eco-friendly strategy for distillery spent wash treatment.

Keywords: Aspergillus niger, Distillery spent wash, Incubation period, Microbial degradation, Pseudomonas fluorescence.

- INTRODUCTION

Waste management is a major global challenge, as the composition of wastewater varies widely among industries. In recent years, the reuse of wastewater in agriculture has gained attention due to its potential to supply nutrients, improve soil properties and enhance crop yield. Among agro-based industries, the sugarcane (Saccharum officinarum L.) industry plays a significant role in the socio-economic development of many countries. Along with sugar production, this industry generates by-products such as bagasse, press mud and distillery spent wash (DSW).

India ranks second in ethanol production in Asia and fourth globally, contributing nearly 65% of alcohol production in the South-East Asian region. Most Indian distilleries use sugarcane molasses as the primary raw material for ethanol production. Approximately 61% of the world’s ethanol is produced from sugar crops (Berg, 2004). During fermentation, carried out for about 50 hours using Saccharomyces cerevisiae, ethanol accumulates up to 8–10%, after which distillation and rectification are performed. The residual liquid waste generated after distillation is known as spent wash (Nandy et al., 2002).

A typical molasses-based distillery produces about 15 L of spent wash per litre of ethanol and more than 30 billion litres are generated annually from approximately 212 distilleries in India. Distillery spent wash is dark brown, acidic, viscous and foul-smelling. Dark brown colour of spent wash is mainly because of the presence of brown polymeric melanoidin pigments, which are highly recalcitrant. Melanoidin pigments are formed by the non-enzymatic aminocarbonyl reaction i.e. Maillard reaction. It has been reported that several refractory components found in molasses effluent, such as xenobiotic substances, anthocyanins, caramel, sugar breakdown products, melanoidin and tannins, are resistant to degradation and remain in the environment. Decomposable organics such skatole, indole and sulphur compounds are the source of its disagreeable smell. It is extremely hazardous to the environment and has a foul stench when dumped into rivers or canals (Sarma et al. 2025). Their discharge of the effluent from the distillery industry into aquatic systems reduces light penetration, dissolved oxygen levels and photosynthetic activity, thereby harming aquatic life (Pazouki et al., 2008).

Although several physico-chemical and biological treatment methods have been explored, many require high chemical inputs and generate excessive sludge. In contrast, microbial treatment offers an eco-friendly, cost-effective and sustainable alternative; therefore, the present study was aimed at evaluating the effectiveness of indigenous microbial inoculants in reducing the toxicity of distillery spent wash and improving its suitability for safe agricultural reuse without adverse effects on soil health.

2. MATERIALS AND METHODS

2.1 Collection of spent wash

The distillery spent wash was collected from the Distillery plant of Gandevi Sugar Factory, Ambheta, Gandevi, which is in the Navsari district of Gujarat. It is      situated between 20.78405 N Latitude and 72.98555 E Longitude. Fresh distillery spent wash was collected from the distillery outlet in pre-cleaned, airtight plastic drums and transported to the laboratory for further analysis and experimentation in the month of October-2024.

2.2 Preparation of 25% Spent Wash

After collecting the raw spent wash from the Distillery Plant of Gandevi Sugar Factory, 25% spent wash solution was prepared by diluting the raw spent wash with water. This diluted solution was then filled into 2.5-liter amber-colored glass bottles for further studies.

2.3 Physio-Chemical Characterization of Distillery Spent Wash Sample

The changes in physico-chemical parameters (COD, BOD, OD, TS, TDS and heavy metals like Fe, Mn, Zn and Cu) in distillery spent wash before and after microbial treatment. All analyses were carried out following the standard procedures outlined by Gilcreas, F. W. (1966).

2.4 Methodology for Microbial degradation of distillery spent wash by microorganisms

Fifteen umber colour bottles of 2.5 L capacity were filled with 25% diluted spent wash collected from the Distillery Plant of Gandevi Sugar Factory, Ambheta, Gandevi. A pure culture of microorganisms (Aspergillus niger at 1 g/L and Pseudomonas fluorescens at 10 mL/L) were inoculated to M1 and M2 treatment, respectively while M0 was kept as uninoculated as per treatment. Each treatment was incubated at room temperature for 30 days after proper mixing in 2.5 L umber colour bottle. The bottles were shaken daily to get complete mixing and chemical reaction of each treatment under study. For periodic sample analysis, three sets of each treatment replicated five times were arranged and each one set was collected for periodic sample analysis of COD, BOD, OD, TS, TDS, Fe, Mn, Zn, Cu of distillery spent wash for periodic analysis on 10th, 20th and 30th days.

Table 1: Properties of raw and 25% spent wash

| Sr. No. | Parameters(mg/L) | Method of analysis | Reference | Raw Spent wash | 25% diluted Spent wash |
| --- | --- | --- | --- | --- | --- |
| 1. | COD | Open Reflux method | Clesceri et al. (1990) | 3,29,840 | 77,280 |
| 2. | BOD | Dissolved oxygen (Winkler method) | Clesceri et al. (1990) | 75,790 | 18,356 |
| 3. | Total Solid (TS) | Gravimetric method | Walter, William. G. (1961) | 3,88,200 | 94,800 |
| 4. | Total Dissolved Solid (TDS) |  | Howard (1933) | 3,33,600 | 83,400 |
| 5. | Iron | Atomic Absorption Spectrophoto-metric | Page et al. (1982) | 75.89 | 32.85 |
| 6. | Manganese |  |  | 13.33 | 3.18 |
| 7. | Zinc |  |  | 12.9 | 3.37 |
| 8. | Copper |  |  | 0.94 | 0.19 |

- RESULT AND DISCUSSION

3.1 Effect of Microbial Inoculation and Incubation Period on COD, BOD and OD of Distillery Spent Wash

The results in Table 2 and Fig. 1 showed that both microbial inoculation (M0, M1, M2) and incubation period (10, 20 and 30 days) significantly influenced the COD of distillery spent wash. The control (M0) recorded the highest mean COD (74,960.13 mg/L), while inoculation with Pseudomonas fluorescens (M2) resulted in the lowest mean COD (31,055.47 mg/L), indicating a 58.57% reduction over M0. M1 also lowered the COD of spent wash by 48.11%. This decline can be attributed to the enhanced metabolic capability of the inoculated microbes, which break down organic pollutants more efficiently. COD values also decreased significantly with increasing incubation time, with the lowest value (44,998.07 mg/L) on the 30th day, reflecting 15.14% and 4.03% reductions over the 10th and 20th days, respectively. The interaction effect (M×D) was significant, with M2D3 showing the minimum COD (28,201.60 mg/L), followed closely by M2D2. Reductions under these treatments exceeded 59–63% over their respective controls, highlighting the benefit of combining effective microorganisms with adequate incubation time. Shayegan et al. (2005) reported that Aspergillus sp. exhibited 84 % reduction in COD of distillery spent wash. Chavan et al. (2006) reported that Pseudomonas sp. was capable for reducing COD from 1,46,380 mg/L to 54,160 mg/L (63% reduction) by converting organic pollutants into CO2, H2O and simpler compounds.

Table 2: Effect of microbial inoculation and incubation period on COD (mg/L) of distillery spent wash

| Microbial inoculation | COD (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 76,748.00 | 75,425.80 | 72,706.60 | 74,960.13 |
| M1 | 46,489.80 | 36,120.00 | 34,086.00 | 38,898.60 |
| M2 | 35,844.40 | 29,120.40 | 28,201.60 | 31,055.47 |
| Mean (D) | 53,027.40 | 46,888.73 | 44,998.07 | 48,304.73 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 394.04 |  | 1,134.78 | 3.15 |
| D | 394.04 |  | 1,134.78 |  |
| M×D | 682.5 |  | 1,965.49 |  |

The data in Table 3 and Fig. 1 exhibited that the highest mean BOD value was recorded in the control (M0) at 17,366.67 mg/L, while inoculation with Pseudomonas fluorescens (M2) resulted in the lowest mean BOD (7,883.53 mg/L), reflecting a 54.61% reduction over M0. Aspergillus niger (M1) also reduced BOD by 47.02%. The decline in BOD with M1 and M2 indicates enhanced microbial degradation of organic matter. Incubation time also played a role, with BOD decreasing from 12,354.87 mg/L on the 10th day to 10,758.00 mg/L on the 30th day, showing that prolonged microbial action gradually lowered theBOD content. The interaction effect (M×D) further confirmed that M2 at 30 days (M2D3) achieved the minimum BOD (7,451.80 mg/L), with reductions exceeding 54–59% over corresponding controls. These findings emphasize that both microbial type and incubation duration are key to efficient BOD removal. Choi (2016) highlighted the effectiveness of microbial degradation in lowering BOD levels in industrial wastewaters. Venkatachalam and Gounder (2017) also reported that Oscillatoria boryana reduced BOD from 72,350 mg/L to 634 mg/L.

Table 3.: Effect of microbial inoculation and incubation period on BOD (mg/L) of distillery spent wash

| Microbial inoculation | BOD (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 18,289.20 | 17,420.40 | 16,390.40 | 17,366.67 |
| M1 | 10,538.60 | 8,631.80 | 8,431.80 | 9,200.73 |
| M2 | 8,236.80 | 7,962.00 | 7,451.80 | 7,883.53 |
| Mean (D) | 12,354.87 | 11,338.07 | 10,758.00 | 11,483.64 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 117.46 |  | 338.26 | 3.96 |
| D | 117.46 |  | 338.26 |  |
| M×D | 203.44 |  | 585.89 |  |

Fig. 1 Effect of microbial inoculation and incubation period on reduction of COD and BOD in distillery spent wash

Table 4 presents the decolourization efficiency of distillery spent wash, assessed through reductions in optical density (OD) under different microbial inoculation treatments (M0, M1, M2), incubation periods (10, 20 and 30 days) and spent wash concentrations (1-5%). Initial OD values increased with concentration and served as the baseline. OD decreased across treatments and incubation periods, with the greatest reduction generally observed on the 10th day. Aspergillus niger (M1) improved decolourization over the control, particularly at higher concentrations, while Pseudomonas fluorescens (M2) consistently showed the maximum OD reduction at all concentrations. At 1% concentration on the 10th day, M2 achieved a 44.63% reduction, outperforming M1 (19.37%) and M0 (4.42%). Overall, M2 proved most effective, followed by M1, indicated that microbial inoculation substantially enhances the breakdown of melanoidins responsible for the dark colour of spent wash. These findings align with earlier reports by Wagh and Nemade (2018) reported that optimal decolourization of 83.64% was achieved on the 12th day of incubation. Singh et al. (2020) also found that maximum decolourization (87.43%) of distillery spent wash was observed by the Aspergillus brasiliensis.

Table 4: Effect of microbial inoculation and incubation period on optical density (475 nm) of distillery spent wash

| Treat. | Days | Spent wash 1% |  | Spent wash 2% |  | Spent wash 3% |  | Spent wash 4% |  | Spent wash 5% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean | % reduction | Mean | % reduction | Mean | % reduction | Mean | % reduction | Mean | % reduction |
|  | Initial | 0.1244 | 0.00 | 0.2513 | 0.00 | 0.3876 | 0.00 | 0.5206 | 0.00 | 0.6590 | 0.00 |
| M0 | 10 | 0.1189 | 4.42 | 0.2394 | 4.75 | 0.3785 | 2.36 | 0.5114 | 1.75 | 0.6455 | 2.04 |
|  | 20 | 0.1182 | 5.05 | 0.2364 | 5.94 | 0.3734 | 3.65 | 0.5026 | 3.45 | 0.6388 | 3.07 |
|  | 30 | 0.1164 | 6.49 | 0.2353 | 6.39 | 0.3695 | 4.67 | 0.4976 | 4.41 | 0.6214 | 5.71 |
| M1 | 10 | 0.1003 | 19.37 | 0.1998 | 20.51 | 0.2540 | 34.46 | 0.3751 | 27.95 | 0.4913 | 25.45 |
|  | 20 | 0.1094 | 12.12 | 0.2094 | 16.69 | 0.3133 | 19.16 | 0.4516 | 13.24 | 0.5623 | 14.67 |
|  | 30 | 0.1129 | 9.24 | 0.2294 | 8.73 | 0.3339 | 13.85 | 0.4726 | 9.22 | 0.6105 | 7.36 |
| M2 | 10 | 0.0689 | 44.63 | 0.1429 | 43.16 | 0.2218 | 42.77 | 0.3106 | 40.33 | 0.3573 | 45.78 |
|  | 20 | 0.0888 | 28.62 | 0.1721 | 31.53 | 0.2814 | 27.39 | 0.3616 | 30.53 | 0.4893 | 25.75 |
|  | 30 | 0.1012 | 18.66 | 0.2106 | 16.20 | 0.3106 | 19.88 | 0.4316 | 17.09 | 0.5266 | 20.09 |

Photo No. 1: Reduction in optical density of distillery spent wash (1%)  due to microbial inoculation

Photo No. 2: Reduction in optical density of distillery spent wash (2%)  due to microbial inoculation

Photo No. 3: Reduction in optical density of distillery spent wash (3%)  due to microbial inoculation

Photo No. 4: Reduction in optical density of distillery spent wash (4%)  due to microbial inoculation

Photo No. 5: Reduction in optical density of distillery spent wash (5%)  due to microbial inoculation

3.2 Effect of Microbial Inoculation and Incubation Period on TS and TDS of Distillery Spent Wash

Table 5 and Fig. 2 showed that microbial inoculation and incubation time significantly reduced the total solids (TS) content of distillery spent wash. The control (M0) recorded the highest mean TS (94,025.34 mg/L), whereas Pseudomonas fluorescens (M2) achieved the lowest (20,064.40 mg/L), reflecting a 78.66% reduction over M0. Aspergillus niger (M1) also lowered TS by 67.24%. This decline suggests that inoculated microbes actively degraded and utilized complex organic matter, with M2 showing greater efficiency due to its stronger metabolic capabilities. TS levels also decreased with longer incubation, with the lowest value (44,455.07 mg/L) observed on the 30th day, indicating that extended microbial activity enhances solids breakdown. The interaction effect (M×D) further confirmed that M2 at 30 days (M2D3) produced the minimum TS (14,875.20 mg/L), achieving more than 83–84% reduction over respective controls. These findings align with earlier reports, such as Yadav and Chandra (2012), also noted substantial TS reduction from 46,516 ± 673 mg/L to 16,829 ± 333 mg/L (63.82 % reduction) in biomethanated distillery spent wash due to bacterial consortium at 8 days of incubation period.

Table 5: Effect of microbial inoculation and incubation period on TS (mg/L) of distillery spent wash

| Microbial inoculation | TS (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 95,710.00 | 93,512.00 | 92,854.00 | 94,025.34 |
| M1 | 40,582.00 | 26,196.00 | 25,636.00 | 30,804.67 |
| M2 | 29,834.00 | 15,484.00 | 14,875.20 | 20,064.40 |
| Mean (D) | 55,375.33 | 45,064.00 | 44,455.07 | 48,298.14 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 86.30 |  | 248.54 | 0.69 |
| D | 86.30 |  | 248.54 |  |
| M×D | 149.48 |  | 430.48 |  |

Fig. 2 Effect of microbial inoculation and incubation period on reduction of TS in distillery spent wash

Table 6 and Fig. 3 showed that microbial inoculation significantly reduced the TDS content of distillery spent wash. The control (M0) recorded the highest mean TDS (83,071.34 mg/L), while Pseudomonas fluorescens (M2) achieved the lowest value (12,974.67 mg/L), followed by Aspergillus niger (M1) with 19,206.67 mg/L, corresponding to reductions of 76.88% and 84.38% over M0. This substantial decline indicates that the inoculated microbes effectively transformed or degraded dissolved solids due to their enzymatic activity and better adaptation to the spent wash environment. Incubation period also influenced TDS levels, with the lowest mean value observed on the 10th day (36,319.33 mg/L). A slight rise on the 20th day followed by a small drop on the 30th day suggests complex microbial dynamics, where early degradation may release soluble compounds before further breakdown occurs later. The interaction effect (M×D) showed that M2 at the 10th day produced the lowest TDS (12,480.00 mg/L), achieving over 84-85% reduction compared to respective controls. Shugaba et al. (2010) also found that the TDS of tannery wastewater decreased from 9,171 mg/L to 1,138 mg/L after treatment with A. niger, which showed efficiency of reducing TDS content (87.59%) of distillery spent wash.

Table 6: Effect of microbial inoculation and incubation period on TDS (mg/L) of distillery spent wash

| Microbial inoculation | TDS (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 83,978.00 | 82,752.00 | 82,484.00 | 83,071.34 |
| M1 | 12,500.00 | 22,800.00 | 22,320.00 | 19,206.67 |
| M2 | 12,480.00 | 13,494.00 | 12,950.00 | 12,974.67 |
| Mean (D) | 36,319.33 | 39,682.00 | 39,251.33 | 38,417.56 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 64.53 |  | 185.83 | 0.65 |
| D | 64.53 |  | 185.83 |  |
| M×D | 111.76 |  | 321.86 |  |

Fig. 3 Effect of microbial inoculation and incubation period on reduction of TDS in distillery spent wash

3.3 Effect of Microbial Inoculation and Incubation Period on Fe, Mn, Zn and Cu Content of Distillery Spent Wash

Treatment M0 (without inoculation) showed significantly the highest mean Fe content (33.36 mg/L), while significantly the lowest Fe content was observed in M1 (19.50 mg/L) which was statistically at par with M2. The data further showed that 41.55 and 37.20 per cent Fe content was decreased due to M1 and M2 over M0. In relation with the incubation period significantly lower Fe content (23.43 mg/L) in distillery spent wash was observed at 10th day of incubation period. The data further showed that decrement in Fe content in distillery spent wash due to M1D1 was 53.12, 50.58 and 43.23 per cent over M0D1, M0D2 and M0D3, while in case of M2D1, decrement in Fe content of spent wash was 52.04, 49.44 and 41.93 per cent over M0D1, M0D2 and M0D3, respectively. Yadav and Chandra (2012) reported that Fe content was decreased from 50.64 ± 2.53 to 10.95 ± 0.43 (78.38 % reduction) by bacterial consortium within 8 days of incubation period. This is because of mechanisms such as biosorption or redox conversion of Fe²⁺ to insoluble forms.

Table 7: Effect of microbial inoculation and incubation period on Fe content (mg/L) of distillery spent wash

| Microbial inoculation | Fe (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 36.07 | 34.22 | 29.79 | 33.36 |
| M1 | 16.91 | 19.97 | 21.63 | 19.50 |
| M2 | 17.30 | 21.99 | 23.56 | 20.95 |
| Mean (D) | 23.43 | 25.39 | 25.00 | 24.60 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 1.12 |  | 3.23 | 17.64 |
| D | 1.12 |  | NS |  |
| M×D | 1.94 |  | 5.59 |  |

Significantly the highest mean Mn content (4.52 mg/L) was found in M0 treatment (without inoculation) and significantly lower Mn content (2.74 mg/L) was observed in distillery spent wash treated by Aspergillus niger. The observed reduction in Mn content with microbial inoculation, particularly with M1 and M2 was 39.38 and 22.79 per cent over M0, respectively. Significantly the lowest Mn content (1.65 mg/L) was found in distillery spent wash treated by Aspergillus niger at 10th day of incubation. Decrement in Mn content of distillery spent wash due to M1D1 was 75.15, 55.88 and 48.11 per cent over M0D1, M0D2 and M0D3, respectively. According to Chaturvedi et al. (2006) microbial treatment can reduce Mn through bioaccumulation and enzymatic transformation into less soluble forms because they observed that bacterial inoculated distillery spent wash reduced 98.34% Mn content. Similar findings reported that both bacteria and fungi are capable of oxidizing Mn2⁺ to Mn4⁺, leading to precipitation and removal from solution.

Table 8: Effect of microbial inoculation and incubation period on Mn content (mg/L) of distillery spent wash

| Microbial inoculation | Mn (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 6.64 | 3.74 | 3.18 | 4.52 |
| M1 | 1.65 | 3.47 | 3.10 | 2.74 |
| M2 | 3.71 | 3.58 | 3.16 | 3.49 |
| Mean (D) | 4.00 | 3.60 | 3.15 | 3.58 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 0.17 |  | 0.50 | 18.86 |
| D | 0.17 |  | 0.50 |  |
| M×D | 0.30 |  | 0.87 |  |

M0 treatment exhibited significantly the highest mean Zn content (5.68 mg/L) in distillery spent wash. With microbial inoculation M1 and M2, the mean Zn content significantly decreased to 2.53 mg/L and 2.80 mg/L, respectively. The results further revealed that 55.45 and 50.70 per cent decrement in Zn content of distillery spent wash was observed with M1 and M2 over M0, respectively. Significantly the lowest Zn content in distillery spent wash was observed in M1D1 with corresponding value of 1.78 mg/L. The data also revealed that the decrement in Zn content of distillery spent wash due to M1D1 was 71.87, 67.58 and 65.77 per cent over M0D1, M0D2 and M0D3, while in case of M2D1, decrement in Zn content of spent wash was 50.87, 43.35 and 40.19 per cent over M0D1, M0D2 and M0D3, respectively. Chau et al. (2023) also found similar result that zinc reduced from 8.1±0.35 mg/L to 7.8±0.32 mg/L in tannery effluent treated by Aspergillus niger via biosorption, complexation and cellular uptake. Similarly, it was also found that Zn removal is enhanced through fungal enzymatic activity and chelation.

Table 9: Effect of microbial inoculation and incubation period on Zn content (mg/L) of distillery spent wash

| Microbial inoculation | Zn (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 6.33 | 5.49 | 5.20 | 5.68 |
| M1 | 1.78 | 2.81 | 2.98 | 2.53 |
| M2 | 3.11 | 2.74 | 2.54 | 2.80 |
| Mean (D) | 3.74 | 3.68 | 3.57 | 3.67 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 0.09 |  | 0.27 | 9.86 |
| D | 0.09 |  | NS |  |
| M×D | 0.16 |  | 0.47 |  |

Significantly the highest mean Cu content (1.81 mg/L) in distillery spent wash was found in M0 (control). However, 28.18 and 18.78 per cent decrement in Cu content of distillery spent wash was observed due to M1 and M2 over M0. Significantly lower mean Cu content (1.43 mg/L) was found at 20th day of incubation period (D2) which was statistically at par with the 10th day of incubation period (D1). Significantly the lowest Cu content (1.16 mg/L) was found in distillery spent wash treated by Aspergillusniger at 10th day of incubation period (M1D1) which was statistically at par with M1D2, M2D1 and M2D2. The data also revealed that the decrement in Cu content in distillery spent wash due to M1D1 was 40.81, 29.26 and 36.26 per cent over M0D1, M0D2 and M0D3, while in case of M2D1, decrement in Cu content of spent wash was 38.77, 26.83 and 34.06 per cent over M0D1, M0D2 and M0D3, respectively. Similar result was found by Agnihotri (2011) that Aspergillus spp. was capable of reducing Cu content from biomethanated distillery effluent as copper declined from 0.097 ppm to 0.0198 ppm (79.59% reduction) because Cu is often sequestered in microbial biomass or transformed into less bioavailable forms during effluent treatment.

Table 10: Effect of microbial inoculation and incubation period on Cu content (mg/L) of distillery spent wash

| Microbial inoculation | Cu (mg/L) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Incubation period (Day) |  |  | Mean (M) |
|  | 10th (D1) | 20th (D2) | 30th (D3) |  |
| M0 | 1.96 | 1.64 | 1.82 | 1.81 |
| M1 | 1.16 | 1.29 | 1.45 | 1.30 |
| M2 | 1.20 | 1.36 | 1.83 | 1.47 |
| Mean (D) | 1.44 | 1.43 | 1.70 | 1.52 |
| Source | S.Em. (±) |  | CD at 5% | CV (%) |
| M | 0.06 |  | 0.18 | 15.84 |
| D | 0.06 |  | 0.18 |  |
| M×D | 0.11 |  | 0.31 |  |

Fig. 4 Effect of microbial inoculation and incubation period on reduction of Fe, Mn, Zn and Cu content in distillery spent wash

- CONCLUSION

The present investigation clearly demonstrated that both Aspergillus niger and Pseudomonas fluorescens played an effective roles in the microbial degradation and Heavy metal reduction in distillery spent wash, though their efficiencies varied across parameters at 10th, 20th and 30th day of incubation period. Pseudomonas fluorescens demonstrated superior performance in reducing biochemical oxygen demand (BOD), chemical oxygen demand (COD), total solids (TS), total dissolved solids (TDS) and optical density (OD), indicating its strong capacity for reducing organic load and colour. On the other hand, Aspergillus niger was particularly efficient in reducing heavy metals like iron (Fe), manganese (Mn), zinc (Zn), copper (Cu) at 10th day incubation period and it also contributed significantly in reductions of BOD, COD and decolourization at 10th day of incubation period over no microbial inoculation. Future studies may focus on optimizing microbial consortia, scaling up the treatment process and evaluating the safe use of treated or diluted distillery spent wash for fertigation or as a nutrient-rich fertilizer in agricultural fields.

Disclaimer (Artificial intelligence)

Option 1:

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

- REFERENCES

- Agnihotri, S. N. (2011). Bioremediation of biomethanated distillery effluent before discharge to reduce contamination of aquatic sources. Water and Society, 1, 253-261. https://doi.org/10.2495/ws110231

- Berg, C. (2004). World fuel ethanol – analysis and outlook. In Sugar Trading Manual. https://doi.org/10.1016/B978-1-85573-457-9.50012-3

- Chaturvedi, S., Chandra, R. and Rai, V. (2006). Isolation and characterization of Phragmites australis (L.) rhizosphere bacteria from contaminated site for bioremediation of colored distillery effluent. Ecological Engineering, 27(3), 202-207. https://doi.org/10.1016/j.ecoleng.2006.02.008

- Chau, T. P., Bulgariu, L., Saravanan, M., Rajkumar, R., Chinnathambi, A., Salmen, S. H. and Jhanani, G. K. (2023). Bioremediation efficiency of free and immobilized form of Aspergillus niger and Aspergillus tubigenesis biomass on tannery effluent. Environmental Research. https://doi.org/10.1016/j.envres.2023.116275

- Chavan, M. N., Kulkarni, M. V., Zope, V. P. and Mahulikar, P. P. (2006). Microbial degradation of melanoidins in distillery spent wash by an indigenous isolate. Indian Journal of Biotechnology, 5(3), 416-421. http://nopr.niscpr.res.in/handle/123456789/38

- Choi, H. J. (2016). Parametric study of brewery wastewater effluent treatment using Chlorella vulgaris microalgae. Environmental Engineering Research, 21(4), 401-408. https://doi.org/10.4491/eer.2016.024

- Clesceri, L. S, Greenberg, A. E. and Trussell, R. R. (17th ed.). Standard methods: For the examination of water and waste water (1990). American Public Health Association. Analytical Biochemistry, 186(1), 183. https://doi.org/10.1016/0003-2697(90)90598-4

- Gilcreas, F. W. (1966). Standard methods for the examination of water and waste water. American Journal of Public Health and the Nations Health, 56(3), 387-388. https://doi.org/10.2105/ajph.56.3.387

- Howard, C. S. (1933). Determination of Total Dissolved Solids in Water Analysis. Industrial and Engineering Chemistry Analytical Edition, 5(1), 4-6. https://doi.org/10.1021/ac50081a004

- Nandy, T., Shastry, S. and Kaul, S. N. (2002). Wastewater management in a cane molasses distillery involving bioresource recovery. Journal of Environmental Management, 65(1), 25-38. https://doi.org/10.1006/jema.2001.0505

- Page, A. L., Miller, R. H. and Keeney, D. R. (Eds.). (1982). Methods of soil analysis. Part 2. Chemical and microbiological properties. American Society of Agronomy; Soil Science Society of America. https://doi.org/10.2134/agronmonogr9.2.2ed

- Pazouki, M., Shayegan, J. and Afshari, A. (2008). Screening of microorganisms for decolorization of treated distillery wastewater. Iranian Journal of Science and Technology, Transaction B: Engineering, 32(B1), 53-60. https://www.researchgate.net/publication/267370006_Screening_of_microorganisms_for_decolorization_of_treated_distillery_wastewater

- Sarma, A., Priyadarshi, N., Chakraborty, R. and Kulkarni, P. (2025). A Review on the Role of Microbes in Degradation of Melanoidin from Distillery Wastewater. International Journal of Environment, Agriculture and Biotechnology, 10(2), 076–085. https://doi.org/10.22161/ijeab.102.8

- Shayegan, J., Pazouki, M. and Afshari, A. (2005). Continuous decolorization of anaerobically digested distillery wastewater. Process Biochemistry, 40(3-4), 1323-1329. https://doi.org/10.1007/s11157-005-1246-z

- Shugaba, A., Wuyep, P. A., Nok, A. J., Ameh, D. A. and Lori, J. A. (2010). Bioremediation of hexavalent chromium and tannic acid in synthetic tannery wastewater using free and calcium alginate–immobilized spores and mycelia of Aspergillus niger and Aspergillus parasiticus. Bioremediation Journal, 14(3), 142-149. https://doi.org/10.1080/10889868.2010.495661

- Singh, T. A.; Singh, T. R.; Singh, R. A.; Pandey, P. K.; Gaur, R. A.; Jamal, F. A.; Patel, S. K. and Bansal, S. H. (2020). Bioremediation of melanoidin contamination in distillery effluent using Aspergillus brasiliensis.  Journal of Biotechnology Computational Biology and Bionanotechnology, 101(3): 205-213. https://doi.org/10.5114/bta.2020.97879

- Venkatachalam, D. and Kaliappa Gounder, S. (2016). A composite approach for remediation of cane distillery spent wash using chitosan modified biomass, electrolysis, and Oscillatoria boryana. Journal of Applied Polymer Science, 134(9). Portico. https://doi.org/10.1002/app.44546

- Wagh, M. P. and Nemade, P. D. (2018). Biodegradation of anaerobically treated distillery spent wash by Aspergillus species from a distillery effluent contaminated site. Desalination and Water Treatment. https://doi.org/10.5004/dwt.2018.21838

- Walter, William. G. (1961). Standard methods for the examination of water and wastewater (11th ed.). American Journal of Public Health and the Nations Health, 51(6), 940–940. https://doi.org/10.2105/ajph.51.6.940-a

- Yadav, S. and Chandra, R. (2012). Biodegradation of organic compounds of molasses melanoidin (MM) from biomethanated distillery spent wash (BMDS) during the decolourisation by a potential bacterial consortium. Biodegradation, 23(4), 609-620. https://doi.org/10.1007/s10532-012-9537-x