# Short Research Article

Development of dragon fruit leather (red-fleshed) by using peel as a byproduct waste utilization

.

ABSTRACT

Aims: The study was carried out to standardize formulations and optimize the drying process parameters by tray drying technique for production of red-fleshed dragon fruit leather by using dragon fruit peel as a by-product waste utilization. Place and Duration of Study: Department of processing and food engineering, College of agricultural engineering, Godhra, Gujarat, between January to May 2024. Methodology: Tray dried red-fleshed dragon fruit leather was prepared by varying process variables, i.e., sugar (10, 20, 30, 40, and 50 g/100g), peel (10, 20, 30, 40, and 50 g/100), temperature (50, 55, 60, 65, and 70°C) and thickness (2, 3, 4, 5, and 6 mm) and its responses like vitamin C, acidity, protein content, total soluble solids, color value -- L*, a*, b*, overall acceptability of red-fleshed dragon fruit leather was analyzed by using Response Surface Methodology (RSM). Results: The optimum value of overall acceptability (7.95), vitamin C (39.08 mg/100g), and color value a*(18.26) of red-fleshed dragon fruit leather prepared by tray drying was found at process parameters of sugar (23.29 g/100g), peel (29.41 g/100g), temperature (69.9°C) and thickness (3mm) by using RSM.

Keywords: Red fleshed dragon fruit, Fruit leather, by product waste utilization, Optimization, RSM

1. INTRODUCTION

Dragon fruit (Hylocereus spp.), a vibrant and nutritious member of the cactaceae family, originates from Central America and Mexico (Wong and Siow, 2015). Characterized by bright red skin, green scales, and either red or white flesh with edible black seeds, it is rich in phytochemicals such as carotenoids, flavonoids, polyphenols, and vitamin C, all contributing to its strong antioxidant potential (Fathordoobady et al., 2019; Kiranmai, 2022).

The flesh of dragon fruit is widely used in products like jams, jellies, juices, ice creams, syrups, and beverages. Its peel and flesh are also utilized as natural coloring agents in the food industry (Wybraniec & Mizrahi, 2002). Fruit leather, a dehydrated fruit-based product with a chewy texture and sweet taste, is made by restructuring fruit pulp with other ingredients through drying (Mahesh et al., 2021).

Dragon fruit peel is the major by-product left over from either fresh consumption or fruit processing. It is accounted for about one-fourth of the whole fruit. The dragon fruit peel may contain some valuable components; thus, its properties are needed to be exploited and understood. Vitamin C content of the red-flesh dragon fruit peel was about two and half times compared to the white-flesh dragon fruit (13 mg/100 g compared to 5 mg/100 g). No significant differences between L* (lightness) and a* (redness) values of both dragon fruit peels were observed, whereas b* value (yellowness) of the red-flesh dragon fruit peel was higher than that of the white-flesh dragon fruit (Liaotrakoon et al. (2013)

Although dragon fruit peel is typically discarded or used as fertilizer, it contains valuable compounds like betacyanins and pectin with functional and sensory benefits (Nur Izalin et al., 2016). In light of rising demand for nutritious, ready-to-eat snacks, this study explores the utilization of red-fleshed dragon fruit peel to develop a value-added product—fruit leather. The focus is on optimizing the sugar-to-peel ratio and drying parameters to enhance product quality and consumer appeal.

Drying is important for removing moisture from a product and extending its shelf life. Excessive drying results in degradation of the nutrients from the product as well as less duration of drying results in retaining of the excess moisture and results in spoilage of the product. Drying technique greatly effects the characteristics of leather like flavor, palatability of food, aroma, hardness, viscosity, enzymatic activity and microbial spoilage (Bandaru, H., & Bakshi, M. 2020)

Fruit leather is the product in which all the nutrients present in the fruit are conserved for a long time. It is generally packed with the nutrients such as fiber, minerals, calcium, phosphorous and iron. The high content of nutrients ensures one’s health and combat fatigue and other health problems Bandaru and Bakshi (2020).

2. material and methods

2.1 Preparation of Samples

Red-fleshed dragon fruit was washed, and peeled manually, and the pulp was homogenized. The mixture was heated to reach the desired total soluble solids (TSS), and additives such as citric acid (0.1g) and potassium metabisulfite (0.1g) were incorporated. The resulting thick puree was spread onto plastic or glass sheets and dried in a tray dryer under optimized conditions. The final fruit leather was then packed and stored at room temperature.

Fig 1: Process flowchart for preparation of red fleshed dragon fruit leather

2.2 Experimental Plan

Red-fleshed dragon fruit leather was prepared using tray drying with various process formulations. Four process variables at five levels each were tested using a central composite rotatable design (CCRD). Thirty experiments were conducted, and response surface methodology (RSM) analyzed the effects and interactions of these variables on the physicochemical, sensory, and color properties of the dried product.

Table 1: Experimental plan for the development of red fleshed dragon fruit leather

| Independent Variables |  |  |
| --- | --- | --- |
| Peel (g/100g of pulp) | : | 10, 20, 30, 40, 50 |
| Sugar (g/100g of pulp) | : | 10, 20, 30, 40, 50 |
| Temperature (°C) | : | 55, 60, 65, 70, 75 |
| Thickness (mm) | : | 2, 3, 4, 5, 6 |
| Dependent Variables |  |  |
| Physico-chemical Parameters | : | Ascorbic acid, pH, TSS., Protein, |
| Color Values | : | L*, a*, b* |
| Sensory Evaluation | : | Color, Taste, Flavor and Overall Acceptability |

2.3 Physico-Chemical Analysis of Red Fleshed Dragon Fruit Leather

Proximate composition represents the gross content of important chemical constituents such as vitamin C, pH, TSS, acidity, and protein. The study of the proximate composition serves as an important base to study the nutritive quality of red-fleshed dragon fruit leather.

Table 2: Actual and coded values of different parameters for preparation of red fleshed dragon fruit leather

| Sr. No. | Coded Value |  |  |  | Actual Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peel (g/100g) | Sugar (g/100g) | Temperature(°C) | Thickness(mm) | Peel (g/100g) | Sugar (g/100g) | Temperature(°C) | Thickness(mm) |
| 1 | -1 | -1 | -1 | -1 | 20 | 20 | 60 | 3 |
| 2 | 1 | -1 | -1 | -1 | 40 | 20 | 60 | 3 |
| 3 | -1 | 1 | -1 | -1 | 20 | 40 | 60 | 3 |
| 4 | 1 | 1 | -1 | -1 | 40 | 40 | 60 | 3 |
| 5 | -1 | -1 | 1 | -1 | 20 | 20 | 70 | 3 |
| 6 | 1 | -1 | 1 | -1 | 40 | 20 | 70 | 3 |
| 7 | -1 | 1 | 1 | -1 | 20 | 40 | 70 | 3 |
| 8 | 1 | 1 | 1 | -1 | 40 | 40 | 70 | 3 |
| 9 | -1 | -1 | -1 | 1 | 20 | 20 | 60 | 5 |
| 10 | 1 | -1 | -1 | 1 | 40 | 20 | 60 | 5 |
| 11 | -1 | 1 | -1 | 1 | 20 | 40 | 60 | 5 |
| 12 | 1 | 1 | -1 | 1 | 40 | 40 | 60 | 5 |
| 13 | -1 | -1 | 1 | 1 | 20 | 20 | 70 | 5 |
| 14 | 1 | -1 | 1 | 1 | 40 | 20 | 70 | 5 |
| 15 | -1 | 1 | 1 | 1 | 20 | 40 | 70 | 5 |
| 16 | 1 | 1 | 1 | 1 | 40 | 40 | 70 | 5 |
| 17 | -2 | 0 | 0 | 0 | 10 | 30 | 65 | 4 |
| 18 | 2 | 0 | 0 | 0 | 50 | 30 | 65 | 4 |
| 19 | 0 | -2 | 0 | 0 | 30 | 10 | 65 | 4 |
| 20 | 0 | 2 | 0 | 0 | 30 | 50 | 65 | 4 |
| 21 | 0 | 0 | -2 | 0 | 30 | 30 | 55 | 4 |
| 22 | 0 | 0 | 2 | 0 | 30 | 30 | 75 | 4 |
| 23 | 0 | 0 | 0 | -2 | 30 | 30 | 65 | 2 |
| 24 | 0 | 0 | 0 | 2 | 30 | 30 | 65 | 6 |
| 25 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |
| 26 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |
| 27 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |
| 28 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |
| 29 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |
| 30 | 0 | 0 | 0 | 0 | 30 | 30 | 65 | 4 |

2.3.1. Determination of Vitamin C

The ascorbic acid of red-fleshed dragon fruit leather was determined using the Rangana (1986) method. The required reagents included 3% meta-phosphoric acid, ascorbic acid standard solution, and dye solution. The dye solution was standardized by titrating it with the standard solution. For the procedure, five grams of red-fleshed dragon fruit leather were blended with 3% meta-phosphoric acid solution, and the liquid was filtered. Then, 5 ml of the filtrate was titrated with 2-6 dichlorophenol-indophenol dye, and the ascorbic acid content was calculated using a formula.

Ascorbic acid(mg100g)=0.5mg×V2ml×50ml×100V2ml×5mlaliquote×Wt. of sample (5g)

2.3.2 Determination of pH

The pH of the red-fleshed dragon fruit leather was measured by using a digital pH meter.

2.3.3 Determination of protein

The quantity of true protein of red-fleshed dragon fruit leather was evaluated by Folin-Lowry assay as per Lowry et al. (1951).

Protein (%)=gfμg×optical density×dilution factor×total vol(ml)×10-6 sample aliquotml×weightof sample (g)×100

2.3.4 Determination of total soluble solids (TSS)

Total soluble solids of red-fleshed dragon fruit leather were determined by using a digital refractometer.

2.3.5 Determination of color value (L*, a*, b*)

The color of the red-fleshed dragon fruit leather was measured in terms of L*, a*, and b* values. All the color measurements were done using a digital photoelectric colorimeter.

2.3.6 Organoleptic evaluation of red-fleshed dragon fruit leather

Dragon fruit leather samples were evaluated for sensory attributes-- color, texture, flavor, and overall acceptability-- by a panel of faculty and postgraduate students from CAET, AAU, Godhra. Panelists rated the samples on a 9-point hedonic scale, with 9 representing liked extremely and 1 representing disliked extremely. The final overall score was the average of all ratings, with a score of "like moderately" considered as the threshold for product acceptability.

3. results and discussion

3.1. Effect of Process Variables on The Quality of Red Fleshed Dragon Fruit Leather

The table shows experimental design and responses (vitamin C, protein, overall acceptability, pH, TSS and color value L*, a*, b*,) collected for optimization of different process variables (peel, sugar, temperature and thickness) for production of red fleshed dragon fruit leather by using Response Surface Methodology techniques. To visualize the combined effects of two factors on the response, three-dimensional response graphs surfaces (Fig 2 to 7) were also generated for each of the fitted models as a function of two variables, while keeping third variable at the central value. The quadratic models were fitted to the experimental data and statistical analysis was carried for each response by using Design Expert Software 13 (Stat Ease Inc.,)

3.2 Analysis of Responses

Various responses (vitamin C, sensory evaluation in terms of overall acceptability, pH, TSS, color values in terms of L*, a*, b*, and protein) of red fleshed dragon fruit leather were discussed here

3.2.1. Effect on Vitamin C

Vitamin C of the red-fleshed dragon fruit leather ranged from 25 to 47.92 mg/100g with an average value of 33.22 mg/100g. The coefficient of the model and other statistics is given in Table 3. The Model F-value of 11.13 implies the model is significant. R2 and adjusted R2 values of the model are 0.91 and 0.83, respectively. It may be seen from Fig. 2 that the increase in sugar proportion in red-fleshed dragon fruit leather formulation resulted in no changes in vitamin C, whereas increases in the peel of red-fleshed dragon fruit leather formulation resulted in an increase in vitamin C up to a certain level. This may be due to the fact that vitamin C is a water-soluble nutrient, and its concentration might not be significantly affected by changes in the sugar contents and the peel of the red-fleshed dragon fruit contain a considerable amount of vitamin C. A similar result was obtained by Singh, J., & Tiwari, R. B. (2019) for fruit leather from a blending of guava and papaya.

Table: 3 ANOVA effect on different process variables on vitamin-C of red fleshed dragon fruit leather

| Source | Sum of Squares | Df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model* | 641.58 | 14 | 45.83 | 11.13 | < 0.0001 |
| A-Sugar | 99.67 | 1 | 99.67 | 24.21 | 0.0002 |
| B-Peel | 0.5075 | 1 | 0.5075 | 0.1233 | 0.7304 |
| C-Temperature | 45.13 | 1 | 45.13 | 10.96 | 0.0048 |
| D-Thickness | 7.25 | 1 | 7.25 | 1.76 | 0.2044 |
| AB | 10.87 | 1 | 10.87 | 2.64 | 0.1250 |
| AC | 162.63 | 1 | 162.63 | 39.50 | < 0.0001 |
| AD | 81.77 | 1 | 81.77 | 19.86 | 0.0005 |
| BC | 15.98 | 1 | 15.98 | 3.88 | 0.0676 |
| BD | 28.06 | 1 | 28.06 | 6.82 | 0.0197 |
| CD | 43.33 | 1 | 43.33 | 10.52 | 0.0054 |
| A² | 8.86 | 1 | 8.86 | 2.15 | 0.1630 |
| B² | 66.46 | 1 | 66.46 | 16.14 | 0.0011 |
| C² | 1.28 | 1 | 1.28 | 0.3106 | 0.5855 |
| D² Complete Model | 49.78 | 1 | 49.78 | 12.09 | 0.0034 |
| Residual | 61.76 | 15 | 4.12 |  |  |
| Lack of Fit | 58.12 | 10 | 5.81 | 7.98 | 0.0167 |
| Pure Error | 3.64 | 5 | 0.7280 |  |  |
| Total | 703.34 | 29 | Adeq Precision 13.88 |  |  |
| R2 0.91 |  |  | Adjusted R2 0.83 |  |  |

* significant

3.2.2 Effect on pH

pH of the red-fleshed dragon fruit leather ranged from 3.0 to 4.1 with an average value of 3.73. The maximum pH is at the combination of (100 g pulp, 30 g sugar, 30 g peel, 65°C temperature, and 3 mm thickness) and the minimum pH is at the combination of (100 g pulp, 20 g sugar, 40 g peel, 60°C temperature, and 3 mm thickness). The coefficient of the model and other statistics is given in Table 4. The Model F-value of 9.81 implies the model is significant. R2 and adjusted R2 values of the model are 0.90 and 0.81, respectively. It may be seen from Fig. 3 that the increase in sugar and peel proportion in red-fleshed dragon fruit leather formulation resulted in decreases in pH. Similar results were obtained by Addai, Z. R., et.al. (2016) for fruit leather made from two cultivars of papaya.

Table 4. ANOVA effect on different process variables on pH of red fleshed dragon fruit leather

| Source | Sum of Squares | df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model* | 0.1229 | 14 | 0.0088 | 2.83 | 0.0273 |
| A-Sugar | 0.0001 | 1 | 0.0001 | 0.0336 | 0.8570 |
| B-Peel | 0.0070 | 1 | 0.0070 | 2.26 | 0.1536 |
| C-Temperature | 0.0100 | 1 | 0.0100 | 3.23 | 0.0926 |
| D-Thickness | 0.0003 | 1 | 0.0003 | 0.1089 | 0.7460 |
| AB | 0.0077 | 1 | 0.0077 | 2.47 | 0.1369 |
| AC | 0.0116 | 1 | 0.0116 | 3.73 | 0.0727 |
| AD | 0.0060 | 1 | 0.0060 | 1.94 | 0.1843 |
| BC* | 0.0371 | 1 | 0.0371 | 11.95 | 0.0035 |
| BD* | 0.0390 | 1 | 0.0390 | 12.58 | 0.0029 |
| CD | 0.0001 | 1 | 0.0001 | 0.0181 | 0.8946 |
| A² | 0.0025 | 1 | 0.0025 | 0.7949 | 0.3867 |
| B² | 0.0009 | 1 | 0.0009 | 0.2904 | 0.5979 |
| C² | 0.0005 | 1 | 0.0005 | 0.1614 | 0.6936 |
| D² Complete Model | 0.0000 | 1 | 0.0000 | 0.0047 | 0.9462 |
| Residual | 0.0465 | 15 | 0.0031 |  |  |
| Lack of Fit | 0.0448 | 10 | 0.0045 | 12.92 | 0.0056 |
| Pure Error | 0.0017 | 5 | 0.0003 |  |  |
| Cor Total | 0.1694 | 29 | Adeq Precision 7.02 |  |  |
| R2 0.73 |  |  | Adjusted R2 0.47 |  |  |

* significant

| Fig. 2 Variations of vitamin C with respect to sugar and peel in red fleshed dragon fruit leather | Fig. 3. Variations of pH with respect to sugar and peel in red fleshed dragon fruit leather |
| --- | --- |

3.2.3 Effect on Protein

Protein of the red-fleshed dragon fruit leather ranged from 5.69 to 10.34%, with an average value of 8.39%. The coefficient of the model and other statistics is given in Table 5. The Model F-value of 10.24 implies the model is significant. R2 and adjusted R2 values of the model are 0.90 and 0.82, respectively. It may be seen from Fig. 4 that the increase in sugar proportion in red-fleshed dragon fruit leather formulation resulted in an increase in protein up to a certain level and increases. Jethva et. al. (2023) worked for the dragon fruit leather and reported the similar results and Jethva et. al. (2024) obtained the similar results of protein for the mango leather.

Table 5. ANOVA effect on different process variables on protein of red fleshed dragon fruit leather

| Source | Sum of Squares | Df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model* | 45.01 | 14 | 3.21 | 10.24 | < 0.0001 |
| A-Sugar | 0.0925 | 1 | 0.0925 | 0.2947 | 0.5952 |
| B-Peel* | 11.97 | 1 | 11.97 | 38.14 | < 0.0001 |
| C-Temperature | 0.0330 | 1 | 0.0330 | 0.1051 | 0.7502 |
| D-Thickness | 0.2970 | 1 | 0.2970 | 0.9463 | 0.3461 |
| AB | 1.02 | 1 | 1.02 | 3.23 | 0.0923 |
| AC* | 6.44 | 1 | 6.44 | 20.51 | 0.0004 |
| AD* | 3.00 | 1 | 3.00 | 9.56 | 0.0074 |
| BC | 0.0001 | 1 | 0.0001 | 0.0002 | 0.9895 |
| BD | 0.0371 | 1 | 0.0371 | 0.1181 | 0.7359 |
| CD | 0.9653 | 1 | 0.9653 | 3.08 | 0.0999 |
| A²* | 19.97 | 1 | 19.97 | 63.62 | < 0.0001 |
| B² | 0.8836 | 1 | 0.8836 | 2.81 | 0.1141 |
| C² | 0.0259 | 1 | 0.0259 | 0.0825 | 0.7778 |
| D² Complete Model | 0.0700 | 1 | 0.0700 | 0.2230 | 0.6435 |
| Residual | 4.71 | 15 | 0.3139 |  |  |
| Lack of Fit | 3.98 | 10 | 0.3982 | 2.74 | 0.1385 |
| Pure Error | 0.7260 | 5 | 0.1452 |  |  |
| Cor Total | 49.72 | 29 | Adeq Precision 10.68 |  |  |
| R² 0.90 |  |  | Adjusted R² 0.82 |  |  |

* significant

3.2.4 Effect on TSS

TSS of the red-fleshed dragon fruit leather ranged from 72 to 94⁰ Brix with an average value of 80⁰ Brix. The maximum TSS is at the combination of (100 g pulp, 40 g sugar, 20 g peel, 60°C temperature, and 5 mm thickness) and the minimum TSS is at the combination of (100 g pulp, 40 g sugar, 40 g peel, 70°C temperature, and 5 mm thickness). The coefficient of the model and other statistics is given in Table 6. The Model F-value of 5.05 implies the model is significant. R2 and adjusted R2 values of the model are 0.82 and 0.66, respectively. The adequate precision value of 9.08 indicates that the model can be used to predict the response within the design space more than 0.2.

It may be seen from Fig. 5 that the increase in sugar proportion in red-fleshed dragon fruit leather formulation resulted in an increase in TSS up to a certain level, then a decrease and an increase in peel of red fleshed dragon fruit leather. This will due to increase the sugar proportion; the concentration of dissolved sugars in the mixture rises, leading to an increase in TSS levels, and the peel of the red-fleshed dragon fruit contains various soluble solids such as sugars, organic acids, and other compounds. Gandhi et. al. (2023) obtained the same results for the mango leather and Jethva et. al. (2024) obtained the similar results of TSS for the mango leather.

Table 6. ANOVA effect on different process variables on TSS of red fleshed dragon fruit leather

| Source | Sum of Squares | Df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model | 708.38 | 14 | 50.60 | 5.05 | 0.0018 |
| A-Sugar | 7.04 | 1 | 7.04 | 0.7022 | 0.4152 |
| B-Peel | 35.04 | 1 | 35.04 | 3.49 | 0.0812 |
| C-Temperature | 5.04 | 1 | 5.04 | 0.5028 | 0.4892 |
| D-Thickness | 45.38 | 1 | 45.38 | 4.52 | 0.0504 |
| AB | 27.56 | 1 | 27.56 | 2.75 | 0.1181 |
| AC | 150.06 | 1 | 150.06 | 14.96 | 0.0015 |
| AD | 22.56 | 1 | 22.56 | 2.25 | 0.1544 |
| BC | 27.56 | 1 | 27.56 | 2.75 | 0.1181 |
| BD | 18.06 | 1 | 18.06 | 1.80 | 0.1995 |
| CD | 126.56 | 1 | 126.56 | 12.62 | 0.0029 |
| A² | 187.50 | 1 | 187.50 | 18.70 | 0.0006 |
| B² | 60.86 | 1 | 60.86 | 6.07 | 0.0263 |
| C² | 15.00 | 1 | 15.00 | 1.50 | 0.2401 |
| D² Complete Model | 51.07 | 1 | 51.07 | 5.09 | 0.0394 |
| Residual | 150.42 | 15 | 10.03 |  |  |
| Lack of Fit | 143.58 | 10 | 14.36 | 10.51 | 0.0090 |
| Pure Error | 6.83 | 5 | 1.37 |  |  |
| Cor Total | 858.80 | 29 | Adeq Precision 9.08 |  |  |
| R² 0.82 |  |  | Adjusted R² 0.66 |  |  |

*significant

| Fig. 4 Variation of protein content with respect to sugar and peel in red fleshed dragon fruit leather | Fig.5 Variations of TSS with respect to sugar and peel in red fleshed dragon fruit leather |
| --- | --- |

3.2.5 Effect on Color Value (a*)

The color value a* of the red-fleshed dragon fruit leather ranged from 5.2 to 28.01, with an average value of 16.63. The coefficient of the model and other statistics is given in Table 7. The Model F-value of 21.80 implies the model is significant. R2 and adjusted R2 values of the model are 0.95 and 0.90, respectively. It may be seen from Fig. 6 that the increase in sugar content initially increases in color value a* up to a certain level, then decrease and peel content in red-fleshed dragon fruit leather. This may be due to increases in browning reaction with sugar increases, and the peel of the red-fleshed dragon fruit contains various pigments, including betalains and anthocyanins, which contribute to the color of the fruit. Jethva et. al. (2024) obtained the similar results of color value (a*) for the mango leather.

Table 7. ANOVA effect on different process variables on color value a* of red fleshed dragon fruit leather

| Source | Sum of Squares | df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model* | 1361.12 | 14 | 97.22 | 21.80 | < 0.0001 |
| A-Sugar | 5.84 | 1 | 5.84 | 1.31 | 0.2704 |
| B-Peel | 57.17 | 1 | 57.17 | 12.82 | 0.0027 |
| C-Temperature | 51.63 | 1 | 51.63 | 11.57 | 0.0039 |
| D-Thickness | 26.38 | 1 | 26.38 | 5.91 | 0.0280 |
| AB* | 42.19 | 1 | 42.19 | 9.46 | 0.0077 |
| AC* | 82.08 | 1 | 82.08 | 18.40 | 0.0006 |
| AD | 3.98 | 1 | 3.98 | 0.8923 | 0.3598 |
| BC* | 109.83 | 1 | 109.83 | 24.62 | 0.0002 |
| BD | 0.9900 | 1 | 0.9900 | 0.2220 | 0.6443 |
| CD | 11.22 | 1 | 11.22 | 2.52 | 0.1335 |
| A²* | 477.24 | 1 | 477.24 | 107.00 | < 0.0001 |
| B²* | 599.15 | 1 | 599.15 | 134.33 | < 0.0001 |
| C²* | 76.95 | 1 | 76.95 | 17.25 | 0.0008 |
| D² Complete Model | 4.04 | 1 | 4.04 | 0.9056 | 0.3564 |
| Residual | 66.91 | 15 | 4.46 |  |  |
| Lack of Fit | 62.31 | 10 | 6.23 | 6.79 | 0.0237 |
| Pure Error | 4.59 | 5 | 0.9182 |  |  |
| Cor Total | 1428.03 | 29 | Adeq Precision 15.75 |  |  |
| R² 0.95 |  |  | Adjusted R² 0.90 |  |  |

*significant

3.2.6 Effect on Overall acceptability

OAA of the red-fleshed dragon fruit leather ranged from 6.8 to 8.3 with an average value of 7.95. The coefficient of the model and other statistics is given in Table 8. The Model F-value of 37.33 implies the model is significant R2 and adjusted R2 values of the model are 0.97 and 0.95, respectively. It may be seen from Fig. 7 that the increase in sugar content may increase in OAA and peel content in dragon fruit leather may decrease in OAA of the red-fleshed dragon fruit leather. This may be due to sugar, which may improve taste and sweetness perception and the peel of the red-fleshed dragon fruit contributes to its flavor profile, including any bitterness. A similar result was found in Gandhi et.al. (2023) for the mango leather.

Table 8. ANOVA effect on different process variables on OAA of red fleshed dragon fruit leather

| Source | Sum of Squares | Df | Mean Square | F-value | p-value |
| --- | --- | --- | --- | --- | --- |
| Model* | 3.28 | 14 | 0.2343 | 37.33 | < 0.0001 |
| A-Sugar | 0.4004 | 1 | 0.4004 | 63.78 | < 0.0001 |
| B-Peel | 0.0204 | 1 | 0.0204 | 3.25 | 0.0914 |
| C-Temperature | 0.0504 | 1 | 0.0504 | 8.03 | 0.0126 |
| D-Thickness | 0.4004 | 1 | 0.4004 | 63.78 | < 0.0001 |
| AB* | 0.0756 | 1 | 0.0756 | 12.05 | 0.0034 |
| AC* | 0.0306 | 1 | 0.0306 | 4.88 | 0.0432 |
| AD* | 0.3906 | 1 | 0.3906 | 62.22 | < 0.0001 |
| BC* | 0.1406 | 1 | 0.1406 | 22.40 | 0.0003 |
| BD | 0.1406 | 1 | 0.1406 | 22.40 | 0.0003 |
| CD* | 0.6806 | 1 | 0.6806 | 108.42 | < 0.0001 |
| A²* | 0.2357 | 1 | 0.2357 | 37.55 | < 0.0001 |
| B²* | 0.4800 | 1 | 0.4800 | 76.46 | < 0.0001 |
| C²* | 0.1336 | 1 | 0.1336 | 21.28 | 0.0003 |
| D² Complete Model | 0.0107 | 1 | 0.0107 | 1.71 | 0.2105 |
| Residual | 0.0942 | 15 | 0.0063 |  |  |
| Lack of Fit | 0.0858 | 10 | 0.0086 | 5.15 | 0.0422 |
| Pure Error | 0.0083 | 5 | 0.0017 |  |  |
| Cor Total | 3.37 | 29 | Adeq Precision 26.33 |  |  |
| R² 0.97 |  |  | Adjusted R² 0.95 |  |  |

| Fig.6 Variation of color value a* with respect to sugar and peel in red fleshed dragon fruit | Fig. 7 Variations of OAA with respect to sugar and peel in red fleshed dragon fruit |
| --- | --- |

3.3 Optimization for Prediction of Process Parameters for Developed Red-Fleshed Dragon Fruit Leather

Optimization of process parameters was done using Design Expert 13 software. Numerical optimization was carried out by putting the values of process parameters within the experimental range and by setting desirable goals for the responses. Optimization of process parameters for preparation of red-fleshed dragon fruit leather by tray-drying techniques was performed on the basis of its dominant quality attributes, such as overall acceptability, vitamin C and color value a*.

The optimum range of process parameters for tray-dried red-fleshed dragon fruit leather was found as follows: sugar-23.30 g/100 g, peel-29.41 g/100 g, temperature-69.90°C, and thickness-3 mm. Corresponding to optimum values of process parameters, predicted values of responses were found as organoleptic score in terms of overall acceptability (7.95), Vitamin C (39.08 mg/100 g) and color value a* (18.26).

4. CONCLUSION

It may be concluded from the study that the tray-dried dragon fruit leather (red-fleshed) by using dragon fruit peel as a by-product utilization having a formulation of sugar: Peel: temperature: thickness was found as 23.30g/100 g, 29.41g/100g: 69.9°C: 3 mm, respectively. Which is extremely useful for satisfying consumer tastes and preferences and optimizing samples having a vitamin C content of 39.08 mg/100 g, overall acceptability of 7.95, and a color value a* of 18.26, respectively.

## Disclaimer (Artificial intelligence)

## Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Addai, Z. R., Abdullah, A., Mutalib, S. A., & Musa, K. H. (2016). EVALUATION OF FRUIT LEATHER MADE FROM TWO CULTIVARS OF PAPAYA. Italian Journal of Food Science/Rivista Italiana di Scienza degli Alimenti, 28(1).

- Bandaru, H., & Bakshi, M. (2020). Fruit Leather: Preparation, packaging and its effect on sensorial and physicochemical properties: A review. Journal of Pharmacognosy and Phytochemistry, 9(6), 1699-1709.

- F., Fathordoobady, M. Y., Manap, J., Selamat, A. P., Singh (2019). Development of supercritical fluid extraction for the recovery of betacyanins from red pitaya fruit (Hylocereus polyrhizus) peel: a source of natural red pigment with potential antioxidant properties. International Food Research Journal, 26(3), 1023.

- Gandhi, R., Jethva, K. R., & Damor, H. (2023). Effect of drying temperature on Physio-chemical properties of protein enriched mango leather, The Pharma Innovation Journal, 12(11): 301-307

- Jethva, K. R., Rathod, S. R., and Pargi, A. S. (2023). Optimization of process variables for development of dragon fruit leather by using fruit peel as by-product utilization. The Pharma Innovation Journal, 12(7): 1154-1159.

- Jethva K. R., Suthar R., and Kumar N (2024). Effect of drying on Physico-chemical properties of protein fortified kesar mango leather. International Journal of Advanced Biochemistry Research, 8(2) 101-107

- Kiranmai, M. (2022). Review of exotic fruit: Nutritional composition, nutraceutical properties and food application of Dragon fruit (Hylocereus spp.). The Pharma Innovation Journal, 11(6), 613-622.

- Liaotrakoon, W., De Clercq, N., Van Hoed, V., & Dewettinck, K. (2013). Dragon fruit (Hylocereus spp.) seed oils: their characterization and stability under storage conditions. Journal of the American Oil Chemists' Society, 90(2), 207-215.

- Lowry, O. H., Rosebrough, N. J., Farr, A. L. and Randall, R. J. (1951). Protein measurement with the Folin phenol reagent. Journal of Biochemistry, 193(1), 265-275.

- Mahesh, A., Champawat, P., Mudgal, V. and Jain, S. (2021). Development of dragon fruit leather. International Journal of Chemical Studies, 9(2), 71–75. https://doi.org/10.22271/chemi.2021.v9.i2b.11969

- Nur Izalin, M. Z., Kharidah, M., Jamilah, B. and Noranizan, M. A. (2016). Functional properties of pectin from dragon fruit (Hylocereus polyrhizus) peel and its sensory attributes. Journal of Tropical Agriculture and Food Science, 44(1), 95-101.

- Ranganna, S. (1986). Analysis and quality control for fruit and vegetable products. New Delhi: Products Tata Mc Graw-Hill Pulishing.

- Singh, J., & Tiwari, R. B. (2019). Development of nutritious fruit leather by blending guava and papaya. International Journal of Current Microbiology Applied Sciences, 8(7), 813-820.

- Wong, Y. M. and Siow, L. F. (2015). Effects of heat, pH, antioxidant, agitation and light on betacyanin stability using red-fleshed dragon fruit (Hylocereus polyrhizus) juice and concentrate as models. Journal of Food Science and Technology, 52(5), 3086–3092. https://doi.org/10.1007/s13197-014-1362-2

- Wybraniec, S., & Mizrahi, Y. (2002). Fruit flesh betacyanin pigments in Hylocereus cacti. Journal of Agricultural and Food Chemistry, 50(21), 6086–6089. https://doi.org/10.1021/jf020145k