Improved Fermenter of Cocoa Beans for Good Fermentation of Small-Scale Farmers

.

ABSTRACT

The primary raw material for the chocolate industry is cacao (Theobroma cacao). Once the pods are harvested and shelled, the beans are subjected to fermentation and then drying as quickly as possible. These two processes have a direct impact on the quality of marketable cacao. The experiments took place at the Félix Houphouët Boigny University of Cocoa site, Abidjan, from September to October 2024. Samples harvested in the Sikensi area , less than 100 km from Department of Technology of the university. This study focuses on improving the fermentation process, an essential phase for developing cacao aromas. In this study, a 60 cm barrel filled to 70% capacity was used for fermentation. The temperatures in this empty fermentation barrel, as well as those in the presence of the beans, were measured over 1 to 5 days. Approximately 30 kg were fermented. A representative sample was taken each day of fermentation and then air-dried to monitor bean browning. A cutting test was also performed on the fermented beans after drying. The results indicate a temperature difference of approximately 1°C between the inside and outside of the empty fermenter, with temperatures remaining nearly identical at different heights within the fermenter. During fermentation, a small average temperature difference of 1.8°C was observed between 13 cm and 39 cm inside the fermentation heap. Browning began on the second day and reached its optimum on the fourth day. The cutting test confirmed the good market quality, with a score of 72% brown beans. These values ​​indicate that our fermenter is a reliable tool for fermenting small quantities of cocoa.

Keywords: Cocoa, improved fermenter, isolation, turning

1. INTRODUCTION

Cocoa production represents a significant portion of the economy for many tropical countries, notably Côte d'Ivoire (Sangare et al., 2009). In order to reach full aromatic potential and be ready for chocolate making, freshcocoa beans must undergo a fermentation process followed by proper drying. The final quality of the cocoa depends largely on these two steps, which directly influence the sensory characteristics of the beans. Among these post-harvest treatments (Wahyuni et al., 2024), fermentation seems to have the greatest impact on the quality of cocoa beans.Indeed, the fermentation of cocoa is a process carried out by a diverse microbial flora composed of yeast, lactic acid bacteria, acetic acid bacteria, and Bacillus (Pereira, 2012; Tejeda et al., 2024). The action of these microorganisms during cocoa fermentation promotes the formation of precursors chocolate aroma, which will be developed during roasting.

However, in many producing countries, fermentation is still carried out using traditional methods (de Melo Pereira et al., 2013), often without adequate control of conditions, resulting in variable quality. To improve the consistency of fermented cocoa bean quality, various research projects have been undertaken to modernize and control this process.

For controlling cocoa fermentation, some researchers propose using microbial starter cultures. This has allowed them to control the cocoa fermentation process in some American countries notably Brazil, equator etc with yeast starter patents (de Melo Pereira et al., 2012; D. H. Ouattara et al., 2014; Rahardjo et al., 2022; Kamarisima et al., 2025). However, the cost of acquiring these starters could be a barrier to the widespread use of this technology, especially for African farmers who complain about the low selling price of their cocoa production. Also, these microbial starters must be paid for by the farmer each time he wants to carry out a fermentation.

The second approach to controlling cocoa fermentation is the development of a fermenter. For this, several fermenters have been designed and tested to carry out the fermentation of cocoa. The tested fermenter prototypes have led to an improvement in the quality of cocoa beans.

This approach offers 3 possibilities. Firstly, involves the design and construction of prototype solid wood fermenter’s to better control fermentation conditions and improve cocoa bean quality while facilitating the work of producers.

However, despite the promising results obtained, several significant limitations have been identified in these prototypes, hindering their adoption by small-scale producers. For exemple high cost of materials and the manufacturing of the fermenters constitute a significant obstacle, as they were made of stainless steel (Castillo et al., 2021; Veira et al., 2019). Furthermore, the fermentation temperature was lower compared to the wooden fermenter for the same quantity of beans processed (de Melo Pereira et al., 2013). Secondly, fermenters constructed from solid wood were susceptible to termite infestations. These insects quickly compromised the structural integrity of the equipment, reducing its durability and efficiency. Thirdly, they are used for harvests exceeding 100 kg (Bankoff et al., 2014) or employ an electric temperature control system (Dircks, 2009), which forces producers to harvest several times before fermentation begins, with the risk of pod rot or bean germination. Therefore, mixing these large quantities involves transferring the beans into another container to facilitate turning This operation induced risk of leaving the beans outside the fermenter for an extended period (Peláez et al., 2016), resulting in heat loss and potentially slowing down the fermentation. Additionally, a technique called "batch micro-fermentation" involves adding nets containing between 10 kg and 20 kg of fresh beans in to a larger fermentation heaps, thus providing these smaller quantities with a more impactful fermentation ecosystem. The major constraint is that all the beans must begin the fermentation process simultaneously (End &Dand, 2015; Fourny&Reynes, 2012). For very small quantities, on the order of 3 to 5 kg, some studies recommend using large quantities of cocoa pod huk in the heap during fermentation (Bobiles & Millena, 2025). Furthermore, producers are not easily adopting new fermentation techniques (Laksono et al., 2025).

The aim of this study is to make available to small farmers, who account for 60% of cocoa producers (Assiri et al., 2009)., a fermenter made from local materials, durable and capable of better fermenting small quantities of cocoa (30 kg) without added starter culture.

2. material and methods

2.1 Materials

2.1.1 Cocoa

The cocoa pods used during this process were harvested in the SIKENSI area (Côte d’Ivoire) and sent to the Félix Houphouët University to be fermented. The beans used were of the Forestero type.

2.1.2 Fermenter using during cocoa fermentation

A 60cm high plastic drum, capable of holding 41 kg of fresh cocoa beans, is used as a fermenter. This material was chosen for its low mass, ease of cleaning, and resistance to termite attack. Furthermore, it is lined with 2cm of coconut fiber for thermal insulation, as its thermal conductivity is between 0.037 W/ mK and 0.045 W/ mK (Fig.1). The use of the plastic sheeting facilitates the installation of this fiber around the fermenter.

Fig.1.Fermenter in horizontal position

Fig.2.Fermenter after isolation

2.1.3 Measuring instrument

The fermenter is equipped probes PT100 (Fig.3) with a Wimesure electronic system (Fig.4) who records the temperatures at 13cm, 26cm and 38cm from the bottom of the fermenter. This device allows for real-time monitoring of the thermal evolution inside the fermenter. In addition, it is connected to a laptop with the cable shown in Fig.3, facilitating setup and data download. The probes are of the temperature PT100 with a range between –​​​200 And​ + 850°C.

| PT100 Probe | Backup Module | Data recorder |
| --- | --- | --- |
| Fig.3: Equipment connected to the fermenter to monitor the temperature changes during cocoa fermentation |  |  |

2.2  Method

2.2.1 Cocoa fermentation with fermenter

Approximately 300 pods were harvested on the same day, and 30 kg of bean were transported to the laboratory, about 100 km away. The beans are cleaned of pod remains and other debris. Banana leaves are used to line the bottom of the fermenter, and then the beans are placed inside. The fermenter filling to 75% capacity allows the beans to move more freely during turning phases. According turning timing, the fermenter will be rotated several times clockwise, then counterclockwise for 5 minutes, to complete turning process without disturbing the beans. Each day, a sample is taken immediately after stirring and sun-dried to stop fermentation. This sample is used to analyze the browning of the almonds. On the morning of the fifth day, all the beans were removed and sun-dried. Turning of the beans during fermentation was carried out using two timings according to current techniques (Arulmari& Visvanathan, 2024; Guehi et al., 2010; Vincent, 1970). The first series (series 1) of brewing was timed at 24h, 48h, 72h and 96h, while series 2 was timed at 48h, 72h and 96h.

2.2.2 Measurement of the temperature of the cocoa mass during the fermentation

A set of three PT100 probes were placed at 13 cm intervals within the fermenter pile (Fig. 4). A fourth probe was positioned in the space between the fermenter heap and fermenter top. ALMEMO Control V5.19 software was used during the experiments for temperature measurement and formatting. Regarding the expected temperature values, it is necessary to remain above 35°C for as long as possible and as quickly as possible, and in the best-case scenario reach 45°C (Barel 2013).

Fig.4.Probe locations

2.2.3 Beans Browning modification

Browning analyzing  allows to assess the evolution of changes within the cocoa bean(De Vuyst & Weckx, 2016; León-Roque et al., 2016; Santos et al., 2019; Thompson et al., 2001). For this measurement, a portion of the sample taken at the beginning of each day during fermentation (0.1 kg of beans) is left to air dry every day of fermentation. Once all the samples are completely dry, the shell are removed and the beans are ground into a fine powder (Nielsen, 2006) . The different powders are sorted according order in which they exit the fermenter. Photographs of the powder are taken, and using open-source software (ImageJ 1.45s) to evaluate the grayscale level throughout entire process.

2.2.4 Cut test

At the end of fermentation process, beans are carefully dried. Market quality is determinate with cutting test. This test involves randomly selecting 100 dry beans, cutting them in half lengthwise to examine their degree of fermentation, as illustrated in Fig.5 below. Beans are sorted according color: purple for insufficiently fermented beans, brown for well-fermented beans, and black or gray for damaged beans (mold or insect).(Guzmán-Armenteros et al., 2023; Tchouatcheu et al., 2019).

Fig.5.Cutting test on a sample of 100 beans

3. results and discussion

After development of fermenter, this document presents its impact on the market quality of cocoa beans .Wewill present the results of the temperature evolution during fermentation according the two most commonly used turning methods. After, we present the evolution of bean’s browning, followed by a cutting test. Finally, we discuss the results.

3.1 Temperature evolution in empty fermenter

Three temperature probes placed inside empty fermenter at different level (T0: 13cm, T1: 26cm, T2: 39cm) from bottom of fermenter, measurements are shown in Fig.6. Regardless of the level of the thermometer probe, the temperature of the empty fermenter is between 27 and 27.4. These values suggests absence of temperature gradient and therefore absence of temperature disturbances throughout the entire fermenter when it is empty, values ​​are very close.

Fig.6.Temperature in the fermenter during the test without cocoa beans

3.2 Temperature of cocoa beans during fermentation by homogenizing the fermentation mass every 24 hours

Fig.7 below shows temperature evolution within the fermenting heap at different level (T1 :13cm,T2: 26cm and T3: 38cm) from bottom of fermenter  during 96-hour fermentation period. Unlike the temperature curves of the empty fermenter, which show temperatures around 27 °C, those of the fermenter containing cocoa beans shows an increasing trend in temperature from the beginning of fermentation ( 28.5°C + 0.52°C ) to the end of the fermentative process( 43°C +0.71°C). Moreover, the temperature curves seem to be specific to our work from the beginning of fermentation until 25hours and from 70 hours until the end of the process. However, between 28 hours and 68 hours, the temperature values seem different, with lower values when the temperature probe is at 13 cm (T1).These temperature values obtained have been reported by several researchers. Furthermore, the average temperature from the beginning to the end of fermentation is 36.1°C. Indeed the average temperature is 28°C, a value lower than the 33°C obtained in several other studies (Hidayat et al., 2019; Veira et al., 2019).  On day 2, our system averaged 35°C and remained above Veira's (2019) and Wahyuni’s (2024), results until the end of the process. Furthermore, the 40°C threshold was reached up to day 3, a finding also observed in other studies (Hernández-Hernández et al., 2016; H. G. Ouattara et al., 2008)  involved quantities ranging from 100 to 1000 kg. Some studies dealing with small quantities of beans, up to 20 kg, indicate temperatures above 40°C by the middle of the second day when the fermenter is insulated (Davrieux et al., 2017).

This result suggests that our system optimizes heat retention during the fermentation of small quantities of beans, through to the use of coconut fibers as insulation. Furthermore, the fact that the beans are not removed from the fermenter during stirring also limits heat loss from the fermenting pile.

Fig.7.Temperature evolution in the fermenting pile at different heights as a function of time in hours (with turning at 24h, 48h,72h, 96 h)

3.3 Temperature of cocoa beans during fermentation by homogenizing the fermentation mass every 24 hours after second day (48 hours) of fermentation

The graph below (Fig.8) shows the temperature evolution at different heights in the fermenter heap (T0, T1, T2) during 96 hours of fermentation. During the first 48 hours, these internal temperatures (T0, T1, T2) follow a similar trend to those in Fig. 7. However, at turning times (48h, 72h, 94h), a brief temperature variation is observed ±1.5°C , probably due to the movement of the beans during turning and microflora activities. . As in the first series of experiments (fermentation with turning beans every 24 hours), the temperature difference in the fermenter heap between the two commonly used stirring times remains small (approximately 1.8°C) with an average value of 36.6°C. This indicates that our system is suitable for the two most frequently used turning methods.

Fig.8.Evolution of temperature in fermenter at different heights as a function of time in hours (with turning for 48 hours,72 hours,94 h)

3.4 Bean browning evolution during  cocoa fermentation

Fig.9​ present​​​​ evolution​​​​​ of browning ​​​​​​of the order ​​​​​​the course of fermentation in the​​fermenters during cocoa fermentation.​​​ The color change observed after 48 hours is similar to the results reported by Nielsen, (2006),which attributes it to the hydrolysis of anthocyanins (insoluble pigments, such as tannins) into anthocyanidins. This hydrolysis process is favored during fermentation by the rise in temperature and the increase in acidity in the kernel. Indeed, after 48 hours, the temperature rise to 42°C as well as the increased acidity in the beans trigger, according Biehl et al., (1989) and Camu (Camu et al., 2008),  to the fusion of lipids. A continuous darkening of the almond's color is thus observed until the process is interrupted.

Furthermore, the analysis of the gray level (Fig.10) (high values ​​indicate a light shade, low values ​​indicate darkening) of the almonds indicates an evolution of the fermentation. This decrease is also observed by Ndukwu and several other authors(León-Roque et al., 2016; Ndukwu & Udofia, 2016; Oliveira et al., 2021).

Day 4 Day 3 Day 2 Day 1 Day 0

Fig.9.Evolution of the cocoa bean browning

Fig.10.Evolution of the grey level during fermentation day

The value close to 70 as a grey level is similar to that obtained for quantities greater than 100kg that underwent 6 days of fermentation (Koffi et al., 2018).

  - Cut test

The results of this test show that 70% of the beans are brown after 96 hours of fermentation, indicating successful and complete fermentation. This brown hue reflects good development of aroma precursors, demonstrating the effectiveness of our system. Conversely, 28% of the beans are distinguished by a purple color, indicating insufficient fermentation. This rate of under fermented beans could suggest an irregularity in the brewing technique. In the sample, we obtained a percentage of moldy beans, slate-colored beans, and defective beans of less than 3%. These results are close to those obtained after 7 days of fermentation (Bankoff et al., 2014; Kongor et al., 2013). These results therefore indicate that our fermenter makes it possible to achieve grade 1 quality from 4 days of fermentation and this for small quantities.

4. Conclusion

An average temperature variation of 1.8°C was observed along the height of the fermentation heap, with temperatures exceeding 40°C from the second day onward. This resulted in normal development, with pronounced browning of the beans beginning on the second day.

Using both brewing methods commonly employed by producers in our fermenter resulted in an average temperature of 36°C, regardless of the brewing method. Since the beans underwent the same thermal shock, the browing was also similar. The resulting beans had a brown proportion of approximately 70%, conforming to export grade 1. Our system is therefore suitable for processing small quantities of cocoa beans.

## Disclaimer (Artificial intelligence)

## Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Arulmari, R, and R Visvanathan ( 2024) Effect of Fermentation Methods and Turning Interval on the Quality of Cocoa Beans (Theobroma Cacao). Agricultural Research 13(3): 586–598.

Bankoff, L, GH Ouattara, TG Karou, et al. (2014) Impacts de La Fermentation Du Cacao Sur La Croissance de La Flore Microbienne et La Qualite Des Feves Marchandes. Agronomie Africaine 25(2): 159–170.

Barel, Michel  2013	Qualité du cacao L’impact du traitement post-récolte. savoir faire. quae.

Biehl, Böle, Bernd Meyer, Gundel Crone, Lutz Pollmann, and Mamot Bin Said (1989)	Chemical and Physical Changes in the Pulp during Ripening and Post-Harvest Storage of Cocoa Pods. Journal of the Science of Food and Agriculture 48(2): 189–208.

Bobiles, SC, and CG Millena 2025	Enhancing Cacao Fermentation for Small-Scale Farmers: Quality Comparison with Traditional Process. Food Research 9(6): 95–105.

Camu, Nicholas, Tom De Winter, Solomon K Addo, et al. (2008) Fermentation of Cocoa Beans: Influence of Microbial Activities and Polyphenol Concentrations on the Flavour of Chocolate. Journal of the Science of Food and Agriculture 88(13): 2288–2297.

Davrieux, Fabrice, Jean-Jacques Rakotomalala, Sophie Assemat, et al.( 2017) Adaptation Du Processus de Fermentation Aux Contraintes Locales: Application Au Cacao Du Sambirano de Madagascar. In . ICCO.

De Vuyst, Luc, and Stefan Weckx( 2016)	The Cocoa Bean Fermentation Process: From Ecosystem Analysis to Starter Culture Development. Journal of Applied Microbiology.

Dircks, HD (2009)	Investigation into the Fermentation of Australian Cocoa Beans and Its Effect on Microbiology, Chemistry and Flavour. The University of New South Wales, Australia.

End, MJ, and R Dand( 2015)	Fèves de Cacao: Exigences de Qualité de L'industrie Du Chocolat et Du Cacao. ECA-Caobisco-FCC Cocoa Research Fund.

Fourny, Gérard, and Max Reynes (2012)	Fermentation À Petite Échelle Pour Une Quantité Limite de Matériel Génétique de Cabosses. In . Alliance des Pays Producteurs de Cacao.

Guehi, S Tagro, Soumaila Dabonne, L Ban-Koffi, D Kra Kedjebo, and G Irie B Zahouli (2010)	Effect of Turning Beans and Fermentation Method on the Acidity and Physical Quality of Raw Cocoa Beans. Advance Journal of Food Science and Technology 2(3): 163–171.

Guzmán-Armenteros, Tania María, Luis Alejandro Ramos-Guerrero, Luis Santiago Guerra, and Jenny Ruales (2023)	Optimization of Cacao Beans Fermentation by Native Species and Electromagnetic Fields. Heliyon 9(4).

Hernández-Hernández, C, Procopio Alejandro López-Andrade, Miguel A Ramírez-Guillermo, Diana Guerra Ramirez, and Juan F Caballero Perez( 2016) Evaluation of Different Fermentation Processes for Use by Small Cocoa Growers in Mexico. Food Science & Nutrition 4(5): 690–695.

Kamarisima, Artnice Mega Fathima, Pingkan Aditiawati, et al.
 2025	Enhancing Fermented Cocoa Bean Quality and Flavor through Short Secondary Fermentation Using Candida Tropicalis as Starter and Citrus Limon Peel. Journal of the Science of Food and Agriculture 105(15): 8616–8624.

Koffi, Aka S., N’Goran Yao, Kouakou Konan, et al.( 2018)	A Numerical Model of Cocoa Beans Drying Kinetics in an Indirect Solar and Air Crossing Dryer. International Journal of Innovation and Applied Studies 23(4): 717–731.

Kongor, John Edem, Jemmy Felix Takrama, Agnes Simpson Budu, Henry Mensah-Brown, and Emmanuel Ohene Afoakwa (2013)	Effects of Fermentation and Drying on the Fermentation Index and Cut Test of Pulp Pre-Conditioned Ghanaian Cocoa (Theobroma Cacao) Beans. Journal of Food Science and Engineering 3(11): 625.

Laksono, P., Widodo, S., Fahmi, D. A., Hanapi, S., Misbah, A., Wanita, Y. P., Djaafar, T. F., Marwati, T., Siswanto, N., Hatmi, R. U., & others. (2025). Small farmers  willingness to adopt bioprocess technology for cocoa beans production in Indonesia. Sustainable Futures, 10, 101217.

León-Roque, Noemí, Mohamed Abderrahim, Luis Nuñez-Alejos, Silvia M Arribas, and Luis Condezo-Hoyos (2016)	Prediction of Fermentation Index of Cocoa Beans (Theobroma Cacao L.) Based on Color Measurement and Artificial Neural Networks. Talanta 161: 31–39.

De Melo Pereira, Gilberto Vinícius, Karina Teixeira Magalhães, Euziclei Gonzaga de Almeida, Irene da Silva Coelho, and Rosane Freitas Schwan (2013) Spontaneous Cocoa Bean Fermentation Carried out in a Novel-Design Stainless Steel Tank: Influence on the Dynamics of Microbial Populations and Physical–chemical Properties. International Journal of Food Microbiology 161(2): 121–133.

Ndukwu, MC, and M Udofia (2016)	Kinetics of Change in Colour and Some Bio-Chemical Composition during Fermentation of Cocoa Bean. Cogent Food & Agriculture 2(1): 1268743.

Nielsen, Dennis Sandris ( 2006) The Microbiology of Ghanaian Cocoa Fermentations. Københavns Universitet’Københavns Universitet’, LUKKET: 2012 Det Biovidenskabelige Fakultet for Fødevarer, Veterin\a ermedicin og NaturressourcerFaculty of Life Sciences, LUKKET: 2012 Institut for FødevarevidenskabDepartment of Food Science, 2012 Institut for Fødevarevidenskab, 2012 FødevaremikrobiologiDepartment of Food Science, Food Microbiology.

Oliveira, Marciano M, Breno V Cerqueira, Sylvio Barbon Jr, and Douglas F Barbin (2021)	Classification of Fermented Cocoa Beans (Cut Test) Using Computer Vision. Journal of Food Composition and Analysis 97: 103771.

Ouattara, Djéneba H, Honoré G Ouattara, Bernadette G Goualie, Liliane M Kouame, and Sébastien L Niamke(2014)	Biochemical and Functional Properties of Lactic Acid Bacteria Isolated from Ivorian Cocoa Fermenting Beans. Journal of Applied Biosciences 77: 6489–6499.

Ouattara, Honoré G, Ban L Koffi, Germain T Karou, et al. (2008) Implication of Bacillus Sp. in the Production of Pectinolytic Enzymes during Cocoa Fermentation. World Journal of Microbiology and Biotechnology 24(9): 1753–1760.

Pereira, Gilberto Vinícius de Melo (2012)	Microbiological and Physicochemical Performance of Cocoa Bean Fermentations Carried out in Different Systems and Screening of Yeast and Bacteria Strains toward Development of a Starter Culture.

Sangare, Abdourahamane, Edmond Koffi, Fataye Akamou, and Chekh Alassane Fall (2009)	Rapport national sur l’état des ressources phytogénétiques pour l’alimentation et l’agriculture état des ressources phytogénétiques pour l’alimentation et l’agriculture: second rapport national. Côte d’Ivoire.

Santos, Felipe A, Eduardo S Palmeira, and GJ Jesus( 2019)   An Image Dataset of Cut-Test-Classified Cocoa Beans. Data in Brief 24: 103916.

Tchouatcheu, Germaine Audrey Ngouambe, Alexandre Mboene Noah, Reinhard Lieberei, and Nicolas Niemenak(2019)	Effect of Cacao Bean Quality Grade on Cacao Quality Evaluation by Cut Test and Correlations with Free Amino Acids and Polyphenols Profiles. Journal of Food Science and Technology 56(5): 2621–2627.

Tejeda, Juan Florencio, Jessica Arango-Angarita, and Jairo Leonardo Cuervo
 2024	Impact of Solar Pre-Drying and Yeast Starter Inoculation Treatments on Volatile Compounds in Cocoa (Theobroma Cacao L.) Beans from Southwestern Colombia. Applied Food Research 4(2): 100559.

Thompson, Sterling S, Kenneth B Miller, and Alex S Lopez (2001)	Cocoa and Coffee. Food Microbiology Fundamentals and Frontiers 3.

Vincent, JC (1970)	Influence de différents traitements technologiques sur la fermentation du cacao et le goût du chocolat. Cafe Cacao The 14: 303–322.

Wahyuni, Rahman Sri, Azis Abdul, and Muhidong Junaedi
 2024	The Effect of Fermentation and Soaking Time on Water Content of Cocoa (Theobroma Cacao L.). Salaga Journal: 50–57.