PERFORMANCE EVALUATION OF OSCILLATING TYPE INTERCULTURAL EQUIPMENT FOR RICE

Abstract

Mechanical weeding is a chemical-free, environmental friendly weeding method that has been carried using mechanical tools which not only effectively controls the weeds, but also loosens the soil of the filed, simultaneously increasing the oxygen content of the soil and thus promoting the crop growth. Cone weeders are developed for weeding operation particularly in paddy cultivation. One of the major constraints in using these hand operated weeders is the physical effort that is needed to push the implement in the slush paddy fields and highly resistant clay soil. It was found that the cone-weeders are difficult to use as they have to be moved back and forth continuously while in operation for effective weed control. To overcome these problems, design of intercultural equipment with oscillating mechanism is necessary which has an advantage of low draft requirement, uprooting of weeds and bury them underneath the soil without damage to the crop.

The performance of the machine was carried under field conditions at 25 DAT. The weeding efficiency of the machine was observed as 81.39 per cent. The plant damage with machine was observed as 6 per cent. The field capacity and field efficiency of the machine was 0.30 ha h-1 and 79.12 per cent, respectively. The wheel slip of the low horse power tractor when operating in rice field with the machine was found to be 12.8 per cent, which is in permissible limit. The performance index of the machine was observed to be 104.32. The cost of weeding with developed machine was Rs. 1703 ha-1 when compared with the manual traditional weeding practice of Rs. 3050 ha-1.

Introduction

Rice (Oryza sativa L.) is India’s prominent crop, and is the staple food for most of the Indians. India has the world’s largest area under rice cultivation and is one of the largest producers of white rice, accounting for 20 per cent of global production. Transplanting is the major method of rice cultivation in India. Irrespective of the method of paddy crop establishment, weed is a major impediment to rice production through its ability to compete for resources and their impact on paddy crop yields. Weeds are known to account for nearly one third of yield reduction (Singh et al., 2005; Savary et al., 2005; Rao and Nagamani, 2007). Thus, weed control is a major prerequisite for improved rice productivity and production.

In developing countries, it is estimated that reduction in yield due to weeds alone is 20 to 30 per cent depending on the crops, weed infestation intensity and location, which might increase up to 50 per cent, if adequate crop management practices were not observed (Basavaraj et al., 2016). Mechanical weeding is a chemical-free, environmental friendly weeding method that has been carried using mechanical tools which not only effectively controls the weeds, but also loosens the soil of the filed, simultaneously increasing the oxygen content of the soil and thus promoting the crop growth (Jinwu et al., 2017).

Cone weeders are developed for weeding operation particularly in paddy cultivation. One of the major constraints in using these hand operated weeders is the physical effort that is needed to push the implement in the slush paddy fields and highly resistant clay soil. Power weeders using rotary action blades with 3.75-5 kW power capacity, these rotary blade machines have not become popular due to frequent entangling of weeds in between the cutting rotary blades in fluffy soil conditions and intermediate cleaning is required when used under higher moisture content conditions prevail especially for paddy crop. Moreover, per cent of plant damage also found high when used these types of weeders. To overcome these problems, design of intercultural equipment with oscillating mechanism is necessary which has an advantage of low draft requirement, uprooting of weeds and bury them underneath the soil without damage to the crop.

Materials and methods

The measurement of various machine and operational parameters and the procedures followed to determine the efficiency of intercultural operation, weeding efficiency, plant damage and field machine index were described. The selected paddy variety was raised in plastic trays and the seeds were uniformly sown in trays by using an automatic seed sowing machine. At the age of 21 days rice seedlings were transplanted with self-propelled riding type 6-row transplanter. The type of soil under which the crop was raised was red sandy loam (sand, silt and clay per cent were 52.79, 24.51 and 22.7, respectively). The row to row spacing of transplanted seedlings was 30 cm.

The tractor speed was kept constant at 2.16 km h-1 (0.6 m s-1) by monitoring the throttle lever at constant engine speed throughout the study and selected variables were kept at different levels. The field in which machine transplanting operation was taken up considered for these evaluation trials and identify most influential level of variables for effective weed control and least plant damage. The dependent variables considered and the levels selected for the study were described below. The rice crop selected for these trails was 25 days after transplanting.

The performance evaluation of developed mini tractor operated oscillating intercultural equipment was conducted field conditions. The observations on weeding efficiency, plant damage, field capacity, performance index were recorded.

Fig. 1. Field operational view of oscillating intercultural equipment

Weeding Efficiency

Weeding efficiency is the ratio between the number of weeds removed by a weeder to the no. of weeds present in a unit area before weeding and expressed as per cent. Number of weeds before and after each test was counted and replicated thrice and average was calculated with different stroke lengths at constant speed.

Weeding Efficiency (%) = W1-W2W1×100

Where, W1	=	 Number of weeds before weeding

W2 	=	 Number of weeds after weeding

Plant damage

Plant damage is expressed in percentage. It is the ratio between the numbers of damaged plants in a row to the number of plants present in a row before operation. Number of plants in a 10 m row length were recorded before and after operation and replicated thrice and average was calculated with different stroke lengths at constant speed (Remesean et al., 2007).

Plant damage, (%) = 1-q p×100

Where, q = Number of plants damaged in a 10 m row length after weeding

p = Number of plants in a 10 m row length before weeding

Theoretical field capacity

It is calculated from the rated field coverage that would be obtained if the equipment was performing its function at 100 per cent of the time in forward speed and covered 100 per cent of its rated width.

FCT = Wt×S 10

Where, FCT	=	Theoretical field capacity, ha h-1

Wt	=	Working width, m

S	=	Forward speed, km h-1

Effective field capacity

It is the actual area covered by the machine based on its total time consumed and actual working width under field condition. It is expressed in terms of area covered per unit time of operation. It can be calculated by formula,

FCE = A T×10000

Where, FCE	=	Effective field capacity, ha h-1

A	= 	Area covered by weeder, m2

T	=	Total time taken, h

Field efficiency

It is the ratio of the actual or effective field capacity to theoretical field capacity. It can be calculated by formula,

FE = FCEFCT

Where, FE	=	Field efficiency, %

FCE	=	Effective field capacity, ha h-1

FCT	=	Theoretical field capacity, ha h-1

Wheel slip

Wheel slip was measured by a measuring the distance covered by the intercultural equipment in grassy land and paddy field in 10 revolutions of the traction wheel using the following formula (Behera, et al., 2009).

Slippage, % =(1- n0 l  l0 n)×100

Where, lo = Distance (m) of travel with number of revolutions of wheels on grassy land.

n = Number of revolutions of wheels for a distance l (m) at the time of weeding.

Performance index of weeder

Performance of the weeder was assessed through performance index (PI) by using the following relation as suggested by Srinivas et al. (2010):

PI = FC X 100-PDX WEP

Where, FC	=	 Field capacity, ha h-1,

PD 	= 	Plant damage, per cent,

WE 	= 	Weeding efficiency, per cent, and

P 	=   	Power, HP

Cost economics of oscillating intercultural equipment

The tractor operated oscillating intercultural equipment developed was evaluated for its field performance for the estimation of cost of operation in Rs. h-1. To calculate the cost of operations some of assumptions were taken for fixed and operational costs. The cost of operation was based on the prevailing market rates during the season and location.

Results and Discussions

The developed oscillating intercultural equipment was evaluated under field conditions.

Bulk density

The soil bulk density in the experimental field was measured randomly at different locations from 0-50 and 50-100 mm depths by means of a core sampler and the bulk density was calculated as mass per unit volume. The bulk density of the experiment field varied in the range of 1.40 to 1.51 g cm-3.

Standing water level

The standing water levels were measured randomly at different locations of the experimental plot by means of steel scale. For the entire experiment, the standing water level was maintained in the range of 45-50 mm.

Row to row spacing

The row to row spacing was set as 30 cm. The rice seedlings were transplanted by the mechanical transplanter at the age of 21days.

Plant Height

The rice plants heights were observed in the range of 26.70 to 27.60 cm and 47.50 to 52.30 cm at 20 and 40 DAT, respectively. The plant height differences among the highest and lowest values of replications were also very low, which was also supported by the coefficient of variation statistical parameter. The overall plant height mean values at 20 and 40 DAT were 27.21 and 49.34 cm, respectively.

Rice hill girth

Before design of any tool that works in between the crop rows, one should have understanding of the extent of space occupied by the plant hills in crops like rice. To know the extent of spread of rice plant hills at the base the NDLR-7 rice variety hill diameter measurements were noted using vernier caliper. The rice plant hill girth diameter varied in the range of 2.5 to 3.25 cm and 5.40 to 6.50 cm at 20 and 40 DAT, respectively. The hill diameter differences among the highest and lowest values of replications were also very low.

Weeding efficiency

The weeding efficiency of oscillating intercultural equipment with optimized values was determined as per the procedure. The observed values of weeding efficiency were presented in Table 1. The recorded values of weeding efficiency at different randomized locations ranged from 78.38 to 83.56 per cent. The mean value of weeding efficiency observed was 81.39 per cent.

Table 1. Weeding efficiency with optimized variables

| S. No. | Weeding efficiency, % |
| --- | --- |
| 1. | 83.08 |
| 2. | 81.94 |
| 3. | 78.38 |
| 4. | 79.71 |
| 5. | 81.69 |
| 6. | 83.56 |
| Average | 81.39 |

Plant damage

The plant damage of the oscillating intercultural equipment operated with optimized parameters was determined following the standard procedure. The observed plant damage values recorded at different locations are presented in Table 2. The plant damage ranged from 4 to 8 per cent, with an average plant damage of 6 per cent. The relatively low level of plant damage indicates safe operation of the equipment under optimized operating conditions.

Table 2. Plant damage with optimized variable

| S. No. | Plant damage, % |
| --- | --- |
| 1. | 6.00 |
| 2. | 8.00 |
| 3. | 6.00 |
| 4. | 4.00 |
| 5. | 6.00 |
| 6. | 6.00 |
| Average | 6.00 |

Theoretical field capacity

During the experiment, the forward speed of the tractor was maintained constant at 2.16 km h⁻¹, and the effective working width of the oscillating intercultural weeder was 1.8 m. Since theoretical field capacity does not account for field losses, it is always greater than the effective field capacity.

FTC (ha h-1) = Width of operation m×Speed (km h-1 )10

= 1.8 m ×2.16 km h-1 10  =   0.388 ha h-1

Field efficiency

Field efficiency is the ratio of effective field capacity to theoretical field capacity, expressed as per cent. Field efficiency was calculated by using below formula.

FE = Effective field capacityTheoretical field capacity×100

= 0.3000.388×100

= 79.12 %

The developed oscillating intercultural weeder theoretical and field capacity were 0.388 and 0.30 ha h-1, respectively. The obtained field efficiency of 79.12% is comparable with the values reported for tractor-operated intercultural weeders in paddy fields under similar operating conditions, indicating satisfactory field performance of the developed oscillating intercultural weeder. The recorded values of effective field capacity were presented in Table 3.

Table 3. Observation values of effective field capacity

| Observations | Area, m2 | Effective field capacity, ha h-1 | Field efficiency, % |
| --- | --- | --- | --- |
| 1. | 400 | 0.300 | 77.16 |
| 2. | 400 | 0.304 | 82.33 |
| 3. | 400 | 0.296 | 78.01 |
| Average | 400 | 0.300 | 79.12 |

Wheel slippage

The wheel slip of the tractor–operated oscillating intercultural weeder was determined by measuring the distance travelled by the traction wheel for a fixed number of revolutions under different surface conditions. On grassy land, the machine covered a distance of 39 m for 10 revolutions of the traction wheel, whereas in the paddy field, the distance covered for the same 10 revolutions was 34 m. Wheel slip was calculated using the following formula:

Slippage, % =(1- n0 l  l0 n)×100

=(1- 10 ×34 39×10)×100

=(1- 0.87)×100

= 12.8 %

The slippage of the wheel was observed as 12.8 per cent, which is in permissible limit.

Performance index of weeder

The overall performance of the weeder was calculated using performance index (PI) formula as suggested by Srinivas et al. (2010).

PI = FC X 100-PDX WEP

= 0.3 X 100-6X 81.3922

= 104.32

In the present study, the effective field capacity of the weeder was 0.3 ha/h, plant damage was limited to 6% and weeding efficiency was 81.39% indicating effective weed removal with minimum damage. The wheel slip of the low horse power tractor when operating in rice field with the machine was found to be 12.8 per cent, which is in permissible limit. The performance index of the machine was observed to be 104.32.

The technical specification of oscillating intercultural equipment and evaluated parameter was given in Table 4.

Table 4. Specifications of oscillating intercultural equipment

| S. No. | Particular | Specification |
| --- | --- | --- |
| 1. | Name of machine | Oscillating intercultural equipment |
| 2. | Power source | Mini-tractor operated |
| 3. | Overall length of machine, mm | 2500 |
| 4. | Overall height of machine, mm | 800 |
| 5. | No. of weeding tynes | 6 |
| 6. | Effective field capacity, ha/h | 0.30 |
| 7. | Weeding efficiency, % | 81.39 |
| 8. | Plant damage, % | 6 |
| 9. | Field efficiency, % | 79.12 |
| 10. | Wheel slippage | 12.8% |
| 11. | Stroke length, mm | 60 |
| 12. | Chain length, mm | 150 |
| 13. | Chain thickness, mm | 5 |

Cost economics of oscillating intercultural machine

The mini tractor operated oscillating intercultural machine developed was evaluated for its field performance and cost of operation was worked out and compared with traditional method of weeding. The cost of weeding with developed machine was Rs. 1703 ha-1 when compared with the manual traditional weeding practice of Rs. 3050 ha-1. There was a saving of Rs. 1347 ha-1 over traditional method of weeding.

Conclusions

The weeding efficiency of the machine was observed as 81.39 per cent. The plant damage with machine was observed as 6 per cent. The field capacity and field efficiency of the machine was 0.30 ha h-1 and 79.12 per cent, respectively. The wheels slip of the low horse power tractor when operating in rice field with the machine was found to be 12.8 per cent, which is in permissible limit. The performance index of the machine was observed to be 104.32. The cost of weeding with developed machine was Rs. 1703 ha-1 when compared with the manual traditional weeding practice of Rs. 3050 ha-1. The developed machine found satisfactory for the intercultural operation in rice

References:

Anil Kumar, D., Joseph Reddy, S., Sanjeeva Reddy, B., Edukondalu, L and Srineevasa Rao, V. 2019. Design and Development of Oscillating Intercultural Equipment for Rice.Int.J.Curr.Microbiol.App.Sci. 8(1): 197-205.

Basavaraj., Surendrakumar, A and Durairaj, C.D. 2016. Study of agronomical and soil parameters in paddy field for development of paddy weeder. International Journal of Agriculture Sciences. 8(30): 1627-1631.

Behera, B.K., Varshney, B.P and Goel, A.K. 2009. Effect of puddling on puddled soil characteristics and performance of self-propelled transplanter in rice crop. Agricultural Engineering International: The CIGR Ejournal. X. 8-20.

Jantra C, Sukcharoenvipharat W, Usaborisut P, Thanpattranon P, Abdullakasim W. Development and performance test of a mechanical rice weeder attached to a narrow steel wheeled tractor. Int J Agric & Biol Eng, 2025; 18(2): 165–168.

Jinwu,W., Han, T., Hongguang, S., Haichao, B and Mingjun, N. 2017. Design and experiment of high clearance roll-waist multifunctional power chassis for paddy field. Transactions of the Chinese Society of Agricultural Engineering. 33(16): 32-40.

Narendra H. Tayade and Birbal Sahu. 2020. Performance Evaluation of Power Weeder for Paddy Crop. Int.J.Curr.Microbiol.App.Sci. 9(02): 2291-2295.

Ragesh K. T. 1, S. V. Jogdand and V. M. Victor (2018). Field Performance Evaluation of Power Weeder for Paddy Crop. Current Agriculture Research Journal. Vol. 6(3): 441-448

Rao, A.N. and Nagamani, A. 2007. Available technologies and future research challenges for managing weeds in dry-seeded rice in India. In: Proc. of the 21st Asian Pacific Weed Science Society Conf. 2 to 6th October, Colombo, Sri Lanka.

Remesan, R., Roopesh, M.S., Remya, N and Preman, P.S. 2007. Wet land paddy weeding- a comprehensive comparative study from south india. Agricultural Engineering International: the CIGR E journal. IX. Pp. 1-21.

Savary, S., Castilla, N.P., Elazegui, F.A and Teng, P.S. 2005. Multiple effects of two drivers of agricultural change, labor shortage and water scarcity, on rice pest profiles in tropical Asia. Field Crops Research. 91: 263-271.

# Singh, S., Singh, G., Singh, V. P. and Singh, A. P. 2005. Effect of establishment methods and weed management practices on weeds and rice in rice-wheat cropping system. Indian Journal of Weed Science. 37: 51-57.

Srinivas, I., Adake, R. V., Reddy, B. S., Korwar, G. R., Thyagaraj, C. R., Dange, A., Veeraprasad, G. and Reddy, R. 2010. Comparative performance of different power weeders in rainfed Sweet sorghum crop. Indian J. Dryland Agric. Res. and Development. 25(2): 63-67.