Original Research Article

Assessment of water quality around Parichha thermal power plant, Jhansi

ABSTRACT

The purpose of this research is to assess the water quality in the surroundings of the Parichha thermal power plant. This work determined physicochemical parameters pH, EC, TDS, Total Hardness, Ca2+, Mg2+, Total Alkalinity, and others in both underground and surface water samples collected near Parichha Thermal Power Plant in Jhansi. The correlation coefficients (r) between the various water quality characteristics of the groundwater samples analysed were determined. The results according to the water quality index (WQI) score showed that Site 1 and Site 2 cannot be used for drinking water, but the other sites 3 and 4 have good WQI scores indicating that the water quality of these sites is good and drinkable.

Keywords: WQI, Physico-chemical properties, Thermal power plant, Fly ash, Water pollution.

1. INTRODUCTION

Coal has long been utilized as a source of thermal energy and as a fuel for electricity-generating thermal power plants. The Indian energy sector has experienced massive growth in recent decades. Coal power plant leave a legacy of mass emissions of solid particles and harmful gases into the atmosphere, as well as chemical discharge, ash, and contaminated water. Near thermal power plants, substantial amounts of fly ash (FA) are produced. Depending on the source of the coal used, FA can contain a lot of heavy metals like mercury, lead, arsenic, and chromium. Infiltration of leachate or slurry leakage into neighbouring water sources is a concern in ash ponds (Du et al. 2018; Demir et al. 2019). Fly ash is leached from ash ponds in the form of a dilute slurry; the process is dependent on the slurry's pH and liquid-to-solid ratio (Lindahl and Bockstaller 2012; Lu et al. 2019).

Natural and anthropogenic contamination of surface and groundwater has put aquatic wildlife and human health in danger all over the world (Akhtar et al., 2021). Groundwater quality and availability will continue to be a major environmental concern. For the preservation and protection of this unique national asset, long-term conservation, sensible development, and management of this natural resource are essential (Deshpande and Gupta, 2004).  The most significant natural effects are geological, hydrological, and climatic, as these have an impact on the quantity and quality of available water (Sundara et al., 2010). The purpose of this study was to see how fly ash disposal affected water quality near the Parichha Thermal Power Plant in Jhansi.

2. MATERIAL AND METHODS

2.1 Study site

The Parichha thermal power station is located in the Jhansi district of Bundelkhand, India, at a latitude of 25° 52′ N and a longitude of 78° 75′ E. For this examination, water samples were taken from four separate sites: 1) the effluent water site of the thermal power plant, 2) the Parichha Dam, 3) the Parichha Ground Water, and 4) the Baratha Village Ground Water (as control site). Water sampling was done in triplicate every month from January 2020 to March 2020. Physico-chemical parameters were measured using APHA standard methods (2012).

2.2 Water quality index and statistical analysis

The WQI was calculated using Brown et al. (1970)’s "weighted arithmetic index approach." The WQI study looked at 11 environmental variables, including pH, EC, TDS, BOD, DO, total alkalinity, total hardness, calcium hardness, magnesium hardness, chloride, and turbidity. The Pearson correlation coefficient was used to determine the correlations between the 12 water-related environmental variables. SPSS 20 for Windows 10 was used for all statistical analyses.

3. results and discussion

3.1 Physico-chemical analysis

In India's arid and semi-arid regions, groundwater is one of the most important sources of water (Moghaddam et al.,2018). As a result, it is critical to assess groundwater quality to conserve it. The 12 physicochemical parameters of water samples obtained near Parichha Thermal Power Plant; Jhansi are presented in this study (Table 1). The highest water temperature and BOD was recorded from Site 1 (22°C; 9.4 mg/l) and the lowest from Site 4 (20°C; 1.34 mg/l). The pH of the four sites ranged from 7.5 to 8.7, while DO levels varied from 3.13 mg/l to 9.98 mg/l. The present findings reveal that the pH value of S2, S3 and S4 is slightly alkaline in nature and is within the acceptable range as compared to WHO (2018) limit, but the value of site 1 is slightly above the WHO limit. Due to its impact on the toxicity and solubility of pollutants, pH is a crucial metric in evaluating the quality of water. Safe water is ensured by keeping pH within the specified range, according to recent research. For example, Prest et al. (2016) stress how pH changes might affect disinfectant stability and the efficiency of water treatment procedures. For aquatic ecosystems that are impacted by organic matter, dissolved oxygen is a crucial metric. The lowest value of dissolved oxygen (3.13 mg/L) was recorded at Site 1, which is the wastewater from the thermal power plant. This indicates a decrease in oxygen levels in water bodies. Low oxygen levels (DO) in wastewater indicate high organic loads and high biochemical oxygen demand, which can have negative effects on receiving water bodies. On the other hand, S2 (7.67 mg/L), S3 (9.98 mg/L) and S4 (9.97 mg/L) showed adequate to exceptional oxygen conditions, indicating that the water quality was better away from the waste discharge site. Similar to this, Kiptui et al. (2025) found that industrially influenced rivers had lower dissolved oxygen levels (average ~4 mg/L), with the Majani-Mingi River showing a statistically significant drop (p < 0.05). According to their research, the discharge of organic waste and associated decomposition processes are responsible for the decline in dissolved oxygen levels.

The maximum turbidity value was observed in Site 1 (9.7 NTU), while the lowest turbidity value was reported in Site 4 (1.7 NTU). According to Lenntech (2023), turbidity is an indicator of the overall transparency of water or the amount of light scattered by dissolved substances in the water when a light source is shone on the water sample. Turbidity gradually decreased from S2 (4.1 NTU) to S4 (1.7 NTU), reflecting sedimentation and dilution effects away from the waste discharge site. The decreasing trend indicates that water becomes cleaner as it moves away from the source of pollution. Total hardness was measured in the range of 151.4 to 348.3 mg/l. Due to the presence of calcium and magnesium in the water, hardness led to increased soap use and scum formation. The highest calcium and magnesium hardness value was found in Site 1, 85.96 mg/l and 44.5 mg/l respectively. The lowest calcium and magnesium hardness was recorded in Site 3 (55.23 mg/l) and Site 2 (25.8 mg/l) respectively. The highest hardness value (348.3 mg/L) was recorded at S1. The increased hardness at S1 indicates possible mineral enrichment due to industrial wastewater and fly ash leachates. On the other hand, the relatively low hardness at sites S2 to S4 indicates limited anthropogenic influence and dilution effects away from the discharge source. According to earlier research, water hardness levels exceeding 300 mg/L can cause scaling in boilers, render them unfit for industrial use, and lower the efficiency of water heaters and other appliances (Memon et al., 2023; García-Ávila et al., 2022; Alemu et al., 2024). The salinity of river water is measured by TDS and EC (Singh et al., 2020). The highest EC (491 µS/cm) was obtained at site 1 while the lowest EC (223 µS/cm) was observed at site 3. Moderate EC values ​​at sites S2 and S4 (295.3 and 267.7 µS/cm, respectively) indicate consistent improvement in water quality away from the discharge source. The high EC at S1 reflects local mineral enrichment due to industrial activity, yet all recorded EC values ​​were within the permissible limits for drinking water. According to WHO (2018), a TDS of less than 600 indicates good palatability (Addisie, 2022). The current study's TDS values varied from 255 mg/L to 714 mg/L; Site 1 had the highest value, at 714 mg/L, which was higher than the WHO-recommended limit of 600 mg/L. This suggests increased dissolved solids as a result of wastewater discharge from thermal power plants. Excellent water quality was demonstrated by sites S2 (255 mg/L) and S3 (273 mg/L), while high quality was suggested by sites S4 (308 mg/L). In contrast, Kiptui et al. (2025) found that river water had significantly lower TDS levels (34–120 mg/L), all of which were within allowable bounds, indicating no industrial influence. Stronger anthropogenic influence is shown in the somewhat greater TDS at Site 1 in the current investigation. Water alkalinity can be caused by the constant presence of strong bases such as sodium hydroxide or potassium hydroxide in water, or by their extremely low concentration. Site 1 had the highest alkalinity of 557 mg/l, whereas site 4 had the lowest, 247 mg/l, exceeding the standard norm. Site 1 had the maximum chloride concentration (128), while Site 2 had the minimum (59.3).

Table: 1. Descriptive analysis of physico-chemical characteristics of different water samples.

| Parameters/Sites |  | pH | Temperature | Turbidity | E.C (µS/cm) | Alkalinity | Total Hardness | Ca2+ | Mg2+ | TDS | Chloride | D.O | B.O.D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | Mean | 8.7 | 22 | 9.7 | 491 | 557 | 348.3 | 85.96 | 44.5 | 714 | 128 | 3.13 | 9.4 |
|  | S. D | 0.6 | 3.2 | 0.7 | 76.02 | 32.4 | 1.89 | 2.5 | 2.7 | 39.5 | 4.16 | 0.35 | 0.1 |
|  | S.E. | 0.33 | 1.86 | 0.41 | 43.89 | 18.68 | 1.09 | 1.44 | 1.56 | 22.81 | 2.40 | 0.20 | 0.06 |
| S2 | Mean | 7.8 | 21 | 4.1 | 295.3 | 461 | 155.4 | 66.26 | 25.8 | 255 | 59.3 | 7.67 | 4.2 |
|  | S. D | 0.3 | 2.1 | 0.6 | 29.37 | 17.6 | 7.2 | 4.74 | 1.94 | 8.8 | 4.16 | 0.4 | 0.36 |
|  | S.E. | 0.15 | 1.20 | 0.35 | 16.95 | 10.14 | 4.15 | 2.74 | 1.12 | 5.08 | 2.40 | 0.23 | 0.21 |
| S3 | Mean | 7.5 | 21 | 2.2 | 223 | 322 | 151.4 | 55.23 | 26.1 | 273 | 73.3 | 9.98 | 2.13 |
|  | S. D | 0.3 | 2.7 | 0.1 | 37.59 | 17 | 0.59 | 0.76 | 1.01 | 8.08 | 2.52 | 0.16 | 0.29 |
|  | S.E. | 0.18 | 1.53 | 0.03 | 21.70 | 9.84 | 0.34 | 0.44 | 0.58 | 4.67 | 1.45 | 0.09 | 0.17 |
| S4 | Mean | 7.6 | 20 | 1.7 | 267.7 | 247 | 251.4 | 68.2 | 28.2 | 308 | 66.3 | 9.97 | 1.34 |
|  | S. D | 0.2 | 2.5 | 0.2 | 41.02 | 25.7 | 14.65 | 5.74 | 1.9 | 56.4 | 7.96 | 0.21 | 0.23 |
|  | S.E. | 0.12 | 1.45 | 0.11 | 23.68 | 14.84 | 8.46 | 3.31 | 1.10 | 32.58 | 4.59 | 0.12 | 0.13 |

Table: 2. Correlation Co-efficient Of Different Parameters of different water samples.

|  | pH | Temp | Turbidity | EC | Alkalinity | TH | Ca2+ | Mg2+ | TDS | Chloride | DO | BOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Temp | .485 | 1 |  |  |  |  |  |  |  |  |  |  |
| Turbidity | .878** | .323 | 1 |  |  |  |  |  |  |  |  |  |
| EC | .859** | .263 | .899** | 1 |  |  |  |  |  |  |  |  |
| Alkalinity | .757** | .201 | .894** | .797** | 1 |  |  |  |  |  |  |  |
| TH | .681* | .226 | .739** | .808** | .428 | 1 |  |  |  |  |  |  |
| Ca2+ | .774** | .297 | .837** | .834** | .652* | .871** | 1 |  |  |  |  |  |
| Mg2+ | .725** | .250 | .892** | .883** | .684* | .898** | .882** | 1 |  |  |  |  |
| TDS | .763** | .292 | .907** | .885** | .689* | .890** | .880** | .991** | 1 |  |  |  |
| Chloride | .769** | .301 | .898** | .834** | .663* | .846** | .745** | .931** | .942** | 1 |  |  |
| DO | -.860** | -.305 | -.992** | -.905** | -.905** | -.731** | -.854** | -.876** | -.894** | -.861** | 1 |  |
| BOD | .833** | .291 | .991** | .888** | .926** | .699* | .820** | .886** | .898** | .871** | -.991** | 1 |
| **. Correlation is significant at the 0.01 level (2-tailed). |  |  |  |  |  |  |  |  |  |  |  |  |
| *. Correlation is significant at the 0.05 level (2-tailed). |  |  |  |  |  |  |  |  |  |  |  |  |

3.2 Correlation analysis

The correlation research minimizes the range of ambiguity in decision-making. To determine the link between the parameters, the correlation coefficient 'r' is utilized. The correlation coefficient (r) between 2 distinct parameters was determined in this study by taking into account the mean values as indicated in table 1. According to the correlation result (table 2), turbidity revealed the strongest positive correlation with Chloride (r = 0.898**), EC (r = 0.899**), Mg²⁺ (r = 0.892**) and TDS (r = 0.907**), which showed that more suspended particles correlated with higher dissolved ionic content. Similarly, EC revealed substantial positive correlation with TH (r = 0.808**), TDS (r = 0.885**), and major cations (Ca²⁺ and Mg²⁺), which revealed that electrical conductivity is mainly affected by dissolved salts and hardness producing ions. TDS showed substantial correlation with chloride (r = 0.942**) and almost perfect correlation with magnesium (r = 0.991**), suggesting common sources such as mineral dissolution and waste discharge. It was also confirmed that the hardness is largely controlled by the high positive association with these divalent cations that is the total hardness exhibited with Ca²⁺ (r = 0.871**) and Mg²⁺ (r = 0.898**). Strong negative associations were observed between dissolved oxygen (DO) and EC (r = -0.905**), turbidity (r = -0.992**), BOD (r = -0.991**) and TDS (r = -0.894**). This suggests that oxygen availability falls as dissolved solids and organic load rise. Alkalinity, Turbidity, and TDS all showed strong positive associations with BOD, indicating that organic pollution has a major impact on the decline of water quality.

3.3 Water quality index

WQI provides single-digit figures that represent all water quality characteristics in a specific place. WQI's main goal is to simplify complex data such that it is understandable and valuable to the general population. The spatial distribution of the water quality index of all four sites is depicted in the Interpolated Inverse Distance Weighted (IDW) map shown in figure 1. Site 1 water is unfit for drinking, according to the water quality status (Brown et al. 1972), because it has a high WQI scores (158.64). While the water at site 2 (WQI =79.17) is not suitable for drinking, it can be utilized for irrigation. Sites 3 (WQI = 47.28) and 4 (WQI = 41.94) both offer acceptable water quality and can be used for drinking, agriculture, and industrial uses.

Table 3:  WQI range, status and possible usage of the water sample (Brown et al. 1972)

| WQI | Water Quality Status(WQS) | Possible usage |
| --- | --- | --- |
| 0-25 | Excellent | Drinking, irrigation, and industrial |
| 26-50 | Good | Drinking, irrigation, and industrial |
| 51-75 | Poor | Irrigation and Industrial |
| 76-100 | Very Poor | Irrigation |
| Above 100 | Unsuitable for drinking and ﬁsh culture | Proper treatment is required before use |

Table 4: Relative weights (Wn) of the parameters used for WQI determination

| Parameter | ICMR/BIS standard (Vs) | Unit weight (Wn) |
| --- | --- | --- |
| pH | 6.5-8.5 | 0.149809 |
| Turbidity | 5 | 0.2546753 |
| E.C (µS/cm) | 300 | 0.0042446 |
| Alkalinity | 120 | 0.0106115 |
| Total Hardness | 300 | 0.0042446 |
| Ca2+ | 75 | 0.0169784 |
| Mg2+ | 30 | 0.0424459 |
| TDS | 500 | 0.0025468 |
| Chloride | 250 | 0.0050935 |
| D.O | 5 | 0.2546753 |
| B.O.D | 5 | 0.2546753 |
| ∑Wn = 1 |  |  |

Table 5: Calculation of WQI in Effluent Water

| Parameters | Vn | Qn | QnWn |
| --- | --- | --- | --- |
| pH | 8.67 | 106.00 | 15.88 |
| Turbidity | 9.74 | 194.73 | 49.59 |
| E.C | 491.00 | 163.67 | 0.69 |
| Alkalinity | 557.00 | 464.17 | 4.93 |
| Total Hardness | 348.26 | 116.09 | 0.49 |
| Ca2+ | 85.96 | 114.62 | 1.95 |
| Mg2+ | 44.54 | 148.46 | 6.30 |
| TDS | 714.00 | 142.80 | 0.36 |
| Chloride | 127.67 | 51.07 | 0.26 |
| D.O | 3.13 | 119.00 | 30.31 |
| B.O.D | 9.40 | 188.00 | 47.88 |
|  | ∑QnWn = 158.64 |  |  |
|  | WQI = 158.64 |  |  |

Table 6: Calculation of WQI in Dam Water

| Parameters | Vn | Qn | QnWn |
| --- | --- | --- | --- |
| pH | 7.8 | 55 | 8.23 |
| Turbidity | 4.09 | 81.8 | 20.83 |
| E.C | 295.33 | 98.44 | 0.41 |
| Alkalinity | 461.33 | 384.44 | 4.07 |
| Total Hardness | 155.41 | 51.80 | 0.21 |
| Ca2+ | 66.26 | 88.35 | 1.50 |
| Mg2+ | 25.82 | 86.07 | 3.65 |
| TDS | 254.70 | 50.94 | 0.12 |
| Chloride | 59.33 | 23.73 | 0.12 |
| D.O | 7.67 | 73 | 18.59 |
| B.O. D | 4.2000 | 84 | 21.39 |
|  | ∑QnWn = 79.17 |  |  |
|  | WQI = 79.17 |  |  |

Table 7: Calculation of WQI in Parichha Ground Water

| Parameters | Vn | Qn | QnWn |
| --- | --- | --- | --- |
| pH | 7.47 | 31.00 | 4.64 |
| Turbidity | 2.15 | 43.00 | 10.95 |
| E.C | 223.00 | 74.33 | 0.32 |
| Alkalinity | 322.33 | 268.61 | 2.85 |
| Total Hardness | 151.44 | 50.48 | 0.21 |
| Ca2+ | 55.23 | 73.64 | 1.25 |
| Mg2+ | 26.13 | 87.09 | 3.70 |
| TDS | 273.33 | 54.67 | 0.14 |
| Chloride | 73.33 | 29.33 | 0.15 |
| D.O | 9.98 | 48.00 | 12.22 |
| B.O.D | 2.13 | 42.60 | 10.85 |
|  | ∑QnWn = 47.28 |  |  |
|  | WQI = 47.28 |  |  |

Table 8: Calculation of WQI in Baratha Ground Water

| Parameters | Vn | Qn | QnWn |
| --- | --- | --- | --- |
| pH | 7.57 | 37.00 | 5.54 |
| Turbidity | 1.69 | 33.80 | 8.61 |
| E.C | 267.67 | 89.22 | 0.38 |
| Alkalinity | 247.00 | 205.83 | 2.18 |
| Total Hardness | 251.36 | 83.79 | 0.36 |
| Ca2+ | 68.20 | 90.93 | 1.54 |
| Mg2+ | 28.20 | 93.99 | 3.99 |
| TDS | 308.33 | 61.67 | 0.16 |
| Chloride | 66.29 | 26.52 | 0.14 |
| D.O | 9.96 | 48.00 | 12.22 |
| BOD | 1.34 | 26.80 | 6.83 |
|  | ∑QnWn = 41.94 |  |  |
|  | WQI = 41.94 |  |  |

4. Conclusion

Based on the findings of this study, it can be determined that various water quality measures surpassed the WHO and BIS permissible standards for drinking water. This could be related to the fly ash dumping near Parichha Thermal Power Station. The water quality of thermal effluent water is usually polluted, and according to the WQI score of this study, the water quality of Parichha dam is also contaminated. It is no longer suitable for drinking. This means that some toxins are released from the thermal plant, causing the water quality of the Parichha Dam to deteriorate. The other two sites have good WQI scores but are in bad shape. As a result, in the current study area, it is required to minimize or reduce further deterioration of groundwater quality. The quality of potable water should be tested regularly.

Figure 1Spatial distribution of WQI in the study area.

DISCLAIMER (ARTIFICIAL INTELLIGENCE)

The authors hereby certify that this work was not written or edited using any generative ai tools, including text-to-image generators and large language models (chatgpt, copilot, etc.).

References

- Addisie MB. Evaluating Drinking Water Quality Using Water Quality Parameters and Esthetic Attributes. Air, Soil and Water Research. 2022;15. doi:10.1177/11786221221075005

- Akhtar, N., Ishak, M.I.S., Ahmad, M.I., Umar, K., Yusuff, M.S.M., Anees, M.T., Qadir, A., 2021. Modification of the water quality index (WQI) process for simple calculation using the multi-criteria decision-making (MCDM) method: a review. Water (Basel) 13, 905. doi: 10.3390/w13070905.

- Alemu CM, Aycheh YF, Angualie GS, Engidayehu SS. Modeling on comprehensive evaluation of groundwater quality status using geographic information system (GIS) and water quality index (WQI): A case study of Bahir Dar City, Amhara, Ethiopia. Water Pract Technol. 2024;19:1084-1098. doi: 10.2166/wpt.2024.076

- American Public Health Association Inc. APHA (2012). Standard Methods for the examination of Water, 22nd Edition, New York, USA.

- Brown RM, McClelland NI, Deininger RA, O’Connor MF (1972) A water quality index—crashing the physiological barrier. Indic Environ Qual 1:173–182

- Demir AEA, Dilek FB, Yetis U (2019) A new screening index for pesti- cides leachability to groundwater. J Environ Manag 231:1193–1202

- Deshpande, R.D.andGupta,S.K., 2004. Water for India in 2050: First Order Assessment of Available Options. Current Science, 86(9), 1216-1224.

- Du W, Zhang CY, Kong XM, Zhuo YQ, Zhu ZW (2018) Mercury release from fly ashes and hydrated fly ash cement pastes. Atmos Environ 178:11–18

- García-Ávila F, Zhindón-Arévalo C, Valdiviezo- Gonzales L, Cadme-Galabay M, Gutiérrez-Ortega H, del Pino LF. A comparative study of water quality using two quality indices and a risk index in a drinking water distribution network. Environ Technol Rev. 2022;11:49-61. doi: 10.1080/21622515.2021.2013955

- Kiptui, Jackline & Kebenei, Sella. (2025). Assessment of the Impacts of Sisal Processing Industries o Water Resources i. 9. 001-014. 10.5281/zenodo.15367122.

- Lenntech (2023). Turbidity. Retrieved August 4, 2023, from https://www.lenntech.com/turbidity.htm

- Lindahl AM, Bockstaller C (2012) An indicator of pesticide leaching risk to groundwater. Ecol Indic 2:95–108

- Lu J, Bai Z, Velthof GL, Wu Z, Chadwick D, Ma L (2019) Accumulation and leaching of nitrate in soils in wheat-maize production in China. Agric Water Manag 212:407–415

- Memon YI, Qureshi SS, Kandhar IA, et al. Statistical analysis and physicochemical characteristics of groundwater quality parameters: A case study. Int J Environ Anal Chem. 2023;103:2270-2291. doi: 10.1080/03067319.2021.1890064

- Moghaddama, A., Moteallemib, A., Joulaeic, F. and Peirovi, R. (2018). A spatial variation study of groundwater quality parameters in the Gonabad Plain using deterministic and geostatistical models. J. of Desalination and Wat. Treat. 103: 261-269.

- Prest EI, Hammes F, van Loosdrecht MCM, Vrouwenvelder JS. Biological stability of drinking water: Controlling factors, methods, and challenges. Front Microbiol. 2016;7:1-24. doi: 10.3389/fmicb.2016.00045

- Singh, G., Patel, N., Jindal, T., Srivastava, P., Bhowmik, A., 2020. Assessment of spatial and temporal variations in water quality by the application of multivariate statistical methods in the Kali River, Uttar Pradesh India. Environ. Monit. Assess. 192 (394), 1–26. doi: 10.1007/s10661-020-08307-0.

- Sundara Kumar K., Sundara Kumar P., Dr. M. J. Ratakanth Babu and Dr. Rao. “Assessment and mapping of ground water quality using geographical information system”. International Journal of Engineering Science and Technology. Vol. 2(11), 2010, 6035-6046.

- WHO. (2018). A global overview of national regulations and standards for drinking-waterquality. World Health Organization.