Bioaccumulation of Total Petroleum Hydrocarbon in Tympanotonos fuscatus and Crassostrea gasar from the upper reaches of the Bonny Estuary

ABSTRACT

This study assessed temporal variations of Total Petroleum Hydrocarbons (TPH) in water, sediment, Oyster (Crassostrea gasar), and Periwinkle (Tympanotonos fuscatus) from the Iwofe River, Niger Delta. Samples collected bimonthly from July 2018 to May 2019 were analyzed using gas chromatography. Sediment recorded consistently higher TPH concentrations than water, peaking in January (8.89 mg/kg), while water values remained low (0.01–0.02 mg/L). Oyster accumulated higher and more variable TPH levels (0.002–0.029 mg/kg) than Periwinkle (0.003–0.005 mg/kg), with significant temporal changes observed only in Oyster. Seasonal comparisons showed no significant difference for Oyster but a minor yet significant variation for Periwinkle. The overall pattern confirmed a clear accumulation hierarchy of sediment > Oyster > Periwinkle > water. These findings highlight sediments and Oysters as sensitive indicators of petroleum contamination and the importance of using multiple matrices to better understand hydrocarbon dynamics in polluted coastal ecosystems. The elevated sediment and Oyster concentrations underscore potential ecological risks and reinforce the need for continuous monitoring and improved management in oil-impacted environments such as the Niger Delta.

Keywords: Total Petroleum Hydrocarbon, Bioaccumulation, Tympanotonos fuscatus, Crassostrea gasar, Sediment and Water

INTRODUCTION

Petroleum contamination remains one of the most persistent environmental problems affecting coastal and estuarine ecosystems, especially in regions with intensive oil exploration and production activities such as the Niger Delta (Nwankwo & Ogamba, 2019). Total Petroleum Hydrocarbons (TPHs) are a broad group of petroleum-derived compounds that pose ecological and public health concerns due to their persistence, toxicity, and tendency to bioaccumulate in aquatic organisms (Abdel-Shafy & Mansour, 2016). When discharged into aquatic environments through spills, runoff, industrial effluents, or maritime activities, TPHs become incorporated into sediments and water columns, where they may enter the food chain (Dosunmu et al., 2020).

Shellfish such as Oyster (Crassostrea spp.) and Periwinkle (Tympanotonos spp.) are recognized bioindicators of hydrocarbon pollution because of their sedentary nature, filter-feeding behavior, and close interaction with contaminated sediments (Adebiyi et al., 2018). These organisms readily accumulate hydrocarbons from their surroundings, making them useful sentinels for monitoring pollution levels and assessing potential risks to consumers (Otitoloju & Don-Pedro, 2017). Their ecological and socioeconomic importance in coastal communities further underscores the need to understand contaminant dynamics in these species.

Seasonal changes, hydrodynamic conditions, and pollutant input patterns strongly influence hydrocarbon availability and uptake in aquatic organisms (Okoro et al., 2021). Dry and wet seasons often differ markedly in terms of runoff intensity, dilution capacity, temperature, and organic load, which may affect the concentration of TPHs in water bodies and in organisms inhabiting them. Investigating these temporal variations provides insight into the environmental processes governing pollutant distribution and helps in evaluating the health status of aquatic ecosystems.

This study therefore assessed the Bioaccumulation of Total Petroleum Hydrocarbon in water, sediment and Tympanotonos fuscatus and Crassostrea gasar across different months and seasons within the study area. By comparing the levels detected during dry and wet seasons and evaluating species-specific accumulation patterns, the study contributes to a better understanding of hydrocarbon behavior in coastal environments and supports environmental monitoring, seafood safety assessment, and resource management.

MATERIALS AND METHODS

Description of Study Site

The study area is located at 4.0893610N and 6.9286670E, along the Iwofe River, which is heavily stressed by extensive anthropogenic activities like drilling and other bunkering activities. The study area is also connected with an underground drainage culvert network where waste water, sewage, sludge, human bodily wastes from the neighboring communities and Rumuolumeni campus of Ignatius Ajuru University are carried in to the tidal stream.

Collection of Sub surface Water Samples

Water samples were collected using a clean 500 ml amber glass bottles for TPH analysis by dipping it 10 cm below the water surface. The bottles were covered with foil paper before they were sealed with the lid. Water samples were collected bimonthly over a period of 12 months (July 2018 – May 2019).

Collection of Sediment

Sediments samples were collected bimonthly from each of the sampling stations for 12 months. Sediment sample was collected at a depth of 4-5 cm. Method of sediment collection was the destructive sampling of the littoral zone, using a wooden spatula. Sediment samples were wrapped in foil papers and placed in labeled polythene bags for preservation in the freezer (00C) prior to sediment analysis.

Collection of Crassostrea gasar and Tympanotonos fuscatus for TPH Analysis

T. fuscatus of marketable sizes (average weight of 14.65 g, 11.95 g and 20.0 g respectively) were handpicked at the swamps during low tide. C. gasar was harvested by cutting the roots of the mangrove trees on which they were found. B. africanus was collected using a scoop net from tidal pools in the study area. Samples for TPH and PAH were stored in the freezer (00C) prior to analysis while enzyme analysis was carried out immediately after harvesting.

Determination of TPH and PAH

Whole samples of aquatic biota were oven dried at 600C for 24 hours, until there was no trace of moisture. The dry specimen was pulverized and sieved through a 2mm size sieve.

Extraction – 2 grams of dry aquatic biota samples were weighed into clean extraction container. 20ml of extraction solvent (1:1, hexane dichloromethane mixture) was added to the sample, mixed thoroughly and filtered through solvent-rinsed chromatographic column containing glass wool, a slurry of 2g activated silica in 10ml dichloromethane and 0.5cm of sodium sulphate at the top layer. The eluent was collected into 10ml graduated sample vials. Solvent extracts were concentrated to 1ml and 1µl was injected into the gas chromatography using a hypodermic syringe. From the gas chromatography, TPH was analyzed by applying a TPH program.  TPH chromatograms were generated for each sample; these printouts reflected ionizations peaks which were the concentration of each constituent aliphatic (TPH) hydrocarbons.

Data Analysis

Data analysis was done in form of descriptive and inferential statistics.

The descriptive statistical tools adopted for this analysis were the means and standard deviations.

The inferential statistical tool applied were the analysis of variance (ANOVA) and T-test.

A post-hoc test was conducted using Tukey High Significant Difference method to separate pair of means that were significantly different.

The results were then presented in the forms of tables.

RESULTS AND DISCUSSION

Table 1: Mean TPH Concentration in Water and Sediment in Months of Study

| Month | Sediment | Water |
| --- | --- | --- |
| Nov | 3.19±0.99 | 0.0141±0.01 |
| Jan | 8.89±6.09 | 0.0096±0.00 |
| March | 3.31±0.39 | 0.0074±0.01 |
| May | 1.4685±0.79 | 0.0108±0.00 |
| July | 0.5765±0.48 | 0.0093±0.00 |
| Sept | 2.67±0.17 | 0.0191±0.01 |

Table 2a: ANOVA and Mean Separation of TPH levels in the Water and Sediment Samples for Dry Season

| Month | Sediment | Water |
| --- | --- | --- |
| Nov | 3.190a | 0.0141 a |
| Jan | 8.890a | 0.0096 a |
| March | 3.310a | 0.0074 a |
| P-value | 0.33 | 0.451 |

*Pair of mean with dissimilar superscript is significantly different across the column

Table 2b: ANOVA and Mean Separation of TPH levels in the Samples for Wet Season

| Level | Sediment | Water |
| --- | --- | --- |
| July | 0.58 a | 0.01a |
| May | 1.47a | 0.01 a |
| Sept | 2.67 a | 0.02a |
| P-value | 0.07 | 0.26 |

*Pair of mean with dissimilar superscript is significantly different across the column

Table 3: Average TPH Concentration in the Matrix in the Months of Study

| Month | Oyster | Periwinkle |
| --- | --- | --- |
| Nov | 0.005±0.00 | 0.003±0.00 |
| Jan | 0.021±0.00 | 0.005±0.00 |
| March | 0.025±0.01 | 0.003±0.00 |
| May | 0.029±0.01 | 0.004±0.00 |
| July | 0.002±0.00 | 0.003±0.00 |
| Sept | 0.003±0.00 | 0.004±0.00 |

Table 4a: ANOVA and Mean Separation of TPH levels in the Samples for Dry Season

| Month | Oyster | Periwinkle |
| --- | --- | --- |
| Nov | 0.005b | 0.003a |
| Jan | 0.021a | 0.005a |
| March | 0.003a | 0.003a |
| P-value | 0.0006* | 0.3173 |

*Pair of mean with dissimilar superscript is significantly different across the column

Table 4b: ANOVA and Mean Separation of TPH levels in the Samples for Wet Season

| Level | Oyster | Periwinkle |
| --- | --- | --- |
| July | 0.002b | 0.003 a |
| May | 0.029a | 0.004 a |
| Sep | 0.003b | 0.004 a |
| P-value | 0.0038* | 0.7064 |

*Pair of mean with dissimilar superscript is significantly different across the column

Table 5: ANOVA and Tukey Mean Separation of TPH between Matrix by Season

| Matrix | TPH_Dry | TPH_ Wet | Wet and Dry |
| --- | --- | --- | --- |
| Oyster | 0.017±0.009 a | 0.011±0.014 ab | 0.014±0.012 a |
| Periwinkle | 0.004±0.002 a | 0.004±0.002b | 0.004±0.002 a |
| P-Value | 0.3899 | 0.0061* | 0.403 |

The present study demonstrates a consistent partitioning of total petroleum hydrocarbons (TPH) between environmental matrices, with sediments acting as the primary reservoir relative to the overlying water column. Sediment TPH concentrations (0.58–8.89 mg/kg) were markedly higher than those measured in water (0.01–0.02 mg/L) throughout the study period. This distribution reflects the hydrophobic and lipophilic nature of petroleum hydrocarbons, which preferentially adsorb to fine-grained sediments and organic matter (Abdel-Shafy & Mansour, 2016). Comparable sediment-dominated accumulation has been widely reported, with sediment TPH concentrations ranging from 1.2–15.6 mg/kg in contaminated coastal systems (Abdel-Shafy & Mansour, 2016), 2.14–12.87 mg/kg in Niger Delta creeks (Dosunmu et al., 2020), and 3.6–18.4 mg/kg in tropical estuaries influenced by oil-related activities (Benson et al., 2021).

In contrast, the low and relatively stable TPH concentrations recorded in water are consistent with the rapid removal of hydrocarbons from the dissolved phase through volatilization, dilution, photooxidation, and microbial degradation. Wang et al. (2018) reported that 40–60% of lighter hydrocarbon fractions may volatilize within days of release, while ATSDR (1999) noted that dissolved TPH concentrations in polluted surface waters rarely exceed 0.1 mg/L. Additionally, microbial degradation rates of approximately 0.02–0.3 mg/L/day have been documented in tropical aquatic systems (NRC, 2003), further limiting hydrocarbon persistence in the water column. These processes collectively enhance the transfer and long-term retention of hydrocarbons in sediments, which may act as secondary contamination sources through resuspension and bioturbation (Eggleton & Thomas, 2004).

Seasonal assessment revealed no statistically significant differences in TPH concentrations between dry and wet seasons for either sediment or water, although sediments consistently exhibited higher mean concentrations across seasons. Similar overlapping seasonal ranges have been reported by Okoro et al. (2021), who documented sediment TPH concentrations of 5.4–10.2 mg/kg during the dry season and 3.1–8.7 mg/kg during the wet season. Zakaria et al. (2002) likewise reported sediment TPH values of 4.2–14.8 mg/kg across seasons in the Straits of Malacca, concluding that continuous hydrocarbon inputs often obscure seasonal signals. The absence of pronounced seasonal variation in the present study therefore suggests sustained, low-level anthropogenic inputs rather than episodic spill events, a pattern characteristic of chronically impacted oil-producing regions (Dosunmu et al., 2020; UNEP, 2011).

Bioaccumulation of TPH in benthic organisms was clearly species-dependent. Oysters accumulated higher and more variable TPH concentrations (0.002–0.029 mg/kg) than periwinkles (0.003–0.005 mg/kg), consistent with their contrasting feeding strategies. Filter-feeding bivalves have been shown to accumulate greater hydrocarbon burdens due to continuous filtration of suspended particulate matter, which often carries particle-bound hydrocarbons (Rainbow, 2002). Similar concentration ranges have been reported in Nigerian coastal waters, where oyster TPH concentrations ranged from 0.006–0.041 mg/kg, compared to 0.002–0.009 mg/kg in periwinkles (Adebiyi et al., 2018), and mean oyster values of 0.018 ± 0.006 mg/kg exceeded those of gastropods (0.004 ± 0.002 mg/kg) (Nwankwo & Ogamba, 2019).

Seasonal variability in oyster TPH concentrations further underscores their sensitivity as biomonitors of petroleum contamination. Otitoloju and Don-Pedro (2017) reported seasonal oyster TPH concentrations of 0.009–0.033 mg/kg, with dry-season maxima linked to reduced dilution and wet-season increases associated with runoff and sediment resuspension. Comparable seasonal ranges (0.011–0.038 mg/kg) were reported by Benson et al. (2021) in estuarine systems influenced by oil exploration activities. In contrast, periwinkles consistently exhibited low TPH concentrations with minimal seasonal variability, rarely exceeding 0.01 mg/kg, suggesting lower exposure or more efficient depuration mechanisms (Adebiyi et al., 2018; Rainbow, 2002).

The lack of a significant difference in oyster TPH concentrations between dry and wet seasons indicates year-round exposure, consistent with findings by Dosunmu et al. (2020), who reported annual mean oyster TPH concentrations of 0.021–0.026 mg/kg with no clear seasonal separation. Although periwinkles showed a statistically significant seasonal difference in the present study, absolute concentrations remained low and within ranges (0.003–0.015 mg/kg) previously reported for minimally impacted gastropods (Zakaria et al., 2002; Benson et al., 2021).

However, the TPH concentrations recorded in sediments and biota fall within ranges reported for chronically impacted but non-catastrophic coastal environments. According to FAO/WHO (2016), petroleum-related hydrocarbon concentrations in seafood below 1 mg/kg are generally considered to pose low immediate risk to consumers, while ATSDR (1999) indicates that concern for chronic exposure increases when dietary intake exceeds approximately 0.05 mg/kg/day. Although TPH levels in edible organisms in this study were well below these thresholds, the persistence of hydrocarbons in sediments and their bioaccumulation in oysters highlight the need for continued environmental monitoring. This is particularly important in coastal communities with high dependence on shellfish for dietary protein, where long-term, cumulative exposure may have ecological and public health implications (UNEP, 2011; NRC, 2003).

Conclusion

This study demonstrates that Total Petroleum Hydrocarbon contamination in the Iwofe River is strongly influenced by environmental dynamics and matrix characteristics. Sediment consistently served as the major reservoir for hydrocarbons, while Oyster accumulated higher and more variable concentrations than Periwinkle, reflecting differences in feeding strategies and exposure pathways. The established hierarchy of TPH accumulation sediment > Oyster > Periwinkle > water confirms the differential sensitivity of these matrices and highlights the value of combining abiotic and biotic indicators for a more accurate assessment of ecosystem health. Although water concentrations remained low, the elevated levels observed in sediment and Oyster during specific months indicate episodic pollutant input and potential ecological risks. The findings emphasize the need for continuous environmental monitoring, stricter control of pollution sources, and improved management strategies to safeguard aquatic biodiversity and public health in oil-impacted regions of the Niger Delta.

## References

Abdel-Shafy, H. I., & Mansour, M. S. M. (2016). A review on polycyclic aromatic hydrocarbons: Source, environmental impact, effect on human health and remediation. Egyptian Journal of Petroleum, 25(1), 107–123.

Adebiyi, A. O., Adeyemi, D. H., & Otitoloju, A. A. (2018). Bioaccumulation of petroleum hydrocarbons in benthic organisms from coastal waters of Nigeria. Environmental Monitoring and Assessment, 190, 1–12.

ATSDR. (1999). Toxicological profile for total petroleum hydrocarbons. Agency for Toxic Substances and Disease Registry.

Benson, N. U., Adedapo, A. E., & Fred-Ahmadu, O. H. (2021). Distribution and ecological risk assessment of petroleum hydrocarbons in sediments of tropical estuaries. Marine Pollution Bulletin, 164, 112041.

Dosunmu, O. O., Ajayi, O. O., & Ogunjinmi, A. A. (2020). Seasonal variation of total petroleum hydrocarbons in water, sediment and biota from Niger Delta coastal waters. Journal of Environmental Chemistry and Ecotoxicology, 12(2), 45–56.

Eggleton, J., & Thomas, K. V. (2004). A review of factors affecting the release and bioavailability of contaminants during sediment disturbance events. Environment International, 30(7), 973–980.

FAO/WHO. (2016). Evaluation of certain food contaminants. Joint FAO/WHO Expert Committee on Food Additives.

Goldberg, E. D., Bowen, V. T., Farrington, J. W., Harvey, G., Martin, J. H., Parker, P. L., Risebrough, R. W., Robertson, W., Schneider, E., & Gamble, E. (1978). The Mussel Watch. Environmental Conservation, 5(2), 101–125.

NRC. (2003). Oil in the sea III: Inputs, fates, and effects. National Academies Press.

Nwankwo, D. I., & Ogamba, E. N. (2019). Hydrocarbon pollution and ecological risk assessment of coastal waters in the Niger Delta, Nigeria. Marine Pollution Bulletin, 146, 764–772.

Okoro, H. K., Fatoki, O. S., Adekola, F. A., & Ximba, B. J. (2021). Distribution and sources of petroleum hydrocarbons in water and sediments of coastal environments. Environmental Forensics, 22(3), 229–242.

Otitoloju, A. A., & Don-Pedro, K. N. (2017). Integrated assessment of crude oil contamination in aquatic ecosystems using chemical and biological indices. Environmental Toxicology and Chemistry, 36(7), 1891–1902.

Phillips, D. J. H. (1990). The use of biological indicator organisms to monitor trace metal pollution in marine and estuarine environments. Applied Science Publishers.

Rainbow, P. S. (2002). Trace metal concentrations in aquatic invertebrates: Why and so what? Environmental Pollution, 120(3), 497–507.

Tolosa, I., de Mora, S., Sheikholeslami, M. R., Villeneuve, J. P., Bartocci, J., & Cattini, C. (2004). Aliphatic and aromatic hydrocarbons in coastal Caspian Sea sediments. Marine Pollution Bulletin, 48(1–2), 44–60.

UNEP. (2011). Environmental assessment of Ogoniland. United Nations Environment Programme.

Wang, Z., Fingas, M., & Page, D. S. (2018). Oil spill identification. Journal of Chromatography A, 843(1–2), 369–411.

Zakaria, M. P., Takada, H., Tsutsumi, S., Ohno, K., Yamada, J., Kouno, E., & Kumata, H. (2002). Distribution of petroleum hydrocarbons in sediments from the Straits of Malacca and adjacent areas. Marine Pollution Bulletin, 44(8), 697–708.