Original Research Article

SOIL-SITE SUITABILITY EVALUATION FOR OIL PALM IN THE SOILS OF KHAMMAM AND BHADRADRI KOTHAGUDEM DISTRICTS OF TELANGANA

Abstract

Five distinctive soil profiles (pedons) from the oil palm (Elaeisguineensis Jacq.) cultivation areas in Khammam and Bhadradri Kothagudem districts of Telangana were analyzed and categorized. The findings revealed that these soils ranged from sandy loam to sandy clay in texture and had moderate permeability. They were slightly acidic to neutral in pH, non-saline, with medium to high organic carbon content, a mixture of mineral types, moderately deep to very deep, and moderately to well-drained. These soils were evaluate soil suitability for oil palm in soils of Khammam and Bhadradri Kothagudem districts of Telangana using limitation method. The soil properties were correlated with crop suitability criteria and relatively better soils were identified for frame work development. The soils belong to Alfisols and Inceptisols soil order. According to the proposed criteria (frame work) Aswaraopeta, Apparaopeta, Gattugudem, Buggapadu and Chandrugonda were evaluated for current suitability were marginally suitable (S3) with climatic factors such as annual rainfall and relative humidity and soil fertility factors (pH, organic matter) being common limitations for all the pedons. The suitability classes can be improved if the correctable limitations (Soil fertility characteristics) are altered through soil amelioration measures. Potentially all the pedons were evaluated to be moderately suitable (S2) for oil palm production due to limitations that can be corrected such as soil fertility, drainage by following appropriate measures. Management techniques, including conservational tillage and manure application, which improve the physical and fertility qualities of the soils should be used.

Key words: Land evaluation, suitability model, Oil palm-growing soils.

- Introduction

Oil palm (Elaeisguineensis Jacq.) cultivation is a global practice, with countries such as Indonesia and Malaysia leading in palm oil production and exports. Recognizing the significance of this crop, India is actively working to expand the acreage dedicated to oil palm farming to help meet the country's growing demand for vegetable oil (Behera et al., 2019). Oil palm cultivation covers an area of 19,000 hectares in Telangana, producing approximately 1.4 lakh metric tons of fresh fruit bunches (FFB) per hectare. In India, the total crude palm oil production is around 2.8lakh metric tons per hectare, with Telangana contributing approximately 39,000 metric tons per hectare (INDIASTAT 2020-2021). Oil palm is grown in Telangana across various soil types and resource conditions, primarily under irrigated conditions in flat to gently sloping upland areas. The soils in the region vary and include red, lateritic, black, alluvial, and colluvial soils, each with differing nutrient-supplying capabilities.

Soil is vital natural resource on whose proper use depends the life supporting system of a country and the socio-economic development of its people. With the increase of demand for land, land evaluation has become more important as people strive to make better use of the limited land resources. Land evaluation is the process of assessment land performance for specified purposes. Yield of any crop is influenced by kind of soils occurring in the area, prevailing climate, topography and management levels. The crop management practices based soil and site suitability criteria may help to overcome the constraints of crop planning for maximizing the production. It also helps in appraisal of suitability of a particular crop in specific soil / area. Land suitability evaluation is the process of estimating the potential of land for land use planning (F. A. O., 1976, Sys et al., 1991, Ram Prasad et al., 20213, Pragnya et al., 2023).

These suitability models provide guidelines to decide the policy of growing most suitable crops depending on the capacities of each soil unit (Sehgal, 1986). There are number of variations in growth and disparities in the packages adopted by farmers in crop cultivation in the study area. Very little attention was paid to study these soils. Most of the studies conducted earlier were only broad based and were conducted as a part of their study of soils of country or state. There are number of variations in growth and disparities in the packages adopted by farmers. This leads nutrient imbalance in soil and ultimately results in soil degradation and declines soil productivity. So, it is essential to understand the land suitability for rice and wheat crops in order to encourage the rice- wheat production system. Keeping these considerations in view, an investigation was carried out to develop suitability evaluation for the oil palm for optimum utilization of soil and land resources of Khammam and Bhadrdri Kothagudem districts in Telangana with a better harvest index.

- Materials and Methods

The study area, depicted in Figure 1, is located between North Latitude 17°29'31.1" and East Longitude 81°07'28.8" and encompasses the central Telangana zone, specifically focusing on the oil palm growing soils in the Badradri Kothagudam and Khammam districts. This region experiences a semi-arid monsoon climate characterized by distinct rainy, winter, and summer seasons. The average annual precipitation is 1191 mm, with approximately 74 percent of this rainfall occurring during the peak southwest monsoon period, which extends from mid-June to mid-September. Temperature extremes in the research area range from as high as 39.3°C to as low as 18.3°C, with an annual average air temperature of 28°C. Relative humidity in the region averages around 64 percent. In this study, five representative soil profiles (referred to as pedons P1 to P5) were examined from locations in Aswaraopeta, Apparaopeta, Guttagudem, Buggapadu, and Chandrugonda, where all soil horizons were observable.

Figure 1. Location of the study area (Khammam and Bhadradri Kothagudem districts)

Each pedon was subjected to morphological analysis, and soil samples were collected from every horizon for the analysis of various soil properties based on standard procedures. These properties included particle-size distribution using the hydrometer method, bulk density determined by the core method, pH measured in a 1:2.5 soil water solution, organic carbon content assessed using the methods of Walkley and Black (1934) and cation exchange capacity (CEC) determined with neutral normal NH4OAc (Ashwini et al., 2016). Soil classification was conducted in accordance with soil taxonomy guidelines (Soil Survey Staff, 1998; Soil Survey Staff, 1998; and Soil Survey Staff, 2006).

Physiographically, the study area is located at an average height of 166 m above the mean sea level (MSL). It is characterized by semi arid climatic condition, with the average rainfall of 1038 mm and 1191 mm in Khammam and Bhadrdri Kothagudem districts (Table 2) respectively, of which 76 % is received during southwest monsoon, 15 % during northeast monsoon and 9 % during summer season. The study area forms a part of Deccan Plateau. The general slope of the land is from north to south and their characteristics are presented in table 1. The soil samples representing each horizon of the pedons were collected and characterized for important physical and physico-chemical properties using standard procedures. The landscape and soil site characteristics (Table 2) were used to evaluate soil suitability for oil palm as per the guidelines given in Sys et al. (1991). The land suitability has been assessed by comparing the landscape and soil characteristics with crop requirements at different limitation levels: no (0), slight (1), moderate (2), severe (3) and very severe (4) (Table 3). Thus, the evaluation was done by comparing the land characteristics with the limitation levels of the crop requirement tables (sys et al. 1993). The number and degrees of limitations suggested the suitability class of each soil for a particular crop. The potential land suitability subclasses were determined after considering the improvement measures to correct the limitations (Sys et al. 1991; FAO 1993a).

Table 1. Meteorological data of Khammam and Bhadradri Kothagudem

| Location/ Period | 1999 - 2019 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Rainfall (mm) | Temperature(0C) |  | Relative Humidity (%) |
|  |  | Maximum | Minimum |  |
| Khammam / Mean | 1038 | 32.8 | 23.3 | 63% |
| Bhadradri Kothagudem / Mean | 1191 | 32.5 | 23.3 | 64% |

- Results and Discussion

The physical characteristics of pedon soil samples have been given in table 2. The study area experiences a semi-arid monsoon climate. The soils were sandy loam to sandy clay loam and exhibit sub-angular blocky structures. Soil colour vary from dark brown to dark yellowish brown and from red to reddish brown. The soils were slightly hard to hard, friable when moist and slightly sticky when wet. Gravel content, highest in Chendrugonda, generally increases with depth. The soils were slightly acidic to slightly alkaline in reaction. Soils were non-saline in nature. Organic carbon (OC) content varied from 1.9 to 11.1 g kg-1 and CEC ranged from 13.88 to 26.3 cmol (p+) kg-1. Exchangeable bases were in the order of Ca2+ > Mg2+ > Na+ > K+.

Table 2. Physical properties of oil palm growing soils of Badradri Kothagudem and Khammam districts of Telangana– Profile samples

| Pedon No. and location | Horizon | Depth (cm) | Mechanical composition (%) |  |  | Texture | Gravel % | B.D (Mg m-3) | Sand/ Silt ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sand | Silt | Clay |  |  |  |  |
| Pedon 1. Fine- loamy, mixed, isohyperthermic, KanhaplicRhodustalfs |  |  |  |  |  |  |  |  |  |
| P1. Aswaraopeta | Ap | 0-15 | 72.6 | 8.0 | 19.4 | sl | - | 1.44 | 9.08 |
|  | Bt1 | 15-39 | 67.2 | 9.3 | 23.5 | scl | - | 1.51 | 7.23 |
|  | Bt2 | 39-68 | 55.4 | 7.4 | 37.2 | sc | - | 1.58 | 7.78 |
|  | BC | 68-115+ | 56.2 | 7.7 | 36.1 | sc | - | 1.60 | 6.94 |
| Pedon 2. Fine- loamy, mixed, isohyperthermic, TypicHaplustalfs |  |  |  |  |  |  |  |  |  |
| P2. Apparaopeta | Ap | 0-13 | 65.4 | 11.6 | 23.0 | scl | - | 1.43 | 5.64 |
|  | Bt1 | 13-29 | 62.3 | 8.0 | 29.7 | scl | - | 1.52 | 7.79 |
|  | Bt2 | 29 -50 | 59.2 | 8.2 | 32.6 | scl | - | 1.55 | 7.22 |
|  | Bt3 | 50-68 | 52.1 | 8.7 | 39.2 | sc | - | 1.64 | 5.99 |
|  | BC | 68-105+ | 61.2 | 12.2 | 26.6 | scl | - | 1.43 | 5.02 |
| Pedon 3. Fine- loamy, mixed, isohyperthermic, TypicHaplustepts |  |  |  |  |  |  |  |  |  |
| P3. Gattugudem | Ap | 0-24 | 64.8 | 11.3 | 23.9 | scl | - | 1.38 | 4.62 |
|  | Bw1 | 24-44 | 56.4 | 18.3 | 25.3 | scl | - | 1.49 | 3.08 |
|  | Bw2 | 44-77 | 62.2 | 17.7 | 22.1 | scl | - | 1.57 | 3.51 |
|  | Bw3 | 77-100 | 57.7 | 19.1 | 23.2 | scl | - | 1.57 | 3.02 |
|  | BC | 100+ | 65.0 | 16.3 | 18.7 | sl | - | 1.59 | 3.66 |
| Pedon 4. Loamy- skeletal, mixed, isohyperthermic, TypicHaplustepts |  |  |  |  |  |  |  |  |  |
| P4. Buggapadu | Ap | 0-15 | 56.2 | 6.6 | 37.2 | gsc | 29.60 | 1.39 | 6.58 |
|  | Bw1 | 15-30 | 61.7 | 10.2 | 28.1 | gscl | 45.70 | 1.48 | 6.05 |
|  | Bw2 | 30-58 | 60.5 | 11.6 | 27.9 | gscl | 54.12 | 1.54 | 5.22 |
|  | Bw3 | 58-82 | 68.6 | 12.5 | 18.9 | gsl | 63.32 | 1.56 | 5.02 |
| Pedon 5. Loamy- skeletal, mixed, isohyperthermic, VerticHaplustepts |  |  |  |  |  |  |  |  |  |
| P5. Chandrugonda | Ap | 0-26 | 64.6 | 11.5 | 23.9 | gscl | 7.75 | 1.38 | 5.62 |
|  | Bw1 | 26-44 | 61.9 | 13.3 | 24.8 | gscl | 49.30 | 1.51 | 4.65 |
|  | Bw2 | 44-70 | 63.3 | 13.6 | 23.1 | gscl | 53.78 | 1.62 | 4.65 |
|  | BC | 70-95 | 69.4 | 12.4 | 18.2 | gsl | 71.50 | 1.63 | 5.60 |

According to "Keys to Soil Taxonomy" (Soil Survey Staff, 2018), soils were classifed up to family level based on their morphological description and analytical data (Table 3). Alfisol and Inceptisol were the two soil orders represented by the soils in Khammam and Bhadradri Kothagudam. Aswaraopet, Apparaopeta placed under Alfisols and rest were under Inceptisols (Ram Prasad, M. and Govardhan, V. (2011) and Pragnya et al. 2023). Following were the soil classifications Khammam and Bhadradri Kothagudam.

Table 3. Classification of different soils in Bhadradri Kothagudem and Khammam districts (USDA Soil Taxonomy)

| P. No & location | Order | Classification |
| --- | --- | --- |
| 1. Aswaraopeta | Alfisol | Fine- loamy, mixed, iso hyperthermic, Kanhaplic Rhodustalfs |
| 2. Apparaopeta | Alfisol | Fine- loamy, mixed, iso hyperthermic, Typic Haplustalfs |
| 3. Gattugudem | Inceptisol | Fine- loamy, mixed, iso hyperthermic, Typic Haplustepts |
| 4. Buggapadu | Inceptisol | Loamy- skeletal, mixed, iso hyperthermic, Typic Haplustepts |
| 5. Chandrugonda | Inceptisol | Loamy- skeletal, mixed, iso hyperthermic, Vertic Haplustepts |

Climate, soil-sitel characteristics (weighted averages) of the soils of study area are presented in table 4. The pH of the soils varied from 7.0 to 7.4. This increase in soil pH down the slope could be due to leaching of bases from higher topography and getting deposited at lower elevations. The electrical conductivity of pedons ranged from 0.08 to 0.14 dS m-1 indicating slight salt content in these pedons.These results were similar to those of Sitanggag et al. (2006) in soils of Shikohpur watershed in Gurgaon district of Haryana. The distribution of organic carbon in these profiles is mainly associated with physiography. These findings are in conformity with those findings of Vara Prasad Rao et al. (2008) and Basava Raju et al. (2005).  The CEC in soils ranged from 16.0 to 24.0 c mol (p+) kg-1, which corresponds to clay content in the horizons.  The base saturation was found to vary from 70.17 to 80.50 per cent.

Table 4. Soil-site characteristics of oil palm growing soils of Bhadradri Kothagudem and Khammam districts

| Physiographic unit | Soil-site characteristics | Pedon 1 | Pedon 2 | Pedon 3 | Pedon 4 | Pedon 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Climate requirements (c) | Annual rainfall (mm) | 1191 | 1191 | 1191 | 1191 | 1191 |
|  | Mean temperature (oC) | 28 | 28 | 28 | 28 | 28 |
|  | Relative humidity (%) | 64 | 64 | 64 | 64 | 64 |
| Soil physical characteristics (s) (weighted averages) | Effective soil depth (cm) | 115 | 105 | 100 | 82 | 95 |
|  | Sand (%) | 62.85 | 60.04 | 61.22 | 61.75 | 64.80 |
|  | Silt (%) | 8.10 | 9.74 | 16.54 | 10.23 | 12.70 |
|  | Clay (%) | 29.05 | 30.22 | 22.64 | 28.03 | 22.50 |
|  | Soil texture | SCL | SCL | SCL | SCL | SCL |
|  | Stoniness (%) | Nil | Nil | Nil | 48.18 | 45.58 |
| Topography(t) | Slope (%) (s) | 2-3 | <2 | 0-1 | <2 | <2 |
| Wetness (w) | Drainage (w) | WD | WD | MD | MD | MD |
|  | Depth of water table (cm) | >500 | >500 | >500 | >500 | >500 |
| Soil Fertility Status (f) (weighted averages) | pH (1:2.5) | 7.4 | 7.3 | 7.3 | 7.1 | 7.0 |
|  | Salinity (EC) (dS m-1) | 0.14 | 0.08 | 0.09 | 0.08 | 0.11 |
|  | Sodicity (ESP %) | Non-sodic | Non-sodic | Non-sodic | Non-sodic | Non-sodic |
|  | CEC (cmol(p+) kg-1) | 15.99 | 20.92 | 19.82 | 24.13 | 22.85 |
|  | Organic matter (g kg-1) | 8.1 | 6.6 | 5.9 | 5.3 | 4.6 |
|  | Base saturation (%) | 77.92 | 80.50 | 75.15 | 70.17 | 71.48 |

The climate and soil-site suitability criteria for oil palm are presented in table 5. The soils with no or only four slight limitations are grouped under suitability class S1 (highly suitable); the soils with more than four slight limitations, and/or with only three moderate limitations under moderately suitability class S2; the soil with more than three moderate limitations, and /or one or more severe limitations(s) under marginally suitable class (S3); the soils with very severe limitations which can be corrected under N1 ; the soils with very severe limitations which cannot be corrected grouped under unsuitable class N2 (NBSS & LUP , 1994 and Sys et al. 1991).

Table 5: Climate and soil-site suitability criteria for oil palm

| Soil-site characteristics | Suitability class and degree of limitation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Highly suitable(S1) | Moderately suitable (S2) | Marginally suitable (S3) | Not suitable(N) |
| Climate requirements (c) |  |  |  |  |
| Annual rainfall (mm) | 2000-2800 | 1500-2000 | 1000-1500 | <1000 |
| Mean temperature (oC) | 28-33 | 27-26, 34-35 | >35 |  |
| Relativehumidity (%) | >75 | 75-60 | 50-60 | <50 |
| Months with rainfall <100 (mm) | <3 | 3-4 | >4 |  |
| Soil physical characteristics (s) |  |  |  |  |
| Effective soil depth (cm) | >100 | 75-100 | 50-75 | <50 |
| Soil texture | cl, scl, l, sil | sl, sicl, c, sic | ls, s, c (swelling) |  |
| Stoniness (%) | Nil | >15-35 | >35 | >35 |
| Topography (t) |  |  |  |  |
| Slope (%) (s) | <5-10 | 10-17 | 18-25 | >25 |
| Wetness (w) |  |  |  |  |
| Drainage (w) | WD | MW to IM | ED to P | PD |
| Depth of water table (cm) | >250 | 100-250 | <100 |  |
| CaCO3 (%) |  |  |  |  |
| Soil Fertility Status (f) |  |  |  |  |
| pH (1:2.5) | 5.0-6.5 | 6.6-8.0; 4.0-4.9 | >8.0; <4.0 |  |
| Salinity (ECe) (dS m-1) | 1-2 | 2-4 | 4-8 | >8 |
| Sodicity (ESP %) | Non-sodic | <10 | 10-15 | - |
| CEC (cmo1+kg-1 ) | >16 | <10 | <10 | <5 |
| Organic matter(g kg-1 ) | >8 | 5-8 | 3-5 | <3 |
| Base saturation (%) | >35 | 20-35 | 10-20 | <10 |

*Textural classes: s-sand, si-silt, l-loam, c-clay, e.g.sil:-silt loam

Soil-site suitability classifications for oil palm cultivation were made based on the crop ideal growth requirements in relation to climate and soil-site parameters such as drainage, flooding, surface stoniness, texture, depth and other soil properties like organic carbon, CaCO3, CEC, base saturation, pH, and EC. The overall suitability of the soils was computed based on the kind and extent of constraints for a particular unit. Soil suitability was finally established based on the amount and severity of constraints. In Bhadradri Kothagudem and Khammam, the soil site suitability parameters for an oil palm were matched with the research area soil characteristics. Each mapping unit nature and degree of constraints were assessed, and the findings were shown in Tables 6.

Table 6. Soil-site suitability evaluation for oil palm cultivation in the soils of Bhadradri Kothagudem and Khammam districts

| Soil-site characteristics | Pedon 1 | Pedon 2 | Pedon 3 | Pedon 4 | Pedon 5 |
| --- | --- | --- | --- | --- | --- |
| Climate requirements (c) |  |  |  |  |  |
| Annual rainfall (mm) | S3 | S3 | S3 | S3 | S3 |
| Mean temperature (oC) | S1 | S1 | S1 | S1 | S1 |
| Relative humidity (%) | S2 | S2 | S2 | S2 | S2 |
| Soil physical characteristics (s) |  |  |  |  |  |
| Effective soil depth(cm) | S1 | S1 | S1 | S2 | S2 |
| Soil texture | S1 | S1 | S1 | S1 | S1 |
| Stoniness (%) | S1 | S1 | S1 | S3 | S3 |
| Topography (t) |  |  |  |  |  |
| Slope (%) (s) | S1 | S1 | S1 | S1 | S1 |
| Wetness (w) |  |  |  |  |  |
| Depth of water table (cm) | S1 | S1 | S1 | S1 | S1 |
| Drainage (w) | S1 | S1 | S2 | S2 | S2 |
| Soil Fertility Status (f) |  |  |  |  |  |
| pH (1:2.5) | S2 | S2 | S2 | S2 | S2 |
| Salinity (EC) (dS m-1) | S1 | S1 | S1 | S1 | S1 |
| Sodicity (ESP %) | S1 | S1 | S1 | S1 | S1 |
| CEC (cmol(p+) kg-1) | S1 | S1 | S1 | S1 | S1 |
| Organic matter (g kg-1 ) | S1 | S2 | S2 | S2 | S3 |
| Base saturation (%) | S1 | S1 | S1 | S1 | S1 |
| Current suitability class | S3cf | S3cf | S3cwf | S3cswf | S3cswf |
| Potential suitability class | S2cf | S2cf | S2cwf | S2cswf | S2cswf |

Based on the soil suitability criteria based on NBSS & LUP (1994) and Sys et al. 1991, soils of the Khammam and Bhadradri Kothagudam evaluated to be marginally suitable (S3). Aswaraopeta, Apparaopeta, Gattugudem, Buggapadu and Chandrugonda were evaluated for current suitability were marginally suitable (S3) with climatic factors such as annual rainfall and relative humidity and soil fertility factors (pH, organic matter) being common limitations for all the pedons. Potentially all the pedons were evaluated to be moderately suitable (S2) for oil palm production due to limitations that can be corrected such as soil fertility, drainage by following appropriate measures. Management techniques, including conservational tillage and manure application, which improve the physical and fertility qualities of the soils should be used.

REFERENCES

Ashwini, A., Chitragar, Sneha, M., Vasi., Sujata Naduvinamani., Akshata, J., Katigar. and Taradevi I. Hulasogi. (2016). Nutrients Detection in the Soil: Review Paper. International Journal on Emerging Technologies. 7(2), 257-260.

Basava Raju, D., Naidu, M. V. S., Ramavatharam, N., Venkaiah, K., Rama Rao, G and Reddy, K. S. 2005. Characterization, classification and evaluation of soils in Chandragiri mandal of Chittoor district, Andhra Pradesh. Agropedology. 15: 55–62.

FAO. 1993a. Frame work for Land Evaluation. Soils Bulletin. 32, Rome.

FAO Statistics. 1976. Food and Agricultural Organisation. URL: http://www.fao.org/.

INDIASTAT. (2020 – 2021). http://www.indiastat.com/agriculture/2/stats.aspx

NBSS, LUP. Proc. National meets on soil-site suitability criteria for different crops. Feb. 7-8, 1994 held at NBSS and LUP (ICAR), New Delhi, 1994.

Pragnya M, Ram Prasad M, Jayasree G and Srinivasa Reddy I.V (2023).  characterization and classification of oil palm growing soils of khammam and bhadradri kothagudem districts of telangana. Biological Forum – An International Journal, 15(9): 932-937.

Ram Prasad, M. and Govardhan, V. (2011). Characterization and Classification of Soil and Land Resource Environs of Deccan Platue. Journal of Research ANGRAU, 39 (1&2), 1-5.

Ram Prasad, M., Govardhan, V., Surekha,K., Praveen Rao,V. and Bhave,M.H.V. (2013). Characterization and Classification of Rice Growing Soils of Central Telngana Region of Andhra Pradesh. Journal of Research ANGRAU, 41(2), 52-58.

Sehgal, J. L. 1986. Soil site suitability evaluation for land use planning. In Introductory Pedology. Kalyani publishers, New Delhi, pp. 174-201.

Sitanggang, M., Rao, Y. S., Nayan Ahmed and Mahapatra, S. K. 2006. Characterization and classification of soils in watershed area of Shikohpur, Gurgaon district, Haryana. Journal of the Indian Society of Soil Science. 54: 106–110.

Soil Survey Staff. (1998). Keys to Soil Taxonomy. Eighth edition, National Resource Conservation Centre, USDA, Blacksburg, Virginia.

Soil Survey Staff. (2006). Keys to Soil Taxonomy. U.S. Dept. Agric., Natural Resources Conservation Service, Washington D. C. Oxford & IBH Publishing Co., New Delhi.

Soil Survey Staff. (2014). Keys to soil taxonomy. Washington: Natural Resources Conservation Service and Agriculture Department.

Soil Survey Staff. 2018. Keys to Soil Taxonomy. Natural Conservation Resources Service, USDA, Blacksburg, Virginia.

Sys, C., Van Ranst, E and Debaveye, J. 1991. Land Evaluation, part 2. Methods in land evaluation. Agricultural publication No 7. Belgium.

Sys, C., Van Ranst, E, Debaveye, J and Beernaert, F. 1993. Land Evaluation, part 3. Crop requirements. Agricultural publication No. 7. Belgium.

Vara Prasad Rao, A. P., Naidu, M. V. S., Ramavatharam, N and Rama Rao, G.  2008.  Characterisation, classification of soils of different land farms in Ramachandrapuram mandal of Chittoor District in Andhra Pradesh for sustainable land use planning. Journal of the Indian Society of Soil Science. 56 (1): 23-33.

Walkley, A. and Black, C.A. (1934). Estimation of organic carbon by chromic acid titration method. Soil Science. 37,  29-34.